3GPP TSG RAN WG1 Meeting #98bis	R1-1910085

Chongqing, China, October 14 – 20, 2019

Agenda Item:	6.2.1.6

Source:	Huawei, HiSilicon

Title:	On MPDCCH performance improvement

Document for:	Discussion and Decision

Introduction

In RAN1 #98 meeting, the following agreements were concluded on MPDCCH performance improvement [1].

Agreement

A fixed subset of the candidates in the MPDCCH search space(s) are reserved for the precoder cycling fallback. The subset of candidates is designed in such a way that the UE complexity is taken into account for the following aspects:

- Number of channel estimations

- LLR storage

- Blind decodes are not increased

For comparable performance, the alternative with the smaller complexity will be selected.

Agreement

Rank-1 precoder is used for precoder cycling in distributed MPDCCH.

- FFS whether and how predefined pairs of Rank-1 precoders specified.

Agreement

The precoder set for 4Tx is the set of rank-1 precoders with indices 12, 13, 14, and 15

Agreement

For both distributed and localized MPDCCH, the granularity of precoding in the frequency domain is only 1 PRB.

Agreement

Select one of the following alternatives (to be done in RAN1#98bis):

Precoder cycling in time domain is done

- Alt1: In a pseudo-random manner

- Alt2: Sequentially and in a cyclic manner

In this contribution, further considerations on MPDCCH performance improvement are provided.

Discussion

Precoder cycling granularity in frequency domain

For distributed MPDCCH, two DMRS ports are used. Each RB uses two rank-1 precoders. As agreed in previous several meetings, the precoder set for 4Tx is the set of rank-1 precoders with indices 12, 13, 14, and 15, and the precoder set for 2Tx is the set of rank-1 precoders with indices 0, 1. Moreover, for both distributed and localized MPDCCH, the granularity of precoding in the frequency domain is only 1 PRB.

In our view, the performance of predefined pairs of Rank-1 precoders could be similar as random pairs of Rank-1 precoders. For 2Tx, predefined pairs of rank-1 precoders {0,1} and {1,0} can be cyclically used in the frequency domain. For 4Tx, predefined pairs of rank-1 precoders {12,13} and {14,15} can be cyclically used in the frequency domain

Proposal 1: Predefined pairs of Rank-1 precoders is supported.

Order of precoding cycling

In [2], it is proposed that the assigned order and pattern of the used precoder shall be changed after a given number of subframes/ PRBs/ repetitions within an MPDCCH packet, and between every MPDCCH packet, for example by initiating the precoder cycling in a pseudo-random manner.

In [3], it is proposed that all the precoder index(es) in frequency domain increase(s) (or decrease(s)) by an offset value in time direction across time domain granularities.

In [4], it is proposed that RAN1 to downselect between:

- Option 1: The sequence of precoders is selected based on the output of a binary pseudo-random sequence.

- Option 2: The sequence of precoders is defined in a deterministic manner.

In [5], it is proposed that the precoders #0 to N-1 in the precoder set are used sequentially and cyclically.

In [6], it is proposed that predefined precoding pattern can be supported for UEs in idle mode or connected mode.  For example, this could be predefined as a function of the narrowband.

In our view, for simplicity of implementation, the precoders #0 to N-1 in the precoder set are used sequentially and cyclically, but with adaptive adjustment based on the allocated number of PRBs and/or repetition numbers.

Proposal 2: The precoders #0 to N-1 in the precoder set are used sequentially and cyclically with adaptive adjustment based on the allocated number of PRBs and/or repetition numbers.

Conclusions

In this contribution, based on the agreement in previous meeting, further considerations on MPDCCH performance improvement are provided and following proposals are given:

Proposal 1: Predefined pairs of Rank-1 precoders is supported.

Proposal 2: The precoders #0 to N-1 in the precoder set are used sequentially and cyclically with adaptive adjustment based on the allocated number of PRBs and/or repetition numbers.

- References

- RAN1 Chairman notes, RAN1 #98, Prague, CZ, August 26th – 30th, 2019

- R1-1908023	MPDCCH performance improvement in LTE-MTC	Ericsson

- R1-1908524	Discussion on MPDCCH performance improvement	LG Electronics

- R1-1908830	Usage of CRS for MPDCCH	Qualcomm Incorporated

- R1-1908260	Discussion on MPDCCH performance improvement	ZTE

- R1-1908296	MPDCCH performance improvement	Nokia, Nokia Shanghai Bell