3GPP TSG RAN WG1 #98	R1- 1910519

Chongqing, China, October 14th – 20th, 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion of Support of NR Uu configuring LTE sidelink

Agenda item:	7.2.4.7

Document for:	Discussion and Decision

# Introduction

At RAN#83, a new work item “5G V2X with NR sidelink” (5G_V2X_NRSL) was approved. The WID has recently been revised ‎[1]. One of the objectives deals with the cross-RAT scenario of control of the LTE sidelink by NR Uu:

2. Specify support for NR Uu to provide control for LTE sidelink - Sidelink mode 4 as per the study outcome [RAN2, RAN1]; and - Sidelink mode 3-like RRC-configured SPS scheduling with DCI-based activation/deactivation as per the agreement in RAN1#97 [RAN1, RAN2].

The following agreements have been reached in previous meetings:

Agreements: - RRC-based activation/deactivation is not supported - DCI-based activation/deactivation is supported - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI - Z is the same timing offset in current LTE V2X specs - X>0. FFS value(s) of X, and if one or multiple values of X are possible Draft LS to RAN to be prepared in R1-1907826 (Phillippe, Huawei), which is approved with the following update: for LTE sidelink for mode 3 with final LS in R1-1907909.

Agreements: - A new RNTI is introduced to scramble the NR DCI used for scheduling LTE PC5.

Agreements: - X is dynamically indicated using a field in the DCI - FFS whether the DCI field provides an index to a table or the value of X - The minimum value of X is subject to UE capability - UE reports a single value subject to UE capability

# Discussion

## Configuration for LTE sidelink mode 4

RAN1 have already agreed that it is supported that NR Uu provides necessary semi-static configuration for mode-4 LTE SL communications and that the signalling details are up to RAN2. In our view, no further work is required in RAN1.

Observation 1: No further RAN1 work on configuration for LTE sidelink mode 4 is needed.

## Scheduling of LTE sidelink mode 3 (SPS)

RAN1 have agreed to support DCI-based activation/deactivation of LTE sidelink SPS. To set the context, Figure 1 shows what the overall message flow might look like. The exchanges between “NR UE” and “LTE UE” of course take place over an internal interface and are not subject to specification.

- The NR UE indicates to the gNB its cross-RAT SL scheduling capability, along with the required processing delay for DCI activation/deactivation (RAN1 have agreed that “Support of LTE PC5 scheduling by NR Uu (mode 3-like) is based on UE capability”)

- The gNB may provide dedicated LTE sidelink configuration, which the NR UE passes on to the LTE UE via the internal interface.

- When the LTE UE has periodic sidelink traffic to transmit it provides SL traffic pattern information to the NR UE over the internal interface; the NR UE passes this on to the gNB over NR Uu in an RRC UE assistance information message. This can re-use the existing LTE RRC IEs. The SL traffic pattern information includes packet timing information; for LTE, this is relative to the LTE Uu’s SFN, so for the cross-RAT case some communication between NR UE and LTE UE about timing may be required.

- gNB provides the LTE SL SPS configuration over NR Uu to the NR UE, which in turn passes it on to the LTE UE via the internal interface.

- Finally, the gNB activates the LTE SL SPS configuration by transmitting a DCI over NR Uu to the NR UE, which in turn passes it on to the LTE UE via the internal interface

Figure 1: Sequence diagram of what cross-RAT scheduling might look like

The RAN1 tasks to enable this are

- Design of a new NR DCI format for activation/deactivation of LTE SL SPS.

- Definition of the timing relationship between time of DCI reception on NR Uu and time of activation of the corresponding LTE SL SPS configuration (that is, time of first PSCCH/PSSCH transmission on the LTE sidelink).

### The definition of the timing relationship between DCI and SPS activation

In the pure LTE case, the processing time is fixed in the specification to 4 subframes (this corresponds to the value Z in the agreement reached at RAN1#97). In the cross-RAT scenario, however, any signalling received from the gNB over NR Uu must be forwarded from the NR module to the LTE module. This forwarding will lead to additional delay, and that delay may be highly implementation-dependent. Moreover, for the NR Uu the gNB benefits from scheduling flexibility in the time domain, in the sense that the timing offset between transmission of the DCI containing an uplink grant and the scheduled UL resources is very flexible. In detail, the DCI contains a field “time domain resource assignment”, which is an index into a table; each entry in the table contains a slot offset K2 between the slot with the scheduling DCI and the slot for PUSCH transmission. The table can be configured using RRC signalling or specified in TS 38.214. Similar scheduling flexibility is desirable for scheduling of the LTE sidelink.

Proposal 1: The DCI field indicating the timing offset X provides an index into a table of slot offsets. It is supported to configure the table by both common and dedicated RRC signalling.

The numerology and hence slot duration on the Uu downlink, which conveys the DCI, is not fixed and can be different from the time units on the LTE sidelink. Since this signalling is handled by the UE’s NR Uu module, the following is proposed:

Proposal 2: The slot offset indicated by the DCI is in units of slot for the subcarrier spacing for PDCCH.

### DCI Contents

The new NR DCI format should re-use the contents of LTE’s DCI format 5A.

Proposal 3: Re-use the contents of LTE’s DCI format 5A in the new NR DCI format for activation/deactivation of LTE SL SPS.

# Conclusions

In this contribution we discussed some aspects of NR Uu control of the LTE V2X sidelink and make the following observations and proposals:

Observation 1: No further RAN1 work on configuration for LTE sidelink mode 4 is needed.

Regarding scheduling of LTE sidelink mode 3:

Proposal 1: The DCI field indicating the timing offset X provides an index into a table of slot offsets. It is supported to configure the table by both common and dedicated RRC signalling

Proposal 2: The slot offset indicated by the DCI is in units of slot for the subcarrier spacing for PDCCH.

Proposal 3: Re-use the contents of LTE’s DCI format 5A in the new NR DCI format for activation/deactivation of LTE SL SPS.

# References

- RP-191723, “Revised WID: 5G V2X with NR sidelink”