3GPP TSG RAN WG1 Meeting #98bis 	R1-1911079

Chongqing, China, 14th – 20th October, 2019

Agenda Item: 7.2.6.2

Source: MediaTek Inc.

Title: Multiple HARQ procedures and intra-UE UCI prioritization

Document for: Discussion and Decision

# Introduction

The first part of the contribution addresses

- the complementary, simultaneous use of HARQ codebooks and codebook-less HARQ feedback. We argue that this would allow keeping the sub-slot size as large as possible (slot or half-slot) for flexible HARQ multiplexing, while meeting the latency requirements with corner case traffic patterns as well. The solution optimizes downlink and uplink spectral efficiency and potentially simplifies traffic prioritization, too.

- the solution options for the dynamic indication of HARQ-ACK codebook, and other  agreed open issues (FFS) on PUCCH configuration

The second part of the contribution addresses intra-UE traffic prioritization. The proposals take into account the latest agreements on down-scoping the potential changes to Rel-15. Priority level for SR and PUSCH are addressed. A method for delayed eMBB HARQ-ACK is proposed, which re-uses the skeleton of A-CSI report scheduling.

# Multiple HARQ-ACK feedback per slot

In RAN1#98 the following agreement was reached:

Agreements: At least one sub-slot configuration for PUCCH can be UE-specifically configured to a UE. - At least support following two sub-slot configurations for PUCCH: “2-symbol*7” and “7-symbol*2”. - FFS other configurable sub-slot configurations, e.g. 4, 14 sub-slots in a slot. - For the above two sub-slot configurations (“2-symbol*7” and “7-symbol*2”), support a single configuration for PUCCH resource following R15 applicable for all the sub-slots in a slot. - FFS whether or not to additionally support that PUCCH resource configuration can be different for different sub-slots - FFS for other sub-slot configurations, if any. - FFS: If a PUCCH resource across sub-slot boundary is supported.

Both agreements on the replication of PUCCH configurations over sub-slots and on the support for very short sub-slots (of just two OS) suggest that configured PUCCH resources should be allowed crossing sub-slot boundaries. If, however, this results for some PUCCH resources (being replicated across sub-slots) in crossing slot boundaries as well, then such PUCCH resources should be treated as invalid. Similar goes for configured PUCCH resources that overlap with DL symbols. If invalid resources are scheduled, the UE should treat them as error cases, just as the case when two otherwise valid but in time overlapping PUCCH resources are scheduled from different sub-slots.

Proposal 1: Allow PUCCH resources to cross sub-slot boundary. If the configured PUCCH resource overlaps with a slot boundary or a DL symbol then it is invalid. The UE shall treat it as an error case if an invalid PUCCH resource is scheduled or PUCCH resources overlapping in time are scheduled from different sub-slots.

Should Rel-16 support further configurability of partitioning size as raised by the agreement FFS? As we argued in [1], small sub-slots inhibit PUCCH-overriding hence flexible HARQ-ACK codebook segmentation. Instead, we propose supporting codebook-less HARQ described in the next Section. In this case, further enhancement to sub-slot partitioning needs not be considered.

Observation 1: Further enhancements to sub-slot partitioning (e.g. to sub-slot size and separate configurability of PUCCH resources per sub-slot) could be avoided by supporting complementary, codebook-less HARQ procedure.

# Complementary use of codebook-less HARQ-ACK

Agreements (RAN1#96bis): For supporting multiple PUCCHs for HARQ-ACK within a slot for constructing HARQ-ACK codebook, support sub-slot-based HARQ-ACK feedback procedure. - A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs [i.e., HARQ-ACK codebook, as was clarified in on-line session] starts in a sub-slot. - PDSCH transmission is not subject to sub-slot restrictions (if any) - FFS: PDSCH-to-sub-slot association. - FFS: Allowing PUCCH across sub-slot boundary or not. - R15 HARQ-codebook construction is applied in unit of sub-slot at least for Type II HARQ-ACK codebook. - FFS for Type I HARQ-ACK codebook. - R15 PUCCH resource overriding procedures is applied in unit of sub-slot. - Number or length of UL sub-slots in a slot is UE-specifically semi-statically configured. - FFS: Limit of number of PUCCH transmissions carrying HARQ-ACKs in a slot. - FFS: K1 definition. - FFS: Details of PUCCH resource configuration and determination. FFS: Use “Codebook-less HARQ” as a complementary or not.

We consider allowing codebook-less HARQ feedback as a simultaneous, complementary option for the gNB, i.e. as a component of a hybrid scheme. The goal is to relax the constraints on the sub-slot partitioning for the codebook-based procedure, and to arrive at configurations of larger sub-slots, so as to allow more HARQ multiplexing. As illustrated by Figure 1, in the hybrid scheme, the partitioning is decided based on the frequent patterns (e.g. 80%-percentile) rather than the infrequent ones (e.g. 99%-percentile).

Signalling and configuration overhead can be negligible as shown in the next section.

Figure 1: Flow-charts presenting sub-slot size selection; 
comparison between sub-slot based and hybrid HARQ procedures

Observation 2: Complementary codebook-less HARQ procedure can allow choosing coarser or no partitioning for the codebook-based procedure. This benefits scheduling and HARQ multiplexing.

Proposal 2: Use “Codebook-less HARQ” as a complementary procedure, simultaneously to the codebook-based procedure. The number of PUCCH’s carrying codebook-less HARQ in a (sub-)slot needs not be restricted.

## Operation details

Codebook-less HARQ mode differs from Type-2 HARQ codebook determination procedure in that:

- DAI counters are not used for HARQ codebook determination

- PUCCH resource over-riding is not applied

- the number of PUCCH’s need not be limited per (sub-)slot, (at least, not for segmentation)

- PUCCH timing and PUCCH resource assignment can be simplified (as detailed further below)

Self-recovery

A lost DCI indicating codebook-less HARQ must not cause any harm to the operation of the other HARQ procedure(s). Therefore, codebook-less HARQ mode would operate as a separate procedure, decoupled from the codebook segmentation, (as opposed to just enabling/disabling the over-riding rule). This guarantees that the DAI counter and PUCCH resource overriding mechanism of the codebook-based procedure(s) cannot be affected by a DCI reception failure.

Slot-partitioning

The codebook-based procedure could either use slots or half-slots for the codebook segmentation (as the unit of K1) and the PUCCH resource allocation. The codebook-less procedure does not use sub-slots for segmentation, the only configuration issue is the PUCCH allocation.

PUCCH allocation and assignment

By default, codebook-based procedure and codebook-less HARQ procedure may use the same PUCCH allocation. For both procedure(s) the PUCCH assignment and timing can follow the Rel-15 procedures adapted to sub-slots.

PUCCH timing

For the codebook-less procedure the Rel-15 mechanism can be used to select the sub-slot where the PUCCH resource is transmitted using a fixed K1 value, which is otherwise not used Alternatively, the sub-slot can also be inferred using the N1 UE processing timeline and the earliest feasible instance of the PRI-selected PUCCH resource from PUCCH resource set 0.

Signalling

One of the signalling options used for codebook selection can be used with codebook-less HARQ as well. However, notice that K1 indication is not needed for the timing. Hence, an economic way to select codebook-less sending is to define a special K1 index that selects codebook-less sending when high-priority HARQ transmission has been indicated. This is illustrated in Figure 3. If K1 is not inferred, then multiple special K1 indexes need to be selected.

Figure 3: Special K1 index to select between codebook-based and codebook-less HARQ

Proposal 3: To avoid signalling overheads, a configurable special K1 (index) value could select codebook-less HARQ_ACK sending.

Proposal 4: For codebook-less procedure, the PUCCH resource assigned by the PRI should be sent in the earliest sub-slot for which the PUCCH resource abides by the N1 UE timeline.

# Support of HARQ-ACK feedback for mixed URLLC and eMBB traffic in a UE

## Multiple HARQ procedures

The following agreement was made at the previous RAN1 meeting:

Agreements (RAN1#98): When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, following can be separately configured for different HARQ-ACK codebooks: - PUCCH-SpatialRelationInfo - Sub-slot configuration (only applied for the sub-slot-based HARQ-ACK codebook) - FFS whether or not to support the case when there are at least two HARQ-ACK codebooks configured with sub-slots, with the same or different sub-slot configurations

To support coexistence of one eMBB and one URLLC flow, one procedure (slot based) following the Rel-15 configuration options and another one that is sub-slot based supporting the new PUCCH configuration features would be sufficient in principle. We are not aware of presented scenarios that would justify the potential need for multiple codebooks configured with sub-slots of potentially different sub-slot sizes.

If large variations in inter-packet-arrival time exist or if there are multiple URLLC flows with very different latency (and reliability) requirements, or when operation mode change is possible (e.g. to emergency mode) then this might be accommodated by allowing a complementary codebook-less HARQ procedure, which does not involve extra burden on signalling (refer back to Figure 3) or separate PUCCH configurations.

Proposal 5: Support at most two HARQ codebook procedures, and one complementary codebook-less HARQ procedure using the PUCCH configurations of one or the other codebook-based procedure.

- If two codebooks are configured than at least one of them should follow Rel-15 configurations only.

- Any Rel-15 codebook has low-priority and any Rel-16 codebook as well as the codebook-less procedure have high-priority.

## HARQ codebook indication for dynamically scheduled PDSCH

Agreements from RAN1#96bis: When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, for both Type I (if supported) and Type II HARQ-ACK codebooks (if supported), and for dynamically-scheduled PDSCH, down-select from below for the PHY identification for identifying a HARQ-ACK codebook: - Opt.1: By DCI format - Opt.2: By RNTI - Opt.3: By explicit indication in DCI (FFS: new field or reuse existing field) - Opt.4: By CORESET/search space - FFS additional option(s) for Type I HARQ-ACK codebook FFS: For SPS PDSCH (including SPS release PDCCH) Agreements RAN1#98: When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, the PHY identification of HARQ-ACK codebook is also used to determine the priority of the HARQ-ACK codebook for collision handling.

Introducing a new DCI field (Opt-3a), increases the size, hence slightly impairs the reliability, which should be handled with scrutiny. All the other options avoid that, but each solution comes with its own drawback, as we discuss below.

Opt.1 by DCI format: causes either inflexibility to the scheduling or requires additional CCE’s/BD’s for PDCCH monitoring by the UE:

- Both codebook-based and codebook-less HARQ feedback should be able to use the conventional DCI format to minimize the number of formats. E.g. compact DCI might cause some restriction why URLLC traffic needs to use DCI 1_1, or e.g. it would be advantageous to use compact DCI with eMBB traffic when PDCCH scheduling is a bottle neck.

- Otherwise, DCI size alignment would be required to maintain the complexity of PDCCH monitoring by the UE.

Opt.2 by RNTI:

- If new RNTI’s are introduced then that increases the false alarm probability

- If MCS-C-RNTI is tied in with low latency HARQ feedback then this represents considerable restriction for the scheduling flexibility:

  - If URLLC needs to use MCS-C-RNTI for the MCS table, it must use the low latency HARQ feedback even when no more retransmission occasions are available. Normally, in such a case URLLC would use the same codebook as eMBB, and HARQ-ACK would only be used for gathering statistics by gNB.

  - URLLC cannot be scheduled with high-efficiency MCS table and use low latency HARQ codebook simultaneously

Opt.3 by explicit indication in DCI (new DCI field or reuse an existing DCI field):

Advantage: Explicit indication allows flexible scheduling and reliability is not impaired.

3a) new DCI field: this would increase the DCI size by an extra 1 bit, which affects reliability. Backward compatibility with Rel-15 can also be an issue.

These drawbacks can be alleviated, if really necessary, by reusing existing fields.

3b) reuse HARQ process ID field:

This solution consists of mapping HARQ processes ID’s to ‘slow’ or ‘fast’ codebooks within a sub-slot. Thus the HARQ process ID field is used for codebook indication as well, which implies that each HARQ procedure has a configurable allowance of HARQ process ID’s.

Note: Further (hierarchical) signalling to select codebook-less HARQ-ACK feedback could employ a reserved/special value of K1 or K1 index as explained earlier in Figure 3.

Rationale: The solution is not restrictive for the scheduler, does not require more soft-buffer, and does not impact HARQ process ID bitwidth either because of the following:

- Currently 16 HARQ processes are supported: this is twice the number of maximum HARQ processes in LTE! So, even if 8 HARQ processes were reserved to URLLC this would still leave plenty of processes for eMBB. However, this is far from being the case. The number of required HARQ processes is correlated to the HARQ Round Trip Time. Since RTT is short in URLLC, it only requires a few HARQ processes (e.g. 1-4). Notice that the eMBB downlink throughput is already restricted by the URLLC downlink traffic, and TCP-IP behavior also favors lower RTT for eMBB HARQ feedback, hence less eMBB HARQ processes are sufficient.

- Without this signaling method, URLLC takes the HARQ processes it requires. In theory, eMBB can use all the remaining processes. However, note that ‘borrowing’ HARQ processes cannot really work without this method either, because URLLC and eMBB usage fluctuate on a different time scale due to the difference in RTT. Without safe, fixed allowances it could also occur that all the HARQ processes are already in use when a new URLLC downlink transmission needs to be scheduled. In this case an eMBB HARQ process needs to be terminated by toggling the NDI, which, in turn, causes RLC overhead. Therefore, when this method configures a fixed allowance for URLLC, it suppresses only a small fluctuation in the number of processes available to eMBB.

- It also follows that HARQ process ID indication bitwidth is not impacted either by the fix allowances. In Rel-15 the HARQ process ID bitwidth is fixed (4 bits).

Hence, fixed allowances between ‘slow’ and the ‘fast’ HARQ codebooks can have but negligible impact on eMBB throughput, and would probably be applied anyways by the gNB scheduler.

Opt.4: By CORESET/search space: also introduces unnecessary scheduling constraint, and it can potentially increase the number of CCEs/BDs that a UE needs to monitor.

From the above list, option 3) has the flexibility to down-select the optimal trade-off. Option 3b), is the most efficient solution.

Proposal 6: For dynamically scheduled PDSCH, use explicit indication in DCI for the PHY identification for identifying a HARQ-ACK codebook when at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE.

- FFS: introduce new field or reuse existing field: HARQ_process_ID.

## HARQ codebook indication for SPS PDSCH

Agreements RAN1#98: When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, - In case of SPS PDSCH, the following options for identifying a HARQ-ACK codebook (to down-select, combinations are not precluded) - Opt.1: By SPS PDSCH configurations - Opt.2: By the DCI activating the SPS PDSCH - Opt.3: By the CORESET where the activating DCI is received

Unlike the other options, Opt-1 has no real drawback and eliminates all coupling with the decision for dynamic grant.

Proposal 7: With SPS PDSCH the HARQ codebook should be selected based on semi-static SPS PDSCH configuration.

# Intra-UE multiplexing and prioritization of UCI

In RAN Plenary #85 the enhancements for intra-UE multiplexing and prioritization have been down-scoped to the potential cases for prioritization behaviour in the case of resource collisions between different priority level traffic. In all other cases Rel-15 handling is maintained.

Table 1: Labels used with collision resolution scenarios

|  | URLLC SR | URLLC HARQ-ACK | CSI | URLLC PUSCH |
| --- | --- | --- | --- | --- |
| URLLC HARQ-ACK | Scenario-01 |  |  |  |
| CSI | Scenario-02 | Scenario-03 |  |  |
| URLLC PUSCH | Scenario-04 | Scenario-05 | Scenario-06 |  |
| eMBB SR | Scenario-07 | Scenario-08 | Scenario-09 | Scenario-10 |
| eMBB HARQ-ACK | Scenario-11 | Scenario-12 | Scenario-13 | Scenario-14 |
| eMBB PUSCH | Scenario-15 | Scenario-16 | Scenario-17 | Scenario-18 |
| Note: Scenarios open for discussion are underlined. colours match scenarios similar in channel types. |  |  |  |  |

## HARQ-ACK

### URLLC HARQ vs eMBB HARQ (scenario-12)

Since the collision scenario does not exist in Rel-15 and the current WID has been down-scoped to introduce prioritization where needed, it follows that the lower priority HARQ transmission needs to be dropped. In [1] we presented arguments why this is the best option, too.

Proposal 5-1: When two HARQ transmissions of different priority levels collide in time, drop the – possibly started – low-priority HARQ-ACK transmission and transmit the high-priority HARQ-ACK.  (Scenario-12)

Rather than dropping eMBB HARQ codebook definitely, HARQ-information should be recovered by sending the codebook in full in a later slot. A possible solution for the delayed sending is that HARQ codebook gets queried by gNB similarly to a CSI report over UL-DCI/PUSCH. The special grant or RRC configuration required for this could reuse the existing UL-DCI format used with A-CSI. An alternative solution would be to use PUCCH for the delayed sending, which for small codebook sizes could result in a more efficient encoding, but the insignificant gains would be outweighed by the standardization difficulties of an appropriate solution: we only refer to the fact that PUCCH resource set selection is based on size. Any simplifying solution could mean that the efficiency advantage is lost. A further advantage of UL-DCI is that the uplink retransmission mechanism can handle further de-prioritization or unsuccessful decoding by regular PUSCH retransmissions.

It is important to note that delayed eMBB HARQ sending could be used in other collision scenarios where eMBB is dropped, too, as opposed to PUCCH multiplexing, which only addresses the HARQ vs. HARQ collision. Even if multiplexing is supported, the increased eMBB HARQ codebook decoding failure rate could call for delayed re-sending.

Proposal 5-2: The UE should store any HARQ codebook transmitted on PUCCH that has been deprioritized (dropped), along with the slot/sub-slot, for later re-sending in full.  FFS: memory depth.  (Scenario-12, …)

Proposal 5-3: For querying the UE’s history of deprioritized HARQ codebooks (Scenario-12, …), reuse Rel-15 UL-DCI features used for scheduling A-CSI measurement and reporting. FFS: precise report format

### HARQ vs P/SP-CSI (scenarios 03 and 13)

Multiplexing URLLC HARQ-ACK information with P/CSI can result in late HARQ feedback. Although this may be countered by single-shot transmissions, however, the collision can occur way too often, severely impacting downlink efficiency. Therefore, URLLC HARQ should be prioritized over P-CSI.

Assuming one URLLC HARQ codebook on average per slot (i.e. less than 0.5 ms average packet inter-arrival time at 30 kHz, as an extreme worst case), the chance that P/SP-CSI gets dropped can be in the order of 30% in certain cases. In the case of long CSI period, this could be compensated for by reduced CSI periodicity. On the other hand, with frequent CSI the dropping can be tolerated because RI, PMI vary more slowly and the occasional loss of CQI do not have dramatic affect. In typical scenarios the occurrences have lower probability and the losses are more affordable.

Proposal 5-4: When high-priority HARQ-ACK collides with P/SP-CSI (scenarios 03 and 13), then UE should drop CSI and transmit HARQ.

Meanwhile Rel-15 handling should be maintained for “eMBB”/”slow” HARQ codebook, based on the PHY indication signal that will support the codebook selection. As a side benefit, when multiplexing can meet URLLC HARQ latency and reliability, and a “slow” codebook is not yet scheduled in the current slot, the URLLC DL-DCI could select HARQ/CSI multiplexing by indicating “slow” codebook. This could further reduce the probability that CSI is dropped.

## SR colliding with HARQ/CSI (scenarios 01, 02, 07, 08, 09, 11)

### SR colliding with HARQ (Scenarios 01, 08 and 11)

Table 3: Rel-15 multiplexing rules between SR and HARQ

| Collision case | SR with PF0 | SR with PF1 |
| --- | --- | --- |
| HARQ-ACK with PF0 | Transmit both HARQ-ACK and SR on PF0 for HARQ-ACK | Transmit both HARQ-ACK and SR on PF0 |
| HARQ-ACK with PF1 | Transmit HARQ-ACK on PF1, drop SR | Transmit HARQ-ACK on PF1 for SR when SR is positive; transmit HARQ-ACK on PF1 for HARQ-ACK when SR is negative. |
| HARQ-ACK with PF2 | Transmit both HARQ-ACK and SR on PUCCH resource for HARQ-ACK based on UCI payload and PRI |  |
| HARQ-ACK with PF3 |  |  |
| HARQ-ACK with PF4 |  |  |

We first consider collision with HARQ transmitted in PF 0/1. Here, Rel-15 rules are not optimal but can be maintained:

- Rel-15 rule for multiplexing PF1 SR with PF1 HARQ preserves both reliability and latency of SR.

- Rel-15 rule for multiplexing PF0 SR with PF0 HARQ, preserves both reliability and – if properly used – latency of SR.

- Rel-15 rule for dropping PF0 SR when it collides with PF1 HARQ is harmful but could be circumvented. This rare scenario can only be produced when a downlink URLLC flow has much higher reliability target than uplink URLLC flow. Either a PF1 is configured for SR to avoid it, or the SR period is shortened. In the latter case, the prohibit timer will only start after SR gets sent and the gNB will assume that SR collided with HARQ and reduce the delay budget by one SR period.

- Rel-15 rule to multiplex PF1 SR with PF0 HARQ on HARQ resource may compromise SR reliability. In cases when it does, it should not be used.

  - Encoding of positive/negative SR on two symbol PF0 can be as reliable as the on/off encoding of positive SR on PF1 on four symbols.

  - If SR is frequent then HARQ reliability might need to be adjusted by using PF1 instead of PF0, else if PF1 SR is infrequent then changing the rule from multiplexing to SR prioritization could enhance efficiency.

In the case of more than two HARQ bits, scheduling PF2 HARQ around SR can mitigate situations when multiplexing would fail to meet reliability. When multiplexing, the scheduler has the choice to opt for a more robust PF3 or PF4 resource. Although URLLC SR reliability target might be stricter than that of URLLC HARQ codebook, however, multiplexing over a more robust resource is still the most efficient solution unless the chance of positive SR is negligible. Note that if PDCCH fails, SR gets transmitted; else SR+HARQ is transmitted with the same false-negative-SR probability as NACK-to-ACK probability by default.

When Reed-Muller encoding is used and CRC is not attached, it is still possible to bias SR detection towards positive SR. Hence, e.g., if we assume a 10^-6 overall reliability target in both uplink and downlink, NACK-to-ACK probability may target e.g. 10^-5, with 0.5% retransmission per packet, whereas SR decoding could favour a 5*10^-7 positive-to-negative error rate at the expense of a 2-5% negative-to-positive false SR rate.

Remark: It may occur that the HARQ reliability target is much lower than the SR target: e.g. when multiple retransmission occasions are available. In such situations scheduling PF2 around SR occasions can be a solution.

Finally we note that Rel-15 handling of collision between URLLC SR and eMBB HARQ is preferable to dropping eMBB HARQ (unless delayed HARQ transmission is supported, as in the proposal), since one or the other of the following two options to avoid multiplexing (when URLLC requirements cannot be met) results in smaller sacrifices to efficiency:

- The scheduler can assume positive SR without waiting for the UE’s transmission. Trading off uplink- for downlink resource utilization.

- The scheduler may opt to segment the HARQ-ACK feedback into more but smaller codebooks, and schedule these on PF2, so that scheduling around SR occasions becomes feasible.

eMBB SR colliding with URLLC HARQ are handled safely using Rel-15 rules, which already handles HARQ as higher priority than SR. Furthermore, eMBB SR occasion periodicity is relatively long, because it is not sensitive to latency, and thus the probability of eMBB SR – URRLC HARQ collision is low.

Proposal 5-5: Maintain Rel-15 handling rules between HARQ and SR irrespective of their PUCCH formats and priority levels. (Scenario 01, 08, 09 and 11).

### URLLC/eMBB SR colliding with CSI (Scenarios 02 and 09)

CSI parameters can vary considerably in measurement content and periodicity as well as report size. In general, multiplexing URLLC SR with CSI risks a considerable loss in reliability and/or latency. Therefore, dropping CSI should be considered when colliding URLLC positive SR. The collision between CSI and positive SR is less frequent than the collision between CSI and SR. Dropping CSI can be tolerated because RI, PMI vary more slowly and the occasional loss of CQI do not have dramatic affect. The periodicity of CSI should account for variability in the interference, hence CSI measurement and reporting can be more frequent than what would be justified by the rate of channel variation.

Since eMBB SR periodicity is usually high, the probability of CSI vs. positive SR collision is low, thus distinction from URLLC SR is not necessary in this case.

Proposal 5-6: In the case of P-SP/CSI colliding with positive SR (of any priority level), transmit SR and drop CSI at least on and after the overlapping symbol. (Scenarios 02 and 09)

### URLLC SR colliding eMBB SR (Scenario-07)

This scenario is not realistic. If L2 passes a URLLC positive SR while eMBB SR is already being transmitted then eMBB SR should be dropped on and after the colliding symbol and URLLC SR transmitted.

## PUSCH vs. SR priority (scenarios 04, 10, 15)

RAN2 has agreed that PUSCH is transmitted and positive SR is dropped in every case when they overlap in time except when SR has higher priority, in which case PUSCH should be dropped at least on and after the colliding symbol. How the priority levels should be known by the UE has not been agreed on in full detail.

SR priority

RAN2 has agreed that when SR collides with PUSCH then SR priority is derived from the LCP-level of the logical channel that has triggered SR. Whether LCP-levels are used directly or via mapping to derive SR priority has not been decided. This is primarily due to the fact that SR priority needs to be compared against PUSCH priority and the agreement has left PUSCH priority as an open issue. The other reason is that RAN2 waits for progress on the handling of SR vs. HARQ/CSI and PUSCH vs. HARQ/CSI collision cases in RAN1.

PUSCH priority

Two main questions need to be decided:

- Should LCP-levels be used directly for prioritization?

- Is it necessary to introduce signalling in the UL-DCI for dynamic grant, or indication in the configured grant?

For the first issue, we argue that LCP-levels should be mapped to high- and low-priority by a configurable threshold before being applied in prioritization between SR and PUSCH because LCP levels already serve a different purpose, namely the prioritization of PDU’s onto available grants. Prioritization means a different thing in the latter case, as an available grant is not dropped. When the grant is dropped due to collision with higher priority SR (or PUSCH) the cost in efficiency is higher. E.g. two eMBB flows could differ in LCP-level but SR should never be prioritized. Similar applies to two colliding URLC flows.

Concerning the second issue we do not see a case where signalling from gNB would be necessary. Even if such a signalling was introduced for handling the case of PUSCH vs. HARQ, that behaviour should not be coupled with this case.

Proposal 5-7: In the case of PUSCH vs. positive SR collision:

- According to RAN2 agreement: If SR is higher priority than PUSCH then SR is transmitted and PUSCH is dropped at least on and after the colliding symbols. Otherwise SR is dropped (prohibition timer is not started) and PUSCH is transmitted.

- According to RAN2 agreement: PHY layer priority level for SR should be derived from LCP level of the triggering logical channel in MAC.

- PHY layer priority level for PUSCH should be derived from LCP level of the triggering logical channel in MAC.

- A configurable thresholds should determine the monotonous mapping from LCP-levels to two PHY layer priority levels: low and high.

## PUSCH vs HARQ (scenarios 05, 14, 16)

In R-1908409 we argued for not distinguishing between lower and higher PUSCH priority levels but to take into account the temporal order of the scheduling DCI’s.  The latter distinction can be used to allow overriding earlier transmissions under certain conditions (e.g. the last DCI schedules a high-priority HARQ, causing drop of PUSCH). Since the plenary agreements have down-scoped the choices, and consensus needs to be reached, a simpler behaviour is proposed here.

High and low priority PUSCH should be derived from LCP-priority values: using a configurable threshold on LCP levels.

When HARQ and PUSCH overlapping in time have the same priority levels then they should be handled by Rel-15 multiplexing (as has been agreed by the down-scoping). Since a URLLC HARQ can be scheduled by a DL-DCI in a later monitoring occasion the beta-offset should be selected sufficiently high.

When low-priority HARQ collides with high-priority PUSCH then Rel-15 handling should be maintained. Ideally, beta-offset should be selected low for best latency and efficiency. In practical scenarios, collision with eMBB HARQ is known at the time PUSCH is scheduled, and beta-offset selection can take this into account.   However, earlier we assumed that a URLLC HARQ colliding with PUSCH may be scheduled by a DL-DCI in a later monitoring occasion and this hypothesis suggests selecting a high beta-offset. Do we need to deal with contradictory requirements then? On one hand, the parameter alpha could still be used to limit the resources spent on eMBB HARQ, even with a high beta-offset.  On the other, the scenario is very rare where PUSCH is scheduled without the knowledge of a collision with a later HARQ that still meets the UE multiplexing timeline, and can practically be eliminated when a collision with eMBB HARQ is also foreseen. Since URLLC HARQ is prioritized in collisions with eMBB HARQ, eMBB HARQ should be dropped when two HARQs overlap with the same PUSCH. This is clearly undesirable. It is still more efficient to schedule URLLC HARQ around PUSCH and eMBB HARQ, and potentially assume the loss of a retransmission occasion.

When low-priority PUSCH collides with high-priority HARQ then PUSCH should be dropped. gNB would only schedule HARQ when this is necessary to maintain HARQ-ACK latency and losing a retransmission occasion would be a worse option. Otherwise, it would send URLLC HARQ-ACK with low-priority HARQ codebook multiplexed onto PUSCH. Beta-offset should be set sufficiently high for this case to ensure HARQ reliability. Based on earlier considerations this case may be the assumed. One concern could be that HARQ-over-eMBB-PUSCH is dropped when it collides with URLLC positive SR. This case is relatively rare however, and URLLC HARQ could only be saved by dropping eMBB PUSCH on each collision when PUSCH and HARQ collide.

Proposal 5-8: When PUSCH and HARQ overlap in time apply the following collision resolution:

- High-priority PUSCH vs low-priority HARQ: apply Rel-15 handling rules

- High-priority HARQ vs low-priority PUSCH: drop PUSCH and transmit HARQ

- Same priority levels: apply Rel-15 handling rules

## PUSCH vs CSI (scenarios 06 and 17)

The scheduler is aware of the collision when configuring the grant for PUSCH. Beta-offset and alpha-factor allow to set the appropriate code rate for CSI. The impact of multiplexing onto latency can be accounted for by scheduling.

Proposal 5-9: When PUSCH colliding with CSI (Scenarios-06, 17), reuse Rel-15 handling.

## P/SP-CSI vs P/SP-CSI collision

When two CSI collide due to differences in periodicity then Rel-15 handling should be applied to multiplex them on a multi-CSI resource when one is configured. If no such resource is configured then the BLER target of the table used with each CSI should be taken into account for the prioritization between them: when BLER targets differ (10^-5 and 10^-1) then the one using the lower BLER target should be transmitted and the other dropped.

Proposal 5-9: In the case of P/SP-CSI vs P/SP-CSI collision, when a multi-CSI resource is not configured and one CSI uses BLER target 10^-5 and the other 10^-1 then only transmit the one with the lower BLER target.

## PUSCH vs. UCI Collision on different CC’s

In Rel-15 PUSCH vs. UCI collisions are handled across CC’s because this allows more UCI-over-PUSCH multiplexing. However, when collision resolution results in dropping one of the transmissions there is no benefit from handling CC’s together. In fact, colliding transmissions could be transmitted simultaneously.

Proposal 5-10: If PUSCH and UCI collide on different CC’s then consider transmitting them separately without UCI over PUSCH multiplexing or prioritization.

## Summary

Table 2 offers an overview of the proposed behaviour.

Table 2: Collision resolution proposals by scenario

|  | URLLC SR | URLLC HARQ-ACK | CSI | URLLC PUSCH |
| --- | --- | --- | --- | --- |
| URLLC SR |  |  |  |  |
| URLLC HARQ-ACK | Reuse Rel-15 |  |  |  |
| CSI | Drop CSI. | Drop CSI. |  |  |
| URLLC PUSCH | Drop SR. | Reuse Rel-15 | Reuse Rel-15. |  |
| eMBB SR | Drop eMBB SR | Reuse Rel-15 | Reuse Rel-15. | Drop eMBB SR |
| eMBB HARQ-ACK | Reuse Rel-15 | Drop eMBB HARQ. Delayed sending of eMBB HARQ-ACK. | Reuse Rel-15. | Reuse Rel-15. |
| eMBB PUSCH | Drop PUSCH | Drop PUSCH. | Reuse Rel-15. | Drop eMBB PUSCH. |

# Conclusion

In the first part of the contribution we made the following observation and proposal on sub-slot partitioning:

Proposal 1: Allow PUCCH resources to cross sub-slot boundary. If the configured PUCCH resource overlaps with a slot boundary or a DL symbol then it is invalid. The UE shall treat it as an error case if an invalid PUCCH resource is scheduled or PUCCH resources overlapping in time are scheduled from different sub-slots.

Observation 1: Further enhancements to sub-slot partitioning (e.g. to sub-slot size and separate configurability of PUCCH resources per sub-slot) could be avoided by supporting complementary, codebook-less HARQ procedure.

On multiple HARQ procedures, we had the following proposals in the second part of the contribution:

Observation 2: Complementary codebook-less HARQ procedure can allow choosing coarser or no partitioning for the codebook-based procedure. This benefits scheduling and HARQ multiplexing.

Proposal 2: Use “Codebook-less HARQ” as a complementary procedure, simultaneously to the codebook-based procedure. The number of PUCCH’s carrying codebook-less HARQ in a (sub-)slot needs not be restricted.

Proposal 3: To avoid signalling overheads, a configurable special K1 (index) value could select codebook-less HARQ_ACK sending.

Proposal 4: For codebook-less procedure, the PUCCH resource assigned by the PRI should be sent in the earliest sub-slot for which the PUCCH resource abides by the N1 UE timeline.

Proposal 5: Support at most two HARQ codebook procedures, and one complementary codebook-less HARQ procedure using the PUCCH configurations of one or the other codebook-based procedure.

- If two codebooks are configured than at least one of them should follow Rel-15 configurations only.

- Any Rel-15 codebook has low-priority and any Rel-16 codebook as well as the codebook-less procedure have high-priority.

Proposal 6: For dynamically scheduled PDSCH, use explicit indication in DCI for the PHY identification for identifying a HARQ-ACK codebook when at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE.

- FFS: introduce new field or reuse existing field: HARQ_process_ID.

Proposal 7: With SPS PDSCH the HARQ codebook should be selected based on semi-static SPS PDSCH configuration.

In the last part of the contribution we had the following observations and proposals on intra-UE prioritization and multiplexing of UCI:

Proposal 5-1: When two HARQ transmissions of different priority levels collide in time, drop the – possibly started – low-priority HARQ-ACK transmission and transmit the high-priority HARQ-ACK.  (Scenario-12)

Proposal 5-2: The UE should store any HARQ codebook transmitted on PUCCH that has been deprioritized (dropped), along with the slot/sub-slot, for later re-sending in full.  FFS: memory depth.  (Scenario-12, …)

Proposal 5-3: For querying the UE’s history of deprioritized HARQ codebooks (Scenario-12, …), reuse Rel-15 UL-DCI features used for scheduling A-CSI measurement and reporting. FFS: precise report format

Proposal 5-4: When high-priority HARQ-ACK collides with P/SP-CSI (scenarios 03 and 13), then UE should drop CSI and transmit HARQ.

Proposal 5-5: Maintain Rel-15 handling rules between HARQ and SR irrespective of their PUCCH formats and priority levels. (Scenario 01, 08, 09 and 11).

Proposal 5-6: In the case of P-SP/CSI colliding with positive SR (of any priority level), transmit SR and drop CSI at least on and after the overlapping symbol. (Scenarios 02 and 09)

Proposal 5-7: In the case of PUSCH vs. positive SR collision:

- According to RAN2 agreement: If SR is higher priority than PUSCH then SR is transmitted and PUSCH is dropped at least on and after the colliding symbols. Otherwise SR is dropped (prohibition timer is not started) and PUSCH is transmitted.

- According to RAN2 agreement: PHY layer priority level for SR should be derived from LCP level of the triggering logical channel in MAC.

- PHY layer priority level for PUSCH should be derived from LCP level of the triggering logical channel in MAC.

- A configurable thresholds should determine the monotonous mapping from LCP-levels to two PHY layer priority levels: low and high.

Proposal 5-8: When PUSCH and HARQ overlap in time apply the following collision resolution:

- High-priority PUSCH vs low-priority HARQ: apply Rel-15 handling rules

- High-priority HARQ vs low-priority PUSCH: drop PUSCH and transmit HARQ

- Same priority levels: apply Rel-15 handling rules

Proposal 5-9: When PUSCH colliding with CSI (Scenarios-06, 17), reuse Rel-15 handling.

Proposal 5-10: If PUSCH and UCI collide on different CC’s then consider transmitting them separately without UCI over PUSCH multiplexing or prioritization.

# References

- R1-1908409  “Multiple HARQ procedures and intra-UE UCI prioritization”, MTK, RAN1#98, Czech, August, 2019