3GPP TSG RAN WG1 Meeting #98bis	R1-1911711

Chongqing, China, October 14-20, 2019

Agenda Item:	7.2.6.1

Source:	Huawei

Title:	Summary of Friday offline discussion on PDCCH enhancements

Document for:	Discussion and decision

Introduction

In the RAN #83 meeting, a new WID [1] on Physical Layer Enhancements for NR URLLC was approved with the following objective for PDCCH enhancements:

- Specification of PDCCH enhancements [RAN1]

  - DCI format(s) with configurable sizes for some fields, with a minimum DCI size targeting a reduction of 10~16 bits relative to Rel-15 DCI format 0_0/1_0 and a maximum DCI size that can be larger than Rel-15 DCI format 0_0/1_0, and provide the possibility to align with the size of the DCI format 0_0/1_0 (including possible zero padding if any)

  - Increased PDCCH monitoring capability on at least the maximum number of non-overlapped CCEs per slot for channel estimation for at least one SCS subject to restrictions including, but not necessary limited to, those identified in TR 38.824. Enhancements for PDCCH monitoring capability on the maximum number of monitored PDCCH candidates per slot (with potential restrictions) can be further considered.

This document summarizes the key issues discussed under agenda item 7.2.6.1 based on the views in [1][2][3][4][5][6][7][8][9][10][11][12][13][14][15][16][17][18][19][20][21][22]. The agreements related to PDCCH enhancements achieved in the previous meetings are listed in Appendix A.

DCI format scheduling Rel-16 URLLC

According to the contributions submitted to RAN1#98bis meeting, companies mainly provide views on detailed design of the information fields, e.g. fields from Rel-15 DCI need to be resized or removed for DCI format scheduling Rel-16 URLLC, fields from Rel-15 DCI could be reused without change for DCI format scheduling Rel-16 URLLC, fields from Rel-15 non-fallback DCI needs to be present for DCI format scheduling Rel-16 URLLC and new fields need to be added due to some URLLC features. In addition, some companies also provide views on other aspects like DCI size alignment and how to differentiate DCI formats if the size of the DCI format scheduling Rel-16 URLLC is aligned with that of Rel-15 DCI. This section summarizes the views on these aspects.

DL DCI format scheduling Rel-16 URLLC

As to the detailed design of the DL DCI format, many companies provide detailed views as summarized in Table 1 below. Note that the fields highlighted in green were agreed in previous meetings.

Table 1 Potential DL DCI design for Rel-16 URLLC

| Fields | DCI format 1_0 | DCI format 1_1 |  | DL DCI for R16 URLLC |
| --- | --- | --- | --- | --- |
| Identifier for DCI formats | 1 bit | 1 bit |  | 1 bit |
| Frequency domain resource assignment | RA type 1 with size depending on the assumed BWP | RA type 0 and RA type 1 with size depending on the active BWP |  | Details seen in section 2.1.1 For resource allocation type 1, support a single configurable scheduling granularity applicable for both the starting point and length indication. A new RRC parameter to configure the scheduling granularity. Candidate values for the new RRC parameter: Option 1: {2, 4, 8, 16} with default value of 1 Support: Ericsson Option 2: {1, 2, 4, 8, 16} with default value of 1 Support: ZTE, CATT, Vivo, Huawei For resource allocation type 0, Option 1: Support also resource allocation type 0 with configurable granularity for the new DCI format scheduling PDSCH (and PUSCH) Support: Nokia Option 2: Support FDRA type 0 with a scaling factor K to RBG size for Rel-16 DCI format Support: DCM Option 3: Resource allocation type 0 is not supported for URLLC scheduling Support: Qualcomm, MTK, Panasonic Option 4: Resource allocation type 0 is supported without any change Support: Sony, Ericsson Support dynamic switching between resource allocation Type 0 and resource allocation Type 1 Support: Nokia |
| Time domain resource assignment | 4 bits |  | 0 or 1 or 2 or 3 or 4 bits | For the bit width, Configurable # of bits (0 or 1 bit or 2 bits or 3 bits or 4 bits) depending on the configured TDRA table For the reference of the SLIV, Option 1: Changing the reference from slot boundary to some PDCCH symbol (e.g. the starting symbol of PDCCH) Support: Nokia, Qualcomm, Huawei, MTK, Vivo, Spreadtrum, Panasonic, OPPO, Samsung, DCM, ZTE, Vivo, CATT, Sony, WILUS - Applied if K0=0, otherwise the reference is slot boundary as in Rel-15 Support: Sony, WILUS, CATT Option 2: Use slot boundary as a SLIV reference for TDRA as in Rel-15 Support: Ericsson |
| VRB-to-PRB mapping | 1 bit |  | 0 or 1 bit | Option 1: 0 bit (removed) Support: MTK, Intel Option 2: Configurable # of bits (0 or 1 bit) Support: Ericsson, Huawei, DCM, Vivo, Samsung, Nokia, spreadtrum, Qualcomm, CATT, Sharp Option 3: 1 bit Support: ZTE |
| Modulation and coding scheme | 5 bits |  | 5 bits | Details seen in section 2.1.1 Option 1: Configurable size for the MCS field for the DCI scheduling Rel-16 URLLC Support: Nokia, Huawei, NTT DOCOMO, Vivo, Intel, OPPO, Samsung, InterDigital, CATT, Sharp, Sequans - Alt 1: by configuring an anchoring index and the number of bits in the DCI Support: Sequans - Alt 2: Configurable MCS table size and the entries - Alt. 3: limiting the number of rows to be indicated Support: Samsung (3~5 bits) Option 2: No change compared to Rel-15 DCI Support: Qualcomm, Ericsson, MTK, Spreadtrum, Sony, Panasonic Option 4: Joint coding of MCS and RV Support: ZTE |
| New data indicator | 1 bit |  | 1 bit | 1 bit (i.e. No change compared to Rel-15 DCI) |
| Redundancy version | 2 bits |  | 2 bits | Details seen in section 2.1.1 Option 1: 2 bits (i.e. No change compared to Rel-15 DCI) Support: Qualcomm Option 2: Support a configurable redundancy version field size of 0, 1 or 2 bits. Support: Nokia, DCM, Intel, Panasonic, MTK, Samsung, Huawei, CATT, Sony (1 or 2 bits), Sharp, Ericsson (1 or 2 bits) Option 3: Support a configurable redundancy version field size of 0 or 1 bit. Support: Spreadtrum, Vivo, China Unicom, Option 4: 1 bit (i.e. limited set of RV sequences) Support: InterDigital Option 5: Joint encoding of MCS and RV Support: ZTE |
| HARQ process number | 4 bits |  | 4 bits | Configurable # of bits (2 or 3 or 4 bits, FFS 0 or 1 bit) Whether to support candidate value 0 - Yes: Nokia, MTK, Spreadtrum, Samsung, Vivo - No: Qualcomm - Need to be decided in connection with support for multiple UL CG/SPS configurations and interaction between HARQ process ID of the initial UL CG/DL SPS transmission and the retransmission of the UL CG/SPS transmission: Ericsson Whether to support candidate value 1 - Yes: Nokia, MTK, Spreadtrum, Samsung, Vivo - No: Qualcomm |
| Downlink assignment index | 2 bits |  | 0 or 2 or 4 bits | Option 1: Post-pone the discussions due to potential impacts from other functionalities/features under discussion Support: Qualcomm, Nokia, Panasonic Option 2: Configurable # of bits Support: Ericsson, Huawei, MTK, Vivo Option 3: 2 bits Support: CATT, ZTE |
| TPC command for scheduled PUCCH | 2 bits |  | 2 bits | Option 1: 2 bits (i.e. No change compared to Rel-15 DCI) Support: Qualcomm, Ericsson, Huawei, CATT, MTK, DCM, ZTE, Spreadtrum, Nokia, Intel Option 2: 0 bit to 2 bits in configurable manner Support: Vivo Option 3: 0 bit or 2 bits in configurable manner Support: Samsung |
| PUCCH resource indicator | 3 bits |  | 3 bits | Option 1: Post-pone the discussion due to potential impacts from other functionalities/features under discussion Support: Qualcomm, Nokia, CATT, Panasonic Option 2: 2 bits Support: Ericsson, CATT, Vivo Option 3: 3 bits Support: Intel, ZTE Option 4: Configurable # of bits (0 or 1 or 2 or 3 bits) Support: DCM, Huawei, MTK |
| PDSCH-to-HARQ_feedback timing indicator | 3 bits |  | 0 or 1 or 2 or 3 bits | Option 1: Post-pone the discussion due to potential impacts from other functionalities/features under discussion Support: Qualcomm, Nokia, Panasonic, CATT Option 2: 0 or 2 bits Support: Ericsson Option 3: 1 bit Support: OPPO Option 4: Configurable # of bits (0 or 1 or 2 or 3 bits) Support: DCM, Intel, MTK, Vivo, Samsung, Spreadtrum, ZTE, Sharp |
| Fields only from Rel-15 non-fallback DCI (i.e. DCI format 1_1) |  |  |  |  |
| Carrier indicator | N/A |  | 0 or 3 bits | configurable # of bits (0 or at least one non-zero bit) Option 1: up to 3 bits (i.e. 0 or 1 bit or 2 bits or 3 bits) Support: Ericsson, CATT, NTT DOCOMO, OPPO, Samsung, Panasonic, Nokia, Intel, ZTE Option 2: up to 2 bits (i.e. 0 or 1 bit or 2 bits) Support: Qualcomm The number of bits for “carrier indicator” in the DCI for DL and UL scheduling should be separately configured Support: Qualcomm, |
| PRB bundling size indicator | N/A |  | 0 or 1 bit | 0 or 1 bit |
| Rate matching indicator | N/A |  | 0 or 1 or 2 bits | 0 or 1 bit or 2 bits |
| ZP CSI-RS trigger | N/A |  | 0 or 1 or 2 bits | 0 or 1 bit or 2 bits |
| Antenna port(s) | N/A |  | 4 or 5 or 6 bits | Option 1: configurable # of bits (0 or 1 or 2 or 4 or 5 or 6 bits) Support: DCM, Intel, Ericsson, Samsung - 0 bit (single port DMRS, e.g., port 1000 is always used as in DCI 1_0) - 1 bit for UE that supports up to two layers (e.g. Value 0 or Value 11 from Table 7.3.1.2.2-1 can be indicated). - 2 bits for UE that supports up to four layers (e.g. Values 0, 11, 9, 10 from Table 7.3.1.2.2-1 can be indicated). - 4, 5, 6 bits to support the same configurations as in DCI format 1_1 Option 2: configurable # of bits (0 or 4 or 5 or 6 bits) Support: Nokia, Qualcomm (separate configuration for eMBB and URLLC), Panasonic Option 3: configurable # of bits (0 or 1 or 2 bits) Support: Spreadtrum, CATT, ZTE |
| Transmission configuration indication | N/A |  | 0 or 3 bits | Option 1: configurable # of bits (0 or 1 or 2 or 3 bits) Support: Ericsson, ZTE, CATT, DCM, Panasonic, MTK, Nokia Option 2: 0 or 3 bits as in Rel-15 Support: Intel, Spreadtrum, Qualcomm |
| SRS request | N/A |  | 2 bits for UEs not configured with SUL 3 bits for UEs configured with SUL | Option 1: configurable # of bits (0 or 1 or 2) Support: MTK Option 2: configurable # of bits (0 or 1 or 2 or 3 bits) Support: CATT, DCM, ZTE, Samsung, Intel, Panasonic Option 3: configurable # of bits (0 or 2 or 3 bits) Support: Nokia Option 4: 2-bit Support: Qualcomm |
| DMRS sequence initialization | N/A |  | 1 bit | Option 1: 0 bit Support: Ericsson, CATT, Spreadtrum Option 2: Configurable # of bits (0 bit or 1 bit) Support: DCM, Nokia, MTK, Panasonic, ZTE Option 3: 1 bit Support: Intel, Qualcomm, Sharp |
| BWP indicator | N/A |  | 0 or 1 or 2 bits | Option 1: N/A Support: Ericsson, Spreadtrum Option 2: Configurable # of bits (0 or 1 or 2 bits) Support: NTT DOCOMO, Samsung, OPPO, MTK, Intel, Panasonic, ZTE, Qualcomm |
| Modulation and coding scheme for TB 2 | N/A |  | 5 bits | 0 |
| New data indicator for TB 2 | N/A |  | 1 bit | 0 |
| Redundancy version for TB 2 | N/A |  | 2 bits | 0 |
| CBG transmission information | N/A |  | 0 or 2 or 4 or 6 or 8 bits | 0 |
| CBG flushing information | N/A |  | 0 or 1 bit | 0 |
| New Fields proposed to be added compared to Rel-15 DCI |  |  |  |  |
| Repetition factor | N/A |  |  | Option 1: Post-pone the discussion due to potential impacts from other functionalities/features Support: ZTE, Panasonic Option 2: configurable # of bits (0 or 1 or 2 or 3) Support: CATT, NTT DOCOMO, Sequans |
| New format indicator | N/A |  |  | Depending on the outcome of the discussion of DCI format differentiation |
| Virtual CRC | N/A |  |  | Support: Panasonic, Huawei |
| Priority indicator | N/A |  |  | Depending on the outcome of the discussion of service differentiation |
| Physical layer configuration set/option indication | N/A |  |  | Depending on the outcome of the discussion of service differentiation Option 1: Configurable # of bits for physical layer configuration set/option indication. Support: Ericsson, InterDigital (field should include priority indicator) |
| HARQ-ACK codebook indication | N/A |  |  | Depending on the outcome of the discussion of HARQ-ACK codebook identification Option 1: 1 bit (Post-pone the discussion due to potential impacts from other functionalities/features) Support: ZTE, Nokia |
| AL8/AL16 identifier | N/A |  |  | Option 1: Configurable # of bits (0 or 1 bit) Support: Huawei |

In addition to the summary of the above table, some additional information are also provided below for some key aspects on the design for DL DCI format scheduling Rel-16 URLLC.

Fields from both DCI format 1_0 and DCI format 1_1 to be resized or removed

In the RAN1#AH 1901 meeting, it was agreed to support reduction of the number of bits for at least one of the following fields compared to Rel-15 DCI

- Frequency domain resource assignment

- Time domain resource assignment

- Modulation and coding scheme

- HARQ process number

- Redundancy version

- PUCCH resource indicator

- PDSCH-to-HARQ_feedback timing indicator

- Downlink assignment index

- Note: Reduction of other fields are not precluded

In addition, some companies also provide the views that potential reduction of the number of bits for the following fields is feasible:

- VRB-to-PRB mapping

Based on the summary in Table 1, it is reasonable to post-pone the discussion of the following fields due to potential impacts from other functionalities/features under discussion:

- PUCCH resource indicator

- PDSCH-to-HARQ_feedback timing indicator

- Downlink assignment index

Conclusion 2.1.1-1: For DL DCI format scheduling Rel-16 URLLC, post-pone the discussion of the following fields due to potential impacts from other functionalities/features under discussion:

- PUCCH resource indicator

- PDSCH-to-HARQ_feedback timing indicator

- Downlink assignment index

Frequency domain resource assignment

In the RAN1#98 meeting, it was agreed to support a single configurable scheduling granularity applicable for both the starting point and length indication for resource allocation type 1. A new RRC parameter is used to configure the scheduling granularity. There are mainly two remaining issues for frequency domain resource assignment, the candidate values for the new RRC parameter for configuring the scheduling granularity for resource allocation type 1, and whether/how to support resource allocation type 0.

Regarding the candidate values for the new RRC parameter,

Option 1: {2, 4, 8, 16} with default value of 1

Support: Ericsson

Option 2: {1, 2, 4, 8, 16} with default value of 1

Support: ZTE, CATT, Vivo, Huawei

In addition, [MTK, R1-1911078] proposed to consider frequency hopping when designing the FD-RA bit-field, e.g. a scaling could be introduced for the UE to determine the new scheduling granularity from the signaled granularity. [WILUS, R1-1911315] proposed that RBs should be grouped by considering PRB grid alignment. Companies are encouraged to check.

Proposal 2.1.1-1: For resource allocation type 1 for frequency domain resource assignment for the DCI format scheduling Rel-16 URLLC, the candidate values for the scheduling granularity for starting point and length indication is {1, 2, 4, 8, 16}.

- The default value is 1 PRB.

Regarding whether/how to support resource allocation type 1:

- Option 1: Support also resource allocation type 0 with configurable granularity for the new DCI format scheduling PDSCH (and PUSCH)

  - Support: Nokia

  - Reasons:

    - Provide more flexibility for inter-UE eMBB and URLLC multiplexing by allowing schedule a URLLC PDSCH in a slot around some already allocated eMBB PDSCH whereas RA type 0 was used

    - Improved frequency diversity

- Option 2: Support FDRA type 0 with a scaling factor K to RBG size for Rel-16 DCI format.

  - Support: DCM

- Option 3: Resource allocation type 0 is supported without any change

  - Support: Sony, Ericsson

- Option 4: Resource allocation type 0 is not supported for URLLC scheduling

  - Support: Qualcomm, MTK, Panasonic

The main concern from companies to support resource allocation type 1 is that more bits are needed for indication compared to resource allocation type 1 assuming same scheduling granularity. On the other hand, additionally supporting resource allocation type 0 does bring more flexibility. However, it seems among the proponents of supporting resource allocation type 0, it seems there are different views on how to support it. Some more discussion is needed first.

Proposal 2.1.1-2: Further study whether/how to support resource allocation type 0 for frequency domain resource assignment for the new DCI format scheduling Rel-16 URLLC.

Time domain resource assignment

In Rel-15, the starting symbol of PDSCH transmission is determined with a reference to the slot boundary. Thus, for a same PDSCH duration, different SLIVs will be needed to schedule PDSCH that start at different symbols in a slot. For example, to enable multiple 2-symbol PDSCH receptions within one slot, gNB may need to configure 7 different SLIVs (with different starting OFDM symbol but same length). This can be very inefficient. As shown in the Table 1 and also copied below for convenience, many companies proposed to change the reference as below:

- Option 1: Changing the reference from slot boundary to some PDCCH symbol (e.g. the starting symbol of PDCCH)

  - Support: Nokia, Qualcomm, Huawei, MTK, Vivo, Spreadtrum, Panasonic, OPPO, Samsung, DCM, ZTE, Vivo, CATT, Sony, WILUS

  - Applied if K0=0, otherwise the reference is slot boundary as in Rel-15

  - Support: Sony, WILUS, CATT

  - Reason: For a same PDSCH duration, different SLIVs will be needed to schedule PDSCH that start at different symbols in a slot, which will increase the number of entries need to be indicated thus require more number of bits for time domain resource assignment. With changing the reference to PDCCH, then much less SLIVs to be indicated. [MTK, R1-1911078] provides some analysis on the reduction of bits as below:

| TD-RA allocation Length | #PDCCH MOs = 2 | #PDCCH MOs = 4 | #PDCCH MOs = 7 |
| --- | --- | --- | --- |
| Any Length (1-14 OS) | 0 | 1 | 2 |
| 7 OS | 0 | 1 | 2 |
| 4 OS | 1 | 2 | 3 |
| 2 OS | 1 | 2 | 3 |

- Option 2: Use slot boundary as a SLIV reference for TDRA as in Rel-15

  - Support: Ericsson

  - Reasons

    - The reduction (e.g. 1 bit) may not be too much

    - Only applied to the case of same slot scheduling

    - Need an indicator on whether the reference is based on slot boundary or the new reference

One issue brought up during the offline discussion in the previous meetings is that it may not be compatible with semi-static HARQ-ACK codebook. However, as discussed in [Nokia, R1-1910798] and [Huawei, R1-1910066], no issue is observed for the support of semi-static HARQ-ACK codebook with the new reference. In addition, with an appropriate design, it can be expected that the reduction of the bits for time-domain resource assignment can even go to 3 bits, thus the reduction is still promising. Therefore, it is encouraged to go to the majority view. In addition, as discussed in [Sony, R1-1910768][WILUS, R1-1911315], if the scheduler does want to do cross slot scheduling, it will have to configure multiple entries in the TDRA table for a single allocation, thus they proposed that whether to use the new reference or the slot boundary can depend on the value of K0. Companies are encouraged to check this issue.

Proposal 2.1.1-3: For time domain resource allocation indication for PDSCH for Rel-16 URLLC in the new DCI format, using the starting symbol of the PDCCH monitoring occasion in which the DL assignment is detected as the reference of the SLIV is supported.

- FFS whether using the new reference is configurable or not

- FFS: new reference is applied if K0=0, otherwise the reference is slot boundary as in Rel-15

Regarding the bit width for time domain resource assignment, it was agreed to support configurable TDRA table as in Rel-15 DCI format 1_1 (i.e. 0, 1, 2, 3 or 4 bits for time domain resource assignment) for the DL DCI format scheduling Rel-16 URLLC.

In addition, the following points were also mentioned in some contributions. Companies are encouraged to check.

- Configure a separate TDRA table for eMBB and URLLC for a UE that supports multiple services

  - Support: Qualcomm, MediaTek

- Some scheduling parameters (e.g. K0, K1, and K2) are implicitly indicated to the UE

  - Support: MediaTek

Modulation and coding scheme

For modulation and coding scheme, the following enhancements are proposed:

- Option 1: Configurable size for the MCS field for the DCI scheduling Rel-16 URLLC

  - Support: Nokia, Huawei, NTT DOCOMO, Vivo, Intel, OPPO, Samsung, InterDigital, CATT, Sharp, Sequans

    - Alt 1: by configuring an anchoring index and the number of bits in the DCI

      - Sequans

    - Alt 2: Configurable MCS table size and the entries

      - Nokia

    - Alt 3: limiting the number of rows to be indicated

      - Samsung (3~5 bits)

  - Reasons

    - To provide the chance to reduce the number of bits for MCS thus enable a smaller DCI size

    - For URLLC, some entries in the existing MCS table is typically not used, e.g. the entries for 64QAM

- Option 2: No change compared to Rel-15 DCI

  - Support: Qualcomm, Ericsson, MTK, Spreadtrum, Sony, Panasonic

  - Concerns

    - Reducing the size of the MCS would lead to coarser MCS granularity which lead to inefficient allocation of resources

    - Localizing to a subset of the Rel-15 MCS table may not be suitable if the UE moves to a different radio condition

- Option 3: Joint coding of MCS and RV

  - Support: ZTE

Looking at the concerns, if the size is configurable, then full MCS table still can be used if needed, while provide the chance to reduce the number of bits for the applicable use case. In addition, it seems the main concern is that it may bring too much restrictions, for compromise, we can limit the smallest number of bits can only go down to 2 bits. Therefore, companies are encouraged to check and go with the following proposal.

Proposal 2.1.1-4: Support configurable size for “modulation and coding scheme” for the DCI format scheduling Rel-16 URLLC.

- FFS: the range of the MCS field is 2~5 bits

- FFS: the entries to be indicated

Some more detailed thinking from companies about the enhancements are copied below for your convenience:

Contribution [Nokia, R1-1908436] As for URLLC transmission the link adaption algorithm is usually designed conservatively to guarantee the reliability requirement, less entries in the MCS table may be needed for URLLC compared to eMBB transmission. There was a proposal to use the lowest 8 entries for URLLC transmission to reduce the size of the MCS field to 3bits, however this hard reduction overlooks the possibility that UE might be in a good channel condition and could use the opportunity to transmit with higher MCS. One way to enhance the proposal, compress the MCS field while keep the possibility to schedule UE to transmit within a wide range of channel conditions without losing performance, is to add an anchoring index combined with 4 (2bit), 8 (3bit) or 16 (4bit) continuous entries within the MCS table. Figure 3-1: The anchoring index and 8 continuous entries in MCS table. As shown in Fig. 3-1, the anchoring index is RRC configured for the UE and could be any entry in the legacy MCS table. The 8 continuous MCS entries in the example of Figure 1 starting from the anchoring index are signalled with 3 bits in DCI and represent the deviation from the anchoring index. Together with the anchor index, the gNB may configure the number of bits in the DCI field to define the number of different MCS entries which can be dynamically signalled in the DCI (0-4 bits). If the URLLC UE stays within a stable environment and the gNB would not need the option of link adaptation, in principle 0bits for MCS in the DCI could be configured and the anchoring index could directly give the applicable MCS for all UL-SCH & DL-SCH communication leading bits saving up to 5 bits compared to fallback DCI. Another alternative discussed already during the Rel-15 discussions would be to make the MCS table size as well as the related MCS entries fully configurable (e.g. using a 32bit bitmap), resulting in a higher RRC signalling overhead. This enables any combination of MCS entries, including the possibility of having the entries non-continuous and spread over a larger range of MCS values as well as the inclusion of selected ‘reserved’ MCS entries for HARQ re-transmission. This may be especially of interest, if the same DCI is used to schedule eMBB and URLLC traffic for a single UE and therefore a different range of MCS entries may be required for the eMBB and URLLC operation. Following the general design principle of the new configurable DCI format, to provide as much flexibility as possible and to reduce the DCI size at the same time by configuration, we propose to adopt fully configurable MCS table entries where the resulting size of the MCS table defines the MCS field size. Proposal 2-8: Support fully configurable MCS table entries where the number of configured entries determines the size of the MCS field.

Contribution [ZTE, R1-1908235] For URLLC scenario, it is preferable that redundancy versions with incremental redundancy are supported for LDPC coding or polar coding. RV can bring performance gain by incremental redundancy. However, mainly lower code rates are used in URLLC scenario. Thus the number of RVs could be limited for certain code rates. According to [4], payload reduction of DCI is up to 3 bits when careful MCS&RV joint coding is introduced and an example of 4 bits MCS&RV is shown in Table 3. Table 3. An example of 4 bits MCS & RV Table MCS Index IMCS; Modulation Order Qm; Target code Rate x [1024] R; Spectral efficiency; Redundancy Version rvidx; Explanation. 0; 2; 40; 0.0781; 0; CQI-1. 1; 2; 78; 0.1523; 0; CQI-2. 2; 2; 120; 0.2344; 0; CQI-3. 3; 2; 193; 0.3770; 0; CQI-4. 4; 2; 308; 0.6016; 0; CQI-5. 5; 2; 449; 0.8770; 0; CQI-6. 6; 2. 7; 4; 378; 1.4766; 0; CQI-8. 8; 2. 9; 4; 616; 2.4063; 0; CQI-10. 10; 2. 11; 6; 567; 3.3223; 0; CQI-12. 12; 2. 13; 2; reserved; 2. 14; 4; reserved; 2. 15; 6; reserved; 2.

Contribution [Samsung, R1-1908490] Configurable from 3 to 5. No need to always support QAM64 and possibly QAM16. No functional specification impact (higher entries of MCS table are not addressable if the field has less than 5 bits)

Redundancy version

For redundancy version, the views from companies are summarized as shown in Table 1 and also copied here for convenience:

- Option 1: 2 bits (i.e. No change compared to Rel-15 DCI)

  - Support: Qualcomm

- Option 2: Support a configurable redundancy version field size of 0, 1 or 2 bits.

  - Support: Nokia, DCM, Intel, Panasonic, MTK, Samsung, Huawei, CATT, Sony (1 or 2 bits), Sharp, Ericsson (1 or 2 bits)

- Option 3: Support a configurable redundancy version field size of 0 or 1 bit.

  - Support: Spreadtrum, Vivo, China Unicom

- Option 4: 1 bit (i.e. limited set of RV sequences)

  - Support: InterDigital

- Option 5: Joint encoding of MCS and RV

  - Support: ZTE

The main reasons proposed by companies to reduce the number of bits for redundancy version mainly include:

- To provide the chance to reduce the number of bits for RV thus enable a smaller DCI size

- For URLLC, reduced number of retransmissions within latency limit

- For very low spectral efficiency /MCS operation (typical for highly reliable transmissions), the gain of incremental redundancy will be very much limited

Based on the above positions from companies and the above analysis, it can be observed that a simpler compromised solution is to support option 2 to provide the fully flexibility. Therefore, companies are encouraged to go with option 2. Note that the design on redundancy version can be the same for DL DCI and UL DCI. However, based on the contributions for PUSCH enhancements, it seems some discussion may be related to redundancy version also. So it seems better to postpone the discussion for uplink till the PUSCH enhancements is clear.

In case the number of bit for the RV field is configured to be 0, then some rule might be needed. For example, [AT&T, R1-1907168] mentioned that RV0 can always be used for initial transmission and RV3 can always be used for retransmission to save the bits. [Nokia, R1-1908436] mentioned that RV0 and RV3 are to be dynamically indicated in case of 1-bit for the RV field.

Proposal 2.1.1-5: Support configurable number of bits (0 or 1 or 2 bits) for “Redundancy version” in DL DCI format scheduling Rel-16 URLLC.

- FFS: If 0 bit is configured, RV0 is used.

- FFS: If 1 bit is configured, RV0 and RV3 are indicated dynamically

HARQ process number

It was agreed in RAN1#98 meeting that the HARQ process number field of the new DCI format is configurable with 2, 3, or 4 bits. FFS 0 or 1 bit.

Whether to support candidate value 0

- Yes: Nokia, MTK, Spreadtrum, Samsung, Vivo

- Reasons:

  - With the fast UE processing time, the round-trio time for PDSCH can be fairly small

  - Within the tight latency bound there may not be even the option to perform HARQ retransmission

- No: Qualcomm

- Need to be decided in connection with support for multiple UL CG/SPS configurations and interaction between HARQ process ID of the initial UL CG/DL SPS transmission and the retransmission of the UL CG/SPS transmission: Ericsson

Whether to support candidate value 1

- Yes: Nokia, MTK, Spreadtrum, Samsung, Vivo

- No: Qualcomm

Based on the above views, it seems the main concern to support a value less than 2 bits is that it may bring much restriction. Based on the views from companies, maybe a compromise way is to support 1 bit but not 0 bit.

Proposal 2.1.1-6: Support the HARQ process number field to be also configurable to 1 bit.

VRB-to-PRB mapping

For VRB-to-PRB mapping, as summarized in Table 1, some companies think it can be removed because semi-static configuration can be used, while some proposed using configurable manner. ZTE proposed to keep it as 1 bit as in Rel-15 fallback DCI. Companies are encouraged to go with the majority (i.e. Option 2) since it can provide full flexibility here and it can be the compromise solutions.

- Option 1: 0 bit (removed)

  - Support: MTK, Intel

- Option 2: Configurable # of bits (0 or 1 bit)

  - Support: Ericsson, Huawei, DCM, Vivo, Samsung, Nokia, spreadtrum, Qualcomm, CATT, Sharp

- Option 3: 1 bit

  - Support: ZTE

Proposal 2.1.1-7: Support configurable number of bits (0 or 1 bit) for “VRB-to-PRB mapping” in new DCI format scheduling Rel-16 URLLC.

TPC command for scheduled PUCCH

For TPC command for scheduled PUCCH, the views from companies are summarized as shown in Table 1 and also copied here for convenience:

- Option 1: 2 bits (i.e. No change compared to Rel-15 DCI)

  - Support: Qualcomm, Ericsson, Huawei, CATT, MTK, DCM, ZTE, Spreadtrum, Nokia, Intel

- Option 2: 0 bit to 2 bits in configurable manner

  - Support: Vivo

- Option 3: 0 bit or 2 bit in configurable manner

  - Support: Samsung

It seems the majority view is to reuse the TPC command for scheduled PUCCH from Rel-15 DCI. Companies are encouraged to check the views from Samsung. For now, it is suggested to go to the majority view.

Proposal 2.1.1-8: Reuse “TPC command for scheduled PUCCH (2 bits)” from Rel-15 DCI for the new DCI format scheduling Rel-16 URLLC.

| Company | View |
| --- | --- |

Fields only from Rel-15 DCI format 1_1 to be present in DCI scheduling Rel-16 URLLC

For achieving fully flexibility, several fields from Rel-15 non-fallback DCI (i.e. DCI format 1_1) are proposed to be added to the DL DCI format scheduling Rel-16 URLLC.

Carrier indicator

For carrier indicator, it was agreed in RAN1#96 bits that 0 or at least one non-zero bit should be supported. Regarding the non-zero bit(s), the views from companies are summarized as shown in Table 1 and also copied here for convenience:

- Option 1: up to 3 bits (i.e. 0 or 1 bit or 2 bits or 3 bits)

  - Support: Ericsson, CATT, NTT DOCOMO, OPPO, Samsung, Panasonic, Nokia, Intel, ZTE

- Option 2: up to 2 bits (i.e. 0 or 1 bit or 2 bits)

  - Support: Qualcomm

It can be observed that may be a simpler compromised solution is to support option 1. Companies are encouraged to go with option 1.

Proposal 2.1.2-1: Support configurable number of bits (0 or 1 or 2 or 3 bits) for “Carrier indicator” for DCI format scheduling Rel-16 URLLC.

During the discussion of RRC parameter, Qualcomm raised the question that whether the number of bits for “carrier indicator” in the DCI for DL and UL scheduling should be separately configured, e.g. for the cases that there are different number of component carrier for DL and UL. It seems there is no harm to allow separate configuration. Companies are encouraged to check and provide your inputs.

Proposal 2.1.2-2: Further study whether the number of bits for “carrier indicator” in the DCI for DL and UL scheduling can be separately configured.

BWP indicator

For BWP indicator, the views from companies are summarized as shown in Table 1 and also copied here for convenience:

- Option 1:  Configurable # of bits (0 or 1 or 2 bits) as in Rel-15

  - Support: NTT DOCOMO, Samsung, OPPO, MTK, Intel, Panasonic, ZTE, Qualcomm

- Option 2: N/A

  - Support: Ericsson, Spreadtrum

The main reason from companies supporting option 2 is that BWP switching may increase the delay for URLLC thus not needed. However, for some use case for URLLC the latency requirement is not that tight, at least in those cases BWP switching is still possible. Therefore, I suggest companies to go to the majority of option 1.

Proposal 2.1.2-3: Keep “BWP indicator (0 or 1 or 2 bits)” without any change from Rel-15 non-fallback DCI in the new DCI format scheduling Rel-16 URLLC.

Based on the summary in Table 1 above, companies also provide views on the following fields:

- Antenna port(s)

- Transmission configuration indication

- SRS request

- DMRS sequence initialization

The above fields are related to MIMO features and also it may be related to multi-TRP discussion in MIMO work item. Based on the views from companies, it seems that the majority view is that these fields should be included in the new DCI format for DL scheduling for sure, and tend to make it configurable and at least the value of “0” should be one candidate value. The main uncertain issues are whether some other smaller values which don’t exist in Rel-15 should be included or not. Therefore, companies are encouraged to check the following proposal:

Proposal 2.1.2-4: For the new DCI format for DL scheduling for Rel-16 URLLC, support configurable number of bits for the following fields:

- Antenna port(s) (0 or 4 or 5 or 6 bits)

  - FFS 1, 2 bits

- Transmission configuration indication (0 or 3 bits)

  - FFS 1, 2 bits

- SRS request (0 or 2 or 3 bits)

  - FFS 1 bit

- DMRS sequence initialization (0 or 1 bit)

| Company | View |
| --- | --- |

New fields need to be added compared to Rel-15 DCI

Based on the above table 1, it can be observed that several new fields compared to Rel-15 DCI are proposed to add in the DL DCI format scheduling Rel-16 URLLC, whether to include repetition factor, new format indicator, priority indicator, physical layer configuration set/option indication, HARQ-ACK codebook indication would depend on the discussion of other functionalities/features, thus the discussion of these fields can be delayed. Especially, for repetition factor it may depend on the discussion in MIMO.

Conclusion 2.1.3-1: For DL DCI format scheduling Rel-16 URLLC, post-pone the discussion of the following potential new fields due to potential impacts from other functionalities/features under discussion:

- Repetition factor (may depend on MIMO discussion)

- New format indicator

- Priority indicator/Physical layer configuration set/option indication

- HARQ-ACK codebook indication

More information of other proposed newly added fields (e.g. virtual CRC, AL8/16 identifier) can be found in [Panasonic, R1-190989][Huawei, R1-1910066]. Companies are encouraged to check the proposals and provide your views on whether these new fields are needed or not.

Proposal 2.1.3-1: Further study the following potential new fields for DL DCI format scheduling Rel-16 URLLC:

- Virtual CRC

- AL8/AL16 identifier

| Company | View |
| --- | --- |

UL DCI format scheduling Rel-16 URLLC

As to the detailed design of the UL DCI format, many companies provide detailed views as summarized in the table below. Note that the fields highlighted in green were agreed in RAN1#96bis meeting.

Table 2 Potential UL DCI design for Rel-16 URLLC

| Fields | DCI format 0_0 | DCI format 0_1 | UL DCI for R16 URLLC |
| --- | --- | --- | --- |
| Identifier for DCI formats | 1 bit | 1 bit | 1 bit |
| Frequency domain resource assignment | RA type 1 with size depending on the assumed BWP | RA type 0 and RA type 1 with size depending on the active BWP | Details seen in section 2.1.1, same as DL |
| Time domain resource assignment | 4 bits | 0 or 1 or 2 or 3 or 4 bits | Depend on the outcome of PUSCH enhancements |
| Frequency hopping flag | 1 bit | 0 or 1 bit | Option 1: 1 bit (No change compared to Rel-15 DCI format 0_0) Support: Qualcomm, Vivo, ZTE, Spreadtrum Option 2: Configurable # of bits (0 or 1 bit) Support: Ericsson, DCM, Intel, MTK, Samsung, Panasonic, CATT, Sharp Option 3: Configurable # of bits (0 or 1 bit) as in Rel-15 DCI format 0_1 Support: Nokia |
| Modulation and coding scheme | 5 bits | 5 bits | Details seen in section 2.1.1, same as DL |
| New data indicator | 1 bit | 1 bit | 1 bit |
| Redundancy version | 2 bits | 2 bits | Depend on the outcome of PUSCH enhancements |
| HARQ process number | 4 bits | 4 bits | Configurable # of bits (2 or 3 or 4 bits, FFS 0 or 1 bit) Remaining details seen in section 2.1.1, same as DL |
| TPC command for scheduled PUSCH | 2 bits | 2 bits | Option 1: 2 bits (i.e. No change compared to Rel-15 DCI) Support: Qualcomm, Ericsson, Huawei, CATT, MTK, DCM, ZTE, Spreadtrum, Nokia, Intel, Vivo, Spreadtrum Option 2: 0 bit or 2 bits in configurable manner Support: Samsung Option 3: 0 bit to 2 bits in configurable manner Support: Vivo |
| UL/SUL indicator | 0 or 1 bit (depending on if padding bit available) | 0 bit or 1 bit | Option 1: As in Rel-15 DCI 0_1 (0 or 1 bit) Support: Huawei, NTT DOCOMO, Intel, MTK, Samsung, Vivo, Sptreadtrum, Panasonic, Nokia, ZTE Option 2: N/A Support: Ericsson |
| Fields only from Rel-15 non-fallback DCI (i.e. DCI format 0_1) |  |  |  |
| Carrier indicator | N/A | 0 or 3 bits | Details seen in section 2.1.1, same as DL |
| SRS resource indicator | N/A | 0 or 1 or 2 or 3 or 4 bits | Option 1: Configurable # of bits (0 or 1 or 2 or 3 or 4 bits) Support: ZTE, Samsung, MTK, Ericsson Option 2: Configurable # of bits (0 or 1 or 2) Support: DCM, Spreadtrum |
| Precoding information and number of layers | N/A | 0 or 1 or 2 or 3 or 4 or 5 or 6 bits | Option 1: Configurable # of bits (0 or 1 or 2 bits) Support: Ericsson, DCM (up to 3), MTK, ZTE Option 2: As in Rel-15 Support: Samsung, Spreadtrum, Intel |
| Antenna port(s) | N/A | 2 or 3 or 4 or 5 bits | Option 1: configurable # of bits (0 or 1 bit or 2 bits) Support: ZTE, Spreadtrum, Qualcomm (maybe) Option 2: configurable # of bits (0 or 1 bit or 2 or 3 or 4 or 5 bits) Support: DCM, Intel Option 3: configurable # of bits (0 or 2 or 3 or 4 or 5 bits) Support: Nokia, Ericsson, Sharp, Panasonic 0 bit: Port 0, 0/1, 0/1/2 or 0/1/2/3 for rank 1,2,3 and 4 respectively Single TB with up to 4 layers for PUSCH for URLLC Support: Qualcomm |
| SRS request | N/A | 2 bits for UEs not configured with SUL 3 bits for UEs configured with SUL | Option 1: configurable # of bits (0 or 1) Support: Ericsson, Spreadtrum Option 2: configurable # of bits (0 or 1 or 2) Support: MTK Option 3: configurable # of bits (0 or 1 or 2 or 3) Support: DCM, Samsung, Intel , ZTE, Intel, Sharp, Panasonic |
| CSI request | N/A | 0 or 1 or 2 or 3 or 4 or 5 or 6 bits | Option 1: Configurable # of bits (0 or 1 or 2 bits) Support: ZTE (up to 3 bits), Intel, OPPO, MTK Option 2: Configurable # of bits (0 or 1 or 2 or 3 or 4 or 5 or 6 bits) Support: DCM, Nokia, Intel, Samsung, Ericsson, Sharp, Huawei |
| beta offset indicator | N/A | 0 or 2 bits | Option 1: Configurable # of bits (0 or 1, 2 bits) Support: Ericsson, ZTE, DCM, Nokia, MTK Option 3: Configurable # of bits (0 or 1 bit) Support: Huawei Option 5: Configurable # of bits (0 or 2 bit) Support: Samsung, Intel. Qualcomm, Sharp, Panasonic |
| BWP indicator | N/A | 0 or 1 or 2 bits | Details seen in section 2.1.1, same as DL |
| DMRS-PTRS association | N/A | 0 or 2 bits | Option 1: N/A Support: Ericsson, Spreadtrum Option 2: 0 or 2 bits Support: DCM, Samsung, MTK, Sharp, ZTE |
| CBGTI | N/A | 0 or 2 or 4 or 6 or 8 bits | 0 |
| DMRS sequence initialization | N/A | 0 or 1 bit | Option 1: 0 or 1 bit Support: MTK, Panasonic, DCM, ZTE, Samsung, Ericsson, Sharp, Intel, Nokia Option 3: 1 bit Support: Qualcomm |
| UL-SCH indicator | N/A | 1 bit | Option 1: Configurable # of bits (0 or 1 bit) Support: DCM, MTK, Panasonic, Intel, ZTE, Nokia Option 2: 1 bit Support: Samsung, Qualcomm, Sharp |
| Downlink assignment index | N/A | 1 or 2 or 4 bits | Depend on discussion under UCI enhancements |
| New Fields proposed to be added compared to Rel-15 DCI |  |  |  |
| New format indicator | N/A |  | Depending on the outcome of the discussion of DCI format differentiation |
| Repetition factor | N/A |  | Depend on discussion under PUSCH enhancements |
| Virtual CRC | N/A |  | Support: Panasonic, Huawei |
| Waveform indicator | N/A |  | Option 1: Add waveform indicator (1 bit) in the UL DCI Support: Qualcomm, ZTE |
| Priority indicator | N/A |  | Depending on the outcome of the discussion of service differentiation |
| Physical layer configuration set/option indication | N/A |  | Depending on the outcome of the discussion of service differentiation Option 1: Configurable # of bits for physical layer configuration set/option indication. Support: Ericsson |
| Open-loop indicator | N/A |  | Depend on discussion under inter-UE multiplexing |

In addition to the summary of the above Table 2, some additional information are also provided below for some key aspects on the design for UL DCI format scheduling Rel-16 URLLC. Note that the discussion of the common aspects between DL DCI format and UL DCI format can be found in section 2.1.1.

Fields from both DCI format 0_0 and DCI format 0_1 to be resized or removed

As described in section 2.1.1, some fields from both DCI format 1_0 and DCI format 1_1 are proposed to be resized or removed for DCI format scheduling Rel-16 URLLC, where the design for some fields for UL DCI is the same as that DL DCI, including frequency domain resource assignment, modulation and coding scheme and HARQ process number.

Based on the summary in Table 2, it is reasonable to post-pone the discussion of the following fields due to potential impacts from other functionalities/features under discussion:

- Time domain resource assignment

- Redundancy version

Conclusion 2.2.1-1: For the new DCI format for UL scheduling for Rel-16 URLLC, post-pone the discussion of the following field due to potential impacts from PUSCH enhancements under discussion:

- Time domain resource assignment

- Redundancy version

Frequency hopping flag

For frequency hopping flag, the views from companies are summarized as shown in Table 2 and also copied here for convenience:

- Option 1: 1 bit (No change compared to Rel-15 DCI format 0_0)

  - Support: Qualcomm, Vivo, ZTE, Spreadtrum

- Option 2: Configurable # of bits (0 or 1 bit)

  - Support: Ericsson, DCM, Intel, MTK, Samsung, Panasonic, CATT, Sharp

Based on the above positions from companies, it is possible that a simpler compromised solution is option 2, which can provide full flexibility also.

Proposal 2.2.1-1: Support configurable number of bits (0 or 1 bit) for “Frequency hopping flag” in the new DCI format for UL scheduling for Rel-16 URLLC.

UL/SUL indicator

For UL/SUL indicator, the views from companies are summarized as shown in Table 2 and also copied here for convenience:

- Option 1: As in Rel-15 DCI 0_1 (0 or 1 bit)

  - Support: NTT DOCOMO, Intel, MTK, Samsung, Vivo, Sptreadtrum, Huawei Panasonic, Nokia, ZTE

- Option 2: N/A

  - Support: Ericsson

It seems no clear reason was given for not supporting configurable size for this field in the DCI scheduling Rel-16 URLLC. Therefore, it is more straightforward to keep it in the way as in Rel-15, which provide full flexibility here.

Proposal 2.2.1-2: Keep “UL/SUL indicator (0 or 1 bit)” without any change from Rel-15 DCI format 0_1 in the new DCI format for UL scheduling for Rel-16 URLLC.

TPC command for scheduled PUSCH

For TPC command for scheduled PUSCH, the views from companies are summarized as shown in Table 1 and also copied here for convenience:

- Option 1: 2 bits (i.e. No change compared to Rel-15 DCI)

  - Support: Qualcomm, Ericsson, Huawei, CATT, MTK, DCM, ZTE, Spreadtrum, Nokia, Intel, Spreadtrum

- Option 2: 0 bit or 2 bits in configurable manner

  - Support: Samsung

- Option 3: 0 bit to 2 bit in configurable manner

  - Support: Vivo

It seems the majority view is to reuse the TPC command for scheduled PUSCH from Rel-15 DCI. Companies are encouraged to check the views from Samsung and Vivo. For now, it can be suggested to go to the majority.

Proposal 2.2.1-3: Reuse “TPC command for scheduled PUSCH (2 bits)” from Rel-15 DCI for the new DCI format for UL scheduling for Rel-16 URLLC.

| Company | View |
| --- | --- |

Fields only from Rel-15 DCI format 0_1 to be present in DCI scheduling Rel-16 URLLC

For achieving fully flexibility, several fields from Rel-15 non-fallback DCI (i.e. DCI format 0_1) are proposed to be added to the UL DCI format scheduling Rel-16 URLLC.

CSI request

For CSI, the views from companies are summarized as shown in Table 2 and also copied here for convenience:

- Option 1: Configurable # of bits (0 or 1 or 2 bits)

  - Support: ZTE (up to 3 bits), Intel, OPPO, MTK

- Option 2: Configurable # of bits (0 or 1 or 2 or 3 or 4 or 5 or 6 bits)

  - Support: DCM, Nokia, Intel, Samsung, Ericsson, Sharp, Huawei

Similar as other fields, it seems more flexible to provide the fully configurability by option 2. Therefore, unless significant problem or strong reasons are found to limit the maximum value to be configured, companies are suggested to go with option 2.

Proposal 2.2.1-4: Keep “CSI request (0 or 1 or 2 or 3 or 4 or 5 or 6 bits)” without any change from Rel-15 DCI format 0_1 in the new DCI format for UL scheduling Rel-16 URLLC.

UL-SCH indicator

For UL-SCH indicator, the views from companies are summarized as shown in Table 2 and also copied here for convenience:

- Option 1: 0 or 1 bit

  - Support: DCM, MTK, Panasonic, Intel, ZTE, Nokia

- Option 2: 1 bit

  - Support: Samsung, Qualcomm, Sharp

It seems companies want to include 0 bit for flexibility. For flexibility, option 1 can be considered for UL DCI format scheduling Rel-16 URLLC. However, once 0 bit is configured, the UE behavior needs to be further clarified though. For example, if 0 bit is configured, does it mean that aperiodic CSI is not allowed to be transmitted on the corresponding PUSCH? [Vivo, R1-1910221] proposed that aperiodic CSI is always multiplexed with UL-SCH when CSI request is triggered if 0 bit is configured. To align with the principle for other fields in the new DCI format, companies are encouraged to go with configurable size also.

Proposal 2.2.1-5: Support configurable number of bits (0 or 1 bit) for “UL-SCH indicator” in the new DCI format for UL scheduling for Rel-16 URLLC.

- FFS: If 0 bit is configured, aperiodic CSI is multiplexed with UL-SCH when the CSI request is triggered.

Based on the summary in Table 2, it seems better to post-pone the discussion of the following fields due to potential impacts from other functionalities/features under discussion:

- Beta offset indicator

- Downlink assignment index

Conclusion 2.2.1-2: For the new DCI format for UL scheduling for Rel-16 URLLC, post-pone the discussion of the following fields due to potential impacts from other functionalities/features under discussion:

- Beta offset indicator

- Downlink assignment index

Based on the summary in Table 2 above, companies also provide views on the following fields:

- SRS resource indicator

- Precoding information and number of layers

- Antenna port(s)

- SRS request

- DMRS-PTRS association

- DMRS sequence initialization

Similar as the DCI format for DL scheduling, the above fields are related to MIMO features and also it may be related to multi-TRP discussion in MIMO work item. Based on the views from companies, it seems that the majority view is that these fields should be included in the new DCI format for DL scheduling for sure, and tend to make it configurable and at least the value of “0” should be one candidate value. The main uncertain issues are whether some other smaller values which don’t exist in Rel-15 should be included or not. Therefore, companies are encouraged to check the following proposal:

Proposal 2.1.2-6: For the new DCI format for UL scheduling for Rel-16 URLLC, support configurable number of bits for the following fields:

- SRS resource indicator (0 or 1 or 2 or 3 or 4 bits, same as Rel-15 DCI format 0_1)

- Precoding information and number of layers (0 or 1 or 2 or 3 or 4 or 5 or 6 bits, same as Rel-15 DCI format 0_1)

- Antenna port(s) (0 or 2 or 3 or 4 or 5 bits)

  - FFS 1 bit

- SRS request (0 or 2 or 3 bits)

  - FFS 1 bit

- DMRS sequence initialization (0 or 1 bit, as in Rel-15 DCI format 0_1)

- DMRS-PTRS association (0 or 2 bits, as in Rel-15 DCI format 0_1)

| Company | View |
| --- | --- |

New fields need to be added compared to Rel-15 DCI

Based on the above table 2, it can be observed that several new fields compared to Rel-15 DCI are proposed to add in the UL DCI format scheduling Rel-16 URLLC. However, the fields of repetition factor, priority indicator and power adjustment indicator should depend on the discussion of other functionalities/features, thus the discussion of these fields can be delayed.

Conclusion 2.2.1-1: For UL DCI format scheduling Rel-16 URLLC, post-pone the discussion of the following potential new fields due to potential impacts from other functionalities/features under discussion:

- Repetition factor

- Priority indicator/Physical layer configuration indication

- Open-loop indicator

- New format indicator

More information of other proposed newly added fields (e.g. waveform indicator) can be found in [Qualcomm, R1-1909264]. Companies are encouraged to check the proposal and provide the views whether the new field is needed or not.

More discussion or views needed on the necessity of this new field.

Proposal 2.2.3-1: Further study the following potential new field for UL DCI format scheduling Rel-16 URLLC:

- Waveform indicator

| Company | View |
| --- | --- |

Other key issues related to DCI format design

Based on the contributions from companies, the following issues related to DCI format design are also discussed. Companies are encouraged to check and provide your inputs.

Issue 1: How to address the impact on DCI size budget due to the introduction of new DCI formats?

- Option 1: Enhance the UE DCI size budget to at least “4+1” for Rel-16 URLLC

  - Support: Nokia, WILUS, Sharp, Sequans, Ericsson, ZTE, CATT

  - Two DCI formats out of {DCI format 0_1, DCI format 1_1, DCI format 0_2, DCI format 1_2} can be configured for size alignment if needed

  - Support: ZTE

  - Reason

    - Increasing the DCI size budget should not be an issue, since R16 URLLC UE is expected to be more powerful considering enhanced PDCCH monitoring capability.

- Option 2: No change of the UE DCI size budget “3+1” and perform DCI size alignment if needed for Rel-16 URLLC

  - Support: Samsung, Panasonic, DCM, WILUS

  - Cons: More restriction

- Option 3: Enhance the UE DCI size budget to 4+1 for Rel-16 URLLC only if the support for the new PDCCH monitoring capability is reported and configured separately from those of the Rel-15.

  - Support: Qualcomm

More views are needed before making any decision on this issue. Companies are encouraged to provide their views on this.

| Company | View |
| --- | --- |

Issue 2: How to differentiate DCI formats if the size of the new DCI formats scheduling Rel-16 URLLC is aligned with that of Rel-15 DCI?

- Option 1: Different DCI formats are transmitted in different search space type/different search space sets, e.g. Rel-15 fallback DCI and the new DCI format scheduling Rel-16 URLLC can be transmitted in CSS and USS respectively.

  - Support: LG, DCM

  - Pros:

    - No increase on DCI size budget

  - Cons:

    - Scheduling restriction, especially for UEs with mixed eMBB and URLLC

    - May increase the required number of #CCEs for monitoring

- Option 2: Different DCI formats CRC are scrambled with different RNTIs

  - Support: Huawei, LG, Panasonic, WILUS

  - Pros:

    - No scheduling restriction

    - No increase on PDCCH blind decode

  - Cons:

    - Increase false alarm ratio

- Option 3: Different DCI formats are transmitted in different CORESETs with the condition that UE reports enhanced PDCCH monitoring capability separately from those of the Rel. 15.

  - Support: Qualcomm

  - Pros:

    - Avoids search space sharing between eMBB and URLLC DCIs, which is an issue when the number of BDs/CCEs are increased or when different minimum processing time capabilities are supported on the same carrier.

  - Cons:

    - Only applied to the case that UE has two independent processing units for a serving to process eMBB and URLLC separately

- Option 4: 1 bit to differentiate DCI format 0_0/1_0 and the new DCI formats scheduling Rel-16 URLLC

  - 1 new bit can be added to DCI format 0_0/1_0 in USS

  - Support: Nokia, DCM

  - Pros:

    - Less scheduling restriction

    - No increase on PDCCH blind decode

  - Cons:

    - Only applicable in USS

- Option 5: Different DCI sizes for URLLC and eMBB

  - Support: Qualcomm

  - Pros: Avoids search space sharing between eMBB and URLLC DCIs, which is an issue when the number of BDs/CCEs are increased or when different minimum processing time capabilities are supported on the same carrier.

  - Cons: Only applied to the case that the DCI size budget can be increased

- Option 6: Different DCI sizes for URLLC and eMBB without increasing the DCI size budget

  - Align the DCI size for DCI format 0_1 and DCI format 1_1 if the number of DCI sizes exceed the DCI size budget

  - Support: Intel

  - Pros: No need to increase the DCI size budget

  - Cons: Some restriction

More views are needed before making any decision on this issue, and it is also related to the discussion of service differentiation under UCI enhancements. Companies are encouraged to provide their views on this.

| Company | View |
| --- | --- |

Issue 3: Whether to extend the existing DCI size alignment procedure with the introduction of the new DCI format?

- Option 1: Introduce an extension of Rel-15 DCI size alignment procedure to support new DCI formats and improved DCI-size budget limit.

  - Only applied if the new DCI formats are configured

  - Support: Ericsson

More views are needed before making any decision on this issue. Companies are encouraged to provide their views on this.

| Company | View |
| --- | --- |

Issue 4: Whether to allow Rel-16 new DCI format to be transmitted in CSS?

- Yes: Intel

  - Avoid unnecessary scheduling constraints

More views are needed before making any decision on this issue. Companies are encouraged to provide their views on this.

| Company | View |
| --- | --- |

Summary of the remaining potential proposals on DCI format for discussion in RAN1#98bis

For convenience, this section summarize the remaining potential proposals planned to be further discussed in the meeting for DCI format. Details can be seen in section 2.1 to section 2.3. Note that the proposals agreed in the meeting were removed from the previous version.

Based on the summary on the new DCI format for DL scheduling in section 2.1:

PUCCH resource indicator

Proposal 2.4-1: Support configurable size for “PUCCH resource indicator (0 or 1 or 2 or 3 bits)” for the new DCI format for DL scheduling.

- New RRC parameter is introduced to for the configuration

PDSCH-to-HARQ_feedback timing indicator

Proposal 2.4-2: Support configurable size for “PDSCH-to-HARQ_feedback timing indicator (0 or 1 or 2 or 3 bits)” for the new DCI format for DL scheduling.

- New RRC parameter is introduced to for the configuration

beta offset indicator

Proposal 2.4-3: Support configurable size for “beta offset indicator (0 or 1 or 2 bits)” for the new DCI format for UL scheduling.

- New RRC parameter is introduced to for the configuration

Downlink assignment index

Proposal 2.4-4: Support configurable size for “Downlink assignment index (0 or 1 or 2 or 4 bits)” for the new DCI format for UL scheduling.

- New RRC parameter is introduced to for the configuration

Proposal 2.4-5: Support configurable size for “Downlink assignment index (0 or 1 or 2 or 4 bits)” for the new DCI format for DL scheduling.

- New RRC parameter is introduced to for the configuration

--------------------------------------------------------------------------------

1st downlink assignment index – 1 or 2 bits:

-	1 bit for semi-static HARQ-ACK codebook;

-	2 bits for dynamic HARQ-ACK codebook.

-	2nd downlink assignment index – 0 or 2 bits:

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks;

-	0 bit otherwise.

-	Downlink assignment index – number of bits as defined in the following

-	4 bits if more than one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI;

-	2 bits if only one serving cell is configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 bits are the counter DAI;

----------------------------------------------------------------------------------

Modulation and coding scheme

Proposal 2.1.1-4: Support configurable size for “modulation and coding scheme” for the new DCI formats scheduling Rel-16 URLLC.

- The range of the MCS field is 2~5 bits

- The entries to be indicated

  - Alt.1: the entries to be indicated are configurable by RRC signaling

  - Alt.2: The entries from 0 to 2^(the number of bits)-1 in the existing tables are to be indicated

Company positions for the above proposal are as below:

-------------------------------------------------------------------------------------------------------------------------

- Option 1: Configurable size for the MCS field for the DCI scheduling Rel-16 URLLC

  - Support: Nokia, Huawei, NTT DOCOMO, Vivo, Intel, OPPO, Samsung, InterDigital, CATT, Sharp, Sequans

    - Alt 1: by configuring an anchoring index and the number of bits in the DCI

    - Alt 2: Configurable MCS table size and the entries

    - Alt 3: limiting the number of rows to be indicated

  - Reasons

    - To provide the chance to reduce the number of bits for MCS thus enable a smaller DCI size

    - For URLLC, some entries in the existing MCS table is typically not used, e.g. the entries for 64QAM

- Option 2: No change compared to Rel-15 DCI

  - Support: Qualcomm, Ericsson, MTK, Spreadtrum, Sony, Panasonic

  - Concerns

    - Reducing the size of the MCS would lead to coarser MCS granularity which lead to inefficient allocation of resources

    - Localizing to a subset of the Rel-15 MCS table may not be suitable if the UE moves to a different radio condition

- Option 3: Joint coding of MCS and RV

  - Support: ZTE

-------------------------------------------------------------------------------------------------------------------------

For time domain resource assignment

The following proposal were came up in the Wednesday online discussion:

Proposal: For time domain resource allocation indication for PDSCH for Rel-16 URLLC in new DCI format, using the starting symbol of the PDCCH monitoring occasion in which the DL assignment is detected as the reference of the SLIV is supported.

- FFS whether using the new reference is configurable or not

- To down-select:

  - Alt 1: The new reference is applied if K0=0 for all the configured TDRA entries, otherwise the reference is slot boundary as in Rel-15

  - Alt 2: The new reference is applied for TDRA entries with K0=0, otherwise the reference is slot boundary as in Rel-15

Potential proposal: For time domain resource allocation indication for PDSCH for Rel-16 URLLC in new DCI format, using the starting symbol of the PDCCH monitoring occasion in which the DL assignment is detected as the reference of the SLIV is supported.

- A RRC parameter is used to enable the utilization of the new reference

- To down-select:

  - Alt 1: The new reference is applied if K0=0 for all the configured TDRA entries, otherwise the reference is slot boundary as in Rel-15

  - Alt 2: The new reference is applied for TDRA entries with K0=0, otherwise the reference is slot boundary as in Rel-15

Question 1: What PDSCH mapping type to be used with new reference?

The current mapping type:

Table 5.1.2.1-1: Valid S and L combinations

| PDSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | {0,1,2,3} (Note 1) | {3,…,14} | {3,…,14} | {0,1,2,3} (Note 1) | {3,…,12} | {3,…,12} |
| Type B | {0,…,12} | {2,4,7} | {2,…,14} | {0,…,10} | {2,4,6} | {2,…,12} |
| Note 1: S = 3 is applicable only if dmrs-TypeA-Posiition = 3 |  |  |  |  |  |  |

Proposal: Only PDSCH mapping type B is supported with new reference.

Question 2: Whether to configure the monitoring of the new DCI formats for DL scheduling and UL scheduling separately or jointly for a search space set

- Option 1: Configure the monitoring of the new DCI formats for DL scheduling and UL scheduling jointly as that for DCI format 0_0/1_0 and DCI format 0_1/1_1.

  - Support:

- Option 2: Configure the monitoring of the new DCI formats for DL scheduling and UL scheduling separately.

  - Support:

Proposal: Configure the monitoring of the new DCI formats for DL scheduling and UL scheduling jointly as that for DCI format 0_0/1_0 and DCI format 0_1/1_1

Company positions for the above proposal are as below:

-------------------------------------------------------------------------------------------------------------------------

- Option 1: Changing the reference from slot boundary to some PDCCH symbol (e.g. the starting symbol of PDCCH)

  - Support: Nokia, Qualcomm, Huawei, MTK, Vivo, Spreadtrum, Panasonic, OPPO, Samsung, DCM, ZTE, Vivo, CATT, Sony, WILUS

  - Applied if K0=0, otherwise the reference is slot boundary as in Rel-15

  - Support: Sony, WILUS, CATT

  - Reason: For a same PDSCH duration, different SLIVs will be needed to schedule PDSCH that start at different symbols in a slot, which will increase the number of entries need to be indicated thus require more number of bits for time domain resource assignment. With changing the reference to PDCCH, then much less SLIVs to be indicated.

| TD-RA allocation Length | #PDCCH MOs = 2 | #PDCCH MOs = 4 | #PDCCH MOs = 7 |
| --- | --- | --- | --- |
| Any Length (1-14 OS) | 0 | 1 | 2 |
| 7 OS | 0 | 1 | 2 |
| 4 OS | 1 | 2 | 3 |
| 2 OS | 1 | 2 | 3 |

- Option 2: Use slot boundary as a SLIV reference for TDRA as in Rel-15

  - Support: Ericsson

  - Reasons

    - The reduction (e.g. 1 bit) may not be too much

    - Only applied to the case of same slot scheduling

    - Need an indicator on whether the reference is based on slot boundary or the new reference

-------------------------------------------------------------------------------------------------------------------------

For frequency domain resource assignment

Proposal 2.1.1-2: Further study whether/how to support resource allocation type 0 for frequency domain resource assignment for the new DCI format scheduling Rel-16 URLLC.

-------------------------------------------------------------------------------------------------------------------------

- Option 1: Support also resource allocation type 0 with configurable granularity for the new DCI format scheduling PDSCH (and PUSCH)

  - Support: Nokia

  - Reasons:

    - Provide more flexibility for inter-UE eMBB and URLLC multiplexing by allowing schedule a URLLC PDSCH in a slot around some already allocated eMBB PDSCH whereas RA type 0 was used

    - Improved frequency diversity

- Option 2: Support FDRA type 0 with a scaling factor K to RBG size for Rel-16 DCI format.

  - Support: DCM

- Option 3: Resource allocation type 0 is supported without any change

  - Support: Sony, Ericsson

- Option 4: Resource allocation type 0 is not supported for URLLC scheduling

  - Support: Qualcomm, MTK, Panasonic

-------------------------------------------------------------------------------------------------------------------------

Summary of agreeable proposals from Saturday offline discussion

Increased PDCCH monitoring capability

In Rel.15 NR, the limits of PDCCH BDs/CCEs are specified as following in TS 38.213:

Table 10.1-2: Maximum number of monitored PDCCH candidates per slot for a single serving cell as a function of the subcarrier spacing value kHz, Maximum number of monitored PDCCH candidates per slot and per serving cell. 0; 44. 1; 36. 2; 22. 3; 20. Table 10.1-3: Maximum number of non-overlapped CCEs per slot for a single serving cell as a function of the subcarrier spacing value kHz, Maximum number of non-overlapped CCEs per slot and per serving cell. 0; 56. 1; 56. 2; 48. 3; 32.

According to the agreements achieved in RAN1#96 meeting and also as shown in the eURLLC WID [1], specification will be done on increased PDCCH monitoring capability on the maximum number of non-overlapped CCEs per slot for channel estimation for at least one SCS subject to some restrictions, while enhancements for PDCCH monitoring capability on the maximum number of monitored PDCCH candidates per slot (with potential restrictions) can be further considered. In the following sections, the key issues about increase PDCCH monitoring capability are summarized.

The maximum number of non-overlapped CCEs per slot for channel estimation

As discussed in the study item phase, increasing the maximum number of non-overlapped CCEs per slot for channel estimation can bring benefits like potential reducing latency and improving the PDCCH blocking, however UE complexity is the main concern. Therefore, careful consideration is needed towards achieving an optimal balance between scheduling flexibility and improved blocking performance against UE complexity and power consumption. To make sure that the increase of number of non-overlapping CCEs does not bring significant UE processing complexity, restrictions should be defined.

The per-CC limit on the maximum number of non-overlapping CCEs per monitoring span

In the RAN1#97 meeting, a working assumption for defining the limit on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span was agreed, and some further agreements were achieved in the RAN1#98 meeting, with some remaining issues for FFS. In order to determine the specific value of C for a combination of (X, Y), several aspects need to be discussed first.

Issue 1:  Whether to support (3, 3) and/or (3, 2) in addition to (2, 2), (4, 3) and (7, 3) as given in UE feature 3-5b?

- (3, 2)

  - Yes: Nokia, LG, Panasonic, DCM

    - To achieve 4 uniform PDCCH monitoring spans within a lot

    - It is sub-optimal to use the combination (4, 3) to achieve 4 PDCCH monitoring occasions within a slot

  - No: ZTE, Vivo, OPPO, Spreadtrum, Huawei, CATT, Sharp

- (3, 3)

  - Yes: Ericsson, Qualcomm, DCM, Panasonic

    - To achieve 4 PDCCH monitoring spans within a lot

  - No: ZTE, Vivo, OPPO, Spreadtrum, Huawei, CATT, OPPO, Sharp

According to the definition of span, more than one PDCCH monitoring occasions can be allocated within a span, therefore there is no problem to support 4 PDCCH monitoring occasions using the combination of (4, 3), though the duration and separation might not be uniform. Therefore, the gain we can get from a combination of (3, 3) or (3, 2) might be not that significant. Generally speaking, unless there is strong motivation or gain, it is preferred not to add more combination, since it may increase the UE implementation complexity. However, companies are encouraged to check and provide the views since not much input for now.

Proposal 3.1-1: Further study whether to support combination (3, 3) and/or (3, 2) for Rel-16 PDCCH monitoring capability.

Proposal: No additional support of combination (3, 3) and (3, 2) for Rel-16 PDCCH monitoring capability.

Issue 2:  How to handle the co-existence of the new PDCCH monitoring capability with the existing Rel-15 slot-level PDCCH monitoring capability?

In the RAN1#98 meeting, two options were agreed to be further down-selected for this issue.

- Option 1: PDCCH monitoring based on Rel-15 capability for eMBB and PDCCH monitoring based on Rel-16 capability for URLLC can be configured to a UE on the same carrier

  - UE monitors PDCCH for eMBB following reported Rel-15 capability, and monitors PDCCH for URLLC following reported Rel-16 capability

  - For Rel-16 PDCCH monitoring capability, the limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span is the same across different spans within a slot. Each span for Rel-16 PDCCH only cover USS for URLLC (FFS for CSS)

  - Support: Ericsson, Qualcomm, LG, OPPO, Panasonic, Spreadtrum, InterDigital

  - Pros:

    - Different processing unit for eMBB and URLLC, thus no PDCCH dropping or DCI budget issue for URLLC, no impact on the existing limit for eMBB

  - Cons:

    - Only applied to the case that UE has two independent processing units for a serving cell to process eMBB and URLLC separately

    - Need to distinguish between eMBB and URLLC

    - No any sharing of PDCCH monitoring between eMBB and URLLC, which may result in a waste of processing power at the UE especially if the eMBB and URLLC uses the same DCI format, and/or the CORESET for eMBB and URLLC are overlapped

    - The number of CCs that a UE can support would become smaller since two “virtual” CCs will be used to handle eMBB and URLLC on a serving cell

- Option 2: PDCCH monitoring for both eMBB and URLLC can be configured based on either Rel-15 capability or Rel-16 capability

  - gNB configures which capability is used

  - For Rel-16 PDCCH monitoring capability,

    - The limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span is the same across different spans within a slot, each span can cover CSS and/or USS

  - Support: Intel, MediaTek, CATT, DCM, Vivo, Huawei

  - Pros:

    - Transparent to service and traffic types

    - No restriction on UE implementation that two independent processing units for a serving cell to process eMBB and URLLC separately

    - Allow sharing of the PDCCH processing power for eMBB and URLLC

  - Cons:

    - Need to ensure that the Rel-16 capability is sufficiently increased to accommodate the cases of UE running multiple service

    - More specification effort on discussing dropping rule or DCI size budget, etc

- Modified option 1: Option 1 is supported with changing eMBB to Rel-15 DCI formats and URLLC to new DCI formats, i.e., PDCCH monitoring based on Rel-15 capability for legacy DCI in Rel-15 and PDCCH monitoring based on Rel-16 capability for new DCI in Rel-16 can be configured to a UE on the same carrier

  - Support: ZTE

Based on the views in the contributions, before making decision on which option to choose, some further clarification are needed for both option 1 and option 2 first. The aspects mentioned in the contributions to be further clarified are summarized as below with some of them maybe better to be clarified before making decision:

For option 1,

Question 1: How to define the separation/identification of the PDCCH monitoring based on Rel-15 PDCCH monitoring capability and the PDCCH monitoring based on the Rel-16 PDCCH monitoring capability?

- CORESET level

  - Qualcomm: Under Option 1, the configuration of PDCCH monitoring occasions for monitoring Rel. 15 and Rel. 16 DCI formats are separated by using different CORESETs or DCI sizes

- Search space set level

- DCI format level

  - Qualcomm: Under Option 1, the configuration of PDCCH monitoring occasions for monitoring Rel. 15 and Rel. 16 DCI formats are separated by using different CORESETs or DCI sizes

    - Question to be clarified: In this case, same CORESET or different CORESETs for Rel-15 PDCCH monitoring and Rel-16 PDCCH monitoring?

  - Pro for doing separation:

    - Avoid sharing of PDCCH monitoring between eMBB and URLLC, thus reduce the processing time needed for URLLC

  - Cons for doing separation:

    - No any sharing of PDCCH monitoring between eMBB and URLLC, which may result in a waste of processing power at the UE especially if the eMBB and URLLC uses the same DCI format, and/or the CORESET for eMBB and URLLC are overlapped

| Company | View |
| --- | --- |
| InterDigital | In Option 1 the UE should know in advance which subset of PDCCH candidates might correspond to URLLC-scheduling DCIs so that most time-critical processing can start immediately. From configuration perspective this corresponds to an additional search space set associated to a different Coreset and/or different DCI format/size. The network can decide how the differentiation is achieved. |

Note that [Intel, R1-1910660] has some analysis on the above three aspects, companies are encouraged to check and provide your views.

[Nokia, R1-1910798] mentioned that the impacts on CA needs to be clarified also. In addition, [Huawei, R1-1910066] [Intel, R1-1908645] mentioned regardless of option 1 or option 2, how to define the limit considering CA with same or different numerology needs to be discussed under the enhanced PDCCH monitoring capability.

Question 2: How does UE report its capability with CA? e.g., does UE additionally report the number of CCs for Rel-15 monitoring capability only and report the number of CCs for Rel-16 monitoring capability only?

| Company | View |
| --- | --- |

Question 3: Do we allow the number of CCs configured with Rel-16 monitoring capability to be larger than the reported UE capability? If yes, how to scale the monitoring capability if the number of CCs configured is larger than the reported capability

| Company | View |
| --- | --- |

Question 4: For Rel-16 monitoring capability, is overbooking still allowed? Do we allow the number of CCs configured with Rel-16 monitoring capability to be larger than the reported UE capability?

- Option 1: The CCE/BD overbooking is only performed on the PCell and for the PDCCH configurations that are based on the Rel. 15 UE capability. For the PDCCH configurations that are based on the enhanced PDCCH monitoring capability, overbooking is not supported.

  - Support: Qualcomm

| Company | View |
| --- | --- |

Question 5: Does option 1 depend on any other factors, e.g. UE processing cap#1 vs cap#2?

| Company | View |
| --- | --- |

For option 2,

Question 1: How does UE report its capability with CA? e.g., does UE additionally report the number of CCs for Rel-15 monitoring capability only, and report the number of CCs for Rel-16 monitoring capability only?

- Option 1: UE reports its capability on the number of CCs to be supported for Rel-15 monitoring capability and for Rel-16 monitoring capability separately

  - Support:

| Company | View |
| --- | --- |

Question 2: How to scale the monitoring capability if the number of CCs configured is larger than the reported capability?

| Company | View |
| --- | --- |

Proposal 3.1-2: Further study whether option 1 or option 2 should be supported for Rel-16 PDCCH monitoring capability.

Issue 3:  What is the detailed value of C for a certain combination?

Regarding the value of C, some companies provide some views as summarized below.

|  | X | Y | C |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | =0 | =1 | =2 | =3 |  |
| Combination 1 | 2 | 2 | [48] | [48] | [36] | [24] | Nokia |
|  | 2 | 2 | [48] | [48] | - | - | Samsung |
|  | 2 | 2 | 32 | 32 | 32 | 32 | DCM |
|  | 2 | 2 | 16 | 16 | - | - | OPPO |
|  | 2 | 2 | 32 | 32 | - | - | Vivo |
|  | 2 | 2 | 16 | - | - | - | ZTE |
|  | 2 | 2 | 28 | - | - | - | Huawei, |
|  | 2 | 2 | 24 | 24 | 16 | 16 | Ericsson |
|  | 2 | 2 | 16 | 16 | 16 | - | CATT |
|  | 2 | 2 | 16 | 16 | - | - | Sharp, |
|  | 2 | 2 | 16 | 16 | - | - | Spreadtrum |
|  | 2 | 2 | 24 | 24 | - | - | China Unicom |
|  | 2 | 2 | 24 | 24 | - | - | Panasonic (Option 2) |
| Combination 2 | 4 | 3 | 56 | 56 | - | - | Samsung |
|  | 4 | 3 | 56 | 32 | 32 | 32 | DCM |
|  | 4 | 3 | 74 | 74 | - | - | Vivo |
|  | 4 | 3 | [56] | [56] | [48] | [32] | Nokia |
|  | 4 | 3 | 36 | - | - | - | ZTE |
|  | 4 | 3 | 40 | 40 | 32 | 24 | Ericsson |
|  | 4 | 3 | 32 | 32 | - | - | Spreadtrum |
|  | 4 | 3 | 36 | 36 | 32 | - | Sharp |
|  | 4 | 3 | 32 | 32 | - | - | OPPO |
|  | 4 | 3 | 32 | 32 | 24 | - | CATT |
|  | 4 | 3 | 36 | 36 | - | - | China Unicom |
|  | 4 | 3 | 36 | 36 | - | - | Huawei |
|  | 4 | 3 | 34 | 34 | - | - | Panasonic (Option 2) |
| Combination 3 | 7 | 3 | 56 | 56 | 48 | 32 | Nokia |
|  | 7 | 3 | 56 | 56 | 48 | - | Samsung |
|  | 7 | 3 | 64 | 64 | - | - | OPPO |
|  | 7 | 3 | 112 | 112 | - | - | Vivo |
|  | 7 | 3 | 56 | - | - | - | ZTE |
|  | 7 | 3 | 56 | 48 | 32 | 32 | DCM, |
|  | 7 | 3 | 56 | 56 | - | - | Spreadtrum |
|  | 7 | 3 | 56 | 56 | 48 | - | Sharp |
|  | 7 | 3 | 56 | 56 | 48 | - | CATT |
|  | 7 | 3 | 56 | 56 | 48 | 32 | Ericsson |
|  | 7 | 3 | 56 | 56 | - | - | China Unicom |
|  | 7 | 3 | 56 | 56 | - | - | Huawei |
|  | 7 | 3 | 44 | 44 | - | - | Panasonic (Option 2) |

Note that the value from Vivo and Panasonic is for the case of UEs supporting both URLLC and eMBB. For UEs only supporting URLLC, a small value is provided [Vivo, R1-1910221].  Based on the above table, it seems the values are very diverse for combination (2, 2) and (4, 3). For (7, 3), it seems the majority view is to support 56 per span. Maybe we can try to get some conclusion for combination (7, 3), while further discussion is needed for the combination (2, 2) and (4, 3).

Proposal 3.1-3: The value of C for combination (7, 3) for 15 kHz and 30 kHz is 56.

- FFS C for combination (2, 2) and (4, 3)

| Company | View |
| --- | --- |

Issue 4:  What is the actual CCE limit per monitoring span if there is empty span(s) within a slot?

[Ericsson, R1-1910545] raised the issue that if there is empty spans within a slot, it may result in that the total limit in some slot(s) is worse than Rel-15 capability. Therefore, empty spans should be taken into account when determining the actual CCE limit per monitoring span.  The following options are proposed by companies.

- Option 1: If UE reports the support of more than one combination of C(X, Y) for a given SCS, and for any slot j, if multiple combinations of C(X, Y) are valid for the span pattern, the per-span limit for slot j is given by

- maxmaxC(X,Y) valid for the span pattern, the per-slot limit #non-empty spans in a slot j

  - Support: Ericsson, Spreadtrum

- Option 2: The actual PDCCH monitoring span is determined following the following rule:

  - In a given slot, the numbers of BDs/CCEs in a span may equal the slot-level limit only if all PDCCH candidates in the span correspond to PDCCH CSS sets including Types 0, 0A, 1, 3, and 2 (without RRC configuration) PDCCH CCS sets. In this case, there can only be a single PDCCH span in the particular slot.

  - If the span includes PDCCH candidates from UE-SS or Type 2 CSS that are UE-specifically configured to the UE or there are multiple spans within a slot duration, the span-level limits (that are no larger than the slot-level limits) apply.

  - Support: Intel

- Option 3: If UE reports the support of one or more combination of C(X, Y) for a given SCS, the maximum number of non-overlapped CCEs per slot per cell is applied if there is only one span in the span pattern in every slot

  - Support: ZTE

- Option 4: If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y) are valid for the span pattern, the maximum value of C of the valid combinations is applied for each non-overlapped in time domain PDCCH resource set

  - Support: OPPO

- Option 5: The number of CCEs for an empty span is added to a span before the empty span, subject to capability constraint

  - Support: Asia Pacific Telecom

Proposal 3.1-4: Further study the impact from empty span.

Issue 5:  What is the UE behavior if the obtained PDCCH monitoring span arrangement is invalid?

- Option 1: Dropping some PDCCH monitoring occasions till the span arrangement is valid.

  - Support: MTK

[MTK, R1-1906565] mentioned this issue and proposed option 1 above. More discussion are needed here to see whether to consider this as error case first, if not then discuss the potential UE behavior. Companies are encouraged to provide views on this.

| Company | View |
| --- | --- |

Applicable SCS(s) for increased PDCCH monitoring capability on non-overlapped CCEs

In the RAN1#98 meeting, it was agreed to support 15 kHz and 30 kHz for enhanced PDCCH monitoring capability in Rel-16. FFS 60 kHz and 120 kHz.

For 120 kHz,

- No need to enhance the PDCCH monitoring capability for 120 kHz

  - MTK, OPPO, Spreadtrum, Huawei, Vivo, China Unicom, Intel, Panasonic

- All supported combinations (X, Y) are applicable also for 120 kHz

  - Nokia, Ericsson, DCM

For 60 kHz,

- No need to enhance the PDCCH monitoring capability for 60 kHz

  - MTK, OPPO, Spreadtrum, Huawei, Vivo, China Unicom, Intel, Panasonic

- All supported combinations (X, Y) are applicable also for 60 kHz

  - Nokia, Ericsson, CATT, DCM

  - Reasons

    - 4 monitoring occasions per slot are needed to allow 1 HARQ retransmission within 1 ms latency, and 7 monitoring occasions per slot are needed to allow 2 HARQ retransmission within 1 ms latency

- Support combination (7, 3) for 60 kHz

  - Samsung

- Support combination (7, 3) and (4, 3) for 60 kHz

  - Sharp

It seems the views are still controversial. Further discussion is needed before making the decision.

Proposal 3.1-5: Further study whether/how to apply Rel-16 PDCCH monitoring capability for 60 kHz and 120 kHz.

Proposal: Rel-16 PDCCH monitoring capability is not supported for 60 kHz and 120 kHz.

| Company | View |
| --- | --- |

Impact on PDCCH overbooking and dropping rules

With the introduction of PDCCH monitoring capability based on span, it seems the PDCCH candidate dropping rule should be updated. It seems straightforward that the PDCCH candidate dropping is performed per PDCCH monitoring span as discussed in [Ericsson, R1-1910545][Nokia, R1-1910798].

- PDCCH dropping is performed per span if Rel-16 enhanced PDCCH monitoring capability is applied for PDCCH monitoring.

  - Ericsson, Nokia, ZTE, Vivo

As to the details of how to perform the dropping, some candidate solutions are proposed by companies as summarized below:

- Option 1: If the number of non-overlapping CCE for channel estimation of the configured PDCCH candidates to monitor in some monitoring span j exceeds the CCE limit per monitoring span of the span j, UE can skip monitoring all PDCCH candidates in span j.

- Option 2: If the number of non-overlapping CCE for channel estimation of the configured PDCCH candidates to monitor in some monitoring span j exceeds the CCE limit per monitoring span of the span j, UE can skip monitoring all PDCCH candidates in the search space sets with highest search space set indices in span j until the number of non-overlapping CCE of remaining PDCCH candidates to monitor in the monitoring span j does not exceed the CCE limit per monitoring span for span j.

- Option 3: If the number of non-overlapping CCE for channel estimation of the configured PDCCH candidates to monitor in some monitoring span j exceeds the CCE limit per monitoring span of the span j, UE can skip monitoring some PDCCH candidates in the search space sets with highest search space set indices in span j until the number of non-overlapping CCE of remaining PDCCH candidates to monitor in the monitoring span j does not exceed the CCE limit per monitoring span for span j.

  - Ericsson,

Based on the above inputs, and considering the potential new definition on the limitation of increased PDCCH monitoring capability and higher priority of URLLC, it seems reasonable to further study on PDCCH dropping rule. However, [Qualcomm, R1-191118] [Samsung, R1-1910483] describe that CCE/BD overbooking is not needed for URLLC, because DCI for URLLC will only be transmitted in USS.

Proposal 3.1-6: Further study enhancements on PDCCH overbooking, including BD/CCE counting and the PDCCH dropping rule.

In addition, [Motorola, R1-1911299] proposed to study solutions to reduce scheduling delay in case of overlap of a PDCCH monitoring occasion with an UL transmission, and specified if needed. Companies are encouraged to check and provide views.

Enhancements for the maximum number of monitored PDCCH candidates per slot

In the RAN1#96 meeting, it was agreed that enhancements for PDCCH monitoring capability on the maximum number of monitored PDCCH candidates per slot (with potential restrictions) can be further considered.

Some analysis from study item phase can be kept here for study whether enhancements for PDCCH monitoring capability on the maximum number of monitored PDCCH candidates per slot are needed or not.

Maximum number of PDCCH monitoring occasions needed for URLLC per slot

The key aspect related to whether to enhance the PDCCH capability is the potential number of PDCCH monitoring occasions needed for URLLC per slot. Several companies provides the views as below:

- At least 3: MediaTek (only for SCS of 15 kHz)

- At least 4: CATT (for FDD and 60 kHz), Nokia, Ericsson

- 7: Vivo, Qualcomm, Nokia, NTT DOCOMO, ZTE, OPPO, Asia Pacific Telecom, ETRI

Some companies provide detailed analysis on how to get the numbers:

Contribution [MediaTek, R1-1906565] Figure 1 and Figure 2 show the CDF of the latency for a single shot transmission for SCS = 15 kHz and SCS = 30 kHz. Various PDCCH monitoring periodicities are evaluated to determine the periodicity needed to meet the latency requirement. For example for SCS = 15 kHz and with a single shot transmission, PDCCH monitoring configuration with 4 OS periodicity is needed to meet the 1ms latency required, which is equivalent to at least 3 PDCCH monitoring occasions configured. Figure 1: SCS =15 kHz; Figure 2: SCS =30 kHz.

Contribution [Qualcomm, R1-1903004] Focusing on the DL direction and considering that completing two transmissions within the latency budget of 1ms is critical for an efficient operation, we analyze the achievable latency under Case 1-1 and Case 2 with different number of monitoring occasions as follows (Note that in the analysis below, we have assumed multiple HARQ-ACK reporting per slot is allowed): - Case 2 with SCS = 30KHz, N1 = N3 = 4.5 symbols, a half-symbol propagation delay and four monitoring occasions per slot: Figure 2: eURLLC latency assuming four PDCCH monitoring occasion per slot (Case 2). - Case 2 with SCS = 30KHz, N1 = N3 = 4.5 symbols, a half-symbol propagation delay and seven monitoring occasions per slot: Figure 3: eURLLC latency assuming seven PDCCH monitoring occasion per slot (Case 2). Observation 1: For eURLLC with stringent latency requirements, a frequent PDCCH monitoring, e.g., in units of every 2, is necessary. Proposal 1: To enable fast scheduling for eURLLC, RAN1 considers the feasibility of increasing the number of BD/CCE limit. The required conditions and relaxations should be studied.

Contribution [CATT, R1-1902002, R1-1902005] For the reference Case 1, where the gNB processing time is associated with the UE N1/N2 processing time, it was observed in [3] that for FDD and 60 KHz SCS at least four monitoring occasions may be required per slot to achieve 1ms latency budget for DL scheduling when provisioning for at least one HARQ retransmission. Contribution [CATT, R1-1902005] We assumed two cases for gNB processing time assumptions to model different assumptions on the base station load as follow where X=2 for 30kHz SCS and X=4 for 60kHz SCS, - Case 1: Processing time for scheduling the initial PDSCH is N2/2 + X and decoding time for the last PUSCH is N1/2+X - Case 2: Processing time for scheduling the initial PDSCH is N2+X and decoding time for the last PUSCH is N1+X. Table 1: Latency analysis under Rel-15 N1/N2 values (FDD) gNB proc time assumption; SCS (kHz); # MO/slot; TTI (OS); DL; UL ConfiguredGrant. 1 Tx (ms); 2 Tx (ms); 1 Tx (ms); 2 Tx (ms). Case 1; 30; 4; 2; 0.58; 1.22; 0.39; 1.07. 4; 0.72; 1.51; 0.61; 1.29. 7; 0.94; 1.94; 0.75; 1.54. 7; 2; 0.51; 1.15; 0.39; 1.04. 4; 0.65; 1.37; 0.61; 1.29. 7; 0.87; 1.87; 0.75; 1.54. 60; 4; 2; 0.46; 0.96; 0.32; 0.86. 4; 0.53; 1.1; 0.43; 0.96. 7; 0.63; 1.21; 0.5; 1.13. 7; 2; 0.42; 0.92; 0.32; 0.82. 4; 0.49; 1.03; 0.43; 0.96. 7; 0.6; 1.17; 0.5; 1.13. Case 2; 30; 4; 2; 0.68; 1.32; 0.47; 1.15. 4; 0.82; 1.61; 0.69; 1.37. 7; 1.04; 2.04; 0.83; 1.62. 7; 2; 0.61; 1.25; 0.47; 1.12. 4; 0.75; 1.46; 0.69; 1.37. 7; 0.96; 1.96; 0.83; 1.62. 60; 4; 2; 0.55; 1.05; 0.4; 0.94. 4; 0.63; 1.2; 0.51; 1.04. 7; 0.73; 1.3; 0.58; 1.21. 7; 2; 0.52; 1.02; 0.4; 0.9. 4; 0.59; 1.13; 0.51; 1.04. 7; 0.7; 1.27; 0.58; 1.21.

Observation 3.2-1: At least 3 PDCCH monitoring occasions per slot are needed for SCS of 15 kHz assuming one shot transmission for URLLC within 1 ms latency budget. 1 PDCCH monitoring occasion per slot may be sufficient for SCS of 30 kHz, 60 kHz and 120 kHz assuming one shot transmission for URLLC within 1 ms latency bound.

Observation 3.2-2: 7 PDCCH monitoring occasions per slot are needed for SCS of 30 kHz assuming two transmissions for URLLC within 1 ms latency budget.

Observation 3.2-3: At least 4 PDCCH monitoring occasions per slot are needed for SCS of 60 kHz assuming two transmissions for URLLC within 1 ms latency budget.

The related issues are discussed under the agenda item of enhanced scheduling/HARQ processing timeline also. More observations can be drawn from that session maybe.

| Company | View |
| --- | --- |

Evaluation on NR Rel-15 PDCCH capability

In Rel.15 NR, the limits of PDCCH BDs/CCEs are specified as following in TS 38.213:

Table 10.1-2: Maximum number of monitored PDCCH candidates per slot for a single serving cell as a function of the subcarrier spacing value kHz, Maximum number of monitored PDCCH candidates per slot and per serving cell. 0; 44. 1; 36. 2; 22. 3; 20.

Some companies provide some analysis and/or evaluation on NR Rel-15 PDCCH capability. Some examples are given below:

Contribution [Nokia, R1-1910798] To understand better the impact of the number of BDs, we have performed some evaluation on the blocking probability with different assumptions on the number of BDs. The evaluation is based on some simplified assumptions: - A total of 34 CCEs in the CORESET - This corresponds to the case of 2-symbol CORESET with 40 MHz bandwidth and 30kHz SCS. - Distribution of ALs (based on PDCCH link level performance assume 10-5 target BLER and the geometry from the system level) is as follows: AL; 1; 2; 4; 8; 16. Probability; 27%; 28%; 33%; 11.5%; 0.5%. - The number of UEs to be scheduled per monitoring occasion: 2 to 8 (an input parameter) - The number of candidates for each AL is also an input parameter. The following numbers for ALs (1, 2, 4, 8, 16) has been simulated: - (2, 2, 2, 1, 1) – 8 candidates per monitoring occasion - (4, 4, 2, 2, 2) – 14 candidates per monitoring occasion - (6, 6, 2, 2, 2) – 18 candidates per monitoring occasion - The output is the blocking probability. - The simulation performs an exhaustive search among all the feasible candidates (with ALs that can satisfy 10-5 target BLER) of all the UEs to maximize the number of UEs that can be scheduled at a time. Each monitoring occasion is independently simulated. Figure. 3-2 Blocking probability for different number of UEs and different number of candidates for each AL. As can be seen from Figure 3-2, the blocking probability improves as more candidates become available. Note that the assumed number of candidates is already beyond what can be supported in Rel-15 in case of 4 monitoring occasions per slot. Reducing the blocking probability by one order (e.g. from 10-2 to 10-3) can be critical for URLLC because it immediately affects the latency of the final packet delivery. Observation 3-2: The maximum number of BDs in Rel-15 is not sufficient for URLLC with multiple monitoring occasions per slot. Increasing the limit would alleviate its impact on PDCCH blocking probability and improve URLLC performance.

Contribution [Spreadtrum, R1-1908954] For blind decoding, we do not prefer to use larger BD limits, other implementation methods such as fewer PDCCH candidates can be configured for every monitoring occasion. As shown in Table 2, more occasions are configured, the current blind decoding number can at least provide 2 PDCCH candidates for every PDCCH monitoring occasion. So we prefer not to increase the PDCCH blind decoding number for Rel-16 URLLC. Table 2: Max no. of PDCCH BDs per monitoring occasion SCS; Max no. of PDCCH BDs per slot; Max no. of PDCCH BDs per monitoring occasion. 7 monitoring occasions per slot; 4 monitoring occasions per slot. 15 kHz; 44; 6; 11. 30 kHz; 36; 4; 9. 60 kHz; 22; 3; 5. 120 kHz; 20; 2; 5. - The number of PDCCH blind decoding does not increase in Rel-16 NR URLLC.

In addition, based on the maximum number of PDCCH monitoring occasions observed in section 3.2.1, we can do a simple calculation as shown in the following Tables:

Table 3 Number of available BDs per PDCCH monitoring occasion based on Rel-15 PDCCH capability assuming 7 PDCCH candidates for CSS

|  | Average number of PDCCH candidates per monitoring occasion in USS |
| --- | --- |
| 15kHz SCS, 3 monitoring occasions per slot assuming one short transmission for URLLC within 1ms latency budget | ~12 |
| 30kHz SCS, 7 monitoring occasions per slot assuming two transmissions for URLLC within 1ms latency budget | ~4 |
| 60kHz SCS, 4 monitoring occasions per slot assuming two transmissions for URLLC within 1ms latency budget | ~4 |

From Table 3 there are about 12 PDCCH candidate available per PDCCH monitoring occasions for SCS of 15 kHz, it may be sufficient. For SCS of 30 kHz and 60 kHz, the available number of PDCCH candidates per PDCCH monitoring occasion is about 4. From single URLLC only UE perspective, it may be sufficient.

Observation 3.2-4: For SCS of 15 kHz, Rel-15 PDCCH monitoring capability may be sufficient from the maximum number of monitored PDCCH candidates per slot perspective, assuming 3 PDCCH monitoring occasions per slot.

Observation 3.2-5: For SCS of 30 kHz and 60 kHz, Rel-15 PDCCH monitoring capability may be sufficient from the maximum number of monitored PDCCH candidates per slot perspective at least for UEs only monitoring URLLC service, assuming 7 PDCCH monitoring occasions per slot for 30 kHz and 4 PDCCH monitoring occasions per slot for 60 kHz.

Observation 3.2-6: The current UE capability on the maximum number of BDs and the maximum number of CCEs for channel estimation for Case 2 in NR is much lower than for Rel-15 LTE sTTI.

| Company | View |
| --- | --- |

Impact on UE complexity

In theory, the increase of PDCCH monitoring capability can provide more flexibility for URLLC scheduling and provide more chances to reduce the latency. However, according to the discussion in Rel-15, it was observed that the limit of BDs do have much impact on UE complexity. Therefore, if enhancements on the maximum number of monitored PDCCH candidates per slot is necessary, then restrictions similar as that for the maximum number of non-overlapped CCEs per slot for channel estimation should be defined.

Enhancements on the maximum number of monitored PDCCH candidates per slot

Some companies provide the views on whether enhancements on the maximum number of monitored PDCCH candidates per slot needed as summarized below:

- Option 2: Enhance the maximum number of monitored PDCCH candidates per slot

  - Company position: Nokia, LG, CATT, Ericsson, NTT DOCOMO, ZTE, Panasonic, Aisa Pacific Telecom,

- Option 3: No enhancements on PDCCH monitoring capability

  - Company position: Spreadtrum, Huawei

Based on the initial analysis above, it seems the motivation to support the enhancements on the maximum number of monitored PDCCH candidates per slot is not strong. However, it seems not many companies provide very clear view on this, thus further inputs may be needed before making decision. Though companies are encouraged to go to the majority view.

Proposal 3.2-1: Further study on whether enhancement on the maximum number of monitored PDCCH candidates per slot is needed or not.

In addition, once the design for increased PDCCH monitoring capability on the maximum number of non-overlapped CCEs per slot for channel estimation is clear, similar frame work can be used for increased PDCCH monitoring capability for the maximum number of monitored PDCCH candidates per slot.

| Company | View |
| --- | --- |

Limit of the maximum number of monitored PDCCH candidates

Some companies [Nokia, R1-1910798] provide the views on the potential maximum number of BDs if enhanced PDCCH monitoring capability on the maximum number of monitored PDCCH candidates per slot is adopted.

- The same span based framework for enhanced PDCCH monitoring capability on the maximum number of non-overlapped CCEs per slot for channel estimation, is also used for the enhanced PDCCH monitoring capability on the maximum number of monitored PDCCH candidates per slot

  - Support: Nokia, Ericsson

From feature lead perspective, if in the end we agree to support enhanced PDCCH monitoring capability on the maximum number of monitored PDCCH candidates per slot, then same frame work for the maximum number of non-overlapping CCEs for channel estimation should be reused.

Summary of potential proposals on PDCCH capability for discussion in RAN1#98bis

For convenience, this section summarize the potential proposals planned to be further discussed in the meeting for increased PDCCH monitoring capability. Details can be seen in section 3.1 to section 3.2. In addition, if time allowed, we will discuss whether to support enhancement on the maximum number of monitored PDCCH candidates per slot.

However, based on the current situation, it seems difficult for us to have any proposal unfortunately. Companies are encouraged to provide your views and then we can see what direct to go.

Proposal 3.1-2: Further study whether option 1 or option 2 should be supported for Rel-16 PDCCH monitoring capability.

Company positions for option 1 and option 2 as below:

-------------------------------------------------------------------------------------------------------------------------

- Option 1: PDCCH monitoring based on Rel-15 capability for eMBB and PDCCH monitoring based on Rel-16 capability for URLLC can be configured to a UE on the same carrier

  - UE monitors PDCCH for eMBB following reported Rel-15 capability, and monitors PDCCH for URLLC following reported Rel-16 capability

  - For Rel-16 PDCCH monitoring capability, the limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span is the same across different spans within a slot. Each span for Rel-16 PDCCH only cover USS for URLLC (FFS for CSS)

  - Support: Ericsson, Qualcomm, LG, OPPO, Panasonic, Spreadtrum, InterDigital

  - Pros:

    - Different processing unit for eMBB and URLLC, thus no PDCCH dropping or DCI budget issue for URLLC, no impact on the existing limit for eMBB

  - Cons:

    - Only applied to the case that UE has two independent processing units for a serving cell to process eMBB and URLLC separately

    - Need to distinguish between eMBB and URLLC

    - No any sharing of PDCCH monitoring between eMBB and URLLC, which may result in a waste of processing power at the UE especially if the eMBB and URLLC uses the same DCI format, and/or the CORESET for eMBB and URLLC are overlapped

    - The number of CCs that a UE can support would become smaller since two “virtual” CCs will be used to handle eMBB and URLLC on a serving cell

- Option 2: PDCCH monitoring for both eMBB and URLLC can be configured based on either Rel-15 capability or Rel-16 capability

  - gNB configures which capability is used

  - For Rel-16 PDCCH monitoring capability,

    - The limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span is the same across different spans within a slot, each span can cover CSS and/or USS

  - Support: Intel, MediaTek, CATT, DCM, Vivo, Huawei

  - Pros:

    - Transparent to service and traffic types

    - No restriction on UE implementation that two independent processing units for a serving cell to process eMBB and URLLC separately

    - Allow sharing of the PDCCH processing power for eMBB and URLLC

  - Cons:

    - Need to ensure that the Rel-16 capability is sufficiently increased to accommodate the cases of UE running multiple service

    - More specification effort on discussing dropping rule or DCI size budget, etc

- Modified option 1: Option 1 is supported with changing eMBB to Rel-15 DCI formats and URLLC to new DCI formats, i.e., PDCCH monitoring based on Rel-15 capability for legacy DCI in Rel-15 and PDCCH monitoring based on Rel-16 capability for new DCI in Rel-16 can be configured to a UE on the same carrier

  - Support: ZTE

Based on the views in the contributions, before making decision on which option to choose, some further clarification are needed for both option 1 and option 2 first. The aspects mentioned in the contributions to be further clarified are summarized as below with some of them maybe better to be clarified before making decision:

For option 1,

Question 1: How to define the separation/identification of the PDCCH monitoring based on Rel-15 PDCCH monitoring capability and the PDCCH monitoring based on the Rel-16 PDCCH monitoring capability?

- CORESET level

  - Qualcomm: Under Option 1, the configuration of PDCCH monitoring occasions for monitoring Rel. 15 and Rel. 16 DCI formats are separated by using different CORESETs or DCI sizes

- Search space set level

- DCI format level

  - Qualcomm: Under Option 1, the configuration of PDCCH monitoring occasions for monitoring Rel. 15 and Rel. 16 DCI formats are separated by using different CORESETs or DCI sizes

    - Question to be clarified: In this case, same CORESET or different CORESETs for Rel-15 PDCCH monitoring and Rel-16 PDCCH monitoring?

  - Pro for doing separation:

    - Avoid sharing of PDCCH monitoring between eMBB and URLLC, thus reduce the processing time needed for URLLC

  - Cons for doing separation:

    - No any sharing of PDCCH monitoring between eMBB and URLLC, which may result in a waste of processing power at the UE especially if the eMBB and URLLC uses the same DCI format, and/or the CORESET for eMBB and URLLC are overlapped

| Company | View |
| --- | --- |

Note that [Intel, R1-1910660] has some analysis on the above three aspects, companies are encouraged to check and provide your views.

[Nokia, R1-1910798] mentioned that the impacts on CA needs to be clarified also. In addition, [Huawei, R1-1910066] [Intel, R1-1908645] mentioned regardless of option 1 or option 2, how to define the limit considering CA with same or different numerology needs to be discussed under the enhanced PDCCH monitoring capability.

Question 2: How does UE report its capability with CA? e.g., does UE additionally report the number of CCs for Rel-15 monitoring capability only and report the number of CCs for Rel-16 monitoring capability only?

| Company | View |
| --- | --- |

Question 3: Do we allow the number of CCs configured with Rel-16 monitoring capability to be larger than the reported UE capability? If yes, how to scale the monitoring capability if the number of CCs configured is larger than the reported capability

| Company | View |
| --- | --- |

Question 4: For Rel-16 monitoring capability, is overbooking still allowed? Do we allow the number of CCs configured with Rel-16 monitoring capability to be larger than the reported UE capability?

- Option 1: The CCE/BD overbooking is only performed on the PCell and for the PDCCH configurations that are based on the Rel. 15 UE capability. For the PDCCH configurations that are based on the enhanced PDCCH monitoring capability, overbooking is not supported.

  - Support: Qualcomm

| Company | View |
| --- | --- |

Question 5: Does option 1 depend on any other factors, e.g. UE processing cap#1 vs cap#2?

| Company | View |
| --- | --- |

For option 2,

Question 1: How does UE report its capability with CA? e.g., does UE additionally report the number of CCs for Rel-15 monitoring capability only, and report the number of CCs for Rel-16 monitoring capability only?

- Option 1: UE reports its capability on the number of CCs to be supported for Rel-15 monitoring capability and for Rel-16 monitoring capability separately

  - Support:

| Company | View |
| --- | --- |

Question 2: How to scale the monitoring capability if the number of CCs configured is larger than the reported capability?

| Company | View |
| --- | --- |

-------------------------------------------------------------------------------------------------------------------------

Proposal 3.1-3: The value of C for combination (7, 3) for 15 kHz and 30 kHz is 56.

- FFS C for combination (2, 2) and (4, 3)

Other PDCCH enhancements

Some companies provide some other thinking on PDCCH enhancements:

- CATT (R1-1910341) proposed configured scheduling assignments in conjunction with some DCI indication to solve the problem brought by PDCCH blocking.

| Company | View |
| --- | --- |

- Reference

- R1-1910027	Discussion on PDCCH enhancements for URLLC	Spreadtrum Communications

- R1-1910066	PDCCH enhancements for URLLC	Huawei, HiSilicon, China Unicom

- R1-1910100	On PDCCH enhancements for NR URLLC	ZTE

- R1-1910221	PDCCH enhancements for URLLC	vivo

- R1-1910341	PDCCH enhancements for URLLC	CATT

- R1-1910483	DL Control for URLLC	Samsung

- R1-1910545	PDCCH Enhancements for NR URLLC	Ericsson

- R1-1910619	PDCCH enhancements for URLLC	OPPO

- R1-1910660	On PDCCH enhancements for URLLC	Intel Corporation

- R1-1910768	DCI Format for URLLC	Sony

- R1-1910798	On PDCCH enhancements for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1910826	PDCCH enhancements for NR URLLC	LG Electronics

- R1-1910849	PDCCH enhancements for NR URLLC	Asia Pacific Telecom co. Ltd

- R1-1910933	Discussion on PDCCH enhancements for NR eURLLC	Sharp

- R1-1910989	PDCCH enhancements for NR URLLC	Panasonic Corporation

- R1-1911078	PDCCH enhancements for eURLLC	MediaTek Inc.

- R1-1911118	PDCCH Enhancements for eURLLC	Qualcomm Incorporated

- R1-1911175	PDCCH enhancements for URLLC	NTT DOCOMO, INC.

- R1-1911285	PDCCH enhancements for URLLC	China Unicom

- R1-1911299	PDCCH enhancements for URLLC	Motorola Mobility, Lenovo

- R1-1911309	PDCCH enhancements for URLLC	Sequans Communications

- R1-1911315	On DCI enhancement for NR URLLC	WILUS Inc.

- Appendix A: Agreements in the past meetings

NR RAN1 AH Meeting 1901

Observation: For carrier frequency 700MHz with antenna configuration of 2 Tx/2 Rx, channel model of TDL-C 300 ns, 20 MHz and a CORESET with 2 symbols, five sources show that Rel-15 NR PDCCH (e.g. DCI payload size 40 bits and AL=16) can meet the reliability of 99.9999% at the 5%-tile SINR geometry, and two sources show that Rel-15 NR PDCCH (e.g. DCI payload size 40 bits and AL=16) cannot meet the reliability of 99.9999% at the 5%-tile SINR geometry. Observation: For carrier frequency 4 GHz with antenna configuration of 4 Tx/4 Rx, channel model of TDL-C 300 ns and a CORESET with 1 or 2 symbols, 12 sources show that Rel-15 NR PDCCH (e.g. DCI payload size 40 bits and AL=16) can meet the reliability of 99.9999% at the 5%-tile SINR geometry. Observation: Eight sources show that compact DCI targeting a reduction of 16 bits compared to (e.g. 40 bits) Rel-15 DCI can provide 0.6dB ~ 1 dB gain for AL=16 assuming 4 GHz, 1e-5 or 1e-6 target BLER, 4 Rx at UE side, TDL-C and a CORESET with 1 or 2 symbols in time domain and 40 MHz in frequency domain. Observation: Three sources show that compact DCI targeting a reduction of 16 bits compared to (e.g. 40 bits) Rel-15 DCI can provide 0.7dB ~ 1 dB gain for AL=16 assuming 700 MHz, 1e-6 target BLER, 2 Rx at UE side, TDL-C and a CORESET with 2 symbols in time domain and 20 MHz in frequency domain. Observation: Two sources show that compact DCI targeting a reduction of 16 bits compared to (e.g. 40 bits) Rel-15 DCI can save 14 % ~ 20% PDCCH resource used for URLLC UEs assuming 700 MHz, 1e-5 or 1e-6 target BLER for single PDCCH transmission, 2 Rx at UE side, TDL-C and a CORESET with 2 symbols in time domain, 20 MHz in frequency domain. Observation: - Three sources show that compact DCI targeting a reduction of 16 bits compared to (e.g. 40 bits) Rel-15 DCI can save 14 % ~ 16% PDCCH resource used for URLLC UEs assuming 4 GHz, 1e-5 or 1e-6 target BLER for single PDCCH transmission, 4Tx/4Rx at gNB side and 4 Rx at UE side, TDL-C and a CORESET with 1 or 2 symbols in time domain, 40 MHz in frequency domain. - One source shows that compact DCI targeting a reduction of 16 bits compared to (e.g. 40 bits) Rel-15 DCI can save 7 % ~ 11% PDCCH resource used for URLLC UEs assuming 4 GHz, 1e-5 target BLER for single PDCCH transmission, 16 Tx/16 Rx at gNB side and 2 Tx/4 Rx at UE side for SINR CDF geometry, 2 Tx/4 Rx for PDCCH BLER, TDL-C and a CORESET with 1 or 2 symbols in time domain, 40 MHz in frequency domain. Agreements: For the DCI format scheduling Rel-16 NR URLLC, - Support potential reduction of the number of bits for at least one of the following fields compared to Rel-15 DCI - Frequency domain resource assignment - Time domain resource assignment - Modulation and coding scheme - HARQ process number - Redundancy version - PUCCH resource indicator - PDSCH-to-HARQ_feedback timing indicator - Downlink assignment index - Note: Reduction of other fields are not precluded - Down-select one of the following options for the DCI format size – targeting down-selection in RAN1#96 (not to be captured in the TR for now) - Option 1: Fixed DCI size targeting a reduction of 10~16 bits reduction compared to the DCI format size of Rel-15 fallback DCI - Option 2: aligned with Rel-15 fallback DCI - Option 3: configurable DCI size with the limitation as below - Minimum DCI size should target 10~16 bits reduction compared to the DCI format size of Rel-15 fallback DCI - Maximum size should be equal to the DCI format size of Rel-15 fallback DCI - Option 4: DCI with configurable sizes for some fields, while - The maximum DCI size can be larger than Rel-15 fallback DCI - The minimum DCI size target a reduction of 10~16 bits less than the DCI format size of Rel-15 fallback DCI - Provide the possibility to align with the size of the Rel-15 fallback DCI (including possible zero padding if any) - Option 5: no introduction of new DCI format due to this SI Note: The DCI format may be impacted by other objectives in this study item and/or the following work item, e.g. PDCCH repetition mechanism and/or UCI enhancement, or may be impacted by objectives in other study item and/or work item, e.g. multi-TRP transmission from Rel-16 work item Conclusion on PDCCH repetition - PDCCH repetition is not considered further in this study item

RAN1 #96 meeting

Agreements: Support increased PDCCH monitoring capability on at least the maximum number of non-overlapped CCEs per slot for channel estimation for Rel-16 NR URLLC for at least one SCS subject to the following restrictions: - Explicit limitation on the maximum number of BDs/non-overlapping CCEs per monitoring occasion and/or per monitoring span, and - The set of applicable SCS(s) to be finalized during the WI phase - Additional restrictions (e.g., impact # of CCs if any, potential limitations on PDSCH/PUSCH processing, impact of wideband RS for CCE counting if any, etc.) can be considered during the WI phase Agreements: - Enhancements for PDCCH monitoring capability on the maximum number of monitored PDCCH candidates per slot (with potential restrictions) for Rel-16 NR URLLC can be further considered in work item phase. Agreements: For the DCI format(s) (may or may not be new format, to be finalized in the WI phase) scheduling Rel-16 NR URLLC, - Support configurable sizes for some fields, while - The maximum DCI size can be larger than Rel-15 fallback DCI - The minimum DCI size target a reduction of 10~16 bits less than the DCI format size of Rel-15 fallback DCI - Provide the possibility to align with the size of the Rel-15 fallback DCI (including possible zero padding if any) - Support at least one of the following configurable fields – the set of configurable field(s) including bitwidths to be finalized during the WI phase (which may further depend on DL vs. UL assignments) - Antenna port(s) [0~2 bits] - Transmission configuration indication [0~3 bits] - Rate matching indicator [0~2 bits] - SRS request [0~3 bits] - PRB bundling size indicator [0~1 bit] - Carrier indicator [0~3 bits] - CSI request [0~3 bit] - ZP CSI-RS triggering [0~2 bits] - Beta offset indicator [0~2 bits] - SRS resource indicator [0~4 bits] - Repetition factor [0~2 bits] - Priority indication [0~3 bits] - Note: Other field(s) can be considered if needed - Note: This doesn’t imply the necessity to increase the DCI size budget (i.e. “3 +1”) compared to Rel-15

RAN1 #96bis meeting

Agreements: Support configurable number of bits for the following fields for DL DCI format scheduling Rel-16 URLLC. - Carrier indicator (0 bit or at least one non-zero bit) - PRB bundling size indicator (0 or 1 bit) - Rate matching indicator (0, 1 or 2 bits) - ZP CSI-RS trigger (0, 1 or 2 bits) Agreements: The following fields from Rel-15 DCI format 1_1 are not included (in case new DCI format) or can be configured to be absent (0 bit) as in Rel-15 (in case reusing the existing format) in the DL DCI format scheduling Rel-16 URLLC. - Modulation and coding scheme for TB 2 - New data indicator for TB 2 - Redundancy version for TB 2 - CBG transmission information - CBG flushing information Agreements: Keep the following two fields without any change from Rel-15 DCI in DCI format scheduling Rel-16 URLLC: - Identifier for DCI formats (1 bit) (when applicable) - New data indicator (1 bit) Agreements: The following field from Rel-15 DCI format 0_1 are not included (in case new DCI format) or can be configured to be absent (0 bit) as in Rel-15 (in case reusing the existing format) in the UL DCI format scheduling Rel-16 URLLC: - CBG transmission information

RAN1 #97 meeting

Agreement: - Support configurable TDRA table as in Rel-15 DCI format 1_1 (i.e. 0, 1, 2, 3 or 4 bits for time domain resource assignment) for the DL DCI format scheduling Rel-16 URLLC Agreements: Support at least resource allocation type 1 for frequency domain resource assignment for the DCI format scheduling Rel-16 DL URLLC with one of the following modifications compared to Rel-15: - Option 1: a single configurable scheduling granularity applicable for both the starting point and length indication - Alt.1: The scheduling granularity reuses the RBG sizes for RA 0 and can be configured between configuration 1 and 2 as in Rel-15 - Alt. 2: A new RRC parameter to configure the scheduling granularity Option 2: Separate configurable starting point granularity and length indication granularity Agreements: Take the following framework as the working assumption for defining the limit on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span: - PDCCH monitoring span follows the definition in UE feature 3-5b as a starting point - FFS whether any modification needed Agreements: - The per-CC limit on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span for a certain combination (X, Y, ) is C - FFS aspects related to UE capability - FFS the limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span is same or different across different spans within a slot - Example of combinations as shown in the following table: - FFS the value of C - Companies are encouraged to report the potential aspects that have impact on the value of C X; Y; C. =0; =1; =2; =3. Combination 1. Combination 2. …. Note: The table here doesn’t mean increased PDCCH monitoring capability is supported for all SCS. N/A can be filled in the corresponding cell for the SCS not applicable. - FFS interaction with Rel-15-based limitation, e.g., whether to increase the limit for PDCCH monitoring case 1 under the increased PDCCH monitoring capability on the maximum number of non-overlapped CCEs per slot for channel estimation

RAN1 #98 meeting

Agreements: - Introduce one new DCI format for DL scheduling and one new DCI format for UL scheduling with configurable sizes for some fields in Rel-16. Agreements: Support separate configurable number of bits (2 or 3 or 4 bits) for “HARQ process number” for new DCI formats for scheduling DL and UL - FFS 0 or 1 bits Note: The following agreement was approved by email discussion post RAN1#98 meeting Agreements: - For resource allocation type 1 for frequency domain resource assignment for the DCI format scheduling Rel-16 URLLC, support the following modification compared to Rel-15: - A single configurable scheduling granularity applicable for both the starting point and length indication. - A new RRC parameter to configure the scheduling granularity Agreements: Support (2, 2) (4, 3) (7, 3) defined in UE feature 3-5b as the combination (X, Y) for Rel-16 PDCCH monitoring capability on the per-CC limit on the maximum number of non-overlapping CCEs for URLLC. - Combination (2, 1) (4, 1) (4, 2) (7, 1) (7, 2) are not additionally introduced - FFS (3, 3) or (3,2) - UE reports the supported combinations per SCS - (2, 2)(4, 3)(7, 3) applicable for 15 kHz and 30 kHz - FFS for 60 kHz and 120 kHz Agreements: For a Rel-16 UE supporting enhanced PDCCH monitoring capability, down-select between option 1 and option 2: - Option 1: PDCCH monitoring based on Rel-15 capability for eMBB and PDCCH monitoring based on Rel-16 capability for URLLC can be configured to a UE on the same carrier - UE monitors PDCCH for eMBB following reported Rel-15 capability, and monitors PDCCH for URLLC following reported Rel-16 capability - For Rel-16 PDCCH monitoring capability, the limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span is the same across different spans within a slot. Each span for Rel-16 PDCCH only cover USS for URLLC (FFS for CSS) - Option 2: PDCCH monitoring for both eMBB and URLLC can be configured based on either Rel-15 capability or Rel-16 capability - gNB configures which capability is used - For Rel-16 PDCCH monitoring capability, - The limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span is the same across different spans within a slot, each span can cover CSS and/or USS - Note: the value C is to be separately discussed Agreements: If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y) are valid for the span pattern, the maximum value of C of the valid combinations is applied. - A combination C(X, Y) is valid if the span pattern satisfies X and Y of the given combination in every slot, including cross slot boundary - FFS the impact from empty span(s) on the span pattern