3GPP TSG RAN WG1 Meeting #98bis	R1-1911715

Chongqing, China, October 14-20, 2019

Agenda Item:	7.2.4.7

Source:	Futurewei

Title:		Feature lead summary #3 of support of NR Uu controlling LTE sidelink

Document for:	Discussion and decision

# Introduction

A new WID on NR-V2X was approved at RAN#83, with one of the objectives as follows:

2. Specify support for NR Uu to provide control for LTE sidelink

- Sidelink mode 4 as per the study outcome [RAN2, RAN1]; and

- Sidelink mode 3-like RRC-configured SPS scheduling with either RRC-based activation/deactivation as per the study outcome or DCI-based activation/deactivation [RAN1, RAN2].

  - RAN1 to make a decision on which option is supported until RAN#84.

- 15 companies submitted contributions for this AI ([1]-[15]). This summary aims to collect views and suggest proposals based on the submitted contributions.

# Decisions at previous meetings

## RAN1#96

Agreements:

- Scheduling by gNB using RRC for LTE sidelink scheduled mode is supported from RAN1 perspective under the premise that there is sufficient time for coordination between the NR and LTE modules. No DCI to activate/release

  - RRC message delivers the SPS grant configuration and releases the SPS configuration.

  - Support of this scheduling mode is subject to UE capability (may or may not have capability for both LTE & NR)

  - Note: some specification LTE change is needed to support the reception of a grant through RRC

    - RRC message contains mode 3 grant content and timing

    - Up to the Editor to capture it as mode 3 or new LTE sidelink mode

  - No intention to have additional NR & LTE specification change (other than those described above) for this function in Rel-16

- RAN1 studied the feasibility of SPS scheduling by gNB for LTE sidelink with DCI activation/release, but there is no consensus to support it

## RAN1#96bis

Agreements:

Regarding RRC-based versus DCI-based activation/release of LTE sidelink SPS, RAN1 agrees to make the choice on the basis of at least:

- Spec impact

- Flexibility

- Performance, including latency

- Implementation complexity

- Timing of the activation/deactivation

## RAN2#105bis

Agreements:

For scheduling LTE SL UEs, the gNB uses RRC messages to deliver the SPS grant configuration.

Separate system information block should be designed to support LTE resource pool configuration via NR Uu. It will be defined as a container (OCTET STRING) and actual information follows what defined in LTE RRC.

gNB should be able to configure the LTE V2X mode 4 sidelink resource pool via dedicated signalling. In addition, gNB should be able to configure mode3 SL resources via dedicated signaling. It will be defined as a container (OCTET STRING) and actual information follows what defined in LTE RRC.

## RAN1#97

Agreements:

- DCI-based activation/deactivation is supported

  - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability

  - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling

  - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V

    - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V

  - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI

    - Z is the same timing offset in current LTE V2X specs

    - X>0. FFS value(s) of X, and if one or multiple values of X are possible

## RAN1#98

Agreements:

- A new RNTI is introduced to scramble the NR DCI used for scheduling LTE PC5.

- X is dynamically indicated using a field in the DCI

  - FFS whether the DCI field provides an index to a table or the value of X

  - The minimum value of X is subject to UE capability

    - UE reports a single value subject to UE capability

# Mode-3 operation

Support for mode-3 was agreed at RAN1#96, with additional details at RAN1#97, and in particular, activation/deactivation by DCI. Most companies discuss NR DCI timing and NR DCI design.

## DCI timing

An important aspect that was not concluded at the last meeting was whether the time at which the DCI applies (X) was signaled in the DCI as a value or as an index to table:

- X is dynamically indicated using a field in the DCI

  - FFS whether the DCI field provides an index to a table or the value of X

14 companies discuss this aspect. Two options are discussed:

- Option 1: The NR DCI field provides an index to a table. 11 companies consider this option ([1]-[4], [6]- [14])

- Option 2: The value of X is directly signalled in the NR DCI. 2 companies consider this option ([5], [15])

Option 1 has a large majority. It is thus proposed to go with this option. In order to accommodate concerns of companies supporting option 2, however, it is further proposed that the table offers sufficient flexibility, and a relatively large number of X values (up to 8).

Some companies discuss what the value of X should be. For instance, [5] states that “Unless there is a clear need and well justified reason of why the X value in the NR DCI can be larger than the reported value, it is recommended that the X value signaled in the DCI should be the same as the UE reported value in UE capability signaling.” [8], [9] and [13] clarify that the value of X signaled in the NR DCI should not be lower than what is reported in the UE capability.

Proposal for offline discussion:

- The NR DCI field to indicate X provides an index to a table of values

  - A UE does not expect to be signaled a value of X smaller than the reported capability

  - The NR DCI field is encoded with [3] bits

Proposal after of first offline discussion:

- The NR DCI field to indicate X provides an index to a table of values

  - The table of values is configurable, and has at most 8 values

  - Alt 1 The size of the DCI field is derived from the size of the table

    - A “TDRA-like” field can be used

  - Alt 2 The size of the DCI field is fixed

Agreements after of first online discussion:

Agreements:

- The NR DCI field to indicate X provides an index to a table of values

  - The table of values is configurable, and has 8 values

  - The size of the DCI field is fixed at 3 bits

The configuration of the table is discussed in [8], [9], [10], [13], [14]. [8] suggests configuration by the gNB, [7] and [14] suggests configuration by RRC signaling. In addition, [7] suggests supporting both common and dedicated signaling. [10] and [13] provide tables, and [14] provides bounds for X (no larger than the shortest expected periodicity for the traffic).

Proposal for offline discussion:

- The table to indicate X is provided by the gNB by higher-layer signalling

  - The X values are selected between 0 and 10ms

It was clear in the first online discussion that the table of values would be passed to the RAN1 specs in the form a higher layer parameter, and that no additional agreement was needed to include the parameter in the RRC list and the capability in the capability list. However, there was no discussion on companies’ preferences of values that could be configured in the table, which could be provided as a RAN1 input to RAN2.

Proposal for email discussion or conclusion:

- Companies are encouraged to provide and justify values of X that that can be configured in the table

Note that another proposal on timing and on the definition of Z is given in [9]. This agreement does not seem necessary, as the difference in timing could be accommodated by the choice of X.

In [1], it is proposed to count the start symbol of DCI in NR DCI decoding time.

## DCI design

The general guidelines of the NR DCI for activation/deactivation of a LTE sidelink resource allocation were agreed at RAN1#97. In particular:

- NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling

- The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V

At RAN1#98, the following was agreed:

- A new RNTI is introduced to scramble the NR DCI used for scheduling LTE PC5.

From the contributions, there seems to be general agreement that a new NR DCI format containing the fields of LTE DCI 5A should be defined. In addition, it was observed that LTE DCI 5A is small by NR standards (e.g., [8], [10); thus the NR DCI used for scheduling LTE PC5 should be padded to match the size of one of the mode-1 NR DCIs.

Proposal for online discussion:

- The NR DCI for scheduling LTE PC5 is padded to match the mode-1 NR DCI size

With these proposals, the NR DCI for scheduling LTE PC5 is as indicated on Table 1.

| Field | Size |
| --- | --- |
| Carrier indicator | 3 bits |
| Lowest index of the subchannel allocation to the initial transmission | log2(NsubchannelSL)< 5 bits (for up to 20 sub-channels) |
| SCI format 1 field - Frequency resource location of initial transmission and retransmission | log2(NsubchannelSL(NsubchannelSL+1)/2)< 8 bits (for up to 20 sub-channels) |
| SCI format 1 field - Time gap beween initial transmission and retransmission | 4 bits |
| SL index (present only for TDD configuration 0-6) | 2 bits |
| SL SPS configuration index | 3 bits |
| Activation/release indication | 1 bit |
| X index | 3 bits |
| Padding to match the mode 1 DCI size | TBD |

Table 1. Content of the NR DCI for scheduling LTE PC5

Note that other proposals are also discussed in the contributions. [2] proposes to have the UE send a confirmation message after receiving a DCI for activation/deactivation. [10] proposes to send an LS to RAN2 to make sure that an information element with the content of the semiPersistSchedIntervalSL LTE is available.

## Others

One company [2] discusses which synchronization applies for the grant and proposes that in the case of NR Uu controlling LTE sidelink, the gNB should be considered as the synchronization source when determining the timing of SPS configuration. It is, however, not clear that an agreement is needed, because the UE receiving an NR DCI would automatically assume that the DCI content applies with the NR timing

Three companies ([2], [3], [6]) discuss how the TDD configuration and bandwidth are communicated to the LTE UE. These configuration parameters can be discussed by RAN2.

Two companies ([4], [16]) discuss NR UE capability signaling for the support of LTE PC5 scheduling by NR Uu. Several companies have proposals related to UE capability ([5], [11], [14], among others). The FL suggestion is to address the capability definition in a broader NR V2X capability discussion.

One company ([9]) discusses power control on the LTE sidelink. It is unclear why the existing LTE power control procedures would not automatically apply.

Two companies ([9], [14]) discuss scheduling assistance information (e.g., UE location reporting, BSR signaling). While this is not absolutely necessary to complete the functionality, such aspects could be considered if there is sufficient interest in the group.

One company [6] indicates that the LTE bandwidth and TDD configuration needs to be known and that RAN2 impact is needed. In addition, [6] indicates that it would be beneficial to have RAN3 considering LTE-NR sidelink coordination.  These two issues can be discussed by RAN2 and RAN3, respectively, without any input from RAN1 at this stage.

No proposal is made here for these aspects. Companies are encouraged to further consider them for contributions at RAN1#99.

# Mode 4

Within the context of this AI, mode-4 configuration is also discussed by 6 companies ([4], [7], [9], [10], [14]). 4 companies indicate that no work is needed in RAN1 (([4], [7], [9], [10]). One company [14] indicates that NR Uu control of LTE mode-4 operation should be a UE capability. The FL suggestion is to let RAN2 work on mode-4 configuration, and to discuss support of this UE capability separately in the overall UE capability discussion for NR V2X.

Proposal for offline discussion:

- No further RAN1 work on configuration for LTE sidelink mode 4 is needed.

- Inform RAN2 of this conclusion

# References

| No. | Tdoc | Type | Source | Title |
| --- | --- | --- | --- | --- |
| 1 | R1-1910061 | decision | Huawei, HiSilicon | NR Uu control for LTE sidelink |
| 2 | R1-1910219 | discussion | vivo | Support of NR Uu controlling LTE sidelink |
| 3 | R1-1910282 | discussion | ZTE, sanechips | Discussion on NR Uu controlling LTE sidelink |
| 4 | R1-1910334 | discussion | CATT | Disucssion on NR Uu controlling LTE sidelink |
| 5 | R1-1910383 | discussion | OPPO | On DCI signalling to control LTE sidelink and UE capability reporting |
| 6 | R1-1910477 | discussion | Samsung | On NR Uu controlling LTE sidelink |
| 7 | R1-1910519 | decision | Nokia, Nokia Shanghai Bell | Discussion of NR Uu controlling LTE sidelink |
| 8 | R1-1910540 | decision | Ericsson | NR Uu controlling LTE sidelink transmissions |
| 9 | R1-1910655 | discussion | Intel | NR Uu control of LTE sidelink communication |
| 10 | R1-1910701 | decision | Futurewei | Support of NR Uu controlling LTE sidelink |
| 11 | R1-1910785 | discussion | LGE | Discussion on NR Uu controlling LTE sidelink |
| 12 | R1-1910965 | discussion | Apple | On NR Uu Controlling LTE Sidelink |
| 13 | R1-1911071 | discussion | MediaTek | Support of NR-Uu controlling LTE sidelink |
| 14 | R1-1911112 | discussion | Qualcomm | NR Uu control of LTE sidelink |
| 15 | R1-1911282 | discussion | InterDigital | On NR Uu Controlling LTE Sidelink |