3GPP TSG RAN WG1 Meeting #99	R1-1911863

Reno, Nevada, USA, November 18 – 22, 2019

Agenda Item:	7.2.2.1.1

Source:	Huawei, HiSilicon

Title:	Initial access signal and channels in NR unlicensed band

Document for:	Discussion and decision

# Introduction

In this contribution, we discuss NR-U DRS design including SSB pattern and the corresponding CORESET 0, RMSI PDSCH rate matching, PBCH content and transmission of RMSI in SCell. Furthermore, extensive PRACH preamble evaluations are provided for ZC sequences with/without repetitions.

# NR–Unlicensed DRS design

## SSB pattern and CORESET 0

Two alternatives on the combination of SSB pattern and CORESET 0 in half slot duration were discussed in RAN1#96bis.

- Alt 1: Legacy SSB positions in a slot

  - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 at least for the first SSB in a slot

  - Support Type0-PDCCH in symbol (#6, #7) for length-2 coreset 0 and symbol #6 or symbol #7 for length-1 coreset 0 for the second SSB in a slot

    - FFS: configurable between symbols #6 and #7 for the length-1 coreset 0 for the second SSB in a slot

- Alt2: New SSB positions in a slot

  - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset or the first SSB in a slot

  - Support Type0-PDCCH in symbol (#7, #8) for length-2 coreset 0 and symbol (#7) for length-1 coreset 0 for the second SSB in a slot

A conclusion as following language was achieved in RAN1#97.

There is no consensus between Alternative 1 and Alternative 2 on the topic of SSB positions and type-0 PDCCH monitoring in a slot. No further online or scheduled offline discussions will be conducted on this topic. If there is consensus on such enhancements, the topic can be revisited in the Rel-16 NR-U work item.

For alternative 1 (legacy SSB pattern) with length-2 CORESET 0, it was argued that 2 remaining symbols at the end of slot could be used for PUCCH transmission or be allocated as RACH Occasions. However, switching transmission direction in DRS transmission is undesirable since it increases the probability of losing channel occupancy. Configuring CSI-RS at the last 2 symbols separated apart from the QCLed SSB is also not preferred as it requires additional beam switching operation. Furthermore, CORESET 0 associating with the 2nd SSB in a slot starts at OS#6 will impede UE to perform half-slot processing. On the other hand, if length-1 CORESET 0 associating with 2nd SSB in a slot is at OS#7, then the relative position among SSB, CORESET 0 and RMSI PDSCH will be different from the 1st SSB within the same slot, which also increases UE’s decoding complexity.

For alternative 2 (shifting 2nd SSB in a slot by one symbol), the gNB could also configure 1 symbol CSI-RS occupying the full initial BWP at OS#6 and OS#13, which is assumed to have the same QCL assumption with the corresponding SSB (CSI-RS transmitted at OS#6 corresponding to SSB 0 and CSI-RS transmitted at OS#13 corresponding to SSB 1) and hence no beam switching back and forth is needed. Besides, alternative 2 also supports length-2 CORESET 0 for each SSB even if 2 SSBs are transmitted in one slot. Furthermore, the symmetric structure between two half-slots could facilitate UE implementation. In TR38.214 [1], Table 5.1.2.1.1-2: Default PDSCH time domain resource allocation A for normal CP already included entries to schedule Type A PDSCH of 4 symbols in 1st half-slot (corresponding to row index 14). Row index 6 could be used to schedule Type B PDSCH of 4 symbols (S=9, L=4).  Considering all discussion above, alternative 2 is preferred to be adopted as SSB pattern for NR-U.

If finally in Rel-16 there were no consensus on the enhancement, the existing NR Rel-15 SSB pattern (Case C 30 kHz SCS) and CORESET0 configuration would be used. It only allows length-1 CORESET 0 when gNB expects to transmit 2 SSB together with their associated CORESET0s in a slot. The capacity and coverage of CORESET0 is limited when RMSI PDCCH requires higher aggregation level or PDCCH scheduling OSI or paging expects to share CORESET0 with RMSI. On the other side, the RE on symbol 6, 7, 12 and 13 cannot be used if there is no other channel multiplexed with DRS. Neither reservation signal nor gap is preferred from the channel access perspective. The minimum shift granularity of DRS would also be restricted to one slot (0.5ms for 30 kHz SCS and 1ms for 15 kHz SCS) which results in less channel access opportunities in a 5ms DRS transmission window.

Figure 1.  Illustration of candidate combination of NR-U SSB pattern and COREST 0

Observation 1: SSB pattern and CORESET0 configuration in NR Rel-15 is not efficient when gNB expect to transmit 2 SSB and their associated CORESET0 in the same slot.

Proposal 1: Alternative 2 shall be adopted in NR-U as SSB position and Type-0 PDCCH monitoring in a slot.

- Support Type0-PDCCH in symbol (#0, #1) for length-2 CORESET 0 and symbol (#0) for length-1 CORESET 0 for the first SSB in a slot.

- Support Type0-PDCCH in symbol (#7, #8) for length-2 CORESET and symbol (#7) for length-1 CORESET 0 for the second SSB in a slot.

## RMSI PDSCH rate matching

In RAN4#92, RAN4 agreed to place SSB close to the edge of sub-bands. One of the motivations from RAN1 perspective is that it allows more RE available for RMSI PDSCH.  In NR Rel-15, PDSCH scheduled by Type-0 PDCCH should be contiguous in frequency domain and cannot rate-match around SSBs. It is efficient only when RMSI PDSCH of 4 symbols is multiplexed with SSB in FDM manner. Large number of RE will be wasted when RMSI PDSCH has more than 4 symbols and rate-matching around SSB is not supported. In Figure 2, we illustrated two multiplexing schemes between RMSI PDSCH and one SSB in a slot. Scheme 1 is the existing Rel-15 scheme where RMSI PDSCH rate matching is not supported. Scheme 2 allows gNB mapping RMSI PDSCH on the same PRB as SSB after symbol 6.  The maximum number of RE available for RMSI PDSCH transmission were calculated in Table 1.  It is observed that gNB could schedule at most 50% more RE if allowing RMSI PDSCH rate-matching around SSB. In SCell, gNB may transmit SSB not on the sync raster which can avoid UE to perform initial access on the cell. Meanwhile, UE can be configured to monitor RMSI associating with off-sync SSB on the SCell for ANR purpose. The capacity of RMSI would still be restricted if rate matching were not supported.

Observation 2: More RE is available for RMSI PDSCH transmission if rate matching around SSB is allowed

Figure 2 multiplexing between RMSI PDSCH and SSB

Table 1. Available REs for PDSCH within a DRS block with 30 kHz SCS.

| NR-U SSB pattern | L=4 | L=12 | L=13 |
| --- | --- | --- | --- |
| scheme 1 | 1344 | 4032 | 4368 |
| scheme 2 | 1344 (0%) | 5952 (47.6%) | 6528 (49.5%) |

* The length is not supported in the table of “Default PDSCH time domain resource allocation A for normal CP”. New entries should be introduced in Table 5.1.2.1.1-2 if supported.

Considering the fixed location of SSB in PCell and assuming RMSI PDSCH is always transmitted in the same slot as the associated SSB, as shown in Figure 1, UE is able to identify rate-matched RE in RMSI PDSCH without explicit signaling.

Observation 3: No explicit signaling in DCI is required for UE to identify RMSI PDSCH RE rate-matched by associated SSB if RMSI is always transmitted together with associated SSB in the same slot.

When DMRS of RMSI PDSCH is in the same symbol as SSB, there is no PDSCH DMRS available for UE to acquire channel estimation on the PRB occupied by SSB if RMSI PDSCH rate-matching were supported. For example in Figure 3, an RMSI PDSCH is scheduled in time domain with row index 1 (Type A, S=2, L=12) with dmrs-TypeA-Position indicating 2 in Table 5.1.2.1.1-2 of TS38.214 [1]. gNB cannot transmit PDSCH DMRS on the PRB occupied by SSB in symbol 2. In order to recover channel information of all PRB allocated to RMSI PDSCH, UE could use DMRS in PDSCH and PBCH jointly, considering the QCL assumption between RMSI PDSCH and associated SSB.

Figure 3.  PDSCH DMRS position

Observation 4: DMRS in both RMSI PDSCH and PBCH could be used to decode RMSI PDSCH when RMSI PDSCH rate matching around SSB is allowed.

Proposal 2: NR-U shall support that RMSI PDSCH scheduled by Type-0 PDCCH is rate-matched around the corresponding SSBs.

## PBCH content

In RAN1#97, RAN1 agreed not to increase the PBCH payload size from NR Rel-15, i.e. , where  is size of MIB and 8 bit is in PBCH payload. Meanwhile, 10-bits SFN and half-frame indicator are indicated as in Rel-15. In RAN1#98 meeting, 3 bits PBCH payload not in MIB were further determined. In summary, the following bit in PBCH payload was specified in NR-U.

- aA ,  aA+1 ,  aA+2 ,  aA+3 : 4th , 3rd , 2nd , and 1st LSB of SFN

- aA+4 : half-frame bit as in Rel-15

- aA+6 : 5th bit of SSB candidate position for 30 kHz SCS

- aA+7aA+6 : 4th bit of SSB candidate position for 15 kHz SCS and 30kHz SCS

In TS38.331, MIB in NR Rel-15 consists of following fields:

MIB ::= 							SEQUENCE {

systemFrameNumber					BIT STRING (SIZE (6)),

subCarrierSpacingCommon				ENUMERATED {scs15or60, scs30or120},

ssb-SubcarrierOffset				INTEGER (0..15),

dmrs-TypeA-Position					ENUMERATED {pos2, pos3},

pdcch-ConfigSIB1					INTEGER (0..255),

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

spare								BIT STRING (SIZE (1))

}

The 1-bit field of subCarrierSpacingCommon indicating subcarrier spacing for SIB1, Msg.2/4 for initial access and broadcast SI-messages. In FR1, it indicates whether CORESET0 using 15 kHz or 30 kHz SCS. As agreed in RAN1#96, the SCS for all SSBs and CORESET #0 on a carrier is always the same for operation of NR in unlicensed spectrum. This field is not necessary because UE will assume same SCS as that of SSB it detects.

The 8-bit field of pdcch-ConfigSIB1 consists of 4-bit controlResourceSetZero and 4-bit searchSpaceZero. In NR Rel-15, controlResourceSetZero provides CORESET0 configuration defined from Table 13-1 to Table 13-6 in section 13 TS38.213 [2].  According to the agreement in RAN1#96, CORESET #0 frequency domain resource configuration should be 48 RBs for 30KHz SCS and 96 RBs for 15KHz SCS. It was also agreed in RAN1#96bis that only CORESET #0 lengths of 1 and 2 symbols are supported for NR-U. RAN4 agreed to introduce single default sync raster for each sub-band in RAN4#92. Thus, the entries in Table 13-1 and 13-4 can be simplified as following Table 2 and Table 3. The offset value X and Y should be derived from the sync raster (GSCN) in each of 20MHz channel bandwidth assuming CORESET0 locates in the middle of each 20MHz channel. It allows enough flexibility for gNB to configure intra-carrier guard band at both edges of an LBT bandwidth without affecting CORESET0 configuration. Considering RAN4 only defines sync raster for 30 kHz, we assume same sync raster applies to 15kHz SCS SSB if gNB hope to transmit  of 15kHz For 15KHzThe mapping between sync raster and offset X and Y is given in Table 4. Compared with NR Rel-15, at most 3 bit can be saved in the field of controlResourceSetZero.

Table 2 Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15, 15} kHz for unlicensed bands

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 96 | 1 | X |
| 1 | 1 | 96 | 2 | X |

Table 3 Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 30} kHz for unlicensed bands

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 48 | 1 | Y |
| 1 | 1 | 48 | 2 | Y |

Table 4 Mapping between sync raster and CORESET0 offset X and Y

| GSCN | X | Y | GSCN | X | Y | GSCN | X | Y |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8996 | 14 | 2 | 9232 | 13 | 2 | 9385 | 15 | 3 |
| 9010 | 15 | 3 | 9246 | 14 | 2 | 9402 | 12 | 1 |
| 9024 | 15 | 3 | 9260 | 15 | 3 | 9416 | 13 | 2 |
| 9037 | 8 | 0 | 9274 | 15 | 3 | 9430 | 13 | 2 |
| 9051 | 9 | 0 | 9287 | 8 | 0 | 9444 | 14 | 2 |
| 9065 | 10 | 0 | 9301 | 9 | 0 | 9458 | 15 | 3 |
| 9079 | 11 | 1 | 9315 | 10 | 0 | 9471 | 8 | 0 |
| 9093 | 12 | 1 | 9329 | 11 | 1 | 9485 | 9 | 0 |
| 9107 | 13 | 2 | 9343 | 12 | 1 | 9499 | 10 | 0 |
| 9121 | 14 | 2 | 9357 | 13 | 2 | 9513 | 11 | 1 |
| 9218 | 12 | 1 | 9371 | 14 | 2 |  |  |  |

Proposal 3: The table of Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set can be simplified as Table 2 and Table 3, corresponding to 1 bit controlResourceSetZero in MIB. The Offset X and Y from smallest RB index of CORESET0 to the smallest RB index of the common RB overlapping with the first RB of the corresponding SS/PBCH block on the sync raster can be derived from the GSCN according to Table 4.

searchSpaceZero provides Type0-PDCCH monitoring occasions. For SSB and CORESET0 multiplexing pattern 1, the system frame number SFNc and starting slot n0 of Type0-PDCCH monitoring occasion associated with SSB candidate position index i is calculated with parameter O and M. 
There were 16 configurations defined in Table 13-11 in TS38.213 [2] for type0-PDCCH CSS set-SSB multiplexing pattern 1 in FR1. The monitoring occasions with M/O parameters given in Table 13-11 are illustrated in Figure 4 assuming 20 SSB candidate positions with 30 kHz SCS defined in NR-U.

Figure 4 SFN and slot mapping of Type-0 PDCCH monitoring occasion

In our companion contribution [3], we proposed to allow UE monitor Type0-PDCCH CSS in the same slot as the associated SSB. It is beneficial for UE to reduce latency when receiving system information. However, it is observed that only two entries with index=1(O=0, M=1/2) and index=5 (O=5, M=1/2) can meet the requirement if DRS burst is transmitted in the 1st or 2nd half frame in a frame with even SFN. The DRS transmission periodicity should be also in the unit of 20ms. Either introducing additional offset values of O=10 and O=15 or defining PDSCH monitoring occasions explicitly with the SFN and slot index of the associated SSB can be considered. In Rel-15, UE is always configured 2 search space sets per slot corresponding to 2 SSB in a slot sequentially when M=1/2. In order to configure UE to detect RMSI PDCCH/PDSCH with full slot length and associated SSB in the same slot, it is necessary to support number of search space sets per slot to be 1 when M=1/2. The “first symbol index” in the table should also be updated depending on whether new SSB pattern and CORESET0 monitoring occasion discussed in section 2.1 will be agreed.  The new entries are summarized in Table 4 with both ways of M/O parameters and explicit SFNC and nC.    If NR-U only allow Type0-PDCCH monitoring occasion in the same slot as the associated SSB, at most 3-bit searchSpaceZero can be saved.

Table 5 PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 in unlicensed band

- New/updated entries by M/O parameters

| Index |  | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- | --- |
| New | 0 | 1 | 1/2 | 0 |
| 1 | 0 | 2 | 1/2 | {0, if is even}, {, if is odd}, or {0, if is even}, {6, if is odd} if Alt 1, or {0, if is even}, {7, if is odd} if Alt 2 |
| New | 5 | 1 | 1/2 | 0 |
| 5 | 5 | 2 | 1/2 | {0, if is even}, {, if is odd}, or {0, if is even}, {6, if is odd} if Alt 1, or {0, if is even}, {7, if is odd} if Alt 2 |
| New | 10 | 1 | 1/2 | 0 |
| New | 10 | 2 | 1/2 | {0, if is even}, {, if is odd}, or {0, if is even}, {6, if is odd} if Alt 1, or {0, if is even}, {7, if is odd} if Alt 2 |
| New | 15 | 1 | 1/2 | 0 |
| New | 15 | 2 | 1/2 | {0, if is even}, {, if is odd}, or {0, if is even}, {6, if is odd} if Alt 1, or {0, if is even}, {7, if is odd} if Alt 2 |

- New entries with by SFNC and nC.

| Index | PDCCH monitoring occasions (SFN and slot number) | Number of search space sets per slot | First symbol index |
| --- | --- | --- | --- |
| new |  | 1 | 0 |
| new |  | 2 | {0, if is even}, {, if is odd}, or {0, if is even}, {6, if is odd} if Alt 1, or {0, if is even}, {7, if is odd} if Alt 2 |

In NR Rel-15 FR1, 4-bit field ssb-SubcarrierOffset in MIB and 1-bit aA+4 in PBCH payload was used to indicate the subcarrier offset kSSB from subcarrier 0 in common resource block NCRBSSB to subcarrier 0 of the SS/PBCH block in the unit of 15 kHz SCS. In RAN4, a simplified channel plan and sync raster is under discussion. The number of kSSB bit can be reduced if sync raster are carefully selected.

Proposal 4: The entries in Table 5 should be included in the table of Parameter for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1, in order to facilitate UE to monitor Type0-PDCCH in the same slot as its associated SS/PBCH block..

It was agreed in TR38.889 [4] that RMSI, OSI and paging could optionally be transmitted in NR-U DRS. It is beneficial to reduce the required number of channel access attempts and better fulfill the OCB requirement in NR-U. On the other side, it is undesirable to always include all these channels/signals in NR-U DRS due to limited resources available for the DRS transmission. In NR Rel-15, the CORESET for Type0-PDCCH CSS is configured in the field of pdcch-ConfigSIB1 in MIB. UE could determine the existence of CORESET0 for Type0-PDCCH CSS through the value of kSSB in MIB. Considering the limited RE resource in DRS, the Type0A-PDCCH CSS and Type2-PDCCH CSS could share same configuration as Type0-PDCCH CSS if gNB expect to transmit OSI and Paging together with SSB in DRS. An indicator in MIB would be helpful for UE to determine whether to also blind detect Type0A-PDCCH CSS and Type2-PDCCH CSS in the CORESET0.

In the discussion on initial access procedure, Q (={1,2,4,8}) value is used by UE to acquire SSB QCL for both serving cell and neighbor cell. If the Q value is carried in MIB, UE is able to distinguish only RMSIs with QCL assumption for soft combining, which simplify UE implementation. The detail description can be found in our companion contribution [3].

Proposal 5: Indicator for OSI and paging and Q value should be included in MIB.

## Transmission of RMSI in Scell

In unlicensed spectrum, different operators may deploy cells in the same unlicensed carrier. Due to limit coordination among operators, cells from different operators might choose same physical layer cell ID (PCI) causing PCI collision. This issue also exists in LTE LAA and other scenario such as Femto. It could be alleviated by some SON algorithms without standard impact. For example, gNB would select a PCI not used after sensing the candidate carrier. It is not necessary to always transmit RMSI in Scell.

Observation 5: It is not necessary to always transmit RMSI in Scell.

In RAN1#98bis, work assumption based on Alt 1 was achieved as following.

Working assumption:

For RMSI transmission for ANR purpose on a carrier with an SSB not on a sync raster, the PBCH in SSB not on a sync raster does not directly provide the location of the CORESET 0 for RMSI reception.

- The frequency domain difference between an off-sync SS/PBCH block and its associated CORESET #0 is determined at least based on

- The offset between the frequency location of the off-sync SS/PBCH block configured by gNB (high layer parameter ssbFrequency) and the frequency location corresponding to the GSCN of the synchronization raster entry within the same LBT bandwidth.

- Also based on the offsets signaled in PBCH payload (including MIB).

  - FFS: How many offsets

- Note: For ANR purpose, the SSB and and the associated CORESET0 are expected to be in the same LBT bandwidth

- Note: This working assumption assumes that there is only one sync raster point defined per 20 MHz. If RAN4 decides that there is more than one sync raster point per 20 MHz, then this working assumption is not valid and will be revisited

Given the agreement in RAN4 that there is only one sync raster in a 20MHz channel bandwidth. The work assumption should be confirmed. The number of offsets signaled in PBCH payload was discussed in section 2.3

Proposal 6: Confirm the work assumption in RAN1#98bis on the derivation of location of the CORESET0 for RMSI reception on a carrier with an SSB not on a sync raster.

# NR-Unlicensed PRACH Preamble

In 3GPP RAN1#97 meeting, the following agreement was made:

Agreement

For a new enhanced design of NR-U PRACH in addition to the Rel-15 design (sequence length of 139) further discussion is limited to the following options

- ZC sequence of the following lengths

- 15 kHz: Choose one of L_RA= [571, 1151]

- 30 kHz: Choose one of L_RA= [283, 571]

- Repetition of Rel-15 PRACH sequences in frequency domain with potentially some mechanisms to improve the cubic metric

  - Consider one of 2 and 4 repetitions for 30 kHz and one of 4 and 8 repetitions for 15 kHz

- Note: Decision will be based on previously agreed evaluation metrics, capacity per cell (i.e., number of preambles per RACH occasion and number of RACH occasions) for the same time and frequency resources, specification impact and implementation complexity.

- Note: Companies should state any deviations in assumptions from the agreed evaluation assumptions.

In [5], the complete solution and specification impact for introducing ZC sequences of length 571 and 1151 were given, which are for convenience also included in Appendix B.

## Numerical evaluations

Table A-1 of Appendix A lists the evaluated PRACH schemes (e.g., from [6] denoted ‘LGE’ and from [7] denoted ‘Ericsson’), and the results are shown in Figures A-1 to A-3 of Appendix A. In particular, the length-139 ZC sequence based Rel-15 PRACH scheme is evaluated with both formats A1 and A3. Their performance with 30 kHz SCS are summarized in Table 6 below, where the 95% CM value is computed as the CM value x measured at PrCM≤x=0.95, where CM=(RCM-1.52)/1.56 and RCM is the raw CM of the PRACH preambles. The following observations are made:

Observation 6: Compared to a single long ZC sequence, a repeated length-139 ZC sequence of similar length performs worse, and the degradation increases with more repetitions. For 30 kHz SCS:

- The mis-detection probability is ~0.01-0.21 dB worse

- The 95-percentile cubic metric (CM) value is ~0.15-1.84 dB larger

- The maximum coupling loss (MCL) is ~0-1.41 dB lower

- The PRACH capacity is ~62-81% smaller

Table 6. Performance of different PRACH schemes with 30 kHz SCS

| Scheme | NZC | R | NFDM | SCS*NZC*R (MHz) | Np (dBm) | SNRFD (dB) | Pmax (dBm) | 95% CM (dB) | PTx (dBm) | MCL (dB) | PRACH capacity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ZC139 (A1) | 139 | 1 | 4 | 4.17 | -102.80 | -4.05 | 16.12 | 2.33 | 16.12 | 122.97 | 6072 (92 cells) |
| ZC139 (A3) | 139 | 1 | 4 | 4.17 | -102.80 | -8.49 | 16.12 | 2.33 | 16.12 | 127.41 | 6072 (92 cells) |
| ZC139x2 (Ericsson) | 139 | 2 | 2 | 8.34 | -99.79 | -7.54 | 19.13 | 1.80 | 19.13 | 126.46 | 3036 (46 cells) |
| ZC139x2 (LGE) | 139 | 2 | 2 | 8.34 | -99.79 | -7.63 | 19.13 | 2.52 | 19.13 | 126.55 | 3036 (46 cells) |
| ZC283* | 283 | 1 | 2 | 8.49 | -99.71 | -7.64 | 19.20 | 2.33 | 19.20 | 126.55 | 7896 (112 cells) |
|  |  |  |  |  |  |  |  | 1.84 |  |  | 6440 (92 cells) |
|  |  |  |  |  |  |  |  | 0.68 |  |  | 3220 (46 cells) |
| ZC139x4 (Ericsson) | 139 | 4 | 1 | 16.68 | -96.78 | -10.97 | 22.14 | 0.50 | 22.14 | 129.89 | 1518 (23 cells) |
| ZC139x4 (LGE) | 139 | 4 | 1 | 16.68 | -96.78 | -11.05 | 22.14 | 2.15 | 20.85 | 128.68 | 1518 (23 cells) |
| ZC571* | 571 | 1 | 1 | 17.13 | -96.66 | -11.18 | 22.25 | 2.33 | 20.67 | 128.51 | 7980 (114 cells) |
|  |  |  |  |  |  |  |  | 1.83 | 21.17 | 129.01 | 6440 (92 cells) |
|  |  |  |  |  |  |  |  | 0.35 | 22.25 | 130.09 | 1610 (23 cells) |

*For the single long ZC sequences, we provide the 95% CM values using all preambles or by choosing a subset of preambles with the lowest CM values that results in the same PRACH capacity as of the length-139 ZC sequence based schemes.

Table 6 also shows that a single long ZC sequence always provides the lowest 95% CM value and the largest MCL value under the same target PRACH capacity. The results for 15 kHz SCS are summarized in Table 7 and basically, similar trends as those for 30 kHz SCS can be observed.

Observation 7: For 15 kHz SCS, for a same PRACH capacity, a length-139 ZC sequence with 8 repetitions results in 1.84 dB lower MCL. The PRACH capacity is 92% lower than a length-1151 ZC sequence.

Table 7. Performance of different PRACH schemes with 15 kHz SCS

| Scheme | NZC | R | NFDM | SCS*NZC*R (MHz) | Np (dBm) | SNRFD (dB) | Pmax (dBm) | 95% CM (dB) | PTx (dBm) | MCL (dB) | PRACH capacity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ZC139 (A1) | 139 | 1 | 8 | 2.085 | -105.81 | -3.64 | 13.17 | 2.33 | 13.17 | 122.62 | 18768 (272 cells) |
| ZC139 (A3) | 139 | 1 | 8 | 2.085 | -105.81 | -7.58 | 13.17 | 2.33 | 13.17 | 126.56 | 18768 (272 cells) |
| ZC139x4 (Ericsson) | 139 | 4 | 2 | 8.34 | -99.79 | -10.53 | 19.19 | 0.50 | 19.19 | 129.51 | 4692 (68 cells) |
| ZC139x4 (LGE) | 139 | 4 | 2 | 8.34 | -99.79 | -10.70 | 19.19 | 2.15 | 19.19 | 129.68 | 4692 (68 cells) |
| ZC571* | 571 | 1 | 2 | 8.565 | -99.67 | -10.74 | 19.31 | 2.33 | 19.31 | 129.72 | 30780 (380 cells) |
|  |  |  |  |  |  |  |  | 1.70 |  |  | 22032 (272 cells) |
|  |  |  |  |  |  |  |  | 0.32 |  |  | 5508 (68 cells) |
| ZC139x8 (LGE) | 139 | 8 | 1 | 17.205 | -96.78 | -13.89 | 22.20 | 2.26 | 20.74 | 131.41 | 2346 (34 cells) |
| ZC1151* | 1151 | 1 | 1 | 17.265 | -96.63 | -14.27 | 22.35 | 2.33 | 20.67 | 131.57 | 29900 (383 cells) |
|  |  |  |  |  |  |  |  | 1.68 | 21.32 | 132.22 | 21216 (272 cells) |
|  |  |  |  |  |  |  |  | -0.05 | 22.35 | 133.25 | 2652 (34 cells) |

*For the single long ZC sequences, we provide the 95% CM values using all preambles or by choosing a subset of preambles with the lowest CM values that results in the same PRACH capacity as of the length-139 ZC sequence based schemes.

Thereto, Tables 6 and 7 show an unacceptable consequence that a repeated length-139 ZC sequence results in smaller PRACH capacity than in NR Rel-15!

Observation 8: Compared to the NR Rel-15 length-139 ZC sequence without repetition, a repeated length-139 ZC sequence results in ~50-87.5% lower PRACH capacity.

Regarding the timing estimation error, it can be seen from Figures A-1(c) and A-2(c) that a repeated ZC sequence and a single long ZC sequence of similar length have almost the same performance, both having less timing error than the NR Rel-15 length-139 ZC sequence due to increased frequency resource occupation.

In Figure 5, the outage probability is shown for different PRACH schemes as a function of the communication distance between the UE and BS, where the outage probability is defined as the probability that the large scale propagation loss (including path loss and shadow fading) experienced by the PRACH preamble is larger than its MCL value calculated in Tables 6 and 7. Here the NLoS case of the UMi-Street Canyon channel model [8] is considered assuming 8 dBi and 3 dBi antenna gains at the BS and UE, respectively. The outage probability in the LoS case is not included since it is sufficiently low (<10-7) for all PRACH schemes. The results show that, compared to the NR Rel-15 PRACH scheme with format A1, using a PRACH format with more time-domain repetition (e.g., format A3) or using a PRACH preamble with a larger sequence length can both provide up to an order of magnitude lower outage probability thanks to their higher MCL values. In this comparison, we have assumed the maximum PRACH capacity for each sequence, cf., Tables 6 and 7. The outage probability for a single long ZC sequence based PRACH scheme could be even lower if the same PRACH capacity as its corresponding repetition based PRACH scheme is assumed.

(a)                                                                               (b)

Figure 5. Outage probabilities of different PRACH schemes for (a) 15 kHz SCS and (b) 30 kHz SCS, using PRACH format A1 or A3, as a function of the communication distance between UE and BS in the NLoS case of the UMi-Street Canyon channel. The antenna gains are 8 dBi and 3 dBi at the gNB and UE, respectively.

## Issues of repeated ZC sequence

### PRACH capacity

The number of supported cells for a PRACH scheme with 64 preambles per cell on a given time-frequency resource is determined by

Ncell=NZC-164NZC/NCS∙NFDM                                                            (1)

where ∙ and ∙ are the ceiling and floor operators, respectively, and the number of frequency resources NFDM within 20 MHz bandwidth for each scheme is given in Tables 6 and 7. In Figure 6, we use (1) to plot Ncell for different PRACH schemes with 30 kHz SCS where the cyclic shift values  NCS are taken from Table 8 in Section 3.4 and Table A-3 in Appendix A, and where we use equation (3) in Section 3.4 to compute the maximum supportable cell radius from NCS. The results in Figure 6 show that a length-139 ZC sequence with 2 or 4 repetitions has a PRACH capacity loss of 50% and 75%, respectively, over all cell radii compared to that of NR Rel-15. On the other hand, the single long ZC sequences with NZC=283 and  NZC=571 offer a capacity gain compared to NR Rel-15.

Figure 6. Number of supported cells with 64 preambles per cell as a function of cell radius for different PRACH schemes with 30 kHz SCS. The maximum channel delay is τ=0.87 µs and the number of guard samples is ng=2.

Unlicensed networks may not be planned among operators, and hence their cells in vicinity of each other could end up using the same set of PRACH preambles. This would cause worse detection performance, more frequent use of contention resolution, more PRACH attempts and in the worst case, complete PRACH failure. This devastating situation occurs more often for the repetition based PRACH schemes since their low PRACH capacity translates into a smaller reuse distance of the preambles. The probability, p, that the same set of 64 PRACH preambles is configured by more than one operator for a given cell location is calculated as

p=1-i=0NOperator-1Ncell-iNcellNOperator								(2)

where Ncell is the number of cells supported by each PRACH scheme (listed in Tables 6 and 7), and NOperator is the number of coexistent operators. This probability is non-negligible as seen from Figure 7.

Observation 9: The low PRACH capacity of repetition based schemes increases by ~10 times the probability that the same set of preambles is used in cells overlapping from different operators compared to using a single long sequence.

(a)                                                                               (b)

Figure 7. Probability that the same set of preambles is used by different operators in a same cell area for different PRACH schemes with (a) 15 kHz SCS and (b) 30 kHz SCS.

### CM values

In Figure 8, we plot the complementary cumulative distribution functions (CCDFs) of the CM values of different PRACH schemes, where the curves for single long ZC sequence based PRACH schemes are obtained by selecting a preamble subset with lowest CM values that support the same number of cells as their corresponding repetition based PRACH schemes. It can be seen that using a single long sequence gives several dB lower CM than using a repeated length-139 ZC sequence with cyclic shifts/phase ramps/phase shifts, for a same PRACH capacity.

(a)                                                                               (b)

Figure 8. CCDFs of the CM values for different PRACH schemes with (a) 15 kHz SCS and (b) 30 kHz SCS, where the single long ZC sequence based PRACH schemes are measured with a subset of generated preambles with lowest CM values, by which the numbers of supported cells are the same as those supported by their corresponding repetition based PRACH schemes.

In Figure 9, we show the CM curves for preambles of different PRACH schemes generated in an order according to the NCS values in Table A-1 and the principle of Rel-15 logical root index ordering (cf. (7)). The same number of preambles is generated for all PRACH schemes and it is shown that with single long ZC sequence, the CM curves are approximately monotonically increasing, while those for repeated ZC sequences have large fluctuations. This implies that, when a repetition based PRACH scheme is applied according to the Rel-15 NCS values and logical root index ordering, the generated 64 preambles in a cell will have significantly different maximum transmit power, which will lead to unequal performance and coverage among them.

(a)                                                                               (b)

Figure 9. CM curves for different PRACH schemes with (a) 15 kHz SCS and (b) 30 kHz SCS, where the preambles are generated in the order according to the NCS values in Table A-1 and the principle of Rel-15 logical root index ordering. For single long ZC sequence based PRACH schemes, the preambles are generated up to the same number as that can be generated by the corresponding repetition based PRACH schemes.

Observation 10. The CM for repeated ZC sequence is significantly worse (several dBs) compared to that of a single long ZC sequence. When using the Rel-15 cyclic shift table and logical root index ordering, repeated ZC sequence leads to very large variations of the CM within the set of 64 preambles.

Auto-/cross-correlation functions

For single long ZC sequence based PRACH schemes, the generated preambles exhibit ideal periodic auto-/cross-correlation properties. However, a repeated ZC sequence is a form of interlaced waveform, which will not be able to inherit the ideal properties of ZC sequences. This is clear from Figure 10 which shows the periodic auto-correlation functions (a) and the periodic cross-correlation functions between preambles generated by the same ZC root index (b) or different ZC root indices (c) for various PRACH schemes with 30 kHz SCS. When there are frequency gaps between adjacent repetitions (e.g., ZC139x4 (LGE)), the periodic auto-correlation function exhibits sidelobes inside the timing detection window and the preambles are not orthogonal, even when using a same root index. Moreover, when different root indices are used, the periodic cross-correlation functions have a maximum value twice that of the single long ZC sequence (i.e., which is equal to 1/NZC).

(a)

(b)                                                                               (c)

Figure 10. Examples of the normalized periodic auto-correlation functions (a), and the normalized periodic cross-correlation functions between PRACH preambles generated by the same ZC root index (b) or different ZC root indices (c) for different schemes with 30 kHz SCS within the timing detection window of length 2.02 μs (i.e., 1.15 μs maximum round trip delay with 300 m ISD plus 0.87 μs maximum channel delay for CDL-C channel with 100 ns desired delay spread).

Observation 11. Sequence repetition with frequency gaps exhibits sidelobes in the auto-correlation function and results in non-orthogonal preambles, even when using different cyclic shifts for a single root index. For preambles from different root indices, the maximum cross-correlation of sequence repetition, with or without frequency gaps, is two times that of a single long ZC sequence.

Cyclic shift dimensioning and root index ordering

In Sections 3.4 and 3.5, we show that it is straightforward to define the cyclic shifts and the root index ordering for new ZC sequence lengths using the same methods as in NR Rel-15, with which the low cross-correlation between PRACH preambles used in the same cell can be maintained, thanks to the ideal auto-/cross-correlation properties of ZC sequences. As can be seen from Figures A1 b) and A2 b), the single long ZC sequence based PRACH schemes always achieve satisfactory false detection performance due to their low cross-correlation property, lower than the target false alarm probability of 0.1%.

In contrast, a repetition based PRACH preamble is a form of interlaced waveform, whose auto-/cross-correlation functions are no longer ideal, as have been shown in Figure 10. It has been proposed that a repetition based PRACH scheme reuses the cyclic shift and root index ordering tables for the NR Rel-15 length-139 ZC sequence. However, these tables affect the cross-correlation among the generated preambles (and in turn the false detection performance). Figure 11 shows the CCDF of the maximum cross-correlation values between preambles in a set of 64 preambles used in the same cell within the timing detection window of length 2.02 μs with 15 and 30 kHz SCSs. Here the set of 64 preambles is generated based on the NZC values in Table A-1 and the Rel-15 root index ordering (i.e., as shown in (7)) with an arbitrary starting logical root index. Clearly, repetition based PRACH schemes reusing the cyclic shift and root index ordering tables of NR Rel-15 bring higher cross-correlation than their corresponding single long ZC sequence based PRACH schemes.

This higher cross-correlation eventually translates to higher false detection probabilities which are even higher than the target false alarm probability of 0.1% as shown in Figures A1 b)  and A2 b). This corroborates that there are dire consequences of reusing the NR Rel-15 cyclic shift and root index ordering tables for repetition based PRACH schemes. If the detection threshold is increased to reduce the higher false detection probability, the misdetection probability would increase instead. Hence the current cyclic shift and root index ordering tables in NR Rel-15 cannot be directly reused for repetition based PRACH schemes, and new methods are required for specifying the cyclic shifts and the root index ordering.

Observation 12. Sequence repetition causes higher false detection probability than a single long ZC sequence, since it destroys the ideal correlation property of the ZC sequence. Therefore, the cyclic shift table and root index ordering table of the length-139 ZC sequence from NR Rel-15 cannot be reused for a repetition based PRACH scheme.

(a)                                                                               (b)

Figure 11. Normalized maximum cross-correlation within the timing detection window of length 2.02 μs for different PRACH schemes with (a) 15 kHz SCS and (b) 30 kHz SCS.

Preamble implementation/detection complexity

A single long ZC sequence is not new since the Rel-15 NR UE is already capable of transmitting a single ZC sequence with very many lengths for the PRACH, DMRS and SRS, see Table A-2 in Appendix A. The candidate lengths 283, 571 and 1151 for the single long ZC sequence are all contained in Table A-2, and thus there is no additional UE complexity of adopting any of these lengths. Similar to the legacy length-139 ZC sequence based PRACH scheme, the detection of a single long ZC sequence based PRACH preamble can enjoy the following aspects of gNB detection complexity reductions:

- Reduction 1: The power delay profile (PDP) calculation for the PRACH preambles generated by a common ZC root index with different cyclic shifts can be made by a single matched filter, since different cyclic shifts correspond to different timing detection windows. This leads to a reduction of detection complexity by a factor of 1/NZCNCS;

- Reduction 2: The detection of PRACH preambles generated by paired root indices (i.e., root r and NZC - r) can share the multiplication operations involved in the matched filter, thanks to the complex conjugate and central symmetry of them [9], which further brings a reduction of detection complexity by a factor of 1/2.

The Reduction 1 above is achieved based on the fact that preambles generated from a common ZC root sequence are cyclically shifted versions of each other. If a common cyclic shift gap NCS is applied to all the repetitions of a repetition based PRACH scheme, it can be guaranteed that the PRACH preambles generated from a common ZC root sequences are cyclically shifted versions of each other, which is the case for all the repetition based PRACH schemes evaluated in this contribution. Hence these repetition based PRACH schemes can enjoy the detection complexity Reduction 1.

The Reduction 2 above is achieved based on the complex conjugate and central symmetry between ZC sequences with complementary root indices (i.e., root r and NZC - r). However, a repetition based PRACH scheme cannot maintain this symmetry property after applying different cyclic shifts/phase ramps/phase shifts to different repetitions for the purpose of CM reduction. Hence a repetition based PRACH scheme cannot enjoy the detection complexity Reduction 2, and so has at least twice higher detection complexity than a single long ZC sequence based PRACH scheme.

An alternative method of preamble detection for repetition based PRACH schemes is to perform matched filter correlation for each repetition separately, and then combine their matched filter outputs, either coherently or non-coherently, to obtain the final PDP. By this means, each repetition may be regarded as a NR Rel-15 PRACH preamble with certain cyclic shift/phase ramp/phase shift. However, the sampling rate cannot be kept the same as for the NR Rel-15 detector since the bandwidth of the PRACH has increased. Thus, a new detector would be needed anyhow. In addition, the complex conjugate and central symmetry properties of each repetition are destroyed when a phase ramp is applied within the repetition, making it unable to enjoy the above detection complexity Reduction 2. In summary, this alternative detection method for repetition based PRACH schemes suffers the following extra detection complexity compared to their corresponding single long ZC sequence based PRACH schemes: a) X-1 times more matched filter calculations where X is the number of repetitions, b) extra addition/multiplication operations involved in combining multiple matched filter outputs together, and c) unable to enjoy the detection complexity Reduction 2 when there is phase ramping within the repetitions.

Combining the above discussion, we have the following observation.

Observation 13: A ZC sequence repetition based PRACH preamble has at least twice higher detection complexity than that based on a single long ZC sequence.

## Summary of evaluation

### The study above is summarized in this observation.

### Observation 14: A repeated length-139 ZC sequence:

- performs worse than a single long ZC sequence of similar length in terms of CM, MCL and PRACH capacity

- results in a lower PRACH capacity than what NR Rel-15 supports

- needs deployment and specification of sequence manipulations for CM reduction

- needs a new method for specifying the cyclic shifts

- needs a new method for specifying the root index ordering

Therefore, there is no motivation, either from performance point of view, or in terms of specification impact, to adopt a repeated ZC sequence. Regarding the choice between the lengths 571/1151 and 283/571 for single long ZC sequences, the results in Tables 6 and 7 show that larger lengths have ~3 dB higher MCL and in turn lower outage probability as demonstrated in Figure 5. The number of PRACH occasions in frequency will be half of that for a longer sequence but overall there is a net gain in PRACH capacity, as shown in Figure 6. Hence there is no advantage from a sequence that span only 10 MHz, i.e., half of the available 20 MHz.

The WI states that 60 kHz SCS will be supported for control and data channels. A PRACH with 60 kHz SCS is anticipated to be useful for low-latency access and for single-numerology configuration. From the results in Figure A-3, we have not observed any particular technical prohibitive issue which makes 60 kHz SCS PRACH performance problematic. The existing length-139 ZC sequence could be reused without any further optimizations. Whether 60 kHz SCS for PRACH could be optional or not is a future discussion. Hence, the following proposal is made.

Proposal 7: The NR-U PRACH is based on a single ZC sequence with the following lengths:

- For 15 kHz SCS: LRA=1151

- For 30 kHz SCS: LRA=571

- For 60 kHz SCS: LRA=139

## Cyclic shift dimensioning for ZC sequences of length 283, 571 and 1151

Determining the cyclic shifts of a ZC sequence with new length is trivial and the same method which was applied for LTE and NR could be reused. Consider a ZC sequence of length NZC used at a given cell radius Rcell. For orthogonal multiplexing, the cyclic shift, NCS, needs to fulfill

NCS≥2∙Rcellc+τ∙NZC∙∆f+ng	                                                     (3)

where τ is the maximum channel delay, c is the speed of light, ∆f is the SCS and ng is the number of guard samples to account for TX/RX filtering. For a given cyclic shift NCS, the number of preamble sequences that can be generated per ZC root sequence is

N=NZCNCS.                                                                             (4)

In NR, the cyclic shifts are limited to 16 values and NCS is signaled by the RRC parameter zeroCorrelationZoneConfig. Hence, N can be smaller for a given cell radius Rcell, than if any integer would have been possible for NCS (i.e., a smaller NCS would have been feasible). Therefore, the set of NCS values (denoted by {NCSk| k=0, 1, …, 15}) should be chosen so that the number of preambles is as close as possible to what would have been feasible if there were no restrictions on the value of NCS. It has been mathematically proven in [10] that the optimal solution to this problem is

NCSk=NZCNSPR0∙ak+a1-a(ak-1),       k=0,1,⋯,140,                                                                 k=15,                                       (5)

where ∙ denotes rounding to the closest integer and a is the solution to

NSPRk=NSPR0∙ak+a1-aak-1.                                                                  (6)

with NSPRk being a boundary condition for the number of preambles per root sequence. Obviously, setting a large value for NSPRk results in a small supported cell radius Rcell. More precisely, in the optimal solution, the cyclic shifts are chosen such that for any cell radius that is located between the ones supported by NCSk and NCSk+1, respectively, the number of preambles is as close as possible to the number of preambles that could be obtained if there were no restrictions on the value of NCS. The parameter a can be obtained numerically from (6) by setting the boundary conditions NSPR0 and NSPR14. Equation (5) was used for determining the NCS values for unrestricted sets in LTE and NR for the sequence length NZC=839, assuming that it should be possible to multiplex 64 preambles on a single root sequence (i.e., NSPR0=64) and that it should be possible to multiplex at least 2 preambles on a single root sequence (i.e., NSPR14=2). These values result in a=0.8561. The value NCS15=0 denotes that no cyclic shift multiplexing is used, which achieves the maximum Rcell provided by the PRACH format as shown in Table A-4 of Appendix A. Table A-3 in Appendix lists the NCS values for the length-139 ZC sequence in NR together with the supported cell radius for different SCS values according to (3), where τ=0.87 µs (i.e., assuming the CDL-C model with 100 ns desired delay spread) and ng=2.

Similarly, we apply (5) by assuming NSPR0=64, NSPR14=2, τ=0.87 µs and ng=2, and can obtain the cyclic shift tables for ZC sequences of length 283, 571 and 1151 as shown in Table 8 below, where the proposed NCS values for NZC = 571 and NZC = 1151 are the same as those in [5].

Table 8. Cyclic shifts NCS for different ZC sequence lengths NZC and the corresponding maximum cell radii Rcell for 15 kHz or 30 kHz SCS, using NSPR0=64 and NSPR14=2 assuming τ=0.87 µs and ng=2

| zeroCorrelation ZoneConfig | NZC=571, Δf=15 kHz |  | NZC=1151, Δf=15 kHz |  | NZC=283, Δf=30 kHz |  | NZC=571, Δf=30 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NCS | Rcell [m] | NCS | Rcell [m] | NCS | Rcell [m] | NCS | Rcell [m] |
| 0 | 0 | max | 0 | max | 0 | max | 0 | max |
| 1 | 8 | <1 | 17 | <1 | 4 | <1 | 8 | <1 |
| 2 | 10 | 9 | 21 | 34 | 5 | <1 | 10 | <1 |
| 3 | 12 | 44 | 25 | 69 | 6 | <1 | 12 | <1 |
| 4 | 15 | 97 | 30 | 112 | 7 | <1 | 15 | <1 |
| 5 | 17 | 132 | 35 | 156 | 8 | <1 | 17 | <1 |
| 6 | 21 | 202 | 44 | 234 | 10 | 10 | 21 | 35 |
| 7 | 25 | 272 | 52 | 303 | 12 | 46 | 25 | 70 |
| 8 | 31 | 377 | 63 | 399 | 15 | 99 | 31 | 123 |
| 9 | 40 | 534 | 82 | 564 | 20 | 187 | 40 | 202 |
| 10 | 51 | 727 | 104 | 755 | 25 | 275 | 51 | 298 |
| 11 | 63 | 937 | 127 | 955 | 31 | 381 | 63 | 403 |
| 12 | 81 | 1253 | 164 | 1276 | 40 | 540 | 81 | 561 |
| 13 | 114 | 1830 | 230 | 1850 | 56 | 823 | 114 | 850 |
| 14 | 190 | 3161 | 383 | 3179 | 94 | 1494 | 190 | 1515 |
| 15 | 285 | 4825 | 575 | 4847 | 141 | 2325 | 285 | 2347 |

Proposal 8: The following cyclic shifts are used for ZC sequences of length NZC:

- If NZC=283:  NCS∈0,4,5,6,7,8,10,12,15,20,25,31,40,56, 94, 141

- If NZC=571:  NCS∈0,8,10,12,15,17,21,25,31,40,51,63,81,114,190,285

- If NZC=1151:  NCS∈0,17,21,25,30,35,44, 52,63,82,104,127,164,230,383,575

## Root index ordering for ZC sequences of length 283, 571 and 1151

For the length-839 ZC sequence in LTE, the logical root indices i are mapped to the sequence root indices u considering an ordering according to the CM values. The length-139 ZC sequence was introduced in LTE and there were several proposals on the logical root index mapping [11]-[13] which also considered the CM. However, it was agreed to just follow a complex conjugate pair ordering, since this reduces detection complexity with ~50% [9]. Ordering in terms of CM may be even less important for small cells. The same root index ordering was used for the length-139 sequence in NR and it could be straightforwardly applied to any new ZC sequence length, i.e., the mapping is done in pairs of 1, NZC-1,2, NZC-2, 3, NZC-3,⋯. Equivalently, the sequence root index u could be determined from a logical root index i by

ui=yi, i=0,1,⋯,NZC-2                                                                                                
y2k=k+1, y2k+1=NZC-k-1, k=0,1,⋯,NZC2                                        (7)

The same root index ordering in (7) has been adopted in [5] to generate the tables of logical root index for NZC=571 and NZC=1151 as given in Appendix B.

Figure 12. The CM profiles of single long ZC sequences of length 1151/571/283 in the order of logical root indices in (7).

Figure 12 shows that using (7), the CM value is approximately increasing with the logical root index, i.e., larger logical root indices result in higher CM values. This implies that if low CM is critical in a deployment, the gNB could signal a small logical root index from which the UE obtains a preamble with a low CM value and that the preambles of a cell have similar CM values. Hence there is no need to purge certain root indices from the root index set or to develop another ordering than (7) in order to optimize the CM values.

Proposal 9: The logical root indices i=0,1,2,3,4,5,⋯ are mapped to the sequence root indices u as 1, NZC-1,2, NZC-2, 3, NZC-3,⋯.

## Multiplexing between PRACH and PUSCH/PUCCH

Two cases for multiplexing between PRACH and PUSCH/PUCCH are considered:

- FDM between PRACH and PRB-interlaced PUSCH/PUCCH

This would always have been possible if PRB-interlaced PRACH was adopted. Thus, if the PRACH allocation overlaps with the PUSCH/PUCCH allocation, this type of multiplexing should not be allowed. We see no reason to manipulate and rate match the PUSCH/PUCCH resource allocation to fit around the PRACH. If the PRACH allocation does not overlap with the PUSCH/PUCCH allocation (e.g., the allocations are in different LBT subbands), multiplexing is possible. However, this does not require any particular standardized solution.

- FDM between PRACH and contiguous PUSCH/PUCCH

This could be possible by scheduling PUSCH/PUCCH on resources orthogonal to the PRACH. The potential issue is that, if the allocations are in the same LBT subband, PRACH transmission may be blocked by other UEs’ PUSCH/PUCCH transmissions if the UEs are sufficiently close such that they interfere with each other and the timing advance is larger than the RX-to-TX switch time. The agreed 300 m ISD assumed in the evaluations corresponds to a Round Trip Time (RTT) of 1.2 μs, which is smaller than the expected switch time (e.g., 13 μs and 7 μs for FR1 and FR2, respectively). Hence, the issue will not be severe and the gNB could handle it with implementation specific means.

Proposal 10: No solution with specification impact is adopted for handling multiplexing between PRACH and PUSCH/PUCCH.

# Conclusions

In this contribution, we have made the following proposals:

For DRS:

Proposal 1: Alternative 2 shall be adopted in NR-U as SSB position and Type-0 PDCCH monitoring in a slot.

- Support Type0-PDCCH in symbol (#0, #1) for length-2 CORESET 0 and symbol (#0) for length-1 CORESET 0 for the first SSB in a slot.

- Support Type0-PDCCH in symbol (#7, #8) for length-2 CORESET and symbol (#7) for length-1 CORESET 0 for the second SSB in a slot.

Proposal 2: NR-U shall support that RMSI PDSCH scheduled by Type-0 PDCCH is rate-matched around the corresponding SSBs.

Proposal 3: The table of Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set can be simplified as Table 2 and Table 3, corresponding to 1 bit controlResourceSetZero in MIB. The Offset X and Y from smallest RB index of CORESET0 to the smallest RB index of the common RB overlapping with the first RB of the corresponding SS/PBCH block on the sync raster can be derived from the GSCN according to Table 4.

Proposal 4: The entries in Table 5 should be included in the table of Parameter for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1, in order to facilitate UE to monitor Type0-PDCCH in the same slot as its associated SS/PBCH block.

Proposal 5: Indicator for OSI and paging and Q value should be included in MIB.

Proposal 6: Confirm the work assumption in RAN1#98bis on the derivation of location of the CORESET0 for RMSI reception on a carrier with an SSB not on a sync raster.

For PRACH:

Proposal 7: The NR-U PRACH is based on a single ZC sequence with the following lengths:

- For 15 kHz SCS: LRA=1151

- For 30 kHz SCS: LRA=571

- For 60 kHz SCS: LRA=139

Proposal 8: The following cyclic shifts are used for ZC sequences of length NZC:

- If NZC=283:  NCS∈0,4,5,6,7,8,10,12,15,20,25,31,40,56, 94, 141

- If NZC=571:  NCS∈0,8,10,12,15,17,21,25,31,40,51,63,81,114,190,285

- If NZC=1151:  NCS∈0,17,21,25,30,35,44, 52,63,82,104,127,164,230,383,575

Proposal 9: The logical root indices i=0,1,2,3,4,5,⋯ are mapped to the sequence root indices u as 1, NZC-1,2, NZC-2, 3, NZC-3,⋯.

Proposal 10: No solution with specification impact is adopted for handling multiplexing between PRACH and PUSCH/PUCCH.

# References

3GPP TR 38.214, v15.6.0, NR, Physical layer procedures for data, June 2019.

3GPP TR 38.213, v15.6.0, NR, Physical layer procedures for control, June 2019.

Huawei, “Initial access in NR unlicensed”, R1-1910046, Chongqing, China, October 14-20, 2019.

3GPP TR 38.889, v16.0.0, Study on NR-based access to unlicensed spectrum, December 2018.

Huawei, HiSilicon, “Introduction of Zadoff-Chu sequences of length 571 and 1151 for NR-U PRACH,” R1-1911614, Chongqing, China, Oct. 14-20, 2019.

Samsung, LG Electronics, ZTE, Sanechips, vivo, Fujitsu, MediaTek Inc., Nokia, NSB, OPPO, Xiaomi, “Summary of Unofficial Offline Discussion on Wideband PRACH Design for NR-U,” R1-1911696, Chongqing, China, Oct. 14-20, 2019.

Ericsson, “Evaluation results for enhanced PUCCH and PRACH designs,” R1-1909303, Prague, Czech Republic, Aug. 26 – 30, 2019.

3GPP TS 38.901 V15.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”, June 2019.

Huawei, “Efficient matched filters for paired root Zadoff-Chu sequences”, R1-071409, St. Julian’s, Malta, March 26-30, 2007.

Huawei, “Multiple values of cyclic shift increment NCS”, R1-072325, Kobe, Japan, May 7 – 11, 2007.

Huawei, “Root Zadoff-Chu sequence order for PRACH format 4 in FS2 TDD”, R1-081029, Sorrento, Italy, Feb. 11 – 15, 2008.

ZTE, Qualcomm Europe, Texas Instruments, “ZC sequence ordering for short RACH”, R1-081069, Sorrento, Italy, Feb. 11 – 15, 2008.

Ericsson, “ZC sequence sorting for RACH preamble format 4”, R1-080895, Sorrento, Italy, Feb. 11 – 15, 2008.

Chairman’s Notes RAN1 Meeting #96, Athens, Greece, Feb. 25 – Mar. 1, 2019.

Chairman’s Notes RAN1 Ad Hoc Meeting, Taipei, Jan. 21-25, 2019.

# Appendix A:

Table A-1 lists all the candidate PRACH schemes for NR-U. Their related ZC sequence lengths NZC, cyclic shift values NCS and frequency resource allocations are detailed in the table. To generate 64 preambles in one cell for each scheme, we take their root indexes in the order of (7), and set the NCS values according to the supported cell radius (i.e., 173 m) based on Table 8 in Section 3.4 for single long ZC sequence based schemes, and based on the Rel-15 length-139 ZC table (i.e., Table A-3) for repetition based schemes. For single long ZC sequence based PRACH schemes, the cyclically shifted sequence is directly used to generate the corresponding frequency-domain modulation sequence via IDFT. For R-time repetition based PRACH schemes, different extra cyclic shifts/phase ramps/phase shifts are further applied to the cyclically shifted sequences to generate the corresponding segments of frequency domain modulation sequences, which are then concatenated and mapped to the frequency resources according to the RE allocations of one RACH occasion (RO) in Table A-1. The detailed values of extra cyclic shifts/phase ramps/phase shifts are listed in Table A-1, which are cited from [6][7] to reduce the CM of resultant PRACH preambles.

Table A-1. Candidate PRACH schemes for NR-U

| SCS (kHz) | Scheme | Frequency domain sequence generation |  |  |  | NRB per RO | Bandwidth per RO (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NZC | NCS | R | RE allocation of one RO |  |  |
| 15 | ZC139 | 139 | 8 | 1 | 139 contiguous REs | 12 | 2.085 |
|  | ZC139x4 (Ericsson) | 139 | 8 | 4 | 556 contiguous REs with phase ramp step [0, 0, 0.008, 0.008] and common phase shift [0, -0.05, -0.05, ] for each of the four repetitions [7] | 47 | 8.34 |
|  | ZC139x4 (LGE) | 139 | 8 | 4 | REs 2 to 140 of every 12 PRBs, with cyclic shift [a, a+1, a, a+1] and phase shift [1, 1i, 1i, 1] for each of the 4 repetitions [6] | 48 | 8.565 |
|  | ZC571 | 571 | 21 | 1 | 571 contiguous REs | 48 | 8.565 |
|  | ZC139x8 (LGE) | 139 | 8 | 8 | REs 2 to 140 of every 12 PRBs, with cyclic shift [a, a+1, a, a+1, a, a+1, a, a+1] and phase shift [1, 1i, 1i, -1, -1, 1i, 1i, 1] for each of the 8 repetitions [6] | 96 | 17.205 |
|  | ZC1151 | 1151 | 44 | 1 | 1151 contiguous REs | 96 | 17.265 |
| 30 | ZC139 | 139 | 12 | 1 | 139 contiguous REs | 12 | 4.17 |
|  | ZC139x2 (Ericsson) | 139 | 12 | 2 | 278 contiguous REs with phase ramp step [0, -0.008] for each of the two repetitions [7] | 24 | 8.34 |
|  | ZC139x2 (LGE) | 139 | 12 | 2 | REs 2 to 140 of every 12 PRBs, with cyclic shift [a, a+1] for each of the 2 repetitions [6] | 24 | 8.49 |
|  | ZC283 | 283 | 20 | 1 | 283 contiguous REs | 24 | 8.49 |
|  | ZC139x4 (Ericsson) | 139 | 12 | 4 | 556 contiguous REs with phase ramp step [0, 0, 0.008, 0.008] and common phase shift [0, -0.05, -0.05, ] for each of the four repetitions [7] | 47 | 16.68 |
|  | ZC139x4 (LGE) | 139 | 11 | 4 | REs 2 to 140 of every 12 PRBs, with cyclic shift [a, a+1, a, a+1] and phase shift [1, 1i, 1i, 1] for each of the 4 repetitions [6] | 48 | 17.13 |
|  | ZC571 | 571 | 40 | 1 | 571 contiguous REs | 48 | 17.13 |
| 60 | ZC139 | 139 | 19 | 1 | 139 contiguous REs | 12 | 8.34 |

Figures A-1 to A-3 show the performance of all the candidate PRACH schemes listed in Table A-1 with formats A1 and/or A3, which are obtained following the simulation assumptions agreed in [14][15].

(a) 												(b)

(c)

Figure A-1. Performance comparison of PRACH schemes with 15 kHz SCS.

(a) 												(b)

(c)

Figure A-2. Performance comparison of PRACH schemes with 30 kHz SCS.

(a) 												(b)

(c)

Figure A-3. Performance of PRACH scheme with 60 kHz SCS.

Table A-2 lists the lengths of ZC sequences that are supported in Rel-15 NR at UE for PRACH, DMRS and the SRS.

Table A-2. Supported ZC sequence lengths for a Rel-15 NR UE

| NZC | NZC | NZC | NZC | NZC | NZC | NZC | NZC | NZC | NZC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31 | 151 | 271 | 389 | 509 | 647 | 761 | 919 | 1039 | 1163 |
| 47 | 167 | 283 | 401 | 523 | 659 | 773 | 929 | 1051 | 1171 |
| 59 | 179 | 293 | 419 | 547 | 661 | 787 | 947 | 1063 | 1187 |
| 71 | 191 | 311 | 431 | 563 | 683 | 797 | 953 | 1069 | 1193 |
| 83 | 199 | 317 | 443 | 571 | 691 | 811 | 971 | 1091 | 1201 |
| 89 | 211 | 331 | 449 | 587 | 701 | 827 | 983 | 1103 | 1223 |
| 107 | 227 | 347 | 467 | 599 | 719 | 839 | 991 | 1109 | 1231 |
| 113 | 239 | 359 | 479 | 607 | 727 | 863 | 997 | 1123 | 1237 |
| 131 | 251 | 367 | 491 | 619 | 743 | 887 | 1019 | 1129 | 1259 |
| 139 | 263 | 383 | 503 | 631 | 751 | 911 | 1031 | 1151 | - |

Table A-3 lists the NCS values for the length-139 ZC sequence in NR together with the supported cell radius for different SCS values according to (3), where τ=0.87 µs and ng=2. The values marked in green were obtained according to (5) from [10] assuming 3-bit signaling (i.e., 8 NCS values) and the values marked in blue were subsequently manually inserted, since it was later agreed to use a 4-bit table (i.e., 16 NCS values). Notably, the number of NCS values is 8 for the length-139 ZC sequence in LTE. Table A-3 shows that in several cases, the cell radius is less than 1 m. However, it was understood that PRACH is used for other applications than initial access (e.g., contention-free random access, beam management etc.) wherein the receive timing is already known and for which it was considered useful with very small NCS values for high multiplexing capacity.

Table A-3. Cyclic shift values NCS for the length-139 ZC sequence in NR Rel-15 and corresponding maximum cell radii  Rcell [m] based on (3) assuming τ=0.87 µs and ng=2

| zeroCorrelation ZoneConfig | NCS | Rcell [m] for 15 kHz SCS | Rcell [m] for 30 kHz SCS | Rcell [m] for 60 kHz SCS | Rcell [m] for 120 kHz SCS |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | Maximum supported by the PRACH format | Maximum supported by the PRACH format | Maximum supported by the PRACH format | Maximum supported by the PRACH format |
| 1 | 2 | <1 | <1 | <1 | <1 |
| 2 | 4 | 13 | <1 | <1 | <1 |
| 3 | 6 | 157 | 13 | <1 | <1 |
| 4 | 8 | 301 | 85 | <1 | <1 |
| 5 | 10 | 445 | 157 | 13 | <1 |
| 6 | 12 | 588 | 229 | 49 | <1 |
| 7 | 13 | 660 | 265 | 67 | <1 |
| 8 | 15 | 804 | 337 | 103 | <1 |
| 9 | 17 | 948 | 409 | 139 | 4 |
| 10 | 19 | 1092 | 481 | 175 | 22 |
| 11 | 23 | 1380 | 624 | 247 | 58 |
| 12 | 27 | 1668 | 768 | 319 | 94 |
| 13 | 34 | 2171 | 1020 | 445 | 157 |
| 14 | 46 | 3034 | 1452 | 660 | 265 |
| 15 | 69 | 4689 | 2279 | 1074 | 472 |

# Table A-4 lists the maximum supportable cell radius for each NR Rel-15 PRACH format.

Table A-4. Maximum cell radii Rcell [m] for the NR Rel-15 PRACH formats for different SCS assuming τ=0.87 µs and ng=2

| Format | Δf=15 kHz | Δf=30 kHz | Δf=60 kHz | Δf=120 kHz |
| --- | --- | --- | --- | --- |
| A1 | 1276 | 573 | 221 | 45 |
| A2 | 2682 | 1276 | 573 | 221 |
| A3 | 4088 | 1979 | 924 | 397 |
| B1 | 924 | 397 | 133 | 1 |
| B2 | 1627 | 748 | 309 | 89 |
| B3 | 2330 | 1100 | 485 | 177 |
| B4 | 4440 | 2155 | 1012 | 441 |
| C0 | 5924 | 2897 | 1383 | 626 |
| C2 | 9870 | 4870 | 2370 | 1120 |

# Appendix B: Specification changes proposed in R1-1911614

# Specification changes for TS38.211

## PRACH preamble definition

#### 6.3.3.1	Sequence generation

The set of random-access preambles  shall be generated according to

from which the frequency-domain representation shall be generated according to

Where LRA=1151,  , LRA=571 or  depending on the PRACH preamble format as given by Tables 6.3.3.1-1 and 6.3.3.1-2.

## Sequence lengths are added to the PRACH formats

Table 6.3.3.1-2: Preamble formats for  and ΔfRA=15⋅2μ kHz where , and for LRA=1151 where μ=0 and for LRA=571 where μ=1

| Format |  |  |  |  | Support for restricted sets |
| --- | --- | --- | --- | --- | --- |
| A1 | 139, 571, 1151 |  |  |  | - |
| A2 | 139, 571, 1151 |  |  |  | - |
| A3 | 139, 571, 1151 |  |  |  | - |
| B1 | 139, 571, 1151 |  |  |  | - |
| B2 | 139, 571, 1151 |  |  |  | - |
| B3 | 139, 571, 1151 |  |  |  | - |
| B4 | 139, 571, 1151 |  |  |  | - |
| C0 | 139, 571, 1151 |  |  |  | - |
| C2 | 139, 571, 1151 |  |  |  |  |

## Tables of the logical root indices

Table 6.3.3.1-x. Mapping from logical index i to sequence number u for preamble formats with LRA=571.

| i | Sequence number u in increasing order of i |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0-19 | 1 | 570 | 2 | 569 | 3 | 568 | 4 | 567 | 5 | 566 | 6 | 565 | 7 | 564 | 8 | 563 | 9 | 562 | 10 | 561 |
| 20-39 | 11 | 560 | 12 | 559 | 13 | 558 | 14 | 557 | 15 | 556 | 16 | 555 | 17 | 554 | 18 | 553 | 19 | 552 | 20 | 551 |
| 40-59 | 21 | 550 | 22 | 549 | 23 | 548 | 24 | 547 | 25 | 546 | 26 | 545 | 27 | 544 | 28 | 543 | 29 | 542 | 30 | 541 |
| 60-79 | 31 | 540 | 32 | 539 | 33 | 538 | 34 | 537 | 35 | 536 | 36 | 535 | 37 | 534 | 38 | 533 | 39 | 532 | 40 | 531 |
| 80-99 | 41 | 530 | 42 | 529 | 43 | 528 | 44 | 527 | 45 | 526 | 46 | 525 | 47 | 524 | 48 | 523 | 49 | 522 | 50 | 521 |
| 100-119 | 51 | 520 | 52 | 519 | 53 | 518 | 54 | 517 | 55 | 516 | 56 | 515 | 57 | 514 | 58 | 513 | 59 | 512 | 60 | 511 |
| 120-139 | 61 | 510 | 62 | 509 | 63 | 508 | 64 | 507 | 65 | 506 | 66 | 505 | 67 | 504 | 68 | 503 | 69 | 502 | 70 | 501 |
| 140-159 | 71 | 500 | 72 | 499 | 73 | 498 | 74 | 497 | 75 | 496 | 76 | 495 | 77 | 494 | 78 | 493 | 79 | 492 | 80 | 491 |
| 160-179 | 81 | 490 | 82 | 489 | 83 | 488 | 84 | 487 | 85 | 486 | 86 | 485 | 87 | 484 | 88 | 483 | 89 | 482 | 90 | 481 |
| 180-199 | 91 | 480 | 92 | 479 | 93 | 478 | 94 | 477 | 95 | 476 | 96 | 475 | 97 | 474 | 98 | 473 | 99 | 472 | 100 | 471 |
| 200-219 | 101 | 470 | 102 | 469 | 103 | 468 | 104 | 467 | 105 | 466 | 106 | 465 | 107 | 464 | 108 | 463 | 109 | 462 | 110 | 461 |
| 220-239 | 111 | 460 | 112 | 459 | 113 | 458 | 114 | 457 | 115 | 456 | 116 | 455 | 117 | 454 | 118 | 453 | 119 | 452 | 120 | 451 |
| 240-259 | 121 | 450 | 122 | 449 | 123 | 448 | 124 | 447 | 125 | 446 | 126 | 445 | 127 | 444 | 128 | 443 | 129 | 442 | 130 | 441 |
| 260-279 | 131 | 440 | 132 | 439 | 133 | 438 | 134 | 437 | 135 | 436 | 136 | 435 | 137 | 434 | 138 | 433 | 139 | 432 | 140 | 431 |
| 280-299 | 141 | 430 | 142 | 429 | 143 | 428 | 144 | 427 | 145 | 426 | 146 | 425 | 147 | 424 | 148 | 423 | 149 | 422 | 150 | 421 |
| 300-319 | 151 | 420 | 152 | 419 | 153 | 418 | 154 | 417 | 155 | 416 | 156 | 415 | 157 | 414 | 158 | 413 | 159 | 412 | 160 | 411 |
| 320-339 | 161 | 410 | 162 | 409 | 163 | 408 | 164 | 407 | 165 | 406 | 166 | 405 | 167 | 404 | 168 | 403 | 169 | 402 | 170 | 401 |
| 340-359 | 171 | 400 | 172 | 399 | 173 | 398 | 174 | 397 | 175 | 396 | 176 | 395 | 177 | 394 | 178 | 393 | 179 | 392 | 180 | 391 |
| 360-379 | 181 | 390 | 182 | 389 | 183 | 388 | 184 | 387 | 185 | 386 | 186 | 385 | 187 | 384 | 188 | 383 | 189 | 382 | 190 | 381 |
| 380-399 | 191 | 380 | 192 | 379 | 193 | 378 | 194 | 377 | 195 | 376 | 196 | 375 | 197 | 374 | 198 | 373 | 199 | 372 | 200 | 371 |
| 400-419 | 201 | 370 | 202 | 369 | 203 | 368 | 204 | 367 | 205 | 366 | 206 | 365 | 207 | 364 | 208 | 363 | 209 | 362 | 210 | 361 |
| 420-439 | 211 | 360 | 212 | 359 | 213 | 358 | 214 | 357 | 215 | 356 | 216 | 355 | 217 | 354 | 218 | 353 | 219 | 352 | 220 | 351 |
| 440-459 | 221 | 350 | 222 | 349 | 223 | 348 | 224 | 347 | 225 | 346 | 226 | 345 | 227 | 344 | 228 | 343 | 229 | 342 | 230 | 341 |
| 460-479 | 231 | 340 | 232 | 339 | 233 | 338 | 234 | 337 | 235 | 336 | 236 | 335 | 237 | 334 | 238 | 333 | 239 | 332 | 240 | 331 |
| 480-499 | 241 | 330 | 242 | 329 | 243 | 328 | 244 | 327 | 245 | 326 | 246 | 325 | 247 | 324 | 248 | 323 | 249 | 322 | 250 | 321 |
| 500-519 | 251 | 320 | 252 | 319 | 253 | 318 | 254 | 317 | 255 | 316 | 256 | 315 | 257 | 314 | 258 | 313 | 259 | 312 | 260 | 311 |
| 520-539 | 261 | 310 | 262 | 309 | 263 | 308 | 264 | 307 | 265 | 306 | 266 | 305 | 267 | 304 | 268 | 303 | 269 | 302 | 270 | 301 |
| 540-559 | 271 | 300 | 272 | 299 | 273 | 298 | 274 | 297 | 275 | 296 | 276 | 295 | 277 | 294 | 278 | 293 | 279 | 292 | 280 | 291 |
| 560-569 | 281 | 290 | 282 | 289 | 283 | 288 | 284 | 287 | 285 | 286 | - | - | - | - | - | - | - | - | - | - |

Table 6.3.3.1-y. Mapping from logical index i to sequence number u for preamble formats with LRA=1151.

| i | Sequence number u in increasing order of i |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0-19 | 1 | 1150 | 2 | 1149 | 3 | 1148 | 4 | 1147 | 5 | 1146 | 6 | 1145 | 7 | 1144 | 8 | 1143 | 9 | 1142 | 10 | 1141 |
| 20-39 | 11 | 1140 | 12 | 1139 | 13 | 1138 | 14 | 1137 | 15 | 1136 | 16 | 1135 | 17 | 1134 | 18 | 1133 | 19 | 1132 | 20 | 1131 |
| 40-59 | 21 | 1130 | 22 | 1129 | 23 | 1128 | 24 | 1127 | 25 | 1126 | 26 | 1125 | 27 | 1124 | 28 | 1123 | 29 | 1122 | 30 | 1121 |
| 60-79 | 31 | 1120 | 32 | 1119 | 33 | 1118 | 34 | 1117 | 35 | 1116 | 36 | 1115 | 37 | 1114 | 38 | 1113 | 39 | 1112 | 40 | 1111 |
| 80-99 | 41 | 1110 | 42 | 1109 | 43 | 1108 | 44 | 1107 | 45 | 1106 | 46 | 1105 | 47 | 1104 | 48 | 1103 | 49 | 1102 | 50 | 1101 |
| 100-119 | 51 | 1100 | 52 | 1099 | 53 | 1098 | 54 | 1097 | 55 | 1096 | 56 | 1095 | 57 | 1094 | 58 | 1093 | 59 | 1092 | 60 | 1091 |
| 120-139 | 61 | 1090 | 62 | 1089 | 63 | 1088 | 64 | 1087 | 65 | 1086 | 66 | 1085 | 67 | 1084 | 68 | 1083 | 69 | 1082 | 70 | 1081 |
| 140-159 | 71 | 1080 | 72 | 1079 | 73 | 1078 | 74 | 1077 | 75 | 1076 | 76 | 1075 | 77 | 1074 | 78 | 1073 | 79 | 1072 | 80 | 1071 |
| 160-179 | 81 | 1070 | 82 | 1069 | 83 | 1068 | 84 | 1067 | 85 | 1066 | 86 | 1065 | 87 | 1064 | 88 | 1063 | 89 | 1062 | 90 | 1061 |
| 180-199 | 91 | 1060 | 92 | 1059 | 93 | 1058 | 94 | 1057 | 95 | 1056 | 96 | 1055 | 97 | 1054 | 98 | 1053 | 99 | 1052 | 100 | 1051 |
| 200-219 | 101 | 1050 | 102 | 1049 | 103 | 1048 | 104 | 1047 | 105 | 1046 | 106 | 1045 | 107 | 1044 | 108 | 1043 | 109 | 1042 | 110 | 1041 |
| 220-239 | 111 | 1040 | 112 | 1039 | 113 | 1038 | 114 | 1037 | 115 | 1036 | 116 | 1035 | 117 | 1034 | 118 | 1033 | 119 | 1032 | 120 | 1031 |
| 240-259 | 121 | 1030 | 122 | 1029 | 123 | 1028 | 124 | 1027 | 125 | 1026 | 126 | 1025 | 127 | 1024 | 128 | 1023 | 129 | 1022 | 130 | 1021 |
| 260-279 | 131 | 1020 | 132 | 1019 | 133 | 1018 | 134 | 1017 | 135 | 1016 | 136 | 1015 | 137 | 1014 | 138 | 1013 | 139 | 1012 | 140 | 1011 |
| 280-299 | 141 | 1010 | 142 | 1009 | 143 | 1008 | 144 | 1007 | 145 | 1006 | 146 | 1005 | 147 | 1004 | 148 | 1003 | 149 | 1002 | 150 | 1001 |
| 300-319 | 151 | 1000 | 152 | 999 | 153 | 998 | 154 | 997 | 155 | 996 | 156 | 995 | 157 | 994 | 158 | 993 | 159 | 992 | 160 | 991 |
| 320-339 | 161 | 990 | 162 | 989 | 163 | 988 | 164 | 987 | 165 | 986 | 166 | 985 | 167 | 984 | 168 | 983 | 169 | 982 | 170 | 981 |
| 340-359 | 171 | 980 | 172 | 979 | 173 | 978 | 174 | 977 | 175 | 976 | 176 | 975 | 177 | 974 | 178 | 973 | 179 | 972 | 180 | 971 |
| 360-379 | 181 | 970 | 182 | 969 | 183 | 968 | 184 | 967 | 185 | 966 | 186 | 965 | 187 | 964 | 188 | 963 | 189 | 962 | 190 | 961 |
| 380-399 | 191 | 960 | 192 | 959 | 193 | 958 | 194 | 957 | 195 | 956 | 196 | 955 | 197 | 954 | 198 | 953 | 199 | 952 | 200 | 951 |
| 400-419 | 201 | 950 | 202 | 949 | 203 | 948 | 204 | 947 | 205 | 946 | 206 | 945 | 207 | 944 | 208 | 943 | 209 | 942 | 210 | 941 |
| 420-439 | 211 | 940 | 212 | 939 | 213 | 938 | 214 | 937 | 215 | 936 | 216 | 935 | 217 | 934 | 218 | 933 | 219 | 932 | 220 | 931 |
| 440-459 | 221 | 930 | 222 | 929 | 223 | 928 | 224 | 927 | 225 | 926 | 226 | 925 | 227 | 924 | 228 | 923 | 229 | 922 | 230 | 921 |
| 460-479 | 231 | 920 | 232 | 919 | 233 | 918 | 234 | 917 | 235 | 916 | 236 | 915 | 237 | 914 | 238 | 913 | 239 | 912 | 240 | 911 |
| 480-499 | 241 | 910 | 242 | 909 | 243 | 908 | 244 | 907 | 245 | 906 | 246 | 905 | 247 | 904 | 248 | 903 | 249 | 902 | 250 | 901 |
| 500-519 | 251 | 900 | 252 | 899 | 253 | 898 | 254 | 897 | 255 | 896 | 256 | 895 | 257 | 894 | 258 | 893 | 259 | 892 | 260 | 891 |
| 520-539 | 261 | 890 | 262 | 889 | 263 | 888 | 264 | 887 | 265 | 886 | 266 | 885 | 267 | 884 | 268 | 883 | 269 | 882 | 270 | 881 |
| 540-559 | 271 | 880 | 272 | 879 | 273 | 878 | 274 | 877 | 275 | 876 | 276 | 875 | 277 | 874 | 278 | 873 | 279 | 872 | 280 | 871 |
| 560-579 | 281 | 870 | 282 | 869 | 283 | 868 | 284 | 867 | 285 | 866 | 286 | 865 | 287 | 864 | 288 | 863 | 289 | 862 | 290 | 861 |
| 580-599 | 291 | 860 | 292 | 859 | 293 | 858 | 294 | 857 | 295 | 856 | 296 | 855 | 297 | 854 | 298 | 853 | 299 | 852 | 300 | 851 |
| 600-619 | 301 | 850 | 302 | 849 | 303 | 848 | 304 | 847 | 305 | 846 | 306 | 845 | 307 | 844 | 308 | 843 | 309 | 842 | 310 | 841 |
| 620-639 | 311 | 840 | 312 | 839 | 313 | 838 | 314 | 837 | 315 | 836 | 316 | 835 | 317 | 834 | 318 | 833 | 319 | 832 | 320 | 831 |
| 640-659 | 321 | 830 | 322 | 829 | 323 | 828 | 324 | 827 | 325 | 826 | 326 | 825 | 327 | 824 | 328 | 823 | 329 | 822 | 330 | 821 |
| 660-679 | 331 | 820 | 332 | 819 | 333 | 818 | 334 | 817 | 335 | 816 | 336 | 815 | 337 | 814 | 338 | 813 | 339 | 812 | 340 | 811 |
| 680-699 | 341 | 810 | 342 | 809 | 343 | 808 | 344 | 807 | 345 | 806 | 346 | 805 | 347 | 804 | 348 | 803 | 349 | 802 | 350 | 801 |
| 700-719 | 351 | 800 | 352 | 799 | 353 | 798 | 354 | 797 | 355 | 796 | 356 | 795 | 357 | 794 | 358 | 793 | 359 | 792 | 360 | 791 |
| 720-739 | 361 | 790 | 362 | 789 | 363 | 788 | 364 | 787 | 365 | 786 | 366 | 785 | 367 | 784 | 368 | 783 | 369 | 782 | 370 | 781 |
| 740-759 | 371 | 780 | 372 | 779 | 373 | 778 | 374 | 777 | 375 | 776 | 376 | 775 | 377 | 774 | 378 | 773 | 379 | 772 | 380 | 771 |
| 760-779 | 381 | 770 | 382 | 769 | 383 | 768 | 384 | 767 | 385 | 766 | 386 | 765 | 387 | 764 | 388 | 763 | 389 | 762 | 390 | 761 |
| 780-799 | 391 | 760 | 392 | 759 | 393 | 758 | 394 | 757 | 395 | 756 | 396 | 755 | 397 | 754 | 398 | 753 | 399 | 752 | 400 | 751 |
| 800-819 | 401 | 750 | 402 | 749 | 403 | 748 | 404 | 747 | 405 | 746 | 406 | 745 | 407 | 744 | 408 | 743 | 409 | 742 | 410 | 741 |
| 820-839 | 411 | 740 | 412 | 739 | 413 | 738 | 414 | 737 | 415 | 736 | 416 | 735 | 417 | 734 | 418 | 733 | 419 | 732 | 420 | 731 |
| 840-859 | 421 | 730 | 422 | 729 | 423 | 728 | 424 | 727 | 425 | 726 | 426 | 725 | 427 | 724 | 428 | 723 | 429 | 722 | 430 | 721 |
| 860-879 | 431 | 720 | 432 | 719 | 433 | 718 | 434 | 717 | 435 | 716 | 436 | 715 | 437 | 714 | 438 | 713 | 439 | 712 | 440 | 711 |
| 880-899 | 441 | 710 | 442 | 709 | 443 | 708 | 444 | 707 | 445 | 706 | 446 | 705 | 447 | 704 | 448 | 703 | 449 | 702 | 450 | 701 |
| 900-919 | 451 | 700 | 452 | 699 | 453 | 698 | 454 | 697 | 455 | 696 | 456 | 695 | 457 | 694 | 458 | 693 | 459 | 692 | 460 | 691 |
| 920-939 | 461 | 690 | 462 | 689 | 463 | 688 | 464 | 687 | 465 | 686 | 466 | 685 | 467 | 684 | 468 | 683 | 469 | 682 | 470 | 681 |
| 940-959 | 471 | 680 | 472 | 679 | 473 | 678 | 474 | 677 | 475 | 676 | 476 | 675 | 477 | 674 | 478 | 673 | 479 | 672 | 480 | 671 |
| 960-979 | 481 | 670 | 482 | 669 | 483 | 668 | 484 | 667 | 485 | 666 | 486 | 665 | 487 | 664 | 488 | 663 | 489 | 662 | 490 | 661 |
| 980-999 | 491 | 660 | 492 | 659 | 493 | 658 | 494 | 657 | 495 | 656 | 496 | 655 | 497 | 654 | 498 | 653 | 499 | 652 | 500 | 651 |
| 1000-1019 | 501 | 650 | 502 | 649 | 503 | 648 | 504 | 647 | 505 | 646 | 506 | 645 | 507 | 644 | 508 | 643 | 509 | 642 | 510 | 641 |
| 1020-1039 | 511 | 640 | 512 | 639 | 513 | 638 | 514 | 637 | 515 | 636 | 516 | 635 | 517 | 634 | 518 | 633 | 519 | 632 | 520 | 631 |
| 1040-1059 | 521 | 630 | 522 | 629 | 523 | 628 | 524 | 627 | 525 | 626 | 526 | 625 | 527 | 624 | 528 | 623 | 529 | 622 | 530 | 621 |
| 1060-1079 | 531 | 620 | 532 | 619 | 533 | 618 | 534 | 617 | 535 | 616 | 536 | 615 | 537 | 614 | 538 | 613 | 539 | 612 | 540 | 611 |
| 1080-1099 | 541 | 610 | 542 | 609 | 543 | 608 | 544 | 607 | 545 | 606 | 546 | 605 | 547 | 604 | 548 | 603 | 549 | 602 | 550 | 601 |
| 1100-1119 | 551 | 600 | 552 | 599 | 553 | 598 | 554 | 597 | 555 | 596 | 556 | 595 | 557 | 594 | 558 | 593 | 559 | 592 | 560 | 591 |
| 1120-1139 | 561 | 590 | 562 | 589 | 563 | 588 | 564 | 587 | 565 | 586 | 566 | 585 | 567 | 584 | 568 | 583 | 569 | 582 | 570 | 581 |
| 1140-1149 | 571 | 580 | 572 | 579 | 573 | 578 | 574 | 577 | 575 | 576 | - | - | - | - | - | - | - | - | - | - |

## Table of cyclic shifts

Table 6.3.3.1-z: NCS for preamble formats with LRA=571 and LRA=1151

| zeroCorrelationZoneConfig | NCS value for LRA=571 | NCS value for LRA=1151 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 8 | 17 |
| 2 | 10 | 21 |
| 3 | 12 | 25 |
| 4 | 15 | 30 |
| 5 | 17 | 35 |
| 6 | 21 | 44 |
| 7 | 25 | 52 |
| 8 | 31 | 63 |
| 9 | 40 | 82 |
| 10 | 51 | 104 |
| 11 | 63 | 127 |
| 12 | 81 | 164 |
| 13 | 114 | 230 |
| 14 | 190 | 383 |
| 15 | 285 | 575 |

## Frequency position

Table 6.3.3.2-1: Supported combinations of  and , and the corresponding value of .

|  | for PRACH | for PUSCH | , allocation expressed in number of RBs for PUSCH |  |
| --- | --- | --- | --- | --- |
| 839 | 1.25 | 15 | 6 | 7 |
| 839 | 1.25 | 30 | 3 | 1 |
| 839 | 1.25 | 60 | 2 | 133 |
| 839 | 5 | 15 | 24 | 12 |
| 839 | 5 | 30 | 12 | 10 |
| 839 | 5 | 60 | 6 | 7 |
| 139 | 15 | 15 | 12 | 2 |
| 139 | 15 | 30 | 6 | 2 |
| 139 | 15 | 60 | 3 | 2 |
| 139 | 30 | 15 | 24 | 2 |
| 139 | 30 | 30 | 12 | 2 |
| 139 | 30 | 60 | 6 | 2 |
| 139 | 60 | 60 | 12 | 2 |
| 139 | 60 | 120 | 6 | 2 |
| 139 | 120 | 60 | 24 | 2 |
| 139 | 120 | 120 | 12 | 2 |
| 571 | 30 | 15 | 96 | 2 |
| 571 | 30 | 30 | 48 | 2 |
| 571 | 30 | 60 | 24 | 2 |
| 1151 | 15 | 15 | 96 | 1 |
| 1151 | 15 | 30 | 48 | 1 |
| 1151 | 15 | 60 | 24 | 1 |

# Specification changes for TS38.331

# RRC parameters

RACH-ConfigCommon information element

-- ASN1START

-- TAG-RACH-CONFIGCOMMON-START

RACH-ConfigCommon ::=               SEQUENCE {

rach-ConfigGeneric                  RACH-ConfigGeneric,

totalNumberOfRA-Preambles           INTEGER (1..63)                                                     OPTIONAL,   -- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                       OPTIONAL,   -- Need M

groupBconfigured                    SEQUENCE {

ra-Msg3SizeGroupA                   ENUMERATED {b56, b144, b208, b256, b282, b480, b640,

b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB            ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA          INTEGER (1..64)

}                                                                                                       OPTIONAL,   -- Need R

ra-ContentionResolutionTimer            ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64},

rsrp-ThresholdSSB                       RSRP-Range                                                      OPTIONAL,   -- Need R

rsrp-ThresholdSSB-SUL                   RSRP-Range                                                      OPTIONAL,   -- Cond SUL

prach-RootSequenceIndex                 CHOICE {

l839                                    INTEGER (0..837),

l139                                    INTEGER (0..137),

l571                                    INTEGER (0..569),

l1151                                   INTEGER (0..1149)

},

msg1-SubcarrierSpacing                  SubcarrierSpacing                                               OPTIONAL,   -- Cond L139

restrictedSetConfig                     ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB},

msg3-transformPrecoder                  ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

...

}

-- TAG-RACH-CONFIGCOMMON-STOP

-- ASN1STOP