3GPP TSG-RAN WG1 Meeting #99                                	R1-1912247

Reno, USA, 18th – 22nd November 2019

Source: 	Asia Pacific Telecom

Title: 	Discussion on uplink timing advance and RACH procedure for NTN

Agenda item:	7.2.5.3

Document for:	Discussion and Decision

# Introduction

In previous meetings, the following agreement has been made.

Agreement in RAN1#97 For UL frequency compensation at least in LEO systems: - Both open and closed loop can be studied - Beam specific post-compensation of common frequency offset at gNB can be considered - FFS: Further indication of common frequency offset - FFS: Signalling details - FFS: Compensation of common frequency offset at UE side - For Open-loop method: - Estimation of UE-specific frequency offset and pre-compensation at UE side can be conducted based on: - DL RSs - UE location and satellite ephemeris - FFS: Determination of UE location Agreement in RAN1#98 For UL frequency compensation at least in LEO systems, parameter(s) for frequency correction can be indicated by gNB to UE - FFS: Signaling details including whether signalling is broadcast or UE specific, and which parameter(s) are signalled. Agreement in RAN1#98-Bis If compensation of the frequency offset is conducted by the network in the uplink and/or the downlink respectively, indication of compensated frequency offset values by the network is beneficial.

In this contribution, we discuss possible implementation details on UL frequency control.

# Discussion

To summarize the UL frequency control in Rel-16 NTN SI, for UL frequency offset, NW can indicate some assisted information to UE, e.g., UL frequency correction or UL/DL frequency post-/pre-compensation at the NW side. For UE, it is possible to pre-compensate UL frequency by itself, irrespective of the existence of the assisted information (closed-loop) or not (open loop).

One issue is that it is not clear how NR procedures, e.g., the legacy RA and two-step RA, accommodate these new features mentioned in the above agreements. Implementation details are needed since the closed-loop solution and the open-loop solution may preclude each other.

## UL frequency control for legacy RA

For legacy RA, if UE has no GNSS capability and no satellite ephemeris, it is impossible to predict the Doppler shift. Based on the RAN1#98-Bis agreement, NW may indicate/broadcast a UL frequency offset to UEs that prevents UL interference. In this case, the frequency control may be implemented as follows.

- Step 1: UE may obtain the common UL frequency offset;

- Step 2: UE applies the common UL frequency offset for Msg1 transmission;

- Step 3: UL frequency correction is sent via Msg2 transmission;

- Step 4: UE applies the UL frequency correction and for Msg3 transmission;

- Step 5: NW has synchronized with UE.

The same concept is shown in Figure 1, where the notations can be also found in the Figure.

Figure 1: an example of UL frequency control for UE without GNSS for the legacy RA.

## UL frequency control for two-step RA

For 2-step RA, if UE has no GNSS and no satellite ephemeris, it may be implemented as follows.

- Step 1: UE may obtain the common UL frequency offset;

- Step 2: UE applies the common UL frequency offset for MsgA transmission;

- Step 3: UL frequency correction is sent to UE via MsgB transmission;

- Step 4: UE applies the UL frequency correction and frequency alignment has achieved;

Figure 2: an example of UL frequency control for UE without GNSS for the 2-step RA.

For UL frequency compensation, the frequency correction can be indicated NW, where the signalling including both broadcast and UE specific manners shall be supported.

# Conclusion

In this contribution, we have the following observations and proposals.

Proposal 1	For UL frequency compensation, the frequency correction can be indicated NW, where the signalling including both broadcast and UE specific manners shall be supported.

# Reference

Chairman's notes in RAN1#98-Bis, Chongqing, China, October 2019.

Chairman's notes in RAN1#98, Prague, Czech, August 2019.