3GPP TSG RAN WG1 #99	 R1-1912488

# Reno, USA, November 18th – 22nd, 2019

Agenda Item:		7.2.9.3

Source:				Samsung

Title:					UE adaptation to maximum number of MIMO layers

Document for:	Discussion/Decision

Introduction

In RAN1#98bis [1], the following were agreed:

Conclusion:

- No additional spec impact to support adaptation of UL Max #MIMO layers for UL NCB and CB MIMO in Rel-16 power saving WI

Agreements:

- The configured per-BWP DL max MIMO layer value is expected to be less than or equal to the per-cell configured DL Max MIMO layer value (if configured).

Agreements:

- DL-SCH TBSLBRM is invariant across all the DL BWPs, when DL max MIMO layer adaptation is per BWP

  - FFS details

Agreements:

- No spec change for TS38.212 is needed for determining DL-SCH TBSLBRM when downlink max MIMO layer adaptation is configured.

  - Note: DL-SCH TBSLBRM is not dependent on the per-BWP configured maximum number of DL MIMO layers value.

- When maximum number of DL MIMO layers per BWP is configured for all configured DL BWPs for a serving cell,

  - At least one BWP is configured with per-BWP configured maximum number of DL MIMO layers value equal to the per-cell configured maximum number of DL MIMO layers value (if configured).

This contribution considers remaining issues related to UE adaptation to maximum number of MIMO layers.

UE adaptation to maximum number of MIMO layers

Remaining issues

CSI mismatch

In the perspective of both CSI measurement or PDSCH reception the maximum number of DL MIMO layers (Lmax) may not induce the number of active antenna related elements. It means that there could be at least two possible ways for power saving based on dynamic adaptation of the maximum number of DL MIMO layers:

Alt 1) A fixed number of active antenna related elements is assumed per operating band like Rel-15 with dynamic adaptation of Lmax. In this case, fixed number of Rx antenna elements of a UE are always active during CSI measurement and decreasing Lmax only reduces the CSI computation complexity by limiting the maximum RI to Lmax.

Alt 2) The number of active antenna related elements varies with dynamic adaptation of Lmax, i.e., only the smallest number of antenna related elements are activated to receive the Lmax MIMO layers. In this case, decreasing Lmax not only reduces the CSI computation complexity but also reduces RF power consumption by turning off the deactivated antenna related elements.

It is important that the UE and the gNB have the same understanding on the aforementioned issue. If not, the following cases of CSI mismatch may occur:

Case 1) gNB assumes Alt 1) for CSI measurement/report while UE assumes Alt 2). In this case, the reported CQI may be lower than the actual channel quality report that the gNB supposed to receive. Also, PMI mismatch may happen because of the different understanding on the number of activated Rx antenna ports between the gNB and the UE, so that the corresponding precoder generated at the gNB may be different from the optimal one.

Case 2) gNB assumes Alt 2) for CSI measurement/report while UE assumes Alt 1). In this case, the reported CQI may be higher than the actual channel quality report that the gNB supposed to receive. PMI mismatch may also happen as in Case 1).

To avoid such CSI mismatch problem, it needs to be clarified that whether the dynamic adaptation of Lmax can imply the adaptation of the number of active antenna related elements or not. If yes, for gNB and UE to have the same understanding on antenna adaptation for CSI measurement, antenna adaptation related parameter needs to be incorporated into the CSI framework.

Proposal 1: Introduce an antenna adaptation related parameter into CSI framework to avoid CSI mismatch.

QCL mismatch

During initial access procedure, the UE measures SS/PBCH and refines its Rx beam for the corresponding to SS/PBCH. This Rx beam corresponding to the SS/PBCH identified during initial access can be used for QCL information to receive the other downlink physical channels (e.g., PDCCH, PDSCH, etc.). The UE can be configured with M TCI states and each TCI-State contains parameters for configuring a QCL relationship between the reference RSs (e.g., CSI-RS) and target RSs (e.g., DMRS for PDCCH or PDSCH, CSI-RS). The reference RSs can be associated with SS/PBCH with QCL-TypeD. If the UE is indicated with TCI state which refers SS/PBCH with QCL-TypeD for PDCCH or PDSCH, the UE can assume the DMRS of PDCCH or PDSCH is QCLed with SS/PBCH. For other cases, if explicit TCI indication is absent, the UE uses the Rx beam of the SS/PBCH identified during initial access. Regarding PDCCH reception, if a UE has not been provided with a configuration of TCI state for the CORESET or has received configuration of more than one TCI states but has not received an activation command for one of the TCI states, the UE assumes that the DMRS of the PDCCH is QCLed with the SS/PBCH block identified during the initial access. Regarding PDSCH, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE assumes that the DMRS of the PDSCH is QCLed with the SS/PBCH block determined in the initial access.

If antenna adaptation is performed at UE side, the Rx beam can be changed as shown in Figure 2. In the example illustrated in Figure 2, it is assumed that BWP#1 is configured with Lmax=2, BWP#2 is configured with Lmax=1, and the UE receives SS/PBCH and refines the Rx beam for the SS/PBCH using 4 antennas during initial access. For BWP#2, if the UE activates 4 antennas, there is no change in Rx beam. Therefore, the UE can receive the PDCCH/PDSCH which is QCLed with the SS/PBCH as usual. However, for BWP#1, if the UE adapts its activated antennas to 2, the Rx beam corresponding to the SS/PBCH can be changed. In other words, when a UE is indicated with TCI state associated with the SS/PBCH with QCL-TypeD, the Rx beam what the UE assumes in BWP#1 can be different from the Rx beam what the UE refines during initial access procedures. This kind of QCL mismatch problem is happen when the indicated TCI state is associated with SS/PBCH. If the CSI-RS is associated with SS/PBCH and the CSI-RS and SS/PBCH are transmitted in different BWP having different antenna assumption, the QCL mismatch problem can be occurred. For TCI state not associated with SS/PBCH, the QCL mismatch would not be the problem since CSI-RS is per-BWP configuration and the TCI state and the TCI state can refer the CSI-RS in the same BWP.

Observation 1: If UE’s antenna assumption for a SS/PBCH reception is different from that for a reference RS reception which associated with the SS/PBCH in QCL-TypeD, QCL mismatch problem can be happened.

Figure 2. Example of QCL mismatch

Such a QCL-TypeD mismatch problem is more critical when the number of antennas changes from small number to large number.  If a UE refines a Rx beam with large number of antennas, it is possible to re-tune it for the beam for small number of antennas since, in general, the beam obtained from large number of antennas has higher resolution than that from small number of antennas. On the other hand, it is difficult to re-tune the beam for small number of antennas to the beam for the large number of antennas. Therefore, to resolve the QCL-TypeD mismatch problem as described above, it would be desirable that UE measures and reports L1-RSRP for SS/PBCH with default antenna assumption based on its capability signaling (e.g., without any active antenna reductions). Then the Rx beam corresponding to a SS/PBCH can be re-used after some retuning for the Rx beam in the BWP where the UE uses smaller number of antennas.

If the number of antennas assumed for SS/PBCH measurement is different from that for the active BWP, measurement gap should be large enough to ensure additional time delay for antenna switching. As an example, if an active BWP not including SS/PBCH is configured with smaller Lmax , then measurement gap should be decided considering not only BWP switching delay but also antenna switching delay for intra-frequency measurement. In another example, if an active BWP including SS/PBCH is configured with smaller Lmax, then the UE still needs measurement gap to open up more antennas to measures the SS/PBCH.

Observation 2: QCL-TypeD mismatch problem is more critical for the case of small-to-large antenna adaptation than the case of large-to-small antenna adaptation.

Proposal 2: The UE performs measurement and report for the SS/PBCH with default antenna assumption based on its capability signaling.

Proposal 3: Measurement gap for SS/PBCH measurement should take into account antenna switching delay.

Conclusion

This contribution considered UE power saving technique based on UE adaptation to maximum number of MIMO. Following proposals and observations were made:

Proposal 1: Introduce an antenna adaptation related parameter into CSI framework to avoid CSI mismatch.

Proposal 2: The UE performs measurement and report for the SS/PBCH with default antenna assumption based on its capability signaling.

Proposal 3: Measurement gap for SS/PBCH measurement should take into account antenna switching delay.

Observation 1: If UE’s antenna assumption for a SS/PBCH reception is different from that for a reference RS reception which associated with the SS/PBCH in QCL-TypeD, QCL mismatch problem can be happened.

Observation 2: QCL-TypeD mismatch problem is more critical for the case of small-to-large antenna adaptation than the case of large-to-small antenna adaptation.

- References

- RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #98bis