3GPP TSG RAN WG1 #99		R1-1912901

Reno, USA, 18th – 22th November 2019

Source:	Panasonic

Title: 	System level evaluation and link budget for NTN

Agenda Item:		7.2.5.1

Document for:	Discussion

Introduction

A study item on Non Terrestrial Network (NTN) has been started. According to the SID[1], the following is to be studied in RAN1.

Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed  [RAN1]:

- Physical layer control procedures (e.g. CSI feedback, power control)

- Uplink Timing advance/RACH procedure including PRACH sequence/format/message

- Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1]

In RAN1#97, RAN1#98 and RAN1#98bis , system level evaluation assumptions were discussed and agreed. In this document, our evaluation results for calibration are provided. In addition, link budget analysis based on the system level evaluation results are shown.

Evaluation results for calibration

System level simulations (SLS) for calibration were conducted based on the propagation model in [2] and simulation assumptions agreed in RAN1#97, RAN1#98 and RAN1#98bis. Simulation assumptions are based on [3],  [4] and [5]. GEO, LEO 600km and LEO 1200km with parameter set-1 (smaller cell size) and set-2 (larger cell size) are evaluated. Two types of UE defined in [3] (i.e. VSAT for Ka-band and Handheld for S-band) are evaluated. Atmospheric absorption [2] are assumed for both S-band and Ka-band. Ionospheric scintillation loss is assumed to be zero, and tropospheric scintillation is assumed based on [2]. Frequency reuse option 1 (reuse 1), option 2 (reuse 3) and option 3 (reuse 2 + polarization reuse) as shown in Figure 1 are evaluated. For frequency reuse option 1, 19-beams and additional two tiers beams (i.e. 61-beams) are considered, and for frequency reuse option 2 and option 3, 19-beams and additional four tires beams (i.e. 127-beams) are considered. UEs are dropped in the only central 19-beams area. Beams with same frequency and polarization are treated as interference. For statistics, UEs in the central 19-beams are used for all frequency reuse options. Additionally, in RAN1#98, calibration study cases were listed and prioritized [4]. In this document, evaluation results for the listed cases are shown.

In this evaluation, we conducted uplink calibration in addition to downlink calibration. For uplink calibration, we use same beam layout and UE dropping as downlink calibration. The same frequency reuse options are applied. Uplink channel bandwidth for UE transmission is ,as used for link budget analysis, system bandwidth divided by frequency reuse factor for Ka-band, and 360kHz for S-band.

Downlink evaluation results of the coupling loss and geometry SINR distribution for each scenario are shown from Figure 2 to Figure 7. Figure 2, Figure 3 and Figure 4 show results of satellite parameter set-1for GEO, LEO-600km and LEO-1200km, respectively. Figure 5, Figure 6 and Figure 7 show the results of set-2. In each figure, upper side graph is Ka-band result, and lower side graph is S-band result. In the Ka-band graph, geometry SINRs for frequency option 1, 2 and 3 are shown, on the other hand, in the S-band graph, geometry SINRs for frequency option 1 and 2 are shown.

Uplink evaluation results are shown from Figure 8 to Figure 13. Figure 8, Figure 9 and Figure 10 show uplink calibration results of satellite antenna parameter set-1 for GEO, LEO-600km and LEO-1200km, respectively. Figure 11, Figure 12 and Figure 13 show results of satellite antenna set-2. For each figure, coupling loss and geometry SINR are shown as in downlink graphs.

Additionally, evaluation results for downlink and uplink are provided in the attached excel formats.

Option 1                                           Option 2                                               Option 3

Figure 1  frequency reuse options

Considerations

Downlink,

- For frequency reuse option 1 (i.e. reuse factor 1), lower limits of SINR for GEO are -5dB for Set-1(-7dB for Set-2), and those of LEO case are -4dB for Set-1(-6dB for Set-2). And, upper limits of SINR for both GEO and LEO are about 2dB. Therefore, SINR distribution range of NTN seems to be smaller than that of the terrestrial network.

- To apply frequency reuse option 2 (i.e. reuse factor 3), we can see over 5dB improvement in 50%-tile SINR compared to the option 1. Moreover, frequency reuse option 3 (i.e. reuse factor 2 + polarization reuse) has about 1dB gain from the option 2 in terms of SINR improvement.

- For frequency reuse option 2 and 3, SINR distribution range is smaller than that of option 1. The SINR ranges of option 2 and 3 in Set-1 are about 3dB. It is equivalent to the beam area which is defined by 3dB lower than the beam centre level.

Uplink,

- Almost same coupling loss is observed for downlink and uplink in the same evaluation case although downlink and uplink carrier frequencies are different.

- SINR distribution range for uplink is larger than that of downlink, especially in Ka-band.

Proposal 1:   Capture the calibration results in TR38.821.

DL calibration results

< GEO, Satellite antenna parameter Set-1 (Study Case 1-5) >

Ka-band (Study Case 1, 2 and 3)

S-band (Study Case 4 and 5)

Figure 2   DL Calibration results for GEO, Satellite antenna Set-1

< LEO-600km, Satellite antenna parameter Set-1 (Study Case 6-10) >

Ka-band (Study Case 6, 7 and 8)

S-band (Study Case 9 and 10)

Figure 3   DL Calibration results for LEO-600km, Satellite antenna Set-1

< LEO-1200km, Satellite antenna parameter Set-1 (Study Case 11-15) >

Ka-band (Study Case 11, 12 and 13)

S-band (Study Case 14 and 15)

Figure 4   DL Calibration results for LEO-1200km, Satellite antenna Set-1

< GEO, Satellite antenna parameter Set-2 (Study Case 16-20) >

Ka-band (Study Case 16, 17 and 18)

S-band (Study Case 19 and 20)

Figure 5   DL Calibration results for GEO, Satellite antenna Set-2

< LEO-600lm, Satellite antenna parameter Set-2 (Study Case 21-25) >

Ka-band (Study Case 21, 22 and 23)

S-band (Study Case 24 and 25)

Figure 6   DL Calibration results for LEO-600km, Satellite antenna Set-2

< LEO-1200km, Satellite antenna parameter Set-2 (Study Case 26-30) >

Ka-band (Study Case 26, 27 and 28)

S-band (Study Case 29 and 30)

Figure 7   DL Calibration results for LEO-1200km, Satellite antenna Set-2

UL calibration results

<GEO, Satellite antenna parameter Set-1 (Study Case 1-5) >

Ka-band (Study Case 1, 2 and 3)

S-band (Study Case 4 and 5)

Figure 8   UL calibration results for GEO, Satellite antenna Set-1

<LEO-600km, Satellite antenna parameter Set-1 (Study Case 6-10) >

Ka-band (Study Case 6, 7 and 8)

S-band (Study Case 9 and 10)

Figure 9   UL calibration results for LEO-600km, Satellite antenna Set-1

<LEO-1200km, Satellite antenna parameter Set-1 (Study Case 11-15) >

Ka-band (Study Case 11, 12 and 13)

S-band (Study Case 14 and 15)

Figure 10   UL calibration results for LEO-1200km, Satellite antenna Set-1

<GEO, Satellite antenna parameter Set-2 (Study Case 16-20) >

Ka-band (Study Case 16, 17 and 18)

S-band (Study Case 19 and 20)

Figure 11   UL calibration results for GEO, Satellite antenna Set-2

<LEO-600km, Satellite antenna parameter Set-2 (Study Case 21-25) >

Ka-band (Study Case 21, 22 and 23)

S-band (Study Case 24 and 25)

Figure 12   UL calibration results for LEO-600km, Satellite antenna Set-2

<LEO-1200km, Satellite antenna parameter Set-2 (Study Case 26-30) >

Ka-band (Study Case 26, 27 and 28)

S-band (Study Case 29 and 30)

Figure 13   UL calibration results for LEO-1200km, Satellite antenna Set-2

Link budget analysis

Link budget is calculated based on the agreed condition in [4], [5] and [6]. The updated points from our previous results[7] are the elevation angle for GEO and the satellite antenna parameters for uplink in Ka-band. The CIR values are obtained by system level evaluation using the assumptions agreed in RAN1#98bis. For the calculation of averaged CIR for uplink, we used log-scale averaging (i.e. averaging in dB).

Link budget analysis results are shown from Table 1 to Table 4. Table 1 and Table 2 show downlink link budget results for the satellite parameter set-1 and set-2, respectively. Table 3 and Table 4 show uplink link budget results for the satellite parameter set-1 and set-2, respectively.

Link budget results are also provided in the attached excel format.

Proposal 2:   Capture the link budget analysis results in TR38.821.

Table 1 Link budget for DL Set 1, GEO, LEO1200km, LEO600km, frequency reuse option 1, 2 and 3

| Parameters | Unit | Assumptions / Values |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Terminal |  | VSAT(Ka-band) |  |  | handheld(S-band) |  |  |
| Satellite altitude | km | 35786 | 600 | 1200 | 35786 | 600 | 1200 |
| Frequency | GHz | 20 | 20 | 20 | 2 | 2 | 2 |
| Frequency reuse Option 1 (reuse factor 1) |  |  |  |  |  |  |  |
| Frequency reuse |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Bandwidth | MHz | 400 | 400 | 400 | 30 | 30 | 30 |
| Frequency reuse Option 2 (reuse factor 3) |  |  |  |  |  |  |  |
| Frequency reuse |  | 3 | 3 | 3 | 3 | 3 | 3 |
| Bandwidth | MHz | 133.3 | 133.3 | 133.3 | 10 | 10 | 10 |
| Frequency reuse Option 3 (reuse factor 2 + polarization 2) |  |  |  |  |  |  |  |
| Frequency reuse |  | 2 | 2 | 2 | - | - | - |
| Bandwidth | MHz | 200 | 200 | 200 | - | - | - |
| Elevation angle | degree | 12.5 | 30 | 30 | 12.5 | 30 | 30 |
| Maximum distance | km | 40317 | 1075.1 | 1998.9 | 40317 | 1075.1 | 1998.9 |
| Propagation |  |  |  |  |  |  |  |
| Free space path loss | dB | 210.6 | 179.1 | 184.5 | 190.6 | 159.1 | 164.5 |
| Atmospheric path loss | dB | 1.16 | 0.5 | 0.5 | 0.2 | 0.07 | 0.07 |
| Scintillation loss | dB | 1.08 | 0.3 | 0.3 | 2.2 | 2.2 | 2.2 |
| Shadowing margin | dB | 0 | 0 | 0 | 3 | 3 | 3 |
| Additional loss | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Receiver |  |  |  |  |  |  |  |
| Tx power | dBW | - | - | - | - | - | - |
| Cable loss | dB | - | - | - | - | - | - |
| antenna gain | dBi | 58.5 | 38.5 | 38.5 | 51 | 30 | 30 |
| EIRP density | dBW/MHz | 40 | 4 | 10 | 59 | 34 | 40 |
| Frequency reuse Option 1 (reuse factor 1) |  |  |  |  |  |  |  |
| EIRP | dBW | 66.0 | 30.0 | 36.0 | 73.8 | 48.8 | 54.8 |
| EIRP | dBm | 96.0 | 60.0 | 66.0 | 103.8 | 78.8 | 84.8 |
| Frequency reuse Option 2 (reuse factor 3) |  |  |  |  |  |  |  |
| EIRP | dBW | 61.2 | 25.2 | 31.2 | 69.0 | 44.0 | 50.0 |
| EIRP | dBm | 91.2 | 55.2 | 61.2 | 99.0 | 74.0 | 80.0 |
| Frequency reuse Option 3 (reuse factor 2 + polarization 2) |  |  |  |  |  |  |  |
| EIRP | dBW | 63.0 | 27.0 | 33.0 | - | - | - |
| EIRP | dBm | 93.0 | 57.0 | 63.0 | - | - | - |
| Transmitter |  |  |  |  |  |  |  |
| antenna gain | dBi | 39.7 | 39.7 | 39.7 | 0 | 0 | 0 |
| number of element |  | 1 | 1 | 1 | 2 | 2 | 2 |
| polarization loss | dB | 0 | 0 | 0 | 3 | 3 | 3 |
| Noise Figure | dB | 1.2 | 1.2 | 1.2 | 7 | 7 | 7 |
| ambient temprature(T0) | K | 290 | 290 | 290 | 290 | 290 | 290 |
| Antenna temperature(Ta) | K | 150 | 150 | 150 | 290 | 290 | 290 |
| G/T | dB/K | 15.9 | 15.9 | 15.9 | -31.6 | -31.6 | -31.6 |
| Output |  |  |  |  |  |  |  |
| CNR (based on calculation) | dB | 11.6 | 8.6 | 9.2 | 0.0 | 6.6 | 7.2 |
| Frequency reuse Option 1 (reuse factor 1) |  |  |  |  |  |  |  |
| Study case |  | Case 1 | Case 6 | Case 11 | Case 4 | Case 9 | Case 14 |
| CIR (based on geometry) | dB | 1.76 | 1.74 | 1.30 | 5.72 | 1.79 | 1.36 |
| CNIR | dB | 1.36 | 0.92 | 0.64 | -1.03 | 0.55 | 0.36 |
| Frequency reuse Option 2 (reuse factor 3) |  |  |  |  |  |  |  |
| Study case |  | Case 2 | Case 7 | Case 12 | Case 5 | Case 10 | Case 15 |
| CIR (based on geometry) | dB | 13.83 | 12.19 | 11.56 | 13.85 | 12.26 | 11.72 |
| CNIR | dB | 9.59 | 6.99 | 7.19 | -0.17 | 5.57 | 5.91 |
| Frequency reuse Option 3 (reuse factor 2 + polarization 2) |  |  |  |  |  |  |  |
| Study case |  | Case 3 | Case 8 | Case 13 | - | - | - |
| CIR (based on geometry) | dB | 13.07 | 13.59 | 13.29 | - | - | - |
| CNIR | dB | 9.28 | 7.37 | 7.75 | - | - | - |

Table 2 Link budget for DL Set 2, GEO, LEO1200km, LEO600km, frequency reuse option 1, 2 and 3

| Parameters | Unit | Assumptions / Values |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Terminal |  | VSAT(Ka-band) |  |  | handheld(S-band) |  |  |
| Satellite altitude | km | 35786 | 600 | 1200 | 35786 | 600 | 1200 |
| Frequency | GHz | 20 | 20 | 20 | 2 | 2 | 2 |
| Frequency reuse Option 1 (reuse factor 1) |  |  |  |  |  |  |  |
| Frequency reuse |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Bandwidth | MHz | 400 | 400 | 400 | 30 | 30 | 30 |
| Frequency reuse Option 2 (reuse factor 3) |  |  |  |  |  |  |  |
| Frequency reuse |  | 3 | 3 | 3 | 3 | 3 | 3 |
| Bandwidth | MHz | 133.3 | 133.3 | 133.3 | 10 | 10 | 10 |
| Frequency reuse Option 3 (reuse factor 2 + polarization 2) |  |  |  |  |  |  |  |
| Frequency reuse |  | 2 | 2 | 2 | - | - | - |
| Bandwidth | MHz | 200 | 200 | 200 | - | - | - |
| Elevation angle | degree | 20 | 30 | 30 | 20 | 30 | 30 |
| Maximum distance | km | 39551 | 1075.1 | 1998.9 | 39551 | 1075.1 | 1998.9 |
| Propagation |  |  |  |  |  |  |  |
| Free space path loss | dB | 210.4 | 179.1 | 184.5 | 190.4 | 159.1 | 164.5 |
| atmospheric path loss | dB | 0.73 | 0.5 | 0.5 | 0.2 | 0.07 | 0.07 |
| Scintillation loss | dB | 0.48 | 0.3 | 0.3 | 2.2 | 2.2 | 2.2 |
| shadowing margin | dB | 0 | 0 | 0 | 3 | 3 | 3 |
| additional loss | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Transmitter |  |  |  |  |  |  |  |
| Tx power | dBW | - | - | - | - | - | - |
| Cable loss | dB | - | - | - | - | - | - |
| antenna gain | dBi | 50.5 | 30.5 | 30.5 | 45.5 | 24 | 24 |
| EIRP density | dBW/MHz | 32 | -4 | 2 | 53.5 | 28 | 34 |
| Frequency reuse Option 1 (reuse factor 1) |  |  |  |  |  |  |  |
| EIRP | dBW | 58.0 | 22.0 | 28.0 | 68.3 | 42.8 | 48.8 |
| EIRP dBm | dBm | 88.0 | 52.0 | 58.0 | 98.3 | 72.8 | 78.8 |
| Frequency reuse Option 2 (reuse factor 3) |  |  |  |  |  |  |  |
| EIRP | dBW | 53.2 | 17.2 | 23.2 | 63.5 | 38.0 | 44.0 |
| EIRP dBm | dBm | 83.2 | 47.2 | 53.2 | 93.5 | 68.0 | 74.0 |
| Frequency reuse Option 3 (reuse factor 2 + polarization 2) |  |  |  |  |  |  |  |
| EIRP | dBW | 55.0 | 19.0 | 25.0 | - | - | - |
| EIRP dBm | dBm | 85.0 | 49.0 | 55.0 | - | - | - |
| Receiver |  |  |  |  |  |  |  |
| antenna gain | dBi | 39.7 | 39.7 | 39.7 | 0 | 0 | 0 |
| number of element |  | 1 | 1 | 1 | 2 | 2 | 2 |
| polarization loss | dB | 0 | 0 | 0 | 3 | 3 | 3 |
| Noise Figure | dB | 1.2 | 1.2 | 1.2 | 7 | 7 | 7 |
| ambient temprature(T0) | K | 290 | 290 | 290 | 290 | 290 | 290 |
| Antenna temperature(Ta) | K | 150 | 150 | 150 | 290 | 290 | 290 |
| G/T | dB/K | 15.9 | 15.9 | 15.9 | -31.6 | -31.6 | -31.6 |
| Output |  |  |  |  |  |  |  |
| CNR (based on calculation) | dB | 4.8 | 0.6 | 1.2 | -5.3 | 0.6 | 1.2 |
| Frequency reuse Option 1 (reuse factor 1) |  |  |  |  |  |  |  |
| Study case |  | Case 16 | Case 21 | Case 26 | Case 19 | Case 24 | Case 29 |
| CIR (based on geometry) | dB | 1.62 | 1.76 | 1.35 | 1.55 | 2.53 | 2.67 |
| CNIR | dB | -0.07 | -1.89 | -1.75 | -6.14 | -1.54 | -1.12 |
| Frequency reuse Option 2 (reuse factor 3) |  |  |  |  |  |  |  |
| Study case |  | Case 17 | Case 22 | Case 27 | Case 20 | Case 25 | Case 30 |
| CIR (based on geometry) | dB | 13.86 | 12.24 | 11.86 | 13.87 | 14.12 | 13.88 |
| CNIR | dB | 4.32 | 0.27 | 0.81 | -5.38 | 0.43 | 1.00 |
| Frequency reuse Option 3 (reuse factor 2 + polarization 2) |  |  |  |  |  |  |  |
| Study case |  | Case 18 | Case 23 | Case 28 | - | - | - |
| CIR (based on geometry) | dB | 12.98 | 13.88 | 13.87 | - | - | - |
| CNIR | dB | 4.21 | 0.36 | 0.94 | - | - | - |

Table 3 Link budget for UL Set 1, GEO, LEO1200km, LEO600km, frequency reuse option 1, 2 and 3

| Parameters | Unit | Assumptions / Values |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Terminal |  | VSAT(Ka-band) |  |  | handheld(S-band) |  |  |
| Satellite altitude | km | 35786 | 600 | 1200 | 35786 | 600 | 1200 |
| Frequency | GHz | 30 | 30 | 30 | 2 | 2 | 2 |
| Frequency reuse Option 1 (reuse factor 1) |  |  |  |  |  |  |  |
| Frequency reuse |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Bandwidth | MHz | 400 | 400 | 400 | 0.36 | 0.36 | 0.36 |
| Frequency reuse Option 2 (reuse factor 3) |  |  |  |  |  |  |  |
| Frequency reuse |  | 3 | 3 | 3 | 3 | 3 | 3 |
| Bandwidth | MHz | 133.3 | 133.3 | 133.3 | 0.36 | 0.36 | 0.36 |
| Frequency reuse Option 3 (reuse factor 2 + polarization 2) |  |  |  |  |  |  |  |
| Frequency reuse |  | 2 | 2 | 2 | - | - | - |
| Bandwidth | MHz | 200 | 200 | 200 | - | - | - |
| Elevation angle | degree | 12.5 | 30 | 30 | 12.5 | 30 | 30 |
| Maximum distance | km | 40317 | 1075.1 | 1998.9 | 40317 | 1075.1 | 1998.9 |
| Propagation |  |  |  |  |  |  |  |
| Free space path loss | dB | 214.1 | 182.6 | 188.0 | 190.6 | 159.1 | 164.5 |
| atmospheric path loss | dB | 1.16 | 0.5 | 0.5 | 0.2 | 0.07 | 0.07 |
| scintillation loss | dB | 1.08 | 0.3 | 0.3 | 2.2 | 2.2 | 2.2 |
| shadowing margin | dB | 0 | 0 | 0 | 3 | 3 | 3 |
| additional loss | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Transmitter |  |  |  |  |  |  |  |
| Tx power | dBm | 33 | 33 | 33 | 23 | 23 | 23 |
| Cable loss | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| antenna gain | dBi | 43.2 | 43.2 | 43.2 | 0 | 0 | 0 |
| number of element |  | 1 | 1 | 1 | 1 | 1 | 1 |
| EIRP density | dBW/MHz | - | - | - | - | - | - |
| EIRP | dBW | 46.2 | 46.2 | 46.2 | -7.0 | -7.0 | -7.0 |
| EIRP | dBm | 76.2 | 76.2 | 76.2 | 23.0 | 23.0 | 23.0 |
| Receiver |  |  |  |  |  |  |  |
| antenna gain | dBi | 51 | 30 | 30 | 51 | 30 | 30 |
| polarization loss | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Noise Figure | dB | - | - | - | - | - | - |
| ambient temprature(T0) | K | - | - | - | - | - | - |
| Antenna temperature(Ta) | K | - | - | - | - | - | - |
| G/T | dB/K | 28.0 | 13.0 | 13.0 | 19.0 | 1.1 | 1.1 |
| Output |  |  |  |  |  |  |  |
| Frequency reuse Option 1 (reuse factor 1) |  |  |  |  |  |  |  |
| Study case |  | Case 1 | Case 6 | Case 11 | Case 4 | Case 9 | Case 14 |
| CNR (based on calculation) | dB | 0.4 | 18.4 | 13.0 | -10.9 | 2.8 | -2.6 |
| CIR (based on geometry) | dB | 1.3 | 1.91 | 1.37 | 4.86 | 2.15 | 1.73 |
| CNIR | dB | -2.16 | 1.81 | 1.08 | -11.06 | -0.56 | -3.93 |
| Frequency reuse Option 2 (reuse factor 3) |  |  |  |  |  |  |  |
| Study case |  | Case 2 | Case 7 | Case 12 | Case 5 | Case 10 | Case 15 |
| CNR (based on calculation) | dB | 5.2 | 23.1 | 17.7 | -10.9 | 2.8 | -2.6 |
| CIR (based on geometry) | dB | 12.76 | 11.92 | 11.34 | 13.53 | 12.02 | 11.72 |
| CNIR | dB | 4.51 | 11.6 | 10.44 | -10.96 | 2.28 | -2.78 |
| Frequency reuse Option 3 (reuse factor 2 + polarization 2) |  |  |  |  |  |  |  |
| Study case |  | Case 3 | Case 8 | Case 13 | - | - | - |
| CNR (based on calculation) | dB | 3.4 | 21.4 | 16.0 | - | - | - |
| CIR (based on geometry) | dB | 12.24 | 13.26 | 13.02 | - | - | - |
| CNIR | dB | 2.91 | 12.64 | 11.24 | - | - | - |

Table 4 Link budget for UL Set 2, GEO, LEO1200km, LEO600km, frequency reuse option 1, 2 and 3

| Parameters | Unit | Assumptions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Terminal |  | VSAT(Ka-band) |  |  | handheld(S-band) |  |  |
| Satellite altitude | km | 35786 | 600 | 1200 | 35786 | 600 | 1200 |
| Frequency | GHz | 30 | 30 | 30 | 2 | 2 | 2 |
| Frequency reuse Option 1 (reuse factor 1) |  |  |  |  |  |  |  |
| Frequency reuse |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Bandwidth | MHz | 400 | 400 | 400 | 0.36 | 0.36 | 0.36 |
| Frequency reuse Option 2 (reuse factor 3) |  |  |  |  |  |  |  |
| Frequency reuse |  | 3 | 3 | 3 | 3 | 3 | 3 |
| Bandwidth | MHz | 133.3 | 133.3 | 133.3 | 0.36 | 0.36 | 0.36 |
| Frequency reuse Option 3 (reuse factor 2 + polarization 2) |  |  |  |  |  |  |  |
| Frequency reuse |  | 2 | 2 | 2 | - | - | - |
| Bandwidth | MHz | 200 | 200 | 200 | - | - | - |
| Elevation angle | degree | 20 | 30 | 30 | 20 | 30 | 30 |
| Maximum distance | km | 39551 | 1075.1 | 1998.9 | 39551 | 1075.1 | 1998.9 |
| Propagation |  |  |  |  |  |  |  |
| Free space path loss | dB | 213.9 | 182.6 | 188.0 | 190.4 | 159.1 | 164.5 |
| atmospheric path loss | dB | 0.73 | 0.5 | 0.5 | 0.2 | 0.07 | 0.07 |
| Scintillation loss | dB | 0.48 | 0.3 | 0.3 | 2.2 | 2.2 | 2.2 |
| shadowing margin | dB | 0 | 0 | 0 | 3 | 3 | 3 |
| additional loss | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Transmitter |  |  |  |  |  |  |  |
| Tx power | dBm | 33 | 33 | 33 | 23 | 23 | 23 |
| Cable loss | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| antenna gain | dBi | 43.2 | 43.2 | 43.2 | 0 | 0 | 0 |
| number of element |  | 1 | 1 | 1 | 1 | 1 | 1 |
| EIRP density | dBW/MHz | - | - | - | - | - | - |
| EIRP | dBW | 46.2 | 46.2 | 46.2 | -7.0 | -7.0 | -7.0 |
| EIRP dBm | dBm | 76.2 | 76.2 | 76.2 | 23.0 | 23.0 | 23.0 |
| Receiver |  |  |  |  |  |  |  |
| antenna gain | dBi | 54 | 34 | 34 | 45.5 | 24 | 24 |
| polarization loss | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| Noise Figure | dB | - | - | - | - | - | - |
| ambient temprature(T0) | K | - | - | - | - | - | - |
| Antenna temperature(Ta) | K | - | - | - | - | - | - |
| G/T | dB/K | 20.0 | 5.0 | 5.0 | 14.0 | -4.9 | -4.9 |
| Output |  |  |  |  |  |  |  |
| Frequency reuse Option 1 (reuse factor 1) |  |  |  |  |  |  |  |
| Study case |  | Case 16 | Case 21 | Case 26 | Case 19 | Case 24 | Case 29 |
| CNR (based on calculation) | dB | -6.4 | 10.4 | 5.0 | -15.8 | -3.2 | -8.6 |
| CIR (based on geometry) | dB | 1.40 | 1.59 | 1.17 | 1.52 | 2.21 | 2.21 |
| CNIR | dB | -7.04 | 1.05 | 0.34 | -15.86 | -4.32 | -8.96 |
| Frequency reuse Option 2 (reuse factor 3) |  |  |  |  |  |  |  |
| Study case |  | Case 17 | Case 22 | Case 27 | Case 20 | Case 25 | Case 30 |
| CNR (based on calculation) | dB | -1.6 | 15.1 | 9.7 | -15.8 | -3.2 | -8.6 |
| CIR (based on geometry) | dB | 13.23 | 11.53 | 11.27 | 13.58 | 11.74 | 11.72 |
| CNIR | dB | -1.74 | 9.96 | 7.43 | -15.78 | -3.37 | -8.66 |
| Frequency reuse Option 3 (reuse factor 2 + polarization 2) |  |  |  |  |  |  |  |
| Study case |  | Case 18 | Case 23 | Case 28 | - | - | - |
| CNR (based on calculation) | dB | -3.4 | 13.4 | 8.0 | - | - | - |
| CIR (based on geometry) | dB | 12.46 | 13.1 | 13.09 | - | - | - |
| CNIR | dB | -3.47 | 10.22 | 6.81 | - | - | - |

Conclusion

System level evaluation results of both DL and UL for calibration are provided in this contribution. Link budget calculation results are also provided. The evaluation results for calibration and link budget for all cases are included in the attached spread sheets.

Proposal 1:   Capture the SLS calibration results for DL and UL in TR38.821.

Proposal 2:   Capture the link budget analysis for DL and UL in TR38.821.

- Reference

- RP-190710, “Study on solutions for NR to support non-terrestrial networks (NTN)”, Thales

- TR38.811 V15.0.0, “Study on New Radio (NR) to support non terrestrial networks”

- R1-1907847, “Chairman's notes of AI 7.2.5 Study on solutions for NR to support Non-Terrestrial Network (NTN)”

- R1-1909757, “Chairman’s notes of AI 7.2.5 Study on solutions for NR to support Non-Terrestrial Network (NTN)”

- R1-1911510, “Chairman’s notes of AI 7.2.5 Study on solutions for NR to support Non-Terrestrial Network (NTN)”

- R1-1907836, “TP for Link Budget Analysis in TR 38.821”, Nokia, Nokia Shanghai Bell, Thales, MediaTek, Ericsson, ZTE

- R1-1911002, “System level evaluation and link budget for NTN”, Panasonic