3GPP TSG RAN WG1 #99			R1-1913064

Reno, USA, November 18th – 22nd, 2019

Source:	WILUS Inc.

Title:	Channel access procedure for NR-U

Agenda item:	7.2.2.2.1

Document for:	Discussion/Decision

- Introduction

From the RAN1#96 to RAN1#98bis meeting, there was a discussion on channel access mechanisms for DL/UL channels and signals and the followings were agreed [1]-[5].

Agreement at RAN1#96: For initiation of a gNB transmission: - LBT other than Cat 4 is not used for DRS multiplexed with unicast data - LBT other than Cat 4 is not used for PDCCH and/or PDSCH transmission outside of DRS. Note: This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths Agreement at RAN1#96bis: For LBT by a UE prior to transmission of a UL burst within a gNB-initiated channel occupancy as LBE device, for gap durations shorter than 25 microseconds, choose one of the following alternatives - Alt 1: Cat 2 LBT can be indicated (FFS: explicit and/or implicit) to the UE if the gap is 16 microseconds (allowing for implementation tolerances) - Alt 2: Cat 2 LBT is not indicated to the UE for gaps less than 25 microseconds - Notes (applicable to both alternatives): - This means that the gNB ensures that gaps between 16 and 25 microseconds do not occur - This doesn’t change the previous agreement for Cat 1 and Cat 2 LBT for gaps of 16 microseconds or less - FFS: Conditions on channel occupancy after a Cat. 1 or Cat. 2 LBT after a gap of 16 microseconds or less Agreement at RAN1#96bis: The following agreement from the SI is updated as shown: Table 7.2.1.3.1-1: Channel access schemes for initiating a COT by gNB as LBE device Cat 2 LBT; Cat 4 LBT. DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR); When the DRS duty cycle ≤1/20, and the total duration is up to 1 ms: 25 µs Cat 2 LBT is used (as in LAA); When DRS duty cycle is > 1/20, or total duration > 1 ms Cat4 with any channel access priority class value can be used. DRS multiplexed with unicast data; N/A except for the cases discussed in the Note below; Channel access priority class is selected according to the multiplexed data. PDCCH and PDSCH; N/A except for the cases discussed in the Note below; Channel access priority class is selected according to the multiplexed data. Agreement at RAN1#96bis: For a UCI-only transmission on PUSCH in a channel occupancy initiated by the UE, Cat4 with lowest channel access priority class value can be used by the UE. Agreement at RAN1#97: For LBT by a UE prior to transmission of a UL burst within a gNB-initiated channel occupancy as an LBE device, for gap durations shorter than 25 microseconds, Cat 2 LBT can be indicated (FFS: explicit and/or implicit) to the UE if the gap is 16 microseconds (allowing for implementation tolerances) Note: this is the Alt 1 identified in RAN1#96bis Agreement at RAN1#97: Select one of the following alternatives for Cat2 LBT in a 16 us gap. - Alt 1: The 16us measurement period is split into two slots with the first slot having a duration 7us and second slot having a duration of 9us. - Energy measurement is done in the 9us slot with the measurement including averaging for at least 4 us in any portion of the slot. LBT is said to be successful if the measured energy is lower than the ED threshold. - Alt 2: The 16us measurement period is split into two slots with the first slot having a duration 7us and second slot having a duration of 9us. - Energy measurement is done in both the 7us and 9us slot with the measurement including averaging for at least 4 us in any portion of each slot. LBT is said to be successful if the measured energy is lower than the ED threshold in both slots. - Alt 3: Energy measurement is done in any portion of the 16 us duration including averaging for at least 4 us. LBT is said to be successful if the measured energy is lower than the ED threshold. Agreement at RAN1#97: Multi-carrier channel access schemes that are applicable to multiple carriers with a single LBT sub-band per carrier that is the same as the carrier bandwidth are also applicable to multiple LBT sub-bands within a carrier when such carriers are used either as a single wideband carrier or as part of carrier aggregation of a set of carriers. Agreement at RAN1#98: For a gNB initiated channel occupancy the reference duration for CWS adjustment is defined as follows. - For a CO with unicast PDSCH(s) and for each set of LBT bandwidths for which a single contention window is maintained, the reference duration for CWS adjustment is from the beginning of the CO until the end of the first slot where at least one unicast PDSCH is transmitted over all the resources allocated for the PDSCH, or until the end of the first transmission burst by the gNB that contains unicast PDSCH(s) transmitted over all the resources allocated for the PDSCH, whichever occurs earlier. - If the CO has a unicast PDSCH, but doesn’t have any unicast PDSCH transmitted over all the resources allocated for that PDSCH, then, the duration of the first transmission burst by the gNB within the CO that contains unicast PDSCH(s) is the reference duration for CWS adjustment. For a UE initiated channel occupancy the reference duration for CWS adjustment is defined as follows. - For a CO with PUSCH(s) and for each set of LBT bandwidths for which a single contention window is maintained, the reference duration for CWS adjustment is from the beginning of the CO until the end of the first slot where at least one PUSCH is transmitted over all the resources allocated for the PUSCH, or until the end of the first transmission burst by the UE that contains PUSCH(s) transmitted over all the resources allocated for the PUSCH, whichever occurs earlier. - If the CO has a PUSCH, but doesn’t have any PUSCH transmitted over all the resources allocated for that PUSCH, then, the duration of the first transmission burst by the UE within the CO that contains PUSCH(s) is the reference duration for CWS adjustment. For a gNB initiated channel occupancy, for a DL burst without unicast PDSCH and with one or multiple UL grants, and for each set of LBT bandwidths for which a single contention window is maintained, CWS adjustment is based on the success or failure (FFS: CB, CBG, or TB) of reception of PUSCH transmissions in the granted resources - FFS: Details of CWS adjustment based on the reception of a successful transmission - FFS: Handling of PUCCH/PRACH/SRS and PUSCH without UL-SCH Agreement at RAN1#98bis: Sharing of a UE-initiated channel occupancy (either CG-PUSCH or scheduled UL) with gNB is supported, such that the gNB is allowed to transmit control/broadcast signals/channels for any UEs as long as the transmission contains transmissions for the UE that initiated the channel occupancy and/or DL signals/channels (PDSCH, PDCCH, reference signals) meant for the UE that initiated the channel occupancy. - The ED threshold that the UE applies when initiating a channel occupancy to be shared with the gNB is configured by gNB (RRC signaling) - if ED threshold that the UE applies when initiating a channel occupancy to be shared with the gNB is not configured, the transmission of the gNB in UE initiated COT may include only control/broadcast signals/channels transmissions of up to 2/4/8 OFDM symbols in duration for 15/30/60 kHz SCS - When absence of WiFi cannot be assumed based on e.g. regulation, the ED threshold that the gNB configures to the UE to apply when initiating the channel occupancy is determined based on the max gNB TX power - Cat. 2 LBT can be used (for gaps of 16us and 25us). - Cat. 1 LBT can be used under the following conditions. - Gap duration <= 16us - For the transmission of the gNB after the first switch between the UE and the gNB if the gNB transmission contains only control/broadcast signals/channels - For the transmission of the gNB after the first switch between the UE and the gNB if the gNB transmission has a duration below X ms (X >= 0). - FFS: X - FFS: For transmissions after the second and subsequent switches between UE and gNB Agreement at RAN1#98bis: - For the CP extension prior to at least a dynamically scheduled PUSCH transmission, the CP extension is located in the symbol(s) immediately preceding the PUSCH allocation indicated by SLIV. The supported durations for CP extension at the UE are: - 0 (i.e. no CP extension) - C1*symbol length – 25 us - C2*symbol length – 16 us - TA - C3*symbol length – 25 us – TA - C1=1 for 15 and 30 kHz SCS, C1=2 for 60 kHz SCS - FFS: Whether C2/C3 is fixed or implicitly derived based on TA for each subcarrier spacing - The N2 timeline (UL grant to PUSCH delay) needs to be relaxed to take the CP extension into account - FFS: Whether the limit as per the previous agreement bounding the resulting CP extension to be less than or equal to one symbol for the given subcarrier spacing should be relaxed - FFS: Applicability of this to other UL transmissions - FFS: Whether the number of durations for CP extension that the UE can be signalled dynamically can be configured Agreement at RAN1#98bis: - For FBE operation, when the gNB operates as an initiating device - The UE is provided information on the gNB fixed frame period and the starting positions of the fixed frame periods - For the provision of the above information the following is signalled - Indication of the fixed frame period and the starting positions of the fixed frame period (SIBx) - FFS: Whether Rel-15 signaling can be reused - When the UE is not initiating a channel occupancy, UE transmissions within a fixed frame period can occur if DL signals/channels (e.g., PDCCH, SSB, PBCH, RMSI, GC-PDCCH, …) within the fixed frame period are detected. - FFS: Extension of GC-PDCCH configuration to idle UEs - FFS: Signaling for FBE operation, when the UE operates as an initiating device

In this contribution, we discuss and provide our views on 16us Cat-2 LBT detailed design, channel access mechanisms for NR-U DRS, CA/wideband BWP operations, and CWS adjustments.

Discussion on Channel Access Procedure for NR-U operation

On 16us Cat-2 LBT detailed design

At the RAN1#97 meeting, it was agreed that Cat-2 LBT can be indicated (FFS: explicit and/or implicit) to the UE if the gap is 16us (allowing for implementation tolerances) for LBT by a UE prior to transmission of a UL burst within a gNB-initiated COT as an LBE device. And the detailed design for Cat-2 LBT was discussed and the following alternatives were listed up as below:

- Alt 1: The 16us measurement period is split into two slots with the first slot having a duration 7us and second slot having a duration of 9us.

  - Energy measurement is done in the 9us slot with the measurement including averaging for at least 4 us in any portion of the slot. LBT is said to be successful if the measured energy is lower than the ED threshold.

- Alt 2: The 16us measurement period is split into two slots with the first slot having a duration 7us and second slot having a duration of 9us.

  - Energy measurement is done in both the 7us and 9us slot with the measurement including averaging for at least 4 us in any portion of each slot. LBT is said to be successful if the measured energy is lower than the ED threshold in both slots.

- Alt 3: Energy measurement is done in any portion of the 16us duration including averaging for at least 4 us. LBT is said to be successful if the measured energy is lower than the ED threshold.

For both channel access for DRS and Type-2 UL channel access, Cat-2 LBT with 25us sensing interval has been specified in Rel-13/14 LTE-LAA/eLAA. The 25us interval consists of a duration Tf = 16us immediately followed by one slot duration (i.e. Tsl =9us), and the duration 16us includes a slot duration Tsl at start of the 16us. Further, the channel is considered to be idle for 25us interval if it is sensed to be idle during the slot durations of 25us. This means that only slot duration Tsl within Tf = 16us is used for sensing the channel to be idle. There has been no eNB’s or UE’s channel sensing behavior for 7us in a part of Tf = 16us, not including slot duration Tsl.

In addition, when the eNB or the UE performs channel access procedure regardless of whether it performs Cat-4 LBT or Cat-2 LBT, there was no definition for sensing slot duration except Tsl = 9us. Energy detection itself in the eNB or UE was defined to perform within the slot duration (i.e. Tsl = 9us), as defined in TS37.213 [7], and in particular, the interval performing the actual energy detection of 4us was left as implementation in a slot duration (i.e. Tsl = 9us). If an additional definition of slot duration for sensing the channel is introduced as like Alt-2, the specification impact would cause significant coexistence issues between NR-U and LAA/Wi-Fi. Therefore, we propose to support Alt-1 that channel sensing is performed only in the 9us slot with the measurement including averaging for at least 4us in any portion of the slot without any additional definition of slot duration for sensing the channel.

- Proposal 1: We propose to support Alt-1 that channel sensing is performed only in the 9us slot with the measurement including averaging for at least 4us in any portion of the slot without any additional definition of slot duration for sensing the channel.

As another issue on 16us Cat-2 LBT detailed design, it should be further discussed how to handle channel access after failure of 16us Cat-2 LBT by a UE for NR-U operation in the case of multi-slot scheduling without gap by a single grant or multiple grant, as shown in Figure 1.

(a) Multiple slot scheduling by a single UL grant

(b) Multiple slot scheduling by multiple UL grants

Figure 1. 16us Cat-2 LBT and multi-slot scheduling without gap by a single grant or multiple grant

If 16us Cat-2 LBT is failed before the first slot of multi-slot allocated resource and any additional channel sensing for transmission in the next slot at the multi-slot allocated resource is not specified, the scheduled or configured resource can be wasted within a gNB’s initiated channel occupancy by not being provided with channel access opportunity. Therefore, it is necessary to discuss and specify how to handle channel access after failure of 16us Cat-2 LBT by a UE for NR-U operation.

- Proposal 2: It is necessary to discuss and specify how to handle channel access after failure of 16us Cat-2 LBT by a UE for NR-U operation.

Channel access schemes for NR-U DRS when Cat-2 conditions are not met

During the study item phase, it was agreed that NR-U should have a signal containing at least SS/PBCH block burst set transmission. Furthermore, it was agreed for NR-U DRS to include the CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying RMSI) associated with SS/PBCH block(s) at the SS/PBCH burst set in one contiguous burst (tentatively referred to as the NR-U DRS). In case of NR-U DRS with RMSI-CORESET(s) and PDSCH(s) carrying RMSI in one contiguous slot and the case of DRS transmission including multiple SS/PBCH block in one contiguous slot, however, it is necessary to further clarify and discuss the details of the channel access type for NR-U DRS when Cat-2 conditions such as the NR-U DRS duty cycle less than 1/20 and the total duration up to 1ms are not met. Considering the NR-U standalone operation, since the transmission of both RMSI-CORESET and RMSI from the gNB and the reception of RMSI-CORESET and RMSI at the UE would not only be essential but also the reception of SS/PBCH block at the UE is quite inevitable especially for initial access such as cell detection and time/ frequency synchronization and for RRM measurement. Therefore, even if the NR-U DRS is composed of consecutive slots with more than 1ms length under the condition with a duty cycle ≤ 1/20 allowed by the regulation, it seems beneficial to allow transmission of the corresponding NR-U DRS by performing Cat-2 for the partial transmission of NR-U DRS in 1ms units to effectively operate NR-U standalone.

- Proposal 3: For NR-U DRS including RMSI-CORESET and PDSCH carrying RMSI or for NR-U DRS including only SS/PBCH blocks,

  - Even if the NR-U DRS is composed of consecutive slots with more than 1ms length under the condition less with a duty cycle ≤ 1/20 allowed by the regulation, it seems beneficial to allow transmission of the corresponding NR-U DRS by performing Cat-2 for the partial transmission of NR-U DRS in 1ms units to effectively operate NR-U standalone.

Channel access mechanism for wideband operation

In the TR38.889 [6], there are two cases for wideband operation for both DL and UL as following:

- Bandwidth larger than 20 MHz can be supported with multiple serving cells.

- NR-U should support that a serving cell can be configured with bandwidth larger than 20 MHz.

For the first case (CA-based) of wideband operation in NR-U, the channel access procedure in LTE-LAA in TS 37.213, i.e., Type-A and Type-B, can be reused for multiple serving cells, where each cell has 20MHz bandwidth.

For the second case (BWP-based) of wideband operation, multi-carrier channel access procedure in LTE-LAA, i.e., Type-A and Type-B, can be also reused for the BWP having multiple 20MHz sub-band(s) in a carrier with larger BW than 20MHz bandwidth. However, in case of applying the Type-A multi-carrier channel access procedure as in LTE-LAA for the BWP-based wideband operation, it should be further investigated whether or not to allow non-contiguous transmission among multiple 20MHz sub-bands.

- Proposal 4: We propose to reuse multi-carrier channel access procedure as in LTE-LAA for the baseline of wideband operation when the bandwidth is larger than 20 MHz with the multiple serving cells or a BWP is larger than 20MHz in a single cell.

In addition to two cases for wideband operation, it is necessary to further clarify whether or not to support multicarrier/wideband operation in the case of CA scenarios among BWPs with bandwidth larger than 20MHz on different serving cells. If it is regarded as one of CA and wideband scenarios, it should further investigated on channel access mechanism for multicarrier/wideband operation on that scenarios.

- Proposal 5: It is necessary to further clarify whether or not to support multicarrier/wideband operation in the case of CA scenarios among BWPs with bandwidth larger than 20MHz on different serving cells.

CWS update mechanism

For NR, in contrast to LTE-LAA, CBG-based transmission and CBG-based HARQ-ACK feedback were adopted for the reduced HARQ processing delay. In addition, NR also supports the flexible DL/UL scheduling and corresponding HARQ-ACK timing according to the UE capability. Therefore, considering the NR-based scheduling and CBG transmission with relevant HARQ feedback/processing times, the channel access mechanisms for DL and UL as used in the LTE-LAA need to be modified. In particular, when CBG-based transmission is used for NR-U operation, it is necessary to deal with TB- and CBG-based A/Ns, separately, in calculating the NACK ratio for updating CWS, due to the fact that the gNB can receive TB-based A/Ns and CBG-based A/Ns from the UE in a given time depending on the UE-specific transmission methods. Therefore, it is necessary to calculate the NACK ratio after converting CBG-based A/Ns into TB-based A/Ns for the case that TB-based A/N and CBG-based A/N feedback from different UEs are mixed.

- Proposal 6: When CBG-based transmission is used for NR-U operation, it is necessary to calculate the NACK ratio after converting CBG-based A/Ns into TB-based A/Ns for the case that TB-based A/N and CBG-based A/N feedback from different UEs are mixed.

It was agreed at the RAN1#98 meeting how to determine the reference slot for updating CWS in the time domain by using “reference duration” term. However, it has not been concluded yet how the CWS adujusts depending on the availability of HARQ feedback in the reference duration. Similar to LTE-LAA, the reference duration should be considered as valid only if at least some HARQ-ACK feedbacks are expected to be available.

- Proposal 7: The reference duration should be considered as valid only if at least some HARQ-ACK feedbacks are expected to be available.

Since both TB-based and CBG-based HARQ-ACK corresponding unicast PDSCH in the reference duration can be supported for NR-U operation, the following alternatives can be considered when calculating the NACK ratio in the CWS adjustment procedure for NR-U.

- Proposal 8: The following alternatives can be considered as methods for calculating the NACK ratio in the CWS adjustment procedure when CBG-based transmission is used for NR-U.

  - Alt-1: If at least one of CBG-based HARQ-ACKs corresponding unicast PDSCH(s) in a reference duration is ACK, HARQ-ACK feedback(s) corresponding the unicast PDSCH(s) can be counted as ACK(s), otherwise, NACK(s).

  - Alt-2: If the first one of CBGs corresponding unicast PDSCH(s) in a reference duration is NACK, HARQ-ACK feedback(s) corresponding the unicast PDSCH(s) can be counted as NACK(s), otherwise, ACK(s).

  - Alt-3: If all of CBG-based HARQ-ACKs corresponding unicast PDSCH(s) in a reference duration is ACK(s), HARQ-ACK feedback(s) corresponding the unicast PDSCH(s) is counted as ACK(s), otherwise, NACK(s).

As another aspects for CWS adjustment in case that CBG-based transmission is used for NR-U, it is necessary to further discuss how to handle CWS adjustment from the gNB perspective for the case of TB CRC failure after successfully receiving all CBG retransmissions from the UE perspectives. In other words, when the CBG based transmission is configured for a UE and only a part of the total number of CBGs transmitted by the gNB during initial transmission is successfully received by the UE, gNB performs CBG retransmission to the UE. After that, even though the UE decodes the CBG retransmission correctly, the UE transmits all CBG NACKs as CBG based HARQ-ACK feedback if TB CRC failure occurred, according to the UE behavior in Rel-15 NR. In view of the channel access performed by the gNB transmitting the PDSCH including the CBG-based transmission, when the gNB can know that the UE successfully received all CBGs in DL irrespective of TB-level ACK/NACK, it may be desirable to consider this as ACK for CWS update because the channel access is successful in DL. Therefore, we propose to count ACK for the corresponding TB when calculating NACK ratio for CWS adjustment if TB CRC failure is happened after successfully receiving all CBG retransmissions by the UE only when a part of total CBGs is retransmitted by the gNB.

- Proposal 9: It is necessary to further discuss how to handle CWS adjustment from the gNB perspective in case that the UE transmits all CBG NACKs in CBG-based HARQ-ACK by TB CRC failure after successfully receiving all CBG retransmissions by the UE.

- Proposal 10: We propose to count ACK for the corresponding TB when calculating NACK ratio for CWS adjustment if TB CRC failure is happened after successfully receiving all CBG retransmissions by the UE only when a part of total CBGs is retransmitted by the gNB.

Conclusion

In this contribution, we have discussed 16us Cat-2 LBT detailed design, channel access mechanisms for NR-U DRS, CA/wideband BWP operations, CWS adjustments. We summarize our view as follows.

- Proposal 1: We propose to support Alt-1 that channel sensing is performed only in the 9us slot with the measurement including averaging for at least 4us in any portion of the slot without any additional definition of slot duration for sensing the channel.

- Proposal 2: It is necessary to discuss and specify how to handle channel access after failure of 16us Cat-2 LBT by a UE for NR-U operation.

- Proposal 3: For NR-U DRS including RMSI-CORESET and PDSCH carrying RMSI or for NR-U DRS including only SS/PBCH blocks,

  - Even if the NR-U DRS is composed of consecutive slots with more than 1ms length under the condition less with a duty cycle ≤ 1/20 allowed by the regulation, it seems beneficial to allow transmission of the corresponding NR-U DRS by performing Cat-2 for the partial transmission of NR-U DRS in 1ms units to effectively operate NR-U standalone.

- Proposal 4: We propose to reuse multi-carrier channel access procedure as in LTE-LAA for the baseline of wideband operation when the bandwidth is larger than 20 MHz with the multiple serving cells or a BWP is larger than 20MHz in a single cell.

- Proposal 5: It is necessary to further clarify whether or not to support multicarrier/wideband operation in the case of CA scenarios among BWPs with bandwidth larger than 20MHz on different serving cells.

- Proposal 6: When CBG-based transmission is used for NR-U operation, it is necessary to calculate the NACK ratio after converting CBG-based A/Ns into TB-based A/Ns for the case that TB-based A/N and CBG-based A/N feedback from different UEs are mixed.

- Proposal 7: The reference duration should be considered as valid only if at least some HARQ-ACK feedbacks are expected to be available.

- Proposal 8: The following alternatives can be considered as methods for calculating the NACK ratio in the CWS adjustment procedure when CBG-based transmission is used for NR-U.

  - Alt-1: If at least one of CBG-based HARQ-ACKs corresponding unicast PDSCH(s) in a reference duration is ACK, HARQ-ACK feedback(s) corresponding the unicast PDSCH(s) can be counted as ACK(s), otherwise, NACK(s).

  - Alt-2: If the first one of CBGs corresponding unicast PDSCH(s) in a reference duration is NACK, HARQ-ACK feedback(s) corresponding the unicast PDSCH(s) can be counted as NACK(s), otherwise, ACK(s).

  - Alt-3: If all of CBG-based HARQ-ACKs corresponding unicast PDSCH(s) in a reference duration is ACK(s), HARQ-ACK feedback(s) corresponding the unicast PDSCH(s) is counted as ACK(s), otherwise, NACK(s).

- Proposal 9: It is necessary to further discuss how to handle CWS adjustment from the gNB perspective in case that the UE transmits all CBG NACKs in CBG-based HARQ-ACK by TB CRC failure after successfully receiving all CBG retransmissions by the UE.

- Proposal 10: We propose to count ACK for the corresponding TB when calculating NACK ratio for CWS adjustment if TB CRC failure is happened after successfully receiving all CBG retransmissions by the UE only when a part of total CBGs is retransmitted by the gNB.

References

[1] Draft Report of 3GPP TSG RAN WG1 #98bis v0.1.0

[2] Final Report of 3GPP TSG RAN WG1 #98 v1.0.0

[3] Final Report of 3GPP TSG RAN WG1 #97 v1.0.0

[4] Final Report of 3GPP TSG RAN WG1 #96bis v1.0.0

[5] Final Report of 3GPP TSG RAN WG1 #96 v1.0.0

[6] 3GPP TR 38.889 v16.0.0, “Study on NR-based access to unlicensed spectrum (Release 16)”

[7] 3GPP TS 37.213 v15.2.0