3GPP TSG-RAN WG1 Meeting #99	Tdoc R1-1913102

Reno, NV, USA, November 18th – 22nd, 2019

Agenda Item:	7.2.15

Source:	Ericsson

Title:	On RAN1 UE features list for Rel-16 NR

Document for:	Discussion, Decision

# 1	Introduction

After RAN1#98bis a first round of discussion on the Rel-16 NR UE features was done. In this contribution, we discuss the general principles of UE features definitions and provide comments on the feature grouping from [1].

# 2	Discussion

The RAN1 feature list was thoroughly discussed in Rel-15 and the outcome was captured in TR 38.822. For somebody not having followed the discussion, the interpretation of the feature list is challenging, mainly due to the component descriptions not being clear. For Rel-16, RAN1 should spend sufficient time on making the content of the feature groups clear.

The component descriptions must be understandable also to people not having followed the discussion

It is at this point difficult to say which features and sub-features of Rel-16 that will be deployed at what time. The feature group split is crucial in allowing implemented sub-features to be tested without dependence on other sub-features being implemented at the same time. It is also important to avoid defining redundant features that can be derived from combinations of other features. Care is therefore needed when defining the feature groups where the number of feature groups is secondary to the content.

Feature groups need to be defined to allow independent testing and deployment of sub-features

RAN2 led features should be primarily handled by RAN2. For example, the proposed UE power saving features in 19-4, 19-5 are purely RAN2 driven and RAN2 has the expertise to specify the corresponding feature and any sub-features thereof. Such features should not be captured in the RAN1 feature list. If there are RAN1 related/dependent aspects, these should be discussed in RAN1.

RAN1 should only define RAN1 feature groups

# 3	UE feature list

Ericsson specific comments to the feature list can be found in the appendix.

## 3.1	eURLLC and IIoT

For eURLLC and IIoT features, changes are made in the feature table.

- For DCI format, not only new DCI formats are introduced for eURLLC, but also existing non-fallback DCI formats have been enhanced to support new Rel-16 features, for example, Rel-16 PUSCH transmission. Thus, the UE feature list should also include enhanced DCI format 0_1.

- For DL-SPS configuration, the description should follow the principle that DL-SPS features are fairly symmetric to those of UL configured grant, including separate RRC parameters for each configuration, separate activation for each configuration, and separate or joint release of the configuration(s). Hence the UE feature definitions should reflect this.

- It should be noted that currently no UE features are included for out-of-order HARQ-ACK and out-of-order scheduling, due to lack of RAN1 agreements. However, it is expected that UE feature groups are to be added for them.

# 4	Conclusion

In this contribution we discussed the Rel-16 UE feature list and make the following observations:

Observation 1	The component descriptions must be understandable also to people not having followed the discussion

Observation 2	Feature groups need to be defined to allow independent testing and deployment of sub-features

Observation 3	RAN1 should only define RAN1 feature groups

# References

R1-1911751, “RAN1 UE features list for Rel-16 NR”, AT&T, NTT DOCOMO

# Appendix	UE features list

| Features | Index | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional | Ericsson Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9. NR_2step_RACH | 9-1 | - RACH type selection based on RSRP threshold - Fall back from 2-step RACH to 4-step RACH - Separate RO with 4-step RO configuration - Shared RO with 4-step RO configuration - msgA PUSCH occasions configuration - Intra-slot frequency hopping for msgA PUSCH - Up to two msgA PUSCH configurations in an UL BWP - Validation of msgA PRACH and PUSCH - Preamble and PUSCH resource unit mapping - msgA PUSCH scrambling - Multiple DMRS sequences for msgA PUSCH - msgA PRACH power control - msgA PUSCH power control - DCI format with CRC scrambled by msgB-RNTI - Monitoring of msgB - HARQ-ACK feedback to a MsgB that contains successRAR - For MsgA PRACH support all preamble formats - For MsgA PRACH, support 15 kHz and 30 kHz in FR1 and 60 and 120 kHz in FR2. - For MsgA PUSCH, support 15 kHz, 30 kHz and 60 kHz in FR1 (maybe 60 kHz can be optional). Support 60 kHz and 120 kHz for FR2 - For MsgA PUSCH waveform, support both CP-OFDM and DFT-s-OFDM - Support DMRS ports with configuration type 1 and configuration type 2. |  |  | Yes |  | N/A | UE cannot initiate a 2-step RACH process, and thus would not be expected understand the 2-tep RACH configurations | per band | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling | The list of components in 9-1 is very long. A rather more high level listing closer to the WID is better as a starting point. E.g. 1.) msgA PRACH & PUSCH transmission 2.) msgB reception |
|  | 9-2 | - LBT for msgA PRACH and PUSCH - Interlaced msgA PUSCH | 9-1 |  | Yes |  | N/A | UE cannot initiate a 2-step RACH process for NR-U | per band |  |  | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling | Whether we need a separate UE feature 9-2 for NR-U with 2-step is unclear. One way could be that if a UE supports 2-step RACH, 10-1 LBT and 10-3 NR-U interlaced PUCCH/PUSCH, then it supports 2-step RACH for NR-U. We don’t expect LBT behavior should be different for 2-step and 4-step. |
|  | 9-3 | Parallel MsgA and PRACH Msg1/SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA | 9-1, 2-52, 4-1, 2-12, 6-6 |  | Yes |  | N/A | UE cannot transmit an MsgA and other UL transmissions in parallel across CCs in inter-band CA | per band combination |  |  | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |  |
|  | 9-4 | MsgA operations in a band combination including SUL | 9-1, 6-16 |  | Yes |  | N/A | UE does not support msgA operations in a band combination including SUL | per band combination |  |  | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |  |
| 10. NR-unlicensed | 10-1 | 1. Cat 1 LBT 2. Cat 2 16us LBT 3. Cat 2 25us LBT 4. Cat 4 LBT |  |  |  |  | N/A |  | Per band |  |  |  | This can be a basic feature group for operating in unlicensed band. Support of channel access mechanism for LBE operation, including Cat 4 LBT, Cat 2 LBT, Cat 1 LBT, contention window adjustment. UE supports either 16 us or 25 us or both 16 and 25 us for Cat-2 LBT. | TBD |  |
|  | 10-2 | 1. Cat 1 LBT 2. Cat 2 16us LBT (Do we need this for FBE?) 3. Cat 2 25us LBT |  |  |  |  | N/A |  | Per band |  |  |  | This can be a basic feature group for operating in unlicensed band. Support of channel access mechanism for FBE operation, including fixed frame period, Cat 2 LBT, Cat 1 LBT | TBD |  |
|  | 10-3 | 1) PRB interlace PUCCH (EPF0, EPF1, EPF2, EPF3) 2) PRB interlace PUSCH |  |  |  |  | N/A |  | Per band |  |  |  | This can be a basic feature group for operating in unlicensed band. Support of PRB interlace PUCCH and PUSCH | TBD |  |
|  | 10-4 |  | FBE channel access or LBE channel access |  |  |  | N/A |  | Per band |  |  |  | If 16us Cat 2 LBT is supported | Optional with capability signalling |  |
|  | 10-5 |  | FBE channel access or LBE channel access |  |  |  | N/A |  | Per band |  |  |  | If 25us Cat 2 LBT is supported | Optional with capability signalling |  |
|  | 10-6 |  | LBE channel access |  |  |  | N/A |  | Per band |  |  |  | If Cat4 LBT with contention window adjustment is supported | Optional with capability signalling |  |
|  | 10-7 | 1. 20MHz 2. 10MHz |  |  |  |  | N/A |  |  |  |  |  |  |  |  |
|  | 10-8 | Length 3, 5, 6, 8, 9, 10, 11, 12, 13 FFS the capability are separate for each length or some groups are formed to signal the capability together |  |  |  |  | N/A |  | FFS: Per UE or per band |  |  |  | Support of additional length (other than 2/4/7) for type B PDSCH | Optional with capability signalling |  |
|  | 10-9 | Two groups of search space sets |  |  |  |  | N/A |  | FFS: Per UE or per band |  |  |  | Being configured with two groups of search spaces (FFS more than two), and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |  |
|  | 10-10 | RSSI measurement |  |  |  |  | N/A |  | per band |  |  |  | RSSI measurement for channel occupancy. |  |  |
|  | 10-11 |  |  |  |  |  | N/A |  | Per band |  |  |  | Support transmitting SRS starting in all symbols (0,…,13) of a slot | Optional with capability signalling |  |
|  | 10-12 | 1. No OCC 2. OCC2 3. OCC4 |  |  |  |  | N/A |  | Per band |  |  |  | UE OCC capability for EPF2/EFP3 | Optional with capability signalling |  |
|  | 10-13 | Maximum length of CP extension a UE can generate before the first symbol |  |  |  |  | N/A |  | Per band |  |  |  | How long a UE can generate the CP extension | Optional with capability signalling |  |
|  | 10-14 | Non-numerical value for dl-DataToUL-ACK |  |  |  |  | N/A |  | Per band | No | No |  | If non-numerical K1 value is supported | Optional with capability signalling |  |
|  | 10-15 | Support of bit fields signalling PDSCH HARQ group index and NFI in DCI 1_1 |  |  |  |  | N/A |  | Per band | No | No |  | Enhanced dynamic HARQ codebook supporting grouping of HARQ ACK and triggering the retransmission of HARQ ACK in each groups | Optional with capability signalling |  |
|  | 10-16 | HARQ-ACK codebook containing all configured HARQ processes for all configured CCs |  |  |  |  | N/A |  | Per band | No | No |  | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signalling |  |
|  | 10-17 | Support of scheduling more than one PUSCH with one UL grant |  |  |  |  | N/A |  | Per band |  |  |  |  | Optional with capability signalling |  |
|  | 10-18 | Configured grant with retransmission in CG resources |  |  |  |  | N/A |  | Per band |  |  |  | Support configured grant with retransmission in configured grant resource | Optional with capability signalling |  |
|  | 10-19 |  |  |  |  |  | N/A |  | Per band |  |  |  | This is the number of LBT bandwidth a UE can perform separate ED check on simultaneously | Optional with capability signalling |  |
|  | 10-20 |  |  |  |  |  | N/A |  | Per band |  |  |  |  | Optional with capability signalling |  |
| 11. NR_L1enh_URLLC | 11-1 | 1) Supports monitoring DCI format 1_2 for DL scheduling 2) Supports monitoring DCI format 0_2 for UL scheduling 3) Supports monitoring enhanced DCI format 0_1 for UL scheduling |  |  | Yes |  | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |  |
|  | 11-2 | 1) Supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span for combination (X, Y, ) 2) If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y) are valid for the span pattern, the maximum value of C of the valid combinations is applied | 3-5b |  | Yes |  | N/A |  | Per UE | No | No |  | This capability is necessary for SCS 15kHz and 30 kHz. | Optional with capability signalling Candidate value set for (X, Y): {(7, 3), (4, 3) and (7, 3), (2, 2) and (4, 3) and (7, 3)} | Remaining FFS that related to 11-2. FFS: (3,2) (3,3) for (X,Y) FFS: The impact from empty span(s) on the span pattern" under 2) If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y) are valid for the span pattern, the maximum value of C of the valid combinations is applied. |
|  | 11-3 | 1) Supports sub-slot based HARQ-ACK feedback procedure. • A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. • At least supports sub-slot configurations “2-symbol*7” and “7-symbol*2”. At least one sub-slot configuration for PUCCH can be UE specifically configured to a UE. • Supports a single configuration for PUCCH resource for all sub-slots in a slot. The starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot. Any sub-slot PUCCH resource is not across sub-slot boundaries. |  |  | Yes |  | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |  |
|  | 11-4 | 1) Supports up to two HARQ-ACK codebooks with different priorities to be simultaneously constructed. 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. |  |  | Yes |  | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |  |
|  | 11-5 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots. • The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number. • The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition. • The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols • If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is splitted into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. • Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. • The maximum TDRA table size is increased to 64. 3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field. 4) Rel-16 PUSCH transmission scheme is supported for DCI format 0_1 and DCI format 0_b (for DG and type 2 CG). |  |  | Yes |  | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |  |
|  | 11-6 | 1) PUSCH transmission with Rel-15 behavior with or without slot aggregation. • With slot aggregation, the number of repetitions can be either semi-statically configured (as in Rel-15) or dynamically indicated (as agreed for Rel-16). | 2-12, 2-13, 2-14, 2-15 |  | Yes |  | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |  |
|  | 11-7 | 1) Supports group common DCI (i.e. DCI format 2_x) for cancelation indication 2) UL cancelation for PUSCH and SRS |  |  | Yes |  | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |  |
|  | 11-8 | For DG-PUSCH, one bit (separately from SRI) in UL grant is used to indicate the open loop power control parameter set |  |  | Yes |  | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |  |
|  | 11-9 | 1) Supports up to 12 configured grant configurations in a BWP of a serving cell. • Separate RRC parameters for different configured grant configurations • Separate activation for different configured grant Type 2 configurations • Separate release for different configured grant Type 2 configurations |  |  | Yes |  | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |  |
|  | 11-9a | M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, including joint release • M is determined by the bit length for HPN field for each DCI format for activation and release of Type 2 CG | 11-9 |  | Yes |  | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |  |
| 12. NR_IIOT | 12-1 | Support intra-UE multiplexing/prioritization of UL channels/signals with two priority levels - Prioritization between UL channels/signals with different priority levels - Multiplexing/prioritization between UL channels/signals with the same priority level |  |  | Yes |  | N/A |  | Per UE | No | No | Same handling as others |  | Optional with capability signaling |  |
|  | 12-2 | - Support of up to 8 SPS configurations in a BWP of a serving cell. • Separate RRC parameters for different SPS configurations • Separate activation for different SPS configurations • Separate release for different SPS configurations - The related HARQ-ACK enhancements to support multiple SPS configurations |  |  | Yes |  | N/A |  | Per UE | No | No | Same handling as others |  | Optional with capability signaling |  |
|  | 12-2a | M<=4 bits indication in the Release DCI is used for indicating which SPS configuration(s) is/are released, including joint release • M is determined by the bit length for HPN field for each DCI format for activation and release of SPS | 12-2 |  | Yes |  | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |  |
| 13. NR DL positioning | 13-1 | 1. Max # of positioning frequency layers supported by UE for DL PRS RSRP measurement and report 2. Max number of TRPs per positioning frequency layer a UE can be configured for DL PRS RSRP measurement and report 3. Max # of DL PRS resource sets per TRP supported by UE for DL PRS RSRP measurement and report 4. Max number of DL PRS resources per TRP a UE can be configured for DL PRS RSRP measurement and report 5. Max # of DL PRS resources supported by UE for DL PRS RSRP measurement and report (A limit on the maximum number of DL PRS resources configured to the UE for all TRPs within a measurement window is defined) 6. Total number of DL PRS Resources that UE can be configured for DL PRS-RSRP measurements across all positioning frequency layers, TRPs and resource sets 7. Support of inter-frequency DL PRS RSRP measurement and report 8. Support of DL PRS RSRP measurement quality metric |  |  |  |  | N/A |  | [ #1 – per UE (FFS FR1/FR2 differentiation) #2 – per UE (FFS FR1/FR2 differentiation) #3 – per UE (FFS FR1/FR2 differentiation) #4 – per UE (FFS FR1/FR2 differentiation) #5 – per UE (FFS FR1/FR2 differentiation) #6 – per UE (FFS FR1/FR2 differentiation) #7 – per UE #8 – per UE ] | No | TBD |  |  | Optional with capability signaling |  |
|  | 13-2 | 1. Max # of positioning frequency layers supported by UE for DL PRS RSTD measurement and report 2. Max number of TRPs per positioning frequency layer a UE can be configured for DL PRS RSTD measurement and report 3. Max # of DL PRS resource sets per TRP supported by UE for DL PRS RSTD measurement and report 4. Max number of DL PRS resources per TRP a UE can be configured for DL PRS RSTD measurement and report 5. Max # of DL PRS resources supported by UE for DL PRS RSTD measurement and report (A limit on the maximum number of DL PRS resources configured to the UE for all TRPs within a measurement window is defined) 6. Total number of DL PRS Resources that UE can be configured for DL PRS- RSTD measurements across all positioning frequency layers, TRPs and resource sets 7. Support of inter-frequency DL PRS RSTD measurement and report 8. Support of DL PRS RSTD measurement quality metric | 13-1 |  |  |  | N/A |  | [ #1 – per UE (FFS FR1/FR2 differentiation) #2 – per UE (FFS FR1/FR2 differentiation) #3 – per UE (FFS FR1/FR2 differentiation) #4 – per UE (FFS FR1/FR2 differentiation) #5 – per UE (FFS FR1/FR2 differentiation) #6 – per UE (FFS FR1/FR2 differentiation) #7 – per UE #8 – per UE ] | No | NA |  |  | Optional with capability signaling |  |
|  | 13-3 | Prerequisite – support of SSB measurements for RRM (SS-RSRP, SS-RSRQ, SS-RSRPB) based on Rel.15 for NR Positioning in Rel.16 Configuration and reporting of SSB RRM measurements (SS-RSRP, SS-RSRQ, SS-RSRPB) based on Rel.15 for NR Positioning in Rel.16 |  |  |  |  | N/A |  |  | No | NA |  |  | Optional with capability signaling |  |
|  | 13-4 | Prerequisite – support CSI-RS for RRM (CSI-RSRP, CSI-RSRQ) based on Rel.15 for NR Positioning in Rel.16 Configuration and reporting of CSI-RS for RRM (CSI-RSRP, CSI-RSRQ) for NR Positioning in Rel.16 |  |  |  |  | N/A |  |  | No | NA |  |  |  |  |
|  | 13-5 | TBD specific content in RAN2 WG Estimation of location coordinate at UE side and support of corresponding signaling for DL PRS configuration / measurements | 13-2 |  |  |  | N/A |  | [per UE] | No | NA |  |  | Optional with capability signaling |  |
| 13a. NR UL positioning | 13a-1 | 1. Max # of SRS Resource Sets for positioning supported by UE 2. Max # of SRS Resources for positioning supported by UE across all SRS Resource Sets 3. Support of SSB for neighbor cell DL pathloss estimation and SRS for positioning OLPC 4. Support of DL PRS for neighbor cell DL pathloss estimation and SRS for positioning OLPC 5. Spatial Relation of SRS Resources for positioning with DL PRS / SSB 6. Spatial Relation of SRS Resources for positioning with other SRS resources for positioning 7. Support of different SRS resource types for positioning (periodic, semi-persistent, aperiodic) |  |  |  |  | N/A |  | [ #1 – per UE #2 – per UE #3 – per UE #4 – per UE #5 – per UE #6 – per UE #7 – per UE ] | No | NA |  |  | Optional with capability signaling |  |
| 13b. NR DL and UL positioning | 13b-1 | 1. Support of UE Rx-Tx time difference measurement with serving cell 2. [Support of UE Rx-Tx time difference measurement with serving cell and neighbor cells] 3. Support of inter-frequency UE Rx-Tx time difference measurements | 13-2 13a-1 |  |  |  | N/A |  | [ #1 - per UE #2 - FFS #3 - per UE ] | No | NA |  |  | Optional with capability signaling |  |
| 13c. NR RAT Independent Positioning | 13c-1 | [TBD in RAN2 WG] |  |  |  |  | N/A |  |  | No | NA |  |  | Optional with capability signaling |  |
| 14. NR TEI | 14-1 | - Maximum number of LTE-CRS rate matching patterns in total within a NR carrier | 5-28 (Rate-matching around LTE CRS) |  |  |  | N/A |  | Per UE | No | No |  | For DSS | Optional with capability signaling {2, 3, 4, 5, 6} |  |
|  | 14-2 | - Up to two LTE-CRS overlapping rate matching patterns within a part of NR carrier overlapping with a LTE carrier | 14-1 (Multiple LTE-CRS rate matching patterns), x-x (mTRP support) |  |  |  | N/A |  | [Per band] | No | No |  | For DSS | Optional with capability signaling {supported} |  |
|  | 14-3 | - PDSCH Type B mapping of length 9 OFDM symbols PDSCH Type B mapping of length 10 OFDM symbols | 5-6a (PDSCH mapping type B) |  |  |  | N/A |  | Per UE | No | No |  | For DSS | Optional with capability signaling {supported} |  |
| 15. 5G_V2X_NRSL | 15-1 | 1) UE can receive PSCCH/PSSCH transmitted using NR sidelink mode 1 and 2. 2) UE can receive [X] PSCCH in a slot. 3) UE can decode [Y] RBs per slot counting both PSCCH and PSSCH. 4) UE supports normal 64QAM MCS table and 256QAM MCS table. |  |  |  |  | [No] |  | Per band | [No] | [Yes] | N.A. | This is the basic FG for sidelink | Optional with capability signaling. For UE supports NR sidelink, UE must indicate this FG is supported |  |
|  | 15-2 | 1) UE can transmit PSCCH/PSSCH using dynamic scheduling and configured grant type 1 and 2 in NR sidelink mode 1 scheduled by NR Uu. 2) UE supports normal 64QAM MCS table and 256QAM MCS table. |  |  |  |  | [No] |  | Per band [Note: Random selection in the exceptional pool is supported.] | [No] | [Yes] | N.A. |  | Optional with capability signalling |  |
|  | 15-3 | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by NR Uu and preconfiguration. 2) UE supports normal 64QAM MCS table and 256QAM MCS table. | 15-1 |  |  |  | [No] |  | Per band [Note: Random selection in the exceptional pool is supported.] | [No] | [Yes] | N.A. |  | Optional with capability signalling |  |
|  | 15-4 | 1) UE can transmit and receive S-SSB in NR sidelink mode 1 and 2 |  |  |  |  | [No] |  | Per band | [No] | [Yes] | N.A. |  | Optional with capability signalling | Synchronization is mandatory for the V2X feature. |
|  | 15-5 | 1) UE can report CBR measurement to gNB. 2) UE can adjust its radio parameters based on CBR measurement. |  |  |  |  | [No] |  | Per band | [No] | [Yes] | N.A. |  | Optional with capability signalling | Adaptation of number of retransmissions per CBR range is mandatory for Mode 2 transmission |
|  | 15-6 | 1) Support prioritization between LTE sidelink transmission/reception and NR sidelink transmission/reception | At least one of 15-1, 15-2, 15-3 |  |  |  | [No] |  | Per band | [No] | [Yes] | N.A. |  | Optional with capability signalling |  |
|  | 15-7 | 1) UE can transmit PSCCH/PSSCH using SPS in LTE sidelink mode 3 scheduled by NR Uu. |  |  |  |  | [No] |  | Per band | [No] | [Yes] | N.A. |  | Optional with capability signalling |  |
|  | 15-8 | 1) UE reports a value ‘X’ for the minimum value it supports for the additional time indicated in the NR DCI scheduling LTE sidelink mode 3. | 15-7 |  |  |  | [No] |  | Per band | [No] | [Yes] | N.A. |  | Optional with capability signalling X in {value1, value2, …} |  |
|  | 15-9 | 1) UE can transmit PSCCH/PSSCH using LTE sidelink mode 4 configured by NR Uu and preconfiguration. |  |  |  |  | [No] |  | Per band | [No] | [Yes] | N.A. |  | Optional with capability signalling |  |
|  | 15-10 | 1) UE can transmit PSSCH with 256QAM in NR sidelink | At least one of 15-2, 15-3 |  |  |  | [Yes] | UE supports QPSK, 16QAM, and 64 QAM for transmission. | Per band | [No] | [Yes] | N.A. | Reception of 256QAM is FFS. | Optional with capability signalling | Type should be per UE |
|  | 15-11 | [1) UE can transmit and receive PSFCH in NR sidelink mode 1 and 2.] |  |  |  |  | [Yes] |  | Per band | [No] | [Yes] | N.A. |  | [Optional with capability signalling] |  |
|  | 15-12 | 1) UE additionally supports low-spectral efficiency 64QAM MCS table. |  |  |  |  | [Yes] | UE supports normal 64QAM MCS table and 256QAM MCS table. | Per band | [No] | [Yes] | N.A. |  |  | At least one of 15-1, 15-2, 15-3 is pre-requisite |
|  | 15-13 | [1) UE supports NR sidelink transmission or reception using PT-RS.] | At least one of 15-1, 15-2, 15-3 |  |  |  | [Yes] |  | Per band | [No] | [Yes] | N.A. |  | [Optional with capability signalling. Mandatory in FR2] |  |
| 16. NR_eMIMO | 16-1 | - Support of L1-SINR based beam selection - Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH - Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - Simultaneous spatial relation update for multiple PUCCH resources - Default spatial relation for dedicated-PUCCH/SRS - Spatial relation update for AP-SRS via MAC CE - Pathloss reference RS update for PUSCH/SRS via MAC CE - SCell beam failure recovery (max # SCells configured for BFR) | TBD for #1 2-1, 2-4 for #2 2-53, 2-59 for #3-#6 8-2 for #7 2-31 for #8 |  |  |  | N/A |  | Per band for #1, #4, #5, #6 Per UE for #2, #3, #7, #8 | N | N for #1, #2, #7, #8 Y for #3 - #6 |  |  | Optional | 16-1.1: The UE indicates how many SINR values it can report and if group-based is supported 16-1.7: UE indicates separate support for PUSCH and SRS 16-1.8: UE indicates how many cells, and how many candidate beam RSs. |
|  | 16-2 | Multi-DCI-based: - The maximum number of CORESET configured per “PDCCH-Config” - The maximum number of CORESET configured per TRP (i.e. same value of HigherLayerIndexPerCORESET (if configured) per “PDCCH-Config”) - The value of R for BD/CCE - Whether the UE shall rate match around configured CRS patterns which is associated with HigherLayerIndexPerCORESET (if configured) Single-DCI-based: - Whether supporting two PTRS ports - For URLLC scheme 2b, whether the UE can support CW soft combining - For URLLC scheme 3 and 4, supported maximum TBS size | TBD |  |  |  | N/A |  | Per UE | N | TBD |  |  | TBD | This feature should be split into at least two groups. The use cases for multi-DCi and single-DCI are quite different. Multi-DCI is mainly for eMBB as per current agreements. Single-DCI can apply to both eMBB and URLLC. In addition, the following agreement is missed. We should introduce separate capabilities for schemes 2a and 2b for single-DCI: “Scheme 2a and 2b have separate UE capabilities.” |
|  | 16-3 | Regular eType-II - 8 parameter combinations - 2 PMI FD resolutions (R=1 and 2) - Rank 1 to 4 - CBSR and rank restriction - UCI omission Port selection eType-II - 6 parameter combinations (combos with L=6 don’t apply) - 2 PMI FD resolutions (R=1 and 2) - Rank 1 to 4 - CBSR and rank restriction - UCI omission | TBD |  |  |  | N/A |  | Per UE | N | N |  |  | Optional |  |
|  | 16-4 | Low PAPR DMRS for PDSCH | TBD |  |  |  | N/A |  | Per UE | N | N |  |  | Mandatory |  |
|  | 16-5 | UL full power transmission (mode [0], 1, 2) - New UL codebook set(s) - Multiple SRS resources in set with different number of ports - UE indication of TPMI(s) which delivers full power | [2-13, 2-14] for #1-#3 |  |  |  | N/A |  | Per UE | N | N |  |  | TBD | Subfeature 0. is needed so UE can indicate support for mode 0 (full power without mode 1 or mode 2), or mode 1, or mode 2. New UL codebook sets only applies to mode 1. |
|  | 16-6 | - PUSCH without transform precoding - PUSCH with transform precoding and with pi/2 BPSK modulation - PUCCH with transform precoding and with pi/2 BPSK modulation | TBD for #1-#3 |  |  |  | N/A |  | Per UE | N | N |  |  | TBD |  |
| 17. NR_CLI_RIM | 17-1 | CLI measurement (both CLI-RSSI and SRS-RSRP) |  |  |  |  | N/A |  | Per UE |  | No |  | UE operates CLI-RSSI measurement and SRS-RSRP measurement. | Optional with capability signalling |  |
|  | 17-2 | [simultaneousRxPDSCHCLIRSSIMeasResource] | 17-1 |  |  |  | N/A |  | [Per UE] |  | [No] |  | [UE can assume that PDSCH is FDMed with CLI-RSSI measurement resource(s)] | [Optional with capability signalling] |  |
|  | 17-3 | [simultaneousRxPDSCHSRSRSRPMeasResource] | 17-1 |  |  |  | N/A |  | [Per UE] |  | [No] |  | [UE can assume that PDSCH is FDMed with SRS RSRP measurement resource(s).] | [Optional with capability signalling] |  |
| 18. MR-DC/CA enhancement | 18-1 | UL power sharing modes supported by the UE |  |  |  |  | N/A |  | Per band combination |  |  |  | Absence means DC not supported. UE supporting DC must indicate support for at least 1 UL power sharing scheme. Indication for support of any combination of the schemes needs to be possible. {semi-static, dynamic without lookahead, dynamic with lookahead}. TBD if semi-static to be split in two modes | Optional with capability signalling |  |
|  | 18-2 | 1Tx operation for TDD intra-band EN-DC |  |  |  |  | N/A |  | Per band combination | Applicable to intra.band TDD-TDD EN-DC/NE-DC only | Applicable to FR1 only |  | Extension of the R15 capability singleUL-Transmission to intra-band TDD EN-DC | Optional with capability signalling |  |
|  | 18-3 | TDM restriction to an FD-LTE cell in EN-DC |  |  |  |  | N/A |  | Per band combination | Applicable in EN-DC/NE-DC with FD-LTE only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to a 2Tx UE: Indicates that the UE supports restricting the TD-LTE uplink transmission occasions with a TDM pattern. | Optional with capability signalling |  |
|  | 18-4 | SCell dormancy with L1 indication |  |  |  |  | N/A |  | Per band combination | No | No |  | Support L1-based mechanism to indicate dormancy/non-dormancy status for group of Scells | Optional with capability signalling | This feature should be split into two separate features a) “L1 based SCell dormancy indication in active time”, and b) “L1 based SCell dormancy indication in outside active time”. The outside active time feature should have dependency on DCI format 3-0 reception (i.e., feature 19-1 in current version of the table) |
|  | 18-5 | Cross-carrier scheduling with different SCS |  |  |  |  | N/A |  | Per band combination | TBD | No |  | crossCarrierScheduling-OtherSCS Indicates whether the UE supports cross carrier scheduling for the different numerologies with carrier indicator field (CIF) in DL carrier aggregation where numerologies for the scheduling cell and scheduled cell are different. {Scheduling cell of lower SCS and scheduled cell of higher SCS, Scheduling cell of higher SCS and scheduled cell of lower SCS, both} Note: this capability exists in Rel-15 38.306, but the functionality was not completed in Rel-15. | Optional with capability signalling |  |
|  | 18-6 | Cross-carrier A-CSI RS triggering with different SCS |  |  |  |  | N/A |  | Per band combination | No | No |  | Indicates whether the UE supports triggering of aperiodic CSI-RS where the CSI-RS is on a carrier with one sub-carrier spacing and the triggering PDCCH is on another carrier with a different sub-carrier spacing compared to the carrier with CSI-RS. | Optional with capability signalling |  |
|  | 18-7 | CA with non-aligned frame boundaries |  |  |  |  | N/A |  | Per band combination | No | No |  | Defines whether the UE supports carrier aggregation operation where the frame boundaries of the PCell and the SCell are not aligned, while the slot boundaries are. | Optional with capability signalling |  |
|  | 18-8 | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group |  |  |  |  | N/A |  |  | No | No |  | Support HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group. Rel-15 had this per cell group | [Optional with capability signalling or conditioned to support for multiple PUCCH groups] |  |
| 19.UE Power Saving | 19-1 | - PS_offset for the detection of DCI format 3 with CRC scrambling by PS-RNTI before DRX ON duration - DCI format 3_0 - Indication of UE wakeup or not - Indication of UE dormancy-like behavior |  |  |  |  | N/A |  | [Per UE] | No | No |  | PS_offset is determined based on UE capability | [Mandatory without capability signalling] | Text in Note column should be deleted: PS_offset is an RRC parameter. |
|  | 19-2 | - Enhancement of DCI format 0_1 and 1_1 - DCI Indication of minimum scheduling offset |  |  |  |  | N/A |  | [Per UE] | No | No |  |  | Optional with capability signalling | Component (1) should be descriptive: support of “Minimum applicable scheduling offset indicator” field in the DCI formats 0_1 and 1_1 |
|  | 19-3 | - Maximum MIMO layer per DL BWP |  |  |  |  | N/A |  | [Per UE] | No | No |  |  | Optional with capability signalling |  |
|  | 19-4 | - Preferred c-DRX configuration - Preferred aggregated bandwidth - Preferred SCell configuration - Preferred MIMO configuration - Release Request |  |  |  |  | N/A |  | [Per UE] | No | No |  |  | Optional with capability signalling | Since this is RAN2 led feature, propose to delete row from RAN1 feature list. |
|  | 19-5 | - Relaxation of intra and inter-frequency RRM measurements of neighboring cells in RRC_IDLE/Inactive |  |  |  |  | N/A |  | [Per UE] | No | No |  |  | Optional with capability signalling | Since this is RAN2 led feature, propose to delete row from RAN1 feature list. |
| 20. NR_IAB | 20-1 | Support up to 4 STCs configured for an IAB node DU per cell per frequency location, including IAB-specific SSB transmission periodicities |  |  |  | Yes | N/A | Separate configuration of SSB transmissions for access UE and IAB node measurements is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |  |
|  | 20-2 | Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities |  |  |  | Yes | N/A | Separate configuration of SMTC windows for Inter-IAB node discovery and measurement is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |  |
|  | 20-3 | Support RACH configuration separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors |  |  |  | Yes | N/A | Separate configuration of RACH transmissions/receptions for access UEs and IAB nodes is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |  |
|  | 20-4 | TDM not required between IAB-MT and IAB-DU functions |  |  |  | Yes | N/A | Only TDM-based resource configurations can be supported | [per {MT CC, DU cell} pair] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling |  |
|  | 20-5 | Support semi-static and dynamic configuration/indication of UL-Flexible-DL slot formats for IAB-DU and IAB-MT resources |  |  |  | Yes | N/A | Only Rel-15 slot formats can be configured for backhaul links | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling |  |
|  | 20-6 | Support DCI Format [4]_0 based indication of soft resource availability to an IAB node |  |  |  | Yes | N/A | Explicit indication of soft resource availability is not supported | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |  |
|  | 20-7 | Support IAB-DU DL timing alignment with parent node based on NTA+NTA,offset⋅Tc/2+Tdelta |  |  |  | Yes | N/A | Case-1 OTA timing alignment is not supported | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |  |
| 21. DAPS HO | 21-1 | 1.Synchronous/asynchronous |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21-2 | Simultaneous UL transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21-3 | Handover to cells with different SCS |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21-4 | Simultaneous detection of 2 PDCCHs – similar to CA |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21-5 | Simultaneous detection of 2 PDCCHs – similar to CA |  |  |  |  |  |  |  |  |  |  |  |  |  |