3GPP TSG-RAN WG1 Meeting #99bis	R1-1913136

Reno, NV, USA, November 18th – 22nd 2019

Source:	Ericsson

Title:	UL Reference Signals for NR Positioning

Agenda item:	7.2.10.2

Document for:	Discussion and decision

Introduction

During RAN1#98b, the discussion on enhancements to the SRS for positioning resulted progressed to a stable stage for issues impacting RAN2. In this contribution, we aim at finalizing the aspects of SRS design for positioning. We address the issues regarding SRS usage, RRC configuration, downscoping of staggered pattern configuration, cyclic shift implementation, frequency hopping, sequence generation and collision handling. We also treat the discussion regarding the SRS-related higher layer parameter values.

UL SRS design for positioning

SRS configuration

As with all other transmission in a cellular network, the SRS is controlled by the serving cell, which decides what to schedule based on load demand, interference levels, etc. In order to communicate SRS configurations to the non-serving neighbor cells, SRS configurations are conveyed via NRPPA to the location server so that they can be relayed to the neighbor cells. This is a similar setup to the DL-PRS, for which configurations are decided by the transmitting gNodeBs, and then forwarded upon request to the location server.  The obvious reason for this setup is that the network gNBs must know what resources are transmitted at all time in order to optimize the scheduling of the different signals, control/data channels, in a very dynamic environment. If the location server has to transmit configurations to the UE, it would have to first get these configurations, then relay them to the UE, incurring a delay. Moreover, the nodeB is still in charge of the SRS for other non-positioning purposes. In order to schedule these resources over LPP configured-resources, the nodeB would have to request a release of the SRS resources for positioning prior to schedule RRC-based SRS. As can be seen, the situation gets overcomplicated for no visible advantage.

In the positioning framework, the network is in charge of configuring the reference signals used for positioning. The location server controls the  measurement configurations, configuration sharing between nodes and measurement reports.

The SRS for positioning is configured by the UE serving cell via RRC

## SRS usage

During the work item, a number of new features of SRS were agreed. These feature will be usable  by all SRS usages as the network sees fit. However there are specified restrictions as to the number of possible SRS for each of the usages, as well as attached behaviours, which would collide with usage of SRS for positioning. The pre-existing (rel-15) usages for SRS are as follow:

- nonCodebook is aimed, as the name suggests, at enabling non-codebook based PUSCH transmission. This SRS usages is restricted to a single SRS resource set made of up to 4 1-port SRS resources. This SRS configuration is aiming at giving the possibility for the network to confirm or revise the choice of PUSCH precoding by the UE, so that the network can respond by downselecting some of the layers (precoders) selected by the UE via the SRI field in DCI.

- Codebook: is aimed at enabling codebook-based transmission of PUSCH. In this usage the SRS is transmitted for reciprocity-based channel sounding, and the network responds to the SRS transmission by sending the suitable precoding matrix to the UE.  Only a single resource set may be configured with up to two SRS resources.

- beamManagement: is aimed at identifying suitable beam candidates. In this usage, only one resource per resource set may be transmitted at a given time instant.

- Antenna switching: is aimed at reciprocity-based DL CSI acquisition via SRS carrier switching.

As can be seen, all usages are clearly mapped to different application of the SRS and all come with restriction on the number of SRS resource sets. Therefore,  a new usage for positioning should be added to allow e.g. the use of multiple resource sets with separate OLPC parameters and  control the parameters used in positioning (repetition factor, PTRS port index, number of ports, SRS pattern).

Define a SRS usage for positioning

SRS pattern for positioning and resource allocation

During RAN1#98b, an initial downselection of the possible combination of comb size and number of symbols was done, with possible patterns FFS:

Agreement: The supported relative RE offsets for combinations of the number of symbols (1,2,4,8,12) and comb sizes (2,4,8) for SRS are given in the table below. At least one pattern for each pair are to be selected from the following options: Number of symbols / Comb size; 1; 2; 4; 8; 12. 2; {0}; {0, 1}; FFS: {0, 1, 0, 1} and/or {0, 0, 1, 1}; N/A; N/A. 4; N/A; {0, 2}; FFS: {0,2,1,3} and/or {0,1,2,3} and or {0,2,3,1}; FFS: {0,2,1,3, 0,2,1,3} and/or {0,1,2,3, 0,1,2,3} and/or {0,0,1,1,2,2,3,3}; FFS: {0,2,1,3, 0,2,1,3, 0,2,1,3} and/or {0,1,2,3, 0,1,2,3,0,1,2,3} and/or {0,0,0,1,1,1,2,2,2,3,3,3}. 8; N/A; N/A; N/A; FFS: {0,4,2,6,1,5,3,7} and/or {0,4,1,5,2,6,3,7} and/or {0,1,2,3,4,5,6,7} and/or {0,4,6,2,5,7,3,1}; N/A.

Patterns for Interference mitigation

During RAN1#98b, the benefit of multiple patterns for interference randomization was discussed. It was argued that the interference of one UE may potentially be mitigated by spreading over several UEs instead of overlapping on a single UE[1], as shown in Figure 1. The proposed pattern offer a 50% difference in the RE pattern. We have evaluated the case of mixed patterns for the case where the UEs in the same cell or different cells have either   a full overlap, or 50% overlap in the pattern they use. As shown in Figure 2, the interference levels ( SINR distribution) do not show any change when two UEs have an exact overlap or 50% overlap when configured with SRS. Therefore, we do not see a reason to have multiple configurable patterns.

| Cell 1 using {0,2,1,3} | Cell 2 using {0,2,3,1} | Overlapping Res for user #0 in cell 1 are with user #0 (blue) and #2 (red) in cell 2. |
| --- | --- | --- |

Figure 1: interference between two possible patterns in comb 4

There is no SINR gain with scheduling SRS with different patterns across cells or with a cell

Figure 2 Performance of overlapping and semi-overlapping patterns for SRS

Repetitions within an SRS resource

During RAN1#98b, it was decided to list different possible repetition patterns for the SRS for positioning. The patterns agreed to be included for repetitions are:

- symbols with comb-2

- 8 symbols and 12 symbols for comb 4

For both comb 2 and comb 4, the repetition patterns are either for consecutive symbols, or occurring at the end of one stagger cycle. The advantage of the cyclic repetition is that it allow to multiplex rel16 UEs with different repetition factors. It is therefore preferred to use the cyclically repeated patterns.

The SRS pattern over a number of symbols larger than the comb size follows the comb staggered pattern cyclically

Therefore the RE offset table from RAN1#98b can be further downscoped to the following:

| Number of symbols Comb size | 1 | 2 | 4 | 8 | 12 |
| --- | --- | --- | --- | --- | --- |
| 2 | {0} | {0, 1} | FFS: {0, 1, 0, 1} | N/A | N/A |
| 4 | N/A | {0, 2} | FFS: {0,2,1,3} | FFS: {0,2,1,3, 0,2,1,3} | FFS: {0,2,1,3, 0,2,1,3, 0,2,1,3} |
| 8 | N/A | N/A | N/A | FFS: {0,4,2,6,1,5,3,7} | N/A |

In the specification, the relative RE offset can be implemented by an additional offset to the frequency start position equation in 38.211:

k0(pi)=k0(pi)+b=0BSRSKTCMsc,bSRSnb+f(l')

Where the RE offset k’ is dependent of the comb size and the number of symbols. the RE offset k’ values are described for the different comb values and SRS length  in table 1.

Table 1 RE offset f(l’) for the SRS comb values KTC=2,4,8 and number of SRS symbols NsymbSRS

| Comb size KTC | SRS length NsymbSRS | Symbol number l’ |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| 2 | 2 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |
| 4 | 2 | 0 | 2 |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 0 | 2 | 1 | 3 |  |  |  |  |  |  |  |  |
|  | 4 | 0 | 2 | 1 | 3 | 0 | 2 | 1 | 3 |  |  |  |  |
|  | 4 | 0 | 2 | 1 | 3 | 0 | 2 | 1 | 3 | 0 | 2 | 1 | 3 |
| 8 | 8 | 0 | 4 | 2 | 6 | 1 | 5 | 3 | 7 |  |  |  |  |

the  RE starting position equation in 38.211 is changed to equation 1 with the additional RE offset k’ described in table 1

the RE offset k’ that controls the staggering of the RE allocation may be disabled by RRC configuration to allow rel-15 behaviour

symbolBundle is not supported

## use of full range for cyclic shift with staggered comb

The allocation of the cyclic shift over a comb folds the cyclic shift range within the comb duration, so that the time-domain representation of the cyclic shift is periodic with period TKTC. While this is the correct behavior for the non-staggered SRS comb, it should be modified when a fully or partially staggered pattern is used, to utilize the full range of the comb-1 equivalent full stagger.  One way to resolve this is to have the sequence distributed over the full comb so that each symbol in the pattern features a part of the cyclic shift sequence.  The current implementation of cyclic shift for SRS is as follow [38.211]:

ru,v(α,δ)(n)=ejαnru,v(n),0≤n<MZC,  with MZC=mNscRBKTC

For SRS bandwidth covering more than 36 active subcarriers (based on the size of the comb, the corresponding bandwidth is (6PRBs for comb 2,12 PRB for comb 4, 24PRBs for comb 8), the SRS is using a  Zadoff Chu (ZC) base sequence. Shorter base sequences use computer generated sequences (CGS).  The sequence is then mapped onto to the time-frequency grid, accounting for the comb size with

r(pi)n,l'=ru,v(αi,δ)n0≤n≤Msc,bRS-1l'∈0,1,...,NsymbSRS-1

where  δ=log2KTC and KTC is the comb factor. So that the SRS is mapped on the time frequency grid as:

aKTCk'+k0(pi),l'+l0(pi)=1NapβSRSr(pi)(k',l')k'=0,1,…,Msc,bRS-1l'=0,1,...,NsymbSRS-10otherwise

where length of the sounding reference signal sequence Msc,bRS is given by

Msc,bRS=mSRS,bNscRBKTC

where   mSRS,b is the bandwidth allocation in number of PRBs,  NscRB is the PRB size.

Figure X illustrates how the cylic shift is applied on the frequency grid

Figure 4 Cyclic shift allocation for SRS. In this example, the resource spans 4 symbols.

Figure X shows the way cyclic shifts are applied on a staggered comb with the current agreements in place, and how they should be applied to allow a full range use. Once the staggered comb is collapsed into a comb-1 symbol, the cyclic shift sequence spans the whole symbol and is thus full range.

The modified CS needs to account for the staggered comb sequence  is then:

r(pi)n,l'=ru,v(αi,KTC,l´)n

Where

| ru,v(αi,KTC,l')n= ejα(KTC*n+ f(l´))ru,v(n) | (1) |
| --- | --- |

The cyclic shift  of  the UL SRS may be either applied according to REL-15 behaviour, or follow a symbol-based behavior as shown in equation 1

Frequency hopping

As of RAN1#98b, frequency hopping is under working assumption not to be supported. Frequency hopping allows a UE that is power limited to transmit portions of the SRS bandwidth in each symbol of the resource, and use frequency hopping to span the intended bandwidth.  One could argue that the staggered comb used for positioning serves a similar role. Each symbol in the comb uses a fraction of the total number of REs in the symbol. These REs can be boosted by borrowing the unallocated REs power.  If the full transmission pattern is used, the full bandwidth is covered. Thus, the use of frequency hopping on a staggered comb SRS transmission seems redundant.

Do not support frequency hopping for SRS transmission with a staggered comb.

For implementation in the SRS configuration, frequency hopping is activated and controlled by the b-SRS and c-SRS parameters in the freqHopping field of the SRS config. One possible solution is to link the frequency hopping parameters to the usage parameters, so that the UE is not expected to decode the d-SRS and b-hop fields in freqHopping and can assume they are set to zero when configured with a usage parameter set to positioning.

Sequence generation

The current sequence generation for SRS is based on  a low PAPR sequence.  is defined by a cyclic shift  of a base sequence  according to

where MZC=mNscRB2δ is the length of the sequence and and δ is the comb size. Multiple sequences are defined from a single base sequence through different values of α and δ. The definition of the base sequence  depends on the sequence length , with computer generated sequences under or equal to a length of 36, and ZC sequences above.

A sequence belongs to a group u and is assigned a number v in that group.  The SRS can be configured for group or sequence hopping by the higher-layer parameter groupOrSequenceHopping.  The configuration possibility are:

- groupOrSequenceHopping = ‘neither’: the group u is entirely defined by the SRS sequence ID parameter sequenceId. The base sequence in that group is the first sequence ( v=0)

- groupOrSequenceHopping equals 'groupHopping': the group u is defined according to a combination of the SRS sequence ID parameter sequence ID and a pseudo random value initialized by the SRS sequence ID parameter sequenceId at the beginning of each radio frame. The sequence number v is set to zero.

- groupOrSequenceHopping equals 'sequenceHopping', the group u is defined by the sequence ID parameter sequenceId.  The sequence number is set according to a pseudorandom sequence for sequences larger than 6 resource blocks, initialized by the SRS sequence ID parameter sequenceId.

The sequence ID is a 10bits parameter, allowing up to 1024 UEs to use distinctive sequences when sequence hopping is selected, and up to the number of groups otherwise. Since the sequence is used along with comb offset and cyclic shift, the amount of multiplexing available should be sufficient. Therefore we propose to keep initialization as in release 15.

Reuse rel-15 definitions for sequenceID, including the definition and values for groupOrSequenceHopping

Collision handling

In release 15, dropping rules for SRS follow the following principles:

- Collision with PUCCH: if PUCCH carries HARQ/SR, SRS (of all types) symbols are dropped for the overlapping symbols between PUCCH and SRS. If PUCCH carries CSI or RSRP reports, SRS is transmitted and PUCCH is dropped.

- Collision between aperiodic SRS and other types of SRS (semipersistent or periodic): the aperiodic SRS is transmitted and the overlapping periodic/SP SRS symbols are dropped

- Collision between semi persistent SRS and periodic SRS: the periodic SRS is dropped in the overlapping symbol with the SP SRS.

- For carrier aggregation, unless simultaneous transmission of SRS and PUCCH/PUSCH is allowed, the UE is not expected to be configured with an SRS in one carrier and PUCCH/PUSCH in another carrier for the same symbol

- Similarly, in CA, unless PRACH and SRS simultaneous transmission are allowed, theUE is not expected to be configured with an SRS in one carrier and PRACH in the other for the same symbol.

- For carrier switching, the guard symbols are treating as SRS symbol for dropping rules purposes.

Since the serving nodeB is in charge of the SRS, the legacy rules for collision apply. There should not be a need for additional rules.

Keep the legacy (rel15) dropping rules for SRS.

## Number of SRS resources in a set and number of resource sets

The parameters srsResourceSetId and  srsResourceIdList control SRS resources in a resource set, where maxNrofSRS-ResourcesPerSet =16 controls the maximum number of resources possible to allocate. However, based on the configuration of SRS usage, that number can be significantly lower. For positioning, the number of beams per resource set may be increased compared to other usages. That number should be comparable to what the UE may be configure in terms of downlink PRS, to measure  UE RxTx for a pair of PRS and SRS.

The number of SRS resource per set should be a capability, with up to 16 resources per set as in rel15.

The number of resources set is currently specified to 16. More  resource sets are expected since each set is also now expected to be handling a different neighbor cell with a different open loop PC (e.g. up to 12 cell indoors, more outdoor).  Therefore increasing the number of set to 32 seem necessary.

The number of SRS resource set is a capability, with up to 32 resource sets

## Other SRS Configuration parameters

number of SRS ports

as discussed during the work item, a single SRS port is to be supported for positioning. In the RRC configuration, the parameter for the number of ports nrofSRS-Ports may be bypassed and set as default to “port1”, otherwise the parameter may be only configured to “port1” when SRS is used with the usage “positioning”.

nrofSRS-Ports is either not transmitted and assumed to be confirmed to “port1” when SRS is configured with “positioning” usage, or expected to be configured to “port1”.

PTRS port index

In the SRS resource configuration, The PTRS port index field is only relevant when a phase tracking reference signal is configured. The reason for this field is that, in non-codebook based UL MIMO, each PUSCH layer transmission is connected to an SRS resource.  The SRS is configured with the PTRS port index to identify the PTRS to use during PUSCH demodulation.  Therefore the field should not be used when SRS is transmitted in other usage than “non-codebook”

The ptrs-PortIndex field is not configured in SRS for positioning

Repetition factor

Repetition factor is used only when frequency hopping is configured, and therefore the UE should not expect to decode the repetition factor if the UE is configured with the staggered pattern without frequency hopping.

Either ignore or do not configure the parameter “repetitionFactor” in the SRS configuration when configured with a positioning usage

spatialRelationInfo

the content of   spatialRelationInfo should enable SRS to align with respect to a DL PRS, SSB or potentially CSI RS. For DL-PRS, the field must unambiguously map to the DL-PRS resource. Therefore, the field must include information regarding the originating TRP (TRP ID, name to be defined by ran2), resource set ID, resource ID, as well as positioning frequency layer ID.

for DL PRS, the configuration should include TRP, resource set, resource ID as well as PFL ID

power control parameters

the close loop parameters for SRS power control should be reused as they are in release 15.  The only open loop parameter requiring a change is the agreed reference signal for path loss computation, including SSB and DL-PRS.  We note that for a full description of the SSB and PRS from neighbour cells, the SSB configuration should be communicated to the UE (unless the UE has already detected that SSB), and the PRS configuration should also be known to the UE. Such configurations can be conveyed to the UE and the serving gNB via the LPP and NRPPa protocols, respectively.

SRS resource set parameters alpha and P0 are unchanged

SSB index for pathlossReferenceRS should point to an SSB configuration known to the UE either via previous detection or LPP configuration.

The PRS resource for pathlossReferenceRS should be entirely described. For the neighbor cell, this imply knowning the resource ID, set ID as well as the TRP ID. For the serving cell, resource set and resource ID should be configured.

Regarding overhead reduction for SSB configuration, it is possible to skip transmission of the SSB configuration if the UE has already detected that SSB. However, the LMF and/or gnodeBs then need to be made aware of the detected SSBs. It is unclear how much overhead reduction results once the transmission from the UEs regarding detected SSBs are factored in. this is an issue for RAN2 design of the signaling.

The overhead reduction for SSB need to take into account the cost of detected SSB signalling from the UE to the LMF / nodeB.

Conclusions

In the previous sections we made the following observations:

Observation 1	In the positioning framework, the network is in charge of configuring the reference signals used for positioning. The location server controls the  measurement configurations, configuration sharing between nodes and measurement reports.

Observation 2	There is no SINR gain with scheduling SRS with different patterns across cells or with a cell

Observation 3	The overhead reduction for SSB need to take into account the cost of detected SSB signalling from the UE to the LMF / nodeB.

And the following proposals:

Proposal 1	The SRS for positioning is configured by the UE serving cell via RRC

Proposal 2	Define a SRS usage for positioning

Proposal 3	The SRS pattern over a number of symbols larger than the comb size follows the comb staggered pattern cyclically

Proposal 4	the  RE starting position equation in 38.211 is changed to equation 1 with the additional RE offset k’ described in table 1

Proposal 5	the RE offset k’ that controls the staggering of the RE allocation may be disabled by RRC configuration to allow rel-15 behaviour

Proposal 6	symbolBundle is not supported

Proposal 7	The cyclic shift  of  the UL SRS may be either applied according to REL-15 behaviour, or follow a symbol-based behavior as shown in equation 1

Proposal 8	Do not support frequency hopping for SRS transmission with a staggered comb.

Proposal 9	Reuse rel-15 definitions for sequenceID, including the definition and values for groupOrSequenceHopping

Proposal 10	Keep the legacy (rel15) dropping rules for SRS.

Proposal 11	The number of SRS resource per set should be a capability, with up to 16 resources per set as in rel15.

Proposal 12	The number of SRS resource set is a capability, with up to 32 resource sets

Proposal 13	nrofSRS-Ports is either not transmitted and assumed to be confirmed to “port1” when SRS is configured with “positioning” usage, or expected to be configured to “port1”.

Proposal 14	The ptrs-PortIndex field is not configured in SRS for positioning

Proposal 15	Either ignore or do not configure the parameter “repetitionFactor” in the SRS configuration when configured with a positioning usage

Proposal 16	for DL PRS, the configuration should include TRP, resource set, resource ID as well as PFL ID

Proposal 17	SRS resource set parameters alpha and P0 are unchanged

Proposal 18	SSB index for pathlossReferenceRS should point to an SSB configuration known to the UE either via previous detection or LPP configuration.

Proposal 19	The PRS resource for pathlossReferenceRS should be entirely described. For the neighbor cell, this imply knowning the resource ID, set ID as well as the TRP ID. For the serving cell, resource set and resource ID should be configured.

References

- R1-1911207	Remaining Details on Uplink PRS Signal Design	Fraunhofer IIS, Fraunhofer HHI