3GPP TSG RAN WG1 Meeting #99		R1- 1913461

Reno, USA, November 18 – 22, 2019

Agenda Item:	7.2.8.2

Source:	Huawei, HiSilicon

Title:	Summary of Proposals for M-TRP Offline Session on Wednesday

Document for:	Discussion and decision

URLLC

Offline Agreement:

For single-DCI based M-TRP URLLC scheme 2a/2b and PTRS port indication, support single PTRS port which is allocated with the lowest indexed DMRS ports.

- [Note that the UE shall not assume that it can use the PT-RS transmissions from PDSCH resources associated with different TCI states in a joint manner, when using PT-RS for tracking the phase. How to capture this in RAN1 is up to editor.]

# Offline Proposal

# For single-DCI based M-TRP URLLC scheme 4, support candidate values of URLLCRepNum with:

- Option 1: 2 bits{2,4,8,16}

- Option 2: 3 bits{2:1:8,16}

For single-DCI based M-TRP URLLC scheme 4, support maximum size of the TDRA field to be increased to 6 bits (maximum TDRA table size is increased to 64)

# Offline Proposal:

Discuss whether/how to support 1 TCI state and joint schemes

|  | TCI states | CDM groups | URLLCRepNum | URLLCSchemeEnabler | UE Behavior |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 | >=1 | Condition 4 | Not configured | Rel15 eMBB |
| A | 1 | 1 | Condition 1 | Configured or not configured | "Scheme 4" with repetition from the same TRP |
| A’ | 1 | >=1 | Condition 2 | Not configured | Rel 15 eMBB |
| B | 2 | 1 | Condition 1 | Not configured | Scheme 4 |
| C | 2 | >=2 | Condition 2 | Not configured | 1a/NCJT |
| C’ | 2 | >=2 | Condition 4 | Not configured | 1a/NCJT |
| E | 2 | >=2 | Condition 4 | Not configured | 1a/NCJT |
| F | 2 | 1 | Condition 4 | Configured | Scheme 2a/2b/3 |
| D’’’ | 2 | 1 | Condition 2 | Configured | Scheme 2a/2b/3 |
| D | 2 | >=2 | Condition 1 | Configured | 1a/NCJT |
| D’ | 2 | >=2 | Condition 2 | Configured | 1a/NCJT |
| D’’ | 2 | >=2 | Condition 4 | Configured | 1a/NCJT |
| G’ | 1 | >=1 | Condition 2 | Configured | Rel 15 eMBB |
| G | 1 | >=1 | Condition 4 | Configured | Rel 15 eMBB |
| H | 2 | 1 | Condition 1 | Configured | Alt1: Joint scheme 4+2a/2b/3 Alt 2: Scheme 4 |
| I | 2 | 2 | Condition 1 | Not configured | Joint scheme 1a+4 |

Note:

- Condition 1: indicates at least one entry in pdsch-TimeDomainAllocationList containing URLLCRepNum (>1) in TDRA by DCI

- Condition 2: indicates one entry in pdsch-TimeDomainAllocationList having no URLLCRepNum by DCI, but other entry having URLLCRepNum

- Condition 3: indicates at least one entry in pdsch-TimeDomainAllocationList containing URLLCRepNum (=1) in TDRA by DCI

- Condition 4: None of entry in TDRA contains URLLCRelNum

- Green: in spec draft; Yellow: one scheme; Blue: joint schemes

S-DCI

[Draft Offline proposal 13] When 2 TCI states are indicated by a TCI code point, and URLLSchemeEnabler is not configured and URLLCRepNum is not indicated by pdsch-TimeDomainAllocationList in scheduling DCI, at least for DMRS type 1 and type 2,,

- Proposal 13-1: If indicated DMRS ports are from single CDM group,

  - Alt 1: the UE applies one of the TCI states according to the CDM group index, e.g. the first and second TCI states correspond to CDM group 0 and 1 respectively.

  - Alt 2: UE can ignore the DCI

  - Alt 3: a predefined TCI state is applied

- Proposal 13-2: If indicated DMRS ports are from three CDM groups,

  - Alt 1: The first TCI state is applied to the first indicated CDM group and the second TCI state is applied to the rest CDM group(s).

  - Alt 2: UE can ignore the DCI

M-DCI

# Offline Proposal:

# For a DL serving cell configured with multi-PDCCH based multi-TRP/panel transmission, in the case of PDCCH overbooking for PDCCH candidates monitoring for primary cell, if [per-TRP limits are the same as per scheduled cell limits maximum number of BD/CCE per slot requiring a UE to monitor over CORESETs with same CORESETPoolIndex value is the same maximum number of BD/CCE over all configured CORESETs], Rel. 15 overbooking is followed; otherwise

- Alt1: when USS set i does not satisfy either BD/CCE limit of that serving cell or BD/CCE limit of TRP limit, the UE shall drop all remaining USSs (≥i).

- Alt2: when USS set i does not satisfy BD/CCE limit of associated TRP (i.e. identified by the value of CORESETPoolIndex), the UE shall drop remaining USSs (≥i) associated with CORESETs configured with the same value of CORESETPoolIndex.

- Alt3: overbooking is only applicable to USS sets associated with the CORESET(s) that are configured with CORESETPoolIndex =0 if CORESETPoolIndex is configured.

- Alt4: when USS set i does not satisfy enhanced BD/CCE limit of that serving cell, the UE shall drop all remaining USSs (≥i), as Rel-15 UE behavior.

- Alt5: no overbooking for primary cell