3GPP TSG RAN WG1 Ad Hoc meeting for Channel Model	R1-161608

Ljubljana, Slovenia, March 14-16, 2016

Agenda Item:	6.1

Source:	Huawei, HiSilicon

Title:	Frequency dependency for large scale parameters

Document for:	Discussion and decision

# Introduction

In the RAN1#84 meeting, it was proposed jointly by 15 companies in [1] that the channel model achieved in this SI should be consistent in frequency. In the previous 3GPP 3D channel model in [1], the sub-6GHz channel characteristic parameters are frequency independent. In Table 1, part of the parameters is listed as an example. However, considering the millimeter wave characteristic, the higher frequency channel is more sensitive to the frequency varying and quite different with sub-6GHz channel [2]. Then it is necessary to investigate the validity and applicability of current 3GPP model to support a frequency range up to 100GHz.

Table 1 part of the 3GPP channel parameters [2]

| Scenarios |  | 3D-UMi |  |  | 3D-UMa |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O-to-I | LOS | NLOS | O-to-I |
| Delay spread (DS) log10([s]) | DS | -7.19 | -6.89 | -6.62 | -7.03 | -6.44 | -6.62 |
|  | DS | 0.40 | 0.54 | 0.32 | 0.66 | 0.39 | 0.32 |
| AoA spread (σASA) log10([]) | ASA | 1.75 | 1.84 | 1.76 | 1.81 | 1.87 | 1.76 |
|  | ASA | 0.19 | 0.15 | 0.16 | 0.20 | 0.11 | 0.16 |
| ZoA spread (σZSA) log10([])2) | ZSA | 0.60 | 0.88 | 1.01 | 0.95 | 1.26 | 1.01 |
|  | ZSA | 0.16 | 0.16 | 0.43 | 0.16 | 0.16 | 0.43 |

In this contribution, based on the indoor measurement and simulation, a comprehensive frequency dependent channel characteristic has been investigated for above 6GHz frequency bands.

# Measurements and simulations for multiple frequencies

## Measurements at Indoor Scenario

A multi-band ultra wideband channel sounder system is designed for wide spectrum channel measurements, which is demonstrated in Figure 1. The sounder system is based on a vector network analyzer (VNA), which provides an intermediate frequency (IF) signal and also samples the received IF signal from down converter.

Figure 1. Channel sounder system

As shown in Figure 2, several Rx positions around the open office area are set. In the measurement, the receiver rotates 360 degree to receive the testing signal while the transmitter antenna is fixed.

Figure 2. Vertical view of indoor office

Five frequencies and two cases (i.e., LOS and NLOS) are considered in our measurements, and the results of DS, ASA, ZSA are presented in Table 2 respectively.

Table 2. Channel parameter measurement result

| parameters | scenario | frequencies |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 8 GHz | 15 GHz | 28 GHz | 60 GHz | 73 GHz |
| DS[ns] | LOS | 11.47 | 11.7 | 11.62 | 9.11 | 12.47 |
|  | NLOS | 31.28 | N/A | 24.87 | N/A | 25.83 |
| ASA[deg] | LOS | 54.93 | 39.8 | 42.15 | 33.04 | 38.15 |
|  | NLOS | 65.88 | N/A | 57.77 | N/A | 51.73 |
| ZSA[deg] | LOS | 6.75 | N/A | 5.17 | 4.62 | 5.41 |
|  | NLOS | 6.90 | N/A | 6.09 | N/A | 5.17 |

## Simulation at Indoor Scenario

Ray tracing approach is considered to simulate the realistic spatial channel characteristic. A virtual office environment with respect to the measurement scenarios is set up, as shown in Figure 3.

Figure 3. The simulation environment for Ray Tracing

The ray tracing simulation results are listed below:

Table 3. Ray tracing simulation results

| parameter | scenario | frequency |
| --- | --- | --- |
| DS[ns] | LOS | 11.45 |
|  | NLOS | 38.36 |
| ASA[deg] | LOS | 53.21 |
|  | NLOS | 58.98 |
| ZSA[deg] | LOS | 8.07 |
|  | NLOS | 5.79 |

# Frequency dependency and performance

## DS vs. Frequency

In indoor LOS scenario, the delay spread obeys very slight linear decrease as frequency increases in logarithm domain. The DS frequency dependency formula can be expressed as:

(1)

#### In indoor NLOS scenario, the delay spread obeys the linear decrease as the frequency increases in logarithm domain. The DS frequency dependency formula can be expressed as:

(2)

Figure 4. The delay spread frequency dependency characteristic

## ASA vs. Frequency

The weak frequency dependency can be observed of ASA both in the LOS and NLOS open office scenarios. The ASA frequency dependency formula as follow is given by:

(3)

(4)

Figure 5. The AoA spread frequency dependency characteristic

## ZSA vs. Frequency

The very weak frequency dependency can be observed of ZSA both in the LOS and NLOS in the open office scenarios. The ZSA frequency dependency formula can be expressed as:

(5)

(6)

Figure 6. The ZoA spread frequency dependency characteristic

Observation 1: frequency dependency on DS, ASA and ZSA can be observed for frequency range up to 100 GHz.

Proposal 1: frequency dependency on LSP should be modeled to get an accurate model on frequency range up to 100 GHz.

## Frequency dependency characteristic

Based on the analysis in Section 3.1, 3.2, and 3.3, the frequency dependent channel characteristics are listed in Table 4 as follows:

Table 4. Frequency Dependent Channel Characteristic

| scenario | Indoor office |  |
| --- | --- | --- |
|  | LOS | NLOS |
| DS [ns] |  |  |
| ASA [deg] |  |  |
| ZSA [deg] |  |  |

Proposal 2: The frequency dependency characteristic at high frequency proposed in the table 4 can be used as baseline for further investigation.

# Conclusions

In this contribution, based on the indoor measurement and simulation, a comprehensive frequency dependent channel characteristic has been investigated for above 6 GHz frequency bands. Based on the results, an observation is given,

Observation 1: frequency dependency on DS, ASA and ZSA can be observed for frequency range up to 100 GHz.

And the following proposals are proposed:

Proposal 1: frequency dependency on LSP should be modeled to get an accurate model on frequency range up to 100 GHz.

Proposal 2: The frequency dependency characteristic at high frequency proposed in the table 4 can be used as baseline for further investigation.

# References

R1-161142, “WF on channel modeling requirements”, Huawei, HiSilicon, Ericsson, Keysight, CMCC, AT&T, Nokia, NTT DOCOMO, Samsung, CATR, Interdigital, Intel, ALU, China Unicom, ZTE, Malta, Feb. 14-18, 2016.

3GPP TR 36.873 V12.2.0 , June, 2015.

28 GHz and 73 GHz Millimeter-Wave Indoor Propagation Measurements and Path Loss Models, 2015 IEEE International Conference on Communications (ICC), ICC Workshops