3GPP TSG-RAN WG1 #AH Channel Model                            R1-161632

Ljubljana, Slovenia, 14 – 16 March, 2016

Source: 	ETRI

Title: 	Path loss measurement results at 28GHz and 38GHz for UMi-

street canyon scenario

Agenda Item:	5.2

Document for:	Discussion/Decision

- Introduction

Regarding the Study on channel model for frequency spectrum above 6 GHz [1], a Skeleton TR [2], scenarios [3] and requirements [4] were agreed in the RAN1 #84 meeting [5]. Given the time constraint available for completing channel modeling, it would be desirable to focus on the high priority scenarios i.e. UMi (street canyon), indoor (office) and UMa as in the agreed WF [3]. Furthermore, from this ad-hoc meeting, we think that extensive measurements collected in various scenarios/frequencies should be discussed for the stochastic channel modelling methodology as in the working assumption [6].

In this contribution, we present path loss and shadow fading measurement results collected in an UMi-street canyon scenario in the frequency band at 28 and 38 GHz.

- Summary of Channel Measurement Campaign

2.1 Measurement equipment

To explore the spatio-temporal characteristics of a wireless channel including the path loss, ETRI developed a wideband channel sounder. This channel sounder is designed for performing a field measurement campaign and collecting measured data by transmitting and receiving signals with a 500 MHz bandwidth at carrier frequencies of 28 and 38 GHz [7].

Figure 1.	ETRI Channel Sounder.

As shown in Figure 1, the sounder is composed of a baseband module, a transceiver module, a timing module, a 28-GHz or 38-GHz RF module, and antennas (directional horn antennas and an omnidirectional antenna). The 3D-Positioner includes a RFM and an antenna servo unit that generates servo signals for stepping motors to control the orientation of the horn antenna with 1o accuracy (horizontally and vertically). The rubidium oscillator of timing module provides the reference clock maintaining stable time synchronization between TX and RX. Table 1 describes detailed specifications of the channel sounder. The dynamic range of the sounder is sufficient to measure a path loss of up to 170 dB.

Table 1. Specifications of ETRI channel sounder

| System parameters |  | Specifications |
| --- | --- | --- |
| Center frequency |  | 28 / 38 GHz |
| Channel Bandwidth |  | 500 MHz |
| PN code length of probing signal |  | 4095 chips |
| Maximum power of TX (w/o antenna) | 28 GHz | 29 dBm |
|  | 38 GHz | 21 dBm |
| Automatic gain control range of RX |  | 60 dB |
| Multipath resolution |  | 2 ns |
| HPBW of pyramidal horn antenna and gain | 28 GHz | 10º(24.4 dBi), 30º(15.4 dBi), 60º(9.9 dBi) |
|  | 38 GHz | 10º(24.6 dBi), 30º(16.4 dBi), 40º(12.6 dBi) |
| Gain of omnidirectional antenna | 28 GHz | 5 dBi |
|  | 38 GHz | 6 dBi |

2.2 Measurement scenario: UMi – Street canyon

The outdoor measurements were conducted in Dunsan-dong site (located in Daejeon) in Korea, which were selected to represent typical dense urban environment. It is a downtown area inhabited by 7-11 story buildings (20-35 m height) with 24-35 m wide streets between buildings. Note that the measurement site is composed of rectangular flat street grids. Figure 2 depicts a detailed measurement scenario, on which the locations of TX and RX of the sounder are marked for line-of-sight (LOS) and non-LOS (NLOS) situations. To emulate a typical UMi - street canyon scenario, we installed the TX antennas at a height of 10 m and the RX antenna at 1.5 m (pedestrian level) as illustrated in Figure 2.

Figure 2.	Measurement scenario (left) and campaign at Dunsan-dong site (right).

In order to overcome considerable path loss in the high frequency bands above 6 GHz, high-gain directional antennas have been typically used to measure propagation channel characteristics. However, synthesizing omnidirectional characteristics by scanning these directional antenna measurements may cause erroneous results due to improper selections of the antenna beam-width and the rotation step size. Thus, we use an omnidirectional antenna at the receiver side to obtain path loss characteristics by limiting distance range up to 200 m. To expand the communication range between TX and RX of the sounder, a 30º Half-Power-Beam-Width (HPBW) horn antenna is used for TX. The measurement campaigns were taken during daytime and outside of normal rush hours. Furthermore, traffic was light and ran at about 30 km/h, with few people on sidewalks.

- Measurement Results: Path loss and Shadow fading

There are two popular path loss models [8]: the close-in (CI) free space reference distance model and the alpha-beta-gamma (ABG) model.

- The CI PL model is given by:

PLf,d[dB]=FSPLf,1m+10nlogd1m+Xσ

, where n and d denote the path loss exponent (PLE) and distance in meters, respectively, and Xσ is the shadow fading (SF) term in dB. FSPL is the free space path loss at the specified reference distance (in this case the reference distance is 1 m):

FSPLf,1m=20log4πfc.

- The ABG model is summarized as:

PLf,ddB=10αlogd1m+β+10γlogf1GHz+Xσ

, where α and γ denote the dependency coefficients with respect to distance and frequency, respectively, and β is the path loss offset determined typically by fitting with measurement data.

Figures 3 and 4 show scattered plots of the measured path loss data along with the CI model predictions in the UMi-street canyon scenario at 28 and 38 GHz in LOS and NLOS conditions, respectively. The path loss exponent (PLE) of the CI model is 2.1 for both frequencies in LOS, which agree with the free space PLE of 2. In NLOS, the PLE of the CI model is 3.0 for both frequencies. Therefore, we can observe that the PLE of the CI model at both frequencies are similar in LOS and NLOS environments. In LOS, the standard deviation difference of the shadow fading is only a fraction of a dB. In NLOS environments, the difference is about 1.6 dB.

(a) LOS					(b) NLOS

Figure 3.	Measured path loss at 28 GHz in UMi-street canyon scenario.

(a) LOS					(b) NLOS

Figure 4.	Measured path loss at 38 GHz in UMi-street canyon scenario

Table 2 summarizes the path loss parameters in the ABG and CI models in UMi scenario.

Table 2: Parameters in the CI and ABG path loss model in UMi-street canyon scenario

| Scenario | Environment | Distance [m] | Model | Frequency [GHz] | PLE/ | [dB] |  | [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UMi | LOS | 50-180 | CI | 28 | 2.1 | - | - | 1.8 |
|  |  |  |  | 38 | 2.1 | - | - | 1.7 |
|  |  |  | ABG | 28-38 | 1.9 | 40.6 | 1.8 | 1.7 |
|  | NLOS | 40-140 | CI | 28 | 3.0 | - | - | 3.8 |
|  |  |  |  | 38 | 3.0 | - | - | 5.4 |
|  |  |  | ABG | 28-38 | 4.5 | 3.1 | 2.0 | 4.3 |

Based the measurement results, we have the following proposal:

Proposal 1: Table 2 can be considered as the PL parameters for UMi-street canyon scenario at 28 and 38 GHz.

- Summary

In this contribution, we present the path loss and shadow fading measurement results at 28 GHz and 38 GHz for the scenario of UMi street canyon. Based on the results, we made the following proposal.

Proposal 1:

- Table 2 can be considered as the PL parameters for UMi-street canyon scenario at 28 and 38 GHz.

References

[1] RP-151606 "New SID Proposal: Study on channel model for frequency spectrum above 6 GHz", Samsung, Nokia Networks, 3GPP TSG RAN Meeting #69, Phoenix, USA, 14th – 16th September 2015

[2] R1-160587 "Skeleton TR on >6GHz channel modeling", Samsung, 3GPP TSG RAN1 Meeting#84, ST. Julian’s, Malta, 15th - 19th February, 2016

[3] R1-161145 "WF on >6GHz Channel Modeling Scenarios", NTT DOCOMO, AT&T, ALU, ASB, CATT, Ericsson, ETRI, Fraunhofer, Huawei, HiSilicon, Intel, InterDigital, KT, LGE, NEC, Mitsubishi, Nokia, NTT, Qualcomm, Samsung, Sharp, Sony, Vodafone, 3GPP TSG RAN1 Meeting#84, ST. Julian’s, Malta, 15th - 19th February, 2016

[4] R1-161142 "WF on channel modeling requirements", Huawei, HiSilicon, Ericsson, Keysight, CMCC, AT&T, Nokia, NTT DOCOMO, Samsung, CATR, Interdigital, Intel, ALU, China Unicom, ZTE, 3GPP TSG RAN1 Meeting#84, ST. Julian’s, Malta, 15th - 19th February, 2016

[5] R1-161107, Chairman’s notes of agenda item 7.3.5 on channel model for >6GHz, February 2016

[6] R1-161150 "WF on channel modeling methodology", Samsung, 3GPP TSG RAN1 Meeting#84, ST. Julian’s, Malta, 15th - 19th February, 2016

[7] H.-K. Kwon, M.-D. Kim and Y.-J. Chong, “Implementation and performance evaluation of mmWave channel sounding system,” in Proc. IEEE APS/URSI, 2015.

[8] Aalto University, BUPT, CMCC, Ericsson, Huawei, Intel, KT Corporation, Nokia, NTT DOCOMO, New York University, Qualcomm, Samsung, University of Bristol, University of Southern California, “5G Channel Model for bands up to 100 GHz”, Globecom 2015, Dec. 2015