3GPP TSG-RAN WG1 #AH Channel Model		R1-161704

Ljubljana, Slovenia, March 14-16, 2016

Source:	Samsung, NTT DOCOMO

Title:	Discussion on LoS probability

Agenda Item:	5.2

Document for:	Discussion

- Introduction

The LoS probability model depends on details of operating scenarios such as transmitter antenna height, density and height of surrounding buildings and material of any obstacles. The definition of LoS is discussed with several LoS model. The LoS state is determined by map-based approach. The transmitter (BS) and receiver (MS) positions and whether any buildings or any obstacles are blocking the direct path between the BS and the MS. The influence of objects not represented in the map such as trees, streetlamps, signs, or cars are modelled using blocking terms.

# Discussion on LoS Probability

The LoS probability model will need to be developed across the wide range of frequency bands and for operating scenarios. Two LoS model have been considered in the literature: namely, the d1d2 model, is the current 3GPP/ITU model[1][2], and the square model, is the one developed by NYU model[3].

The first LoS probability model considered, the d1d2 model,

pd=mind1d,11-e-dd2+e-dd2				(1)

where d is the 2D distance in meters and d1 and d2 can both be optimized to fit a set of data or the operating scenarios.

The next LoS probability model considered, the square model,

pd=(mind1d,11-e-dd2+e-dd2)2				(2)

where again d is the 2D distance in meters and d1 and d2 can both be optimized to fit a set of data or operating scenarios.

Figure 1. The map of investigation area in Tokyo, Japan

In Figure 1, it can be seen that the map of investigation area in Tokyo. The red circle is BS points. By using measurement data on Docomo, The LoS probability is analyzed by the measurement data. The LoS probability model depends on BS/MS antenna height and kind of antenna. BS antenna height is 10m, MS antenna height is 2.5m in Tokyo. The kind of BS and MS antenna are sleeve antenna type.

Figure 2. LoS probability for the two models in Tokyo

In Figure 2, it can be seen that the d1d2 model is more suitable than the square model in Tokyo UMi scenario. Also, the RMSE of d1d2 model is smaller than the square model in Tokyo. A summary of the results of the two LoS models are given in the Table 1.

Table 1. Comparison of the LoS probability with Tokyo

|  | d1 | d2 | RMSE |
| --- | --- | --- | --- |
| Fitted d1d2 model | 21 | 39 | 0.018 |
| Square model | 16 | 126 | 0.039 |

In Table 1, it can be seen that RMSE of the d1d2 model is smaller than the square model in Tokyo UMi scenario. RMSE of the square model is bigger than RMSE of the d1d2 model over two times.

Table 2. Comparison of the LoS probability with the various environments

|  | d1 | d2 | RMSE |
| --- | --- | --- | --- |
| Fitted d1d2 model in New-york and Tokyo | 23 | 60 | 0.047 |
| Square model in New-york and Tokyo | 25 | 141 | 0.042 |
| Fitted d1d2 model in New-york, Tokyo and Daejeon | 21 | 50 | 0.031 |
| Square model in New-york, Tokyo and Daejeon | 22 | 123 | 0.022 |

In Table 2, it can be seen that RMSE of the the d1d2 model is little larger than the square model. The different of each RMSE is under one percent. RMSE of the d1d2 model is smaller or a little larger than RMSE of square model according to environment of measurements. Because of that, the d1d2 model using for an existing don’t need to change.

For the UMi scenario, the LoS probability was found that the 3GPP LoS probability formula is sufficient for frequencies above 6GHz[1]. But the fitted d1d2 model in (1) provides better other fitted model. That formula is the same as (1) with d1=18m and d2=36m with d being replaced by 2D distance. The three models are compared to the data in Figure 3 and table 3[4].

Table 3. Comparison of the LoS probability for the UMi

|  | d1 | d2 | MSE |
| --- | --- | --- | --- |
| 3GPP UMi | 18 | 36 | 0.023 |
| Fitted d1d2 model | 20 | 39 | 0.001 |
| Square model | 22 | 100 | 0.026 |

The table 3 show MSE of the d1d2 model is smaller than MSE of the square model in UMi scenario. Also, MSE of the 3GPP UMi LoS model is a litter smaller than MSE of the square model in UMi scenario. So the current LoS model has small error over all distance in total UMi scenario.

Figure 3. UMi LoS probability for the three models

In Figure 3, it can be seen that MSE of the 3GPP UMi LoS model and the fitted d1d2 LoS model are smaller than MSE of the square model in the total UMi scenario. The 3GPP UMi model is special case in sub-model of the d1d2 model. So the fitted d1d2 model provides better other fitted model.

The d1d2 model is suitable than square(NYU) model in the UMi scenarios. Because the MSE of d1d2 model is more smaller than square(NYU) model in various UMi scenarios.

Proposal 1: For UMi scenarios, reuse 3GPP 3D LoS probability models

# Conclusions

Based on the discussions the following way forward is proposed,

Proposal 1: For UMi scenarios, reuse 3GPP 3D LoS Probability models.

References

- 3GPP TR 36.873. “Study on 3D channel model for LTE”.

- ITU-R Rep M.2135-1, ”Guidelines for evaluation of radio interface technologies for IMT-Advanced.”

- M. K. Samimi, T. S. Rappaport, and G. R. MacCartney, Jr., “Probabilistic omnidirectional path loss models for millimeter-wave outdoor communications,” in IEEE Wireless Communications Letters, vol. 4, no. 4, pp. 357-360, Aug. 2015.

- White paper on “5G Channel Model for bands up to 100 GHz,” available at http://www.5gworkshops.com/5GCM.html