3GPP TSG-RAN WG1 NR adhoc	R1-1700295

# Spokane, WA, USA, 16th- 20th January 2017

Agenda Item:	5.1.1.1.4

Source:	Ericsson

Title:	Synchronization signal sequence design

Document for:	Discussion, Decision

# Introduction

RAN1#87 made the following agreements on the synchronization signal sequence design:

- For NR-PSS

  - ZC-sequence can be used as the baseline sequence for NR-PSS for study.

    - Other type of sequences are not excluded, e.g. low density power boosted sequence.

  - Study the following alternatives on the NR-PSS sequence length

    - Alt 1: using sequence whose length is longer than LTE.

      - Whether one longer sequence is used or whether the longer sequence is constructed by concatenating multiple sequences which may be same or different sequence and/or length.

    - Alt 2: using sequence whose length is shorter than LTE.

    - Alt 3: using sequence whose length is the same LTE.

  - Study the following alternatives on the sequence repetition

    - Alt 1: no repetition.

    - Alt 2: time-repetitive signal of NR-PSS across OFDM symbols

    - Alt 3: time-repetitive signal of NR-PSS within an OFDM symbol

    - Alt 4: frequency-repetitive NR-PSS sequences within an OFDM symbol (element-wise or sequence-wise).

- For NR-SSS

  - Study the following alternatives for NR-SSS sequence design:

    - Alt 1: interleaving two M-sequences without scrambling using ID in PSS (no cell ID in NR-PSS).

    - Alt 2: interleaving two M-sequences with scrambling using ID in PSS as in LTE.

    - Alt 3: a root sequence cyclically shifted in time and/or frequency domain.

      - E.g. ZC-sequence or M-sequence with cyclic shifts.

    - Alt 4: message-based transmission (CRC and/or channel coding based).

    - Alt 5: element-wise multiplication of the ZC-sequence and PN-sequence with cyclic shifts.

    - Other alternatives are not excluded.

  - Study the following alternatives on the NR-SSS sequence length:

    - Alt 1: using sequence whose length is longer than LTE.

      - Whether one longer sequence is used or whether the longer sequence is constructed by concatenating multiple sequences which may be same or different sequence and/or length.

    - Alt 2: using the same NR-SSS sequence length as in LTE.

  - Study the following alternatives on the sequence repetition/interleaving:

    - Alt 1: no repetition.

    - Alt 2: time-repetitive signal of NR-SSS within or across OFDM symbols.

    - Alt 3: frequency-repetitive sequences of NR-SSS within an OFDM symbol (element-wise or sequence-wise).

    - Alt 4: frequency interleaved sequence of NR-SSS using comb structure within a OFDM symbol.

In this contribution we discuss the NR-PSS and NR-SSS design, and some evaluation results are provided to compare the possible alternatives.

# Sequence selection

In the agreement, ZC-sequence is selected as the baseline sequence for NR-PSS as expressed below.

ZCun=exp-jπ∙u∙n(n+1)NZC, n=0, 1, ⋯, NZC-1,

where u is the root value and NZC is the sequence length. In NR, it is expected to have a wider band reserved for the NR-PSS[1], which could enable the use of a longer ZC sequence. However, it is well known that the ZC sequence used for time and frequency synchronization would have ambiguity problem, especially for a large frequency offset.

The robustness on the frequency offset for the original ZC sequence is much related with the root value selection for a given sequence length. With some simplification, the peaks of the cross correlation of the transmit and receive signals is located at the position of n0+ku mod NZC, where k is the normalized frequency offset, i.e., k=Δf/fSC (for simplicity, k is assumed to be an integer), and n0 is the delay of receive signals relative to the transmit signal. The peaks of the local cross-correlation results are illustrated in Figure 1, where NZC=128, u=11 and u=25 are assumed, respectively.

(a) NZC = 128, u = 11							(b) NZC = 128, u = 25

Figure 1 Illustration of time and frequency offset ambiguity with different root values

It shows that u=11 has more close peaks than u=25 in the selected region, i.e., from -20µs to 20µs in the time domain and -75kHz (i.e., -5 subcarriers spacing) to 75kHz (i.e., 5 subcarrier spacing) in the frequency domain. Then, it can be expected that the ZC sequence with u=11 of course has worse miss detection rate than u=25 with the same detection algorithms. In addition, from Figure 1 (b), if the frequency offset is not larger than ±45kHz (i.e., ±3 subcarriers spacing), the correct values can be better selected from the three peaks in the region, and the other two peaks are lower than the selected one since the corresponding time delay is well outside the cyclic prefix (CP) and can be ignored, which results in fewer hypotheses with high correlation values.

The miss detection rate performance of the longer ZC sequence is evaluated with the above root values and better root value with different maximum frequency offset assumptions as shown in Figure 2, where 15kHz SCS with normal CP[1] is used as reference, the channel model is the TDL-C with 100ns delay spread and 1-Tx/2-Rx antenna configuration. In the receiver, the correlation between the received signals after filtering in the time domain and the local sequence with different frequency offset hypotheses and time shifts is calculated. After scanning the buffered received signals, the time shift and frequency offset hypotheses with the largest correlation values are selected as the estimated time position and the frequency offset. The residual frequency offset will be further reduced by refining the frequency estimate based on the NR-SSS. In the simulation, the miss detection is defined as the detected sequence not being the transmitted sequence or the residual frequency error being larger than a threshold or the time position being larger than a threshold.

In Figure 2 (a), the miss detection performance is evaluated with different root values, which shows a serious degradation of u=11 for the large number of ambiguous values, and u=25 can have much better performance. However, even with the better selection, the performance still degrades with large frequency offset as shown in Figure 2 (b) for the inevitable ambiguous problem.

Figure 2 Miss detection rate of ZC sequence with different root values and maximum frequency offset

The simulation results confirm the above expectations from the ambiguity figures. From this, we can make the following observations:

A longer ZC sequence cannot eliminate the ambiguous problem in case of large frequency offsets, even with good selection of the root value

There could be a big possibility to support the larger frequency offset in NR, especially with the high carrier frequency, thus it is necessary to consider a new sequence design with good robustness with the possibly large frequency offset. The straightforward design is to modify the ZC sequence with acceptable detection complexity and good performance, based on which we make the following proposal:

Modified ZC sequence needs to be considered for NR-PSS to be robust enough with frequency and time offset.

NR-PSS detection performance is mainly decided by the reserved physical resource if there is no ambiguity problem for the selected sequence. Thus, the extension to the frequency domain or time domain with the same effective longer ZC is expected to have similar detection performance, which would be decided by whether the system is bandwidth-limited or not.

We propose two options for the usage of the NR-PSS/SSS band within one OFDM symbol: with and without repetition in the frequency domain, as shown in Figure 3, where (a) is Option A to direct use the whole band with a longer sequence, and (b) is the Option B is to divide the whole band into several parts, each of them has the same short sequence.

(a) Option A: Directly use the whole band with a longer sequence

(b) Option B: Divide the whole band into several parts, each of them has the same sequence

Figure 3 Band usage without (Option A) and with repetition (Option B)

Option A is a straightforward extension to use a longer sequence over a wider band than LTE, and some tones on both sides would be reserved to avoid the emission problem. Option B divides the whole band into serval parts, and each of them carries the same short sequence. Option B benefits the terminals with even narrower band, such as low cost terminals, for the detection, though the system band for NR-PSS/SSS can still be the pre-defined wider bandwidth. Note that the number of used subcarriers in Option B is less than that in Option A, but the power of each of the used tones in Option B can be boosted to compensate for this.

It is necessary for both options to select a proper sequence to avoid the time and frequency ambiguity problem as mentioned above. In this contribution, to achieve robust time and frequency estimation, we propose a sequence design with interleaved two ZC sequences, of which one sequence is generated by the conjugation of the other sequence. The constructed NR-PSS sequence with a total length of NZC is generated as

ZCun=exp(-1)n∙jπ∙u∙n/2∙(n/2+1)NZC/2, n=0, 1, ⋯, NZC-1,

where the unique root value for both sequences needs to be defined. In this case, the odd subcarriers in the tones for NR-PSS carry one ZC sequence, and the even subcarriers carry the other, i.e., the conjugated ZC sequence as illustrated in Figure 4.

Figure 4 NR-PSS sequence design with interleaved two ZC sequences

The time and frequency ambiguity of the proposed interleaved ZC sequences is firstly checked for both Option A and Option B as shown in Figure 5, where the sequence is divided into two parts with the length of 64 used tones in each part for Option B.

(a) Interleaved ZC within Option A	(b) Interleaved ZC within Option B	(c) Single ZC within Option B

Figure 5 Time and frequency offset ambiguity of interleaved ZC sequences

It shows that the proposed sequence with interleaved two ZC sequences can significantly reduce the ambiguity problem for both Option A and Option B in a large frequency offset region, and a much clearer peak in the region can be identified than that of the original ZC sequence in Figure 1. For the single ZC sequence with repetition, i.e., Figure 5 (c), the best peak is also more distinguishable than that in Figure 1.

The miss detection performance of the proposed interleaved ZC sequences is evaluated and compared with the single ZC sequence in Figure 6, using the same parameters and the receiving algorithms described above. The simulation results show that the proposal with interleaved ZC sequences without repetition in the frequency domain (i.e., Option A) is better than the single longer ZC without repetition, even with less frequency offset, due to better selection of the correct peak. With large frequency offset, e.g., ±75kHz, the interleaved ZC is much better the longer ZC for the alleviation of the ambiguity. With repetition (i.e., Option B), both schemes have very close performance for either small or large frequency offset.

(a) Option A (without repetition)				(b) Option B (with repetition)

Figure 6 NR-PSS miss detection rate performance of longer ZC and interleaved ZC

In the above simulation, the frequency offset between the transmitter and the receiver is assumed to be zero, and the different frequency offset hypotheses range is used to identify the impact of the time and frequency ambiguity problem, which is expected to have the similar observations with the non-zero frequency offset. In Figure 7, 16kHz frequency offset between the transmitter and the receiver and the same receiving algorithm as above is assumed. The results show the similar observation that the interleaved ZC sequence has more robust performance with the large frequency offset without repetition and the design with repetition in the frequency domain is also robust.

(a) Option A (without repetition)				(b) Option B (with repetition)

Figure 7 NR-PSS miss detection rate performance of longer ZC and interleaved ZC with frequency offset between transmitter and receiver

Based on the evaluation results, we have the following proposals for the modified (or enhanced) ZC sequence and the band usage of NR-PSS:

A sequence constructed by two interleaved ZC sequences is proposed as the NR-PSS sequence for robustness with large frequency offset

Repetition in the frequency domain can be considered for robust performance and flexible band usage

NR-SSS is expected to deliver the relevant ID information with sequence detection in addition to further synchronization refinement. Thus, the sequence still needs to support a large number of candidates with acceptable detection complexity. The m-sequence with BPSK modulation in LTE can be extended or repeated to support a wider bandwidth.

Note that the system information transmitted together with NR-PSS/SSS, e.g., NR-PBCH[2], is supposed to be demodulated and decoded within the transmission instance. One option is to introduce additional demodulation reference signals for NR-PBCH for the channel estimation, which, however, needs further work on the reference signal pattern design and also more overhead within the limited physical resource. The other option is to re-use the NR-PSS and NR-SSS in this instance to do channel estimation after synchronization. In this case, it is necessary to assume the same transmission ports used by NR-PBCH and NR-SSS. With the typical dual-port assumption for NR-PBCH, NR-SSS needs to support the dual-port transmission too. Thus, the comb-structure with different subcarriers to indicate the corresponding port index can be considered as an option as shown in Figure 8.

Figure 8 NR-SSS with comb structure to support dual ports

NR-SSS with a comb structure needs to be considered

# Conclusion

In section 2 we made the following observations:

Observation 1	A longer ZC sequence cannot eliminate the ambiguous problem in case of large frequency offsets, even with good selection of the root value

Based on the discussion above we have the following proposals:

Proposal 1	Modified ZC sequence needs to be considered for NR-PSS to be robust enough with the frequency and time offset

Proposal 2	A sequence constructed by interleaved two ZC sequences is proposed as NR-PSS for robustness with large frequency offset

Proposal 3	Repetition in the frequency domain can be considered for the robust performance and flexible band usage

Proposal 4	NR-SSS with a comb structure needs to be considered

# References

R1-1700292, “NR synchronization signal bandwidth and multiplexing”, Ericsson, 3GPP TSG-RAN WG1 #87ah, Spokane, USA, January 16-20, 2017

R1-1700296 “Minimum system information delivery”, Ericsson, 3GPP TSG-RAN WG1 #87ah, Spokane, USA, January 16-20, 2017