3GPP TS 36.211 V19.1.0 (2025-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);

Physical channels and modulation

(Release 19)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

E-UTRA, radio, layer 1

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	9

1	Scope	10

2	References	10

3	Symbols and abbreviations	11

3.1	Symbols	11

3.2	Abbreviations	15

4	Frame structure	15

4.1	Frame structure type 1 for FDD	16

4.2	Frame structure type 2	17

4.3	Frame structure type 3	18

4.4	Frame structure type 1 for NTN-TDD	19

5	Uplink	20

5.1	Overview	20

5.1.1	Physical channels	20

5.1.2	Physical signals	20

5.2	Slot structure and physical resources	20

5.2.1	Resource grid	20

5.2.2	Resource elements	22

5.2.3	Resource blocks	22

5.2.4	Narrowbands and widebands	22

5.2.5	Guard period for narrowband and wideband retuning	23

5.3	Physical uplink shared channel	25

5.3.1	Scrambling	25

5.3.2	Modulation	26

5.3.2A	Layer mapping	27

5.3.2A.1	Layer mapping for transmission on a single antenna port	27

5.3.2A.2	Layer mapping for spatial multiplexing	27

5.3.3	Transform precoding	28

5.3.3A	Precoding	28

5.3.3A.1	Precoding for transmission on a single antenna port	28

5.3.3A.2	Precoding for spatial multiplexing	28

5.3.4	Mapping to physical resources	31

5.4	Physical uplink control channel	37

5.4.1	PUCCH formats 1, 1a and 1b	38

5.4.2	PUCCH formats 2, 2a and 2b	41

5.4.2A	PUCCH format 3	42

5.4.2B	PUCCH format 4	44

5.4.2C	PUCCH format 5	44

5.4.3	Mapping to physical resources	45

5.4A	Short Physical Uplink Control Channel	48

5.4A.1	General	48

5.4A.2	SPUCCH formats 1,1a,1b	48

5.4A.2.1	Slot-SPUCCH	48

5.4A.2.2	Subslot-SPUCCH	49

5.4A.3	SPUCCH format 3	50

5.4A.3.1	Slot-SPUCCH	50

5.4A.4	SPUCCH format 4	50

5.4A.4.1	Slot-SPUCCH	50

5.4A.4.2	Subslot-SPUCCH	51

5.4A.5	Mapping to physical resources	51

5.5	Reference signals	55

5.5.1	Generation of the reference signal sequence	55

5.5.1.1	Base sequences of length  or larger	56

5.5.1.2	Base sequences of length less than 	57

5.5.1.3	Group hopping	61

5.5.1.4	Sequence hopping	62

5.5.1.5	Determining virtual cell identity for sequence generation	62

5.5.2	Demodulation reference signal	63

5.5.2.1	Demodulation reference signal for PUSCH	63

5.5.2.1.1	Reference signal sequence	63

5.5.2.1.2	Mapping to physical resources	67

5.5.2.1A	Demodulation reference signal for PUSCH with sub-PRB allocations	68

5.5.2.1A.1	Reference signal sequence using modulation schemes other than π/2-BPSK	68

5.5.2.1A.2	Reference signal sequence using π/2-BPSK modulation scheme	69

5.5.2.1A.3	Group hopping	70

5.5.2.1A.4	Mapping to physical resources	71

5.5.2.2	Demodulation reference signal for PUCCH	71

5.5.2.2.1	Reference signal sequence	71

5.5.2.2.2	Mapping to physical resources	73

5.5.2.3	Demodulation reference signal for SPUCCH	74

5.5.2.3.1	Reference signal sequence	74

5.5.2.3.2	Mapping to physical resources	74

5.5.3	Sounding reference signal	76

5.5.3.1	Sequence generation	76

5.5.3.1.1	Sequence generation for basic SRS	76

5.5.3.1.2	Sequence generation for additional SRS	76

5.5.3.2	Mapping to physical resources	77

5.5.3.2.1	Mapping to physical resources for basic SRS	77

5.5.3.2.2	Mapping to physical resources for additional SRS	79

5.5.3.3	Sounding reference signal subframe configuration	80

5.6	SC-FDMA baseband signal generation	82

5.6A	SC-FDMA baseband signal generation for PUSCH using sub-PRB allocations	83

5.6A.1	Modulation schemes other than π/2-BPSK	83

5.6A.2	Modulation scheme π/2-BPSK	83

5.7	Physical random access channel	84

5.7.1	Time and frequency structure	84

5.7.2	Preamble sequence generation	91

5.7.3	Baseband signal generation	96

5.8	Modulation and upconversion	97

6	Downlink	98

6.1	Overview	98

6.1.1	Physical channels	98

6.1.2	Physical signals	98

6.2	Slot structure and physical resource elements	99

6.2.1	Resource grid	99

6.2.2	Resource elements	100

6.2.3	Resource blocks	101

6.2.3.1	Virtual resource blocks of localized type	102

6.2.3.2	Virtual resource blocks of distributed type	102

6.2.4	Resource-element groups (REGs)	103

6.2.4A	Enhanced Resource-Element Groups (EREGs)	104

6.2.4B	Short Resource-Element Groups (SREGs)	104

6.2.5	Guard period for half-duplex FDD operation	104

6.2.6	Guard Period for TDD Operation	105

6.2.7	Narrowbands and widebands	105

6.2.8	Guard period for narrowband and wideband retuning	106

6.3	General structure for downlink physical channels	106

6.3.1	Scrambling	107

6.3.2	Modulation	108

6.3.3	Layer mapping	108

6.3.3.1	Layer mapping for transmission on a single antenna port	108

6.3.3.2	Layer mapping for spatial multiplexing	109

6.3.3.3	Layer mapping for transmit diversity	110

6.3.4	Precoding	110

6.3.4.1	Precoding for transmission on a single antenna port	110

6.3.4.2	Precoding for spatial multiplexing using antenna ports with cell-specific reference signals	111

6.3.4.2.1	Precoding without CDD	111

6.3.4.2.2	Precoding for large delay CDD	111

6.3.4.2.3	Codebook for precoding and CSI reporting	112

6.3.4.3	Precoding for transmit diversity	113

6.3.4.4	Precoding for spatial multiplexing using antenna ports with UE-specific reference signals	114

6.3.5	Mapping to resource elements	115

6.4	Physical downlink shared channel	116

6.4.1	Physical downlink shared channel for BL/CE UEs	118

6.4.2	Slot/subslot-based physical downlink shared channel	121

6.5	Physical multicast channel	123

6.5.1	Cyclic shift for PMCH	123

6.5.2	Frequency-domain interleaving	124

6.6	Physical broadcast channel	125

6.6.1	Scrambling	125

6.6.2	Modulation	125

6.6.3	Layer mapping and precoding	125

6.6.4	Mapping to resource elements	125

6.6.4.1	PBCH repetition in the cell acquisition subframe	127

6.7	Physical control format indicator channel	127

6.7.1	Scrambling	128

6.7.2	Modulation	128

6.7.3	Layer mapping and precoding	128

6.7.4	Mapping to resource elements	128

6.8	Physical downlink control channel	129

6.8.1	PDCCH formats	129

6.8.2	PDCCH multiplexing and scrambling	129

6.8.3	Modulation	130

6.8.4	Layer mapping and precoding	130

6.8.5	Mapping to resource elements	130

6.8A	Enhanced physical downlink control channel	131

6.8A.1	EPDCCH formats	131

6.8A.2	Scrambling	133

6.8A.3	Modulation	133

6.8A.4	Layer mapping and precoding	133

6.8A.5	Mapping to resource elements	133

6.8B	MTC physical downlink control channel	134

6.8B.1	MPDCCH formats	134

6.8B.2	Scrambling	135

6.8B.3	Modulation	136

6.8B.4	Layer mapping and precoding	136

6.8B.5	Mapping to resource elements	136

6.8C	Short physical downlink control channel (SPDCCH)	140

6.8C.1	SPDCCH formats	140

6.8C.2	Scrambling	142

6.8C.3	Modulation	142

6.8C.4	Layer mapping and precoding	142

6.8C.5	Mapping to resource elements	142

6.9	Physical hybrid ARQ indicator channel	144

6.9.1	Modulation	144

6.9.2	Resource group alignment, layer mapping and precoding	145

6.9.3	Mapping to resource elements	147

6.10	Reference signals	149

6.10.1	Cell-specific Reference Signal (CRS)	149

6.10.1.1	Sequence generation	149

6.10.1.2	Mapping to resource elements	150

6.10.2	MBSFN reference signals	152

6.10.2.1	Sequence generation	152

6.10.2.1.1	Sequence generation for 15 kHz and 7.5 kHz subcarrier spacing	152

6.10.2.1.2	Sequence generation for 1.25 kHz subcarrier spacing	153

6.10.2.1.3	Sequence generation for 2.5 kHz subcarrier spacing	153

6.10.2.1.4	Sequence generation for 0.37 kHz subcarrier spacing	153

6.10.2.2	Mapping to resource elements	153

6.10.2.2.1	Mapping to resource elements for 15 kHz and 7.5 kHz subcarrier spacing	153

6.10.2.2.2	Mapping to resource elements for 1.25 kHz	155

6.10.2.2.3	Mapping to resource elements for 2.5 kHz subcarrier spacing	155

6.10.2.2.4	Mapping to resource elements for 0.37 kHz subcarrier spacing	156

6.10.3	UE-specific reference signals associated with PDSCH	156

6.10.3.1	Sequence generation	157

6.10.3.2	Mapping to resource elements	158

6.10.3A	Demodulation reference signals associated with EPDCCH, MPDCCH, or SPDCCH	165

6.10.3A.1	Sequence generation	166

6.10.3A.2	Mapping to resource elements	167

6.10.4	Positioning reference signals	169

6.10.4.1	Sequence generation	169

6.10.4.2	Mapping to resource elements	169

6.10.4.3	Positioning reference signal subframe configuration	171

6.10.5	CSI reference signals	172

6.10.5.1	Sequence generation	173

6.10.5.2	Mapping to resource elements	173

6.10.5.3	CSI reference signal subframe configuration	182

6.11	Synchronization signals	182

6.11.1	Primary synchronization signal (PSS)	182

6.11.1.1	Sequence generation	182

6.11.1.2	Mapping to resource elements	183

6.11.2	Secondary synchronization signal (SSS)	183

6.11.2.1	Sequence generation	183

6.11.2.2	Mapping to resource elements	185

6.11.3	Resynchronization signal (RSS)	186

6.11.3.1	Sequence generation	186

6.11.3.2	Mapping to resource elements	186

6.11A	Discovery signal	187

6.11B	MTC wake-up signal (MWUS)	188

6.11B.1	Sequence generation	188

6.11B.2	Mapping to resource elements	188

6.12	OFDM baseband signal generation	189

6.13	Modulation and upconversion	189

7	Generic functions	191

7.1	Modulation mapper	191

7.1.1	BPSK	191

7.1.2	QPSK	191

7.1.3	16QAM	191

7.1.4	64QAM	192

7.1.5	256QAM	194

7.1.6	1024QAM	195

7.2	Pseudo-random sequence generation	195

8	Timing	195

8.1	Uplink-downlink frame timing	196

9	Sidelink	198

9.1	Overview	198

9.1.1	Physical channels	198

9.1.2	Physical signals	198

9.1.3	Handling of simultaneous sidelink and uplink/downlink transmissions	198

9.2	Slot structure and physical resources	199

9.2.1	Resource grid	199

9.2.2	Resource elements	199

9.2.3	Resource blocks	200

9.2.4	Resource pool	200

9.2.5	Guard period	200

9.3	Physical Sidelink Shared Channel	200

9.3.1	Scrambling	200

9.3.2	Modulation	201

9.3.3	Layer mapping	201

9.3.4	Transform precoding	201

9.3.5	Precoding	201

9.3.6	Mapping to physical resources	201

9.4	Physical Sidelink Control Channel	202

9.4.1	Scrambling	202

9.4.2	Modulation	202

9.4.3	Layer mapping	202

9.4.4	Transform precoding	202

9.4.5	Precoding	202

9.4.6	Mapping to physical resources	202

9.5	Physical Sidelink Discovery Channel	203

9.5.1	Scrambling	203

9.5.2	Modulation	203

9.5.3	Layer mapping	203

9.5.4	Transform precoding	203

9.5.5	Precoding	203

9.5.6	Mapping to physical resources	203

9.6	Physical Sidelink Broadcast Channel	204

9.6.1	Scrambling	204

9.6.2	Modulation	204

9.6.3	Layer mapping	204

9.6.4	Transform precoding	204

9.6.5	Precoding	204

9.6.6	Mapping to physical resources	204

9.7	Sidelink Synchronization Signals	204

9.7.1	Primary sidelink synchronization signal	205

9.7.1.1	Sequence generation	205

9.7.1.2	Mapping to resource elements	205

9.7.2	Secondary sidelink synchronization signal	205

9.7.2.1	Sequence generation	205

9.7.2.2	Mapping to resource elements	205

9.8	Demodulation reference signals	205

9.9	SC-FDMA baseband signal generation	207

9.10	Timing	207

10	Narrowband IoT	208

10.0	General	208

10.0.1	Frame structure	208

10.0.1.1	Frame structure type 1	208

10.0.1.2	Frame structure type 2	208

10.1	Uplink	209

10.1.1	Overview	209

10.1.1.1	Physical channels	209

10.1.1.2	Physical signals	209

10.1.2	Slot structure and physical resources	209

10.1.2.1	Resource grid	209

10.1.2.2	Resource elements	210

10.1.2.3	Resource unit	210

10.1.3	Narrowband physical uplink shared channel	211

10.1.3.1	Scrambling	211

10.1.3.2	Modulation	211

10.1.3.3	Layer mapping	212

10.1.3.4	Transform precoding	212

10.1.3.5	Precoding	212

10.1.3.6	Mapping to physical resources	212

10.1.4	Demodulation reference signal	214

10.1.4.1	Reference signal sequence	214

10.1.4.1.1	Reference signal sequence for 	214

10.1.4.1.2	Reference signal sequence for 	216

10.1.4.1.3	Group hopping	217

10.1.4.2	Mapping to physical resources	218

10.1.5	SC-FDMA baseband signal generation	219

10.1.6	Narrowband physical random-access channel	220

10.1.6.1	Time and frequency structure	220

10.1.6.2	Baseband signal generation	223

10.1.7	Modulation and upconversion	223

10.2	Downlink	224

10.2.1	Overview	224

10.2.1.1	Physical channels	224

10.2.1.2	Physical signals	224

10.2.2	Slot structure and physical resource elements	224

10.2.2.1	Resource grid	224

10.2.2.2	Resource elements	224

10.2.2.3	Guard period for half-duplex FDD operation	224

10.2.2.4	Guard period for TDD operation	224

10.2.3	Narrowband physical downlink shared channel	225

10.2.3.1	Scrambling	225

10.2.3.2	Modulation	225

10.2.3.3	Layer mapping and precoding	225

10.2.3.4	Mapping to resource elements	225

10.2.4	Narrowband physical broadcast channel	227

10.2.4.1	Scrambling	227

10.2.4.2	Modulation	227

10.2.4.3	Layer mapping and precoding	227

10.2.4.4	Mapping to resource elements	227

10.2.5	Narrowband physical downlink control channel	228

10.2.5.1	NPDCCH formats	228

10.2.5.2	Scrambling	228

10.2.5.3	Modulation	228

10.2.5.4	Layer mapping and precoding	228

10.2.5.5	Mapping to resource elements	229

10.2.6	Narrowband reference signal (NRS)	230

10.2.6.1	Sequence generation	233

10.2.6.2	Mapping to resource elements	233

10.2.6A	Narrowband positioning reference signal (NPRS)	234

10.2.6A.1	Sequence generation	235

10.2.6A.2	Mapping to resource elements	235

10.2.6A.3	NPRS subframe configuration	236

10.2.6B	Narrowband wake up signal (NWUS)	237

10.2.6B.1	Sequence generation	237

10.2.6B.2	Mapping to resource elements	238

10.2.7	Synchronization signals	238

10.2.7.1	Narrowband primary synchronization signal (NPSS)	238

10.2.7.1.1	Sequence generation	238

10.2.7.1.2	Mapping to resource elements	238

10.2.7.2	Narrowband secondary synchronization signal (NSSS)	239

10.2.7.2.1	Sequence generation	239

10.2.7.2.2	Mapping to resource elements	240

10.2.8	OFDM baseband signal generation	240

10.2.9	Modulation and upconversion	241

Annex A (informative): Change history	242