3GPP TSG RAN WG1 #100			R1-2000446

e-Meeting, February 24th – March 6th, 2020

Agenda item:	7.2.4.2.1

Title:	NR Sidelink Mode-1 resource allocation

Source:	MediaTek Inc.

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss the remaining issues on Rel-16 NR V2X resource allocation Mode-1, specifically on the following: 1) DCI format 3_0, and 2) Remaining details on SL configured grants.

# DCI format 3_0

## Payload size alignment with existing NR DCI

In RAN1-99 [1], the following agreement was made on DCI format 3_0 payload size:

Agreements: - Existing DCI size budget is maintained when the UE is configured with SL - (Working assumption): The size of the new DCI format and the size of one of the existing NR DCI formats are aligned.

According to 38.212 v16 [2], the size of each DCI format 3_0 field is listed in Table 1. As the existing DCI format 0_0/1_0 have a payload of at least 34 bits or more depending on active BWP (excluding padding bits), in our view the alignment of payload sizes is feasible between DCI format 3_0 and DCI format 0_0/1_0 by zero padding.

We have the following proposal:

Proposal 1: Confirm working assumption. DCI format 3_0 payload size is aligned with DCI format 0_0/1_0.

Table 1: DCI format 3_0 field size.

| DCI field | Number of bits |
| --- | --- |
| Time gap | [x] bits ( e.g., x is 3-4 bits) |
| HARQ process ID | [x] bits ( e.g., x is 3-4 bits) |
| NDI | 1 bit |
| Lowest index of the subchannel allocation to initial transmission | log2(N subChannel SL) bits |
| SCI frequency resource assignment | log2(N subChannel SLN subChannel SL + 12) or log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits |
| SCI time resource assignment | 5 or 9 bits |
| PSFCH-to-HARQ feedback timing indicator | 3 bits |
| PUCCH resource indicator | 3 bits |
| Configuration index | 0 or [x] bits (e.g., x is 3-4 bits) |

## Minimum value in Time gap field

DCI format 3_0 includes time gap field for indication of slot offset from the DCI reception to the first sidelink transmission. The field indicates a value which points to an entry of a higher-layer table which is configured by RRC parameter timeGapFirstSidelinkTransmission.

In LTE V2X, the minimum value for time offset indication is 4 subframes. Similarly in NR, the minimum slot offset value in DCI 3_0 should not be shorter than a UE minimum processing time. In addition, the minimum value should depend on sidelink numerology.

We propose the following:

Proposal 2: The indicated value in time gap field in DCI 3_0 should not be smaller than a UE minimum processing time, and should depend on sidelink numerology.

## Minimum value in PSFCH-to-HARQ feedback timing field

DCI format 3_0 includes a PSFCH-to-HARQ feedback timing indication field which points to an uplink slot in reference from the last PSFCH slot. For example, if the indicated timing value is k = 0 (see section 16.5, TS 38.213 v16 [3]), the last slot for PUCCH has to overlap with the last PSFCH reception occasion.

The SL-HARQ preparation time on PUCCH should depend on PSFCH decoding time, UCI preparation time, as well as uplink numerology. Unless a sufficient PSFCH-to-HARQ timing is indicated in DCI 3_0, UE may not be guaranteed to prepare HARQ feedback on the indicated uplink slot.

Proposal 3: PSFCH-to-HARQ feedback timing indication in DCI 3_0 should not be smaller than UE’s minimum HARQ preparation time on PUCCH.

# Sidelink configured grants (SL-CG)

## Maximum number of SL-CGs per resource pool

LTE V2X supports a maximum of 8 sidelink SPS configurations whereas Rel-16 NR supports up to 12 configured grants in uplink. Since multiple configured grants are introduced to both Rel-16 eURLLC and V2X for similar motivations (i.e., mainly, multi-service support and low-latency requirements), it is preferable to support at least the same number of maximum configured grants in both uplink and sidelink in Rel-16. In our view, at least 12 configured grants should be configurable in sidelink.

We propose the following:

Proposal 4: At least 12 SL configured grants are supported per resource pool.

## Joint release of multiple SL-CGs by single DCI

Both Rel-16 uplink grant-free and downlink SPS support joint deactivation of multiple configurations. Higher layer parameters configure deactivation states which are mapped to the configuration index signaled in DCI. Similar RRC parameters can be also introduced to SL configured grant deactivation signaling to support joint release of multiple configurations with no specification effort.

Joint deactivation is useful for improving spectral efficiency by fast release of SL resources via single PHY signaling.

We see benefit in supporting joint release of multiple configurations to bring parity among CG/SPS configurations among all uplink, downlink, and sidelink in Release 16.

We propose the following:

Proposal 5: Support joint deactivation of multiple SL Type-2 configured grants by single DCI.

# Conclusions

We have the following proposals:

Proposal 1: Confirm working assumption. DCI format 3_0 payload size is aligned with DCI format 0_0/1_0.

Proposal 2: The indicated value in time gap field in DCI 3_0 should not be smaller than a UE minimum processing time, and should depend on sidelink numerology.

Proposal 3: PSFCH-to-HARQ feedback timing indication in DCI 3_0 should not be smaller than UE’s minimum HARQ preparation time on PUCCH.

Proposal 4: At least 12 SL configured grants are supported per resource pool.

Proposal 5: Support joint deactivation of multiple SL Type-2 configured grants by single DCI.

# References

- Chairman’s notes, RAN1#99, Reno, USA, November 2019.

- TS 38.212 version 16.0.0, “Multiplexing and channel coding”, Release 16, December 2019.

- TS 38.213 version 16.0.0, “Physical layer procedures for control”, Release 16, December 2019.