3GPP TSG RAN WG1 Meeting #100bis-e                                     R1-2001599

e-Meeting, April 20th – 30th, 2020

Source: 	ZTE

Title: 	Maintenance of low PAPR RS

Agenda item:	7.2.6.5

Document for:	Discussion/Decision

# Introduction

In Rel-16, low PAPR RS is introduced including DMRS for CP-OFDM and DFT-S-OFDM. In this meeting, we provide our views on some issues of the latest endorsed specifications.

# Discussion on maintenance

In Rel-16, DCI format 0_2 is introduced for URLLC. It should be clarified if the agreed π/2-BPSK DMRS can be used for PUSCH scheduled by DCI 0_2 or not. In our view, if the higher-layer parameter DMRSuplinkTransformPrecoding-r16 is configured and π/2-BPSK modulation is used for PUSCH, it is natural to support the π/2-BPSK DMRS to achieve low PAPR benefit.

Proposal 1: The agreed π/2-BPSK DMRS can be used for PUSCH scheduled by DCI 0_2.

Based on proposal 1, we provide TP1 and TP2 for 38.211 section 6.4.1.1.1.2 and 38.212 section 7.3.1.1.3 respectively .

## TP1 for TS 38.211 [1]:

- 6.4.1.1.1.2 Sequence generation when transform precoding is enabled If transform precoding for PUSCH is enabled, the reference-signal sequence shall be generated according to where ru,vα,δ(n) with δ=1 and α=0 depends on the configuration: - if the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH, and the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space, ru,vα,δ(n) is given by clause 5.2.3 with cinit given by cinit=217Nsymbslotns,fμ+l+12NIDnSCID+1+2NIDnSCID+nSCIDmod 231 where nSCID=0 unless given by the DCI according to clause 7.3.1.1.2 in [4, TS38.212] for a transmission scheduled by DCI format 0_1 or given by the DCI according to clause 7.3.1.1.3 in [4, TS38.212] for a transmission scheduled by DCI format 0_2 if Antenna ports field in the DCI format 0_2 is not 0 bit or given by the higher-layer parameter antennaPort for a PUSCH transmission scheduled by a type-1 configured grant; and - NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters pi2BPSKscramblingID0 and pi2BPSKscramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_1 or by DCI format 0_2 if Antenna ports field in the DCI format 0_2 is not 0 bit or by a PUSCH transmission with a configured grant; - NID0∈0,1,…,65535 is given by the higher-layer parameter pi2BPSKscramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI or by DCI format 0_2 if Antenna ports field in the DCI format 0_2 is 0 bit; - NIDnSCID=NIDcell otherwise; - otherwise, ru,vα,δ(n) is given by clause 5.2.2. The sequence group u=fgh+nIDRS mod 30, where nIDRS is given by - nIDRS=nIDPUSCH if nIDPUSCH is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE, the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is not configured, and the PUSCH is neither scheduled by RAR UL grant nor scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI according to clause 8.3 in [5, TS 38.213]; - nIDRS=NIDnSCID if the higher-layer parameter dmrs-UplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH, the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space; - nIDRS=NIDcell otherwise where and the sequence number v are given by: - if neither group, nor sequence hopping is enabled - if group hopping is enabled and sequence hopping is disabled where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized with at the beginning of each radio frame - if sequence hopping is enabled and group hopping is disabled where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized with at the beginning of each radio frame. The hopping mode is controlled by higher-layer parameters: - for PUSCH transmission scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, sequence hopping is disabled and group hopping is enabled or disabled by the higher-layer parameter groupHoppingEnabledTransformPrecoding; - for all other transmissions, sequence hopping and group hopping are enabled or disabled by the respective higher-layer parameters sequenceHopping and sequenceGroupHopping if these parameters are provided, otherwise, the same hopping mode as for Msg3 shall be used. The UE is not expected to handle the case of combined sequence hopping and group hopping. The quantity l above is the OFDM symbol number except for the case of double-symbol DMRS in which case l is the OFDM symbol number of the first symbol of the double-symbol DMRS.

## TP2 for TS 38.212 [2]:

- 7.3.1.1.3 Format 0_2 <Unchanged parts are omitted> - Antenna ports – number of bits determined by the following: - 0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 is not configured; - 2, 3, 4, or 5 bits otherwise, - 2 bits as defined by Tables 7.3.1.1.2-6 or 7.3.1.1.2-6A, if transform precoder is enabled, dmrs-Type=1, and maxLength=1; - 4 bits as defined by Tables 7.3.1.1.2-7 or 7.3.1.1.2-7A, if transform precoder is enabled, dmrs-Type=1, and maxLength=2; - 3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transform precoder is disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transform precoder is disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transform precoder is disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook. where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively.

# Discussion on remaining issues of UE feature

Since the design on Low PAPR DMRS for CP-OFDM and for DFT-S-OFDM are completely different, we think FG 16-6a should be split into two FGs. One is for CP-OFDM, the other is for DFT-S-OFDM. The details can also be found in our companion contribution [3].

The reason is that for CP-OFDM, the enhanced low PAPR DMRS is still inserted in frequency domain, the only change compared with Rel-15 is just on sequence initialization.

However, for DFT-S-OFDM, the enhanced low PAPR DMRS is inserted in time domain. Then, DFT process is needed to transform DMRS from the time domain to the frequency domain. The final DMRS sequence in frequency domain is not constant amplitude anymore. Hardware implementation change at both gNB side and UE side may be needed for this feature group. Some UEs with less capability maybe only support the new DMRS for CP-OFDM.

Therefore, we suggest to report the UE capability separately on low PAPR DMRS with and without transform precoding.

Proposal 2: Split FG16-6a into two FGs separately for CP-OFDM and DFT-S-OFDM.

Based on the above proposal, UE feature list for low PAPR RS needs to be updated as follows.

| 16-6a | Low PAPR DMRS for PUSCH without transform precoding | For PUSCH without transform precoding | TBD |
| --- | --- | --- | --- |
| 16-6b | Low PAPR DMRS for PUCCH | For PUCCH format 3 and/or PUCCH format 4 with transform precoding and with pi/2 BPSK modulation | TBD |
| 16-6c | Low PAPR DMRS for PUSCH with transform precoding and with pi/2 BPSK | For PUSCH with transform precoding and with pi/2 BPSK modulation | TBD |

# Conclusion

In this contribution, we provide our proposal and corresponding TPs on low PAPR PUSCH DMRS. The details of the TPs can be found in section 2.

Proposal 1: The agreed π/2-BPSK DMRS can be used for PUSCH scheduled by DCI 0_2.

Proposal 2: Split FG16-6a into two FGs separately for CP-OFDM and DFT-S-OFDM.

# Reference

- 38.211 g10, NR Physical channels and modulations (Release 16)

- 38.212 g10, NR Multiplexing and channel coding (Release 16)

- R1-2001604, NR eMIMO UE features, ZTE