3GPP TSG RAN WG1 #100bis	R1-2001678

e-Meeting, April 20th – 30th, 2020

Source:	vivo

Title:	Discussion on remaining issues on multi-TRP operation

Agenda Item:	7.2.6.2

Document for:	Discussion and Decision

Introductions

This paper discusses some remaining issues and suggests text proposals on Rel-16 MTRP.

Discussion on TCI states

TCI state activation

For multi-PDCCH-based MTRP operation, RAN1 has confirmed TCI state activation per TRP in the Reply LS [3] as follows:

RAN1 would like to support 8 activated TCI states per TRP, i.e. per CORESETPoolIndex. The total number of activated TCI states that a UE supports is subject to UE capability. Further detailed design is up to RAN2.

In the clause 6.1.3.14 of TS38.321 [7], the “R” field in the TCI States Activation/Deactivation for UE-specific PDSCH MAC CE has been updated with “CORESET Pool ID” field to indicate the CORESETPoolIndex to which the activated TCI states are associated. A Rel-16 UE with no CORESETPoolIndex configured would assume the “CORESET Pool ID” bit set to 0. Therefore, a single MAC CE is applied for TCI state activation/deactivation for UE-specific PDSCH for both cases when no CORESETPoolIndex is configured or only one value of CORESETPoolIndex is configured, and when two different values of CORESETPoolIndex are configured.

For single-PDCCH-based MTRP operation, RAN1 replied to the question about TCI state pairing and total number of activated TCI states in [8] as:

From RAN1 perspective, MAC CE based enhancement is preferred so that the pairing of the TCI states can be flexible. Each TCI state can be dynamically paired with another TCI state, assuming dynamic here means pairing via MAC-CE. From RAN1 perspective, some restrictions on pairing may be introduced but this will not impact RAN2 work.

The maximum number of activated TCI states in mTRP operation is 8. The number of bits of TCI field in DCI is 3 if higher layer parameter tci-PresentInDCI is enabled. The total number of simultaneously activated TCI states is up to 8.

To implement the single-PDCCH-based MTRP requirement, RAN2 has introduced an Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE in clause 6.1.3.24 of [7]. Hence, the text in clause 5.1.5 of [5] should be revised as follows.

Adopt the text proposal in clause 5.1.5 of TS 38.214.

------------------------------------------Start of Text Proposal ---------------------------------- 5.1.5 Antenna ports quasi co-location < Unchanged parts are omitted > The UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321] or in clause [6.1.3.x] of [10, TS 38.321], used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' in one CC/DL BWP or in a set of CCs/DL BWPs, respectively. When a set of TCI state IDs are activated for a set of CCs/DL BWPs, where the applicable list of CCs is determined by indicated CC in the activation command, the same set of TCI state IDs are applied for all DL BWPs in the indicated CCs. When two different values of CORESETPoolIndex are configured, the mapping is applied to the DCI field 'Transmission Configuration Indication' of the PDCCH corresponding to the CORESETPoolIndex indicated by the activation command. When a UE supports two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' the UE may receive an activation command, as described in clause [6.1.3.X24] of [10, TS 38.321], the activation command is used to map up to 8 combinations of one or two TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. The UE is not expected to receive more than 8 TCI states in the activation command. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Default QCL for PDSCH

As depicted in [5], “if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states.” However, there may be several cases and more details of the default QCL(s) for PDSCH to be specified depending on DCI format and the transmission schemes. If at least one TCI codepoint indicates two TCI states, the UE may still buffer with the two default different TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states.

2.2.1 Default QCL when TCI field in DCI is not present

The issue of default QCL assumption for PDSCH in single-PDCCH-based multi-TRP when tci-PresentInDCI is not configured, i.e., TCI field not present in the DCI, was addressed in [9] but no consensus was reached. Some companies say it can be up to UE implementation to select one of the two default TCI states which are from the TCI states of the lowest codepoint among the TCI codepoints containing two different TCI states. However, other companies have concerns on the UE complexity and performance if it is up to UE implementation because it will impact channel estimation by DMRS and PDSCH demodulation and decoding. To simplify the UE implementation and let both network and UE have same understanding of default QCL assumption when TCI field is not present in DCI, we have the following proposal.

If the PDSCH is scheduled by a DCI format not having the TCI field present, the PDSCH would have one default QCL, which can be one of the two default different TCI states, e.g., the first TCI state of the two default different TCI states, or the TCI state of the CORESET for the PDCCH transmission with the DCI if the TCI state of the CORESET for the PDCCH is one of the two default TCI states.

2.2.2 Default QCL when indicating one TCI state

The issue of default QCL assumption for PDSCH in single-PDCCH based multi-TRP when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and the PDSCH is indicated with one TCI state by the scheduling DCI was also discussed in [9] but no consensus was reached either. Same arguments as those in the case of TCI field not present in the DCI were raised. We have similar proposal as follows.

When the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL if applicable, if the PDSCH is indicated with one TCI state by the scheduling DCI, the PDSCH would have one default QCL, which can be one of the two default different TCI states, e.g., the first TCI state of the two default different TCI states, or the indicated TCI state if it is one of the two default different TCI states, or the TCI state of the CORESET for the PDCCH transmission with the DCI if the TCI state of the CORESET for the PDCCH is one of the two default TCI states.

2.2.3 Default QCL when indicating two TCI states

One of the remaining issues is the default QCL assumption when time-domain repetition schemes such as scheme 3 or scheme 4 is dynamically indicated by DCI. In scheme 3 and scheme 4, each PDSCH repetition applies one of the two TCI states indicated in the DCI. If the offset between the reception of the DL DCI and partial or all scheduled PDSCH transmission occasions are less than the threshold timeDurationForQCL, it is reasonable that the UE applies one default TCI state out of both buffered data for one PDSCH repetition. Low UE implementation complexity and aligned network and UE assumption on the default TCI state are desired as well. Therefore, we have following proposal.

If the PDSCH is indicated with two TCI states and scheme 3 or scheme 4 by the scheduling DCI, and if the offset between the reception of the DL DCI and partial or all scheduled PDSCH transmission occasions are less than the threshold timeDurationForQCL, each of the PDSCH transmission occasion has one default QCL, which can be one of the two default different TCI states,

For scheme 3, the first and second TCI states of the two default TCI states or the TCI states indicated by the scheduling DCI are applied to the first and second PDSCH transmission occasions respectively. If the offset between the reception of the DL DCI and one of the two PDSCH transmission occasions is less than the threshold timeDurationForQCL, default TCI states are applied, otherwise TCI states indicated by the scheduling DCI is applied;

For scheme 4, when CycMapping is enabled, the first and second TCI states of the default TCI states or the TCI states indicated by the scheduling DCI are applied are applied to the first and second PDSCH transmission occasions, respectively, and so on. When SeqMapping is enabled, the first TCI state of the default TCI states or the TCI states indicated by the scheduling DCI is applied to the first and second PDSCH transmissions, and the second TCI state of the default TCI states or the TCI states indicated by the scheduling DCI is applied to the third and fourth PDSCH transmissions, and so on. If the offset between the reception of the DL DCI and one of the PDSCH transmission occasions is less than the threshold timeDurationForQCL, default TCI states are applied to the PDSCH transmission occasion, otherwise TCI states indicated by the scheduling DCI is applied.

Based on above proposals, we have the TP proposal in Appendix.

Adopt text proposal in clause 5.1.5 and 5.1.2.1 of TS 38.214 given in Appendix.

PDSCH default QCL for cross-carrier scheduling

For non-CA case, the default QCL(s) of a scheduled PDSCH depends on the maximum number of TCI states mapped to a codepoint in the MAC CE activation command, or the configured number of CORESETPoolIndexes, supporting both single-TRP and multi-TRP cases. However, for cross-carrier scheduling, the default QCL for a PDSCH follows the activated TCI state with the lowest ID in the active BWP which was obviously specified for single TRP. To align with non-CA, following default QCL determination can be applied to the cross-carrier scheduling for both single-PDCCH-based and multi-PDCCH-based multi-TRP cases for cross carrier scheduling.

For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present,

If two different values of CORESETPoolIndex are configured, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH corresponding to the CORESETPoolIndex, which is same as the CORESETPoolIndex of PDCCH scheduling that PDSCH in the active BWP of the scheduled cell.

If at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states applicable to PDSCH in the active BWP of the scheduled cell.

For multi-TRP in cross carrier scheduling, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present,

If two different values of CORESETPoolIndex are configured, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH corresponding to the CORESETPoolIndex, which is same as the CORESETPoolIndex of PDCCH scheduling that PDSCH in the active BWP of the scheduled cell.

If at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states applicable to PDSCH in the active BWP of the scheduled cell.

Default QCL for aperiodic CSI-RS

Current specification on default QCL for aperiodic CSI-RS doesn’t work for MTRP operation. For both single-PDCCH-based and multi-PDCCH-based MTRP, the default QCL for aperiodic CSI-RS may follow the default QCL for PDSCH if there is not any other DL signal with an indicated TCI state in the same symbols as the CSI-RS. Thus, we have following proposal on default QCL for aperiodic CSI-RS.

If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, and if a UE configured with two different values of CORESETPoolIndex, or at least one TCI codepoint indicates two TCI states, when receiving the aperiodic CSI-RS,

if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies indicated one of the one or two of QCL assumption of the other DL signal when receiving the aperiodic CSI-RS.

if there is no other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the indicated one of the one or two of the QCL assumptions for PDSCH reception.

Multi-PDCCH Based Multi-TRP

CRS rate matching

In RAN1 #98bis [2], following agreements were made on CRS rate matching:

Agreement For multi-DCI based multi-TRP/panel transmission, the UE shall rate match around: - Configured CRS patterns which optionally associated with a higher layer signaling index per CORESET (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same higher layer index. - This is a UE optional feature with separate UE capability signalling - If UE does not support this feature, the default UE behaviour is the following: - For multi-DCI based multi-TRP/panel transmission, the UE shall rate match PDSCH around configured CRS patterns from multiple TRPs

In our view, “optionally” in the above agreement implies that a UE may not be configured with parameter CRSPatternList-CORESETPoolIndex even if the UE supports the capability of [separate-lte-CRS-ToMatchAround]. In this case, the UE shall rate match PDSCH around configured CRS patterns from multiple TRPs. It is beneficial for a UE supporting the capability to operate in MTRP scenarios with severe interference among TRPs. In addition, it should be captured that configured CRS patterns associated with a CORESETPoolIndex (if configured) are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same CORESETPoolIndex. In [10], two lists of LTE CRS patterns, lte-CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16, are configured associating to two different values of CORESETPoolIndex respectively. Therefore, we have following text proposal.

Adopt following text proposal in clause 5.1.4.2 of TS 38.214.

------------------------------------------Start of Text Proposal ---------------------------------- 5.1.4.2 PDSCH resource mapping with RE level granularity < Unchanged parts are omitted > - If the UE is configured by higher layer parameter PDCCH-Config with two different values of CORESETPoolIndex in ControlResourceSet and also may be configured by the higher layer parameter LTE lte -CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH: - REs indicated by the CRS pattern(s) in lte-CRS-PatternList-r16 if the PDSCH is associated with CORESETPoolIndex = 0, or the CRS pattern(s) in lte-CRS-PatternListSecond-r16 if the PDSCH is associated with CORESETPoolIndex = 1 CRSPatternList-CORESETPoolIndex for a UE supporting the capability of [separate-lte-CRS-ToMatchAround]; - REs indicated by the CRS pattern(s) in both lte-CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16, if configured, in ServingCellConfig for a UE not supporting the capability of [separate-lte-CRS-ToMatchAround]otherwise. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

PUCCH for HARQ-ACK feedback and other UCIs

In RAN1 #98bis [2], PUCCH/PUSCH collision between different TRPs is avoided by implementation when separate HARQ-ACK feedback is enabled.

Agreement For multi-PDCCH based multi-TRP/panel transmission, when separated ACK/NACK feedback is enabled, - PUCCH/PUSCH collision between different TRPs can be avoided by implementation and UE doesn’t expect overlapping PUCCHs/PUSCHs transmission toward different TRPs. For PUCCH/PUSCH transmission toward the same TRP, Rel-15 multiplexing rules apply. - Note that PUCCH resources can be associated with values of higher layer index per CORESET so that indices may be used to differentiate TRP to determine whether there is overlapping among TRPs. PUSCH can be differentiated by scheduling CORESET in terms of targeted TRP.

As stated in [4], a HARQ-ACK information reporting is associated with a CORESET through a reception of a PDCCH with a DCI format triggering the reporting of the HARQ-ACK information by the UE. And a UE configured with separate HARQ-ACK feedback does not expect a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the first CORESETs to overlap with a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the second CORESETs.

For MTRP operation, it may be required to transmit PUCCH for CSI and SR/BFR. However, it is not clear the association between the CORESETPoolIndex and a PUCCH for P/SP-CSI and SR/BFR. Thus, we prefer to clarify the corresponding behavior when a PUCCH for CSI and SR collides with PUCCH for HARQ-ACK. Considering the above agreements, some restriction can be addressed as follows:

A UE doesn’t expect a PUCCH for CSI/SR overlap with more than one PUCCH for HARQ-ACK.

If a PUCCH with HARQ-ACK overlaps with a PUCCH with SR/CSI information, the UE shall apply the procedures specified in Rel-15.

Adopt following text proposal in section 9 of TS 38.213.

------------------------------------------Start of Text Proposal ---------------------------------- 9 UE procedure for reporting control information < Unchanged parts are omitted > If a UE - is not provided CORESETPoolIndex or is provided CORESETPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided CORESETPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ACKNACKFeedbackMode = SeparateFeedback the UE shall separately apply the procedures described in Clauses 9.1 and 9.2.3 for reporting HARQ-ACK information associated with the first CORESETs on active DL BWP of the serving cells and for reporting HARQ-ACK information associated with the second CORESETs on active DL BWP of the serving cells. HARQ-ACK information reporting is associated with a CORESET through a reception of a PDCCH with a DCI format triggering the reporting of the HARQ-ACK information by the UE. If a PUCCH with HARQ-ACK overlaps with a PUCCH with SR/CSI information, the UE shall apply the procedures described in Clause 9.2. A UE doesn’t expect a PUCCH with SR/CSI information overlap with more than one PUCCH with HARQ-ACK information. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Support of sub-slot HARQ-ACK feedback for MTRP

Sub-slot-based HARQ-ACK feedback was agreed in RAN1 #96bis [6].

Agreements: For supporting multiple PUCCHs for HARQ-ACK within a slot for constructing HARQ-ACK codebook, support sub-slot-based HARQ-ACK feedback procedure. - A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot.

The following agreements were achieved in RAN1 #98 on separate HARQ-ACK feedback [1].

Agreement For multi-DCI based multi-TRP transmission with separate ACK/NACK feedback - UE is allowed to transmit two TDMed long PUCCHs within a slot - UE is allowed to transmit TDMed short PUCCH and long PUCCH within a slot - UE is allowed to transmit TDMed short PUCCH and short PUCCH within a slot FFS whether/how to use PRI indication with the granularity of sub-slot for eMBB with M-TRP

From the above agreement, no more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. It implies that for sub-slot-based HARQ-ACK feedback, more than one PUCCHs carrying HARQ-ACK information are allowed in a slot independent of separate ACK/NACK feedback for MTRP is configured or not. Considering the agreement in MTRP session, for multi-PDCCH-based MTRP transmission with separate HARQ-ACK feedback, we can further restrict that a UE is allowed to transmit two TDMed PUCCHs within not a sub-slot but a slot. To make it clear, we have following proposal.

Adopt following text proposal in clause 9.2 and 9.2.3 of TS 38.213.

------------------------------------------Start of Text Proposal ---------------------------------- 9.2 UCI reporting in physical uplink control channel UCI types reported in a PUCCH include HARQ-ACK information, SR, LRR, and CSI. UCI bits include HARQ-ACK information bits, if any, SR information bits, if any, LRR information bit, if any, and CSI bits, if any. The HARQ-ACK information bits correspond to a HARQ-ACK codebook as described in Clause 9.1. For the remaining of this clause, any reference to SR is applicable for SR and/or for LRR. A UE may transmit one or two PUCCHs on a serving cell in different symbols within a slot. When the UE transmits two PUCCHs in a slot and the UE is not provided ACKNACKFeedbackMode = SeparateFeedback, at least one of the two PUCCHs uses PUCCH format 0 or PUCCH format 2. If a UE is provided ACKNACKFeedbackMode = SeparateFeedback, the UE may transmit up to two PUCCHs with HARQ-ACK information in different symbols within a slot. In Clauses 9.2.3, 9.2.5.1 and 9.2.5.2, a UE assumes 11 CRC bits if a number of respective UCI bits is larger than or equal to 360; otherwise, the UE determines a number of CRC bits based on the number of respective UCI bits as described in [5, TS 38.212]. < Unchanged parts are omitted > 9.2.3 UE procedure for reporting HARQ-ACK If a UE is provided ACKNACKFeedbackMode = SeparateFeedback and it is not provided subslotLength-ForPUCCH, the UE may transmit up to two PUCCHs with HARQ-ACK information in different symbols within a slot. Otherwise, aA UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

BD/CCE for M-PDCCH based MTRP

For M-DCI based MTRP, a UE determines, for the purpose of reporting pdcch-BlindDetectionCA, a number of serving cells as Ncells,0DL+R⋅Ncells,1DL, where R=TBD if the UE does not report a value of R. We recommend the TBD value be one.

Adopt the text proposal in section 10 of TS 38.213.

------------------------------------------Start of Text Proposal ---------------------------------- 10 UE procedure for receiving control information < Unchanged parts are omitted > If a UE can support - a first set of Ncells,0DL serving cells where the UE is either not provided CORESETPoolIndex or is provided CORESETPoolIndex with a single value for all CORESETs on all DL BWPs of each serving cell from the first set of serving cells, and - a second set of Ncells,1DL serving cells where the UE is provided CORESETPoolIndex with a value 0 for a first CORESET and with a value 1 for a second CORESET on any DL BWP of each serving cell from the second set of serving cells the UE determines, for the purpose of reporting pdcch-BlindDetectionCA, a number of serving cells as Ncells,0DL+R⋅Ncells,1DL where R is either a value reported by the UE or R=TBD R=1 if the UE does not report a value of R. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Single-PDCCH Based Multi-TRP

Scheme switching

In RAN1 #99, conditions for identifying transmission schemes were agreed as the following table.

Agreement Following TCI state and joint schemes are supported TCI states; CDM groups; URLLCRepNum; URLLCSchemeEnabler; UE Behavior. 0 (in spec draft); 1; >=1; Not applicable; Not applicable; Rel 15. A (one scheme); 1; 1; Condition 1; Configured or not configured; "Scheme 4" with repetition from the same TRP Limitations agreed for Scheme 4 apply. A’ (one scheme); 1; >=1; Condition 2; Not configured; Rel 15. B (in spec draft); 2; 1; Condition 1; Not configured; Scheme 4. C (in spec draft); 2; 2; Condition 2; Not configured; 1a/NCJT. E (in spec draft); 2; 2; Condition 4; Not configured; 1a/NCJT. F (in spec draft); 2; 1; Condition 4; Configured; Scheme 2a/2b/3. D’’ (one scheme); 2; 2; Condition 4; Configured; 1a/NCJT. G’ (one scheme); 1; >=1; Condition 2; Configured; Rel 15. G (one scheme); 1; >=1; Condition 4; Configured; Rel 15. Note: - Condition 1: indicates one entry in pdsch-TimeDomainAllocationList containing URLLCRepNum (>1) in TDRA by DCI - Condition 2: indicates one entry in pdsch-TimeDomainAllocationList having no URLLCRepNum by DCI, but at least one entry having URLLCRepNum - Condition 4: None of entry in TDRA contains URLLCRepNum

In the current specification [5], condition 4, i.e., none of entry in TDRA contains RepNumR16, is missing for identifying scheme 2a (FDMSchemeA)/2b (FDMSchemeB)/c (TDMSchemeA); The condition of “RepSchemeEnabler is not configured” is missing for scheme 4. Therefore, we have following text proposal.

Adopt following text proposal in 5.1 in TS 38.214.

------------------------------------------Start of Text Proposal ---------------------------------- 5.1 UE procedure for receiving the physical downlink shared channel < Unchanged parts are omitted > If a UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE may expect to receive multiple PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain. The UE may expect the reception of fully/partially-overlapped PDSCHs in time and frequency domain only when PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of CORESETPoolIndex. For a ControlResourceSet without CORESETPoolIndex, the UE may assume that the ControlResourceSet is assigned with CORESETPoolIndex as 0. When the UE is scheduled with fully/partially/non-overlapped PDSCHs in time and frequency domain, the full scheduling information for receiving a PDSCH is indicated and carried only by the corresponding PDCCH, the UE is expected to be scheduled with the same active BWP and the same SCS. When the UE is scheduled with fully/partially-overlapped PDSCHs in time and frequency domain, the UE can be scheduled with at most two codewords simultaneously. When PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of CORESETPoolIndex, the following operations are allowed: - For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH associated with a value of CORESETpoolIndex ending in symbol i, the UE can be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH associated with a different value of CORESETpoolIndex that ends later than symbol i. - In a given scheduled cell, the UE can receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j, and a second PDSCH associated with a value of CORESETpoolindex different from that of the first PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK assigned to be transmitted in a slot before slot j. If PDCCHs that schedule corresponding PDSCHs are associated to the same or different ControlResourceSets having the same value of CORESETPoolIndex, the UE procedure for receiving the PDSCH upon detection of a PDCCH follows Clause 5.1. When a UE is configured by higher layer parameter RepSchemeEnabler set to one of ‘FDMSchemeA’, ‘FDMSchemeB’, ‘TDMSchemeA’, and it is configured with none of entries in pdsch-TimeDomainAllocationList containing RepNumR16 in PDSCH-TimeDomainResourceAllocation, if the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”. - When two TCI states are indicated in a DCI and the UE is set to ‘FDMSchemeA’, the UE shall receive a single PDSCH transmission occasion of the TB with each TCI state associated to a non-overlapping frequency domain resource allocation as described in Clause 5.1.2.3. - When two TCI states are indicated in a DCI and the UE is set to ‘FDMSchemeB’, the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping frequency domain resource allocation with respect to the other PDSCH transmission occasion as described in Clause 5.1.2.3. - When two TCI states are indicated in a DCI and the UE is set to ‘TDMSchemeA’, the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping time domain resource allocation with respect to the other PDSCH transmission occasion and both PDSCH transmission occasions shall be received within a given slot as described in Clause 5.1.2.1. When a UE is configured by the higher layer parameter PDSCH-config that indicates at least one entry in pdsch-TimeDomainAllocationList containing RepNumR16 in PDSCH-TimeDomainResourceAllocation, the UE may expect to be indicated with one or two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' together with the DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNum16 in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”. - When two TCI states are indicated in a DCI with ‘Transmission Configuration Indication’ field, and the UE is not configured with higher layer parameter RepSchemeEnabler, the UE may expect to receive multiple slot level PDSCH transmission occasions of the same TB with two TCI states used across multiple PDSCH transmission occasions as defined in Clause 5.1.2.1. - When one TCI state is indicated in a DCI with ‘Transmission Configuration Indication’ field, the UE may expect to receive multiple slot level PDSCH transmission occasions of the same TB with one TCI state used across multiple PDSCH transmission occasions as defined in Clause 5.1.2.1. < Unchanged parts are omitted > 5.1.2.1 Resource allocation in time domain < Unchanged parts are omitted > When a UE is configured by the higher layer parameter RepSchemeEnabler set to ‘TDMSchemeA’ and indicated DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, and it is configured with none of entries in pdsch-TimeDomainAllocationList containing RepNumR16 in PDSCH-TimeDomainResourceAllocation, the number of PDSCH transmission occasions is derived by the number of TCI states indicated by the DCI field 'Transmission Configuration Indication' of the scheduling DCI. < Unchanged parts are omitted > When a UE configured by the higher layer parameter PDSCH-config that indicates at least one entry in pdsch-TimeDomainAllocationList contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, - If two TCI states are indicated by the DCI field ‘Transmission Configuration Indication’ together with the DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, and the UE is not configured with higher layer parameter RepSchemeEnabler, the same SLIV is applied for all PDSCH transmission occasions, the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Conclusion

We have following proposals regarding to Rel-16 MTRP.

- Adopt the text proposal in clause 5.1.5 of TS 38.214.

------------------------------------------Start of Text Proposal ---------------------------------- 5.1.5 Antenna ports quasi co-location < Unchanged parts are omitted > The UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321] or in clause [6.1.3.x] of [10, TS 38.321], used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' in one CC/DL BWP or in a set of CCs/DL BWPs, respectively. When a set of TCI state IDs are activated for a set of CCs/DL BWPs, where the applicable list of CCs is determined by indicated CC in the activation command, the same set of TCI state IDs are applied for all DL BWPs in the indicated CCs. When two different values of CORESETPoolIndex are configured, the mapping is applied to the DCI field 'Transmission Configuration Indication' of the PDCCH corresponding to the CORESETPoolIndex indicated by the activation command. When a UE supports two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' the UE may receive an activation command, as described in clause [6.1.3.X24] of [10, TS 38.321], the activation command is used to map up to 8 combinations of one or two TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. The UE is not expected to receive more than 8 TCI states in the activation command. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

If the PDSCH is scheduled by a DCI format not having the TCI field present, the PDSCH would have one default QCL, which can be one of the two default different TCI states, e.g., the first TCI state of the two default different TCI states, or the TCI state of the CORESET for the PDCCH transmission with the DCI if the TCI state of the CORESET for the PDCCH is one of the two default TCI states.

When the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL if applicable, if the PDSCH is indicated with one TCI state by the scheduling DCI, the PDSCH would have one default QCL, which can be one of the two default different TCI states, e.g., the first TCI state of the two default different TCI states, or the indicated TCI state if it is one of the two default different TCI states, or the TCI state of the CORESET for the PDCCH transmission with the DCI if the TCI state of the CORESET for the PDCCH is one of the two default TCI states.

If the PDSCH is indicated with two TCI states and scheme 3 or scheme 4 by the scheduling DCI, and if the offset between the reception of the DL DCI and partial or all scheduled PDSCH transmission occasions are less than the threshold timeDurationForQCL, each of the PDSCH transmission occasion has one default QCL, which can be one of the two default different TCI states,

For scheme 3, the first and second TCI states of the two default TCI states or the TCI states indicated by the scheduling DCI are applied to the first and second PDSCH transmission occasions respectively. If the offset between the reception of the DL DCI and one of the two PDSCH transmission occasions is less than the threshold timeDurationForQCL, default TCI states are applied, otherwise TCI states indicated by the scheduling DCI is applied;

For scheme 4, when CycMapping is enabled, the first and second TCI states of the default TCI states or the TCI states indicated by the scheduling DCI are applied are applied to the first and second PDSCH transmission occasions, respectively, and so on. When SeqMapping is enabled, the first TCI state of the default TCI states or the TCI states indicated by the scheduling DCI is applied to the first and second PDSCH transmissions, and the second TCI state of the default TCI states or the TCI states indicated by the scheduling DCI is applied to the third and fourth PDSCH transmissions, and so on. If the offset between the reception of the DL DCI and one of the PDSCH transmission occasions is less than the threshold timeDurationForQCL, default TCI states are applied to the PDSCH transmission occasion, otherwise TCI states indicated by the scheduling DCI is applied.

Adopt text proposal in clause 5.1.5 and 5.1.2.1 of TS 38.214 given in Appendix.

For multi-TRP in cross carrier scheduling, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present,

If two different values of CORESETPoolIndex are configured, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH corresponding to the CORESETPoolIndex, which is same as the CORESETPoolIndex of PDCCH scheduling that PDSCH in the active BWP of the scheduled cell.

If at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states applicable to PDSCH in the active BWP of the scheduled cell.

If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, and if a UE configured with two different values of CORESETPoolIndex, or at least one TCI codepoint indicates two TCI states, when receiving the aperiodic CSI-RS,

if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies indicated one of the one or two of QCL assumption of the other DL signal when receiving the aperiodic CSI-RS.

if there is no other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the indicated one of the one or two of the QCL assumptions for PDSCH reception.

Adopt following text proposal in clause 5.1.4.2 of TS 38.214.

------------------------------------------Start of Text Proposal ---------------------------------- 5.1.4.2 PDSCH resource mapping with RE level granularity < Unchanged parts are omitted > - If the UE is configured by higher layer parameter PDCCH-Config with two different values of CORESETPoolIndex in ControlResourceSet and also may be configured by the higher layer parameter LTE lte -CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH: - REs indicated by the CRS pattern(s) in lte-CRS-PatternList-r16 if the PDSCH is associated with CORESETPoolIndex = 0, or the CRS pattern(s) in lte-CRS-PatternListSecond-r16 if the PDSCH is associated with CORESETPoolIndex = 1 CRSPatternList-CORESETPoolIndex for a UE supporting the capability of [separate-lte-CRS-ToMatchAround]; - REs indicated by the CRS pattern(s) in both lte-CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16, if configured, in ServingCellConfig for a UE not supporting the capability of [separate-lte-CRS-ToMatchAround]otherwise. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Adopt following text proposal in section 9 of TS 38.213.

------------------------------------------Start of Text Proposal ---------------------------------- 9 UE procedure for reporting control information < Unchanged parts are omitted > If a UE - is not provided CORESETPoolIndex or is provided CORESETPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided CORESETPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ACKNACKFeedbackMode = SeparateFeedback the UE shall separately apply the procedures described in Clauses 9.1 and 9.2.3 for reporting HARQ-ACK information associated with the first CORESETs on active DL BWP of the serving cells and for reporting HARQ-ACK information associated with the second CORESETs on active DL BWP of the serving cells. HARQ-ACK information reporting is associated with a CORESET through a reception of a PDCCH with a DCI format triggering the reporting of the HARQ-ACK information by the UE. If a PUCCH with HARQ-ACK overlaps with a PUCCH with SR/CSI information, the UE shall apply the procedures described in Clause 9.2. A UE doesn’t expect a PUCCH with SR/CSI information overlap with more than one PUCCH with HARQ-ACK information. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Adopt following text proposal in clause 9.2 and 9.2.3 of TS 38.213.

------------------------------------------Start of Text Proposal ---------------------------------- 9.2 UCI reporting in physical uplink control channel UCI types reported in a PUCCH include HARQ-ACK information, SR, LRR, and CSI. UCI bits include HARQ-ACK information bits, if any, SR information bits, if any, LRR information bit, if any, and CSI bits, if any. The HARQ-ACK information bits correspond to a HARQ-ACK codebook as described in Clause 9.1. For the remaining of this clause, any reference to SR is applicable for SR and/or for LRR. A UE may transmit one or two PUCCHs on a serving cell in different symbols within a slot. When the UE transmits two PUCCHs in a slot and the UE is not provided ACKNACKFeedbackMode = SeparateFeedback, at least one of the two PUCCHs uses PUCCH format 0 or PUCCH format 2. If a UE is provided ACKNACKFeedbackMode = SeparateFeedback, the UE may transmit up to two PUCCHs with HARQ-ACK information in different symbols within a slot. In Clauses 9.2.3, 9.2.5.1 and 9.2.5.2, a UE assumes 11 CRC bits if a number of respective UCI bits is larger than or equal to 360; otherwise, the UE determines a number of CRC bits based on the number of respective UCI bits as described in [5, TS 38.212]. < Unchanged parts are omitted > 9.2.3 UE procedure for reporting HARQ-ACK If a UE is provided ACKNACKFeedbackMode = SeparateFeedback and it is not provided subslotLength-ForPUCCH, the UE may transmit up to two PUCCHs with HARQ-ACK information in different symbols within a slot. Otherwise, aA UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Adopt the text proposal in section 10 of TS 38.213.

------------------------------------------Start of Text Proposal ---------------------------------- 10 UE procedure for receiving control information < Unchanged parts are omitted > If a UE can support - a first set of Ncells,0DL serving cells where the UE is either not provided CORESETPoolIndex or is provided CORESETPoolIndex with a single value for all CORESETs on all DL BWPs of each serving cell from the first set of serving cells, and - a second set of Ncells,1DL serving cells where the UE is provided CORESETPoolIndex with a value 0 for a first CORESET and with a value 1 for a second CORESET on any DL BWP of each serving cell from the second set of serving cells the UE determines, for the purpose of reporting pdcch-BlindDetectionCA, a number of serving cells as Ncells,0DL+R⋅Ncells,1DL where R is either a value reported by the UE or R=TBD R=1 if the UE does not report a value of R. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Adopt following text proposal in 5.1 in TS 38.214.

------------------------------------------Start of Text Proposal ---------------------------------- 5.1 UE procedure for receiving the physical downlink shared channel < Unchanged parts are omitted > If a UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE may expect to receive multiple PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain. The UE may expect the reception of fully/partially-overlapped PDSCHs in time and frequency domain only when PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of CORESETPoolIndex. For a ControlResourceSet without CORESETPoolIndex, the UE may assume that the ControlResourceSet is assigned with CORESETPoolIndex as 0. When the UE is scheduled with fully/partially/non-overlapped PDSCHs in time and frequency domain, the full scheduling information for receiving a PDSCH is indicated and carried only by the corresponding PDCCH, the UE is expected to be scheduled with the same active BWP and the same SCS. When the UE is scheduled with fully/partially-overlapped PDSCHs in time and frequency domain, the UE can be scheduled with at most two codewords simultaneously. When PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of CORESETPoolIndex, the following operations are allowed: - For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH associated with a value of CORESETpoolIndex ending in symbol i, the UE can be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH associated with a different value of CORESETpoolIndex that ends later than symbol i. - In a given scheduled cell, the UE can receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j, and a second PDSCH associated with a value of CORESETpoolindex different from that of the first PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK assigned to be transmitted in a slot before slot j. If PDCCHs that schedule corresponding PDSCHs are associated to the same or different ControlResourceSets having the same value of CORESETPoolIndex, the UE procedure for receiving the PDSCH upon detection of a PDCCH follows Clause 5.1. When a UE is configured by higher layer parameter RepSchemeEnabler set to one of ‘FDMSchemeA’, ‘FDMSchemeB’, ‘TDMSchemeA’, and it is configured with none of entries in pdsch-TimeDomainAllocationList containing RepNumR16 in PDSCH-TimeDomainResourceAllocation, if the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”. - When two TCI states are indicated in a DCI and the UE is set to ‘FDMSchemeA’, the UE shall receive a single PDSCH transmission occasion of the TB with each TCI state associated to a non-overlapping frequency domain resource allocation as described in Clause 5.1.2.3. - When two TCI states are indicated in a DCI and the UE is set to ‘FDMSchemeB’, the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping frequency domain resource allocation with respect to the other PDSCH transmission occasion as described in Clause 5.1.2.3. - When two TCI states are indicated in a DCI and the UE is set to ‘TDMSchemeA’, the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping time domain resource allocation with respect to the other PDSCH transmission occasion and both PDSCH transmission occasions shall be received within a given slot as described in Clause 5.1.2.1. When a UE is configured by the higher layer parameter PDSCH-config that indicates at least one entry in pdsch-TimeDomainAllocationList containing RepNumR16 in PDSCH-TimeDomainResourceAllocation, the UE may expect to be indicated with one or two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' together with the DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNum16 in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”. - When two TCI states are indicated in a DCI with ‘Transmission Configuration Indication’ field, and the UE is not configured with higher layer parameter RepSchemeEnabler, the UE may expect to receive multiple slot level PDSCH transmission occasions of the same TB with two TCI states used across multiple PDSCH transmission occasions as defined in Clause 5.1.2.1. - When one TCI state is indicated in a DCI with ‘Transmission Configuration Indication’ field, the UE may expect to receive multiple slot level PDSCH transmission occasions of the same TB with one TCI state used across multiple PDSCH transmission occasions as defined in Clause 5.1.2.1. < Unchanged parts are omitted > 5.1.2.1 Resource allocation in time domain < Unchanged parts are omitted > When a UE is configured by the higher layer parameter RepSchemeEnabler set to ‘TDMSchemeA’ and indicated DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, and it is configured with none of entries in pdsch-TimeDomainAllocationList containing RepNumR16 in PDSCH-TimeDomainResourceAllocation, the number of PDSCH transmission occasions is derived by the number of TCI states indicated by the DCI field 'Transmission Configuration Indication' of the scheduling DCI. < Unchanged parts are omitted > When a UE configured by the higher layer parameter PDSCH-config that indicates at least one entry in pdsch-TimeDomainAllocationList contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, - If two TCI states are indicated by the DCI field ‘Transmission Configuration Indication’ together with the DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, and the UE is not configured with higher layer parameter RepSchemeEnabler, the same SLIV is applied for all PDSCH transmission occasions, the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Appendix: Text Proposal in Clause 5.1.5 of TS38.214

------------------------------------------Start of Text Proposal ---------------------------------- 5.1.5 Antenna ports quasi co-location < Unchanged parts are omitted > Independent of the configuration of tci-PresentInDCI and tci-PresentInDCI-ForFormat1_2 in RRC connected mode, if no TCI codepoints are mapped to two different TCI states and the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). If none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. If a UE configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, for both cases, when tci-PresentInDCI is set to 'enabled' and tci-PresentInDCI is not configured in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH associated with a value of CORESETPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest CORESET-ID among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. If the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI states for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', and at least one TCI codepoint indicates two TCI states, - If the PDSCH is scheduled by a DCI format not having the TCI field present, for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the TCI state of the CORESET for the PDCCH transmission with the DCI if the TCI state of the CORESET for the PDCCH is one of the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states or the first TCI state of the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states otherwise. - if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. - If the PDSCH is scheduled by a DCI indicating one TCI state, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the indicated TCI state if it is one of the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states, or the first TCI state of TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states otherwise. - When a UE is configured by higher layer parameter RepSchemeEnabler set to ‘TDMSchemeA’, if two TCI states are indicated by the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, the first and second TCI states of the two TCI states either in the lowest codepoint among the TCI codepoints containing two different TCI states or the two TCI states indicated by the DCI are applied to the first and second PDSCH transmission occasions respectively. If the offset between the reception of the DL DCI and the PDSCH transmission occasion is less than the threshold timeDurationForQCL, TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states are applied, otherwise TCI states indicated by the DCI are applied; - If two TCI states are indicated by the DCI field ‘Transmission Configuration Indication’ together with the DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, when the value indicated by RepNumR16 in PDSCH-TimeDomainResourceAllocation equals to two, the first and second TCI state of the two TCI states either in the lowest codepoint among the TCI codepoints containing two different TCI states or indicated by the DCI are applied to the first and second PDSCH transmission occasions, respectively; when the value indicated by RepNumR16 in PDSCH-TimeDomainResourceAllocation is larger than two, the UE may be further configured to enable CycMapping or SeqMapping in RepTCIMapping. - When CycMapping is enabled, the first and second TCI states of the two TCI states either in the lowest codepoint among the TCI codepoints containing two different TCI states or indicated by the DCI are applied to the first and second PDSCH transmission occasions, respectively, and the same TCI mapping pattern continues to the remaining PDSCH transmission occasions. - When SeqMapping is enabled, first TCI state of the two TCI states either in the lowest codepoint among the TCI codepoints containing two different TCI states or indicated by the DCI is applied to the first and second PDSCH transmissions, and the second TCI state of the two TCI states either in the lowest codepoint among the TCI codepoints containing two different TCI states or indicated by the DCI is applied to the third and fourth PDSCH transmissions, and the same TCI mapping pattern continues to the remaining PDSCH transmission occasions. - If the offset between the reception of the DL DCI and the PDSCH transmission occasion is less than the threshold timeDurationForQCL, TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states are applied, otherwise TCI states indicated by the DCI is applied. < Unchanged parts are omitted > 5.1.2.1 Resource allocation in time domain < Unchanged parts are omitted > When a UE is configured by the higher layer parameter RepSchemeEnabler set to ‘TDMSchemeA’ and indicated DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, the number of PDSCH transmission occasions is derived by the number of TCI states indicated by the DCI field 'Transmission Configuration Indication' of the scheduling DCI. - If two TCI states are indicated by the DCI field ‘Transmission Configuration Indication’, the UE is expected to receive two PDSCH transmission occasions, where the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1. The second TCI state is applied to the second PDSCH transmission occasion, and tThe second PDSCH transmission occasion shall have the same number of symbols as the first PDSCH transmission occasion. If the UE is configured by the higher layers with a value K in StartingSymbolOffsetK, it shall determine that the first symbol of the second PDSCH transmission occasion starts after K symbols from the last symbol of the first PDSCH transmission occasion. If the value K is not configured via the higher layer parameter StartingSymbolOffsetK, K = 0 shall be assumed by the UE. The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 applied respectively to the first and second TCI state. - Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Clause 5.1.2.1. When a UE configured by the higher layer parameter PDSCH-config that indicates at least one entry in pdsch-TimeDomainAllocationList contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, - If two TCI states are indicated by the DCI field ‘Transmission Configuration Indication’ together with the DCI field “Time domain resource assignment’ indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation and DM-RS port(s) within one CDM group in the DCI field “Antenna Port(s)”, the same SLIV is applied for all PDSCH transmission occasions, the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1. When the value indicated by RepNumR16 in PDSCH-TimeDomainResourceAllocation equals to two, the second TCI state is applied to the second PDSCH transmission occasion. When the value indicated by RepNumR16 in PDSCH-TimeDomainResourceAllocation is larger than two, the UE may be further configured to enable CycMapping or SeqMapping in RepTCIMapping. - When CycMapping is enabled, the first and second TCI states are applied to the first and second PDSCH transmission occasions, respectively, and the same TCI mapping pattern continues to the remaining PDSCH transmission occasions. - When SeqMapping is enabled, first TCI state is applied to the first and second PDSCH transmissions, and the second TCI state is applied to the third and fourth PDSCH transmissions, and the same TCI mapping pattern continues to the remaining PDSCH transmission occasions. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

References

- RAN1 #98 Chairman's Notes.

- RAN1 #98bis Chairman's Notes.

- R1-1913463, Reply LS on multi PDCCH-based and single PDCCH-based multi-TRP operation, RAN1 #99.

- 3GPP TS 38.213 V16.1.0.

- 3GPP TS 38.214 V16.1.0.

- RAN1 #96bis Chairman's Notes.

- 3GPP TS 38.321 V16.0.0.

- R1-1911550, Reply LS on single PDCCH-based multi-TRP operation, RAN1 #98bis.

- R1-2001377, Outcome of email thread [100e-NR-eMIMO-multiTRP-01], RAN1 #100-e.

- 3GPP TS 38.331 V16.0.0.