3GPP TSG RAN WG1 Meeting #100-Bis-e	R1-2001976

e-Meeting, April 20th – 30th, 2020

Agenda item:		7.2.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Two-step RACH related LS received from RAN2

Document for:		Discussion and Decision

# Introduction

For the last RAN1 e-meeting, RAN2 sent an LS[1], which was not treated during the meeting itself, but deferred to be treated af the RAN1 100-e meeting. For this current e-meeting, RAN2 has sent three additional LS [2], [3], [4]. In this contribution we will shortly discuss these LS and how they should preferably be treated.

# Discussion

In [1] RAN2 is asking whether there is any issues or concerns with supporting support for CSI-RS based 2-step CFRA. During the discussions for Rel-16 two-step RACH, this topic was not discussed since the main focus was on defining the physical channel structure and associated procedures for facilitating the overall concept. When looking at the current specification text for 38.213 [5], the general part for the association to the CSI-RS (if configured) is already covered in section 8.1, but the actual procedural text is missing. For the 4-step RACH procedure, there is already existing text, which could incorporated in a similar way for 2-step RACH procedure. According to our understanding there should be no problem or issues with adopting this approach as well for 2-step RACH procedure. Therefore, we propose the following:

Proposal 1: Adopt the following text proposal to include support for CSI-RS based 2-step CFRA:

## 8.2A 	Random access response - Type-2 random access procedure

<Omitted text>

If the UE detects the DCI format 1_0 with CRC scrambled by a C-RNTI and a transport block in a corresponding PDSCH within the window, the UE transmits a PUCCH with HARQ-ACK information having ACK value if the UE correctly detects the transport block or NACK value if the UE incorrectly detects the transport block and the time alignment timer is running [11, TS 38.321].

If the UE detects a DCI format 1_0 with CRC scrambled by the corresponding MSGB-RNTI and receives a transport block in a corresponding PDSCH, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block or a CSI-RS resource the UE used for PRACH association, as described in Clause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format 1_0.

The UE does not expect to be indicated to transmit the PUCCH with the HARQ-ACK information at a time that is prior to a time when the UE applies a TA command that is provided by the transport block.If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure [11, TS 38.321]. If requested by higher layers, the UE is expected to transmit a PRACH no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

In [2] there is a short discussion of two alternatives for indicating dedicated MsgA PUSCH resources. The two alternatives are both providing or indicating a method for obtaining such indication. The first approach is using specific configuration values indicating the number of contention free preambles as well as a start offset to potentially be used for indicating the start offset of the contention free preambles. With this approach there two configured values will create a pointer to the dedicated resources for the CFRA procedure (both preamble and PUSCH resource). With the second approach, also two parameters are used to derive the exact start of the PUSCH resources for MsgA transmission ((PUSCH occasion index and ra-PreambleIndex). These two approaches would in essence result in the same outcome (defining the dedicated MsgA PUSCH resources), but would either be using direct signalling (approach 1), or indirect/derived signalling (approach 2). Since RAN2 further indicated that there is a majority for the first approach, and the outcome of the two approaches would be the same, Nokia would have a preference for supporting the first approach as well.

Proposal 2: Respond to RAN2 with reference to [2] that Alt1 is the preferred method for indicating the allocation of dedicated MsgA PUSCH resources.

In [3], RAN2 raises the issue as to whether the starring point of MsgB window is clear from RAN1 specification point of view. Upon observing the current specification text, there MsgB window is defined to start at the first symbol of the earliest CORESET the UE is configured to monitor, which is occurring after the last symbol of MsgA PUSCH transmission. There are no conditions capturing the behaviour in case the UE does not transmit the MsgA PUSCH due to lack of RO to PUSCH association. Since RAN1 decided to support MsgA preamble transmission even for cases without an associated PUSCH, this needs correction. Since the Type-2 random access procedure may happen either with PUSCH transmission or without PUSCH transmission, it is proposed to split the description to account for these two separate cases.

Proposal 3: Adopt the following text proposal in 38.213 to correct the starting point of MsgB window for cases where MsgA preamble is transmitted without MsgA PUSCH transmission.

## 8.2A	 Random access response - Type-2 random access procedure

In response to a transmission of a PRACH and a PUSCH, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PUSCH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

In response to a transmission of a PRACH for Type-2 random access procedure without an associated PUSCH, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the Type-2 PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

In [4], RAN2 asks RAN1 to include the agreements listed in the LS. Especially the part b is relevant in the context of 2-step RACH, as this relates to how the two LSB of the SFN are interpreted in the SuccessRAR processing for MsgB reception. This is discussed in the following.

The LS from RAN2 indicates that

RAN2 agreed a downlink assignment is valid for successful RAR reception if the two LSB bits of the SFN indicated in DCI format 1_0 scrambled by RA-RNTI or msgB-RNTI correspond to the PRACH occasion used to transmit the Random Access Preamble. RAN2 agreed that TS 38.213 should capture such validation, i.e. to determine whether a downlink assignment is valid for successful RAR reception.

To capture this in 38.213, we propose to capture the validation in the following way:

Proposal 4: Adopt the following text proposals in 38.213 to capture the successRAR validation as requested by RAN2.

## 8.2	Random access response - Type-1 random access procedure

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in Clause 10.1. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

If the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI, and the SFN indicated in DCI format 1_0 (if configured) scrambled by RA-RNTI corresponds to the PRACH occasion used to transmit the Random Access Preamble, and a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers parse the transport block for a random access preamble identity (RAPID) associated with the PRACH transmission. If the higher layers identify the RAPID in RAR message(s) of the transport block, the higher layers indicate an uplink grant to the physical layer. This is referred to as random access response (RAR) UL grant in the physical layer.

<unchanged text omitted>

## 8.2A	 Random access response - Type-2 random access procedure

In response to a transmission of a PRACH and a PUSCH, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PUSCH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding RA-RNTI, and the SFN indicated in DCI format 1_0 scrambled by MSGB-RNTI corresponds to the PRACH occasion used to transmit the Random Access Preamble, and a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

-	an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Clause 8.2 when the UE detects a RAR UL grant, or

-	transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where

-	a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon

-	a slot for the PUCCH transmission is indicated by a PDSCH-to-HARQ_feedback timing indicator field of 3 bits in the successRAR having a value k from {1, 2, 3, 4, 5, 6, 7, 8} and, with reference to slots for PUCCH transmission having duration Tslot, the slot is determined as ceiln+k+∆+t∆Tslot where n is a slot of the PDSCH reception, ∆ is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214], and t∆≥0

-	the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5+t∆ msec where NT,1 is the PDSCH processing time for UE processing capability 1 [6, TS 38.214]

-	the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission

# Conclusion

In this contribution we propose the following:

Proposal 1: Adopt the following text proposal to include support for CSI-RS based 2-step CFRA:

## 8.2A 	Random access response - Type-2 random access procedure

<Omitted text>

If the UE detects the DCI format 1_0 with CRC scrambled by a C-RNTI and a transport block in a corresponding PDSCH within the window, the UE transmits a PUCCH with HARQ-ACK information having ACK value if the UE correctly detects the transport block or NACK value if the UE incorrectly detects the transport block and the time alignment timer is running [11, TS 38.321].

If the UE detects a DCI format 1_0 with CRC scrambled by the corresponding MSGB-RNTI and receives a transport block in a corresponding PDSCH, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block or a CSI-RS resource the UE used for PRACH association, as described in Clause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format 1_0.

The UE does not expect to be indicated to transmit the PUCCH with the HARQ-ACK information at a time that is prior to a time when the UE applies a TA command that is provided by the transport block.If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure [11, TS 38.321]. If requested by higher layers, the UE is expected to transmit a PRACH no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

Proposal 2: Respond to RAN2 with reference to [2] that Alt1 is the preferred method for indicating the allocation of dedicated MsgA PUSCH resources.

Proposal 3: Adopt the following text proposal in 38.213 to correct the starting point of MsgB window for cases where MsgA preamble is transmitted without MsgA PUSCH transmission.

## 8.2A	 Random access response - Type-2 random access procedure

In response to a transmission of a PRACH and a PUSCH, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PUSCH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

In response to a transmission of a PRACH for Type-2 random access procedure without an associated PUSCH, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the Type-2 PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

Proposal 4: Adopt the following text proposals in 38.213 to capture the successRAR validation as requested by RAN2.

## 8.2	Random access response - Type-1 random access procedure

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in Clause 10.1. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

If the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI, and the SFN indicated in DCI format 1_0 (if configured) scrambled by RA-RNTI corresponds to the PRACH occasion used to transmit the Random Access Preamble, and a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers parse the transport block for a random access preamble identity (RAPID) associated with the PRACH transmission. If the higher layers identify the RAPID in RAR message(s) of the transport block, the higher layers indicate an uplink grant to the physical layer. This is referred to as random access response (RAR) UL grant in the physical layer.

<unchanged text omitted>

## 8.2A	 Random access response - Type-2 random access procedure

In response to a transmission of a PRACH and a PUSCH, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PUSCH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding RA-RNTI, and the SFN indicated in DCI format 1_0 scrambled by MSGB-RNTI corresponds to the PRACH occasion used to transmit the Random Access Preamble, and a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

-	an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Clause 8.2 when the UE detects a RAR UL grant, or

-	transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where

-	a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon

-	a slot for the PUCCH transmission is indicated by a PDSCH-to-HARQ_feedback timing indicator field of 3 bits in the successRAR having a value k from {1, 2, 3, 4, 5, 6, 7, 8} and, with reference to slots for PUCCH transmission having duration Tslot, the slot is determined as ceiln+k+∆+t∆Tslot where n is a slot of the PDSCH reception, ∆ is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214], and t∆≥0

-	the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5+t∆ msec where NT,1 is the PDSCH processing time for UE processing capability 1 [6, TS 38.214]

-	the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission

# References

- R1-2001236, “LS to RAN1 on Support of 2-step CFRA”, RAN2, ZTE

- R1-2001508, ”LS to RAN1 on preamble-to-PRU mapping for 2-step CFRA	”, RAN2, Ericsson

- R1-2001511, ”LS to RAN1 on the starting point of MSGB window”, RAN2, ZTE

- R1-2001506, ”LS on random access procedure in NR-U”, RAN2, InterDigital

- TS 38.213, v. 16.0.0, ” Physical layer procedures for control (Release 16)”, 3GPP