3GPP TSG RAN WG1 Meeting #100bis		R1-2002293

e-Meeting, April 20th – 30th, 2020

Agenda item:       7.2.6.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance on Rel-16 CSI enhancements

Document for:     Discussion and Decision

# 1	Introduction

In this contribution we briefly discuss two text proposals to 38.214 intended to clarify the conditions on the PUSCH for which low-latency aperiodic CSI timing requirements apply and the processing of an aperiodic CSI report occupies all NCPU CPUs.

# 2	FD basis indicator for Mν=1

In subclause 5.2.2.2.5 of TS38.214, a formula is provided to compute the value of the FD basis indicator i1,6,l from the FD component indices n3,l(f), when N3≤19, which reads

| i1,6,l=f=1Mυ-1C2Mυ-1-n3,lf,Mυ-f. | (1) |
| --- | --- |

An algorithm to invert this formula is also provided, which calculates the FD component indices n3,l(f) from the FD basis indicator i1,6,l.

When N3≤19, there are possible codebook configurations resulting in Mν=1, i.e., the FD basis consists of a single component index n3,l(0)=0. In this case, both formula (1) and the inversion algorithm are undefined (f=0). In fact, the indicator i1,6,l should not be reported because the only FD component, n3,l(0)=0, is always included in the FD basis and known to the gNB. However, the current codebook description does not specify the UE’s behaviour in this case.

This problem can be easily fixed, for example, by the following text proposal

TP to TS 38.214, Sec. 5.2.2.2.5: ## <Unchanged text is omitted> For all values of N3, n3,l(0)=0 for l=1,…,υ. The nonzero elements of n3,l, identified by n3,l(1), …, n3,lMυ-1, are found from i1,6,l (l=1,…,υ), if reported, for N3≤19, and from i1,6,l (l=1,…,υ) and Minitial, for N3>19, using Cx,y as defined in Table 5.2.2.2.5-4 and the algorithm: s0=0 for f=1,…,Mυ-1 Find the largest x*∈Mυ-1-f,…,N3-1-f in Table 5.2.2.2.5-4 such that i1,6,l -sf-1≥Cx*,Mυ-f ef=Cx*,Mυ-f sf=sf-1+ef if N3≤19 n3,l(f)=N3-1-x* else nl(f)=2Mυ-1-x* if nl(f)≤Minitial+2Mυ-1 n3,l(f)=nl(f) else n3,l(f)=nl(f)+(N3-2Mυ) end if end if ## <Unchanged text is omitted> When n3,l and Minitial are known, i1,5 and i1,6,l (l=1,…,υ) are found as follows: - If N3≤19, i1,5=0 and is not reported. If Mν=1, i1,6,l=0, for l=1,…,ν, and is not reported. If Mν>1, i1,6,l=f=1Mυ-1CN3-1-n3,lf,Mυ-f, where Cx,y is given in Table 5.2.2.2.5-4 and where the indices f=1,…,Mυ-1 are assigned such that n3,l(f) increases as f increases. - If N3>19, Minitial is indicated by i1,5, which is reported and given by i1,5=MinitialMinitial=0 Minitial+2MυMinitial<0 Only the nonzero indices n3,l(f)∈IntS, where IntS={Minitial+i mod N3, i=0,1,…,2Mυ-1}, are reported, where the indices f=1,…,Mυ-1 are assigned such that n3,l(f) increases as f increases. Let nl(f)=n3,lf n3,lf≤Minitial+2Mυ-1 n3,lf-(N3-2Mυ)n3,lf>Minitial+N3- 1, then i1,6,l=f=1Mυ-1C2Mυ-1-nlf,Mυ-f, where Cx,y is given in Table 5.2.2.2.5-4. <Unchanged text is omitted>

# 3	On the minimum value of K0

In subclause 5.2.2.2.5 there is no lower limit specified to the value that parameter K0 can take. Therefore, given the definition

| K0=β2LM1 | (2) |
| --- | --- |

there exist possible codebook configurations for which the value of K0 is so small that

case 1. 	No coefficient amplitude/phase can be reported for some layer, i.e. K0<2 for reported rank ν≤2, or K0<ν  for rank 2<ν≤4

or

case 2. 	Not all the selected beams can be used in a layer, i.e. K0<L for rank ν≤2, or K0<νL2  for rank 2<ν≤4.

Both situations can arise for some parameter combinations when the number of subbands, NSB, in the CSI reporting band is small, namely for NSB≤4.

The last column of Table 1 shows the minimum value of K0 and the number of subbands for which it occurs, for all supported parameter combinations.

Table 1. Minimum value of K0 and number of subbands NSB for which it is achieved for all parameter combinations.

| paramCombination-r16 | L | pυ |  | β | min K0 (NSB) |
| --- | --- | --- | --- | --- | --- |
|  |  | υ ∈1,2 | υ ∈3,4 |  |  |
| 1 | 2 | ¼ | 1/8 | ¼ | 1 (NSB≤4) |
| 2 | 2 | ¼ | 1/8 | ½ | 2 (NSB≤4) |
| 3 | 4 | ¼ | 1/8 | ¼ | 2 (NSB≤4) |
| 4 | 4 | ¼ | 1/8 | ½ | 4 (NSB≤4) |
| 5 | 4 | ¼ | ¼ | ¾ | 6 (NSB≤4) |
| 6 | 4 | ½ | ¼ | ½ | 4 (NSB≤2) |
| 7 | 6 | ¼ | - | ½ | 6 (NSB≤4) |
| 8 | 6 | ¼ | - | ¾ | 9 (NSB≤4) |

Note that it is possible for a UE not to report any nonzero combination coefficients for some polarisations or beams, if it chooses to do so, however when case 1 above occurs, a UE is forced to do so regardless of a UE’s preferred PMI calculation. Besides, in case 1, some reported indicators become redundant: for example, if no amplitude/phase coefficients can be reported for a layer the bitmap, which is still reported, serves no useful purpose.

Similarly, a UE could decide, as a result of a PMI calculation, not to report any coefficient for certain beams in a layer. However, if this behaviour is forced by the CSI report configuration, like in case 2, because K0 is too small to report at least one nonzero coefficient for all L beams, effectively it is as if a lower value of L was configured.

The critical configurations described here are not typical and may be regarded as corner cases, however the issue can be easily fixed by introducing a minimum value of K0, for example by modifying the definition of K0 as follows

| K0=maxx,β2LM1 | (3) |
| --- | --- |

where x can be chosen from one the following alternatives:

1a. 	x=2, to ensure that a UE can report one NZC per polarisation for rank ν≤2

1b. 	x=max⁡{2,ν}, to ensure that a UE can report one NZC per polarisation for rank ν≤4

2a. 	x=L, to ensure that a UE can report one NZC for each selected beam for rank ν≤2

2b. 	x=L2⋅max⁡{2,ν}, to ensure that a UE can report one NZC for each selected beam for rank for ν≤4

# 4	PUSCH conditions for maximum CPU occupancy and low-latency timing requirement in A-CSI report

This is a TP to capture a shadow CR for Rel-16, corresponding to the CR in [4] for Rel-15.

In subclauses 5.2.1.6 and 5.4 of TS38.214, similar wording is used to describe the conditions of the PUSCH for which the maximum number of NCPU CPUs are occupied by an A-CSI report (Sec. 5.2.1.6) and the low latency timing requirements of Table 5.4-1 apply (Sec. 5.4).

In subclause 5.2.1.6 the sentence reads:

“if a CSI report is aperiodically triggered without transmitting a PUSCH with either transport block or HARQ-ACK or both...”

whilst in subclause 5.4 the words used are:

“if the CSI is triggered without a PUSCH with either transport block or HARQ-ACK or both”

The correct interpretation, confirmed by the session notes in [1]-[3], is that the maximum CPU occupancy of an A-CSI report and the low-latency timing requirements apply when the PUSCH carries no transport block nor HARQ-ACK, i.e., it only carries a single A-CSI report.

However, the choice of words used to describe these conditions, “without... with either..., or...., or both”, may lead to a different interpretation. Namely that the maximum CPU occupancy of an A-CSI report and the low-latency timing requirements apply when the PUSCH carries only one of either HARQ-ACK or transport block, or none of them, i.e., it carries a single CSI report + HARQ-ACK, or a single CSI report + transport block, or only a single CSI report.

To remove this ambiguity, we suggest the following text proposals

TP to TS 38.214, Sec. 5.2.1.6: ## <Unchanged text is omitted> A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings.Processing of a CSI report occupies a number of CPUs for a number of symbols as follows: - OCPU=0 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info configured - OCPU=1 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR', 'ssb-Index-SINR' or 'none' (and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured) - for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI', - if a CSI report is aperiodically triggered with a PUSCH with no transport block and no HARQ-ACK when L = 0 CPUs are occupied, where the CSI corresponds to a single CSI with wideband frequency-granularity and to at most 4 CSI-RS ports in a single resource without CRI report and where codebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', OCPU=NCPUOCPU=NCPU, - otherwise, OCPU=KsOCPU=KS, where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement. ## <Unchanged text is omitted>

TP to TS 38.214, Sec. 5.4: ## <Unchanged text is omitted> Z, Z' and µ are defined as: Z=maxm=0,…,M-1(Z(m))⁡ and Z'=maxm=0,…,M-1(Z'(m)), where M is the number of updated CSI report(s) according to Clause 5.2.1.6, (Z(m),Z'(m)) corresponds to the m-th updated CSI report and is defined as - (Z1,Z1') of the table 5.4-1 if the CSI is triggered with a PUSCH with no transport block and no HARQ-ACK when L = 0 CPUs are occupied (according to Clause 5.2.1.6) and the CSI to be transmitted is a single CSI and corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or - (Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or - (Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the reportQuantity is set to 'ssb-Index-SINR', or reportQuantity is set to 'cri-SINR', or - (Z3,Z3') of the table 5.4-2 if reportQuantity is set to 'cri-RSRP' or 'ssb-Index-RSRP', where Xµ is according to UE reported capability beamReportTiming and KBl is according to UE reported capability beamSwitchTiming as defined in [13, TS 38.306], or - (Z2,Z2') of table 5.4-2 otherwise. ## <Unchanged text is omitted>

# 5	Clarification on which UE capability component indicates NCPU

This is a TP to capture a shadow CR for Rel-16, corresponding to the CR in [5] for Rel-15.

In subclause 5.2.1.6 of TS38.214, the parameter NCPU is defined as the number of simultaneous CSI calculations supported by a UE. The text specifies that this parameter is indicated by the UE, but it does not say by which UE capability component it is indicated.

It seems common undertanding that this paramer is indicated by components simultaneousCSI-ReportsPerCC and simultaneousCSI-ReportsAllCC of FG 2-35. However, the name and description of these components refer to the number of simultaneous CSI reports rather then CSI calculations a UE can process. Because a single CSI report may require multiple CSI calculations, without some clarification a UE may under report its NCPU capability as illustrated in the following case.

For a single CSI report configured with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI', and that is not a low-latency report, the number of simultaneous CSI calculations (i.e., occupied CPUs) equals the number of CSI-RS resources, Ks, in the resource set. Therefore, it is not clear if a UE indicating simultaneousCSI-ReportsPerCC=1 can support this CSI report if Ks>1.

The proposed text clarifies that the number of simultaneous CSI calculations supported by a UE is indicated by simultaneousCSI-ReportsPerCC for a single CC and by simultaneousCSI-ReportsAllCC across all CCs in case of carrier aggregation.

If a clarification is not added, a gNB may not be able to schedule a CSI report that requires multiple simultaneous CSI calculations, if the reported capabilities simultaneousCSI-ReportsPerCC and simultaneousCSI-ReportsAllCC indicate the number of supported simultaneous CSI reports instead of the number of simultaneous CSI calculations.

TP to TS 38.214, Sec. 5.2.1.6: The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports across all configured cells. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Subclause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L holds. A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings.Processing of a CSI report occupies a number of CPUs for a number of symbols as follows: - OCPU=0 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info configured - OCPU=1 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP' or 'none' (and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured) - for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI', - if a CSI report is aperiodically triggered without transmitting a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied, where the CSI corresponds to a single CSI with wideband frequency-granularity and to at most 4 CSI-RS ports in a single resource without CRI report and where codebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', OCPU=NCPUOCPU=NCPU, - otherwise, OCPU=KsOCPU=KS, where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement. ## <Unchanged text is omitted>

# 6	Conclusion

In this contribution, we proposed a couple four text proposals to TS38.214, summarised as follows.

Text Proposal 1. 	Fix an undefined UE’s behaviour regarding the reporting of the FD basis indicator i1,6,l, when Mν=1

Text Proposal 2.	Modify the definition of  K0=maxx,β2LM1, by introducing a minimum value, x, from one of the following alternatives

1a. 	x=2, to ensure that a UE can report one NZC per polarisation for rank ν≤2

1b. 	x=max⁡{2,ν}, to ensure that a UE can report one NZC per polarisation for rank ν≤4

2a. 	x=L, to ensure that a UE can report one NZC for each selected beam for rank ν≤2

2b. 	x=L2⋅max⁡{2,ν}, to ensure that a UE can report one NZC for each selected beam for rank for ν≤4

Text Proposal 3.	[Shadow CR from Rel 15] Clarification on the number of occupied CPUs and delay requirements for CSI reports using PUSCH

Text Proposal 4.	[Shadow CR from Rel 15] Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations NCPU

# References

- 3GPP, “RAN1 Chairman's Notes RAN1#93,” Busan, Korea, May 2018

- Ad-hoc chair (Samsung), R1-1807785 “Chairman’s notes of AI 7.1.2 MIMO,” Busan, Korea, May 2018

- Ericsson, R1-1807810, “CSI reporting offline session notes Thursday morning,” Busan, Korea, May 2018

- Nokia/NSB, R1-2002291, “Clarification on the number of occupied CPUs and delay requirements for CSI reports using PUSCH,” RAN1#100bis-e, April 2020

- Nokia/NSN, R1-2002292, “Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations NCPU,” RAN1#100bis-e, April 2020