3GPP TSG RAN WG1 #100bis	R1-2002385

e-Meeting, April 20th – 30th, 2020

Source:	Sharp

Title:	Remaining issues on wideband operation for NR-U

Agenda Item:	7.2.2.2.5

Document for:	Discussion and Decision

Background

The following conclusion and agreements were achieved at RAN1#99 meeting [1].

Agreement: - The RRC parameters intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 include a mechanism to indicate that no intra-carrier guard-bands are configured - Note: This configuration may be used for the case where transmission only occurs in a BWP if LBT is successful in all RB sets within the BWP - For a carrier with intra-carrier guard bands, the UE does not expect that the dedicated BWP is configured to include parts of a RB set.

Discussion

RB set configuration for no GB case

As agreed above, the higher layer parameter intraCellGuardBand-r16 will include a mechanism to indicate that no intra-cell guard-bands are configured. For this case, whether/how to define RB-sets is FFS.

RB-sets have various use cases. For downlink reception, available RB-sets are indicated by DCI format 2_0 to accommodate PDSCH reception by the UE. For LBT purpose, availability of Cat.2 LBT in frequency domain is indicated through the available RB-sets. For UL scheduling, the PUCCH/PUSCH resource is indicated by RB-sets.

Even if no intra-cell guard-bands are configured for a carrier, an RB-set should be defined for cross-carrier indication of available RB-sets. For example, if one carrier without intra-cell guard-bands are configured in an SCell, DCI format 2_0 in another carrier configured in the PCell should have a functionality to indicate whether the SCell is available or not. Thus, an RB-set (i.e. equivalent to the carrier) should be defined for indicating the SCell is available or not.

Proposal 1: An RB-set which includes all RBs in a carrier is defined for the carrier without intra-cell guard-bands.

Proposal 2: Adopt the Text proposal #1.

Text proposal #1 --------- beginning of text proposal for TS 38.214 - 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set-1 intra-cell guard bands on a carrier, each defined by a starting and an ending CRB, GB sstart,μ and GB send,μ , respectively. The UE determines NRB-set RB-sets, each defined by a starting and an ending CRB, RB sstart,μ and RB send,μ, respectively. UE determines RB 0start,μ=Ngridstart,μ, RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ, and the remaining starting and ending CRBs as RB send,μ=GB sstart,μ-1 and RB s+1start,μ=GB send,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard bands and corresponding RB-sets according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard bands and corresponding RB-sets according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. For a carrier with intra-cell guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with an RB-set. RB-sets within a BWP form a set SRB-sets of cardinality NRB-setBWP. -------- Unchanged contents are omitted --------- end of text proposal

UL/DL separation

Some of the parameter names for RB-sets and intra-cell guard bands should be clarified as follows.

Proposal 3: Adopt the Text proposal #2.

Text proposal #2 --------- beginning of text proposal for TS 38.214 - 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier, each defined by start and end CRB, GB s,xstart,μ and GB s,xend,μ , respectively. The intra-cell guard bands separate NRB-set,x RB-sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set,x-1end,μ=Ngrid,xstart,μ+Ngrid,xsize,μ , and the remaining start and end CRBs as RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xend,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngrid,xsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngrid,xsize,μ]. For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets,x of cardinality NRB-set,xBWP. The subscript x for NRB-set,x, GB s,xstart,μ, GB s,xend,μ, RB s,xstart,μ, RB s,xend,μ and SRB-sets,x are set to DL for downlink, and set to UL for uplink. When there is no risk for confusion, the subscript x may be dropped. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] -------- Unchanged contents are omitted --------- end of text proposal

PDCCH candidate and CCE mapping for SS set configuration with freqMonitorLocations-r16

In RAN1#100_e, Qualcomm [2] provided the text proposal to capture “For a search space set configuration configured with freqMonitorLocations-r16, the number of PDCCH candidates to be monitored (i.e., nrofCandidates or nrofCandidates-SFI) is assigned for each RB set.” We also believe it should be captured. The only necessary change is to modify the PDCCH candidates to which the hashing function applies.

Proposal 4: Adopt the following Text proposal #3.

Text proposal #3 --------- beginning of text proposal for TS 38.213 - 10.1 UE procedure for determining physical downlink control channel assignment -------- Unchanged contents are omitted For a search space set associated with CORESET , - if the UE is not provided with freqMonitorLocations-r16, the CCE indexes for aggregation level corresponding to PDCCH candidate of the search space set in slot for an active DL BWP of a serving cell corresponding to carrier indicator field value - if the UE is provided with freqMonitorLocations-r16 and if the (k+1)’th bit in the bitmap in freqMonitorLocations-r16 is ‘1’, in RB set k, the CCE indexes for aggregation level corresponding to PDCCH candidate of the search space set in slot for an active DL BWP of a serving cell corresponding to carrier indicator field value are given by where for any CSS, ; for a USS, , , for , for , for , and ; ; is the number of CCEs, numbered from 0 to , in CORESET ; is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, ; , where is the number of PDCCH candidates the UE is configured to monitor for aggregation level of a search space set for a serving cell corresponding to ; for any CSS, ; for a USS, is the maximum of over all configured values for a CCE aggregation level of search space set ; the RNTI value used for is the C-RNTI. -------- Unchanged contents are omitted --------- end of text proposal

Conclusion

In this contribution, we have the following proposals:

Proposal 1: An RB-set which includes all RBs in a carrier is defined for the carrier without intra-cell guard-bands.

Proposal 2: Adopt Text proposal #1.

Proposal 3: Adopt Text proposal #2.

Proposal 4: Adopt Text proposal #3.

References

- Chairman’s note at RAN1#99

- R1-2000960, “TP for wideband operation for NR-U”, Qualcomm Incorporated, RAN1#100_e, March, 2020.