3GPP TSG-RAN WG1 #100bis-e		R1-2002413

e-Meeting, April 20th – 30th, 2020

Source:	Ericsson

Title:	RAN1 aspects of UL Tx switching

Agenda Item:	5.1

Document for:	Discussion and Decision

# Introduction

In this document, we discuss some remaining RAN1 aspects for the below objective of the RAN4 led WI on RF requirements for NR frequency range 1 (FR1).

# Discussion

## Impact on PUSCH preparation time

Several aspects related to impact on UE PUSCH preparation procedure were discussed in RAN1#100-eMeeting and related agreements are captured in [2].

Regarding how to capture the increased PUSCH preparation time in the specification, our preference is to increment N2 by the length of switching duration, i.e., replace N2 with N2' where N2'=N2+dtxs and  with being the UE capability reported for length of UL switching period as agreed in RAN4 [1].

In addition to the update for Tproc,2, it should also be clarified whether the increased N2 is also used in computation of Tproc,2mux and aperiodic SRS switching delay.

Proposal 1

- For capturing impact of increased PUSCH processing time, replace N2 with N2' where N2'=N2+dtxs and  with being the UE capability reported for length of UL switching period.

Observation 1

- It should be clarified whether the increased N2 is also used in computation of Tproc,2mux and aperiodic SRS switching delay.

## Condition and Presence of switching period for UL CA

This issue was also discussed in RAN1#100-eMeeting and related agreements are captured in [3]. The main open issue for this area is the below Proposal (from [3]) related to Option 1 and Option 2

Proposal 1:

- For inter-band UL CA, make down-selection on the following two options in RAN1#100bis:

- Option 1: If uplink Tx switching is configured, UE is not expected to be scheduled or configured with UL transmission on carrier 2 for case 1.

  - Semi-static switching between uplink Tx switching mode and normal UL CA operation is supported by RRC reconfiguration.

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |

- Option 2: If uplink Tx switching is configured, UE can be scheduled or configured with UL transmission on both carrier 1 and carrier 2 for case 1.

  - UE can be scheduled or configured with UL transmission on either carrier 1 or carrier 2

  - UE can be scheduled or configured with UL transmission on both carrier 1 and carrier 2 simultaneously

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P, 1P+1P, [0P+1P] |
| Case 2 | 0T+2T | 0P+2P, [0P+1P] |

- If no consensus on the down-selection in RAN1#100bis, both options are supported, UE can report which option is supported based on UE capability. UE can report via capability signaling which Option (between Option 1 and Option 2) is supported for the case when uplink Tx switching is configured.

As discussed in detail in our previous contribution [4], and email discussion [100e-5.1LS-TxSwitching-03], our preference is to support Option 2.

In RAN1#100-eMeeting, a ‘compromise proposal’ to support both Option 2 and Option 1 as different UE capabilities was discussed. If such capability signaling is introduced, we prefer to define the capability as shown in Proposal 3.

Proposal 2

- Option 2 (i.e., “UE can be scheduled UL transmission on both carrier 1 and carrier 2 for case 1 simultaneously”) should be supported for defining the condition and presence of switching periods for UL tx switching with CA case.

Proposal 3

If UE capability between Option 1 and Option 2 is introduced, the capability is defined as follows:

- For an inter-band band-combination for which the UE indicates support for UL CA (i.e., “CA case”)

  - Introduce additional UE capability to indicate the supported UE behavior when UL Tx switching is configured for the UE

  - The supported UE behavior can be according to Option 1 or Option 2

    - Option 1: When configured for UL Tx switching, the UE is not expected to be scheduled or configured with UL transmission on both carrier 1 and carrier 2 simultaneously.

    - Option 2: When configured for UL Tx switching, the UE can be scheduled or configured with UL transmission on both carrier 1 and carrier 2 for case 1.

      - UE can be scheduled or configured with UL transmission on either carrier 1 or carrier 2

      - UE can be scheduled or configured with UL transmission on both carrier 1 and carrier 2 simultaneously

# Conclusions

In this document we discuss some remaining RAN1 aspects of UL tx switching and make the following proposals and observation.

Proposal 1

- For capturing impact of increased PUSCH processing time, replace N2 with N2' where N2'=N2+dtxs and  with being the UE capability reported for length of UL switching period.

Observation 1

- It should be clarified whether the increased N2 is also used in computation of Tproc,2mux and aperiodic SRS switching delay.

Proposal 2

- Option 2 (i.e., “UE can be scheduled UL transmission on both carrier 1 and carrier 2 for case 1 simultaneously”) should be supported for defining the condition and presence of switching periods for UL tx switching with CA case.

Proposal 3

If UE capability between Option 1 and Option 2 is introduced, the capability is defined as follows:

- For an inter-band band-combination for which the UE indicates support for UL CA (i.e., “CA case”)

  - Introduce additional UE capability to indicate the supported UE behavior when UL Tx switching is configured for the UE

  - The supported UE behavior can be according to Option 1 or Option 2

    - Option 1: When configured for UL Tx switching, the UE is not expected to be scheduled or configured with UL transmission on both carrier 1 and carrier 2 simultaneously.

    - Option 2: When configured for UL Tx switching, the UE can be scheduled or configured with UL transmission on both carrier 1 and carrier 2 for case 1.

      - UE can be scheduled or configured with UL transmission on either carrier 1 or carrier 2

      - UE can be scheduled or configured with UL transmission on both carrier 1 and carrier 2 simultaneously

# References

[1] R1-2001522 – “LS on Tx switching between two uplink carriers2”, RAN4  RAN1,RAN2 LS, RAN1#100bis-e-Meeting, April 2020.

[2] R1-2001274 – “[100e-5.1LS-TxSwitching-02] Email discussion/approval on remaining issues on PUSCH preparation procedure”, China Telecom, RAN1#100-e, February 2020.

[3] R1-2001275 – “[100e-5.1LS-TxSwitching-03] Email discussion/approval on remaining issues on inter-band UL CA”, China Telecom, RAN1#100-e, February 2020.

[4] R1-2000883 – “RAN1 aspects of UL Tx switching”, Ericsson, RAN1#100-e, February 2020.