3GPP TSG RAN WG1 Meeting #100bis-e	                                                                          R1-2002534

April 20th – April 30th, 2020

Agenda item:    7.2.2.2.5

Source:             Qualcomm Incorporated

Title:                  TP for wideband operation for NR-U

Document for:  Discussion and Decision

# Introduction

This paper lists TPs for a few issues identified for agenda item “wideband operation for NR-U”.

# BWP configuration limitation

It is agreed that UE does not expect the BWP configure to be partially overlapping with defined RB sets. This is captured in 38.214 section 7. There is an error in computing RBNRB-set-1end,μ, and it is preferred to capture the restriction on BWP starting and ending positions accurately.

====TP for 38.214 7==============

7	UE procedures for transmitting and receiving on a carrier with intra-cell guard bands

For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with  NRB-set-1  intra-cell guard bands on a carrier, each defined by start and end CRB, GB sstart,μ  and  GB send,μ , respectively. The intra-cell guard bands separate  NRB-set RB-sets, each defined by start and end CRB, RB sstart,μ and  RB send,μ, respectively.  UE determines RB 0start,μ=Ngridstart,μ, RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ-1  , and the remaining start and end CRBs as RB send,μ=GB sstart,μ-1 and RB s+1start,μ=GB send,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ].

For a carrier with intra-carrier guard bands, the UE expects RBBWP,istart, μ=RBs0start,μ , and RBBWP,isize, μ=RBs1end,μ-RBs0start,μ+1 for 0≤s0≤s1≤NRB-set-1.SRB-setsNRB-setBWP

[The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.]

========================

# Available RB set bitmap in COT-SI

When the active BWP contains multiple LBT bandwidth, for indication of available LBT bandwidth, a bitmap can be configured in the DCI 2_0 with each bit corresponds to one LBT bandwidth. There are a few issues we identified

- In current version of 38.213, when the bit in the bitmap indicates ‘0’ or ‘1’, the corresponding RB set is considered as available or unavailable respectively. It may be more intuitive to use ‘0’ and ‘1’ for ‘unavailable’ and ‘available’ respectively

- For the COT-SI transmission at the beginning of a COT, since the COT-SI encoding needs processing time, the DCI format 2_0 cannot carry valid available RB set information. A default value is needed to indicate the available RB set information is not yet available. We propose to use all 0 bitmap to indicate that.

- Also need to consider the case that the bitmap is not configured, in which case, the only UE assumption makes sense is assume all RB sets available when the DCI 2_0 is detected.

============TP for 38.213 Section 11.1.1====================================

--Unchanged part omitted------------------------

For each serving cell in the set of serving cells, the UE can be provided:

-	an identity of the serving cell by servingCellId

-	a location of a SFI-index field in DCI format 2_0 by positionInDCI

-	a set of slot format combinations by slotFormatCombinations, where each slot format combination in the set of slot format combinations includes

-	one or more slot formats indicated by a respective slotFormats for the slot format combination, and

-	a mapping for the slot format combination provided by slotFormats to a corresponding SFI-index field value in DCI format 2_0 provided by slotFormatCombinationId

-	for unpaired spectrum operation, a reference SCS configuration  by subcarrierSpacing and, when a supplementary UL carrier is configured for the serving cell, a reference SCS configuration  by subcarrierSpacing2 for the supplementary UL carrier

-	for paired spectrum operation, a reference SCS configuration  for a DL BWP by subcarrierSpacing and a reference SCS configuration  for an UL BWP by subcarrierSpacing2

-	for operation with shared spectrum channel access, a location of a bitmap in DCI format 2_0, having a one-to-one mapping with a set of RB sets [6, TS 38.214] of the serving cell, where a value of '0' indicates that an RB set is available for receptions and a value of '1' indicates that an RB set is not available for receptions, by availableRB-SetPerCell-r16, and the bitmap in DCI format 2_0. The RB set remains available or unavailable until the end of the indicated channel occupancy duration. When all bits in the bitmap are ‘1’, in the availableRB-SetPerCell-r16, the availability for all RB sets for reception are considered as unknown, till another DCI format 2_0 is received.

-	When availableRB-SetPerCell-r16 is not configured for a serving cell configured with CO-DurationPerCell-r16 or slotFormatCombinations, the UE considered all RB sets available for reception when the DCI format 2_0 is detected

-	a location and a length of a channel occupancy duration field in DCI format 2_0, by CO-DurationPerCell-r16, that indicates a remaining channel occupancy duration for the serving cell starting from a slot where the UE detects the DCI format 2_0 by providing a value from CO-DurationList-r16. If CO-DurationPerCell-r16 is not provided, the remaining channel occupancy duration for the serving cell is a number of slots, starting from a slot where the UE detects the DCI format 2_0, that the SFI-index field value provides corresponding slot formats

-	a location of a search space set group switching field in DCI format 2_0, by SearchSpaceSwitchTrigger-r16, that indicates a group from two groups of search space sets for PDCCH monitoring for scheduling on the serving cell as described in Clause 10.4.

--Unchanged part omitted------------------------

===============================================================