3GPP TSG RAN WG1 Meeting #100bis-e 	R1-2002587

E-meeting, April 20 - April 30, 2020

Agenda Item:	7.2.11.8

Source:	Huawei, HiSilicon

Title:	Rel-16 UE features for NR positioning

Document for:	Discussion and Decision

Introduction

In RAN#87-e, a preliminary discussion on UE capability signal provided an informative summary in [1] clarifying the terminology used for Rel-16 UE capability discussion.

- Terminology definitions based on Rel-15 (TR38.822) - “Feature(s)”: It is a highest level grouping. In Rel-16, it is per-WI grouping. - “Feature group(s)”: It is a kind of “subfeature(s)” within a “feature”, and is defined by each row in the UE features list. - “Component(s)”: One feature group contains one or multiple components. When UE reports support of the feature group, basically it is applied to all components in the feature group.

It also clarified how to handle the so-called basic feature groups/components, used as “baseline feature bits that covers the minimum parts of a feature and use additional bits for parts that can be used together with the baseline feature” in the previous RAN2 guideline in [2].

Based on the guideline from RAN2 [2] and information from RAN [1], we think the Rel-15 UE capability methodology is reused for Rel-16. However, for NR positioning part, we observed some discrepancy between the Rel-15 approach and the current UE feature list.

Feature group

In the following, we will propose to address the following issues

- Keep a single “type” for components within a group

In Rel-15, a feature group (FG) is mapped to a single field/IE in TS 38.306 and TS 38.311 that contains all component fields listed. A dedicated signaling granularity is associated with a parent IE, with the following example.

| Signaling granularity type | Corresponding exemplary RAN2 parent IE |
| --- | --- |
| Per UE | Phy-Parameters MeasAndMobParameters |
| Per band | BandNR |
| Per band per band combination | FeatureSetDownlink FeatureSetUplink |
| Per CC per band per band combination | FeatureSetDownlinkPerCC FeatureSetUplinkPerCC |

It is thus not preferred to put components with potentially different signal granularity type under a single FG.

Proposal 1: The feature groups for positioning needs to be restructured to comply with Rel-15 grouping convention.

- Consider using the feature list in Appendix A as a baseline.

Values for the features

Due to the page limit, the recommended values of the components in each feature group based on Appendix A can be found in Appendix B.

New capability proposals

Simultaneous processing of LTE and NR PRS

Since processing both LTE and NR PRS will require UE processing capability, it would be reasonable to provide UE capability to inform LMF whether UE supports it. If UE does not support it, it is expected that LMF will not trigger both LTE OTDOA positioning and NR DL/RTT positioning at the same time, or the performance requirements for both LTE OTDOA and NR DL/RTT positioning will not be met at the same time.

Proposal 2: Add the following UE capability under common NR DL PRS processing feature.

| 13. Common NR DL PRS processing | 13-x | Concurrent LTE/NR PRS processing | Support of concurrent processing of LTE PRS and NR PRS | FFS 13-1 | Yes |  | Network does not know whether OTDOA and NR DL/RTT positioning can be used together. | 1) Per UE | No | No | NA |  | Optional with capability signalling {supported, notSupported} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Maximum number of SRS on a symbol for intra-band CA

As proposed in our companion contribution [5], we suggest to add the capability of maximum number of SRS on symbol for intra-band CA. The capability is reported per band, and the number should include at least {1, 2}.

Proposal 3: Add the following UE capability under SRS for positioning.

| 13d. NR Multi-RTT 13e. SRS for positioning | 13d-x 13e-y | Simultaneous transmission | Max number of SRS resources for positioning on a symbol for intra-band CA | FFS | Yes |  | Network does not know whether it is allowed to configure SRS for positioning across CC for intra-band CA | 2) Per band | No | No | NA |  | Optional with capability signalling {1, 2} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Maximum number of SRS resources in a slot per BWP

As proposed in our companion contribution [5], we suggest to add the capability of maximum number of SRS resources in a slot per BWP. The capability is reported per [band], following the guideline from [2] to reduce FS or FSPC. The definition itself is per BWP, and the number should include at least {1, 2}.

Proposal 4: Add the following UE capability under SRS for positioning.

| 13d. NR Multi-RTT 13e. SRS for positioning | 13d-x 13e-y | SRS resources in a slot per BWP | Max number of SRS resources for positioning in a slot per BWP | FFS | Yes |  | Network does not know whether it is allowed to configure SRS for positioning in a slot | 2) Per band | No | No | NA |  | Optional with capability signalling {1, 2} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Conclusions

In this contribution, we discuss the feature groups for NR positioning, and propose the values for UE to report. In addition, some new capability proposals are presented. Below are the proposals.

Proposal 1: The feature groups for positioning needs to be restructured to comply with Rel-15 grouping convention.

- Consider using the feature list in Appendix A as a baseline.

Proposal 2: Add the following UE capability under common NR DL PRS processing feature.

| 13. Common NR DL PRS processing | 13-x | Concurrent LTE/NR PRS processing | Support of concurrent processing of LTE PRS and NR PRS | FFS 13-1 | Yes |  | Network does not know whether OTDOA and NR DL/RTT positioning can be used together. | 1) Per UE | No | No | NA |  | Optional with capability signalling {supported, notSupported} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: Add the following UE capability under SRS for positioning.

| 13d. NR Multi-RTT 13e. SRS for positioning | 13d-x 13e-y | Simultaneous transmission | Max number of SRS resources for positioning on a symbol for intra-band CA | FFS | Yes |  | Network does not know whether it is allowed to configure SRS for positioning across CC for intra-band CA | 2) Per band | No | No | NA |  | Optional with capability signalling {1, 2} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: Add the following UE capability under SRS for positioning.

| 13e. SRS for positioning | 13d-x 13e-y | SRS resources in a slot per BWP | Max number of SRS resources for positioning in a slot per BWP | FFS | Yes |  | Network does not know whether it is allowed to configure SRS for positioning in a slot | 2) Per band | No | No | NA |  | Optional with capability signalling {1, 2} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- References

3GPP RP-200502, Informational summary on email discussion: [Rel16_UE_capabilities] Exchange of views, NTT DOCOMO, RAN#87-e, e-Meeting, 16 – 19 Mar., 2020.

3GPP R2-2002378, Guidelines for UE capability definitions, RAN2, RAN2#109e, Electronic meeting, 24 Feb – 6 Mar., 2020.

3GPP TS 37.355, LTE Positioning Protocol (LPP).

R1-2001484, RAN1 UE features list for Rel-16 NR after RAN1#100-E, AT&T, NTT DOCOMO, RAN1#100-e, e-Meeting, 24 Feb. – 6 Mar., 2020.

R1-2001559, Maintenance of SRS for NR positioning, Huawei, HiSilicon, RAN1#100bis-e, 20 Apr. – 30  Apr., 2020.

- Appendix A.

In this appendix, we provide the proposed feature groups.

| 13. Common NR DL PRS processing | 13-0 | FFS Basic PRS processing | FFS whether or not to introduce basic FG. |
| --- | --- | --- | --- |
|  | 13-1 | PRS processing capability | - Maximum DL PRS bandwidth in MHz, supported by UE. Values = [20, 50, 100, 200, 400] in MHz - Duration of DL PRS symbol in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. Values for T = [0.125, 0.25, 0.5, 1, 40, 80, 160, 320, 640, 1280] ms |
| 13a. NR E-CID | 13a-1 | NR E-CID DL SSB RRM measurements for NR Positioning | Configuration and reporting of SSB RRM measurements (SS-RSRP, SS-RSRQ) based on Rel.15 for NR Positioning in Rel.16 |
|  | 13a-2 | NR E-CID DL CSI-RS RRM measurements for NR Positioning | Configuration and reporting of CSI-RS for RRM (CSI-RSRP, CSI-RSRQ) for NR Positioning in Rel.16 |
| 13b. NR DL-AoD | 13b-0 | FFS Basic DL-AoD positioning | FFS whether or not to introduce basic FG. |
|  | 13b-1 | PRS configuration | - Max number of positioning frequency layers supported by UE for DL PRS measurement report. Values = {1, [2, 3], 4} - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE for DL PRS measurement report. Values = {1,2} - Max number of DL PRS Resources per DL PRS Resource Set a UE can be configured for DL PRS measurement report. Values = [1, 4, 8, 16, 32, 64] - Max number of DL PRS Resources supported by UE for DL PRS measurement report across all frequency layers, TRPs and DL PRS Resource Sets. Values = [64, 128, 192, 256, 512, 1024, 2048] - Max number of TRPs across all positioning frequency layers per UE for DL PRS measurement report. Values = [16, 32, 64, 128, 256] - Max number of DL PRS Resources per positioning frequency layer for DL PRS measurement report. Values = [32, 64, 128, 256, 512, 1024] - [Max number of DL PRS resources per TRP across all frequency layers. Value set: {4,8,16,32,64,128}] |
|  | 13b-2 | Multiple RSRP report | Max number of DL PRS RSRP measurements on different PRS resources from the same TRP supported by the UE Values = {1, 2, 3, 4, 5, 6, 7, 8} |
|  | 13b-3 | Inter-frequency measurement | Support of inter-frequency DL PRS RSRP measurement report in RRC_CONNECTED state |
|  | 13b-4 | PRS QCL support | - [Support of SSB from serving cell as QCL Type C source of a DL PRS resource from serving cell] - [Support of SSB from neighbour cells as QCL Type C source of a DL PRS resource from neighbour cells] - [Support of SSB from serving cell as QCL Type D source of a DL PRS resource from serving cell] - [Support of SSB from neighbour cells as QCL Type D source of a DL PRS resource from neighbour cells] - [Support of a DL PRS resource from serving cell as QCL Type D source of another DL PRS resource from serving cell] - [Support of a DL PRS resource from neighbour cells as QCL Type D source of another DL PRS resource from neighbour cells] |
| 13c. NR DL-TDOA | 13c-0 | FFS Basic DL-TDOA positioning | FFS whether or not to introduce basic FG. |
|  | 13c-1 | PRS configuration | - Max number of positioning frequency layers supported by UE for DL PRS measurement report. Values = {1, [2, 3], 4} - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE for DL PRS measurement report. Values = {1,2} - Max number of DL PRS Resources per DL PRS Resource Set a UE can be configured for DL PRS measurement report. Values = [1, 4, 8, 16, 32, 64] - Max number of DL PRS Resources supported by UE for DL PRS measurement report across all frequency layers, TRPs and DL PRS Resource Sets. Values = [64, 128, 192, 256, 512, 1024, 2048] - Max number of TRPs across all positioning frequency layers per UE for DL PRS measurement report. Values = [16, 32, 64, 128, 256] - Max number of DL PRS Resources per positioning frequency layer for DL PRS measurement report. Values = [32, 64, 128, 256, 512, 1024] - [Max number of DL PRS resources per TRP across all frequency layers. Value set: {4,8,16,32,64,128}] |
|  | 13c-2 | Multiple RSTD report | Max number of DL RSTD measurements per pair of TRPs. Values = {1, 2, 3, 4}. |
|  | 13c-3 | Inter-frequency measurement | Support of inter-frequency DL RSTD/DL PRS RSRP measurement report in RRC_CONNECTED state |
|  | 13c-4 | PRS QCL support | - [Support of SSB from serving cell as QCL Type C source of a DL PRS resource from serving cell] - [Support of SSB from neighbour cells as QCL Type C source of a DL PRS resource from neighbour cells] - [Support of SSB from serving cell as QCL Type D source of a DL PRS resource from serving cell] - [Support of SSB from neighbour cells as QCL Type D source of a DL PRS resource from neighbour cells] - [Support of a DL PRS resource from serving cell as QCL Type D source of another DL PRS resource from serving cell] - [Support of a DL PRS resource from neighbour cells as QCL Type D source of another DL PRS resource from neighbour cells] |
|  | 13c-5 | Quality report | [Support of DL RSTD measurement quality metric] |
| 13d. NR Multi-RTT | 13d-0 | FFS Basic Multi-RTT positioning | FFS whether or not to introduce basic FG. |
|  | 13d-1 | PRS configuration | - Max number of positioning frequency layers supported by UE for DL PRS measurement report. Values = {1, [2, 3], 4} - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE for DL PRS measurement report. Values = {1,2} - Max number of DL PRS Resources per DL PRS Resource Set a UE can be configured for DL PRS measurement report. Values = [1, 4, 8, 16, 32, 64] - Max number of DL PRS Resources supported by UE for DL PRS measurement report across all frequency layers, TRPs and DL PRS Resource Sets. Values = [64, 128, 192, 256, 512, 1024, 2048] - Max number of TRPs across all positioning frequency layers per UE for DL PRS measurement report. Values = [16, 32, 64, 128, 256] - Max number of DL PRS Resources per positioning frequency layer for DL PRS measurement report. Values = [32, 64, 128, 256, 512, 1024] - [Max number of DL PRS resources per TRP across all frequency layers. Value set: {4,8,16,32,64,128}] |
|  | 13d-2 | Multiple UE Rx – Tx time difference report | Max number of UE Rx – Tx time difference measurements per TRP DL PRS Resource Set/Resource |
|  | 13d-3 | Intra/Inter-frequency measurement | - Support of UE Rx – Tx time difference measurement with serving cell - Support of UE Rx – Tx time difference measurement with neighbouring cells - Support of UE Rx – Tx time difference measurements across different positioning frequency layers for DL PRS processing - [Support of UE Rx-Tx time difference measurements across different component carriers for SRS for positioning. |
|  | 13d-4 | PRS QCL support | - [Support of SSB from serving cell as QCL Type C source of a DL PRS resource from serving cell] - [Support of SSB from neighbour cells as QCL Type C source of a DL PRS resource from neighbour cells] - [Support of SSB from serving cell as QCL Type D source of a DL PRS resource from serving cell] - [Support of SSB from neighbour cells as QCL Type D source of a DL PRS resource from neighbour cells] - [Support of a DL PRS resource from serving cell as QCL Type D source of another DL PRS resource from serving cell] - [Support of a DL PRS resource from neighbour cells as QCL Type D source of another DL PRS resource from neighbour cells] |
|  | 13d-5 | Quality report | [Support of UE Rx – Tx time difference measurement quality metric] |
|  | 13d-6 | SRS configuration | - Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 16}. Other values FFS - Max number of aperiodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. - Max number of semi-persistent SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. - Max number of periodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. |
|  | 13d-7 | Open loop power control | - Support of SSB for serving cell DL pathloss estimation and OLPC for SRS for positioning. - Support of SSB for neighbour cell(s) DL pathloss estimation and OLPC for SRS for positioning. - Support of DL PRS for serving cell DL pathloss estimation and OLPC for SRS for positioning. - Support of DL PRS for neighbour cell(s) DL pathloss estimation and OLPC for SRS for positioning. |
|  | 13d-8 | Number of pathloss estimates | Max number of pathloss estimates across all SRS resource sets for positioning in addition to the up to four pathloss estimates per serving cell specified for PUSCH/PUCCH/SRS transmission. It is indicated from the following set of values {0, [1], 4, 8, 16} |
|  | 13d-9 | SRS Tx beamforming | - Support of spatial relation of SRS Resources for positioning with DL PRS Resources from serving cell - Support of spatial relation of SRS Resources for positioning with DL PRS Resources from neighbour cells - Support of spatial relation of SRS Resources for positioning with SSB from serving cell - Support of spatial relation of SRS Resources for positioning with SSB from neighbour cells - Support of spatial Relation of SRS Resources for positioning with other SRS resources |
|  | 13d-10 | Number of spatial relations | [Max number of active spatial relations including both DL PRS Resources and SSB. Values = [FFS]] |
| 13e. SRS for positioning | 13e-0 | FFS Basic SRS for positioning | FFS whether or not to introduce basic FG. |
|  | 13e-1 | SRS configuration | - Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 16}. Other values FFS - Max number of aperiodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. - Max number of semi-persistent SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. - Max number of periodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. |
|  | 13e-2 | Open loop power control | - Support of SSB for serving cell DL pathloss estimation and OLPC for SRS for positioning. - Support of SSB for neighbour cell(s) DL pathloss estimation and OLPC for SRS for positioning. - Support of DL PRS for serving cell DL pathloss estimation and OLPC for SRS for positioning. - Support of DL PRS for neighbour cell(s) DL pathloss estimation and OLPC for SRS for positioning. |
|  | 13e-3 | Number of pathloss estimates | Max number of pathloss estimates across all SRS resource sets for positioning in addition to the up to four pathloss estimates per serving cell specified for PUSCH/PUCCH/SRS transmission. It is indicated from the following set of values {0, [1], 4, 8, 16} |
|  | 13e-4 | SRS Tx beamforming | - Support of spatial relation of SRS Resources for positioning with DL PRS Resources from serving cell - Support of spatial relation of SRS Resources for positioning with DL PRS Resources from neighbour cells - Support of spatial relation of SRS Resources for positioning with SSB from serving cell - Support of spatial relation of SRS Resources for positioning with SSB from neighbour cells - Support of spatial Relation of SRS Resources for positioning with other SRS resources |
|  | 13e-5 | Number of spatial relations | [Max number of active spatial relations including both DL PRS Resources and SSB. Values = [FFS]] |

- Appendix B.

In this appendix, we provide detailed values each FG.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13. Common NR DL PRS processing | 13-0 | FFS Basic PRS processing | FFS whether or not to introduce basic FG. |  |  |  |  |  |  |  |  |  |  |
|  | 13-1 | PRS processing capability | - Maximum DL PRS bandwidth in MHz, supported by UE. Values = [20, 50, 100, 200, 400] in MHz - Duration of DL PRS symbol in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. Values for T = [0.125, 0.25, 0.5, 1, 40, 80, 160, 320, 640, 1280] ms | FFS 13-0 | Yes |  | Network does not know how to provide the assistance data to the UE | 2) Per band | No | Yes | NA | - UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value - UE DL PRS processing capability is defined for a single positioning frequency layer - UE DL PRS processing capability is agnostic to DL PRS comb factor configuration - FFS if UE DL PRS processing capability is agnostic to the configured SCS settings of DL PRS - FFS if reported values of T are the same across bands within a FR or across FRs - FFS cases w/ and w/o configuration of measurement gap | Optional with capability signalling #1: {5MHz, 20MHz, 50MHz, 100MHz} for FR1 bands, {50MHz, 100MHz, 200MHz, 400MHz} for FR2 bands #2: N = {0.5, 1, 2, 4, 5, 6, 8, 10, 15, 20, 40} ms T = {1, 2, 5, 10, 20, 40, 80, 160, 320, 640, 1280[, equalToN]} ms |
| 13a. NR E-CID | 13a-1 | NR E-CID DL SSB RRM measurements for NR Positioning | Configuration and reporting of SSB RRM measurements (SS-RSRP, SS-RSRQ) based on Rel.15 for NR Positioning in Rel.16 | [FG 1-1] | Yes |  | LMF may not trigger RRM measurement report in E-CID based on SSB | 1) Per UE | No | No | NA | Note: No new UE capability is assumed on top of the R15 capabilities. | Optional with capability signalling {supported, notSupported} |
|  | 13a-2 | NR E-CID DL CSI-RS RRM measurements for NR Positioning | Configuration and reporting of CSI-RS for RRM (CSI-RSRP, CSI-RSRQ) for NR Positioning in Rel.16 | [FG 1-4 FG 1-5] | Yes |  | LMF may not trigger RRM measurement report in E-CID based on CSI-RS | 1) Per UE | No | No | NA | Note: No new UE capability is assumed on top of the R15 capabilities. | Optional with capability signalling {supported, notSupported} |
| 13b. NR DL-AoD | 13b-0 | FFS Basic DL-AoD positioning | FFS whether or not to introduce basic FG. |  |  |  |  |  |  |  |  |  |  |
|  | 13b-1 | PRS configuration | - Max number of positioning frequency layers supported by UE for DL PRS measurement report. Values = {1, [2, 3], 4} - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE for DL PRS measurement report. Values = {1,2} - Max number of DL PRS Resources per DL PRS Resource Set a UE can be configured for DL PRS measurement report. Values = [1, 4, 8, 16, 32, 64] - Max number of DL PRS Resources supported by UE for DL PRS measurement report across all frequency layers, TRPs and DL PRS Resource Sets. Values = [64, 128, 192, 256, 512, 1024, 2048] - Max number of TRPs across all positioning frequency layers per UE for DL PRS measurement report. Values = [16, 32, 64, 128, 256] - Max number of DL PRS Resources per positioning frequency layer for DL PRS measurement report. Values = [32, 64, 128, 256, 512, 1024] - [Max number of DL PRS resources per TRP across all frequency layers. Value set: {4,8,16,32,64,128}] | FFS 13b-0 | Yes |  | Network does not know how to provide the assistance data | 1) Per UE | No | Yes | FFS |  | Optional with capability signalling #1: {1,2,3,4} for FR1 and FR2 #2: {1,2} for both FR1 and FR2 #3: {2, 8} for FR1, {8, 64} for FR2. #4: {6, 24, 128, 512} for FR1, {24, 96, 512, 2048} for FR2. #5: {3, 12, 64, 256} for FR1 and FR2. #6: {6, 24, 128} for FR1, and {24, 96, 512} for FR2 #7: FFS |
|  | 13b-2 | Multiple RSRP report | Max number of DL PRS RSRP measurements on different PRS resources from the same TRP supported by the UE Values = {1, 2, 3, 4, 5, 6, 7, 8} | FFS 13b-0 | No |  | At most 2 RSRP measurements per TRP will be requested. | 1) Per UE | No | No | NA |  | Optional with capability signalling {2,4,8} |
|  | 13b-3 | Inter-frequency measurement | Support of inter-frequency DL PRS RSRP measurement report in RRC_CONNECTED state | FFS 13b-0 | No |  | Only intra-frequency RSRP measurement is supported. | 1) Per UE | No | No | NA |  | Optional with capability signalling {supported, notSupported} |
|  | 13b-4 | PRS QCL support | - [Support of SSB from serving cell as QCL Type C source of a DL PRS resource from serving cell] - [Support of SSB from neighbour cells as QCL Type C source of a DL PRS resource from neighbour cells] - [Support of SSB from serving cell as QCL Type D source of a DL PRS resource from serving cell] - [Support of SSB from neighbour cells as QCL Type D source of a DL PRS resource from neighbour cells] - [Support of a DL PRS resource from serving cell as QCL Type D source of another DL PRS resource from serving cell] - [Support of a DL PRS resource from neighbour cells as QCL Type D source of another DL PRS resource from neighbour cells] | FFS 13b-0 | No |  | LMF may or may not configure QCL information with PRS. | 1) Per UE | No | No | NA | This feature group is not needed from our perspective. | Optional with capability signalling {supported, notSupported} |
| 13c. NR DL-TDOA | 13c-0 | FFS Basic DL-TDOA positioning | FFS whether or not to introduce basic FG. |  |  |  |  |  |  |  |  |  |  |
|  | 13c-1 | PRS configuration | - Max number of positioning frequency layers supported by UE for DL PRS measurement report. Values = {1, [2, 3], 4} - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE for DL PRS measurement report. Values = {1,2} - Max number of DL PRS Resources per DL PRS Resource Set a UE can be configured for DL PRS measurement report. Values = [1, 4, 8, 16, 32, 64] - Max number of DL PRS Resources supported by UE for DL PRS measurement report across all frequency layers, TRPs and DL PRS Resource Sets. Values = [64, 128, 192, 256, 512, 1024, 2048] - Max number of TRPs across all positioning frequency layers per UE for DL PRS measurement report. Values = [16, 32, 64, 128, 256] - Max number of DL PRS Resources per positioning frequency layer for DL PRS measurement report. Values = [32, 64, 128, 256, 512, 1024] - [Max number of DL PRS resources per TRP across all frequency layers. Value set: {4,8,16,32,64,128}] | FFS 13c-0 | Yes |  | Network does not know how to provide the assistance data | 1) Per UE | No | Yes | FFS |  | Optional with capability signalling #1: {1,2,3,4} for FR1 and FR2 #2: {1,2} for both FR1 and FR2 #3: {1, 8} for FR1, {8, 64} for FR2. #4: {3, 24, 128, 512} for FR1, {24, 96, 512, 2048} for FR2. #5: {3, 12, 64, 256} for FR1 and FR2. #6: {3, 24, 128} for FR1, and {24, 96, 512} for FR2 #7: FFS |
|  | 13c-2 | Multiple RSTD report | Max number of DL RSTD measurements per pair of TRPs. Values = {1, 2, 3, 4}. | FFS 13c-0 | No |  | Single RSTD measurement per TRP pair will be requested. | 1) Per UE | No | No | NA | Note: This is a max number of DL RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets. All the RSTD measurements in a single report should have a single reference timing. | Optional with capability signalling {1,2,3,4} |
|  | 13c-3 | Inter-frequency measurement | Support of inter-frequency DL RSTD/DL PRS RSRP measurement report in RRC_CONNECTED state | FFS 13c-0 | No |  | Only intra-frequency RSTD/RSRP measurement is supported. | 1) Per UE | No | No | NA |  | Optional with capability signalling {supported, notSupported} |
|  | 13c-4 | PRS QCL support | - [Support of SSB from serving cell as QCL Type C source of a DL PRS resource from serving cell] - [Support of SSB from neighbour cells as QCL Type C source of a DL PRS resource from neighbour cells] - [Support of SSB from serving cell as QCL Type D source of a DL PRS resource from serving cell] - [Support of SSB from neighbour cells as QCL Type D source of a DL PRS resource from neighbour cells] - [Support of a DL PRS resource from serving cell as QCL Type D source of another DL PRS resource from serving cell] - [Support of a DL PRS resource from neighbour cells as QCL Type D source of another DL PRS resource from neighbour cells] | FFS 13c-0 | No |  | LMF may or may not configure QCL information with PRS. | 1) Per UE | No | No | NA | This feature group is not needed from our perspective. | Optional with capability signalling {supported, notSupported} |
|  | 13c-5 | Quality report | [Support of DL RSTD measurement quality metric] | FFS 13c-0 | No |  | UE will not provide RSTD quality in the measurement report | 1) Per UE | No | No | NA | This feature group is not needed from our perspective. | Optional with capability signalling {supported, notSupported} |
| 13d. NR Multi-RTT | 13d-0 | FFS Basic Multi-RTT positioning | FFS whether or not to introduce basic FG. |  |  |  |  |  |  |  |  |  |  |
|  | 13d-1 | PRS configuration | - Max number of positioning frequency layers supported by UE for DL PRS measurement report. Values = {1, [2, 3], 4} - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE for DL PRS measurement report. Values = {1,2} - Max number of DL PRS Resources per DL PRS Resource Set a UE can be configured for DL PRS measurement report. Values = [1, 4, 8, 16, 32, 64] - Max number of DL PRS Resources supported by UE for DL PRS measurement report across all frequency layers, TRPs and DL PRS Resource Sets. Values = [64, 128, 192, 256, 512, 1024, 2048] - Max number of TRPs across all positioning frequency layers per UE for DL PRS measurement report. Values = [16, 32, 64, 128, 256] - Max number of DL PRS Resources per positioning frequency layer for DL PRS measurement report. Values = [32, 64, 128, 256, 512, 1024] - [Max number of DL PRS resources per TRP across all frequency layers. Value set: {4,8,16,32,64,128}] |  | Yes |  | Network does not know how to provide the assistance data | 1) Per UE | No | Yes | FFS |  | Optional with capability signalling #1: {1,2,3,4} for FR1 and FR2 #2: {1,2} for both FR1 and FR2 #3: {1, 8} for FR1, {8, 64} for FR2. #4: {3, 24, 128, 512} for FR1, {24, 96, 512, 2048} for FR2. #5: {3, 12, 64, 256} for FR1 and FR2. #6: {3, 24, 128} for FR1, and {24, 96, 512} for FR2 #7: FFS |
|  | 13d-2 | Multiple UE Rx – Tx time difference report | Max number of UE Rx – Tx time difference measurements per TRP DL PRS Resource Set/Resource | FFS 13d-0 | No |  | Single UE Rx – Tx time difference measurement per TRP will be requested. | 1) Per UE | No | No | NA |  | Optional with capability signalling {1,2,3,4} |
|  | 13d-3 | Intra/Inter-frequency measurement | - Support of UE Rx – Tx time difference measurement with serving cell - Support of UE Rx – Tx time difference measurement with neighbouring cells - Support of UE Rx – Tx time difference measurements across different positioning frequency layers for DL PRS processing - [Support of UE Rx-Tx time difference measurements across different component carriers for SRS for positioning. | FFS 13d-0 | No |  | Only UE Rx – Tx time difference measurement with serving cell is supported. PRS and SRS are from the same cell. | 1) Per UE | No | No | NA | #3: Note: Covers scenario when DL PRS are processed across different DL PRS frequency layers associated with a given component carrier used for SRS for positioning #4: Note: Covers scenario when SRS for positioning is transmitted in different component carriers than the component carrier to which DL PRS is configured] | Optional with capability signalling {supported, notSupported} |
|  | 13d-4 | PRS QCL support | - [Support of SSB from serving cell as QCL Type C source of a DL PRS resource from serving cell] - [Support of SSB from neighbour cells as QCL Type C source of a DL PRS resource from neighbour cells] - [Support of SSB from serving cell as QCL Type D source of a DL PRS resource from serving cell] - [Support of SSB from neighbour cells as QCL Type D source of a DL PRS resource from neighbour cells] - [Support of a DL PRS resource from serving cell as QCL Type D source of another DL PRS resource from serving cell] - [Support of a DL PRS resource from neighbour cells as QCL Type D source of another DL PRS resource from neighbour cells] | FFS 13d-0 | No |  | LMF may or may not configure QCL information with PRS. | 1) Per UE | No | No | NA | This feature group is not needed from our perspective. | Optional with capability signalling {supported, notSupported} |
|  | 13d-5 | Quality report | [Support of UE Rx – Tx time difference measurement quality metric] | FFS 13d-0 | No |  | UE will not provide UE Rx – Tx time difference quality in the measurement report | 1) Per UE | No | No | NA | This feature group is not needed from our perspective. | Optional with capability signalling {supported, notSupported} |
|  | 13d-6 | SRS configuration | - Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 16}. Other values FFS - Max number of aperiodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. - Max number of semi-persistent SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. - Max number of periodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. | FFS 13d-0 | Yes |  | LMF will not know how to request SRS configuration from the serving gNB. Serving gNB will not know how to configure SRS. | 1) Per UE | No | Yes | NA | Support of AP SRS is still under discussion in RAN2. | Optional with capability signalling. #1: {1,2,4} for FR1, and {1,4,16} for FR2. #2: {0,1,2,4} for FR1, and {0,4,16,64} for FR2. #3: {0,1,2,4} for FR1, and {0,4,16,64} for FR2 #4: {1,2,4} for FR1 and {4,16,64} for FR2. |
|  | 13d-7 | Open loop power control | - Support of SSB for serving cell DL pathloss estimation and OLPC for SRS for positioning. - Support of SSB for neighbour cell(s) DL pathloss estimation and OLPC for SRS for positioning. - Support of DL PRS for serving cell DL pathloss estimation and OLPC for SRS for positioning. - Support of DL PRS for neighbour cell(s) DL pathloss estimation and OLPC for SRS for positioning. | FFS 13d-0 | No |  | SRS pathloss will only be based on the Rel-15 reference signal from the serving cell. | 1) Per UE | No | Yes | NA |  | Optional with capability signalling. {supported, notSupported} |
|  | 13d-8 | Number of pathloss estimates | Max number of pathloss estimates across all SRS resource sets for positioning in addition to the up to four pathloss estimates per serving cell specified for PUSCH/PUCCH/SRS transmission. It is indicated from the following set of values {0, [1], 4, 8, 16} | FFS 13d-0, 13d-7 | No |  | SRS pathloss will only be based on the DL reference signal from the serving cell. | 2) Per band | No | Yes | NA |  | Optional with capability signalling. {0,4,8,16} |
|  | 13d-9 | SRS Tx beamforming | - Support of spatial relation of SRS Resources for positioning with DL PRS Resources from serving cell - Support of spatial relation of SRS Resources for positioning with DL PRS Resources from neighbour cells - Support of spatial relation of SRS Resources for positioning with SSB from serving cell - Support of spatial relation of SRS Resources for positioning with SSB from neighbour cells - Support of spatial Relation of SRS Resources for positioning with other SRS resources | FFS 13d-0 | No |  | SRS spatial relation will only be based on the Rel-15 reference signal from the serving cell. | 1) Per UE | No | Yes | NA |  | Optional with capability signalling. {supported, notSupported} |
|  | 13d-10 | Number of spatial relations | [Max number of active spatial relations including both DL PRS Resources and SSB. Values = [FFS]] | FFS 13d-0, 13d-9 | No |  | The spatial relation of SRS for positioning will only be based spatial relations for Rel-15 SRS/PUSCH/PUCCH. | 2) Per band | No | Yes | NA |  | Optional with capability signalling {0,4,8,16} |
| 13e. SRS for positioning | 13e-0 | FFS Basic SRS for positioning | FFS whether or not to introduce basic FG. |  |  |  |  |  |  |  |  |  |  |
|  | 13e-1 | SRS configuration | - Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 16}. Other values FFS - Max number of aperiodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. - Max number of semi-persistent SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. - Max number of periodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. | FFS 13e-0 | Yes |  | Serving gNB will not know how to configure SRS. | 1) Per UE | No | Yes | NA | Support of AP SRS is still under discussion in RAN2. | Optional with capability signalling. #1: {1,2,4} for FR1, and {1,4,16} for FR2. #2: {0,1,2,4} for FR1, and {0,4,16,64} for FR2. #3: {0,1,2,4} for FR1, and {0,4,16,64} for FR2 #4: {1,2,4} for FR1 and {4,16,64} for FR2. |
|  | 13e-2 | Open loop power control | - Support of SSB for serving cell DL pathloss estimation and OLPC for SRS for positioning. - Support of SSB for neighbour cell(s) DL pathloss estimation and OLPC for SRS for positioning. - Support of DL PRS for serving cell DL pathloss estimation and OLPC for SRS for positioning. - Support of DL PRS for neighbour cell(s) DL pathloss estimation and OLPC for SRS for positioning. | FFS 13e-0 | No |  | SRS pathloss will only be based on the Rel-15 reference signal from the serving cell. | 1) Per UE | No | Yes | NA |  | Optional with capability signalling. {supported, notSupported} |
|  | 13e-3 | Number of pathloss estimates | Max number of pathloss estimates across all SRS resource sets for positioning in addition to the up to four pathloss estimates per serving cell specified for PUSCH/PUCCH/SRS transmission. It is indicated from the following set of values {0, [1], 4, 8, 16} | FFS 13e-0, 13e-2 | No |  | SRS pathloss will only be based on the DL reference signal from the serving cell. | 2) Per band | No | Yes | NA |  | Optional with capability signalling. {0,4,8,16} |
|  | 13e-4 | SRS Tx beamforming | - Support of spatial relation of SRS Resources for positioning with DL PRS Resources from serving cell - Support of spatial relation of SRS Resources for positioning with DL PRS Resources from neighbour cells - Support of spatial relation of SRS Resources for positioning with SSB from serving cell - Support of spatial relation of SRS Resources for positioning with SSB from neighbour cells - Support of spatial Relation of SRS Resources for positioning with other SRS resources | FFS 13e-0 | No |  | SRS spatial relation will only be based on the Rel-15 reference signal from the serving cell. | 1) Per UE | No | Yes | NA |  | Optional with capability signalling. {supported, notSupported} |
|  | 13e-5 | Number of spatial relations | [Max number of active spatial relations including both DL PRS Resources and SSB. Values = [FFS]] | FFS 13e-0, 13e-4 | No |  | The spatial relation of SRS for positioning will only be based spatial relations for Rel-15 SRS/PUSCH/PUCCH. | 2) Per band | No | Yes | NA |  | Optional with capability signalling {0,4,8,16} |