3GPP TSG RAN WG1 #100bis	R1-2002611

e-Meeting, April 20th – 30th, 2020

Agenda item:	7.2.10.4

Source:	Moderator (Nokia)

Title:	FL summary on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH

Document for:	Discussion and Decision

# 1	Introduction

This contribution is a pre-meeting summary of the 6 documents submitted for the AI7.2.10.4 on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH.

# 2 Summary of issues addressed in the Tdocs

## 2.1 Identified issues

| Issue # | Description | Source |
| --- | --- | --- |
| 1 | Scaling of the beam switching timing d to the applied SCS is missing from three places | Huawei, vivo |
| 2 | In agreements of RAN1 #100-e meeting, the value of beam switching timing d is added for cross-carrier aperiodic CSI-RS triggering. But the description of d value only is for the case that the scheduling offset is smaller than the threshold. The same description of d value can be supplemented for the case that the scheduling offset is equal to or greater than the threshold. | vivo |
| 3 | Incorrect location for the RAN1#100e TP “including the case that the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states”, introduced to 5.2.1.5.1 instead of the intended 5.2.1.5.1a. | Oppo, MTek, Ericsson |
| 4 | The agreement for default QCL assumption without configured CORESET is only captured in 5.2.1.5.1a for the case that PDCCH and the triggered aperiodic CSI-RS have different SCSs but not in 5.2.1.5.1 for the case that PDCCH and the triggered aperiodic CSI-RS have the same SCS. | Qualcomm |

## 2.2 Spec text alignment/corrections

| Issue # | Description | Source |
| --- | --- | --- |
| 1 | RRC parameter name alignment | Ericsson |

# 3 FL proposal

Issue #1: This seems like a clear bug left due to different TPs were not properly integrated together. Should be easy to agree.

Issue #2: Should be straight forward to determine whether to address the raised issue or not, and if the answer is “yes”, should be easy to agree on the exact wording

Issue #3: This seems like a clear misinterpretation from the FL side last time, leading to the correct text being introduced to an incorrect location of the spec

Issue #4: Should be straight forward to determine whether to address the raised issue or not, and if the answer is “yes”, should be easy to agree on the exact wording.

Spec text alignments/corrections: Issue #1 is an obvious misalignment between the RRC naming and the 38.214 and should be easy to agree.

FL proposal: Address all the raised issues in RAN1#100bis e-meeting

# 4 References

- R1-2001543	Remaining issues on the A-CSI RS triggering with different numerology	Huawei, HiSilicon

- R1-2001690	Remaining issues on aperiodic CSI-RS triggering	vivo

- R1-2001737	Text proposals for aperiodic CSI-RS triggering with different numerologies	OPPO

- R1-2001836	Remaining issues on X-carrier ACSI-RS triggering with different SCS	MediaTek Inc.

- R1-2002496	Correction to aperiodic CSI-RS triggering with mixed numerology	Ericsson

- R1-2002561	Remaining issues for aperiodic CSI-RS triggering with different numerology	Qualcomm Incorporated

# Annex – Proposals of the submitted Tdocs

R1-2001543	Remaining issues on the A-CSI RS triggering with different numerology	Huawei, HiSilicon

Proposal 1: Adopt the following TP in TS 38.214.

q=-1q=-1

--------------------------------- Start TP for TS 38.214 sub-clause 5.2.1.5.1a --------------------------------------

<Unchanged parts omitted>

5.2.1.5.1a	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies

When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the behavior defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions:

Beam switch timing:

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero

-	if one of the associated trigger states has the higher layer parameter qcl-Type set to 'QCL-TypeD',

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS;

-	else,

-	if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored.

-	else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received.

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols, when the reported value is one of the values of {14,28,48}, or is equal to or greater than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values of {224, 336}, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.

<Unchanged parts omitted>

--------------------------------- End TP for TS 38.214 sub-clause 5.2.1.5.1a --------------------------------------

R1-2001690	Remaining issues on aperiodic CSI-RS triggering	vivo

Proposal 1: Replace the original text “48+ d” with  “48+ d in CSI-RS symbols”

TS 38.214 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies < Unchanged parts are omitted > Beam switch timing: - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48+ d in CSI-RS symbols, when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero - if one of the associated trigger states has the higher layer parameter qcl-Type set to 'QCL-TypeD', - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d in CSI-RS symbols when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS; - else, - if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. - else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming + d in CSI-RS symbols, when the reported value is one of the values of {14,28,48}, or is equal to or greater than 48+d in CSI-RS symbols, when the reported value of beamSwitchTiming is one of the values of {224, 336}, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI. < Unchanged parts are omitted >

Proposal 2: Supplement the description of d value for the case that the scheduling offset is equal to or greater than the threshold.

TS 38.214 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies < Unchanged parts are omitted > Beam switch timing: - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48+ d when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero - if one of the associated trigger states has the higher layer parameter qcl-Type set to 'QCL-TypeD', - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d in CSI-RS symbols when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS; - else, - if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. - else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming + d in CSI-RS symbols, when the reported value is one of the values of {14,28,48}, or is equal to or greater than 48+d when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI. < Unchanged parts are omitted >

R1-2001737	Text proposals for aperiodic CSI-RS triggering with different numerologies	OPPO

// TP for 38.214//

##### 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 16, 24} slots. If the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states , the CSI-RS triggering offset is fixed to zero. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement. ##### 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies Aperiodic CSI-RS timing: - When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset, including the case that the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states. The CSI-RS triggering offset has the values of {0, 1,…31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with CA-slot-offset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where (omitted part)

R1-2001836	Remaining issues on X-carrier ACSI-RS triggering with different SCS	MediaTek Inc.

Proposal 1: Modify the contents in 38.214 CR ([2], R1-2001443) as below:

38.214 5.2.1.5.1 “Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology” with the deleted contents marked in red:

When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset, including the case that the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 16, 24} slots. If the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states , the CSI-RS triggering offset is fixed to zero. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement.

38.214 5.2.1.5.1a “Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies” with the additional contents marked in red:

When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset, including the case that the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement. When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the behavior defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions:

R1-2002496	Correction to aperiodic CSI-RS triggering with mixed numerology	Ericsson

##### 5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

For CSI-RS resource sets associated with Resource Settings configured with the higher layer parameter resourceType set to 'aperiodic', 'periodic', or 'semi-persistent', trigger states for Reporting Setting(s) (configured with the higher layer parameter reportConfigType set to 'aperiodic') and/or Resource Setting for channel and/or interference measurement on one or more component carriers are configured using the higher layer parameter CSI-AperiodicTriggerStateList. For aperiodic CSI report triggering, a single set of CSI triggering states are higher layer configured, wherein the CSI triggering states can be associated with any candidate DL BWP. A UE is not expected to receive more than one DCI with non-zero CSI request per slot. A UE is not expected to be configured with different TCI-StateId's for the same aperiodic CSI-RS resource ID configured in multiple aperiodic CSI-RS resource sets with the same triggering offset in the same aperiodic trigger state. A UE is not expected to receive more than one aperiodic CSI report request for transmission in a given slot. When a UE is triggered with a CSI report for a DL BWP that is non-active when expecting to receive the most recent occasion, no later than the CSI reference resource, of the associated NZP CSI-RS, the UE is not expected to report the CSI for the non-active DL BWP and the CSI report associated with that BWP is omitted. When a UE is triggered with aperiodic NZP CSI-RS in a DL BWP that is non-active when expecting to receive the NZP CSI-RS, the UE is not expected to measure the aperiodic CSI-RS. In the carrier of the serving cell expecting to receive that associated NZP CSI-RS, if the active DL BWP when receiving the NZP CSI-RS is different from the active DL BWP when receiving the triggering DCI,

-	the last symbol of the PDCCH span of the DCI carrying the BWP switching shall be no later than the last symbol of the PDCCH span of the DCI carrying the CSI trigger, irrespective of whether they are in the same carrier of a serving cell or not and irrespective of whether they are in the same SCS or not;

-	the UE is not expected to have any other BWP switching in that carrier after the last symbol of the PDCCH span covering the DCI carrying the CSI trigger and before the first symbol of the triggered NZP CSI-RS or CSI-IM.

A trigger state is initiated using the CSI request field in DCI.

-	When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

-	When the number of CSI triggering states in CSI-AperiodicTriggerStateList is less than or equal to , the CSI request field in DCI directly indicates the triggering state.

-	For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in Clause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCI-State's for the aperiodic CSI-RS resources associated with the CSI triggering state. If a State referred to in the list is configured with a reference to an RS associated with 'QCL-TypeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP.

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48 when the reported value of beamSwitchTiming is one of the values of {224, 336}.

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS;

-	else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored.

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48}, or is equal to or greater than 48 when the reported value of beamSwitchTiming is one of the values of {224, 336}, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.

-	A non-zero codepoint of the CSI request field in the DCI is mapped to a CSI triggering state according to the order of the associated positions of the up to 2NTS-1 trigger states in CSI-AperiodicTriggerStateList with codepoint '1' mapped to the triggering state in the first position.

For a UE configured with the higher layer parameter CSI-AperiodicTriggerStateList, if a Resource Setting linked to a CSI-ReportConfig has multiple aperiodic resource sets, only one of the aperiodic CSI-RS resource sets from the Resource Setting is associated with the trigger state, and the UE is higher layer configured per trigger state per Resource Setting to select the one CSI-IM/NZP CSI-RS resource set from the Resource Setting.

When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 16, 24} slots. If the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states , the CSI-RS triggering offset is fixed to zero. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement.

----- unchanged text omitted -------

##### 5.2.1.5.1a	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies

When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the behavior defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions:

Beam switch timing:

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d  in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48+ d when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero

-	if one of the associated trigger states has the higher layer parameter qcl-Type set to 'QCL-TypeD',

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d  in CSI-RS symbols when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS;

-	else,

-	if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored.

-	else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received.

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming + d  in CSI-RS symbols, when the reported value is one of the values of {14,28,48}, or is equal to or greater than 48+d when the reported value of beamSwitchTiming is one of the values of {224, 336}, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.

Table 5.2.1.5.1a-1: Additional beam switching timing delay d

| µPDCCH | d [PDCCH symbols] |
| --- | --- |
| 0 | 8 |
| 1 | 8 |
| 2 | 14 |

Aperiodic CSI-RS timing:

-	When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset, including the case that the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states. The CSI-RS triggering offset has the values of {0, 1,…31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with CA-slot-offset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where

-	n is the slot containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset,

-	μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively,

-	Nslot, offset, PDCCHCA and μoffset,PDCCHare the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured CA-slot-offset for the cell receiving the PDCCH respectively, Nslot, offset, CSIRSCA and  μoffset,CSIRS are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured CA-slot-offset for the cell transmitting the CSI-RS respectively, as defined in [4, TS 38.211] clause 4.5

-	If the µPDCCH < µCSIRS, the UE is expected to be able to measure the aperiodic CSI RS, if the CSI-RS starts no earlier than the first symbol of the CSI-RS carrier's slot that starts at least Ncsirs PDCCH symbols after the end of the PDCCH triggering the aperiodic CSI-RS.

-	If the µPDCCH > µCSIRS, the UE is expected to be able to measure the aperiodic CSI RS, if the CSI-RS starts no earlier than at least Ncsirs PDCCH symbols after the end of the PDCCH triggering the aperiodic CSI-RS.

Table 5.2.1.5.1a: Ncsirs as a function of the subcarrier spacing of the triggering PDCCH

| µPDCCH | Ncsirs [symbols] |
| --- | --- |
| 0 | 4 |
| 1 | 5 |
| 2 | 10 |
| 3 | [14] |

## 3.2	Text proposal on RRC parameter alignment

##### 5.2.1.5.1a	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies

When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the behavior defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions:

Beam switch timing:

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d  in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48+ d when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero

-	if one of the associated trigger states has the higher layer parameter qcl-Type set to 'QCL-TypeD',

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d  in CSI-RS symbols when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS;

-	else,

-	if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored.

-	else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received.

----- Unchanged text omitted ----

R1-2002561	Remaining issues for aperiodic CSI-RS triggering with different numerology	Qualcomm Incorporated

Proposal 1: In case of same numerology A-CSI RS triggering, when the offset between A-CSI RS and triggering DCI is less than beamSwitchTiming, capture the default QCL agreement in specification. Adopt the proposed text proposal in 5.2.1.5.1 in TS 38.214

- If no CORESET configured on the carrier for receiving the A-CSI RS, UE receives the A-CSI RS by applying the QCL parameters of the activated PDSCH TCI state with lowest ID.

The corresponding text proposal is as follows

---------------------------------------- Start of text proposal to Section 5.2.2.2.5 in TS 38.214 ---------------------------------- >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without the higher layer parameter repetition is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48 when the reported value of beamSwitchTiming is one of the values of {224, 336}. - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS; - else if the active BWP of the serving cell for receiving the aperiodic CSI-RS has configured ControlResourceSet, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored.; - else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< ----------------------------------------------------------- End of text proposal -------------------------------------------------------

Proposal 2: Update specification text to include cross-carrier scheduling with different SCS in the description of PDCCH limits for secondary cells. Adopt the corresponding text proposal in Section 10.1 in TS 38.213.

---------------------------------------- Start of text proposal to Section 10.1 in TS 38.213 --------------------------------------- >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< ## 10.1 UE procedure for determining physical downlink control channel assignment >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< For same cell scheduling or for cross-carrier scheduling, a UE does not expect a number of PDCCH candidates, and a number of corresponding non-overlapped CCEs per slot on a secondary cell to be larger than the corresponding numbers that the UE is capable of monitoring on the secondary cell per slot. >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< ----------------------------------------------------------- End of text proposal --------------------------------------------------------