3GPP TSG RAN WG1 #100bis-E								R1-2002771

e-Meeting, April 20th – 30th, 2020

Source:	Moderator (Intel Corporation)

Title:	Summary of the RAN1 WG E-mail Discussion [100b-e-NR-Pos-02]

Agenda item:	7.2.8.1

Document for:	Discussion and Decision

- Introduction

This contribution is a part of the Rel.16 maintenance work on NR Positioning. It provides summary of the e-mail discussion #2 for NR Positioning organized during the RAN1#100bis E-meeting based on the submitted contributions [1] - [11].

[100b-e-NR-Pos-02] Email discussion/approval on the following issues by 4/23; if necessary, followed by endorsing the corresponding TPs by 4/28 – Alexey (Intel)

Corrections to 38.211

  - PRS/SSB collision handling for neighbour cells

  - Missing value in muting pattern

  - Periodicity in slots for DL PRS transmission

  - Relationship b/w number of symbols and comb factor

NR-TimingMeasQuality for RSTD

- Corrections to TS 38.211

  - PRS/SSB Collision Handling for Neighbor Cells

The following TPs were submitted to clarify UE assumption on PRS/SSB transmission/collision for neighbor cells.

Option 1 based on TP in [2]

- the symbol l is not used by any SS/PBCH block used by the serving cell for downlink PRS transmitted from the serving cell or not used by any SS/PBCH block used by the same non-serving cell indicated by the higher-layer parameter SSB-positionInBurst for downlink PRS transmitted from a non-serving cell;

Option 2 based on TP in [3]

- the symbol l is not used by any SS/PBCH block used by the serving cell for downlink PRS transmitted from the serving cell or any SS/PBCH block from a non-serving cell indicated by the higher-layer parameter SSB-positionInBurst for downlink PRS transmitted from a the same non-serving cell;

- Companies are invited to comment on above TPs, select the preferred one and propose any modification, if it is necessary.

Table 1: PRS/SSB Collision Handling for Neighbor Cells

| Company | Comments |
| --- | --- |
| Huawei/HiSilicon | We think that duplicated text should be avoided. Since TS 38.214 fixed that issue, we believe a better wayforward would be - the symbol l is not used by any SS/PBCH block from the same cell according to clause 5.1.6.5 of [6, TS 38.214]. Between Option 1 and Option 2, we think Option 2 is better, if we really want to echo TS 38.214. |
| OPPO | Both options are similar. Huawei’s proposal seems better |
| CMCC | We prefer HW’s layout to Options 1 & 2 |
| LG | We are fine with both options and Huawei’s proposal. |
| ZTE | Both options and Huawei’s proposal are fine for us. Our intention is to fix the problem. |
| Nokia/NSB | We are okay with Option 2 or HW’s proposal. |
| Qualcomm | We are OK with HW’s proposal or Option 2. |
| Intel | Prefer Option 2 – gives more complete picture. |
| CATT | We are OK with HW’s proposal or Option 2. |
| Futurewei | Ok with either Option |
| vivo | We are OK with Option 2 or Huawei/HiSilicon’s proposal in their above comment. |
| mtk | We consider to adopt HW proposal |
| Samsung | Option 2 |
| Ericsson | Either option is fine, with a preference for option 2. |

  - Missing value in muting pattern

Text proposal to capture in the TS38.211 the missing value of DL PRS muting pattern configuration was provided in [3]:

TS 38.211-g10 7.4.7.1.4 Mapping to slot in a downlink PRS resource set < Unchanged parts are omitted > - bi1 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS 2μTmutingPRSTperPRSmod L in the bitmap given by the higher-layer parameter DL-PRS-MutingPattern where L∈2, 4, 6, 8, 16, 32 is the size of the bitmap; < Unchanged parts are omitted >

- Companies are invited to provide feedback on whether this change is needed in table below.

Table 2: Missing value in muting pattern

| Company | Comments |
| --- | --- |
| Huawei/HiSilicon | Support |
| OPPO | Support |
| CMCC | Support |
| LG | Support |
| ZTE | Support |
| Nokia/NSB | Support |
| Qualcomm | Support |
| Intel | Support |
| CATT | Support |
| Futurewei | Support |
| Vivo | Support with one correction of typo. This TP is for TS 38.211-g10, 7.4.1.7.4 Mapping to slots in a downlink PRS resource set. |
| Mtk | Good catch by vivo. Same view as above |
| Samsung | Support |
| Ericsson | Support |

  - Periodicity in slots for DL PRS transmission

Correction of the DL PRS periodicity values in order to align with the TS 38.214 and TS 37.355 was proposed in [4] with the following changes

TS 38.211-g10 7.4.7.1.4 Mapping to slot in a downlink PRS resource set <Text omitted> For a downlink PRS resource in a downlink PRS resource set, the UE shall assume the downlink PRS resource being transmitted when the slot and frame numbers fulfil Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod 2μTperPRS∈iTgapPRSi=0TrepPRS-1 <Text omitted> - bi1 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS 2μTmutingPRSTperPRSmod L in the bitmap given by the higher-layer parameter DL-PRS-MutingPattern where L∈2, 4, 8, 16, 32 is the size of the bitmap; - bi2 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod 2μTperPRS TgapPRS mod TrepPRS in the bitmap given by the higher-layer parameter DL-PRS-MutingPattern; <Text omitted> the periodicity TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 is given by the higher-layer parameter dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16; <Text omitted>

Companies are invited to provide their feedback on the proposed above modification in table below.

Table 3: Periodicity in slots for DL PRS transmission

| Company | Comments |
| --- | --- |
| Huawei/HiSilicon | Support. The first change is not aligned with R1-2001731, it should be mod 2μTperPRS. Is it correct understanding that the alignment of the parameter of dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 for a DL PRS resource is taken as lower priority and will handled by the spec editor? |
| OPPO | Support |
| CMCC | Support |
| LG | Support |
| ZTE | Support. We prefer to let editor handle alignment issues. |
| Nokia/NSB | Support the TP. |
| Qualcomm | OK |
| Intel | Support. Note: Comment from Huawei is reflected in TP above (i.e. it is updated) |
| CATT | Support. |
| Futurewei | OK |
| Vivo | OK with one correction of typo. This TP is for TS 38.211-g10, 7.4.1.7.4 Mapping to slots in a downlink PRS resource set. |
| Mtk | Okay |
| Samsung | Support |
| Ericsson | Support |

  - Relationship b/w number of symbols and comb factor

It was noticed in [4], that relationship b/w number of DL PRS symbols and comb factor is not captured in RAN1 specification. Therefore, it was proposed to adopt the following TP in the TS 38.214:

Option 1. Proposed changes in the TS 38.214.

DL-PRS-NumSymbols defines the number of symbols of the DL PRS resource within a slot where the allowable values are given in Clause 7.4.1.7.1 of [4, TS38.211]. For the combination of {number of symbols, comb size} for a PRS resource, the UE can be configured with one of { 2, 2},{ 4, 2}, { 6, 2}, { 12, 2}, { 4, 4}, { 12, 4}, { 6, 6}, { 12, 6} and { 12, 12}

Alternatively, the similar change can be made in the TS 38.211

Option 2. Proposed changes in the TS 38.211.

TS 38.211-g10 7.4.7.1.4 Mapping to slot in a downlink PRS resource set --------------------------------------------------------------Text omitted--------------------------------------------------------------- - the size of the downlink PRS resource in the time domain LPRS∈2,4,6,12 is given by the higher-layer parameter DL-PRS-NumSymbols;. The value of LPRS is also a function of the configured parameter KcombPRS as given by Table 7.4.17.3-2; --------------------------------------------------------------Text omitted--------------------------------------------------------------- Table 7.4.1.7.3-2: The number of symbols LPRS as a function KcombPRS LPRS; KcombPRS. 2; 4; 6; 12. 2; 2; NA; NA; NA. 4; 2; 4; NA; NA. 6; 2; NA; 6; NA. 12; 2; 4; 6; 12. --------------------------------------------------------------Text omitted---------------------------------------------------------------

- Companies are invited to provide comments on above TPs, select the preferred one and propose any modification, if it is necessary.

Table 4: Relationship b/w number of symbols and comb factor

| Company | Comments |
| --- | --- |
| Huawei/HiSilicon | Support Option 1 in principle (although the proposal is to change TS 38.214 instead of TS 38.211). The restriction may be easily removed for future releases. One question is why there is an extra space between “{” and the digit? For Option 2, the number in the table is not clear in itself; e.g. what does 2 and 6 mean in the row composed of (2, NA, 6, NA)? |
| OPPO | Support Option 1. Regarding Huawei’s question for Option 1, the extra space between “{” and the digit is a typo and can be removed |
| CMCC | Option 1 looks clearer to us. |
| LG | We prefer Option 1 with a minor change as follows. For the combination of {DL-PRS-NumSymbols, DL-PRS-CombSizeN} for a PRS resource, the UE can be configured with one of {2, 2},{4, 2}, {6, 2}, {12, 2}, {4, 4}, {12, 4}, {6, 6}, {12, 6} and {12, 12}. |
| ZTE | Option 1 for simplicity. |
| Nokia/NSB | We support option 1 as the topic is about configuration options it makes sense to have in 38.214 from our view. Update: If the group prefers to have the TP in 38.211 as suggested by QC that is okay for us. |
| Qualcomm | We prefer Reference signal configurations to be part of 38.211, since 38.214 is supposed to be about procedures. We can add the text of Option 1 into 38.211 as shown below: 7.4.7.1.4 Mapping to slot in a downlink PRS resource set -------------------------------Text omitted--------------------------------------------------------------- and where - lstartPRS is the first symbol of the downlink PRS within a slot and given by the higher-layer parameter DL-PRS-ResourceSymbolOffset; - the size of the downlink PRS resource in the time domain LPRS∈2,4,6,12 is given by the higher-layer parameter DL-PRS-NumSymbols; - the comb size KcombPRS∈2, 4, 6,12 is given by the higher-layer parameter transmissionComb;,, such that combination of {DL-PRS-NumSymbols, transmissionComb} is one of {2, 2},{4, 2}, {6, 2}, {12, 2}, {4, 4}, {12, 4}, {6, 6}, {12, 6} and {12, 12}; - the resource-element offset koffsetPRS∈0,1,…,KcombPRS-1 is given by the higher-layer parameter combOffset; - the quantity k' is given by Table 7.4.1.7.3-1. -------------------------------Text omitted--------------------------------------------------------------- |
| Intel | We prefer TP to be captured in 211. Agree to change proposed by Qualcomm. |
| CATT | Our preference is to have the changes in TS 38.211. QC’s proposal is fine to us. |
| Futurewei | TP should be into 38.211 (Option 2) |
| vivo | We also prefer to modify 38.211 instead of 38.214. We support Qualcomm’s above TP except one correction to the section number. It should be for TS 38.211-g10, 7.4.1.7.3 Mapping to physical resources in a downlink PRS resource |
| mtk | It is better to capture RS structure in 211. So, we are okay for QC’s proposal |
| Samsung | Option 1 |
| Ericsson | Configuration is better suited for 214, but if this closes the issue we can have it in 211. |

- Clarification on NR-TimingMeasQuality for RSTD

  - Phase 1 Discussion on NR-TimingMeasQuality for RSTD

- In R1-2001560, it was mentioned that the parameter NR-TimingMeasQuality for DL RSTD measurements may have multiple interpretations, i.e. has ambiguity. Therefore, it was proposed that RAN1 clarifies which interpretation should be used for the timing measurement quality for RSTD.

- Interpretation 1: σk2+σ12

- Interpretation 2: σk

  - Where σ1 is the TOA quality of the reference and σk is the TOA quality of the TRP k.

Table 5: Clarification on NR-TimingMeasQuality for RSTD

| Company | Comments |
| --- | --- |
| Huawei/HiSilicon | We think the issue that interests more people is whether reference measurement (reference TRP/resource set/resource) can also have the quality. Our preference is Yes. For the issue listed here, as we said earlier, the formula is only informative, and we are not intending to introduce any new measurement. For two interpretations, we can accept either way, but we slightly prefer Interpretation 2, so that the quality of reference TRP and the neighbouring TRP will be interpreted in the same way, and even the RSTD quality and UE Rx – Tx time difference quality will also be interpreted in the same way. Note that we have quite a lot of TOA measurements to report considering RAN1 introduced resource specific timing reporting and RAN2 introduced additional path reporting. |
| OPPO | Both interpretation can work since the reference TRP TOA meas quality is reported. We prefer Interpretation 2 as the rang of its value is narrower and more accurate quantitation can be achieved |
| LG | We prefer that quality of timing measurement (RSTD, UE RX-TX time difference) is defined/unified as TOA measurement quality. In case of RSTD measurement quality, it inherently contains both errors for timing measurement for a reference and a timing measurement for PRS resource from a neighbor TRP. It might be reasonable that the UE report TOA measurement quality for reference and neighbor TRP, rather than reporting TOA measurement quality for a reference TRP and reporting RSTD measurement quality for neighbor TRPs. |
| ZTE | We prefer to interpretation 2 for consistency with other timing measurement metods. |
| Nokia/NSB | We are okay to support interpretation 2. Is the intention to make an agreement which is sent to RAN4 or just have it in the chairmen’s notes? We would also be okay to have the reference measurement also have a quality as we see this as useful. |
| Qualcomm | We are OK to go with Interpretation 2, even though our initial understanding was similar to LTE (option 1). In either case, we again there cannot be any formulas written as an agreement. We don’t think there needs to be something sent to Ran4. We may prefer to just have a conclusion, in case someone in the future needs to refer to, but we don’t see any spec impact in RAN1, or RAN2 or RAN4. |
| Intel | We are OK to confirm Interpretation 2. We wonder what is indicated by UE in case of single shot DL PRS processing. |
| CATT | We prefer the interpretation of NR-TimingMeasQuality as the TOA quality instead of RSTD quality to be consistent with all timing measurement. We also don’t see the need to define any formula at this moment. |
| Futurewei | Slight preference for 2 |
| vivo | Interpretation 2 is our understanding. A question from our side, is the intention of the proponent to specify this in any specification? RAN1, and/or other WGs? |
| MTK | We also think that quality should be in terms of the standard deviation per TOA. Another thing we want to further mention is, we can further consider whether to define confidence level (twice of three times of the standard deviation) |
| Samsung | Interpretation 2 is our preference |
| Ericsson | Support interpretation 2. We also have a similar question to Vivo and other on the spec impact. If this is not to define a measurement, how is the assumption captured? |

Proposal for agreement/conclusion #1

- Interpretation #2 is confirmed, i.e. timing measurement quality for RSTD is defined by σk

  - where σk is the TOA quality of the TRP k, σ1 is the TOA quality of the reference TRP

  - Phase 2 Discussion on NR-TimingMeasQuality for RSTD

Companies are invited to provide view on the following additional questions:

- Whether proposal for potential agreement/conclusion #1 is supported

- Whether and how to capture the potential agreement/conclusion on interpretation of NR-TimingMeasQuality for RSTD

  - Send LS to RAN2

  - Make a conclusion in chair notes or capture in specification

  - Etc.

- In addition, companies are encouraged to provide their understanding on how the TOA quality of the TRP k, represented by σk is defined in case of single shot DL PRS measurements (i.e. only one measurement is available).

Table 5: Further clarification on NR-TimingMeasQuality for RSTD

| Company | Comments |
| --- | --- |
| Huawei/HiSilicon | 1) Support in principle. Suggest the following revision. Proposal for conclusion: - It is RAN1 understanding that the NR-TimingMeasQuality is the TOA quality. - Note: No RAN1 spec change is required. 2) We think a conclusion in the chairman’s notes will be sufficient. An LS to RAN2 to capture it in LPP would also be OK for us, but we assume no RAN1 spec change is required for Rel-16. 3) We think it would be entirely up to UE implementation in Rel-16. In addition, we have the following comment: Should we have another proposal for conclusion as following? - It is RAN1 understanding that the measurement for RSTD reference can also have NR-TimingMeasQuality. - Note: No RAN1 spec change is required. |
| Intel | - We have some preference to make an agreement - We prefer to inform RAN2 on RAN1 understanding by sending LS. We agree that no RAN1 spec change is needed. - It seems that “up to UE implementation” is the only feasible option in R16. |
| Nokia/NSB | - We are okay with the proposed conclusion. As TOA is not a defined measurement we don’t think the first bullet from HW should be added. - We think a conclusion in the chairmen’s notes is a good way forward. An LS to RAN2 is okay for us if the group prefers that perhaps with a note that no RAN1 spec will be changed. - Up to UE implementation. |
| Qualcomm | - We prefer just a conclusion. Proposal for conclusion: - It is RAN1 understanding that the NR-TimingMeasQuality is the TOA quality. - Note: No RAN1 spec change is required. - We think a conclusion in the chairman’s notes will be sufficient. No LS is needed - Up to UE implementation |
| Ericsson | - A conclusion should be enough if there is no spec impact. - We are okay with an LS to RAN2 as information, reflecting this is not an agreement with spec impact in ran1. - Up to UE implementation (just as for multiple instance of PRS). |

- Summary of Issues

  - Corrections to TS 38.211

Based on analysis of responses received so far, the following proposals are made to correct the TS 38.211. Please note that corrections corresponding to issues 2.2 and 2.3 are merged in one Text Proposal#2.

Proposal 2

- Text proposals #1, #2, #3 provided in this section are agreed by RAN1 WG

Table 6 - Text Proposal #1

TS 38.211-g10 7.4.1.7.3 Mapping to physical resources in a downlink PRS resource - the symbol l is not used by any SS/PBCH block used by the serving cell for downlink PRS transmitted from the serving cell or any SS/PBCH block from a non-serving cell indicated by the higher-layer parameter SSB-positionInBurst for downlink PRS transmitted from a the same non-serving cell;

Table 7 - Text Proposal #2

TS 38.211-g10 7.4.1.7.4 Mapping to slot in a downlink PRS resource set <Text is omitted> For a downlink PRS resource in a downlink PRS resource set, the UE shall assume the downlink PRS resource being transmitted when the slot and frame numbers fulfil Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod 2μTperPRS∈iTgapPRSi=0TrepPRS-1 <Text is omitted> - bi1 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS 2μTmutingPRSTperPRSmod L in the bitmap given by the higher-layer parameter DL-PRS-MutingPattern where L∈2, 4, 6, 8, 16, 32 is the size of the bitmap; - bi2 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod 2μTperPRS TgapPRS mod TrepPRS in the bitmap given by the higher-layer parameter DL-PRS-MutingPattern; <Text is omitted> the periodicity TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 is given by the higher-layer parameter dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16; <Text is omitted>

Table 7 - Text Proposal #3

TS 38.211-g10 7.4.1.7.3 Mapping to physical resources in a downlink PRS resource <Text is omitted> and where - lstartPRS is the first symbol of the downlink PRS within a slot and given by the higher-layer parameter DL-PRS-ResourceSymbolOffset; - the size of the downlink PRS resource in the time domain LPRS∈2,4,6,12 is given by the higher-layer parameter DL-PRS-NumSymbols; - the comb size KcombPRS∈2, 4, 6,12 is given by the higher-layer parameter transmissionComb;, such that combination of {LPRS, KcombPRS} is one of {2, 2},{4, 2}, {6, 2}, {12, 2}, {4, 4}, {12, 4}, {6, 6}, {12, 6} and {12, 12}; - the resource-element offset koffsetPRS∈0,1,…,KcombPRS-1 is given by the higher-layer parameter combOffset; - the quantity k' is given by Table 7.4.1.7.3-1. <Text is omitted>

Table 5: Views on TPs for corrections of the TS 38.211

| Company | Comments |
| --- | --- |
| vivo | TP#1 and TP#3 are for TS 38.211-g10, 7.4.1.7.3 Mapping to physical resources in a downlink PRS resource. TP#2 is for TS 38.211-g10, 7.4.1.7.4 Mapping to slots in a downlink PRS resource set. |
| Huawei/HiSilicon | Would that be easier if we change {DL-PRS-NumSymbols, transmissionComb} to LPRS, KcombPRS so that the spec editor can has less work to do when it comes to parameter name alignment? |
| Intel | As a moderator updated TPs as follows: - Added correct sections to each TP based on comment from vivo - Implemented comment from Huawei We support the above TPs |
| Qualcomm | We are OK, but quick question: Is the parameter called SSB-positionInBurst or SSB-positionsInBurst ? |
| Samsung | Fine for us. |
| Ericsson | Support |

  - NR-TimingMeasQuality for RSTD

Based on discussion in Section 3, the following proposal has been made for endorsement by RAN1 WG:

Proposal

- Capture the following conclusion in chair notes:

  - It is RAN1 understanding that the NR-TimingMeasQuality is the quality for time of arrival measurements

  - NR-TimingMeasQuality is left up to UE implementation

  - Notes:

    - No RAN1 spec change is required

    - NR-TimingMeasQuality measurement is also applicable for the reference timing used in RSTD measurements

- Outcome of E-mail Discussion [100b-e-NR-Pos-02]

Based on further discussion over RAN1 WG e-mail reflector, the following was finally agreed by RAN1 WG:

- Three text proposals captured in Section 4.1 of this document

- Capture in chair notes the following conclusion for NR-TimingMeasQuality

Conclusion:

  - It is RAN1 understanding that the NR-TimingMeasQuality is the quality for time of arrival measurements

  - NR-TimingMeasQuality is left up to UE implementation

  - Notes:

    - No RAN1 spec change is required

    - NR-TimingMeasQuality measurement is also applicable for the reference timing used in RSTD measurements

- References

- R1-2001558	Maintenance of DL PRS for NR positioning	Huawei, HiSilicon

- R1-2001600	Maintenance of DL reference signals for NR positioning	ZTE

- R1-2001685	Discussion on remaining issues on DL RS for NR positioning	vivo

- R1-2001731	Remaining Issues on DL Positioning Reference Signal	OPPO

- R1-2001954	Remaining details of DL Reference signals for NR positioning	LG Electronics

- R1-2002046	Remaining details on DL Reference Signals	Futurewei

- R1-2002095	Remaining issues on DL PRS for NR Positioning	CATT

- R1-2002144	DL Reference Signals for NR Positioning	Samsung

- R1-2002288	Corrections to DL reference signals for NR positioning	Intel Corporation

- R1-2002557	Discussion on DL PRS processing & related UE capabilities	Qualcomm Incorporated

- R1-2002620	Maintenance of rel16 DL reference signals for NR positioning	Ericsson