3GPP TSG RAN WG1 Meeting #100bis							R1-2002954

eMeeting, April 20th – 30th 2020

Title:	Draft LS on RAN1 agreements related to resource multiplexing in IAB

Response to:

Release:	Rel-16

Work Item:	NR_IAB-Core

Source:	Ericsson [RAN1]

To:	RAN2

CC:

Contact Person:

#### Name:	Magnus Åström

E-mail Address:	magnus.astrom (AT) ericsson.com

1. Overall Description:

RAN1 has discussed Mechanisms for resource multiplexing among backhaul and access links within the Rel-16 NR WI Integrated Access and Backhaul (IAB). In that discussion, RAN1 made some agreements that may be of interest to RAN2.

2. Actions:

To RAN2 group.

ACTION:

RAN1 respectfully informs RAN2 about the following agreements that have been made by RAN1:

Agreement: Confirm that from a RAN1 perspective all Rel-15 UE common search space types are also applicable to Rel-16 IAB nodes. Signaling details are left to RAN2.

Agreement: Confirm DCI Format 2_0 and DCI Format 2_5 can be monitored by an IAB-MT in at least a common search space. The same number of aggregation levels and candidates can be separately configured for both DCI Format 2_0 and DCI Format 2_5.

Agreement: DCI Format 2_0 is not monitored by an IAB-MT in a UE(MT)-specific search space. DCI Format 2_5 can be additionally monitored by an IAB-MT in a UE(MT)-specific search space. Signaling details (e.g. whether the configuration is in the existing UE-specific search space configuration or a new MT-specific search space configuration) is left up to RAN2.

Agreement:  maxAI-DCI-PayloadSize = maxSFI-DCI-PayloadSize = 128.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #101	25 May – 5 June 2020		eMeeting

TSG-RAN WG1 Meeting #102 	24 Aug – 28 Aug 2020		Toulouse, France