3GPP TSG RAN WG1 #100bis	R1-2002981

e-Meeting, April 20th – 30th, 2020

Source:	moderator (vivo)

Title:	Editorial corrections on UL full power transmission

Agenda Item:	7.2.6.4

Document for:	Discussion and Decision

Editorial corrections

Reason for change:  RRC parameter names are not aligned in multiple specs, and some editorial corrections for better reading of spec.

Summary of change:  editorial corrections including RRC parameter names alignment for multiple specs.

Consequences if not approved: RRC parameters in RAN1 and RAN2 specs are not aligned

Clauses affected: 38.212 section 7.3.1.1.2, 7.3.1.1.3. 38.213 section 7.1. 38.214 section 6.1.1.1

Issue 8: TP for RRC parameter alignment with 38.331 (Editorial)

TS 38.212

7.3.1.1.2	Format 0_1

<Unrelated part omitted>

-	Precoding information and number of layers – number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ULFPTxModes ul-FullPowerTransmission is either not configured or configured to Mode2 fullpowerMode2, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank, and codebookSubset;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ULFPTxModes ul-FullPowerTransmission =Mode1 fullpowerMode1, maxRank=2, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ULFPTxModes ul-FullPowerTransmission =Mode1 fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ULFPTxModes ul-FullPowerTransmission is either not configured or configured to Mode2 fullpowerMode2, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ULFPTxModes ul-FullPowerTransmission =Mode1 fullpowerMode1, maxRank=1, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ULFPTxModes ul-FullPowerTransmission is either not configured or configured to Mode2 fullpowerMode2, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ULFPTxModes ul-FullPowerTransmission =Mode1 fullpowerMode1, transform precoder is disabled, maxRank=2, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ULFPTxModes ul-FullPowerTransmission is either not configured or configured to Mode2 fullpowerMode2, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ULFPTxModes ul-FullPowerTransmission =Mode1 fullpowerMode1, maxRank=1, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

For the higher layer parameter txConfig=codebook, if ULFPTxModes ul-FullPowerTransmission is configured to Mode2 fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in an SRS resource set with usage set to 'codebook' and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used.

…

Table 7.3.1.1.2-2: Precoding information and number of layers, for 4 antenna ports, if transform precoder is disabled, maxRank = 2 or 3 or 4, and ULFPTxModes ul-FullPowerTransmission is either not configured or configured to Mode2 fullpowerMode2

…

Table 7.3.1.1.2-2A: Precoding information and number of layers for 4 antenna ports, if transform precoder is disabled, maxRank = 2, and ULFPTxModes ul-FullPowerTransmission =Mode1 fullpowerMode1

…

Table 7.3.1.1.2-2B: Precoding information and number of layers for 4 antenna ports, if transform precoder is disabled, maxRank = 3 or 4, and ULFPTxModes ul-FullPowerTransmission =Mode1 fullpowerMode1

…

Table 7.3.1.1.2-3: Precoding information and number of layers for 4 antenna ports, if transform precoder is enabled and ULFPTxModes ul-FullPowerTransmission is either not configured or configured to Mode2 fullpowerMode2, or if transform precoder is disabled, maxRank = 1, and ULFPTxModes ul-FullPowerTransmission is either not configured or configured to Mode2 fullpowerMode2

…

Table 7.3.1.1.2-3A: Precoding information and number of layers for 4 antenna ports, if transform precoder is enabled and ULFPTxModes ul-FullPowerTransmission =Mode1 fullpowerMode1, or if transform precoder is disabled, maxRank = 1, and ULFPTxModes ul-FullPowerTransmission =Mode1 fullpowerMode1

…

Table 7.3.1.1.2-4: Precoding information and number of layers, for 2 antenna ports, if transform precoder is disabled, maxRank = 2, and ULFPTxModes ul-FullPowerTransmission is either not configured or configured to Mode2 fullpowerMode2

…

Table 7.3.1.1.2-4A: Precoding information and number of layers, for 2 antenna ports, if transform precoder is disabled, maxRank = 2, and ULFPTxModes ul-FullPowerTransmission =Mode1 fullpowerMode1

…

Table 7.3.1.1.2-5: Precoding information and number of layers, for 2 antenna ports, if transform precoder is enabled and ULFPTxModes ul-FullPowerTransmission is either not configured or configured to Mode2 fullpowerMode2, or if transform precoder is disabled, maxRank = 1, and and ULFPTxModes ul-FullPowerTransmission is either not configured or configured to Mode2 fullpowerMode2

…

Table 7.3.1.1.2-5A: Precoding information and number of layers, for 2 antenna ports, if transform precoder is enabled and ULFPTxModes ul-FullPowerTransmission =Mode1 fullpowerMode1, or if transform precoder is disabled, maxRank = 1, and ULFPTxModes ul-FullPowerTransmission =Mode1 fullpowerMode1

…

Table 7.3.1.1.2-32: SRI indication for codebook based PUSCH transmission, if ULFPTxModes ul-FullPowerTransmission is not configured, or ULFPTxModes ul-FullPowerTransmission =Mode1 fullpowerMode1, or ULFPTxModes ul-FullPowerTransmission =Mode2 fullpowerMode2 and NSRS=2

…

Table 7.3.1.1.2-32A: SRI indication for codebook based PUSCH transmission, if ULFPTxModes ul-FullPowerTransmission =Mode2 fullpowerMode2 and NSRS=3

…

Table 7.3.1.1.2-32B: SRI indication for codebook based PUSCH transmission, if ULFPTxModes ul-FullPowerTransmission =Mode2 fullpowerMode2 and NSRS=4

…

Issue 6: TP for clarity on bitwidth of precoding information (Editorial)

TS 38.212

7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

<unchanged part removed>

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in an SRS resource set with usage set to ‘codebook’. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

Issue 5: TP for UL full power transmission with DCI 0_2

TS 38.212

7.3.1.1.3	Format 0_2

-	Precoding information and number of layers – number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is either not configured or configured to fullpowerMode2, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank-ForDCIFormat0_2, and codebookSubset-ForDCIFormat0_2;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission=fullpowerMode1, the values of higher layer parameters maxRank-ForDCIFormat0_2=2, transform precoder is disabled, and codebookSubset-ForDCIFormat0_2;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission=fullpowerMode1, the values of higher layer parameters maxRank-ForDCIFormat0_2=3 or 4, transform precoder is disabled, and codebookSubset-ForDCIFormat0_2;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is either not configured or configured to fullpowerMode2, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank-ForDCIFormat0_2, and codebookSubset-ForDCIFormat0_2;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission=fullpowerMode1, the values of higher layer parameters maxRank-ForDCIFormat0_2=1, and according to whether transform precoder is enabled or disabled, and codebookSubset-ForDCIFormat0_2;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is either not configured or configured to fullpowerMode2, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank-ForDCIFormat0_2 and codebookSubset-ForDCIFormat0_2;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission=fullpowerMode1, transform precoder is disabled, the values of higher layer parameters maxRank-ForDCIFormat0_2=2, and codebookSubset-ForDCIFormat0_2=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is either not configured or configured to fullpowerMode2, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank-ForDCIFormat0_2 and codebookSubset-ForDCIFormat0_2.

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission=fullpowerMode1, the values of higher layer parameters maxRank-ForDCIFormat0_2=1, and according to whether transform precoder is enabled or disabled, and codebookSubset-ForDCIFormat0_2;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, the values of higher layer parameters maxRank-ForDCIFormat0_2 is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in an SRS resource set with usage set to 'codebook' and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in an SRS resource set with usage set to ‘codebook’. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

Issue 7: TP on RRC parameter names and maximum number of configurable spatial relation (Editorial)

TS 38.213

------------------------------------------------------------------------------------------------------

7.1	Physical uplink shared channel

<Unrelated part omitted>

For a PUSCH transmission on active UL BWP , as described in Clause 12, of carrier  of serving cell , a UE first calculates a linear value  of the transmit power , with parameters as defined in Clause 7.1.1. For a PUSCH transmission scheduled by a DCI format or configured by ConfiguredGrantConfig or semiPersistentOnPUSCH, if txConfig in PUSCH-Config is set to 'codebook',

-	if ULFPTx ul-FullPowerTransmission in PUSCH-Config is provided and codebookSubset in PUSCH-Config is set to nonCoherent or partialAndNonCoherent, the UE scales PPUSCH,b,f,c(i,j,qd,l) by s where:

-	if ULFPTxModes ul-FullPowerTransmission in PUSCH-Config is set to Mode1 fullpowerMode1, and each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port', s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over the maximum number of SRS ports supported by the UE in one SRS resource

-	if ULFPTxModes ul-FullPowerTransmission in PUSCH-Config is set to Mode2 fullpowerMode2, s=1 for full power TPMIs reported by the UE [16, TS 38.306], and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for remaining TPMIs, where the number of SRS ports is associated with a SRS resource indicated by SRI if more than one SRS resources are is configured in the SRS-ResourceSet with usage set to 'codebook', or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', and

-	if ULFPTxModes in PUSCH-Config is not provided ul-FullPowerTransmission in PUSCH-Config is set to fullpower, s=1

-	else, if each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of SRS ports supported by the UE in one SRS resource.

…

-----------------------------------------------------------------------------------------------------------

TS 38.214

6.1.1.1	Codebook based UL transmission

--------------------------------------------------------------------

<Unrelated part omitted>

When higher layer parameter ULFPTxModes is set to 'Mode 2',

-	the UE can be configured with one SRS resource or multiple SRS resources with same or different number of SRS ports within an SRS resource set with usage set to 'codebook'.

-	up to 2 different spatial relations (maxNumberConfiguredSpatialRelations) can be configured for all SRS resources with usage set to 'codebook'.

-	subject to UE capability, a maximum of 2 or 4 SRS resources are supported in an SRS resource set with usage set to 'codebook'

…

------------------------------------------------------------------------

------------------------------------------------------------------------

up to 2 different spatial relations (maxNumberConfiguredSpatialRelations) can be configured for all SRS resources in the SRS resource set with usage set to 'codebook' when multiple SRS resources are configured in the SRS resource set.

------------------------------------------------------------------------

TS 38.214

6.1.1.1	Codebook based UL transmission

<Unrelated part omitted>

For codebook based transmission, the UE may be configured with a single SRS-ResourceSet with usage set to 'codebook' and only one SRS resource can be indicated based on the SRI from within the SRS resource set. Except when higher layer parameter ULFPTxModes ul-FullPowerTransmission is set to 'Mode 2 fullpowerMode2', the maximum number of configured SRS resources for codebook based transmission is 2. If aperiodic SRS is configured for a UE, the SRS request field in DCI triggers the transmission of aperiodic SRS resources.

The UE shall transmit PUSCH using the same antenna port(s) as the SRS port(s) in the SRS resource indicated by the DCI format 0_1 or 0_2 or by configuredGrantConfig according to clause 6.1.2.3.

The DM-RS antenna ports  in Clause 6.4.1.1.3 of [4, TS38.211] are determined according to the ordering of DM-RS port(s) given by Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 in Clause 7.3.1.1.2 of [5, TS 38.212].

Except when higher layer parameter ULFPTxModes ul-FullPowerTransmission is set to 'Mode 2 fullpowerMode2', when multiple SRS resources are configured by SRS-ResourceSet with usage set to 'codebook', the UE shall expect that higher layer parameters nrofSRS-Ports in SRS-Resource in SRS-ResourceSet shall be configured with the same value for all these SRS resources.

When higher layer parameter ULFPTxModes ul-FullPowerTransmission is set to 'Mode 2 fullpowerMode2',

-	the UE can be configured with one SRS resource or multiple SRS resources with same or different number of SRS ports within an SRS resource set with usage set to 'codebook'.

-	up to 2 different spatial relations (maxNumberConfiguredSpatialRelations) can be configured for all SRS resources with usage set to 'codebook'.

-	subject to UE capability, a maximum of 2 or 4 SRS resources are supported in an SRS resource set with usage set to 'codebook'

…

References

[1] R1-2002746, “Summary of prep email discussion on ULFPTx”, RAN1#100bis-e