3GPP TSG RAN WG1 #100bis			                                   					       												        R1-2003101

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.7

Source:	Moderator (AT&T)

Title:	Summary of Email Approval [100b-e-NR-UEFeatures-PowSav-03]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email approval [100b-e-NR-UEFeatures-PowSav-03] during RAN1 #100bis-e. According to the Chairman’s Notes:

[100b-e-NR-UEFeatures-PowSav-03] Email discussion/approval on the medium priority item as in proposal 2 as in R1-2001869 by (4/24-4/28)– Ralf (ATT)

The following was discussed and agreed during RAN1 #100bis-e within the scope of [100b-e-NR-UEFeatures-PowSav-03] “Email discussion/approval on the medium priority item as in proposal 2 as in R1-2001869” [1].

# Summary of Email Approval [100b-e-NR-UEFeatures-PowSav-03]

The following is the proposal in [1] for approval in this email discussion:

FL Proposal 2 (19-2):

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 19-2 | Cross Slot Scheduling | - Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset - minimumSchedulingOffset K2 configuration for PUSCH |  |  |  |  | N/A |  | Per UE | No or Yes | No or Yes |  |  | Optional with capability signalling |

- Medium priority

  - Agree on xDD/FRx differentiation

Companies are asked to provide their views and comments in the following table.

xDD differentiation

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | No |
| MediaTek | No |
| ZTE | NO |
| CATT | No |
| Intel | No |
| Apple | No |
| Ericsson | No |
| Qualcomm | No |

FRx differentiation

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | No |
| MediaTek | No |
| ZTE | NO |
| CATT | No |
| Intel | No |
| Apple | No |
| Ericsson | Yes |
| Qualcomm | Yes |

There are other fields empty that we may want to try to agree on to conclude the entire row. You are invited to provide your views below.

Prerequisite feature groups

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | No |
| Qualcomm | 5-30, 5-30a, or 5-31 |

Need for the gNB to know if the feature is supported

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Yes |
| CATT | Yes if this feature is optional with UE capability |
| Apple | Yes |
| Ericsson | Yes, need to enable feature for the UE |
| Qualcomm | Yes |

Consequence if the feature is not supported by the UE

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Dynamic adaptation of cross-slot scheduling is not supported. |
| Apple | Dynamic adaptation of the minimum value of K0min/K2min for cross-slot scheduling is not supported. |

Capability interpretation for mixture of FDD/TDD and/or FR1/FR2

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Conclusions

After further discussion on the RAN1 email reflector the following was agreed:

Agreement:

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 19-2 | Cross Slot Scheduling | (1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset (2) minimumSchedulingOffset K2 configuration for PUSCH |  | FFS |  | Yes | N/A | Dynamic adaptation of the minimum value of K0min/K2min for cross-slot scheduling is not supported | Per UE | No | No | N/A |  | Optional with capability signalling |

# References

- R1-2001869, Summary on UE features for UE power savings, Moderator (AT&T)