3GPP TSG RAN WG1 #100bis			                                   					       												        R1-2003103

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.7

Source:	Moderator (AT&T)

Title:	Summary of Email Approval [100b-e-NR-UEFeatures-PowSav-05]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email approval [100b-e-NR-UEFeatures-PowSav-05] during RAN1 #100bis-e. According to the Chairman’s Notes:

[100b-e-NR-UEFeatures-PowSav-05] Email discussion/approval on the medium priority item as in proposal 4 as in R1-2001869 by (4/24-4/28)– Ralf (ATT)

The following was discussed and agreed during RAN1 #100bis-e within the scope of [100b-e-NR-UEFeatures-PowSav-05] “Email discussion/approval on the medium priority item as in proposal 4 as in R1-2001869” [1].

# Summary of Email Approval [100b-e-NR-UEFeatures-PowSav-05]

The following is the proposal in [1] for approval in this email discussion:

FL Proposal 4 (19-4):

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 19-4a | UE assistance information | Support of reporting preferred minimum K0/K2 via UE assistance information - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots | 19-2 |  | N/A |  | Per UE | No | No |  | FFS: This capability is indicated only if UE supports the UE capability of dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB. The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted FFS: The minimum applicable value candidates of K0/K2 are {1, 2, 4, 6} for BWP with 15kHz/ 30kHz SCS and {2, 4, 8, 12} for BWP with 60kHz/ 120kHz SCS. FFS: UE preferred K0/K2 for Cross-carrier scheduling is FFS in RAN1 | Optional with capability signalling |

- Medium priority

  - Conclude on candidate component values

Companies are asked to provide their views and comments in the following table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| CATT | Three FFS points in the note are not needed. The prerequisite of 19-2 already captures the first FFS. The 2nd part has been included in the component description. We never have agreement of the 3rd FFS |
| Ericsson | The first two FFS points are not needed. 3rd one reflects RAN1 agreement and good to keep it until FFS resolved. |

There are other fields empty that we may want to try to agree on to conclude the entire row. You are invited to provide your views below.

Need for the gNB to know if the feature is supported

| Company | Comments/Questions/Suggestions |
| --- | --- |
| CATT | Yes |

Consequence if the feature is not supported by the UE

| Company | Comments/Questions/Suggestions |
| --- | --- |
| CATT | No consequence |

Capability interpretation for mixture of FDD/TDD and/or FR1/FR2

| Company | Comments/Questions/Suggestions |
| --- | --- |
| CATT | No differentiation between FDD/TDD and between FR1/FR2 |

# Conclusions

After further discussion on the RAN1 email reflector the following was agreed:

Agreement:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 19-4a | UE assistance information | Support of reporting preferred minimum K0/K2 via UE assistance information - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots | 19-2 | Yes | N/A |  | Per UE | No | No | N/A | The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted | Optional with capability signalling |

# References

- R1-2001869, Summary on UE features for UE power savings, Moderator (AT&T)