3GPP TSG RAN WG1 Meeting #101-e                                      R1-2003474

e-Meeting, May 25th – June 5th, 2020

Source:         ZTE

Title:            Maintenance of UE and gNB measurements for NR Positioning

Agenda item:    7.2.8.3

Document for:	Discussion and Decision

Introduction

In this contribution, we present our view on text changes of UE and gNB measurements for NR positioning.

The reference to determine time stamp

The following agreement was made in RAN1#99 [2],

Agreement:

A UE measurement can be associated with a time stamp. For UE RSTD, DL PRS RSRP and UE Rx-Tx time difference measurement report, the time stamp can include the SFN, as well as the slot number for a subcarrier spacing. These values correspond to the reference provided by the DL-PRS-RstdReferenceInfo.

In addition, UE can choose to use a different reference than the one indicated by the network for DL-TDOA measurement, which is reported by nr-DL-PRS-ReferenceInfo-r16 in nr-DL-TDOA-SignalMeasurementInformation-r16. However, it’s unclear in current specification whether the time stamp is determined by the reference indicated by network or the reference reported by UE. For this reason, we propose to have text changes to make it clear.

Proposal 1: To have a clear reference to determine time stamp, we propose to have following text changes,

========================TS 38.214 clause 5.1.6.5 unchanged parts omitted======================

For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE can report an associated higher layer parameter nr-TimeStamp-r16. The nr-TimeStamp-r16 can include the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by nr-DL-PRS-ReferenceInfo-r16 in nr-DL-PRS-AssistanceData-r16 or reported by nr-DL-PRS-ReferenceInfo-r16 in nr-DL-TDOA-SignalMeasurementInformation-r16 if the UE chooses to use a different reference than indicated by the network.

================================unchanged parts omitted================================

Conclusions

In this contribution, we have discussed possible text changes for the reference to determine time stamp. Based on the discussion, we have following proposal.

Proposal 1: To have a clear reference to determine time stamp, we propose to have following text changes,

========================TS 38.214 clause 5.1.6.5 unchanged parts omitted=======================

For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE can report an associated higher layer parameter nr-TimeStamp-r16. The nr-TimeStamp-r16 can include the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by nr-DL-PRS-ReferenceInfo-r16 in nr-DL-PRS-AssistanceData-r16 or reported by nr-DL-PRS-ReferenceInfo-r16 in nr-DL-TDOA-SignalMeasurementInformation-r16 if the UE chooses to use a different reference than indicated by the network.

==================================unchanged parts omitted===============================

References

[1] 3GPP TS 38.214 V16.1.0 (2020-05), Physical layer procedures for data (Release 16)

[2] Final Report of 3GPP TSG RAN WG1 #99 v1.0.0 (Reno, USA, 18th–22nd November 2019)