3GPP TSG RAN WG1 #101			R1-2003670

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.4.2.1

Title:	Sidelink mode-1 resource allocation

Source:	MediaTek Inc.

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss the remaining issues on Rel-16 NR V2X mode-1 resource allocation. We discuss minimum DCI-to-PSCCH/PSSCH processing times, DCI 3_0 content and payload size, SL-HARQ reporting to gNB.

# PSCCH/PSSCH preparation time

The minimum PSCCH/PSSCH preparation time in LTE V2X is 4 subframes (i.e. 4ms). Similarly in NR, the minimum slot offset indication in DCI 3_0 should not be shorter than a UE minimum processing time. The minimum value should also depend on sidelink numerology.

We propose the following:

Proposal 1: The minimum preparation time from DCI to PSCCH/PSSCH transmission should not be shorter than NR PUSCH preparation time values according to active sidelink numerology.

# DCI 3_0 content and payload size

In RAN1-99 [1], the following agreement was made on DCI format 3_0 payload size:

Agreements: - Existing DCI size budget is maintained when the UE is configured with SL - (Working assumption): The size of the new DCI format and the size of one of the existing NR DCI formats are aligned.

According to 38.212 v16.1 [2], the size of each DCI format 3_0 field is listed in Table 1. As the existing DCI format 0_0/1_0 have a payload of at least 34 bits or more depending on active BWP (excluding padding bits), the alignment of payload sizes between DCI format 3_0 and DCI format 0_0/1_0 appears to be feasible by zero padding.

We have the following proposal:

Proposal 2: Confirm working assumption. DCI format 3_0 payload size is aligned with DCI format 0_0/1_0 by zero padding.

Table 1: DCI format 3_0 field size.

| DCI field | Number of bits |
| --- | --- |
| Time gap | [x] bits ( e.g., x is 3-4 bits) |
| HARQ process ID | [x] bits ( e.g., x is 3-4 bits) |
| NDI | 1 bit |
| Lowest index of the subchannel allocation to initial transmission | log2(N subChannel SL) bits |
| SCI frequency resource assignment | log2(N subChannel SLN subChannel SL + 12) or log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits |
| SCI time resource assignment | 5 or 9 bits |
| PSFCH-to-HARQ feedback timing indicator | 3 bits |
| PUCCH resource indicator | 3 bits |
| Configuration index | 0 or [x] bits (e.g., x is 3 bits) |

In addition, the size of DCI 3_0 should be the same for dynamic grants and configured grant type-2 activation/deactivation. Size alignment can be achieved by using zero padding.

Proposal 3: DCI format 3_0 size is aligned using zero padding between dynamic grants and configured grants type-2 activation/deactivation.

# SL-HARQ reporting to gNB

In RAN1#100bis-e [3], the following agreement was made on minimum preparation time for SL-HARQ reporting on UL.

Agreements: - A UE does not expect to be scheduled to transmit the UL report corresponding to a PSFCH reception earlier than Tprep after the end of the PSFCH. - This includes the effect of time advance. - Tprep = (N+X) ∙ (2048+144) ∙ k ∙ 2 –μ ∙ T_c where: - Working assumption: N is 14, 18, 28 and 32 corresponds to the SCS configuration μ of 0, 1, 2 and 3, μ = min(μ_SL, μ_UL) - k = T_s / T_c (parameters as defined in 38.211) - FFS X (including the possibility of value 0)

We propose the following:

Proposal 4: Confirm the working assumption:

- N is 14, 18, 28 and 32 corresponds to the SCS configuration μ of 0, 1, 2 and 3, μ = min(μ_SL, μ_UL).

In RAN1#100bis-e [3], the following agreements were made on PUCCH resource indication for SL-HARQ reports on UL.

Agreements: - Higher layer signaling is used to configure the values of the PSFCH to PUCCH gap (NOTE: this is referred to as sl-FeedbackToUL-ACK in the following) - The field PSFCH-to-HARQ_feedback timing indicator: - Selects one of the configured values of the PSFCH to PUCCH gap, except in the case that, together with PUCCH resource indicator, it indicates that no PUCCH resource is provided. - FFS Presence in DCI format 3_0 and size (0-3 bits).

We propose the following for the PUCCH indicator bitfield size in DCI 3_0:

Proposal 5: The PUCCH indicator field in DCI 3_0 contains 3 bits.

# Conclusions

We have the following proposals:

Proposal 1: The minimum preparation time from DCI to PSCCH/PSSCH transmission should not be shorter than NR PUSCH preparation time values according to active sidelink numerology.

Proposal 2: Confirm working assumption. DCI format 3_0 payload size is aligned with DCI format 0_0/1_0 by zero padding.

Proposal 3: DCI format 3_0 size is aligned using zero padding between dynamic grants and configured grants type-2 activation/deactivation.

Proposal 4: Confirm the working assumption:

- N is 14, 18, 28 and 32 corresponds to the SCS configuration μ of 0, 1, 2 and 3, μ = min(μ_SL, μ_UL).

Proposal 5: The PUCCH indicator field in DCI 3_0 contains 3 bits.

# References

- Chairman’s notes, RAN1#99, Reno, USA, November 2019.

- TS 38.212 version 16.1.0, “Multiplexing and channel coding”, Release 16, March 2020.

- Chairman’s notes, RAN1#100bis, e-Meeting, April 2020.