3GPP TSG RAN WG1 #101	                                                                 R1-2003905

# e-Meeting, May 25th – June 5th, 2020

Agenda item:	8.1.3

Source:	Samsung

Title:	Evaluation assumptions for SLS and LLS

Document for:	Discussion and Decision

# Introduction

In RAN #86, a new study item for Rel-17 was approved to extend NR to up to 71 GHz [1]. This contribution will discuss the evaluation assumptions for SLS and LLS to study the penitential changes to NR for support new numerology and carrier bandwidth assuming same waveform of DL and UL are reused, as well as the channel access mechanism, considering potential interference to/from other nodes, to comply with regulation requirement and to guarantee fair coexistence with other RAT operating on 60 GHz unlicensed band.

# Evaluation Assumptions for SLS

As identified in our accompany contribution [2], for 60 GHz unlicensed band, at least indoor hotspot scenario should be prioritized for study and evaluation. The layout for indoor hotspot scenario can reuse the one for indoor evaluation of sub7 GHz as described in TR 38.889 [3], wherein two operators each with 3 gNBs are deployed in a room of size 120 meters by 80 meters as shown in Figure 1, and the gNB of the same color belongs to the same operator. The parameters are of value a=20 meters, b=40 meters, c=20 meters, and d=40 meters. The deployment scenario is selected to achieve a target serving link RSSI distribution with 10%-15% serving link below -72dBm.

Figure 1: Indoor hotspot layout for SLS.

The evaluation assumptions of SLS for the indoor hotspot layout can be summarized in Table 1, wherein the parameters are updated from TR 38.889 [3].

Table 1: SLS evaluation parameters for indoor hotspot scenario

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 60 GHz |
| Carrier Channel Bandwidth | 2.16 GHz |
| Number of carriers | 1 |
| Number of users per operator | 5 UEs associated with each gNB per 2.16 GHz |
| Numerology | 960 kHz SCS with NCP and/or ECP |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 18 dBm (total across all TX antennas) |
| UE/STA Tx Power | 21 dBm (total across all TX antennas) |
| BS/AP Antenna gain | 5 dBi |
| UE/STA Antenna gain | 5 dBi |
| BS/AP Noise Figure | 7 dB |
| UE/STA Receiver Noise Figure | 10 dB |
| Minimum received power from serving cell for UE dropping | -82 dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (8, 16, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

Proposal 1: For indoor hotspot development scenario, use Table 1 as a baseline for SLS evaluation parameters.

# Evaluation Assumptions for LLS

To study the potential issues with the new numerology and carrier bandwidth for carrier frequency between 52.6 GHz and 71 GHz, a set of LLS is needed to evaluate the performance of SS/PBCH block, RACH, and data transmission. The evaluation assumptions for the above targets are described in Table 2, Table 3, and Table 4, correspondingly.

Table 2: LLS evaluation parameters for initial cell search

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 60 GHz |
| Channel Model | TDL/CDL |
| Numerology | 960 kHz SCS with NCP and/or ECP |
| Antenna Configuration at gNB | (M, N, P, Mg, Ng) = (8, 16, 2, 1, 1), dH = dV = 0.5 λ |
| Antenna Configuration at UE | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 1), dH = dV = 0.5 λ |
| UE speed | 3 km/h |
| Initial CFO | 5 ppm |
| Phase Noise Model | Phase noise profile according to TR 38.803 |
| Target SNR | ≥ -6 dB |
| Evaluation Metric | PCID one-shot detection performance with 1% false alarm rate; CDF of timing and frequency residual offset |

Table 3: LLS evaluation parameters for PRACH preamble

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 60 GHz |
| Channel Model | TDL/CDL |
| Numerology | 960 kHz SCS with NCP and/or ECP |
| Antenna Configuration at gNB | (M, N, P, Mg, Ng) = (8, 16, 2, 1, 1), dH = dV = 0.5 λ |
| Antenna Configuration at UE | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 1), dH = dV = 0.5 λ |
| UE speed | 3 km/h |
| Frequency Offset | 0.1 ppm |
| Timing Offset | A uniform distribution in [0, max differential delay] |
| Phase Noise Model | Phase noise profile according to TR 38.803 |
| Target SNR | ≥ -6 dB |
| Evaluation Metric | PRACH detection rate; False alarm rate (based on the preamble pool size is not less than 64); CDF of estimation error for frequency/timing |

Table 4: LLS evaluation parameters for data transmission

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 60 GHz |
| Channel Model | TDL/CDL |
| Numerology | 960 kHz SCS with NCP and/or ECP |
| Antenna Configuration at gNB | (M, N, P, Mg, Ng) = (8, 16, 2, 1, 1), dH = dV = 0.5 λ |
| Antenna Configuration at UE | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 1), dH = dV = 0.5 λ |
| UE speed | 3 km/h |
| Frequency Offset | 0.1 ppm |
| Channel Estimation | Realistic |
| Phase Noise Model | Phase noise profile according to TR 38.803 |
| Target SNR | ≥ -6 dB |
| Evaluation Metric | BLER; Throughput |

Proposal 2: Use Table 2, Table 3, and Table 4 as a baseline for LLS evaluation parameters.

# Conclusion

In this contribution, we have discussed the evaluation assumptions for SLS and LLS. Proposals of this contribution are summarized as follows.

Proposal 1: For indoor hotspot development scenario, use Table 1 as a baseline for SLS evaluation parameters.

Proposal 2: Use Table 2, Table 3, and Table 4 as a baseline for LLS evaluation parameters.

# Reference

[1] RP-193259, New SID: Study on supporting NR from 52.6GHz to 71 GHz, Intel Corporation, 2019.