3GPP TSG RAN WG1 #101		R1-2004050

e-Meeting, May 25th – June 5th, 2020

Source:	OPPO

Title:	Text proposals for low PAPR RS

Agenda Item:	7.2.6.5

Document for:	Discussion and Decision

Introduction

In this contribution, we present our views on the remaining issues of the new low PAPR DMRS introduced in Rel-16 and propose some text proposals to update the specification.

Discussion

Low-PAPR sequence generation type 2

In RAN1#100bis-e meeting, regarding the cyclic shift of lower PAPR RS, we had the following agreement [1]

Agreement Remove specification text related to cyclic shifts for low PARP RSs.

According to the agreement, an CR for 38.211 is implemented as follows [2]

###### 6.4.1.3.3.1 Sequence generation The reference-signal sequence rlm shall be generated according to where MscPUCCH,s is given by clause 6.3.2.6.3 and ru,vα,δ(m) depends on the configuration: - if the higher-layer parameter dmrs-UplinkTransformPrecodingPUCCH-r16 is configured, and π2-BPSK is used for PUCCH, ru,vα,δ(m) is given by clause 5.2.3 with cinit given by clause 6.4.1.3.2.1. The sequence group u and the sequence number v depend on the sequence hopping in clause 6.3.2.2.1α. <omitted text>

In the current version of TS 38.211, the description on the low-PAPR sequence generation type 2 is as follows [3]

### 5.2.3 Low-PAPR sequence generation type 2 The low-PAPR sequence ru,v(α,δ)n is defined by a base sequence ru,vn according to ru,v(α,δ)n=ru,vn, 0≤n<M where M=mNscRB2δ is the length of the sequence. Multiple sequences are defined from a single base sequence through different values of α and δ.

The highlighted part is confusing as it seems  α and δ can still control the generation of low-PAPR sequences. It is not real. Thus we propose to fix it to avoid the confusion.

Proposal 1: Clarify that the low-PAPR sequence of type 2 will not change for different values of α and δ in Section 5.2.3 with the ones in TS 38.211

The following gives the corresponding text proposal for Proposal 1:

In TS 38.211 Section 5.2.3 <omitted text> ### 5.2.3 Low-PAPR sequence generation type 2 The low-PAPR sequence ru,v(α,δ)n is defined by a base sequence ru,vn according to ru,v(α,δ)n=ru,vn, 0≤n<M where M=mNscRB2δ is the length of the sequence, α=0, δ=0 . Multiple sequences are defined from a single base sequence through different values of α and δ. Base sequences ru,vn are divided into groups, where u∈0,1,…,29 is the group number and v is the base sequence number within the group, such that each group contains one base sequence (v=0) of length M=mNscRB2δ, 12≤m2δ. The sequence ru,v0,…,ru,vM-1 is defined by ru,vn=1Mi=0M-1ru,vie-j2πinM n=0,…,M-1 where the definition of ru,vi depends on the sequence length. <omitted text>

PDSCH DMRS

There is an alignment for RRC parameter.

Proposal 2: Align the following RRC parameters in Section 7.4.1.1.1 with its counterpart in TS 38.331

- dmrsDownlink-r16   ->  dmrs-Downlink-r16

The following gives the corresponding text proposal for Proposal 3:

In TS 38.211 Section 7.4.1 <omitted text> - 7.4.1.1 Demodulation reference signals for PDSCH ##### 7.4.1.1.1 Sequence generation The UE shall assume the sequence is defined by . where the pseudo-random sequence is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with cinit=217Nsymbslotns,fμ+l+12NIDnSCIDλ+1+217λ2+2NIDnSCIDλ+nSCIDλmod 231 where is the OFDM symbol number within the slot, ns,fμ is the slot number within a frame, and - NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-DownlinkConfig IE if provided and the PDSCH is scheduled by PDCCH using DCI format 1_1 or 1_2 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI - NID0∈0,1,…,65535 is given by the higher-layer parameter scramblingID0 in the DMRS-DownlinkConfig IE if provided and the PDSCH is scheduled by PDCCH using DCI format 1_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI; - NIDnSCIDλ=NIDcell otherwise; - nSCIDλ and λ are given by - if the higher-layer parameter dmrs-Downlink-r16 in the DMRS-DownlinkConfig IE is provided nSCIDλ=nSCIDλ=0 or λ=21-nSCIDλ=1 λ=λ where λ is the CDM group defined in clause 7.4.1.1.2. - otherwise by nSCIDλ=nSCID λ=0 The quantity nSCID∈0, 1 is given by the DM-RS sequence initialization field, if present, in the DCI associated with the PDSCH transmission if DCI format 1_1 or 1_2 in [4, TS 38.212] is used, otherwise nSCID=0. <omitted text>

Conclusion

In this contribution, we present our view on the current specification regarding Rel-16 new DMRS (low PAPR RS). The following proposals and the corresponding text proposals are provided.

Proposal 1: Clarify that the low-PAPR sequence of type 2 will not change for different values of α and δ in Section 5.2.3 with the ones in TS 38.211

Proposal 2: Align the following RRC parameters in Section 7.4.1.1.1 with its counterpart in TS 38.331

- dmrsDownlink-r16   ->  dmrs-Downlink-r16

References

- Chairman’s notes of RAN1#100bis-e meeting

- R1-2003164, Corrections to MIMO enhancements (for TS 38.211)

- TS 38.211, NR; Physical channels and modulation, v16.1.0

- TS 38.331, NR; Radio Resource Control (RRC) protocol specification, v16.0.0