3GPP TSG RAN WG1#101e	        	R1-2004198

E-meeting, May 25th- June 5th, 2020

Agenda Item:					    8.4.2

Source:	Sony

Title:	Techniques for NR coverage enhancement

Document for:	Discussion

# Introduction

The new SID on NR coverage enhancement (CE) [1], makes clear that service quality, CAPEX as well as OPEX are all heavily influenced by the coverage. It is therefore our view that any technique that can potentially be used to enhance the coverage is worth investigating.

In the agenda it is stated that this item is a placeholder and that it will not be treated during the e-meeting. We think that it is still useful that companies provide their initial perspectives on coverage enhancement techniques for later reference. Hence in this document we list and provide initial considerations on techniques that we think can have an influence on the coverage for later reference.

The SID on NR coverage enhancement includes the following objectives (where the de-emphasized text is less relevant to this document).

The objective of this study item is to study potential coverage enhancement solutions for specific scenarios for both FR1 and FR2. The detailed objectives are as follows.

- The target scenarios and services include

  - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1

  - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2.

  - TDD and FDD for FR1.

  - VoIP and eMBB service for FR1.

  - eMBB service as first priority and VoIP as second priority for FR2.

  - LPWA services and scenarios are not included.

- Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation

  - UL channels (including PUSCH and PUCCH) are prioritized for FR1.

  - Both DL and UL channels for FR2.

- Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services

  - The target channels include at least PUSCH/PUCCH

  - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions)

  - Study the additional enhanced solutions for FR2 if any

  - Evaluate the performance of the potential solutions based on link level simulation.

The document is separated into two parts addressing FR1 and FR2 separately.

# Coverage enhancements for FR1

It has been observed in the IMT-2020 self-evaluation study [2] that in some scenarios, coverage is limited by UL channels. In [3], we observe the following:

- In some scenarios, UL coverage is less than DL coverage due to the larger transmit power applied in the DL

- In some indoor scenarios (InH-eMBB), DL coverage is worse than UL coverage

- Increasing the number of antenna elements improves the coverage, but does not have an effect on the link imbalance (between UL and DL)

- Data channels generally have inferior coverage to the control channels

Hence there is a need to improve the coverage of both DL and UL channels, with an emphasis on UL channels (as described in the SID).

The following paragraphs provide our views on a list of techniques that could conceivably improve NR coverage in FR1.

Firstly, we consider a set of techniques applicable to both data and control channels.

Repetition / TTI bundling. These techniques have been used in LTE-MTC. These techniques reduce the data rate in the DL and UL and would not be acceptable for a service with a fixed data rate requirement. Repetition is only really relevant when the UE is being scheduled with a minimum MCS: if the UE had been scheduled with a non-minimum MCS, coverage could alternatively be improved by simply choosing a lower MCS.

Lower data rate. Operation at a lower data rate improves coverage, similarly to repetition, but operation at a lower data rate may not be acceptable from a service perspective. Operation at a lower rate would mainly be an implementation choice, but the specifications could provide further support by defining lower code rate MCS.

High BLER target operation. Operation at a higher BLER target and relying on re-transmissions allows the system to operate at a lower SNR target, which improves coverage. The drawback of higher BLER target operation is that it increases latency and jitter. Operating at a higher BLER target is an implementation choice that does not require specification.

Narrower beams / more antenna elements. More antenna elements at the gNodeB provide higher transmit array gain and higher receiver array gain. The higher transmit array gain improves the link budget in the downlink and the higher receiver array gain improves the link budget in the uplink. To a large extent, the use of more antenna elements and finer beams at the gNodeB is an implementation choice, but the use of finer beams might have some impact on beam tracking or beam measurement procedures. More antenna elements can also be applied at the UE, increasing diversity but at the expense of UE cost and form factor.

Increased number of SSBs. The use of beam refinement gives some flexibility to balance the antenna array gain between e.g. SSB and data channels. Currently the SSB count is limited to 4 SSBs for frequencies below 3 GHz and to 8 SSBs for the remaining part of the FR1 band. Those limitations significantly reduce the flexibility to introduce e.g.  TDD massive MIMO with SRS based precoding. The problem originates from the imbalance caused by massive array gain for the transport channels not being present for the SSBs. It is suggested that the influence of number of SSBs in FR1 is investigated for massive MIMO systems as the coverage otherwise may be limited by the SSBs.

Multi-TRP transmission and reception. Multi-TRP transmission can improve coverage in the UL and DL by increasing diversity. Use of multi-TRP techniques is an implementation choice that is supported in Release-16. SFN operation can improve coverage in the DL. These issues are better addressed in MIMO work item(s).

DMRS enhancements. More DMRS lead to better channel estimation. Fewer DMRS leave more resource elements available for data / control channels, lowering the coding rate applied to those channels, with an associated improvement in SNR performance. The optimal quantity and type of DMRS can be studied in this study item.

Frequency hopping. Frequency hopping is supported in Rel-16 for PUSCH. It is beneficial for the frequency selective channel with flattish fading across the transmission bandwidth, where the scheduler does not know the channel conditions (if the scheduler knew the channel conditions, it could apply frequency selective scheduling instead). The scheduler may not have good knowledge of UL channel conditions at the edge of coverage since measurements on SRS are likely to be less accurate at lower SRS-SNR. Various enhancements to frequency hopping can be considered, including increasing the number of hops and UE feedback information on which hops are more effective (there is no point having the UE transmit on a frequency hop that is faded).

Time interleaving. Time diversity can be achieved if the transmission time of a channel is greater than the coherence time of the channel. Time diversity can be achieved by time interleaving transmissions. A time interleaving technique would have an impact on latency.

Higher TX power. Higher transmit power directly affects the link budget. One of the reasons that DL channels have significantly better coverage in the DU-eMBB coverage analysis in the IMT-2020 self-evaluation is that the total transmit power in the DL is 21dB greater than in the UL. The maximum UE transmit power is limited by regulations (such as SAR regulations), though it may be that some types of UE (such as vehicular UEs) could transmit more power than other UEs (as is the case in LTE). These issues can be considered by RAN4.

Small cells / relays. Coverage can be improved by deploying small cells or relays. However this is not always an attractive solution to a coverage problem as it increases network deployment cost.

Sidelink relay. Sidelink relay can provide an opportunistic method of improving coverage, where coverage is provided to a UE via an intermediate UE. Sidelink relays are already under consideration in other working groups. Most of the specification work on sidelink relays would not need to be done in RAN1, since most of the basic sidelink functionality has already been specified. However RAN1 could consider the coverage implications of the use of sidelink relaying in the study item.

Secondly, we consider a set of techniques applicable to control channels.

Compact DCI. Through transmitting fewer DCI bits in the same physical resource, compact DCI improves processing gain and hence improves SNR performance and link budget. Compact DCI has already been defined for URLLC and its configurability means that it can potentially be used for the purposes of coverage enhancement with little further specification impact. Furthermore, control channels were found to have better coverage than data channels in the IMT-2020 self-evaluation study. Hence compact DCI techniques should be considered to be a low priority in the coverage enhancements SI.

Higher Aggregation Level. Use of a higher aggregation level for PDCCH can improve coverage. This technique can be considered to be a low priority since control channels were not found to be the coverage-limiting factor in the IMT-2020 self-evaluation study.

Compact UCI. Minimisation of uplink control signaling should improve UL coverage. This minimization of UL control signaling can either be achieved through the use a compact UCI format or by compacting / not transmitting some of the CSI information. Minimisation of CSI signaling may impact the performance of DL channels, which would inadvertently help to rectify the DL / UL coverage imbalance.

Based on the discussion above and considering the relative importance of improving the coverage of UL channels, it is proposed that RAN1 focusses on the following aspects for NR coverage enhancements:

- More antenna elements / narrower beams

- DMRS enhancements

- Frequency hopping enhancements

- Time interleaved transmissions

- Increased number of SSBs

Proposal 1: For FR1, RAN1 focusses on the following aspects for NR coverage enhancement:

- More antenna elements / narrower beams

- DMRS enhancements

- Frequency hopping enhancements

- Time interleaved transmissions

- Increased number of SSBs

# Coverage enhancements for FR2.

In general, the following proposals can potentially improve the coverage in FR2. Some of them will furthermore be proposed in the MIMO sessions. This FR2 list is based on our FR1 list where FR2 related aspects have been added.

Firstly, we consider a set of techniques applicable to both data and control channels.

Repetition / TTI bundling. These techniques have been used in LTE-MTC. These techniques reduce the data rate in the DL and UL and would not be acceptable for a service with a fixed data rate requirement. Repetition is only really relevant when the UE is being scheduled with a minimum MCS: if the UE had been scheduled with a non-minimum MCS, coverage could alternatively be improved by simply choosing a lower MCS.

Lower data rate. Operation at a lower data rate improves coverage, similarly to repetition, but operation at a lower data rate may not be acceptable from a service perspective. Operation at a lower rate would mainly be an implementation choice, but the specifications could provide further support by defining lower code rate MCS.

High BLER target operation. Operation at a higher BLER target and relying on re-transmissions allows the system to operate at a lower SNR target, which improves coverage. The drawback of higher BLER target operation is that it increases latency and jitter. Operating at a higher BLER target is an implementation choice that does not require specification.

Narrower beams / more antenna elements at gNB. More antenna elements at the gNodeB provide higher transmit array gain and higher receiver array gain. The higher transmit array gain improves the link budget in the downlink and the higher receiver array gain improves the link budget in the uplink. To a large extent, the use of more antenna elements and finer beams at the gNodeB is an implementation choice, but the use of finer beams might have some impact on beam tracking or beam measurement procedures. Also, the use of beam refinement gives some flexibility to balance the antenna array gain between e.g. SSB and data channels.

UE antenna count. More antenna elements at each panel can be applied at the UE, increasing array gain. UE beam width is so far transparent to the system. The UE beam width selection will influence the general interference level in a cell and potentially influence the coverage.

Multi-TRP transmission and reception. Multi-TRP transmission can improve coverage in the UL and DL by increasing diversity. Use of multi-TRP techniques is an implementation choice that is supported in Release-16. SFN operation can improve coverage in the DL. Multi TRP and related UE multi panel/multi beam operation is expected to be addressed in the MIMO sessions but may have a bearing on the coverage.

DMRS enhancements. More DMRS lead to better channel estimation. Fewer DMRS leave more resource elements available for data / control channels, lowering the coding rate applied to those channels, with an associated improvement in SNR performance. The optimal quantity and type of DMRS can be studied in this study item. The spatially filtered channels at FR2 may motivate variable DMRS granularity in the frequency domain.

Frequency hopping. Frequency hopping is supported in Rel-16 for PUSCH. It is beneficial for the frequency selective channel, where the scheduler does not know the channel conditions (if the scheduler knew the channel conditions, it could apply frequency selective scheduling instead). The scheduler may not have good knowledge of UL channel conditions at the edge of coverage since measurements on SRS are likely to be less accurate at lower SRS-SNR. Various enhancements to frequency hopping can be considered, including increasing the number of hops and UE feedback information on which hops are more effective (there is no point having the UE transmit on a frequency hop that is faded).

Time interleaving. Time diversity can be achieved if the transmission time of a channel is greater than the coherence time of the channel. Time diversity can be achieved by time interleaving transmissions. A time interleaving technique would have an impact on latency.

Higher TX power. Higher transmit power directly affects the link budget. One of the reasons that DL channels have significantly better coverage in the DU-eMBB coverage analysis in the IMT-2020 self-evaluation is that the total transmit power in the DL is 21dB greater than in the UL. The maximum UE transmit power is limited by regulations (such as SAR/MPE regulations), though it may be that some types of UE (such as vehicular UEs) could transmit more power than other UEs (as is the case in LTE). These issues can be considered by RAN4.

Small cells / relays. Coverage can be improved by deploying small cells or relays. However, this is not an attractive solution to any coverage problem as it increases network deployment cost.

Sidelink relay/reflective arrays. Sidelink relay can provide an opportunistic method of improving coverage, where coverage is provided to a UE via an intermediate UE. Sidelink relays are already under consideration in other working groups. Most of the specification work on sidelink relays would not need to be done in RAN1, since most of the basic sidelink functionality has already been specified. However, RAN1 could consider the coverage implications of the use of sidelink relaying in the study item. Furthermore, reflective array nodes have recently seen extensive research interest in order to enhance coverage and/or mitigate shadowing in FR2 scenarios and could be a low-cost option to enhance coverage.

UE TX diversity. Multi UE beam-based diversity and related coherence aspects for UE panels can be studied and will likely be addressed in the MIMO sessions.

UE Spherical coverage. Is specified by RAN4 but highly relevant, affecting the probability of TX diversity at the UE side.

UE Beam correspondence. In relation to the UE spherical coverage, the beam correspondence (BC) is also a critical metric to determine the network coverage. Though RAN1 generally thinks of BC as a mandatory feature for a UE, RAN4 requirement actually allows certain UEs to use limited uplink beam sweeping to meet the spherical coverage requirement. In addition, all UEs will lose the BC capability when the SINR level drops below a certain level due to the estimation error on L1-RSRP. Therefore, it is important for RAN1 to investigate the BC performance and define necessary enhancements in the low SINR scenario as part of this coverage enhancement SI.

Secondly, we consider a set of techniques applicable to control channels.

Compact DCI. Through transmitting fewer DCI bits in the same physical resource, compact DCI improves processing gain and hence improves SNR performance and link budget. Compact DCI has already been defined for URLLC and it is unclear that a different form of compact DCI would be desirable for the purposes of coverage enhancement. Furthermore, control channels were found to have better coverage than data channels in the IMT-2020 self-evaluation study. Hence compact DCI techniques should be considered to be a low priority in the coverage enhancements SI.

Higher Aggregation Level. Use of a higher aggregation level for PDCCH can improve coverage. This technique can be considered to be a low priority since control channel were not found to be the coverage-limiting factor in the IMT-2020 self-evaluation study.

Compact UCI. Minimisation of uplink control signaling should improve UL coverage. This minimization of UL control signaling can either be achieved through the use a compact UCI format or by compacting / not transmitting some of the CSI information. Minimisation of CSI signaling may impact the performance of DL channels, which would inadvertently help to rectify the DL / UL coverage imbalance.

Polarization of Reference signals. In general, the performance at cell edge heavily depends on the channel knowledge. Channel knowledge for UL transmissions has limitations as of today and depends on reciprocity, beam correspondence, and polarization of possibly both UL and DL reference signals.

In the Rel-16 specification the polarization aspect of beams is largely overlooked and in general transparent to the operation. The best approach for identification of the best beam-pair is based on that the receiver is expected to receive with dual polarized antennas. This is however not mandatory for a UE or even possible for any beam angle, and when supported by the UE requires two receivers to be active. Yet another drawback with single polarized RSs is that the entire channel is not sounded. It is therefore not possible for a UE to estimate the true potential of a beam-pair. Various techniques to improve CSI can be employed. The different techniques may have different overhead and different levels of complexity and a careful evaluation of the benefit of each technique is needed. Dual polarized reference signals can be implemented in dedicated resources associated with the existing reference signals or integrated with the existing reference signals with backwards compatibility and low to zero overhead.

Based on the discussion above, it is proposed that RAN1 focusses on the following aspects for NR coverage enhancements:

- More antenna elements / narrower beams at gNB

- UE Antenna configuration (antennas/panel, spherical coverage, multi beam capability, BC)

- DMRS enhancements

- Frequency hopping enhancements

- Time interleaved transmissions

- Sidelink relay/reflective arrays

- Polarization aspects of the UL and/or DL reference signals

Proposal 2: For FR2, RAN1 focusses on the following aspects for NR coverage enhancement:

- More antenna elements / narrower beams at gNB

- UE Antenna configuration (antennas/panel, spherical coverage, multi beam capability, BC)

- DMRS enhancements

- Frequency hopping enhancements

- Time interleaved transmissions

- Sidelink relay/reflective arrays

- Polarization aspects of the UL and/or DL reference signals

# Conclusions

In this tdoc, the following proposals were made:

Proposal 1: For FR1, RAN1 focusses on the following aspects for NR coverage enhancement:

- More antenna elements / narrower beams

- DMRS enhancements

- Frequency hopping enhancements

- Time interleaved transmissions

- Increased number of SSBs

Proposal 2: For FR2, RAN1 focusses on the following aspects for NR coverage enhancement:

- More antenna elements / narrower beams at gNB

- UE Antenna configuration (antennas/panel, spherical coverage, multi beam capability, BC)

- DMRS enhancements

- Frequency hopping enhancements

- Time interleaved transmissions

- Sidelink relay/reflective arrays

- Polarization aspects of the UL and/or DL reference signals

# References

- RP-193240. “New SID on NR coverage enhancement”. RAN plenary # 86. Sitges, Spain. December 2019.

- TR37.910. “Study on self evaluation towards IMT-2020 submission”.

- R1-2004196 “On NR coverage analysis in FR1”. Sony. RAN1#101e. May 2020.