3GPP TSG RAN WG1 #101 		R1-2004249
e-Meeting, May 25th – June 5th, 2020

Agenda item:	8.4.1.1

Source:	Apple Inc.

Title:	On baseline coverage performance for FR1

Document for:	Decision

# Introduction

In RAN#86 meeting, new study item on NR coverage enhancement [1] was agreed. Some of objectives of this study item are showing below,

- The target scenarios and services include

  - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1

  - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2.

  - TDD and FDD for FR1.

  - VoIP and eMBB service for FR1.

  - eMBB service as first priority and VoIP as second priority for FR2.

  - LPWA services and scenarios are not included.

- Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation

  - UL channels (including PUSCH and PUCCH) are prioritized for FR1.

  - Both DL and UL channels for FR2.

In this contribution, we discuss the NR coverage enhancement scenarios and the simulation methodology.

# Baseline coverage for FR1

Before the study item was agreed, there were extensive discussions on the scenarios and objectives of the coverage enhancement over the RAN email reflector. During the discussion, the coverage concerns cover almost all channels. It is obvious deviating the target of this SI. For this study, the focus should identify the coverage bottleneck of specific channel first, then specify the schemes to improve the coverage. The coverage extension should be avoided, it could also cause the working area overlapping with SI reduced capability NR device.

Observation 1: Coverage enhancement study focuses on specific channels with the coverage bottleneck, coverage extension should avoid.

- Identify the coverage bottleneck of PRACH, msg3, VoIP, PUSCH and PUCCH formats in the UL

- Identify the coverage bottleneck of PBCH, PSS/SSS, VoIP, PDCCH formats, PDSCH in the DL

For FR1 Urban (outdoor gNB serving indoor UE) scenario, the following points could be better to clarify first.

- gNB served users are all indoor users.

- TDD NR frame structure in the evaluation, i.e., UL/DL ratio

For the rural area coverage scenario, to support the extreme coverage should not be prioritized, as it is not the typical deployment scenario, the target DL and UL data rate need to be considered additionally.

In Rel.15 NR design, serval techniques were already specified to support the coverage enhancement, such as the time domain transmission/reception repetition, frequency hopping, and PUSCH repetition enhancement in Rel.16, these should be the baseline for the further enhancement of the coverage.

Observation 2: Existing defined coverage enhancement techniques should be the benchmark for further coverage enhancement.

# Evaluation methodology

To identify the coverage issue, MCL based evaluation methodology can be used. The maximum coupling loss (MCL) is the limit value of the coupling loss at which the service can be delivered, and therefore defines the coverage of the service. It is defined in the UL and DL, and is evaluated via the link budget analysis. The following MCL calculation table can be considered for evaluation.

Table1: MCL calculation table

| Physical channel name | Value |
| --- | --- |
| Transmitter |  |
| (1) Tx power (dBm) |  |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) |  |
| (3) Receiver noise figure (dB) |  |
| (4) Interference margin (dB) |  |
| (5) Occupied channel bandwidth (Hz) |  |
| (6) Effective noise power = (2) + (3) + (4) + 10 log(5) (dBm) |  |
| (7) Required SINR (dB) |  |
| (8) Receiver sensitivity = (6) + (7) (dBm) |  |
| (9) MCL = (1) (8) (dB) |  |

For the detailed link level simulation assumption, the following common simulation parameters can be considered in the evaluation. In addition, some channel specific parameters would be required, such as probability of missed detection, probability of false alarm for PRACH and PUCCH evaluation.

Table 2: the common link level simulation parameter

| Parameters | NR |
| --- | --- |
| Carrier frequency | 4GHz |
| Waveform | DL: CP-OFDM UL: CP-OFDM as the starting point |
| Numerology | 15kHz |
| Services and bit rates | VoIP: DL 12.2 kbps, UL 12.2 kbps Uband: DL 10Mbps, UL 1Mbps Rural: DL 1Mbps, UL 100kbps |
| Channel bandwidth | 20 MHz |
| Target BELR | Up to evaluated channel |
| DMRS overhead | Rel.15 NR DMRS |
| Antenna configuration eNB | 8tx, 8rx |
| Antenna configuration UE | 4rx, 1tx |
| Propagation channel & UE velocity | TDL-A 30ns and TDL-C 300ns in TR38.901, 3km/h, |
| Channel estimation | Ideal channel estimation for calibration Realistic channel estimation |
| Metric for calibration | BLER vs. SNR |

Proposal 1: The common LLS assumptions in Table 2 can be considered for performance evaluation, including the carrier frequency, gNB and UE antenna configuration, waveform, numerology, propagation channel, etc.

# Summary

In this contribution, we discuss the coverage enhancement scenarios and simulation assumption, and have the following observations and proposal:

Observation 1: Coverage enhancement study focuses on specific channels with the coverage bottleneck, coverage extension should avoid.

- Identify the coverage bottleneck of PRACH, msg3, VoIP, PUSCH and PUCCH formats in the UL

- Identify the coverage bottleneck of PBCH, PSS/SSS, VoIP, PDCCH formats, PDSCH in the DL

Observation 2: Existing defined coverage enhancement techniques should be the benchmark for further coverage enhancement.

Proposal 1: The common LLS assumptions in Table 2 can be considered for performance evaluation, including the carrier frequency, gNB and UE antenna configuration, waveform, numerology, propagation channel, etc.

# References

- RP-193240, “New SID on NR coverage enhancement”, China Telecom