3GPP TSG RAN WG1 #101			R1-2004268

e-Meeting, May 25th – June 5th, 2020

Agenda item:		7.2.6.5

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance on Rel-16 Low PAPR RS

Document for:		Discussion and Decision

# Introduction

This contribution is providing some missing parts of NR Rel-16 specification regarding to the introduction of “Low PAPR RS” even though we have already good implementation with the excellent effort from the specification editors.

The proposed updates are listed as below.

- Clarification of the condition for applying different identity for the nIDRS.

- Clarification of the number of ports for PUCCH format 3/4 when new DMRS is applied.

# TS 38.211

Though we have agreed on the clarification of the text for nIDRS at the RAN1 #100e meeting, the condition is not clearly captured.

The correct formulation is,

- nIDRS=NIDnSCID is applied if the higher-layer parameter dmrs-UplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH

- nIDRS=nIDPUSCH if nIDPUSCH is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE, the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is not configured or, the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured but π/2-BPSK modulation is not used for PUSCH , the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space;

Also, “configured grant” in a sentence shall be clarified as “a type-1 configured grant”.

===========================================================================

6.4.1.1.1.2	Sequence generation when transform precoding is enabled

If transform precoding for PUSCH is enabled, the reference-signal sequence  shall be generated according to

where ru,vα,δ(n) with δ=1 and α=0 depends on the configuration:

-	if the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH, and the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space, ru,vα,δ(n) is given by clause 5.2.3 with cinit given by

cinit=217Nsymbslotns,fμ+l+12NIDnSCID+1+2NIDnSCID+nSCIDmod 231

where nSCID=0 unless given by the DCI according to clause 7.3.1.1.2 in [4, TS38.212] for a transmission scheduled by DCI format 0_1 or given by the higher-layer parameter antennaPort for a PUSCH transmission scheduled by a type-1 configured grant; and

-	NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters pi2BPSKscramblingID0 and pi2BPSKscramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_1 or by a PUSCH transmission with a type-1 configured grant;

-	NID0∈0,1,…,65535 is given by the higher-layer parameter pi2BPSKscramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI;

-	NIDnSCID=NIDcell otherwise;

-	otherwise, ru,vα,δ(n) is given by clause 5.2.2.

The sequence group u=fgh+nIDRS mod 30, where nIDRS is given by

-	nIDRS=nIDPUSCH if nIDPUSCH is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE and,

- the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is not configured or the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured and π/2-BPSK modulation is not used for PUSCH, and

- the PUSCH is neither scheduled by RAR UL grant nor scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI according to clause 8.3 in [5, TS 38.213];

-	nIDRS=NIDnSCID if the higher-layer parameter dmrs-UplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH, the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space;

-	nIDRS=NIDcell otherwise

<unchanged text omitted>

===========================================================================

There is one agreement for the multiplexing capability for PUCCH format 3/4 when new sequence is used.

Agreement The PUCCH multiplexing capacity when Rel-16 DMRS is configured for pi/2 BPSK PUCCH is a single port

This implies that only single PUCCH port is available when new DM-RS is applied.

In order to avoid misunderstanding, it is proposed that

- Orthogonal sequence index” shall be 0 if dmrsUplinkTransformPrecodingPUCCH-r16 is configured;

##### 6.4.1.3.3	Demodulation reference signal for PUCCH formats 3 and 4

###### 6.4.1.3.3.1	Sequence generation

The reference-signal sequence rlm shall be generated according to

where MscPUCCH,s is given by clause 6.3.2.6.3 and ru,vα,δ(m) depends on the configuration:

-	if the higher-layer parameter dmrsUplinkTransformPrecodingPUCCH-r16 is configured, and π/2-BPSK modulation is used for PUCCH, ru,vα,δ(m) is given by clause 5.2.3 with cinit given by clause 6.4.1.3.2.1. The sequence group u and the sequence number v depend on the sequence hopping in clause 6.3.2.2.1 and the cyclic shift α=0.  The orthogonal sequence index n=0.

-	otherwise, ru,vα,δ(m) is given by clause 6.3.2.2. The cyclic shift α varies with the symbol number and slot number according to clause 6.3.2.2.2 with  for PUCCH format 3 without interlaced mapping and obtained from Table 6.4.1.3.3.1-1 with the orthogonal sequence index n given by clause 6.3.2.6.3 for PUCCH format 3 with interlaced mapping and PUCCH format 4.

Table 6.4.1.3.3.1-1: Cyclic shift index m0 for PUCCH format 3 with interlaced mapping and PUCCH format 4.

| Orthogonal sequence index n | Cyclic shift index |  |  |
| --- | --- | --- | --- |
|  | NSFPUCCH,s=1 | NSFPUCCH,s=2 | NSFPUCCH,s=4 |
| 0 | 0 | 0 | 0 |
| 1 | - | 6 | 6 |
| 2 | - | - | 3 |
| 3 | - | - | 9 |

<unchanged text omitted>

====================================================================================

# Conclusion

In this contribution, we have provided the following CR for the latest updated specification on “Low PAPR RS”

The proposed updates are listed as below.

- Clarification of the condition for applying different identity for the nIDRS.

- Clarification of the number of ports for PUCCH format 3/4 when new DMRS is applied.

# References

- TS38.211 v16.1.0

- Chairman’s Report for RAN1 #100e-bis