3GPP TSG RAN WG1 #101-e	R1-2004543

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.4.2.1

Source:	Ericsson

Title:	Resource Allocation Mode 1 for NR SL

Document for:	Discussion, Decision

# 1	Introduction

In this paper, open issues related to the resource allocation mode 1 AI in NR SL Rel-16 are discussed.

# 2	Dynamic grant

For dynamic grant, one important point that has not been agreed yet is the number of SL HARQ-ACK reports per grant. We see two alternatives:

- SL HARQ-ACK is reported after the last SL transmission (and the corresponding PSFCH reception).

- SL HARQ-ACK is reported each SL transmission (and the corresponding PSFCH reception).

The first option is simpler and identical to the approach taken for CG.

The second option has more flexibility but not without some problems.

- In Mode 2, the time between two consecutive transmission (Z = a + b in the agreements) includes:

  - Time between PSSCH and PSFCH (a), which depends on MinTimeGapPSFCH and periodPSFCHresource.

  - Time between PSFCH and the next PSSCH (b), which is up to UE implementation.

  - It is in principle possible that b is shorter than the guard period.

- In Mode 1, the time between two consecutive transmissions would include:

  - Time between PSSCH and PSFCH, which depends on MinTimeGapPSFCH and periodPSFCHresource.

  - The time between PSFCH and SL-HARQ report in UL, which is determined by the gNB but not less than Tprep = (N+X) ∙ (2048+144) ∙ k ∙ 2 –μ ∙ T_c. For X=0 the values are given in Table 1.

Since SCI can signal resources spaced by at most 32 slots, the second option would result in significantly less flexibility to schedule the resources with a single DCI than the first option (or that of Mode 2, which is the same).

Table 1. PUCCH preparation time for different numerologies.

| μ | Tprep |
| --- | --- |
| 0 | 0.99 ms (~ 1 slot). |
| 1 | 0.64 ms (rounded up to 2 slots). |
| 2 | 0.5 ms (~2 slots). |
| 3 | 0.29 ms (rounded up to 3 slots) |

Given these problems and that multiple DCI grants, each of them for a single SL transmission, can be used to have SL HARQ-ACK reporting to the gNB for each transmission we propose specifying the first alternative only.

For a dynamic grant in Mode 1 when using SL HARQ feedback:

- Only one HARQ-ACK bit is reported to the gNB.

- There is only one PUCCH transmission occasion after the last resource in the set of resources provided by the dynamic grant.

# 3	Configured grant

## 3.1	Timing aspects

For SL carriers, we distinguish the following cases:

- UL and SL are in the same carrier.

- UL and SL are in different carriers but share the same timing.

- UL and SL are in different carriers with different timings. The time difference is known at the gNB.

- UL and SL are in different carriers with different timings. The time difference is not known at the gNB.

In our view, for cases 1-3 there is no ambiguity as to when the UE transmits (over SL or SL reports on Uu). The challenging case is the 4th one. This may correspond to the case that a SL carrier is on the ITS band (with a timing set by some regulator body) and scheduling comes from the Uu carrier of one or multiple operators). We believe that it is not possible to operate all system configurations for case 4. For example, many pool configurations cannot be used because the gNB would have no means to tell their timing. However, there are many relevant configurations (e.g., all slots are SL slots) that work well even when the timing difference is not know at the gNB.

In the asynchronous case:

- The timing difference between Uu and SL is not known at the gNB.

- Some system configurations are not feasible. However, many others are. Like in LTE, RAN1 should strive for supporting the latter case.

In RAN1#100bis-e, the following was agreed:

Agreements: - The slots for sidelink transmission for CG type-1 are determined using the UL formula in 38.321 with the following changes: - Using slot-level granularity instead of symbol-level granularity (i.e., remove numberOfSymbolsPerSlot, “symbol number in the slot”, S in the formula) - periodicity is in number of slots - timeDomainOffset is expressed in number of slots - FFS: frame indexing (e.g., SFN, or virtual frame number) - FFS: whether logical or physical slots are used

The following text from 38.321 describes the configuration of type-1 grants in UL:

Upon configuration of a configured grant Type 1 for a Serving Cell by upper layers, the MAC entity shall: - store the uplink grant provided by upper layers as a configured uplink grant for the indicated Serving Cell; - initialise or re-initialise the configured uplink grant to start in the symbol according to timeDomainOffset and S (derived from SLIV as specified in TS 38.214 [7]), and to reoccur with periodicity. After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider that the uplink grant recurs associated with each symbol for which: [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = (timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), for all N >= 0.

The UL formula is valid for synchronous Uu-SL carriers. For asynchronous Uu-SL carriers, however, it is necessary to define a common reference for all UEs. For DCI-based allocation, a correction term TTA2 has been introduced. We believe that a similar approach can be used here. That is, a new time reference must be defined based on DL timing (i.e., SFN=0) and the TA correction. This is illustrated in Figure 1.

Figure 1. Configuration of a type-1 grant with asynchronous Uu-SL carriers.

In our view, this could be implemented in the specification in the following way.

After a sidelink grant is configured for a sidelink configured grant Type 1, the MAC entity shall consider that the uplink grant recurs associated with each slot for which: [(DFNv × numberOfSlotsPerFrame) + slot number in the frame] = (timeDomainOffset + N × periodicity) modulo (1024 × numberOfSlotsPerFrame), for all N >= 0. where: - DFNv is defined by having value 0 starting with the first SL slot that starts no earlier than TSFN,0-TTA2, where: - TSFN,0 is the time corresponding to the start of SFN=0 in the DL carrier, as perceived by the UE. - TTA is the timing advance value at the time of configuring the grant. - timeDomainOffset is expressed in number of SL slots with respect to DFNv = 0. - periodicity is expressed in number of SL slots.

Note that DFNv is only used by the UE. It is not a replacement for DFN.

For configured grant type 1:

- SFN is replaced by DFNv, which is defined by having value 0 starting with the first SL slot that starts no earlier than TSFN,0 – TTA/2, where TSFN,0 is the time corresponding to the start of SFN=0 in the DL carrier at the UE and TTA is the timing advance value at the time of configuring the grant.

- timeDomainOffset is expressed in number of SL slots with respect to DFNv = 0.

- periodicity is expressed in number of SL slots.

## 3.2	HARQ process ID determination

The following agreement was made in RAN1#100-e:

Agreements: - The HARQ process ID for each transmission in a resource corresponding to a SL configured grant is determined based on the formula used for UL configured grants. - The mapping with the values of HPN in SCI is fixed for a TB, and is up to UE implementation. Note: This corresponds to the HARQ process ID for the interaction between gNB and UE, if any distinction is made.

The following agreement was made in RAN1#100bis-e:

Agreements: - In the determination of the UL slot used for SL HARQ-ACK reporting to the gNB: - k=0 corresponds to a last slot for a PUCCH transmission that would overlap with the last PSFCH reception occasion assuming that the starting time of the frame for sidelink transmission is given by the starting time of the frame for the downlink reception. - FFS: corrections to cover the asynchronous case

For UL CGs, the HPN is determined as described by the following text from TS 38.321:

For configured uplink grants, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation: HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes where CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8]. NOTE 1: CURRENT_symbol refers to the symbol index of the first transmission occasion of a repetition bundle that takes place. NOTE 2: A HARQ process is configured for a configured uplink grant if the configured uplink grant is activated and the associated HARQ process ID is less than nrofHARQ-Processes. NOTE 3: If the MAC entity receives both a grant in a Random Access Response and an overlapping grant for its C-RNTI or CS-RNTI, requiring concurrent transmissions on the SpCell, the MAC entity may choose to continue with either the grant for its RA-RNTI or the grant for its C-RNTI or CS-RNTI.

For synchronous SL/Uu carriers, CURRENT_symbol can be unambiguously determined by both UE and gNB. However, for asynchronous carriers, this is not the case. That is, the value obtained by the gNB (which would be based on SFN) and the value obtained by the UE (which would be based on DFN) will likely be different. The simplest way to fix this is to have CURRENT_symbol refer to the symbol in which PUCCH is transmitted.

In the determination of HPN for configured grant, CURRENT_symbol refers to the symbol index of the PUCCH used for reporting SL HARQ-ACK to the gNB.

# 4	DCI aspects

Section 7.3.1.4.1 in 38.212 (as endorsed in [1]) specifies DCI format 3_0 as follows:

DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell. The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI: - Time gap – [x] bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in clause 8.1.2.1 of [6, TS 38.214] - HARQ process ID – [x] bits as defined in clause 16.4 of [5, TS 38.213] - New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213] - Lowest index of the subchannel allocation to the initial transmission –log2(N subChannel SL) bits as defined in clause 8.1.2.2 of [6, TS 38.214] - SCI format 1-A fields according to clause 8.3.1.1: - Frequency resource assignment. - Time resource assignment. - PSFCH-to-HARQ feedback timing indicator – 3 bits as defined in clause 16.5 of [5, TS 38.213]. - PUCCH resource indicator – 3 bits as defined in clause 16.5 of [5, TS 38.213]. - Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise [x] bits as defined in clause x.x.x of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, this field is reserved for DCI format 3_0 with CRC scrambled by SL-RNTI. - Counter sidelink assignment index – [1 or 2] bits - 2 bits as defined in clause 15.x of [5, TS 38.213] if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic - [1 or 2] bits as defined in clause 15.x of [5, TS 38.213] if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static

## 4.1	Contents

As seen above, some of the fields are not completely specified.

For time gap, we propose that the RRC table can be configured with up to 8 values and that the bitwidth is chosen accordingly.

Up to 8 values may be configured by higher layer for timeGapFirstSidelinkTransmission, which indicate the number of SL (logical) slots. The field time gap in DCI format 3_0 has variable bitwidth that depends on the number of configured values.

For HARQ process ID, we propose to use 4 bits.

16 SL HARQ processes are defined. The field HARQ process ID in DCI format 3_0 consists of 4 bits.

RAN2#108 made the following working assumption

Up to 8 configured grants (including both configured grant type1 and type2) in mode1 can be configured and active for NR sidelink communication (working assumption). Running 38.321 will specify something like “[8]”

Based on this, a 3-bit field is necessary in DCI.

The field ‘Configuration index’ consists of 3 bits, when applicable.

For cSAI when the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static, we propose to use 2 bits and maintain the Rel-15 behavior for Uu.

The field ‘Counter sidelink assignment index’ uses 2 bits when the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

## 4.2	Size alignment

| Field |  | Format 0-0 | Format 0-1 |
| --- | --- | --- | --- |
| Identifier |  | 1 bit | 1 bit |
| Resource information | CFI | – | 0 or 3 bits |
|  | UL/SUL | 0 or 1 bit | 0 or 1 bit |
|  | BWP indicator | – | 0-2 bits |
|  | Freq.-domain allocation | Variable (only type 1) | variable |
|  | Time-domain allocation | 0-4 bits | 0-4 bits |
|  | Frequency hopping | 0 or 1 bit | 0 or 1 bit |
| TB-related | MCS | 5 bits | 5 bits |
|  | NDI | 1 bit | 1 bit |
|  | RV | 2 bis | 2 bis |
| HARQ-related | Process number | 4 bits | 4 bits |
|  | DAI | – | 1-4 bits |
|  | CBGTI | – | 0, 2, 4, or 6 bits |
| Multi-antenna related | DMRS seq initialization | – | 1 bit |
|  | Antenna ports | – | 2-5 bits |
|  | SRI | – | Variable |
|  | Precoding information | – | 0-6 bits |
|  | PTRS-DMRS assoc. | – | 0 or 2 bits |
|  | SRS request | – | 2 bits |
|  | CSI request | – | 0-6 bits |
| Power control | PUSCH power control | 2 bits | 2 bits |
|  | Beta offset | – | 0 or 2 bits |

Even when only 2 sub-channels are configured, the size exceeds 30 bits, meaning that alignment with DCI 0_0 is in general not possible.

When both DCI 0_1 and DCI 3_0 are configured, DCI 3_0 is zero-padded to match the size of DCI 0_1.

Similarly, if in a search space a UE is configured with format 3_0 with CRC scrambled with SL-RNTI and with format 3_0 with CRC scrambled with SL-CS-RNTI, then the former is zero-padded until it has the same size as the latter.

When both DCI 3_0 with CRC scrambled with SL-RNTI and DCI 3_0 with CRC scrambled with SL-CS-RNTI are configured, DCI 3_0 with CRC scrambled with SL-RNTI is zero-padded to match the size of DCI 3_0 with CRC scrambled with SL-CS-RNTI.

We propose to use the following text from 36.212 (with the corresponding changes to format numbers and RNTI names):

If the number of information bits in format 3_0 mapped onto a given search space is less than the payload size of format 0_1 mapped onto the same search space, zeros shall be appended to format 3_0 until the payload size equals that of format 0_1 including any padding bits appended to format 0_1. If the format 3_0 CRC is scrambled by SL-RNTI and if the number of information bits in format 3_0 mapped onto a given search space is less than the payload size of format 3_0 with CRC scrambled by SL-CS-RNTI mapped onto the same search space and format 0_1 is not defined on the same search space, zeros shall be appended to format 3_0 until the payload size equals that of format 3_0 with CRC scrambled by SL-CS-RNTI.

# 5	HARQ procedure over Uu

In RAN1#100bis-e, the following was agreed:

Agreements: - A UE does not expect to be scheduled to transmit the UL report corresponding to a PSFCH reception earlier than Tprep after the end of the PSFCH. - This includes the effect of time advance. - Tprep = (N+X) ∙ (2048+144) ∙ k ∙ 2 –μ ∙ T_c where: - Working assumption: N is 14, 18, 28 and 32 corresponds to the SCS configuration μ of 0, 1, 2 and 3, μ = min(μ_SL, μ_UL) - k = T_s / T_c (parameters as defined in 38.211) - FFS X (including the possibility of value 0)

## 5.1	UE behaviour for generating the HARQ reports

In previous meetings, the following agreements and working assumptions were made:

Agreements: - For dynamic grant, the number of retransmissions of a TB is up to the gNB. - For configured grant, the maximum number of times that a TB can be retransmitted using the resources provided by the configured grant is configured per priority per configured grant. Agreements: - Only one new TB can be transmitted in one period of the configured grant. - FFS any issue with retransmission spanning multiple periods - The DCI scheduling the retransmissions uses the HARQ process ID corresponding to the first transmission of the TB, as agreed for Q2. Working assumption: - In case of reaching the maximum number of HARQ re-transmissions for a TB, the UE sends one bit on the UL resources for SL HARQ-ACK reporting. The specification will specify the UE behavior (what the behavior is: FFS), and specify the contents of the report (what the content is: FFS). Agreements: - The working assumption (as in proposal 3 in the summary) from RAN1#100-e is confirmed.

There have been long discussions on:

- whether the first agreement (for configured grant) refers to the resources provided by the configured grant alone or whether it also includes retransmissions scheduled by a dynamic grant.

- whether the UE can be aware of the maximum number of transmission of a TB or not.

For the first issue, we think that the agreement could hardly be any clearer as it states that “the maximum number of times that a TB can be retransmitted using the resources provided by the configured grant is configured per priority per configured grant“.

We propose avoiding the discussion on the second issue altogether and agreeing to support the default behavior in this casde too. That is, the UE reports ACK/NACK depending on whether it detects PSFCH or not and, if so, its contents. Given that the UE would perform the same as in other cases, there is no need to detect whether the maximum number of transmissions has been reached or not and to discuss it either. The gNB can decide whether to provide additional resources or not based on the SL HARQ-ACK report.

In case of reaching the maximum number of HARQ re-transmissions for a TB using resources provided by a configured grant, the UE reports ACK/NACK based on the corresponding PSFCH reception(s) or absence(s) thereof.

## 5.2	SL HARQ multiplexing

At this point, only the support for SL HARQ-ACK reporting in PUSCH is missing. We discuss the details for each of the codebooks in the next sections. One issue that is common to both codebooks is how to provide the configuration of the beta_offset indicator. In our view, it is reasonable to reuse the value(s) configured for DL HARQ-ACK reporting to the gNB, that the same codebook type is used for SL HARQ-ACK and DL HARQ-ACK reporting (as agreed in the e-mail discussion after RAN1#98bis). Introducing a dedicated configuration does not seem necessary and is not desirable at this point either.

For SL HARQ-ACK reporting to the gNB, beta_offset is indicated using the same signalling and configuration as for DL HARQ-ACK reporting.

### 5.2.1	Type-1 codebook

In our view, SL HARQ-ACK reporting in PUSCH using type-1 codebook can be easily introduced if the following is considered:

- The procedures agreed in RAN1#100bis-e for SL HARQ-ACK reporting in PUCCH using type-1 codebook are the baseline for SL HARQ-ACK reporting in PUSCH using type-1 codebook in the same way as the procedures for DL HARQ-ACK reporting in PUCCH using type-1 codebook are the baseline for DL HARQ-ACK reporting in PUSCH using type-1 codebook.

- There is no need to make a distinction based on the DCI format scheduling the PUSCH transmission. This is because the sidelink assignment index (SAI) only includes the counter SAI.

- Like for supporting SL HARQ-ACK reporting in PUCCH using type-1 codebook a few minor changes to the procedural text are necessary.

In preparing the TP for SL HARQ-ACK reporting in PUSCH using type-1 codebook, at least the following changes are made with respect to the Rel-15 specification:

- The procedures for SL HARQ-ACK reporting in PUCCH using type-1 codebook are the baseline.

- There is no difference in the procedure if PUSCH is scheduled by DCI format 0_0 or by format 0_1.

- ”Candidate PSSCH transmissions with corresponding PSFCH reception occasions” replaces ”candidate PDSCH receptions”

- “PSFCH-to-HARQ_feedback timing indicator” replaces “PDSCH-to-HARQ_feedback timing indicator”

  - sl-PSFCH-ToPUCCH is used to determine PSFCH-to-PUCCH gap for SL CG type-1

- SL BWP replaces DL BWP

- NOTE: This is not intended to change the reporting of DL HARQ-ACK in any way

In preparing the TP for SL HARQ-ACK reporting in PUSCH using type-1 codebook:

- The following parameters and the corresponding parts of the Rel-15 specification are not used:

  - Parameters related to transmission of more than 1 TB

    - harq-ACK-SpatialBundlingPUSCH

- The following functionality from the Rel-15 specification is not supported

  - HARQ-ACK for SPS PDSCH release

- NOTE: This is not intended to change the reporting of DL HARQ-ACK in any way.

In RAN1#100bis-e, the following was agreed:

Agreements: - Counter sidelink index assignment (SAI) is supported. - 2 bits are used for type-2 codebook. - FFS size for type-1 codebook (1 or 2 bits). - The field is always present in DCI format 3_0.

In our view, using two bits for type-1 codebook is the simplest approach.

Counter sidelink index assignment (SAI) uses 2 bits for type-1 codebook.

### 5.2.2	Type-2 codebook

In our view, SL HARQ-ACK reporting in PUSCH using type-2 codebook can be easily introduced if the following is considered:

- The procedures agreed in RAN1#100bis-e for SL HARQ-ACK reporting in PUCCH using type-2 codebook are the baseline for SL HARQ-ACK reporting in PUSCH using type-2 codebook in the same way as the procedures for DL HARQ-ACK reporting in PUCCH using type-2 codebook are the baseline for DL HARQ-ACK reporting in PUSCH using type-2 codebook.

- There is no need to make a distinction based on the DCI format scheduling the PUSCH transmission. This is because the sidelink assignment index (SAI) only includes the counter SAI.

- Like for supporting SL HARQ-ACK reporting in PUCCH using type-2 codebook a few minor changes to the procedural text are necessary.

In preparing the TP for SL HARQ-ACK reporting in PUSCH using type-2 codebook, at least the following changes are made with respect to the Rel-15 specification:

- The procedures for SL HARQ-ACK reporting in PUCCH using type-2 codebook are the baseline.

- There is no difference in the procedure if PUSCH is scheduled by DCI format 0_0 or by format 0_1.

- NOTE: This is not intended to change the reporting of DL HARQ-ACK in any way

In preparing the TP for SL HARQ-ACK reporting in PUSCH using type-2 codebook:

- The following parameters and the corresponding parts of the Rel-15 specification are not used:

  - Parameters related to transmission of more than 1 TB

    - harq-ACK-SpatialBundlingPUSCH

  - Parameters related to CBG transmission

    - PDSCH-CodeBlockGroupTransmission

- The following functionality from the Rel-15 specification is not supported

  - HARQ-ACK for SPS PDSCH release

- NOTE: This is not intended to change the reporting of DL HARQ-ACK in any way.

We have included a TP for TS 38.213 in the appendix.

Agree the TP for TS 38.213 in the appendix.

## 5.3	Collision between SL HARQ-ACK and Uu UCI

In RAN1#99, the following conclusion was made:

Conclusion: - No support of multiplexing of SL HARQ and Uu UCI on PUCCH or PUSCH in Rel-16 - Note: this reverts the agreements made during RAN1#98b email discussion

Thus, the specification needs to clarify what happens if the gNB schedules simultaneous transmission of SL HARQ-ACK reporting and Uu UCI. In principle, one could define prioritization rules that determine which of the two transmission takes place. Unfortunately, such a solution would require functionality that is currently missing. For example, assigning priorities to different types of UCI, identifying the priorities of SL HARQ-ACK reports, etc. We prefer to avoid such discussions altogether and simply assume that the gNB will ensure that the situation does not take place. We note that the gNB indeed has tools to guarantee this, e.g., by selecting the PSDCH-to-PUCCH or PSFCH-to-PUCCH gaps appropriately.

A UE does not expect to be provided UL resources for transmission of SL HARQ-ACK information and of Uu UCI such that they overlap in time.

# 6	Processing times

In RAN1#100bis-e, the following was agreed:

Agreements: - A UE does not expect to be scheduled to transmit the UL report corresponding to a PSFCH reception earlier than Tprep after the end of the PSFCH. - This includes the effect of time advance. - Tprep = (N+X) ∙ (2048+144) ∙ k ∙ 2 –μ ∙ T_c where: - Working assumption: N is 14, 18, 28 and 32 corresponds to the SCS configuration μ of 0, 1, 2 and 3, μ = min(μ_SL, μ_UL) - k = T_s / T_c (parameters as defined in 38.211) - FFS X (including the possibility of value 0)

In our view, X can only be a deterministic value. Otherwise, there would be some ambiguity regarding the time of the UL transmission.

In Tprep, the value of X is always equal to 0.

As for preparation time (i.e., scheduling PDCCH to first PSSCH transmission), the values used in UL are applicable here too. Table 2, obtained from TS 38.214, summarizes the preparation times for PUSCH (i.e., with simplest capabilities).

Table 2. PUSCH preparation times

| μ | Preparation time (Tmin) |
| --- | --- |
| 0 | 10 symbols (0.71 ms) |
| 1 | 12 symbols (0.43 ms) |
| 2 | 23 symbols (0.41 ms) |
| 3 | 36 symbols (0.32 ms) |

A UE does not expect to be scheduled to perform a SL transmission earlier than Tmin after the end of the PDCCH carrying the DCI, where Tmin is given the UL preparation when the UL numerology is equal to μSL.

# 7	Conclusion

Observation 1	In the asynchronous case:

	The timing difference between Uu and SL is not known at the gNB.

	Some system configurations are not feasible. However, many others are. Like in LTE, RAN1 should strive for supporting the latter case.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For a dynamic grant in Mode 1 when using SL HARQ feedback:

	Only one HARQ-ACK bit is reported to the gNB.

	There is only one PUCCH transmission occasion after the last resource in the set of resources provided by the dynamic grant.

Proposal 2	For configured grant type 1:

	SFN is replaced by DFNv, which is defined by having value 0 starting with the first SL slot that starts no earlier than TSFN,0 – TTA/2, where TSFN,0 is the time corresponding to the start of SFN=0 in the DL carrier at the UE and TTA is the timing advance value at the time of configuring the grant.

	timeDomainOffset is expressed in number of SL slots with respect to DFNv = 0.

	periodicity is expressed in number of SL slots.

Proposal 3	In the determination of HPN for configured grant, CURRENT_symbol refers to the symbol index of the PUCCH used for reporting SL HARQ-ACK to the gNB.

Proposal 4	Up to 8 values may be configured by higher layer for timeGapFirstSidelinkTransmission, which indicate the number of SL (logical) slots. The field time gap in DCI format 3_0 has variable bitwidth that depends on the number of configured values.

Proposal 5	16 SL HARQ processes are defined. The field HARQ process ID in DCI format 3_0 consists of 4 bits.

Proposal 6	The field ‘Configuration index’ consists of 3 bits, when applicable.

Proposal 7	The field ‘Counter sidelink assignment index’ uses 2 bits when the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

Proposal 8	When both DCI 0_1 and DCI 3_0 are configured, DCI 3_0 is zero-padded to match the size of DCI 0_1.

Proposal 9	When both DCI 3_0 with CRC scrambled with SL-RNTI and DCI 3_0 with CRC scrambled with SL-CS-RNTI are configured, DCI 3_0 with CRC scrambled with SL-RNTI is zero-padded to match the size of DCI 3_0 with CRC scrambled with SL-CS-RNTI.

Proposal 10	In case of reaching the maximum number of HARQ re-transmissions for a TB using resources provided by a configured grant, the UE reports ACK/NACK based on the corresponding PSFCH reception(s) or absence(s) thereof.

Proposal 11	For SL HARQ-ACK reporting to the gNB, beta_offset is indicated using the same signalling and configuration as for DL HARQ-ACK reporting.

Proposal 12	In preparing the TP for SL HARQ-ACK reporting in PUSCH using type-1 codebook, at least the following changes are made with respect to the Rel-15 specification:

	The procedures for SL HARQ-ACK reporting in PUCCH using type-1 codebook are the baseline.

	There is no difference in the procedure if PUSCH is scheduled by DCI format 0_0 or by format 0_1.

	”Candidate PSSCH transmissions with corresponding PSFCH reception occasions” replaces ”candidate PDSCH receptions”

	“PSFCH-to-HARQ_feedback timing indicator” replaces “PDSCH-to-HARQ_feedback timing indicator”

o	sl-PSFCH-ToPUCCH is used to determine PSFCH-to-PUCCH gap for SL CG type-1

	SL BWP replaces DL BWP

NOTE: This is not intended to change the reporting of DL HARQ-ACK in any way

Proposal 13	In preparing the TP for SL HARQ-ACK reporting in PUSCH using type-1 codebook:

	The following parameters and the corresponding parts of the Rel-15 specification are not used:

o	Parameters related to transmission of more than 1 TB

	harq-ACK-SpatialBundlingPUSCH

	The following functionality from the Rel-15 specification is not supported

o	HARQ-ACK for SPS PDSCH release

NOTE: This is not intended to change the reporting of DL HARQ-ACK in any way.

Proposal 14	Counter sidelink index assignment (SAI) uses 2 bits for type-1 codebook.

Proposal 15	In preparing the TP for SL HARQ-ACK reporting in PUSCH using type-2 codebook, at least the following changes are made with respect to the Rel-15 specification:

	The procedures for SL HARQ-ACK reporting in PUCCH using type-2 codebook are the baseline.

	There is no difference in the procedure if PUSCH is scheduled by DCI format 0_0 or by format 0_1.

NOTE: This is not intended to change the reporting of DL HARQ-ACK in any way

Proposal 16	In preparing the TP for SL HARQ-ACK reporting in PUSCH using type-2 codebook:

	The following parameters and the corresponding parts of the Rel-15 specification are not used:

o	Parameters related to transmission of more than 1 TB

	harq-ACK-SpatialBundlingPUSCH

o	Parameters related to CBG transmission

	PDSCH-CodeBlockGroupTransmission

	The following functionality from the Rel-15 specification is not supported

o	HARQ-ACK for SPS PDSCH release

NOTE: This is not intended to change the reporting of DL HARQ-ACK in any way.

Proposal 17	Agree the TP for TS 38.213 in the appendix.

Proposal 18	A UE does not expect to be provided UL resources for transmission of SL HARQ-ACK information and of Uu UCI such that they overlap in time.

Proposal 19	In Tprep, the value of X is always equal to 0.

Proposal 20	A UE does not expect to be scheduled to perform a SL transmission earlier than Tmin after the end of the PDCCH carrying the DCI, where Tmin is given the UL preparation when the UL numerology is equal to μSL.

# References

- R1-2003169, “Corrections on 5G V2X sidelink features after RAN1#100bis-e” (endorsed), Huawei, RAN1#100bis-e, April 2020.

- R1-2003178, “Corrections on Sidelink” (endorsed), Samsung, RAN1#100bis-e, April 2020.

# Appendix – TP for TS 38.213 (HARQ-ACK codebook generation)

Notes:

- Section 16.5.1 incorporates changes with respect to the endorsed CR in [2].

- Sections 16.5.1.2 and 16.5.2.2 incorporate track changes with respect to sections 9.1.2.2 and 9.1.3.2 in TS 38.213 v15.9.0, respectively.

## 16.5	UE procedure for reporting HARQ-ACK on uplink

<Unchanged parts are omitted>

### 16.5.1	Type-1 HARQ-ACK codebook determination

<Unchanged parts are omitted>

If a UE reports HARQ-ACK information in a PUCCH only for

-	PSFCH reception occasions associated with PSSCH transmissions scheduled by a DCI format 3_0 with counter SAI field value of 1, or

-	PSFCH reception occasions associated with PSSCH transmissions corresponding to a SL configured grant

within the MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions as determined in Clause 16.5.1.1, the UE determines a HARQ-ACK codebook only for the PSFCH reception occasion associated with PSSCH transmission scheduled by DCI format 3_0 or only for the PSFCH reception occasion associated with PSSCH transmission corresponding to a SL configured grant according to corresponding MA occasions, where a value of a counter SAI in DCI format 3_0 is according to Table 16.5.2.1-1. Otherwise, the procedures in Clause 16.5.1.1 and in Clause 16.5.1.2 for a HARQ-ACK codebook determination apply.

#### 16.5.1.1	Type-1 HARQ-ACK codebook in physical uplink control channel

<Unchanged parts are omitted>

#### 16.5.1.2	Type-1 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by DCI format 0_0 or by DCI format 0_1, then

-	if the UE has not been provided a grant scheduling PSSCH transmissions with corresponding PSFCH reception occasions that the UE transmits corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PSFCH-to-HARQ_feedback timing indicator field in a DCI format scheduling the PSSCH transmissions, or on a value of PSFCH-to-HARQ feedback timing indicator field in a DCI format 3_0 activating a SL configured grant Type-2 transmission, or on the value of sl-PSFCH-ToPUCCH for a SL configured grant Type-1, in any of the MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions, as described in Subclause 9.1.2.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else the UE generates the HARQ-ACK codebook as described in Subclause 16.5.1.1, unless the UE receives only PSFCH receptions associated with PSSCH transmissions corresponding to a SL configured grant, or only PSFCH receptions associated with PSSCH transmissions scheduled by a DCI format 3_0 with counter SAI field value of 1 in the MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions in which case the UE generates HARQ-ACK information only for the PSFCH receptions as described in Subclause 9.1.2.

A UE sets to NACK value in the HARQ-ACK codebook any HARQ-ACK information corresponding to PSFCH reception occasions associated with PSSCH transmissions scheduled by DCI format 3_0 that the UE detects in a PDCCH monitoring occasion that starts after a PDCCH monitoring occasion where the UE detects a DCI format 0_0 or a DCI format 0_1 scheduling the PUSCH transmission.

### 16.5.2	Type-2 HARQ-ACK codebook determination

<Unchanged parts are omitted>

#### 16.5.2.1	Type-2 HARQ-ACK codebook in physical uplink control channel

<Unchanged parts are omitted>

#### 16.5.2.2	Type-2 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by DCI format 0_0 or by DCI format 0_1, then

-	if the UE has not received any PDCCH within the monitoring occasions for DCI format 3_0 for scheduling PSSCH transmissions with associated PSFCH reception occasions on any serving cell  and the UE does have a configured grant Type-1 with a value of sl-PSFCH-ToPUCCH for a SL configured grant Type-1 such that the UE is expected to report HARQ-ACK information based on PSFCH reception occasions associated with PSSCH transmissions corresponding to a SL configured grant  to multiplex in the PUSCH, as described in Subclause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else, the UE generates the HARQ-ACK codebook as described in Subclause 9.1.3.1.