3GPP TSG RAN WG1 Meeting #101-e	R1-2004630

E-meeting, May 25th – June 5th, 2020

Agenda Item:	7.1.3

Source:	Huawei, HiSilicon

Title:	Clarification on description of supported components of PDSCH ratematching capabilities in TS 38.306

Document for:	Discussion and Decision

Introduction & Discussion

In TR 38.822 [1], three UE capabilities for PDSCH rate-matching are agreed as

| 5-26 | Semi-static rate-matching resource set configuration for DL | 1) Bitmap 1/2/3 2) controlResourceSet | rateMatchingResrcSetSemi-Static |
| --- | --- | --- | --- |
| 5-27 | Dynamic rate-matching resource set configuration for DL | Bitmap 1/2/3 | rateMatchingResrcSetDynamic |
| 5-27a | Dynamic rate-matching control resource set for DL | Dynamic rate-matching control resource set for DL | rateMatchingCtrlResrcSetDynamic |

Combined with the RRC parameters of PDSCH rate-matching in TS 38.331 [2], as cited in Appendix, we have observations:

- Support of rateMatchingResrcSetSemi-Static means supporting semi-static rate matching for both bitmap 1/2/3 and ControlResourceSet, i.e. support of configuration of RateMatchPattern::patternType::bitmaps and RateMatchPattern::patternType::controlResourceSet in TS 38.331.

- Support of rateMatchingResrcSetDynamic means ONLY supporting dynamic rate matching for Bitmap 1/2/3.

- Support of rateMatchingCtrlResrcSetDynamic means ONLY supporting dynamic rate matching for ControlResourceSet.

However, they are not clearly reflected in current TS 38.306, i.e. the description of the rateMatchingResrcSetDynamic is very similar to that of rateMatchingResrcSetSemi-Static, resulting in ambiguity that the term “RB-symbol level granularity” is referring to both pattern-type of bitmap and ControlResourceSet. In short, the current description of rateMatchingResrcSetDynamic in TS 38.306 does not clearly preclude support of ControlResourceSet, which is not aligned with TR 38.822.

Therefore, we proposal a TP of TS 38.306 as,

Proposal 1: adopt a TP of Rel-15 TS 38.306 as follows,

<Unchanged parts are omitted – 38.306> rateMatchingCtrlResrcSetDynamic Indicates whether the UE supports dynamic rate matching for DL control resource set.; UE; Yes; No; No. rateMatchingResrcSetDynamic Indicates whether the UE supports receiving PDSCH with resource mapping that excludes the REs corresponding to resource sets configured with bitmap pattern-type based on dynamic indication in the scheduling DCI as specified in TS 38.214 [12].; UE; No; No; No. rateMatchingResrcSetSemi-Static Indicates whether the UE supports receiving PDSCH with resource mapping that excludes the REs corresponding to resource sets configured with RB-symbol level granularity following the semi-static configuration as specified in TS 38.214 [12].; UE; Yes; No; No. <Unchanged parts are omitted>

Additionally, we propose a LS to inform RAN2 for an update of TS 38.306,

Proposal 2: Send an LS to RAN2 with the message that

- Support of rateMatchingResrcSetDynamic means only supporting dynamic rate matching for Bitmap pattern-type as described in TS 38.822;

- Request an update to Rel-15 TS 38.306 as the TP above.

Conclusions

In conclusion, we proposal a TP of TS 38.306 as,

Proposal 1: adopt a TP of Rel-15 TS 38.306 as follows,

<Unchanged parts are omitted – 38.306> rateMatchingCtrlResrcSetDynamic Indicates whether the UE supports dynamic rate matching for DL control resource set.; UE; Yes; No; No. rateMatchingResrcSetDynamic Indicates whether the UE supports receiving PDSCH with resource mapping that excludes the REs corresponding to resource sets configured with bitmap pattern-type based on dynamic indication in the scheduling DCI as specified in TS 38.214 [12].; UE; No; No; No. rateMatchingResrcSetSemi-Static Indicates whether the UE supports receiving PDSCH with resource mapping that excludes the REs corresponding to resource sets configured with RB-symbol level granularity following the semi-static configuration as specified in TS 38.214 [12].; UE; Yes; No; No. <Unchanged parts are omitted>

Additionally, we need a LS to inform RAN2 for an update of TS 38.306,

Proposal 2: Send an LS to RAN2 with the message that

- Support of rateMatchingResrcSetDynamic means only supporting dynamic rate matching for Bitmap pattern-type as described in TS 38.822;

- Request an update to Rel-15 TS 38.306 as the TP above.

- References

3GPP TR 38.822: "NR; User Equipment (UE) feature list"

3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities"

- Appendix

Excerpt from TS 38.331

-- ASN1START

-- TAG-RATEMATCHPATTERN-START

RateMatchPattern ::=                SEQUENCE {

rateMatchPatternId                  RateMatchPatternId,

patternType                         CHOICE {

bitmaps                             SEQUENCE {

resourceBlocks                      BIT STRING (SIZE (275)),

symbolsInResourceBlock              CHOICE {

oneSlot                             BIT STRING (SIZE (14)),

twoSlots                            BIT STRING (SIZE (28))

},

periodicityAndPattern               CHOICE {

n2                                  BIT STRING (SIZE (2)),

n4                                  BIT STRING (SIZE (4)),

n5                                  BIT STRING (SIZE (5)),

n8                                  BIT STRING (SIZE (8)),

n10                                 BIT STRING (SIZE (10)),

n20                                 BIT STRING (SIZE (20)),

n40                                 BIT STRING (SIZE (40))

}                                                                                           OPTIONAL,   -- Need S

...

},

controlResourceSet                  ControlResourceSetId

},

subcarrierSpacing                   SubcarrierSpacing                                               OPTIONAL,   -- Cond CellLevel

dummy                               ENUMERATED { dynamic, semiStatic },

...,

[[

controlResourceSet-r16              ControlResourceSetId-r16                                        OPTIONAL    -- Need R

]]

}

-- TAG-RATEMATCHPATTERN-STOP