3GPP TSG-RAN WG1 Meeting #101-e	R1-2004661

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	6.2.5.1

Source:	Ericsson

Title:	On the RAN1 UE feature list for Rel-16 LTE-MTC

Document for:	Discussion, Decision

# Issue #1: Prerequisites for UL early termination

In the RAN1 UE feature list for Rel-16 LTE-MTC in [1], it is FFS whether support for sub-frame level UL resource reservation (1-25/1-26) should be a prerequisite for the multi-TB UL early termination feature (1-17). The rationale for tying the two features to each other is that without means for creating a gap in the UL transmission, it would not be possible for eNB to indicate to the UE that it should terminate its UL transmission. However, that rationale only holds for HD-FDD. In FD-FDD and TDD, it may be possible to transmit the DL indication without a gap in the UL transmission. Therefore, it seems to make sense to remove 1-25/1-26 from the prerequisites for 1-17 and consider adding a note about the HD-FDD case.

For 1-17, remove 1-25 and 1-26 from Prerequisites.

For 1-17, insert an informative Note stating that “For HD-FDD, the necessary UL gaps can be created using feature groups 1-25 and 1-26”.

# Issue #2: Reciprocity-based candidates in CE mode B

In the RAN1 UE feature list for Rel-16 LTE-MTC in [1], it is FFS whether MPDCCH performance improvement with reciprocity-based candidates in TDD (1-34) can apply to CE mode B. Since there is no SRS support in CE mode B, it seems reasonable to limit the support of this feature to CE mode A.

For 1-34, add “for CE mode A” in the Feature group and Components columns.

For 1-34, change Prerequisites from “1-31 or 1-32” to “1-31”.

# Issue #3: CSI-RS-based feedback in CE mode A

In the RAN1 UE feature list for Rel-16 LTE-MTC in [1], for feature groups that are restricted to only one of the two CE modes, the applicable CE mode is indicated already in the Feature group column. CSI-RS-based feedback for non-BL UE (1-35) is only applicable to CE mode A, as can be seen in the Components column, but it is not indicated in the Feature group column.

For 1-35, add “for CE mode A” in the Feature group column.

# Issue #4: Different descriptions for LTE-MTC and NB-IoT

In the RAN1 UE feature list for Rel-16 LTE in [1], the descriptions differ more than necessary for some of the features that are common or similar for LTE-MTC and NB-IoT.

Discuss and decide on updates to align descriptions for LTE-MTC and NB-IoT.

# References

- R1-2003196, “Summary on email discussion [100b-e-LTE-UEFeatures-Remaining]”