3GPP TSG RAN WG1 Meeting #101-E															R1-2004703

e-Meeting, May 25 – June 05, 2020

Source: 	Moderator (Intel Corporation)

Title:	Summary of discussions on supporting NR from 52.6 GHz to 71 GHz

Agenda item:	8.1

Document for:	Discussion

# Introduction

In this contribution, we summarize high-level discussions regarding supporting NR from 52.6 GHz to 71 GHz and related evaluation assumptions and simulation parameters that will help complete the Study Item (SI). Section 2 contains summary of evaluation assumptions and simulation parameters based on submitted contributions from agenda 8.1.1, 8.1.2, and 8.1.3. Section 3 contain summary of high-level issues and considerations based on submitted contributions from the same agenda.

# Summary of Evaluation Assumptions & Simulation Parameters

Companies have provided simulations to obtain useful observations and technical findings. In this section, we provide a summary of evaluation assumptions made for simulations and simulation parameters explained in the submitted contributions. We also provide a summary of objectives of the evaluations performed using the evaluation assumptions and simulation parameters.

## 2.1. Link Level Simulation

Table 1, Table 2, and Table 3 summarize link level simulation assumptions suggested or mentioned in the submitted contribution in RAN1 #101-e meeting. The submitted link level simulations were utilized to obtain analysis for the following purposes:

- Phase noise impact for various numerology (i.e. subcarrier spacing, and CP type)

- Performance analysis for PDSCH/PUSCH

- Performance analysis for SSB

- Channel delay spread impact for various CP type/lengths

Table 1. Summary of link level simulation parameters set 1

| Company | fc [GHz] | BW [MHz] | SCS [kHz] | CP Type | # RB | Channel Model | Antenna Configuration (Mg,Ng,M,N,P) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Apple [19] | 45, 70 | 45 GHz: 400 70 GHz: 400, 800, 1200, 1600, 2000 | 60, 120, 240, 480, 960 | - | - | AWGN | 1x1 |
| Ericsson [11] | 60 | 400, 2000 | 400 MHz: 240, 480, 960, 2 GHz: 960, 1920, 3840 | - | - | InH emulated CDL-B (21ns DS), CDL-D (19.5ns DS), UMi emulated CDL-B (55ns DS), CDL-D (27ns DS) | For InH emulation (1,1,4,8,2) BS (1,1,2,2,2) UE For UMi emulation (1,1,8,16,2) BS (1,1,4,4,2) UE |
| Charter [9] |  | 400 | 960 | - | 34 | TDL-A (5ns DS) | 2x2 |
| Huawei [2] | 70 | 140 | 120, 240, 480, 960 | - | 96, 48, 24, 12 | CDL-D (50ns DS) | (1,1,8,16,2) BS (1,1,2,4,2) UE |
| Intel [62] | 60, 70 | 400 | 120, 240, 480, 960, 1920 | Normal | 256, 128, 64, 32, 16 | TDL-A (10ns DS), TDL-D (1ns DS) | 1x2 |
| Interdigital [20] | 60 | 400, 2000 | 400 MHz: 120, 240, 480, 960 2000 MHz: 960, 1920, 3840 | Normal (120, 240, 480 kHz), Extended (960, 1920, 3840 kHz) | 512, 128, 64, 32 | CDL-B (40, 100, 800ns DS) CDL-D (40, 100, 800ns) with K-factor 10dB | (2,2,4,8,2) BS (dv,dh)=(0.5,0.5), (dgv, dgh)=(2.0,4.0) (1,2,2,4,2) UE (dv,dh)=(0.5,0.5), (dgv, dgh)=(0.0,0.0) |
| Lenovo [3] | 52.6, 60, 71 | - | 120, 240, 480, 960, 1920 | Normal | 2 | CDL-A (10, 30ns DS) | (1,1,1,2,1) BS (1,1,1,2,1) UE |
| Nokia [10] | 60 | 2000 | 120, 240, 480, 960, 1920, 3840 | Normal | 512, 128, 64, 32 | CDL-E (10ns DS) | (1,1,8,16,2) BS (1,1,4,4,2) UE |
| NTT Docomo [25] | 70 | 400 | 120, 240, 480, 960 | Normal (120,240,480 kHz) Extended (960 kHz) | 264, 132, 66, 32 | TDL-A (10, 22, 52ns DS) TDL-D (10ns DS) | 1x1 |
| Samsung [65] | 60 | - | 960 | - | - | TDL/CDL | (1,1,8,16,2) BS (1,1,2,4,2) UE |
| Qualcomm [26] | 60 | 400 | 60, 120, 480, 960 | Normal | 512, 128, 64, 32 | CDL-B (10ns DS) | (2,2,4,8,2) BS (1,2,2,4,2) UE |
| Vivo [4][60] | 60 | 500, 2000 | 480, 960, 1920 | Normal | 500MHz: 80, 40, 20, 2GHz: 320, 160, 80 | TDL-A (5ns DS) | 2x2 |

Table 2. Summary of link level simulation parameters set 2

| Company | PA Model | PN Model Tx | PN Model Rx | EVM | I-Q Imbalance | Channel Estimation | Mobility |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Apple [19] | - | None | Model (Set-B) from [70] | - | - | - | - |
| Ericsson [11] | - | 3GPP 38.803 example 2 BS, PN model in [64] | 3GPP 38.803 example 2 UE, PN model in [64] | - | - | Real | 3 Km/hr |
| Charter [9] | - | 3GPP 38.803 example 1 | 3GPP 38.803 example 1 | - | - | Real | 167 Hz (doppler) |
| Huawei [2] | - | 3GPP 38.803 example 2 BS | 3GPP 38.803 example 2 UE | 3% Tx 5% Rx | - | - | - |
| Intel [62] | - | 3GPP 38.803 example 2 BS, IEEE 802.11ad | 3GPP 38.803 example 2 UE, IEEE 802.11ad | - | - | Real | 3 Km/hr |
| Interdigital [20] | Rapp Model | 3GPP 38.803 example 2 BS | 3GPP 38.803 example 2 UE | - | - | Ideal, Real | 3 Km/hr |
| Lenovo [3] | - | 3GPP 38.803 example 2 BS | 3GPP 38.803 example 2 UE | - | - | Real | 3 Km/hr |
| Nokia [10] | - | 3GPP 38.803 example 2 BS | 3GPP 38.803 example 2 UE | - | - | Real | 3 Km/hr |
| NTT Docomo [25] | IEEE 802.11ad GaAs Model | 3GPP 38.803 example 1 | 3GPP 38.803 example 1 | - | - | - | - |
| Samsung [65] | - | 3GPP 38.803 example ? | 3GPP 38.803 example ? | - | - | - | 3 Km/hr |
| Qualcomm [26] | Modified Rapp Model [26] | 3GPP 38.803 example 1 (scaled) | 3GPP 38.803 example 1 (scaled) | - | - | - | 3 Km/hr |
| Vivo [4][60] | - | 3GPP 38.803 example 2 BS | 3GPP 38.803 example 2 UE | - | -16.88 dBc, -25.98 dBc, -30.44 dBc | Real | 3 Km/hr |

Table 3. Summary of link level simulation parameters set 3

| Company | Rank | SLIV | DMRS Configuration | PTRS Configuration(1) | MCS/TBS | Frequency Offset |
| --- | --- | --- | --- | --- | --- | --- |
| Apple [19] | 1 | - | - | - | - | - |
| Ericsson [11] | 1 | - | - | K = 2, L =1 | QPSK 2/3, 16QAM 2/3, 64QAM 2/3 | - |
| Charter [9] | 1 | - | 1 DMRS Symbol | - | 64 QAM (MCS 19) | - |
| Huawei [2] |  | S=3, L=11 | 1 DMRS Symbol Position (2) | K = 2, L = 1 | QPSK (MCS 1), 16 QAM (MCS 9), 64 QAM (MCS 16) | - |
| Intel [62] | 1 | S=0, L=14 | 2 DMRS Symbol Position (2,11) | K = 4, L = 1 | 16QAM 3/4, 64QAM 3/4 | - |
| Interdigital [20] | 1 | - | - | K = 4, L = 1 | QPSK (MCS 7), 16QAM (MCS 16) 64QAM (MCS22) 256QAM (MCS 23, 27) | - |
| Lenovo [3] | - | - | 1 DMRS Symbol Position | K = 2, L = 1 | MCS 9, MCS 16, MCS 22 | - |
| Nokia [10] | 1, 2 | S=1, L=13 | 1 DMRS Symbol Position (2) | K = 2, L = 1 (CP-OFDM) 8x4, 12x4 (SC-FDM) | QPSK 2/3, 16QAM 2/3, 64QAM 2/3, 256QAM 2/3 | - |
| NTT Docomo [25] | - | - | - | - | MCS 16 | - |
| Samsung [65] | - | - | - | - | - | 5 ppm (initial cell search) 0.1 ppm (PRACH, data) |
| Qualcomm [26] | - | S=2, L=12 | 1 DMRS Symbol Position (3) | K = 4, L = 1 | QPSK (MCS 7), 16QAM (MCS 16), 64 QAM (MCS 22, 28) | - |
| Vivo [4][60] | 1 | S=4, L=8 | 1 DMRS Symbol Position (3) | K = 2, 4, L = 1 | QPSK 1/2, 16QAM 3/4, 64QAM 3/4 | 0, 200, 400, 600 kHz (initial access) |
| Note (1): PTRS per K number of PRBs, and PTRS every L number of OFDM symbols |  |  |  |  |  |  |

Based on submitted simulation parameters for link level simulation, moderator suggests using Table 4 as basis for further discussion.

Table 4. Suggested harmonized link level simulation parameters as baseline for discussion

| Parameter | Value | Notes |
| --- | --- | --- |
| Carrier Frequency [GHz] | 60 GHz Optional: 70 GHz |  |
| Subcarrier Spacing [kHz] | 240 kHz, 480 kHz, 960 kHz Optional: 60 kHz, 120 kHz, 1920 kHz, 3840 kHz |  |
| Bandwidth [MHz] | 2000 MHz Optional: 400 MHz, 500 MHz |  |
| Number of RB | For 2000 MHz: 320 (480 kHz), 160 (960 kHz), 80 (1920 kHz), 40 (3840 kHz) For 400 MHz: 256 (120 kHz), 128 (240 kHz), 64 (480 kHz), 32 (960 kHz), 16 (1920 kHz), 8 (3840 kHz) For 500 MHz: 330 (120 kHz), 165 (240 kHz), 82 (480 kHz), 41 (960 kHz), 20 (1920 kHz), 10 (3840 kHz) | Do not exceed 4k FFT size |
| CP Type | Normal CP, Extended CP |  |
| Channel Model | TDL-A (5ns, 10ns DS) Optional: TDL-D (1ns, 10ns DS) Optional: CDL-A (10ns, 30ns DS) CDL-B (10ns, 20ns, 50ns DS) CDL-D (20ns, 30ns, 40ns, 50ns DS) |  |
| Antenna Configuration (Mg,Ng,M,N,P) | For TDL model: 2x2 Optional: 1x2 For CDL model: (1,1,8,16,2) BS, (1,1,4,4,2) UE with (0.5 dv, 0.5 dH) Optional BS configuration: (1,1,4,8,2), (2,2,4,8,2), (1,1,4,8,2) Optional UE configuration (1,1,2,4,2), (1,2,2,4,2), (1,1,2,2,2) |  |
| PA Model | Optional: Companies to provide modeling | In lieu of pre-loaded Tx EVM |
| Tx PN Model | 3GPP TR38.803 example 2 BS Optional: 3GPP TR38.803 example 1 Companies to provide modeling |  |
| Rx PN Model | 3GPP TR38.803 example 2 UE Optional: 3GPP TR38.803 example 1 Companies to provide modeling |  |
| Pre-loaded Tx EVM | Optional: 3% at Tx | In lieu of PA model |
| Additive Rx EVM | Optional: 5% at Rx |  |
| I-Q Imbalance | Optional: -26dBc, -31dBc |  |
| Channel Estimation | Realistic channel estimation |  |
| Mobility | 3 Km/hr |  |
| Transmission Rank | Rank 1 Optional: Rank1+2 adaptive, Rank 2 |  |
| PDSCH SLIV | (S=2, L=12) Optional: (S=3, L=11), (S=0, L=14) | Starting symbol, S, (indexed from 0) and length, L. |
| DMRS Configuration | Front loaded, 1 DMRS symbol Optional: 2 DMRS symbol at (2,11) symbol index |  |
| PTRS Configuration | (K = 4, L = 1) Optional: (K = 2, L = 1) | PTRS per K number of PRBs, and PTRS every L number of OFDM symbols |
| MCS/TBS | MCS 16 (16QAM), MCS 22 (64QAM) Optional: MCS 1 (QPSK), MCS 7 (QPSK), MCS 23 (256QAM), MCS 27 (256QAM) |  |
| Frequency Offset | Optional: 0.1 ppm (for data channel) 10 ppm (for initial access) |  |

## 2.2. System Level Simulation

Table 5 and Table 6 summarize system level simulation assumptions suggested or mentioned in the submitted contribution in RAN1 #101-e meeting. The submitted system level simulations were utilized to obtain analysis for the following purposes:

- Channel delay spread impact for various CP type/lengths

- NR-NR multi-operator coexistence analysis

- Performance analysis for PDSCH/PUSCH

- Performance impact for using various CCA levels and LBT schemes (e.g. receiver-aided LBT, omni-directional LBT, directional LBT, etc)

Table 5. Summary of system level simulation parameters set 1

| Company | Deployment Scenario/Channel Model | Antenna Configuration (Mg,Ng,M,N,P) | fc [GHz] | BW [MHz] | SCS [kHz] |
| --- | --- | --- | --- | --- | --- |
| Ericsson [39] | InH Office (ISD=20m, 12 BS, 120m x 50m) | (1,1,8,8,2), (1,1,8,4,2), (1,1,2,1,2) BS (dv,dh)=(0.5,0.5) (1,1,4,2,2), (1,1,2,1,2) UE (dv,dh)=(0.5,0.5) | 60 | 2160 | 960 |
| Huawei [69] | InH Office Box (10x10m, 1 BS x 2 operators, min BS-BS d=1m), BS height 3m, UE height 1.5m Outdoor Backhaul (300x10m area, 1 desired link + U=1,2,4,8 interfering links) | For 70 GHz: (1,1,8,16,2) BS (dv,dh)=(0.5,0.5) (1,1,4,4,2) UE (dv,dh)=(0.5,0.5) For 60GHz: (1,1,4,4,2) BS (dv,dh)=(0.5,0.5) (1,1,2,2,2) UE (dv,dh)=(0.5,0.5) | 60, 70 | 500 | - |
| Intel [62] | InH Open Office (ISD=20m, 12 BS), UMi (ISD=150m, 1 layer), InF-DL, InF-SH | (1,1,8,8,2), (1,1,16,16,2), (1,1,32,32,2) BS (1,2,2,2,2), (1,2,4,4,2), (1,2,8,8,2) UE | 60, 70 | - | 480, 960, 1920, 3840 |
| Nokia [38] | Indoor office (3 BS/operator) 5 UE per BS 5GCM Indoor mixed office, LOS only 3 km/hr fading | (1,1,8,16,2) BS (1,1,1,2,1) UE omni-directional | 60 | 2160 | - |
| Samsung [65] | InH Mixed Office (ISD=40m, 3 BS x 2 operators, 80m x 120m) | (1,1,8,16,2) BS (dv,dh)=(0.5,0.5) (1,1,2,4,2) UE (dv,dh)=(0.5,0.5) | 60 | 2160 | 960 |
| Qualcomm [26][56] | InH (ISD=10m, 12, 120m x 50m) 10 UE per gNB InH Open Office (ISD=20m, 12 BS x 2 operators, 10x10m boxes), UMi (ISD=20,50,100m) 10 UE per gNB | (1,1,8,8,2), (1,1,16,8,2) BS for InH (1,1,32,8,2) BS for UMi (dv,dh)=(0.5,0.5) (1,1,2,2,2), (1,1,4,4,2) , (1,2,4,4,2) UE (dv,dh)=(0.5,0.5) | 28, 60 | 500, 2160 | - |
| vivo [32] | InH Mixed Office (ISD=20m, 12 BS x 2 operators, 10x10m boxes), 120 UE in area UMi-Street Canyon (ISD=200m, 2 layers, 1 micro BS per Hexagonal area) 10 UE per micro TRP | (1,1,4,4,2), (1,1,16,8,2) BS (dv,dh)=(0.5,0.5) (1,1,2,2,2), (1,1,2,4,2) UE (dv,dh)=(0.5,0.5) | 60 | 500 (InH), 2000 (Dense Urban) | 960 |
| ZTE [61] | InH Open Office (ISD=20m, 6 BS x 2 operators), UMa (ISD=100m) | (1,1,8,16,2) BS, (dv,dh)=(0.5,0.5) (1,1,1,2,2), (1,1,4,4,2) UE, (dv,dh)=(0.5,0.5) | 60 | 2160 | - |

Table 6. Summary of system level simulation parameters set 1

| Company | BS TxP [dBm] | UE TxP [dBm] | BS AE gain [dBi] | UE AE gain [dBi] | BS NF | UE NF | Traffic Model | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson [39] | 16 dBm (128 AE), 19 dBm (62 AE) 20 dBm (4 AE) | 20 dBm | 5 | 0 | 7 | 13 | FTP-1,3 (27Mbyte/file) |  |
| Huawei [69] | 40 dBm EIRP | 40 dBm EIRP | - | - | 7 | 10, 13 | FTP-3 (0.5Mbyte) |  |
| Intel [62] | - | - | 5 | 5 | - | - | - |  |
| Nokia [38] | 40 dBm EIRP 18 dBm TxP | - | - | - | - | - | FTP-3 (0.5Mbyte) |  |
| Samsung [65] | 18 dBm TxP | 21 dBm TxP | 5 | 5 | 7 | 10 | FTP-1 FTP-3 (0.5Mbyte) | Min received power from serving cell: -82dBm |
| Qualcomm [26][56] | 40 dBm EIRP | 25 dBm EIRP | 5 | 5 | 7 | 13 | Full Buffer |  |
| Vivo [32] | 40 dBm EIRP 23dBm (32 AE) 14 dBm (256 AE) | 25 dBm EIRP 14 dBm (8AE), 8 dBm (32AE) | 5 | 0 | 7 | 10, 13 | Full Buffer, FTP-3 (0.5Mbyte) |  |
| ZTE [61] | 40 dBm EIRP, 14dBm TxP | 21 dBm TxP | 5 | 0 | 7 | 13 | - | -72dBm Rx Sensitivity |

Based on submitted simulation parameters for system level simulation, moderator suggests using Table 7 as basis for further discussion.

Table 7. Suggested harmonized system level simulation parameters as baseline for discussion

| Parameter | Value | Notes |
| --- | --- | --- |
| Carrier Frequency [GHz] | 60 GHz Optional: 70 GHz |  |
| Subcarrier Spacing [kHz] | 960 kHz Optional: 60 kHz, 120 kHz, 240 kHz, 480 kHz, 1920 kHz, 3840 kHz |  |
| Bandwidth [MHz] | 2160 MHz Optional: 500 MHz |  |
| Number of RB | For 2160 MHz: 340 (480 kHz), 178 (960 kHz), 89 (1920 kHz), 44 (3840 kHz) For 500 MHz: 330 (120 kHz), 165 (240 kHz), 82 (480 kHz), 41 (960 kHz), 20 (1920 kHz), 10 (3840 kHz) | Do not exceed 4k FFT size |
| Deployment Scenario | Indoor Office: Scenario A) InH open office model: Office box 120m x 50 m, 12 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, ISD = 20m, BS randomly deployed within 10m x 10m virtual box Optional: Scenario B) small InH open office model: Office box 20m x 20 m, 1 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, BS randomly deployed within 10m x 10m virtual box Optional: Scenario C) InH open office model: Office box 120m x 50 m, 12 BS per operator, 1 operator, BS height at 3m (ceiling), UE height 1m, BS fixed position, ISD = 20m Optional: Scenario D) InH open office model: Office box 120m x 50 m, 6 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, BS fixed position, ISD = 40m Optional: Scenario E) InH open office model: Office box 120m x 80 m, 3 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, BS fixed position, a=20m, b=40m, c=20m, and d=40m Dense Urban: Scenario F) Dense Urban with 1 layer Hexagonal grid, single layer, 3 sectors per site, 19 sites locations, BS height 10m, UE height 1.5m, ISD = 150m Optional: Scenario G) Dense Urban with 2 layers Macro layer (sub 7GHz): Hexagonal grid, single layer, 3 sectors per site, 19 sites locations BS height 25m, UE height 1.5m, ISD = 200m, fixed BS position Micro layer (above 52.6 GHz): BS height 10m, UE height 1.5m, 2 operator, 1 BS per hexgrid per operator, random position within macro hexagonal grid per operator, minimum distance between TRP and UE: 10m, Indoor Factor Hall: Optional: Indoor factory with Dense cluster & low BS (InF-DL) Grid, 300m x 150m x 10m factor hall ISD 50m, BS height 1.5m, UE height 1.5m, Typical clutter size 2m, Clutter height 6m, Clutter density 20% Optional: Indoor factory with sparse clutter & High BS (InF-SH) Grid, 300m x 150m x 10m factor hall ISD 50m, BS height 8m, UE height 1.5m, Typical clutter size 10m, Clutter height 2m, Clutter density 60% |  |
| UE distribution | Average of 10 UE per BS For InH open office: 100% indoor UEs For Dense urban: 100% outdoor UEs For InF: 100% indoor UEs | UE randomly distributed over the entire deployment area |
| Channel Model | InH open office: InH – office channel & PL model from TR38.901 Dense Urban: UMi street canyon channel & PL model from TR38.901 Indoor factor: InF channel & PL model from TR38.901 |  |
| Mobility | 3 Km/hr |  |
| BS Antenna Configuration (Mg,Ng,M,N,P) | (1,1,8,16,2) with (0.5 dv, 0.5 dH) Optional: (1,1,4,4,2), (1,1,8,4,2), (1,1,8,8,2), (1,1,16,16,2), (1,1,32,8,2) |  |
| BS Antenna Pattern | Antenna power pattern given in Table 7.3-1 of TR38.901 (with exception of antenna element gain) |  |
| BS Antenna element gain | 5 dBi |  |
| UE Antenna Configuration (Mg,Ng,M,N,P) | (1,1,2,4,2) with (0.5 dv, 0.5 dH) Optional: (1,1,1,2,2), (1,1,2,2,2), (1,1,4,4,2) |  |
| UE Antenna Pattern | Antenna power pattern given in Table 7.3-1 of TR38.901 (with exception of antenna element gain) |  |
| UE Antenna element gain | 0 dBi Optional: 5dBi |  |
| BS Power Limitation | 40 dBm EIRP Maximum TxP adjusted to meet EIRP limits |  |
| UE Power Limitation | 25 dBm EIRP with 21 dBm max TxP Optional: 40dBm EIRP with 21 dBm max TxP |  |
| BS NF | 7 dB |  |
| UE NF | 13 dB Optional: 10 dB |  |
| Transmission Rank | Rank adaptative transmission between Rank 1 and 2 |  |
| PDCCH Overhead | 2 Symbol per slot |  |
| DMRS Overhead | 1 Symbol per slot |  |
| CSI-RS Overhead | - |  |
| SRS Overhead | - |  |
| Other Overhead | - | This can include overhead from beam management, PRACH, RAR, SR, etc. |
| TDD DL/UL Ratio | - |  |
| CSI feedback | Ideal feedback |  |
| Additive Rx EVM | Optional: 5% at Rx | In lieu of PA model, Tx/Rx PN Model, I-Q imbalance, and other RF impairments |
| Traffic Model | FTP Model 3 (0.5MByte file) Optional: Full buffer, FTP Model 1 (27Mbyte file) FTP Model 3 (27Mbyte file) |  |
| UE Receiver | MMSE-IRC |  |
| Cell selection criteria | Random select from strongest RSRP with 1 dB HO Marginhysterisys |  |
| DL/UL Traffic Ratio | 50% DL, 50% UL Optional: 100% DL, 0% UL, 80% DL, 20% UL 0% DL, 100% UL |  |

# Summary of High-level Issues and Considerations

Based on RAN1 Chairman guidance, discussions for Rel-17 supporting NR from 52.6 GHz to 71 GHz SI for RAN1 #101-e meeting will be limited to simulation assumptions and some high-level issues and considerations. In this section, we summarize candidate high-level issues for discussion during RAN1 #101-e meeting.

- Candidate numerology (SCS, and CP length) to be supported by RAN1 specification.

  - Discussions may include how RAN1 should conclude on determination of the candidate numerologies

  - Discussion may also include identification of any coupling with other system parameters, such as bandwidth (number of PRB), FFT size, etc

- Candidate bandwidths (or range of bandwidth) to be supported by RAN1 specification and related considerations (e.g. maximum FFT size)

  - Discussions may include how RAN1 should conclude on determination of the candidate bandwidths

- Identification of regulatory aspects to consider in channel access (and interference mitigation techniques) for 60GHz unlicensed NR operation

  - Some examples could be CCA sensitivity levels, time unit for measurement and back-off counters, access categories, channel bandwidth occupancy, etc.

- Supported LBT modes of operation (e.g. omni-directional LBT, directional LBT, receiver-aided LBT, no-LBT, etc)

  - Discussion may include how RAN1 should conclude on LBT mode of operations and identification of various consideration aspects (in the decision-making process)

  - Discussions may also include whether to always mandate LBT operations or not

Other issues discussed in submitted contributions are (not an exhaustive list):

- Investigation of directional LBT

- Investigation of receiver-aided LBT

- Shared COT mechanisms

- Beam sweeping issues for SS/PBCH blocks

- Beam failure detection issues

- Potential enhancements to increase the channel access opportunities

- Energy detection threshold calculation to account for instance for the directivity of LBT, or LBT channel bandwidth

- OCB constraints and related specification impact

- FBE operations

- SSB and CORESET#0 multiplexing

# Reference

- R1-2003286, “Channelization in unlicensed spectrum above 52.6GHz and below 71GHz,” Futurewei

- R1-2003293, “Link level evaluation methodology and considerations for PHY design in 52.6-71 GHz using NR waveform,” Huawei, HiSilicon

- R1-2003304, “Discussion on potential physical layer impacts for NR beyond 52.6 GHz,” Lenovo, Motorola Mobility

- R1-2003424, “Discussion on requried changes to NR using existing DL/UL NR waveform,” vivo

- R1-2003461, “Discussion on the required changes to NR for above 52.6GHz,” ZTE, Sanechips

- R1-2003636, “System Analysis and evaluation methodology of NR opration up to 71 GHz,” CATT

- R1-2003679, “On Required changes to NR using existing DL/UL NR waveform for supporting NR from 52.6GHz to 71 GHz,” MediaTek Inc.

- R1-2003764, “Discussion on Required Changes to NR in 52.6 – 71 GHz,” Intel Corporation

- R1-2003777, “Waveform Enhancements for 60GHz operation,” Charter Communications, Inc

- R1-2003811, “Required changes to NR using existing DL/UL NR waveform,” Nokia, Nokia Shanghai Bell

- R1-2003849, “On NR operations in 52.6 to 71 GHz,” Ericsson

- R1-2003903, “Design aspects for extending NR to up to 71 GHz,” Samsung

- R1-2003945, “Discussions on Changes to NR Using Existing DL/UL NR Waveform,” Quectel

- R1-2003961, “Discussion on required changes to NR for above 52.6GHz,” CMCC

- R1-2004004, “Discussion on required changes to NR using existing NR waveform,” Spreadtrum Communications

- R1-2004038, “Consideration on required physical layer changes to support NR above 52.6 GHz,” LG Electronics

- R1-2004096, “Discussion on the waveform for DL/UL at above 52.6 GHz,” OPPO

- R1-2004188, “Considerations on bandwidth and subcarrier spacing for above 52.6 GHz,” Sony

- R1-2004247, “On Required changes to NR using existing DL/UL NR Waveform,” Apple

- R1-2004299, “Consideration on supporting above 52.6GHz in NR,” InterDigital, Inc.

- R1-2004301, “Consideration on required changes to NR using existing NR waveform,” Fujitsu

- R1-2004312, “Study on the required changes to NR using existing DL/UL NR waveform,” NEC

- R1-2004333, “Initial views of required changes of NR DL/UL waveform,” Sharp

- R1-2004340, “Discussion on required changes to NR using existing DL/UL waveform,” ITRI

- R1-2004417, “Evaluation Methodology and Required Changes on NR on 52.6 to 71 GHz,” NTT DOCOMO, INC.

- R1-2004500, “NR using existing DL-UL NR waveform to support operation between 52p6 GHz and 71 GHz,” Qualcomm Incorporated

- R1-2004519, “Discussions on required changes on supporting NR from 52.6GHz to 71 GHz,” CAICT

- R1-2004593, “On NR operation between 52.6 GHz and 71 GHz,” Convida Wireless

- R1-2003287, “Considerations on channel access in the unlicensed spectrum above 52.6GHz and below 71GHz,” Futurewei

- R1-2003294, “Channel access mechanism for 60 GHz unlicensed operation,” Huawei, HiSilicon

- R1-2003305, “Discussion on channel access for NR beyond 52.6 GHz,” Lenovo, Motorola Mobility

- R1-2003425, “Discussion on channel access mechanism,” vivo

- R1-2003462, “Discussion on the channel access mechanism for above 52.6GHz,” ZTE, Sanechips

- R1-2003637, “Channel Access Mechanism in support,” CATT

- R1-2003680, “On channel access mechanism from 52.6GHz to 71GHz,” MediaTek Inc.

- R1-2003765, “Channel Access Procedure for NR in 52.6 - 71 GHz,” Intel Corporation

- R1-2003780, “Channel access mechanisms for 60 GHz NR-U,” Charter Communications, Inc

- R1-2003812, “NR coexistence mechanisms for 60 GHz unlicensed band,” Nokia, Nokia Shanghai Bell

- R1-2003850, “Channel Access Mechanism,” Ericsson

- R1-2003904, “Channel access mechanism for 60 GHz unlicensed spectrum,” Samsung

- R1-2003921, “Discussion on channel access mechanism for supporting NR from 52.6GHz to 71 GHz,” Beijing Xiaomi Mobile Software

- R1-2003962, “Discussion on channel access for above 52.6GHz,” CMCC

- R1-2004005, “Discussion on channel access mechanism for above 52.6GHz,” Spreadtrum Communications

- R1-2004039, “Considerations on channel access mechanism to support NR above 52.6 GHz,” LG Electronics

- R1-2004097, “Discussion on channel access mechanism for above 52.6GHz,” OPPO

- R1-2004174, “Channel access mechanism,” TCL Communication Ltd.

- R1-2004189, “Channel access mechanism for 60 GHz unlicensed spectrum,” Sony

- R1-2004248, “On Unlicensed Channel Access Mechanisms for > 52.6GHz,” Apple

- R1-2004288, “Channel access mechanisms for NR from 52.6-71GHz,” AT&T

- R1-2004300, “On Channel Access Mechanisms,” InterDigital, Inc.

- R1-2004303, “Discussions on channel access mechanism for NR from 52.6GHz to 71 GHz,” Quectel

- R1-2004313, “Study on the channel access mechanism,” NEC

- R1-2004334, “Channel access mechanism,” Sharp

- R1-2004341, “Discussion on channel access mechanism,” ITRI

- R1-2004418, “Channel Access Mechanism for NR in 60 GHz unlicensed spectrum,” NTT DOCOMO, INC.

- R1-2004489, “Channel access mechanism for NR in 52.6 to 71GHz band,” Qualcomm Incorporated

- R1-2004520, “Discussions on channel access mechanism on supporting NR from 52.6GHz to 71 GHz,” CAICT

- R1-2004558, “Discussion on channel access for unlicensed spectrum from 52.6GHz to 71 GHz,” Potevio

- R1-2004594, “Considerations on channel access mechanism for NR supporting from 52.6 to 71 GHz,” Convida Wireless

- R1-2003426, “Evaluation on different numerologies for NR using existing DL/UL NR waveform,” vivo

- R1-2003463, “Discussion on the evaluation assumptions for above 52.6GHz,” ZTE, Sanechips

- R1-2003766, “Considerations on performance evaluation for NR in 52.6-71GHz,” Intel Corporation

- R1-2003813, “Performance of existing DL NR waveform beyond 52.6 GHz,” Nokia, Nokia Shanghai Bell

- R1-2003851, “Enhanced phase noise modeling,” Ericsson

- R1-2003905, “Evaluaton assumptions for SLS and LLS,” Samsung

- R1-2004040, “Consideration on multi-carrier operation for NR above 52.6 GHz,” LG Electronics

- R1-2004098, “Discussion on deployment scenarios for carrier frequency above 52.6GHz,” OPPO

- R1-2004419, “Potential Enhancements for NR on 52.6 to 71 GHz,” NTT DOCOMO, INC.

- R1-2004606, “System level evaluation methodology and results for 60 GHz unlicensed operation,” Huawei, HiSilicon

- R1-163984, “Discussion on phase noise modeling,” Samsung