3GPP TSG-RAN WG1 #101								R1-2004710

e-Meeting, May 25th-June 5th, 2020

Source: 	Moderator (Apple)

Title:                     Outcome of preparation discussion on UL Power Control for NN-DC

Agenda item:	7.2.10.1

Document for:	Discussion and Decision

# 1 Introduction

This contribution summarizes the contributions made under the “Uplink Power Control for Supporting NR-NR Dual-Connectivity” sub-agenda item for 7.2.10.1 Rel-16 Maintenance of Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR).

In addition, the issues selected for this meeting were captured in Section 3 based on the companies’ inputs.

# 2. Discussion

Table 1 summaries the identified issues in accordance to the contributions submitted and more details for each issue were provided in the following sections.

Table 1: Issues scoping based on contributions

## 2.1	Issue-1: RAN2 LS reply on T_offset determination WA

One working assumption was made in ran1 #100-eMeeting related to T_offset determination and corresponding UE capability signaling. According to LS [12], MN is required to process the SCG configuration to identify the T_offset used by the UE; Otherwise, possible largest value of T_offset has to be assumed by MN for dynamic power sharing operation.

The LS reply from RAN2 [13] was received in this meeting and it states the following:

“RAN2 would like to thank RAN1 for the LS on uplink power control for NR-NR Dual-Connectivity. RAN2 is still discussing the reply to RAN1 but has no consensus yet on introducing new inter-node signalling for T_offset. However, RAN2 would like to remind that it was agreed in Rel-15 that MN and SN are not required to comprehend each other’s UE configuration for MR-DC. Therefore, RAN1 making assumption that such comprehension is possible is not correct in RAN2 view.”

In addition, RAN2 is still discussing new functionality to improve the efficiency of DPS by introducing inter-gNB signal and hence avoid assuming the worst T_offset value, even RAN1 did not ask for this.

Several companies [1][3][6][7][8][11] discussed the impact of RAN2 reply LS with following summarized proposal

- Opt.1: No impact and confirm the WA with removing Tproc,CSI from second capability (i.e. Alt.2 in [11]) for T_offset determination [6][11].

- Opt.2: Revising the WA and defining the T_offset values in RAN1 without the need of MN/SN interaction [1][7][10]:

  - Option 2-1: <1, 0.5> ms [1]

    - Introducing one RRC parameter to configure the T_offset value for UEs on a per FR basis [1].

  - Option 2-2: <4, 1.5> ms [10]

  - Reasoning:

- Opt.3: NOT support “look-ahead” with T_offset [8]

- Opt.4: Wait from RAN2 further progress on email discussion [3]

The reasons for these options were summarized in the Table 2 below based on papers:

Table 2:

| Index | No. of companies | Companies | Reasoning |
| --- | --- | --- | --- |
| Opt.1 | 2 | Samsung [6], Qualcomm [11] | - No impact was seen on the RAN1 spec as the situation that “MN and SN do not comprehend each other’s UE configuration for MR-DC” was recognized already when making this agreement. [11] - No impact was seen on the RAN1 spec. When the MN can comprehend the UE configurations of SCG, it helps MN to select a suitable and optimal value for T_offset. If not, MN just set the T_offset based on the possible largest value. [6] - Current WA is better than specifying “absolute value” [11] |
| Opt.2 | 3 | Nokia [7], ZTE [1] (Opt.2-1), Ericsson [10] (Opt.2-2) | - Considering the late stage of the WI discussions (RAN4 is already waiting for RAN1 specs to stabilize to specify performance requirements), our preference is to fix this issue in RAN1 in this meeting rather than wait and rely on other WGs to find a solution. This removes the linking of T_offset with RRC configurations of MCG and SCG and is fully aligned with RAN2 MR-DC framework and also current UE capability framework for dynamic power sharing [10]. |
| Opt.3 | 1 | OPPO [8] | - The current framework for look-ahead operation doesn’t work based on RAN2 LS [8] |
| Opt.4 | 1 | Huawei [3] | - RAN2 email thread discussion is still ongoing about how to accommodate the RAN1 WA and, then MN scheduler could be aware of the Tproc,SCG maxand take the restriction introduced by the Toffset into consideration [3]. |

## 2.2 Issue-2: TPC Commands in DCI format 2_2 and 2_3

DCI format 2_2

NR supports different timeline of group-common TPC in DCI format 2_2 for dynamic PUSCH (DG-PUSCH) and CG-PUSCH. More specifically, for DG-PUSCH, the k2 value in scheduling DCI format is used to limit the last applicable DCI 2_2. While, for CG-PUSCH, minimum K2 value configured in PUSCH-ConfigCommon (i.e. cell-specific) is used to determine the last applicable DCI format 2_2. It is FFS regarding how to handle the case when DCI format 2_2 or 2_3 is received after T0-T_offset, as shown in FIG.1 copied from [11].

Figure 1: Group-TPC commands in DCI 2_2 received after T0 – T_offset.

This issue was extensively discussed in RAN1 #100 bis e-meeting, but no consensus was reached. Ten companies [1][2][3][4][5][6][8][9][10][11] shared views on this open issue and can be generally summarized as follows:

Table 3:

|  | Description | Companies |
| --- | --- | --- |
| Opt.1 | The NW ensures that the group TPC command in the DCI format 2_2 received after T0 – T_offset that is applied to the semi-static MCG UL transmission that overlaps with the SCG UL transmission starting at time T0 does not indicate transmission power increase for the MCG UL transmission. The UE can determine the sum power for the MCG UL transmission based on the information available at time T0 – T_offset. | Qualcomm [11] |
| Opt.2 | When UE has an SCG UL transmission and an overlapping MCG UL transmission, then for adjusting the power of the MCG UL transmission - the UE shall only consider TPC commands that are provided by DCI format 2-2 in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG | Ericsson [10] ZTE [1] Apple [9] Intel [5] |
| Opt.3 | Leave it to UE implementation | VIVO [2] |
| Opt.4 | When UE has an SCG UL transmission and an overlapping MCG UL transmission, the UE is not expected to receive the PDCCH with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG for below cases: - DCI format 2_2 indicated transmission power adjustment for a PUSCH/PUCCH transmission on MCG, or | HW [3] MTK [4] |
| Opt.5 | - When a UE is configured with NR-DC with dynamic power-sharing, for the semi-static MCG UL transmission starting at time T1, regardless of whether it overlaps with a SCG UL transmission, the UE does not apply TPC commands in DCI fomat 2_2 received after T1 – {minimum k2 value configured in PUSCH-Config}. NW ensures that minimum k2 value configured in the PUSCH-Config such that non-ignored DCI format 2_2 is not present after T0 – T_offset if there is an overlapping SCG transmission starting at time T0. |  |

DCI 2_3

As discussed in RAN1 #101bis e-meeting, it should that the DCI format 2_3 is used to trigger SRS carrier switching along with TPC commands for the SRS transmission. One point raised during email discussion of preparation phase is whether or not the DCI format 2_3 itself has time restriction, instead of TPC command only. In other words, since DCI format 2_3 is also used to trigger SRS carrier switching, it should not be allowed to transmit after T_offset, same as scheduling DCI format,

Companies views on this issue was summarized in the following Table 4:

Table 4:

|  | Description | Companies |
| --- | --- | --- |
| Opt.1 | When UE has an SCG UL transmission and an overlapping MCG UL transmission, then for adjusting the power of the MCG UL transmission, the UE does not expect to receive DCI format 2-3 in PDCCH receptions with a last symbol that is not earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG | HW [3] Intel [5] Samsung [6] |
| Opt.2 | …. |  |

## 2.3	Issue-3: Handling UL Transmission Cancelation on MCG

Another open issue discussed in last RAN1 meeting is how to handle UL transmission cancelation on MCG for MCG dynamic grant PUSCH/configured grant PUSCH due to 5.4.3.1.3 of TS38.321 or by DCI format 2_4. Although DCI 2-4 does not schedule uplink transmissions, it would impact on the value of PMCGactual due to UL cancelation. Similarly, for CG-PUSCH transmission, the UE may or may not transmit the PUSCH. For dynamic grant PUSCH, the UE may skip the transmission if some conditions are met which are specified in 5.4.3.1.3 of TS38.321.

For different cancelation cases, companies’ views can be categorized using following table:

Case 1: DCI format 2_0/2_4

Table 5:

|  | Description | Companies |
| --- | --- | --- |
| Opt.1 | For power determination of UL transmission in SCG starting at T0, UE is not required to take into account the skipped MCG UL transmission due to either DCI format 2_0/2_4 or according to the section 5.4.3.1.3 of TS 38.321 received after T0-T for PMCGactual determination for the UL transmission in MCG overlapping with the concerned SCG transmission | ZTE [1] HW [3] Samsung [6] Apple [9] Qualcomm [11] |
| Opt.2 | For power determination of UL transmission in SCG starting at T0, UE does not expect to receive DCI format 2_0/2_4 that is received in a PDCCH with last symbol that is earlier by less than T_offset from a first symbol of a transmission occasion on the SCG | MTK [4] |
| Opt.3: | left for UE implementation | VIVO [2] |

Case 2: CG-PUSCH (different with Case 1 as the cancelation is not associated with DCI format)

Table 6

|  | Description | Companies |
| --- | --- | --- |
| Opt.1 | UE assumes that actual CG-PUSCH transmission exists in every transmission occasion. | ZTE [1] |
| Opt.2 | If the time instance {T1 – Tproc,2} is earlier than {T0 – Toffset}, this CG-PUSCH is considered into the power calculation. Otherwise, if the time instance {T1 – Tproc,2} is later than {T0 – Toffset}, this CG-PUSCH is not considered into the power calculation | ZTE [1] |
| Opt.3: | left for UE implementation | VIVO [2] MTK [4] Intel [5] Qualcomm [11] |

## 2.4	Issue-4: Power Determination for PUCCH and SRS Transmission

Another issue is how to determine the power of periodic PUCCH (e.g. SR, P-/SP-CSI) and P/SP-SRS. Views on this issue was summarized as follows:

Table 7:

|  | Description | Companies |
| --- | --- | --- |
| Opt.1 | - UE assumes there is always UL transmission in the periodic PUCCH (e.g. P-/SP-CSI) resource and P-/SP-SRS resource for PMCGactual determination. - UE is not required to take into account the cancellation of MCG UL transmissions for dynamic power sharing (i.e. it is up to UE implementation whether to take into account for power determination). | ZTE [1] Apple [9] VIVO [2] MTK [4] Intel [5] Qualcomm [11] |

## 2.5 	Issue 5: Removal of earlier text on DPS

This issue was brought up in [11] to remove the following text in TS 38.213, taking into account the agreed WA context and lack of agreements made for supporting the following context:

“

-	if UE transmission(s) in slot i1 of the MCG overlap in time with UE transmission(s) in slot i2 of the SCG and if PMCGactuali1+PSCGactuali2>PTotalNR-DC in any portion of slot i2 of the SCG, the UE reduces transmission power in any portion of slot i2 of the SCG so that PMCGactuali1+PSCGactuali2≤PTotalNR-DC in all portions of slot i2, where PMCGactuali1 and PSCGactuali2 are the UE transmission powers in slot i1 of the MCG and in slot i2 of the SCG, respectively, that the UE determines according to Clauses 7.1 through 7.5 using PMCG and PSCG, respectively, as the maximum transmission powers on the MCG and the SCG and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3]

-	if UE transmission(s) in slot i1 of the MCG or in slot i2 of the SCG do not overlap in time with any UE transmission(s) on the SCG or the MCG, respectively, the UE determines a transmission power in slot i1 of the MCG or in slot i2 of the SCG as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

This issue was discussed in last RAN1 #100 bis e-meeting and no conclusion was made due to concern raised on the potential impact on PRACH transmission on MCG. More specifically, one company wants to clarify how to handle the PRACH transmission first and then to decide whether or not to remove this paragraph.

## 2.6	Issue 6: RRC parameter alignment

One TP was proposed in [8] to align with RAN2 specification as following:

In TS 38.213 Section 7.6 Dual Connectivity CR R1-2003183 ### 7.6.2 NR-DC If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5. If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2-r16 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2-r16 and with an inter-CG power sharing mode by nrdc-PCmode-FR1-r16 nrdc-PC-mode-FR1-r16 for FR1 and/or by nrdc-PCmode-FR2-r16 nrdc-PC-mode-FR2-r16 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range. If a UE is provided semi-static-mode1 fornrdc-PCmode-FR1-r16 nrdc-PC-mode-FR1-r16 or for nrdc-PCmode-FR2-r16 nrdc-PC-mode-FR2-r16, or semi-static-mode2 fornrdc-PCmode-FR1-r16 nrdc-PC-mode-FR1-r16 or for nrdc-PCmode-FR2-r16 nrdc-PC-mode-FR2-r16, the UE does not expect PMCG and PSCG to be configured such that PMCG+PSCG>PTotalNR-DC, where PMCG is the linear value of PMCG, PSCG is the linear value of PSCG, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3]. If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1-r16 nrdc-PC-mode-FR1-r16 or for nrdc-PCmode-FR2-r16 nrdc-PC-mode-FR2-r16, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively. If a UE is provided semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2 - if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG overlapping with slot i2 as described in Clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power - otherwise, the UE determines a power for the transmission on MCG or the SCG overlapping with slot i2, as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively The UE expects to be provided semi-static-mode2 for nrdc-PCmode-FR1-r16 nrdc-PC-mode-FR1-r16 or for nrdc-PCmode-FR1-r16 nrdc-PC-mode-FR2-r16 only for synchronous NR-DC operation [10, TS 38.133]. If the UE indicates a capability for dynamic power sharing between the MCG and the SCG and is provided dynamic for nrdc-PCmode-FR1-r16 nrdc-PC-mode-FR1-r16 or for nrdc-PCmode-FR2-r16 nrdc-PC-mode-FR2-r16 - if UE transmission(s) in slot i1 of the MCG overlap in time with UE transmission(s) in slot i2 of the SCG and if PMCGactuali1+PSCGactuali2>PTotalNR-DC in any portion of slot i2 of the SCG, the UE reduces transmission power in any portion of slot i2 of the SCG so that PMCGactuali1+PSCGactuali2≤PTotalNR-DC in all portions of slot i2, where PMCGactuali1 and PSCGactuali2 are the linear values of UE transmission powers in slot i1 of the MCG and in slot i2 of the SCG, respectively, that the UE determines according to Clauses 7.1 through 7.5 using PMCG and PSCG, respectively, as the maximum transmission powers on the MCG and the SCG and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3] - if UE transmission(s) in slot i1 of the MCG or in slot i2 of the SCG do not overlap in time with any UE transmission(s) on the SCG or the MCG, respectively, the UE determines a transmission power in slot i1 of the MCG or in slot i2 of the SCG as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively If a UE - is provided dynamic for nrdc-PCmode-FR1-r16 nrdc-PC-mode-FR1-r16 or for nrdc-PCmode-FR2-r16 nrdc-PC-mode-FR2-r16, and - indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that - are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, and - overlap with the transmission occasion on the SCG <omitted text>

# 3	Conclusion on the scope of the RAN1#101

Companies inputs about issues prioritization were captured in Table 8 below:

Table 8: Issues ranking for handling in meeting by companies

Based on the inputs, the following was proposed by feature leader to scope the email discussions for this topic:

Feature Leader Proposal:

- Email thread 1: Issue 1/2/3/4/5 in R1-2004237.

# References

- R1-2003324	Remaining Issues of Dynamic Power Sharing for NR-DC	ZTE

- R1-2003410	Remaining issues on uplink power control for NR-NR DC	vivo

- R1-2003501        	UL power control for NR-NR dual connectivity	Huawei, HiSilicon

- R1-2003672        	Remaining issues on uplink power control for supporting NR-NR dual-connectivity				MediaTek Inc.

- R1-2003749	Remaining issues on uplink power control for NN-DC	Intel Corporation

- R1-2003891	Remaining Issues on UL Power Control for NR-DC	Samsung

- R1-2003939	Remaining details of Rel-16 DC uplink power control	Nokia, Nokia Shanghai Bell

- R1-2004057	Text proposals for UL Power Sharing for NR-DC	OPPO

- R1-2004236	Remaining issues of UL Power Control for NN-DC	Apple

- R1-2004361	Remaining issues for NR-DC UL Power Control	Ericsson

- R1-2004473	Remaining issues on NR-DC power-control	Qualcomm Incorporated

- R1-2001421, LS on uplink power control for NR-NR Dual-Connectivity, RAN1, RAN1#100-e

- R1-2003262, LS reply on uplink power control for NR-NR Dual-Connectivity, RAN2, RAN1#101-e