3GPP TSG RAN WG1 Meeting #101-e	R1- 2004724

e-Meeting, 25th May – 5th June 2020

Title:	Final Summary of PDCCH based Power Saving Signal/Channel

Agenda item:	7.2.7.1

Source:	CATT

Document for:	Discussion

Final Summary of Email Discussions and Agreements

Email thread [101-e-NR-NR_UE_Pow_Sav-WUS-01]

The dropping rule for collision between RAR and DCP was heavily discussed based on the questions from RAN2 LS in R1-2003260.     The summary of the discussion in RAN1 is as follows,

- RAN1 common understanding that RAR is critical and should have higher priority than that of DCP when they collide.

- Current RAN1 specification supports RAR prioritizing over DCP in some scenarios – there is no impact to the legacy RAR behavior

  - RAR addressed by RA-RNTI  or MsgB-RNTI

  - RAR addressed by C-RNTI when RAR is configured on Type-3 CSS with lower index than that of DCP

- Scenarios that DCP would be prioritized over RAR – there is impact to the legacy RAR behavior

  - RAR address by C-RNTI when RAR is configured on Type 3 CSS with higher index than that of DCP

  - RAR addressed by C-RNT when RAR is configured on USS

- Different views in RAN1 whether bullet (3) should be addressed with RAN1 specification change – There is no consensus on the need of the changes

  - Pro – RAN1 specification change to ensure RAR is always prioritized

  - Con – gNB implementation could avoid any scenarios in bullet (3).  There is no specification change.

The reply LS is to capture the status of RAN1 specificaiton in reply to priority rule of RAR and DCP collision in RAN2 LS.   During the discussion, the DL channel combination associated with PS-RNTI in Table 6.2-2 of TS38.202 was discussed.  The proposed TP was not agreed and expect to discuss further at next meeting.

The agreements are as follows,

Conclusion:

- There is no consensus on whether the specification change is needed to support the RAR addressed by all RNTI’s prioritizing over DCP in all scenarios

The latest draft reply LS to RAN2 is approved in R1-2004994.

Email thread [101-e-NR-NR_UE_Pow_Sav-WUS-02]

The email discussion on Issue 1 of minimum time gap value and Issue 4 DCI size budget for DCI format 2_6 with the following agreements and conclusions.

Agreements:

- Confirm the WA on the minimum time gap values made in RAN1#100b-e

Conclusion:

- No consensus on additional specification change in DCI size alignment for DCI format 2_6.

Email thread [101-e-NR-NR_UE_Pow_Sav-WUS-03]

Motivation of RAN1/RAN2 specification alignement

- TP for TS38.213:  The proposed TP was agreed to capture the general behavior.  The proposed TP is to clarify the the meaning  of the decoding bit value ‘1’ and ‘0’ from DCI format 2_6 associated with Wake-up and no-Wake-up indication of 38.213 from the agreed editor’s CR in [19].  In addition, RRC parameters ps-PositionDCI-2-6 and sizeDCI-2-6  are updated.

- TP for TS38.214:  RAN2 LS in R1-2003260 indicated that the flag ps-TransmitPeriodicCSI is renamed to ps-TransmitOtherPeriodicCSI.  The agreed TP is to update the name of the RRC parameter in RAN1 spec.

Agreements:

Adopt the TP for clarification of wakeup indication bit shown below

-----------------------------------------------------Start of TP of 38.213---------------------------------------------------------- 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331] - a PS-RNTI for DCI format 2_6 by ps-RNTI - a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in Clause 10.1 - a payload size for DCI format 2_6 by SizeDCI_2-6 sizeDCI_2-6 - a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6 ps-PositionDCI-2-6, where - the UE may not start the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '0', and - a ‘0’ value for the Wake-up indication bit, when reported to higher layers, indicates not to start the drx-onDurationTimer for the next long DRX cycle (as described in [14, TS 38.321]) - a ‘1’ value for the Wake-up indication bit, when reported to higher layers, indicates to start the drx-onDurationTimer for the next long DRX cycle (as described in [14, TS 38.321]) - the UE starts the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '1' *** Unchanged text is omitted *** If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE detects DCI format 2_6, the physical layer of a UE reports the value of the Wake-up indication bit for the UE to higher layers [14, TS 38.321] for the next long DRX cycle. If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE does not detect DCI format 2_6, if the UE is provided ps-WakeupOrNot, the physical layer of the UE does not report a value of the Wake-up indication bit UE is indicated by ps-WakeupOrNot whether the UE may not start or whether the UE shall start the drx-onDurationTimerto higher layers for the next long DRX cycle. if the UE is not provided ps-WakeupOrNot, the UE may not start Active Time indicated by drx-onDurationTimer for the next DRX cycle If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle the physical layer of the UE shall reports a value of 1 for the Wake-up indication bit to higher layers start the drx-onDurationTimer for the next long DRX cycle. -----------------------------------------------------End of TP of 38.213---------------------------------------------------------------

Agreements:

Adopt the TP for Clause 5.1.6.1 and 5.2.2.5 of TS 38.214.

5.1.6.1 CSI-RS reception procedure <omitted text> If the UE is configured with DRX, - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitOtherPeriodicCSI ps-TransmitPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to quantities other than ‘cri-RSRP’ and ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to cri-RSRP when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported. <omitted text>

5.2.2.5 CSI reference resource definition <omitted text> When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI ps-TransmitPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to quantities other than ‘cri-RSRP’ and ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to ‘cri-RSRP’ or ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to ‘cri-RSRP’ if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. <omitted text>

Summary of Email Discussions

Email Discussion [101-e-NR-NR_UE_Pow_Sav-WUS-01]

Email discussion and proposals

The email discussion of collision of DCP and RAR, companies agreed that RAR for BFR is more critical than that of DCP monitoring.   However, there are different views on how the RAR for BFR is prioritized over DCP

- Based on current priority rule in TS38.213 without further specification change – RAR is sent at CSS2 or CSS3 with lower index than that of DCP by gNB implementation.

  - Supported by:  Samsung, Huawei, HiSilicon, Panasonic, OPPO, vivo

- RAR for BFR addressed by all RNTI should be prioritized over DCP

  - Supported by: LG, ZTE, CMCC, Nokia, Qualcomm, Intel, MediaTek, Ericsson, DoCoMo, Interdigital, APPLE, Sony, CATT

The majority companies would support that RAR for BFR addressed by all RNTIs is prioritized over DCP.   From RAN2 LS in R1-2003260, the RAR for BFR could be addressed by both RA-RNTI and C-RNTI outside Active Time with potential collision to DCP within RACH response window.  Similarly the 2 step RACH could have RACH MsgB addressed by C-RNTI.  Thus, the proposal is to have TP supporting RAR for BFR addressed by C-RNTI in USS with higher priority over DCP in type 3 CSS.   In the mean time, the invalid DCP monitoring occasion is updated with reference to Clauses 5.1.4 and 5.1.4a of TS38.321.

The combination of the channels received simultaneously by UE in Clause 6.2 of TS38.202 needs to be updated to include the PDCCH and PDSCH with C-RNTI.

Proposal 1:

RAR for BFR addressed by all RNTIs is prioritized over DCP.

Proposal 2:

TP to Clause 10.1 and Clause 10.3 of TS 38.213

-----------------------------------------------------Start of TP of 38.213---------------------------------------------------------- 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** If a UE - is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and - monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have same or different QCL-TypeD properties on active DL BWP(s) of one or more cells the UE monitors PDCCHs only in a CORESET, and in any other CORESET from the multiple CORESETs having same QCL-TypeD properties as the CORESET, on the active DL BWP of a cell from the one or more cells - the CORESET corresponds to the CSS set with the lowest index and CRC not scrambled by PS-RNTI in the cell with the lowest index containing CSS, if any; otherwise, to the USS set with the lowest index in the cell with lowest index - the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions - for the purpose of determining the CORESET, a SS/PBCH block is considered to have different QCL-TypeD properties than a CSI-RS - for the purpose of determining the CORESET, a first CSI-RS associated with a SS/PBCH block in a first cell and a second CSI-RS in a second cell that is also associated with the SS/PBCH block are assumed to have same QCL-TypeD properties - the allocation of non-overlapping CCEs and of PDCCH candidates for PDCCH monitoring is according to all search space sets associated with the multiple CORESETs on the active DL BWP(s) of the one or more cells - the number of active TCI states is determined from the multiple CORESETs *** Unchanged text is omitted *** 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells *** Unchanged text is omitted *** If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7, 5.1.4 and 5.1.4a of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle the physical layer of the UE reports a value of 1 for the Wake-up indication bit to higher layers for the next long DRX cycle. *** Unchanged text is omitted *** -----------------------------------------------------End of TP of 38.213---------------------------------------------------------------

Proposal 3:

TP to Clause 6.2  of TS 38.202 for the channel combination of DCP and PDCCH+PDSCH addressed by either RA-RNTI or C-RNTI

-----------------------------------------------------Start of TP of 38.202---------------------------------------------------------- - 6.2 Downlink The tables 6.2-1, 6.2-2 describe the possible combinations of physical channels that can be received simultaneously in the downlink by one UE. Table 6.2-1 introduces notation for a "Reception Type" which represents a physical channel and any associated transport channel. Table 6.2-2 describes the combinations of these "Reception Types" which are supported by the UE depending on capabilities [8, TS 38.306], and enumerates how many of each can be received simultaneously. The UE shall be able to receive all TBs according to the indication on PDCCH. Any subset of the combinations specified in table 6.2-2 is also supported. Table 6.2-1: Downlink "Reception Types" "Reception Type"; Physical Channel(s); Monitored RNTI; Associated Transport Channel; Comment. A; PBCH; N/A; BCH. B; PDCCH+PDSCH; SI-RNTI; DL-SCH; Note 1. C0; PDCCH; P-RNTI; N/A; Note 1, Note 2. C1; PDCCH+PDSCH; P-RNTI; PCH; Note 1. D0; PDCCH+PDSCH; RA-RNTI or Temporary C-RNTI or MsgB-RNTI; DL-SCH; Note 3. D1; PDCCH+PDSCH; C-RNTI, CS-RNTI, MCS-C-RNTI; DL-SCH. D2; PDCCH; C-RNTI, CS-RNTI, MCS-C-RNTI; DL-SCH. E; PDCCH; C-RNTI; N/A; Note 4. F0; PDCCH; Temporary C-RNTI; UL-SCH; Note 3. F1; PDCCH; C-RNTI, CS-RNTI, MCS-C-RNTI; UL-SCH. G; PDCCH; SFI-RNTI; N/A. H; PDCCH; INT-RNTI; N/A. J0; PDCCH; TPC-PUSCH-RNTI; N/A. J1; PDCCH; TPC-PUCCH-RNTI; N/A. J2; PDCCH; TPC-SRS-RNTI; N/A. K; PDCCH; SP-CSI-RNTI; N/A. L0; PDCCH; SL-RNTI; SL-SCH. L1; PDCCH; SLCS-RNTI; SL-SCH. M; PDCCH; SL Semi-Persistent Scheduling V-RNTI; SL-SCH; Note 5. N; PDCCH; PS-RNTI; N/A. O; PDCCH; AI-RNTI; N/A. Note 1: These are received from PCell only. Note 2: In some cases UE is only required to monitor the short message within the DCI for P-RNTI. Note 3: These are received from PCell or PSCell. Note 4: This corresponds to PDCCH-ordered PRACH. Note 5: This corresponds to PDCCH scheduling LTE PC5.. Table 6.2-2: Downlink "Reception Type" combinations Supported Combinations; Comment. PCell; PSCell; SCell. 1. RRC_IDLE. A + (B and/or C1 and/or D0) + F0; Note 1. 2. RRC_INACTIVE. A + (B and/or C1 and/or D0) + F0; Note 1. 3. RRC_CONNECTED. (A + C0 + (B and/or (D0 or (m1*D1+m2*D2))) + E + F0 + n*F1 + G + H + J0 + J1 + J2 + K + O + [L0 + L1 + M]) or ((A+B+C0+[D0 or D1]) [and/or] N); (A + (D0 or (m1*D1+m2*D2)) + E + F0 + n*F1 + G + H + J0 + J1 + J2 + K + O + [L0 + L1 + M]) or ((A+B+C0+[D0 or D1]) [and/or] N); m1*D1 + m2*D2 + E + n*F1 + G + H + J0 + J1 + J2 + K + O + [L0 + L1 + M]; Note 2, Note 3, Note 4, Note 5, Note 6, Note 7, Note 8. Note 1: UE is not required to decode more than two PDSCH simultaneously, and decoding prioritization when more than two are received is up to UE implementation. Note 2: For PCell, UE is not required to decode SI-RNTI PDSCH simultaneously with C-RNTI PDSCH, unless in FR1. Note 3: Supported combinations are subject to UE capabilities for dual connectivity, carrier aggregation, receiving of group TPC commands, pre-emption indication and dynamic SFI monitoring. Note 4: The values of m2 ≥ 0 and n≥ 0 in the supported combinations are subject to the UE capability. Note 5: Support of monitoring PDCCH with SL-RNTI, SLCS-RNTI, SL Semi-Persistent Scheduling V-RNTI are subject to UE capability. Note 6: The values of m1 ≥ 1 in the supported combinations are subject to the UE capability [for receiving multi-DCI-based multi-TRP/multi-panel transmissions]. Note 7: In Active time, a UE is not expected to monitor the DCI format for the PDCCH scrambled by PS-RNTI. Note 8: The PDCCH scrambled by PS-RNTI can only be configured on the PCell and PSCell..

Collection of comments during Email Discussions

[101-e-NR-NR_UE_Pow_Sav-WUS-01] Email discussion/approval regarding collison of DCP and RAR

during RAR monitoring window:

- Discussion on issue raised in RAN2 LS R1-2003260. The discussion will also include information from contributions in AI-5

  - R1-2003353    Discussion on DCP Open Issues       vivo

  - R1-2003484    Draft reply LS on  DCP Open Issues            ZTE

  - R1-2003485    Discussion on collision between DCP and RAR      ZTE

  - R1-2003587    Draft LS reply on DCP open issues  CATT

  - R1-2003852    Draft reply LS on RAN2 DCP open issues   Samsung

  - R1-2004113    Reply LS on RAN2 DCP Open Issues          OPPO

  - R1-2004625    Draft reply LS on RAN2 DCP Open Issues Huawei, HiSilicon

  - R1-2004626  Discussion on the collision between DCP and RAR addressed to C-RNTI            Huawei, HiSilicon

- UE PDCCH monitoring by other RNTI in combination with PS-RNTI outside Active Time

  - If agreed, whether TP is needed for TS38.202

by 5/28, with potential TPs by 6/3

| Company | Supporting Issue 2 | Comments |
| --- | --- | --- |
| LG | Support (RAR should be prioritized over DCP) | The BFR procedure can be based on CFRA and/or CBRA. For a CFRA case, either Type3-CSS or USS can be configured for monitoring RAR scrambled by C-RNTI. If a USS is used for RAR monitoring, and if RAR monitoring occasion is fully or partially overlapped with monitoring occaison for DCI format 2_6, a UE skips RAR monitoring by priority rule defined in TS38.213. In our view, BFR procedure should be prioritized rather than wake up signaling. So, we support RAN2’s understanding mentioned in RAN2 LS, i.e., RAR addressed all RNTIs should be prioritized over DCP by the UE. (Or, at least, USS configured for montiroing RAR should be prioritized over DCP.) |
| Samsung | Not support | The BFR search space can be either Type3-CSS or USS. Therefore, the priortization between DCP and RAR addressed to C-RNTI can be fully controlled by gNB. Therefore, we think current prority rule is sufficient and do not see any need of introducing additional priority rule. |
| ZTE | Support (RAR should be prioritized over DCP) | The search space set provided by recoverySearchSpaceId is only monitored after beam failure recovery is triggered. If it collides with DCP and UE prioritizes DCP, it is probable that the CRC check would be failed due to the deteriorated beam link maintenance. What’s more, the likelihood that network sends a not-wake-up indication in this case is low. Hence, it costs additional power for UE to detect the DCP. From the perspectives of UE power saving and link maintenance, RAR should be prioritized over DCP. |
| CMCC | Support (RAR should be prioritized over DCP) | When beam failure is happened, the monitoring of BFR response should be guaranteed to realize fast beam failure recovery. Therefore, when DCP collides with RAR, RAR with CRC scrambled by C-RNTI should be prioritized over DCP. |
| Nokia | Support (RAR should be prioritized over DCP) | Like discussed in our paper, due to the restrictions that are applied to recoverySearchSpaceId SS set configuration, collision between DCP and RAR monitoring can result DCP being prioritized over RAR. To avoid unwanted consequences, we would support capturing behavior to handle this in RAN1 or RAN2 spesification. |
| Huawei, HiSilicon | No need to have specification impact. | We share similar view with Samsung. gNB can configue the RecoverySearchSpace as type3-CSS or USS. Therefore, the gNB can handle the collision between DCI format 2_6 and RAR addressed to C-RNTI. Also, in our view this is not a new issue in Rel-16. It is not necessary to define the new priority between the DCI format 2_6 scrambled by PS-RNTI and the RAR addressed to C-RNTI. |
| Panasonic | Not support | Based on the current priority rule and BFR SS configuration flexibility for RAR addressed to C-RNTI, network is able to control the UE behavior and avoid issue by implementation. Thus we see no issue so far. Even if USS is used for RAR and it is possible that UE may skip RAR monitoring due to DCP, potential delay is forseen but this is also within preparation of gNB as it configures as such. So if this is deemed an issue that needs to address for beam management, it could be discussed in Rel.17. |
| Qualcomm | Support | We agree with RAN2’s view in LS R1-2003260 that RAR addressed all RNTIs should be prioritized over DCP by the UE. At least for RAR addressed RA-RNTI or MsgB-RNTi, the RAR prioritization is already clear according to the following text in TS 38.213, Section 10.1: For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not expect to monitor a PDCCH in a Type0/0A/2/3-PDCCH CSS set or in a USS set if a DM-RS for monitoring a PDCCH in a Type1-PDCCH CSS set does not have same QCL-TypeD properties [6, TS 38.214] with a DM-RS for monitoring the PDCCH in the Type0/0A/2/3-PDCCH CSS set or in the USS set, and if the PDCCH or an associated PDSCH overlaps in at least one symbol with a PDCCH the UE monitors in a Type1-PDCCH CSS set or with an associated PDSCH.. In other words, monitoring of RAR addressed RA-RNTI or MsgB-RNTI (Type1-PDCCH CSS) is proiritized over all other PDCCHs in Type0/0A/2/3 CSS or USS, not just for DCP. Although RAN2 only mentioned DCP in the LS, it seems that the intent is enabling the same level of prioritization as RA-RNTI or MsgB-RNTI monitoring for RAR addressed C-RNTI. In this regard, the spec test above can be revised as follow: For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not expect is not required to monitor a PDCCH in a Type0/0A/2/3-PDCCH CSS set or in a USS set if a DM-RS for monitoring a PDCCH in a Type1-PDCCH CSS set, or for monitoring a PDCCH in the search space set provided by recoverySearchspaceId according to the procedure described in Clause 6, does not have same QCL-TypeD properties [6, TS 38.214] with a DM-RS for monitoring the PDCCH in the Type0/0A/2/3-PDCCH CSS set or in the USS set, and if the PDCCH or an associated PDSCH overlaps in at least one symbol with a PDCCH the UE monitors in a Type1-PDCCH CSS set, or with a PDCCH the UE monitors in the search space set provided by RecoverySearchSpaceId according to the procedure described in Clause 6, or with an associated PDSCH.. According to the above changes, the UE may not be “required” to monitor DCP when it collides with RAR address C-RNTI (during BFR), and it is reagareded as one of the invalid DCP monitoring occasions as described in Section 10.3 in TS 38.213. Thus, no changes in Section 10.3 in TS 38.213 is needed. |
|  |  | According to the suggested TP above, the following changes are suggested in TS 38.202: - Table 6.2-1: D0; PDCCH+PDSCH; RA-RNTI or Temporary C-RNTI or MsgB-RNTI or C-RNTI, MCS-C-RNTI; DL-SCH; Note 3, Note 6. ⁞ Note 6: C-RNTI and MCS-C-RNTI are received in a random access response window.. Note: the intention is to cover the RAR addressed all RNTI in reception type D0. - Table 6.2-2: PCell; PSCell. 3. RRC_CONNECTED. (A + C0 + (B and/or (D0 or (m1*D1+m2*D2))) + E + F0 + n*F1 + G + H + J0 + J1 + J2 + K + O + [L0 + L1 + M]) or ((A+B+C0+B and/or ([D0]) [and/or] N)); (A + (D0 or (m1*D1+m2*D2)) + E + F0 + n*F1 + G + H + J0 + J1 + J2 + K + O + [L0 + L1 + M]) or ((A+B+C0+ ([D0]) [and/or] N)). Note: - For PCell, B (SI-RNTI), D0 (RAR), and N (DCP) can be simultaneously received if their QCL-TypeD properties are the same. - For PSCell, B and C0 are not received (PCell only). |
| Intel | Support (RAR should be prioritized over DCP) | Regarding LS response In our view, SS set prioritization rules that were captured in Section 10.1 of 38.213 mostly applies to active time , although not explicitly captured. We agree with Nokia, ZTE that to avoid unwanted consequences on link performance/maintenance, RAR should be prioritized over DCP. Regarding combination of RNTIs monitored outside active time Currently in Table 6.2-2 of 38.802, the following is captured as combination of RNTIs outside active time in connected mode. ((A+B+C0+[D0]) [and/or] N) Where Table 6.2-1: Downlink "Reception Types" "Reception Type"; Physical Channel(s); Monitored RNTI; Associated Transport Channel; Comment. A; PBCH; N/A; BCH. B; PDCCH+PDSCH; SI-RNTI; DL-SCH; Note 1. C0; PDCCH; P-RNTI; N/A; Note 1, Note 2. D0; PDCCH+PDSCH; RA-RNTI or Temporary C-RNTI or MsgB-RNTI; DL-SCH; Note 3. D1; PDCCH+PDSCH; C-RNTI, CS-RNTI, MCS-C-RNTI; DL-SCH. N; PDCCH; PS-RNTI; N/A. Regardless of whether RAR addressed to C-RNTI is prioritized over DCP or not, it seems clear that RAR addressed to C-RNTI can be received outside active time and the combination ((A+B+C0+[D0]) [and/or] N) does not include RAR addressed to C-RNTI. “Reception Type” D1 can represent RAR addressed to C-RNTI. A note can be added in Table 6.2-1 in this regard. Hence, one way to update the combination is ((A+B+C0+(D0 or D1)) [and/or] N), since at a given either time RAR addressed to C-RNTI or RAR addressed to RA-RNTI/MsgB-RNTI is expected. |
| OPPO | Not needed | The current priority rule for PDCCH minotoring is sufficient with proper gNB configuration. When WUS is configurd for the UE, recoverySearchSpaceId can be configured as Type-3 SS with the ID smaller than that for WUS. |
| vivo | Not needed, no specification impact | PDCCH monitoring for RAR dropping due to different QCL properties can be avoided by proper network implementation. No additional UE behaviors need to be specified. The PDCCH monitoring CSS is prioritized over that in USS, and SS set with lower index is prioritized over the higher ones in the same type of SS set. If PDCCH monitoring in RAR search spaces would be prioritized, it can be configured with lower SS set index than the CSS for WUS. Similarly, for RAR PDCCH monitoring occasions in BFR procedure, the recoverySearchspace can be link to a CSS with lower index compared to that for WUS, to make sure PDCCH monitoring in recoverySearchspace prioritized. Furthermore, network have the flexibility to configure PDCCH monitoring occasions for WUS and RAR in TDM manner. In this case, PDCCH monitoring for RAR dropping due to different QCL properties can be avoided by proper network implementation. |
| MediaTek | Support (RAR should be prioritized over DCP) | We agree that the PDCCH monitoring priority rule is clear in current specification and gNB can prioritize the RAR monitoring by proper configuration. However, we are not sure whether it can be handled by network easily. On the other hand, if WUS/DCP has higher priority than RAR monitoring, it is very likely that UE cannot detect WUS/DCP succussfully due to the deteriorated beam quality. So, monitoring WUS/DCP in this case is not beneficial for both power saving and link maintainence. In order to ensure the performance, we prefer prioritizing RAR monitoring over WUS/DCP. |
| NTT DOCOMO | Support (RAR should be prioritized over DCP) | We think that RAR should be prioritized over DCP. In our understanding, the case of monitoring RAR in type-1 PDCCH CSS, i.e., regular RA and BFR CBRA, is already specified in the current spec so that RAR is prioritized over DCP. More precisely, in such case, monitoring type-1 PDCCH CSS is prioritized over other CSS/USS. Other remaining case is BFR CFRA, and also in this case, RAR should be prioritized similarly as RAR in other cases. Above consideration, we also support the proposal by Qualcomm, i.e., RAR is prioritized over all other PDCCHs in Type0/0A/2/3 CSS or USS, not just for DCP. |
| Ericsson | Support (RAR should be prioritized over DCI 2_6) | Collision between DCI 2_6 and RAR monitoring can result in 2_6 being prioritized over RAR. However, we agree with the RAN2 understanding that RAR should be prioritized over DCI 2_6. We prefer to address this issue through specification instead of adding more NW scheduling restrictions. In our view, the principle should be that link maintainence procedures such as BFR should be prioritized over DCI 2_6 which anyways may not be functional if link is in failure and is under recovery. We prefer to capture behavior in RAN1 and/or RAN2 specs. |
| InterDigital | Support |  |
| Apple | Support (RAR should be prioritized over DCI 2_6), not clear that specification is needed. | We support the prioritization of RAR for BFR over DCP. However, prioritization of RAR for BFR over DCP can be handled by the current proritization rules in Rel-15. The recoverySearchSpaceId can be configured as a Type-3 SS with an ID smaller than that the ID for the Type-2 SS fort he WUS. As such, gNB configuration should be able to handle this. |
| SONY | Support (RAR should be prioritized over DCI 2_6) | We agree with the RAN2 understanding that RAR should be prioritised over DCP. As stated by other companies, if beam failure recovery is triggered, the reliability of DCP is liklely to be low in any case. The UE could / should take the safe option and wake up anyway if it doubts the reliability of DCP. We think that the proportion of times that a RAR / DCP conflict occurs will be low and hence the power consumption impact of the UE taking the “safe option” and waking up is minimal. While the BFR search space can be configured as either CSS or USS, we don’t see why a network would want to configure the BFR search space as CSS just to deal with this issue. However, if network vendors are OK with this limitation, then appropriatey configuring the BFR search space as CSS would be an alternative. |

Email Discussion [101-e-NR-NR_UE_Pow_Sav-WUS-02]

Email Discussions and Proposals

For Issue 1, most companies support the confirmation of working assumption on the minimum time gap values with a couple of companies with partially support.

Proposal 1:

Confirm the WA on the minimum time gap values

- Working assumption: Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows,

| SCS (kHz) | Minimum Time Gap TminimumTimeGap(slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |

Most companies support clarification on the DCI format 2_6 size budget except objection by one company as follows,

- DCI format 2_6 excluding from DCI size budget (3+1) –

  - Nokia, Panasonic, CMCC, vivo, Huawei, HiSilicon, MediaTek, OPPO, ZTE, Intel, APPLE, InterDigital, Sony, ITRI, CATT

- DCI size budget (3+1) remains the same but counting differently outside and within Active Time

  - Qualcomm, Ericsson

- Not change of DCI size budget

  - Samsung

Proposal 2:

DCI format 2_6 exclude from DCI size budget in DCI size alignment

Proposal 3:

TP for TS38.212 excluding DCI format 2_6 in DCI size alignment

-----------------------------------------------------Start of TP of 38.212------------------------------------------------------------------ 7.3.1.0 DCI size alignment *** Unchanged text is omitted *** Step 3: - If both of the following conditions are fulfilled the size alignment procedure is complete - the total number of different DCI sizes configured to monitor, excluding the size of DCI format 2-6, if configured, is no more than 4 for the cell - the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell *** Unchanged text is omitted *** -----------------------------------------------------End of TP of 38.212-----------------------------------------------------------------

Feedbacks from Email discussion

[101-e-NR-NR_UE_Pow_Sav-WUS-02] Email discussion/approval regarding:

- Issue 1:  Whether to confirm Working Assumption of minimum time gap values (section 3.1)

- Issue 4:  DCI size budget for DCI format 2_6 (Section 3.4)

  - Clarification of the specification for DCI size alignment for DCI format 2_6 outside Active Time

by 5/28, with potential TPs by 6/3 – Fangchen (CATT)

Issue 1:  Whether to confirm Working Assumption of minimum time gap values (section 3.1)

- The value of minimum time gap is decoupled with SCell dormancy indication.

- Working assumption: Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows,

| SCS (kHz) | Minimum Time Gap TminimumTimeGap(slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |

| Company | Supporting Issue 1 | Comments |
| --- | --- | --- |
| Nokia | Support | As network needs to account this in the configuration of the DCI format 2_6 (GC-PDCCH) monitoring, it would not be preferable to fragment the UE capabilities any furhter. |
| Panasonic | Support |  |
| CMCC | Support |  |
| Qualcomm | Partially support | For 60kHz in Value 1, we prefer ‘2 slots’. This renders Value 1 equal to the ‘Z’ value (i.e., {1, 1, 2, 2} in TS 38.214, Table 5.3.1-1), which represents the minimum time required for PDCCH processing and reconfiguration of processing elements for subsequent DCI processing. Also, since there are views from some companies that even smaller values, e.g., zero-slot gap, are required, we suggest to consider another option that allows the UE to omit the min time gap capability reporting, which can be implicitly perceived as ‘no restriction’ or ‘zero gap’ at the network. |
| Vivo | Partially support | − For Value 2 , all values is larger than the type 2 BWP switching delay. − For Value 1, some values (15kHz) are larger than type 1 BWP switching delay, but some (60kHz, 120kHz) are not. Although we prefer all the values equal to type 1/type 2 BWP switching time, but we can compromise with the table if consistent design for value 1 exists. for example, value 1 euqals to the value for BWP switching type 1. |
| Huawei, HiSilicon | Support | As we discussed in the last meeting, the value Z of {1,1,2,2} is not a valid argument to have a larger minimum time gap value for Value1 in the working assumption. The value Z in cross-slot scheduling is defined from the start of the slot where the change indication is transmitted to the start of the slot where the indicated minK0 takes effect. Differently, the minimum time gap is defined from the end of the last slot where the UE would be required to monitor DCI format 2_6 to the start of the slot where the ON duration timer would start. Corresponding to the Z value for 60kHz SCS, the corresponding minimum time gap is 1 slot for PDCCH case 1-1. It should be noted that {1, 1, 2, 2} for Z is defined regarding the PDCCH case 1-1. For PDCCH case 1-2 and case2, one more slot willed added. This is actually why we prefer to have different values for PDCCH case 1-1 and PDCCH case 1-2/PDCCH case 2. However, we would be fine to confirm the working assumption as a compromise. |
| MediaTek | Support | We are fine to confirm the working assumption. |
| Samsung | Support |  |
| OPPO | Support |  |
| ZTE | Support | Okay to confirm the working assumption. |
| Ericsson | Support | We prefer to confirm the WA without creating further UE capability signalling. |
| Intel | Support | Although we have some preferences for Value 1 of 60K to be 2 slots and Value 2 of 30K, 60K, 120K to be not exceeding corresponding type 2 BWP switch delay, we are fine to confirm WA as compromise. |
| Apple | Support | We are fine with confirming the working assumption |
| InterDigital | Support |  |
| SONY | Support | While a “value 1” UE has the capability to wake up quickly, operating in a “value 1” mode is likely to consume more power than operating in a “value 2” mode, depending on implementation. Hence, depending on UE state and UE traffic, we think it would be beneficial if the UE could indicate both an MTG capability and a preferred MTG via UE assistance information. In any case, we are OK to confirm the working assumption as it defines the UE capability aspect. |
| ITRI | Support |  |

Issue 4:  DCI size budget for DCI format 2_6 (Section 3.4)

- Clarification of the specification for DCI size alignment for DCI format 2_6 outside Active Time

| Company | Supporting Issue 4 | Comments |
| --- | --- | --- |
| Nokia | Support (excluding DCI format 2_6 from the budget) | As current spesification only consideres the DCI formats monitored in CONNECTED mode, we would not like to split this in two budgets, inside and outside Active Time. We would prefer to simply exclude DCI format 2_6 monitored outside the Active time from the evaluation of the budget limit. |
| Panasonic | Support | It should be clarified that DCI format 2_6 should not be counted in the existing DCI size budget (3+1) |
| CMCC | Support | It should be specified that DCI format 2_6 are not counted in the DCI size budget. |
| Qualcomm | Support | We support separate accounting for DCI sizes within and outside the Active Time. The size of DCI format 2_6 can be counted in the DCI size budget for “outside Active Time”. |
| Vivo | Support | We believe DCI format 2_6 apart from existing DCI size budget is needed. Otherwise, the payload of DCI format 2_6 may need to be padded to align with another DCI format. Thus, this will lead to bad link performance . And the definition of the ‘DCI size budget ‘ is understood regardless UE in active time or outside active time. |
| Huawei, HiSilicon | Support | We are fine to clarify that the DCI format 2_6 is not counted into the DCI size budget (3+1). |
| MediaTek | Support | We think excluding DCI format 2_6 in DCI size budget calculation is beneficial for WUS performance because unnecessary DCI size alignment can be avoided. And we share the similar view as Nokia, there is no need to split the DCI size budget into “outside Active Time” and “inside Active Time”. Simply excluding DCI format 2_6 in budget calculation is sufficient. |
| Samsung | Object | The proposal is against Rel-15 specifications – there is no differentiation for “inside” and “outside” Active Time. The proposal is also irrelevant to UE PS and applies to UEs that do not support UE PS and do not monitor DCI format 2_6 (changes Rel-15 UE behavior). The proposal is unnecessary as the existing specifications are complete. - According to Clause 10.1 of TS38.213, UE counts the DCI format size based on a number of configured PDCCH candidates in respective search space sets, And TS 38.213 also states that “The UE does not monitor PDCCH for detecting DCI format 2_6 during Active Time [11, TS 38.321]”. So, it’s clear that UE counts DCI format 2_6 for DCI sizes only for outside Active Time. - “not counted” is meaningless for a UE implementation. UE needs count DCI format 2_6 if the UE is configured to monitor the DCI format 2_6 according to the respective search space sets. - The size of DCI format 2_6 is configured by sizeDCI-2-6-r16 as described in TS 38.331. There is no DCI size alignment for CSS. |
| OPPO | Support | Share same view with Nokia and MTK. We think excluding DCI format 2_6 in DCI size budget calculation is beneficial for WUS performance because unnecessary DCI size alignment can be avoided. There is no need to split the DCI size budget into “outside Active Time” and “inside Active Time”. Simply excluding DCI format 2_6 in budget calculation is sufficient. |
| ZTE | Support | If DCI format 2_6 is counted in the existing (3+1) size budget, zero-padding is needed for the size alignment, the BLER performance may be impacted. Therefore, we think DCI format 2_6 can be excluded from the size budget. And splitting the size budget into “outside Active Time ” and “within Active Time” is not needeed. |
| Ericsson | Support | This should be clarified to avoid negative impacts on DCI sizes monitored within Active time and to avoid DCI 2_6 size enlargement, due to the text in 38.212, clause 7.3.1.0: Step 3: - If both of the following conditions are fulfilled the size alignment procedure is complete - the total number of different DCI sizes configured to monitor is no more than 4 for the cell - the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell <text omitted> The UE is not expected to handle a configuration that, after applying the above steps, results in - the total number of different DCI sizes configured to monitor is more than 4 for the cell; or Based on discussion from 100bis-e, we updated our proposal to count budgets separately within and outside active time; however we are open to alternative way to resolve this. |
| Intel | Support | We are fine to clarify this in specification |
| Apple | Support | We are fine with excluding DCI format 2_6 from the DCI size budget. We do not see a need to split the size budget between “within” and “outside” the Active Time. |
| InterDigital | Support |  |
| SONY | Support |  |
| ITRI | Support | We share same view with MTK. Excluding DCI format 2_6 in DCI size budget calculation can avoid unnecessary DCI size alignment. For simplicity, excluding DCI format 2_6 in budget calculation is sufficient. |

Email Discussion [101-e-NR-NR_UE_Pow_Sav-WUS-03]

Summary of Email Discussions

All companies agree the intention of TP to Clause 10.3 of TS38.213 for clarification.   There are proposed modification by Samsung and Qualcomm with the number of companies supported as follows,

- Alt 1: Samsung’s TP – Samsung, CMCC, ZTE, OPPO, Huawei, HiSilicon, MediaTek, Apple, Sony, NEC

- Alt 2: Qualcomm’s TP – Qualcomm, vivo, ASUSTek, Apple, Ericsson, Spreadtrum, Intel, InterDigital, Nokia, NSB

  - Alt 2a: removing “when reporting to higher layer” marking in yellow in Alt 2 – AsusTek, Spreadtrum, Intel

A good point was made by one company that we spent a lot of time in RAN1#100bis trying to avoid talking about the drx-onDurationTimer in RAN1 specs (on the understanding that MAC starts the drx-onDurationTimer).      The section in Clause 10.3 is to indicate the information of DCI format 2_6 wakeup bit and SCell dormancy bits.   The action of detection,  miss-detection, or invalid monitor occasion are in the subsequent paragraphs.  Two alternatives of revision receive support from same number of companies.   We need to down select from Alt 1 or Alt 2.

Proposal 1:

TP for clarification of wakeup indication bit (including the CR agreed in RAN1#100b-e in R1-2003177 shown in “purple”

Alt 1:

-----------------------------------------------------Start of TP of 38.213---------------------------------------------------------- 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331] - a PS-RNTI for DCI format 2_6 by ps-RNTI - a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in Clause 10.1 - a payload size for DCI format 2_6 by SizeDCI_2-6 sizeDCI_2-6 - a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6 ps-PositionDCI-2-6, where - the UE may not start the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '0', and - a value ‘0’ of Wake-up indication bit indicates not to wake up - a value ‘1’ of Wake-up indication bit indicates to wake up - the UE starts the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '1' *** Unchanged text is omitted *** If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE detects DCI format 2_6, the physical layer of a UE reports the value of the Wake-up indication bit for the UE to higher layers [14, TS 38.321] for the next long DRX cycle. If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE does not detect DCI format 2_6, if the UE is provided ps-WakeupOrNot, the physical layer of the UE does not report a value of the Wake-up indication bit UE is indicated by ps-WakeupOrNot whether the UE may not start or whether the UE shall start the drx-onDurationTimerto higher layers for the next long DRX cycle. if the UE is not provided ps-WakeupOrNot, the UE may not start Active Time indicated by drx-onDurationTimer for the next DRX cycle If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle the physical layer of the UE shall reports a value of 1 for the Wake-up indication bit to higher layers start the drx-onDurationTimer for the next long DRX cycle. -----------------------------------------------------End of TP of 38.213---------------------------------------------------------------

Alt 2:

-----------------------------------------------------Start of TP of 38.213---------------------------------------------------------- 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331] - a PS-RNTI for DCI format 2_6 by ps-RNTI - a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in Clause 10.1 - a payload size for DCI format 2_6 by SizeDCI_2-6 sizeDCI_2-6 - a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6 ps-PositionDCI-2-6, where - the UE may not start the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '0', and - a ‘0’ value for the Wake-up indication bit, when reported to higher layers, indicates not to start the drx-onDurationTimer for the next long DRX cycle (as described in [14, TS 38.321]) - a ‘1’ value for the Wake-up indication bit, when reported to higher layers, indicates to start the drx-onDurationTimer for the next long DRX cycle (as described in [14, TS 38.321]) - the UE starts the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '1' *** Unchanged text is omitted *** If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE detects DCI format 2_6, the physical layer of a UE reports the value of the Wake-up indication bit for the UE to higher layers [14, TS 38.321] for the next long DRX cycle. If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE does not detect DCI format 2_6, if the UE is provided ps-WakeupOrNot, the physical layer of the UE does not report a value of the Wake-up indication bit UE is indicated by ps-WakeupOrNot whether the UE may not start or whether the UE shall start the drx-onDurationTimerto higher layers for the next long DRX cycle. if the UE is not provided ps-WakeupOrNot, the UE may not start Active Time indicated by drx-onDurationTimer for the next DRX cycle If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle the physical layer of the UE shall reports a value of 1 for the Wake-up indication bit to higher layers start the drx-onDurationTimer for the next long DRX cycle. -----------------------------------------------------End of TP of 38.213---------------------------------------------------------------

No comment was made on the text proposal to update RAN2 parameter TransmitOtherPeriodicCSI

Proposal 2:

TP for Clause 5.1.6.1 and 5.2.2.5 of TS 38.214.

5.1.6.1 CSI-RS reception procedure <omitted text> If the UE is configured with DRX, - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitOtherPeriodicCSI ps-TransmitPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to quantities other than ‘cri-RSRP’ and ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to cri-RSRP when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported. <omitted text>

5.2.2.5 CSI reference resource definition <omitted text> When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI ps-TransmitPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to quantities other than ‘cri-RSRP’ and ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to ‘cri-RSRP’ or ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to ‘cri-RSRP’ if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. <omitted text>

Email discussion of [101-e-NR-NR_UE_Pow_Sav-WUS-03]

[101-e-NR-NR_UE_Pow_Sav-WUS-03] Email discussion/approval the TPs regarding:

- Issue 3: 	Specification alignment and text proposals (Section 3.3)

by 5/27

The proposed TP was agreed to capture the general behavior.  However, the editor of 38.213 deos not capture the TP exactly as those were agreed in [19].  There were discussions to clarify the exact text of the decoding bit value ‘1’ and ‘0’ from DCI format 2_6 associated with Wake-up and no-Wake-up indication.  In addition, RRC parameters ps-PositionDCI-2-6 and sizeDCI-2-6  are updated.

Proposal:

TP to capture value ‘1’ and ‘0’ from DCI format 2_6 associated with Wake-up and no-Wake-up indication

-----------------------------------------------------Start of TP of 38.213----------------------------------------------------------

10.3	PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells

A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331]

-	a PS-RNTI for DCI format 2_6 by ps-RNTI

-	a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in Clause 10.1

-	a payload size for DCI format 2_6 by SizeDCI_2-6 sizeDCI_2-6

-	a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6 ps-PositionDCI-2-6, where

-	the UE may not start the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '0', and

- a value ‘0’ of Wake-up indication bit is the no-Wake-up indication

- a value ‘1’ of Wake-up indication bit is the Wake-up indication

- the UE starts the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '1'

-----------------------------------------------------End of TP of 38.213---------------------------------------------------------------

RAN2 also reach agreements on the update parameters name in [18]

Agreements

1	RAN2 confirms that the flags ps-TransmitPeriodicL1-RSRP and ps-TransmitPeriodicCSI are defined per cell group

2	The flags ps-TransmitPeriodicCSI and ps-TransmitPeriodicL1-RSRP are independent, and it is possible to control UE to report all types of periodic CSI apart from L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

3	The flag ps-TransmitPeriodicCSI is renamed to ps-TransmitOtherPeriodicCSI

TP for Clause 5.1.6.1 and 5.2.2.5 of TS 38.214.

5.1.6.1 CSI-RS reception procedure <omitted text> If the UE is configured with DRX, - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitOtherPeriodicCSI ps-TransmitPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to quantities other than ‘cri-RSRP’ and ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to cri-RSRP when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported. <omitted text>

5.2.2.5 CSI reference resource definition <omitted text> When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI ps-TransmitPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to quantities other than ‘cri-RSRP’ and ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to ‘cri-RSRP’ or ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to ‘cri-RSRP’ if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. <omitted text>

| Company | Supporting Issue 3 | Comments |
| --- | --- | --- |
| Nokia | Support the intention to align. | On the TP for the wake-up indication bit definition, should we align then rest of the Section 10.3? E.g. in last meeting CR: “the physical layer of the UE reports a value of 1 for the Wake-up indication bit to higher layers for the next long DRX cycle.“ Should we rather than talk about wake-up indication bit reporting to the higher layer, we could say for example: “providing/reporting wake-up indication (or no-wake-up) to higher layers“. |
| Panasonic | Support the TP from FL |  |
| Samsung | Support the intention, but need revision on the TP and proposal. | We see the intention to clarify how to interpret the value of wake-up indication bit. But no-Wake-up indication and wake-up indication still look like predefined notations, and they are neither defined in RAN1 nor in RAN2. We suggest following revision for the proposed TP: - a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6 ps-PositionDCI-2-6, where - the UE may not start the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '0', and - a value ‘0’ of Wake-up indication bit is the no-Wake-up indication indicates not to wake up - a value ‘1’ of Wake-up indication bit is the Wake-up indication indicates to wake up and revise the proposal as below: Proposal: TP to capture clarify value ‘1’ and ‘0’ from DCI format 2_6 associated with Wake-up and no-Wake-up indication |
| NEC | Support the intention and agree with revised TP by Samsung. | In addition to the alignment by the TP, we propose the followings: Higher-layer parameter Scell-groups-for-dormancy-outside-active-time needs to be updated. A bitmap is not provided by higher layer for detection of DCI format 2_6 but provided by DCI format 2_6. Therefore some adjustment of description would be needed. According to 38.321, drx-onDurationTimer starts at the beginning of a subframe, i.e. the resolution of start timing is a subframe. We feel “a slot where the drx-onDuarationTimer would start” slightly is not aligned with RAN2 spec. |
| CMCC | Support | We are OK with the TP proposed by Samsung. |
| ZTE | Support | Okay with the TP proposed by Samsung. |
| OPPO | Support | Fine with the modified TP from Samsung. There is really a Gap between RAN1 and RAN2 spec without this TP. |
| Qualcomm | Support with revised TP | We are generally fine with the TP by Samsung. However, the notion of ‘to wake-up’ or ‘not to wake-up’ still seems a bit vague. Although we all know what it means, we should be very accurate to capture it in the specification. In the end, everything is about whether MAC to start the drx-onDurationTimer or not – in TS 38.321, it is captured as: 3> if DCP indication associated with the current DRX Cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or. Therefore, to align with RAN2 spec, we suggest the following TP: - a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6 ps-PositionDCI-2-6, where - a ‘0’ value for the Wake-up indication bit, when reported to higher layers, indicates not to start the drx-onDurationTimer for the next long DRX cycle - a ‘1’ value for the Wake-up indication bit, when reported to higher layers, indicates to start the drx-onDurationTimer for the next long DRX cycle. |
| Huawei, HiSilicon | Support the TP to align the parameter names. Support to further describe the value of wake-up indication with some revision on the proposed TP. | We are fine with Samsung’s revision. |
| MediaTek | Support | We are fine with the TP proposed by Samsung. |
| Vivo | Support of Qualcomm’s revision | We think Qualcomm’s revision is more accurate to capture it in the specification. It accurately elaborates the meaning of ‘not wake-up’ and ‘wake-up’. Therefore we prefer Qualcomm’s version of TP |
| ASUSTeK | Support of Qualcomm’s revision with some addition changes | We agree with Qualcom and Vivo that “to start/not to start timer” is more precise and align with RAN2 formulation. (thoigh we slightly prefer to remove “,when reported to higher layers,”) We think there are some more places requires similar updates “when WUS is not detected” or “when WUS mmonitoring occasion is invalid“, like the following: If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE detects DCI format 2_6, the physical layer of a UE, when reported to higher layers, does not indicate whether to start the drx-onDurationTimer or not reports the value of the Wake-up indication bit for the UE to higher layers [14, TS 38.321] for the next long DRX cycle. […] If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle the physical layer of the UE, when reported to higher layers, indicate to start the drx-onDurationTimer reports a value of 1 for the Wake-up indication bit to higher layers for the next long DRX cycle.. |
| Apple | Support | We are okay with the intention of the TP and feel that Samsung’s TP is okay but Qualcomm’s TP is more technically accurate. |
| Ericsson | Support | OK to align the specification, and the TPs (from Samsung and QC) generally look OK. For QC TP, we think reference to MAC would be helpful (as shown below). - a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6 ps-PositionDCI-2-6, where - a ‘0’ value for the Wake-up indication bit, when reported to higher layers, indicates not to start the drx-onDurationTimer for the next long DRX cycle (as described in [11, TS 38.321]) - a ‘1’ value for the Wake-up indication bit, when reported to higher layers, indicates to start the drx-onDurationTimer for the next long DRX cycle (as described in [11, TS 38.321]) |
| Spreadtrum | Support | We are fine for TP from E///, and “when reported to higher layers” seems not necessary. |
| Intel | Support | Agree with Spreadtrum’s suggestion. “when reported to higher layers” seems not necessary and MAC spec is already cited, so there is no ambiguity. Copied below for easy reference - a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6 ps-PositionDCI-2-6, where - a ‘0’ value for the Wake-up indication bit indicates not to start the drx-onDurationTimer for the next long DRX cycle (as described in [11, TS 38.321]) - a ‘1’ value for the Wake-up indication bit indicates to start the drx-onDurationTimer for the next long DRX cycle (as described in [11, TS 38.321]) |
| InterDigital | Support | We are fine with Qualcomm’s TP. |
| SONY | Support Samsung’s revision | We prefer the Samsung formulation over the Qualcomm formulation. We spent a lot of time in RAN1#100bis trying to avoid talking about the drx-onDurationTimer in RAN1 specs (on the understanding that MAC starts the drx-onDurationTimer). So our preference is that the RAN1 specs just talk about whether the UE is indicated to wake up or not (as per the Samsung proposal) and leave discussion of the drx-onDurationTimer to RAN2. |

Summary of Preparation

The common view is to further discuss 4 issues after the email discussion.    The discussion also include the issue of collision of DCP and RAR addressed by C-RNTI from RAN2 LS R1-2003260.   The reply LS to RAN2 would also be discussed.    During email discussion, the issues of the monitoring RNTI in combination with PS-RNTI outside Active Time in Table 6.2-2 of TS38.202 were raised to be incosnsitent with RAN2 specifcation.  The monitoring RNTI in combination with PS-RNTI would be discussed with collision of DCP and RAR together.  If additional combination were agreed, TP to TS38.202 will be discussed subsequently.

Two email threads are proposed as follows,

[101-e-NR-NR_UE_Pow_Sav-WUS-1] Collison of DCP and RAR

Issue 2:  Collision of RAR and DCP during RAR monitoring window (section 3.2)

  - Discussion on issue raised in RAN2 LS R1-2003260 and reply LS by 5/28.   The discussion will also include information from contributions in AI-5

    - R1-2003353    Discussion on DCP Open Issues       vivo

    - R1-2003484    Draft reply LS on  DCP Open Issues            ZTE

    - R1-2003485    Discussion on collision between DCP and RAR      ZTE

    - R1-2003587    Draft LS reply on DCP open issues  CATT

    - R1-2003852    Draft reply LS on RAN2 DCP open issues   Samsung

    - R1-2004113    Reply LS on RAN2 DCP Open Issues          OPPO

    - R1-2004625    Draft reply LS on RAN2 DCP Open Issues Huawei, HiSilicon

    - R1-2004626  Discussion on the collision between DCP and RAR addressed to C-RNTI            Huawei, HiSilicon

  - UE PDCCH monitoring by other RNTI in combination with PS-RNTI outside Active Time

    - If agreed, whether TP is needed for TS38.202

- [101-e-NR-NR_UE_Pow_Sav-WUS-2]  Specification clarification

Issue 1:  Whether to confirm Working Assumption of minimum time gap values (section 3.1)

Issue 3: 	Specification alignment and text proposals (Section 3.3)

Issue 4:  DCI size budget for DCI format 2_6 (Section 3.4)

  - Clarification of the specification for DCI size alignment for DCI format 2_6 outside Active Time

Email Discussion during Preparation

Issue 1:  Confirm Working Assumption of minimum time gap values (section 3.1)

Issue 2:  RAR is prioritize over DCP during RAR monitoring window (section 3.2)

Issue 3: 	Specification alignment and text proposals (Section 3.3)

Issue 4:  DCI size budget for DCI format 2_6 (Section 3.4)

| Company | Supporting Issues | Comments |
| --- | --- | --- |
| Samsung | Issue 1: OK to discuss. Support confirming WA Issue 2: OK to discuss. Do not support the proposal from FL. Issue 3: OK to discuss. | Issue 4: No need to discuss. No need for any proposal. The arguments in Section 3.4 are incorrect due to the following: - There is no ambiguity on how to account for the total budget of DCI format sizes. UE determines the DCI format size budget based on the search space sets the UE is configured to monitor PDCCH, as described in TS 38.213. - TS 38.213 states that “The UE does not monitor PDCCH for detecting DCI format 2_6 during Active Time [11, TS 38.321]”. So, no need for proposal in counting DCI format 2_6 outside Active Time sepeartely. - “NOT be counted” has no meaning for a UE implementation. We cannot say that DCI format 2_6 is an invisible DCI format for the UE with respect to the number of DCI format sizes the UE has to monitor. |
| ZTE | Issue 1,2,3,4 | We are okay to discuss issue 1,2,3,4 in this meeting. Noted that for issue 2, the discussion is triggered by the LS in AI 5, and we are okay to discuss it in AI 5 or AI 7.2.7.1. In section 3.2, our view is added. |
| Ericsson | Issues 1,2,3,4 |  |
| Intel | Issues 1,2,3,4 | As discussed over email, we believe the combination of RNTIs monitored outside active time can be discussed as part of issue 2. In case issue 2 is discussed in AI 5, we suggest to just add a separate issue in 7.2.7.1 targeting that matter. Issue 5: Combinations of RNTIs monitored outside active time (can be combined with Issue 2 or discussed separately) |
| CMCC | Issues 1,2,3,4 | For issue 2, we also support RAR is prioritize over DCP during RAR monitoring window. In adddtion, we also want to clarify the DCI format 2_6 transimmion behaviour in multiple monitoring occasions. - DCI format 2_6 should be transmitted in only one monitoring occasion or all monitoring occasions. |
| ASUSTeK | Issue 1,2,3,4 | It’s fine to discuss issue 1,2,3,4. Though we are open to discuss issue 4, we think the current text may be sufficient (or with some minor clarification, e.g. DCI size budget only applied for the case within Active Time). |
| Nokia | Issues 1,2,3 and 4 are OK for us. | We agree with Intel, that it would good to clarify 38.202. Current text seems to be contradiction with MAC spesification. |
| MediaTek | Issues 1,2,3,4 | We are also fine to discuss the issue mentioned by Intel to clarify the RNTIs combination monitored outside active time in 38.202. |
| Huawei, HiSilicon | We are fine to discuss Issues 1,2,3,4. | - We think Issue#2 can be discussed in the LS AI. For the combinations with other RNTIs, we think it should be discussed. But we suggest to focus on the case of collision between DCP and RAR addressed to C-RNTI, which is raised in RAN2 LS. For issues newly raised during the preparation phase, it is better to note them this time and give companies time until the next meeting to look into the issue properly. - A number of companies raised the issue to clarify the BWP switching timing considering the DCI format 2_6 triggered dormancy adaptation is different from the BWP switching triggered by scheduling DCI. The same BWP switching due to dormancy adaptation would be transmitted in the DCI format 2_6 which may be transmitted on any one of the multiple occasions. There is no HARQ-ACK transmission for the DCI format 2_6, and therefore the UE and gNB may have different understanding regarding the timing of BWP switching, which means different understanding regarding the interruption time etc. We propose to clearly define/clarify the timing of BWP switching triggered by the DCI format 2_6. Issue#5: clarify/define the start of the BWP swiching triggered by DCI format 2_6. |
| OPPO | Issues 1,2,3,4 | Fine to discuss these issue. For issue 1, we shall discuss how to UE and gNB align BWP switching. For issue 3, we can discuss this issue in RAN 1 but it would be better make the change in RAN2. |
| vivo | Issues 1,2,3,4 We propose also to discuss the starting time of BWP switching after dormancy indication received from DCI format 2_6 | We are fine to discuss the issues 1,2,3,4. For issue 2, (collisoin of DCP and RAR), we see some contributions in AI 5 as well and is not captured in the supporting companies list. It seems some companies think it works well by gNB implementation and some thinks spec impact is needed (with different spec impact consideration). Therefore we kindly suggest to have more discussions during this and next week. In additional , we propose also to discuss the starting time of BWP switching after dormancy indication received from DCI format 2_6 |
| Panasonic | Issue 1, 2, 3, 4 | For Issue 2, we think more discussion is needed rather than starting from the above statement. |

Summary from contributions reviews

Minimum time gap – values

RAN1#99 agreements Agreements:s The minimum time gap between the end of the slot of last DCI format 3_0 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to [3]ms - FFS impact of dormancy/non-dormancy transition RAN1#100-e agreements Agreements PS_offset range from {0.125ms to 15 ms} for all SCS. Agreements The PS_offset resolution is 0.125 ms. Agreements Candidate values for the minimum time gap are specified by RAN1 and shared with RAN4 · Minimum time gap is no more than 3 ms for all SCSs · Two values of minimum time gap for each SCS are proposed as - SCS 15kHz: {TBD, TBD} slots - SCS 30kHz {TBD, TBD} slots - SCS 60kHz {TBD, TBD} slots - SCS 120kHz {TBD, TBD} slots RAN1#100bis-e agreements Proposal 1: - The value of minimum time gap is decoupled with SCell dormancy indication. - Working assumption: Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows, SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24.

During RAN1#100bis-e email discussion,  working assumption was made to have the value of minimum time gap is decoupled with SCell dormancy indication and two values of minimum time gap.

Proposals from companies

- Confirmation of working assumptions – CATT, MediaTek, Samsung, CMCC, OPPO, Ericsson, Nokia

- New values – Huawei, Qualcomm, DoCoMo

  - Huawei

|  | NR Slot length (ms) | Capability Type 1 |  | Capability Type 2 |
| --- | --- | --- | --- | --- |
|  |  | UE not report pdcch-MonitoringAnyOccasions | UE reports pdcch-MonitoringAnyOccasions |  |
| 0 | 1 | 0 | 1 | 1 |
| 1 | 0.5 | 0 | 1 | 2 |
| 2 | 0.25 | 1 |  | 4 |
| 3 | 0.125 | 2 |  | 8 |

  - Qualcomm

    - SCS 15kHz: {1, 3} slots

    - SCS 30kHz: {2, 6} slots

    - SCS 60kHz: {3, 12} slots

    - SCS 120kHz: {6, 24} slots

  - DoCoMo

| SCS (kHz) | Minimum Time Gap TminimumTimeGap(slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 5 |
| 60 | 1 | 9 |
| 120 | 2 | 18 |

Proposal:  Confirm the working assumptions

- The value of minimum time gap is decoupled with SCell dormancy indication.

- Working assumption: Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows,

| SCS (kHz) | Minimum Time Gap TminimumTimeGap (slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |

Collisoin of DCP and RAR

RAN2 LS in [18] would like to ask RAN1 how to resolve the collision between the DCP and the RAR addressed to C-RNTI with different quasi-collocated property during BFR.

1. Collision between DCP and RAR addressed to C-RNTI RAN2 has discussed UE behavior when a DCP monitoring occasion overlaps with the ra-ResponseWindow or msgB-ResponseWindow. RAN2 understanding is that according to current TS 38.213 prioritization rules, if DCP collides with RAR addressed to C-RNTI (e.g. during BFR) and the search spaces are not quasi-collocated, DCP will be prioritized as it is type-3 CSS and thus impacting legacy RAR behavior. RAN2 would like to ask RAN1 the following: - To confirm RAN2 understanding that if DCP and RAR search spaces are not quasi-collocated, a collision between DCP and RAR addressed to C-RNTI will impact legacy RAR handling. - From RAN2 point of view, the understanding is that RAR addressed all RNTIs should be prioritized over DCP by the UE. - RAN2 would like to ask if RAN1 has any concerns with the understanding above? If RAN1 doesn’t have any concerns, what is RAN1 preference on where to capture this behavior e.g. TS 38.213 or in TS 38.321 via a DCP monitoring exception rule similar to overlap with DRX Active time?

The collision of DCP and RAR monitoring were discussed with proposals as follows,

- RAR is prioritized over DCP –

  - gNB implementation with current specification -  vivo, Huawei, Samsung

  - RAR with CRC scrambled by C-RNTI over DCP – CATT, Intel, LG, Ericsson, Nokia,ZTE,CMCC

Proposal:

RAR is prioritize over DCP during RAR monitoring window.  Discuss further

- RAN2 LS reply

- Any RAN1 specification change

Spcification Alignment - Clarification the interaction between PHY and MAC layers

RAN1#100bis-e agreements Proposal 1: L1 procedure of DCI format 2_6 detection - Successful decoding of DCI format 2_6 - L1 sends a positive indication to MAC when the value of wakeup indication bit is “1” - L1 sends a negative indication to MAC when the value of wakeup indication bit is “0” - Miss-detection - all CRC checks fails on DCI format 2_6 - L1 does not send any indication to MAC - All invalid monitoring occasions – following legacy behavior to wakeup - L1 sends a positive indication to MAC TP was approved and captured in [19]

The proposed TP was agreed to capture the general behavior.  However, the editor of 38.213 deos not capture the TP exactly as those were agreed in [19].  There were discussions (ZTE, CATT, AsusTek, Spreadtrum) to clarify the exact text of the decoding bit value ‘1’ and ‘0’ from DCI format 2_6 associated with Wake-up and no-Wake-up indication.  In addition, RRC parameters ps-PositionDCI-2-6 and sizeDCI-2-6  are updated.

Proposal:

TP to capture value ‘1’ and ‘0’ from DCI format 2_6 associated with Wake-up and no-Wake-up indication

-----------------------------------------------------Start of TP of 38.213----------------------------------------------------------

10.3	PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells

A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331]

-	a PS-RNTI for DCI format 2_6 by ps-RNTI

-	a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in Clause 10.1

-	a payload size for DCI format 2_6 by SizeDCI_2-6 sizeDCI_2-6

-	a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6 ps-PositionDCI-2-6, where

-	the UE may not start the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '0', and

- a value ‘0’ of Wake-up indication bit is the no-Wake-up indication

- a value ‘1’ of Wake-up indication bit is the Wake-up indication

- the UE starts the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '1'

-----------------------------------------------------End of TP of 38.213---------------------------------------------------------------

RAN2 also reach agreements on the update parameters name in [18]

Agreements

1	RAN2 confirms that the flags ps-TransmitPeriodicL1-RSRP and ps-TransmitPeriodicCSI are defined per cell group

2	The flags ps-TransmitPeriodicCSI and ps-TransmitPeriodicL1-RSRP are independent, and it is possible to control UE to report all types of periodic CSI apart from L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

3	The flag ps-TransmitPeriodicCSI is renamed to ps-TransmitOtherPeriodicCSI

TP for Clause 5.1.6.1 and 5.2.2.5 of TS 38.214.

5.1.6.1 CSI-RS reception procedure <omitted text> If the UE is configured with DRX, - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitOtherPeriodicCSI ps-TransmitPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to quantities other than ‘cri-RSRP’ and ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to cri-RSRP when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported. <omitted text>

5.2.2.5 CSI reference resource definition <omitted text> When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI ps-TransmitPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to quantities other than ‘cri-RSRP’ and ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to ‘periodic’ and reportQuantity set to ‘cri-RSRP’ or ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to ‘cri-RSRP’ if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. <omitted text>

DCI size budget for DCI format 2_6

Currently the DCI format size budget per UE is determined without separation for the capability between CONNECTED or IDLE/INACTIVE, nor between outside active time and inside active time.  The only separation is done via defining the RNTIs for UE monitoring. There are RNTIs (C-RNTI, P-RNTI, SI-RNTI, RA-RNTI) that UE may need to monitor outside active time which the corresponding DCI formats are accounted in DCI-format size budget.  The intent of the DCI format 2_6 outside active time use would NOT be counted in the total budget of DCI format sizes.   Several companies (vivo, CATT, MediaTek, Ericsson, Qualcomm) have proposals in counting DCI format 2_6 outside Active Time sepeartely and how to account for the total budget of DCI format sizes.

- Proposal: DCI format 2-6 size is aligned to the DCI  size budget outside Active Time.

Others

- The starting time of BWP switching after dormancy indication received from DCI format 2_6  –

  - Inconsistent power saving information (vivo) – no-Wakeup and non-dormant SCell indications for a UE

  - More than one DCI format 2_6 are received (vivo, Huawei) –

  - No DCI format 2_6 monitoring during BWP switching

- Valid moniotoring occasion when more than one avalailable moniotoring occasions in a search space set (LG)

- No restriction on minimum time gap without UE capability feedback (Qualcomm)

Contributions summary and proposals

| vivo[1] | - Proposal 1: PDCCH monitoring for RAR dropping due to different QCL properties can be avoided by proper network implementation. No additional UE behavior need to be specified. - Send LS to RAN2 to inform above decisions. - Proposal 2: UE is not expected to be indicated by PDCCH WUS not to wake up while SCell group is indicated to non-dormancy state. Capture TP in Appendix 1 in R1-2003403 for TS38.213. - Proposal 3: The starting point of BWP switching of Scell dormancy should be defined from the following alternatives, - Alt 1: the starting of BWP switching of Scell dormancy is after the last valid monitoring occasion for DCI format 2-6 - Alt 2: the starting of BWP switching time of Scell dormancy is n slot prior to DRX ON, where n is the Scell dormancy/non-dormancy switching time. - Proposal 4: The size of DCI format 2-6 is not restricted by the existing DCI size budget (3+1) in Rel-15 which is used in Active Time. Capture TP in Appendix 2 in R1-2003043 for TS38.212. |
| --- | --- |
| ZTE [2] | - Proposal 1: The following TP of L1 procedure of the detection of DCI format 2_6 is proposed (wakeup bit). - Proposal 2: Adopt the following TP for Clause 5.1.6.1 and 5.2.2.5 of TS 38.214. (ps-TransmitOtherPeriodicCSI) |
| Huawei, HiSilicon [3] | - Proposal 1: Confirm the working assumption made in RAN1#100b-e that the value of minimum time gap is decoupled with SCell dormancy indication. - Proposal 2: The values of minimum time gap during which the detection of DCI format 2_6 is not required prior to the start of ON Duration Timer are determined per SCS according to the reported capability type of UE as the following table. NR Slot length (ms); Capability Type 1; Capability Type 2. UE not report pdcch-MonitoringAnyOccasions; UE reports pdcch-MonitoringAnyOccasions. 0; 1; 0; 1; 1. 1; 0.5; 0; 1; 2. 2; 0.25; 1; 4. 3; 0.125; 2; 8. - Proposal 3: Further discuss and narrow down between Alt.1, Alt.2 and Alt. 3 for UE behaviour when dormancy indication is configured. - Proposal 4: Adopt TP1 in TS 38.214 to clarify UE behaviour of RRM measurement when DCI format 2_6 is configured. |
| CATT [4] | - Proposal 1: The working assumption is confirmed. Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows, SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. - Proposal 2: RAR indicated by PDCCH with CRC scrambled by C-RNTI or MCS-C-RNTI should be prioritized over DCP by the UE outside Active Time, especially when they have different QCL-TypeD properties. - Proposal 3: The detected value of the Wake-up indication bit from DCI format 2_6 needs to be defined clearly to have ‘1’ associated with Wake-up indication and ‘0’ associated with no-Wake-up indication. - Proposal 4: DCI format 2_6 is not counted in the DCI size alignment. |
| MediaTek[5] | - Proposal 1: Confirm the working assumption for minimum time gap in RAN1 #100b. - The value of minimum time gap is decoupled with SCell dormancy indication. - Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows, SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. - Proposal 2: DCI format 2_6 is not counted in the DCI size budget. - Proposal 3: Adopt the TP in TS 38.213 as follows (prior to). |
| Intel [6] | - Proposal 1: If the DCI format 2_6 monitoring occasion overlaps with ra-ResponseWindow or msgB-ResponseWindow, the UE does not monitor for DCI format 2_6. - Proposal 2: Adopt the following TP |
| Samsung [7] | - Proposal #1: Confirm the working assumption for the values of the minimum time gap - Observation #1: The specifications with respect to the size of DCI format 2_6 are complete. The Rel-15 DCI format size budget is not exceeded. The size of DCI format 2_6 should be counted towards the maximum number of DCI format sizes as for every other DCI format. - Observation #2: There is no need for any specification impact with respect to monitor ‘RAR’ scheduled by a DCI format with a C-RNTI or an MCS-C-RNTI. |
| NEC [8] | - Using the unified wording “PCell or PSCell” is also recommended instead of “PCell or SpCell”. |
| CMCC [9] | - Proposal 1. Support to confirm the working assumption - The value of minimum time gap is decoupled with SCell dormancy indication. - Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows, SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. - Proposal 2. DCI format 2_6 should be transmitted in only one monitoring occasion or all monitoring occasions. |
| Spreadstrum[10] | - Proposal 1: For P-CSI/L1-RSRP measurement/report, consider to adopt TP in Appendix 5.1. - Proposal 2: To clarify the real starting of monitoring is the beginning of the 1st full “duration”, consider to adopt TP in Appendix 5.2. - Proposal 3: To align parameters in RAN2, such as ps-Wakeup, ps-PositionDCI-2-6 and sizeDCI-2-6, consider to adopt TP in Appendix 5.2 and 5.3. |
| LG [11] | - Proposal 1: At least for CFRA, the SS set related to BFR procedure has a higher priority than SS set to be monitored for DCI format 2_6. - Proposal 2: The monitoring occasion which has at least one actually monitored candidate is regarded as a valid monitoring occasion. - Proposal 3: A UE monitors DCI format 2_6 in the first available full duration within a monitoring window. |
| OPPO [12] | - Proposal 1: Confirm the working assumption. - The value of minimum time gap is decoupled with SCell dormancy indication. - Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows, SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. - Proposal 2: when the UE is configured scell dormancy operation, the UE doesn’t need to monitor WUS during its BWP switching delay before DRX ON. |
| AsusTek[13] | - Proposal: RAN1 adopts either TP1 or TP2 to avoid discrepancy on interaction between PHY and MAC on wake up indication |
| Ericsson [14] | - Proposal 1 UE is not required to monitor DCI 2-6 in monitoring occasions that overlaps with the ra-ResponseWindow or beam-failure recovery procedure. - Proposal 2 RAN1 to provide the following response to RAN2 LS (R1-2003260) - RAN1 does not have a concern with the RAN2 understanding and asks RAN2 to capture in 38.321 that the UE wakes up in an upcoming ON duration when a DCI 2-6 monitoring occasion overlaps the ra-ResponseWindow or beam-failure recovery procedure. - Proposal 3 RAN1 to confirm the minimum time gap values in the WA as well as the decoupling of minimum time gap values from Scell dormancy indication. - Proposal 4 DCI sizes in the budget calculation are counted separately for the cases of within Active time and outside Active time. - Proposal 5 Adopt TP1 for 38.212 subclause 7.3.1.0 DCI sizes in the budget calculation are counted separately for the cases of within Active time and outside Active time. |
| NTT DoCoMo [15] | - Proposal 1: Two candidate values of UE reported minimum time gap for each SCS are: SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 5. 60; 1; 9. 120; 2; 18. |
| Qualcomm[16] | - Proposal 1: For consistency, the lower set of values for the minimum time gap should not be smaller than the Z values (i.e., {1, 1, 2, 2}) for the application delay of the minimum scheduling offset restriction. - Proposal 2: For the reported UE capability on the minimum time gap, the following sets of values can be considered: - SCS 15kHz: {1, 3} slots - SCS 30kHz: {2, 6} slots - SCS 60kHz: {3, 12} slots - SCS 120kHz: {6, 24} slots - Proposal 3: For the reported UE capability on the minimum time gap, adding a third option of ‘No restriction’ can be considered. The third option can be implicitly signaled by omitting the capability report for the minimum time gap. - Proposal 4: For the aggregation level and the number of PDCCH candidates for DCI format 2_6, reuse those for DCI format 2_0. - Proposal 5: Separate sets of DCI format sizes are accounted for within and outside the Active Time. The size of DCI format 2_6 is counted in the DCI size budget for “outside Active Time”. |
| Nokia, NSB [17] | - Proposal 1: When monitoring occasions of DCI format 2_6 overlaps with PDCCH monitoring in search space given recoverySearchSpaceId as described in Section 6 of 38.213, UE should follow the legacy DRX operation. - Proposal 2: Send a LS to RAN2 indicating that RAN1 agrees that there could be impact to the legacy RAR monitoring configuration and that the behaviour related to DCP and RAR monitoring could be captured to RAN2 specification. - Proposal 3: Confirm the working assumption take in the RAN1#100bis-e: working assumption: - The value of minimum time gap is decoupled with SCell dormancy indication. - Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows, SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. |

Reference

- R1-2003403		Maintenance of PDCCH-based power saving signal	vivo

- R1-2003486		Remaining issues on WUS PDCCH	ZTE

- R1-2003518		Remaining issues on PDCCH based power saving	Huawei, HiSilicon

- R1-2003630		Remaining issues on the Power Saving Signals/Channels	CATT

- R1-2003664		Remaining issues on PDCCH-based power saving signal	MediaTek Inc.

- R1-2003745		Remaining details of PDCCH-based power saving signal/channel	Intel Corporation

- R1-2003884		Remaining issues for PDCCH-based power saving signal	Samsung

- R1-2003924		TP for further alignment with RAN2 specifications	NEC

- R1-2003957		Remaining issues on power saving signal/channel	CMCC

- R1-2003999		Clarification on power saving signal	Spreadtrum Communications

- R1-2004025		Remaining issues on PDCCH-based power saving signal/channel	LG Electronics

- R1-2004101		Remaining issues for Power saving signal	OPPO

- R1-2004320		Wake up indication for ON duration timer	ASUSTeK

- R1-2004357		Remaining issues for WUS	Ericsson

- R1-2004398		Maintenance for PDCCH-based power saving signal/channel	NTT DOCOMO, INC.

- R1-2004467		Remainign issues in power saving signal/channel	Qualcomm Incorporated

- R1-2004577		On open issues related to DCI format 2_6	Nokia, Nokia Shanghai Bell

- R1-2003260		LS on RAN2 DCP Open Issues, RAN2		 InterDigital.

- R1-2003177		TR38.213 CR 0105 Corrections on UE power savings	 Samsung