3GPP TSG-RAN WG1 Meeting #101	R1-2004937

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	8.4.1.1

Source:	Ericsson

Title:	Simulation Parameters and Initial Results for FR1

Document for:	Decision

# 1	Introduction

In this paper, coverage evaluations for a set of channels in some different scenarios with frequencies in Frequency Range 1 (FR1) are presented. The set is selected to include channels used for the initial access procedure as well as when sending data and considers both uplink as well as downlink and allows to identify potential bottlenecks in terms of channels that significantly limit overall coverage.

For low band, represented by a carrier frequency of 700MHz, the studied scenarios are Low Mobility Large Cell (LMLC), Extreme Rural, and Rural Macro (with and inter-site distance of 7km to stress coverage). A deployment with conventional sector antennas is used.

For mid-band, represented by a carrier frequency of 4GHz, the studied scenarios are Urban Macro and Rural Macro. In both cases the ISD is extended to stress coverage and identify bottlenecks. A system deployment with an Advanced Antenna System (AAS) and user specific beamforming is used.

As discussed in [1] the evaluations involve link simulations to determine the link quality signal-to-noise and interference ratio (SINR) that is required to reach acceptable error rates, and system level simulations that determine coverage, or probability of reaching, those SINRs. Link level assumptions and results are presented in Section 2, and system level models are presented in Section 3.

This contribution is a revision of R1-2004352. We add Section 4 that describes how to determine non-ideal antenna gain and interference margin parameters for use in link budget calculations.

# 2	Link level models and results

For FR1, deployments using subcarrier spacings 15kHz as well as subcarriers spacings 30kHz are foreseen. Furthermore, we also foresee deployments using FDD as well as TDD as well as deployments with advanced antenna system as well as without. Even though it is possible to come up with very many different permutations, the proposal is to consider two different baseline cases for FR1, with carrier frequencies  selected based on [2].

Proposal:

- Consider two configurations as shown in Table 1, a 2x10MHz FDD with 15kHz subcarrier spacing at 700 MHz with 2T/2R at base station and 1T/2R at terminal and a 1x100MHz 3DL1UL TDD with 30kHz subcarrier spacing at 4GHz using up to 32T/32R at base station and 1T/4R at terminal.

| - Carrier frequency | - 700MHz | - 4GHz |
| --- | --- | --- |
| - Duplex | - FDD | - TDD - 3DL:1UL |
| - Bandwidth | - 2x10MHz (52PRBs) | - 1x100MHz (273PRBs) |
| - Subcarrier spacing | - 15kHz | - 30kHz |
| - gNb antenna config | - 2T2R | - 32T32R - Single panel with 8x8 dual polarized element pairs, and element separations (dH, dV) = (0.5, 0.8)λ. Panel is partitioned into 8x2 dual polarized subarrays, each with 4x1 element pairs connected to a pair of TRXs, one for each polarization. - (8x8x2x2x8) |
| - UE antenna config | - 1T2R | - 1T4R - Single panel with 1x2 dual polarized element pairs separated 0.5λ - *Above is Ericsson parameters submitted to 37.910 |
| - Waveform | - DL: OFDM, UL: DFT-S-OFDM |  |

Table 1 Carrier configurations for FR1

- As proposed in [1], the antenna capabilities are modeled partly on link level and partly on system level. Transmit and receive diversity as well as spatial multiplexing operate on fast fading time scale, and it appears natural to model them on link level. To strike a balance between complexity and accuracy while still allowing to account for up to two-layer spatial multiplexing as well as first order benefits of transmit diversity with dual polarized arrays, two transmit antennas is proposed to be used for the downlink whereas a single transmit antenna is consider for the uplink. On the receiver side, two or four antennas should be considered depending on the band as well as link direction in order to capture receiver diversity benefits.

Proposal:

- For the baseline link level simulations, consider 2T for downlink 1T for uplink, whereas 2R and 4R are relevant for both uplink and downlink at 700MHz and 4GHz respectively, see also Table 2.

| - Carrier frequency | - 700MHz | - 4GHz |
| --- | --- | --- |
| - gNB | - 2T2R | - 2T4R |
| - UE | - 1T2R | - 1T4R |

Table 2 Baseline link level simulation antenna configurations

- The link level performance does not only depend on carrier frequency and antenna configurations, assumptions are also needed when it comes to the power delay profile. For simplicity, the so-called tapped delay line channel models serve as a good starting point.

- Terminals with coverage problems are likely to be in non-LOS propagation conditions which implies that TDL A- TDL-B or TDL- C are candidates whereas TDL-E and TDL-F are not.

- When it comes spatial correlation, there are a few different options defined in [3], and the cases with low or medium are good starting points. In order not to over-estimate diversity benefits, the case with medium correlation for dual polarized antennas at both the gNb and the UE is a good starting point.

- Terminals at different positions experience different time dispersion. Based on the tables in Section 7.7.3 [4]  we  note that the median RMS delay spread is 10-7.43≈37ns in the rural macro scenario and 10-6.62≈239ns  for indoor users in the urban macro scenario.   Based on this it is proposed to consider a RMS delays spread of 100ns as a starting point, and later on complement with results for 30ns for 700MHz low band and 300ns for 4GHz,

- Terminals at different positions may move with different speed, and examples of commonly assumed speeds are 3,30, 120km/h and 300km/h.

- It is certainly possible to construct many different parameter combinations. However, as starting points, the following proposals, as summarized below in Table 3, are made.

Proposal:

- For the link level simulations, initially consider TDL-A with medium spatial correlation, RMS delay spread 100ns. Later consider also 30ns delay spread and for the 700MHz case and 300ns for the 4GHz targeting rural macro and urban micro deployments (where a majority of users are indoors). Initially consider terminal speed of 3km/h and later consider also other higher speeds like 30km/h and 120km/h. This summarized in Table 3.

| - Carrier frequency | - 700MHz | - 4GHz |
| --- | --- | --- |
| - Model | - TDL-A |  |
| - Correlation | - medium correlation |  |
| - RMS delay spread | - [30], 100 ns | - 100, [300] |
| - Terminal speed | - 3, [30, 120] | - 3, [30] |

Table 3 Link level simulation power delay profiles

## 2.1	FDD 700MHz

- The link level performance does not only depend on carrier frequency and antenna configurations, assumptions are also needed when it comes to the power delay profile. For simplicity, the so-called tapped delay line channel models serve as a good starting point. As mentioned in the introduction above, in order to find the bottle necks signaling both during initial access as well as uplink and downlink transmission in connected mode will be considered. Not all signals and channels will be explicitly included. For example, system information as well as paging are not explicitly included. They use the PDCCH and PDSCH and are therefore implicitly included. Furthermore, in addition to the commonly assumed error rate of 1%, an error rate of 10% is also considered, meaning that for a UE in poor coverage, an increase in for example access time or latency could be acceptable.

- Below in Table 4 an initial set of channels as well as assumptions and initial results are outlined, and the following proposal is made

Proposal:

- For the coverage analysis for low band, consider, the channels and assumptions in Table 4.

Table 4 Link level assumptions and SNR requirements for 700MHz

| Channel/Signal | Assumptions | SNR [dB] for 10%, 1% error rate |
| --- | --- | --- |
| Initial Access |  |  |
| SSB (P/S-SS and PBCH) | SSB transmitted with 20ms periodicity 10% and 1% residual BLER after 4 retransmissions within MIB TTI of 80ms, UE is not assumed to know the SS/PBCH block index, wideband precoder, cycled for different transmissions | -9.8, -7.1 |
| MSG1 (PRACH) | Format 0 10% and 1% missed detection at 0.1% false alarm probability, with maximum timing estimation error 50% of the normal CP length and 64 preambles per cell Initial timing offset uniformly distributed in [0, 23 µs] corresponding to 6 km intersite distance (ISD). | -13.9, -8.7 |
| MSG2 RAR (PDCCH+PDSCH) | PDSCH with 8 bytes payload, MCS 0 with transport block scale factor 0.25, 12 PRBs, 3 DMRS symbol, 9 symbols with PDSCH (and 2 symbols reserved for PDCCH) precoder cycling | -9, -6.3 |
| MSG3 RRC request (PDCCH+PUSCH) | PUSCH with 7 bytes payload, MCS 0, 2 PRBs, 3 DMRS symbols 11 symbols with PUSCH, With 7 re-transmissions (8 attempts), using different frequency for different attempts. No PDCCH errors. | -10.0, -7.4 |
| Uplink and Downlink Data Transmission |  |  |
| DL assignment or UL Grant (PDCCH) | PDCCH using aggregation level 16 and DCI format 0_0 or 1_0 with payload of 39bits+24bits CRC CORESET 48 PRBs, 2 symbols, non-interleaved mapping, precoder cycling | -9.2,-6.9 |
| DL data (PDSCH) | Link and rank adaption based on 20 slot wideband CSI feedback periodicity and HARQ with up to three retransmissions. 52 PRBs, 2 symbols with DMRS, PDSCH and DMRS mapped to 12 symbols (2 symbols reserved for PDCCH), overhead due to CSI-RS and TRS with 20ms period | depends on rate |
| ACK/NACK (PUCCH) | Format 1 with 1bit, 14 symbols long with 7 DMRS and frequency hopping Pr(DTX to ACK) <=1%, Pr(NACK to ACK) <=0.1%, Pr(ACK error) <=1% or 10% | -8.1, -4.4 |
| CSI feedback (PUSCH or PUCCH) | 5+2 bits for wideband CSI feedback for 2Tx 1 PRB, no HARQ ACK/NACKs PUCCH format 3 with 4 DMRS and frequency hopping PUSCH without multiplexing with data on PUSCH and no frequency hopping | PUSCH: -6.0, -0.4 PUCCH: -5.0, -1.6 |
| UL data (PUSCH) | Link and bandwidth adaption based on DMRS and HARQ with up to three retransmissions. Up to 52 PRBs, 2 symbols with DMRS, PDSCH and DMRS mapped to 14 symbols and no UCI overhead included | depends on rate |

## 2.2	4GHz TDD

For the second carrier configuration the same signals and channels as for the first carrier configuration is considered. Stand-alone operation is considered both for the 700MHz FDD carrier as well as for the 4GHz TDD carrier. This means that payloads for the initial access messages are similar. However, since the subcarrier spacing is different, so is that the slot length and for this reason a different PRACH configuration is needed. Even though the bandwidth is different CORESET with duration two symbols is used, but this assumption is not expected to be critical. Additional, since more antennas are considered, the CSI feedback payloads are also slightly different.

Below in Table 5 an initial set of channels as well as assumptions and initial results are outlined, and the following proposal is made.

Proposal:

- For the coverage analysis for mid band, consider, the channels and assumptions in Table 5.

Table 5 Link level assumptions and SNR requirements for 4GHz

| Channel/Signal | Assumptions | SNR [dB] for 10%, 1% error rate |
| --- | --- | --- |
| Initial Access |  |  |
| SSB (P/S-SS and PBCH) | SSB transmitted with 20ms periodicity residual BLER after 4 retransmissions within MIB TTI of 80ms, UE is not assumed to know the SS/PBCH block index | -12.9, -11.4 |
| MSG1 (PRACH) | Format B4 (12 symbols) 10% and 1% missed detection at 0.1% false alarm probability, with maximum timing estimation error 50% of the normal CP length and 64 preambles per cell Initial timing offset uniformly distributed in [0, 1.9 µs] for 500 m ISD and [0, 6.7 µs] 1732 m ISD | -20.0, -17.0 for 500 m ISD -19.1, -16.1 for 1732 m ISD |
| MSG2 RAR (PDCCH+PDSCH) | PDSCH with 8 bytes payload, MCS 0 with transport block scale factor 0.25, 12 PRBs, 3 DMRS symbol, 9 symbols with PDSCH (and 2 symbols reserved for PDCCH) precoder cycling | -12, -9.7 |
| MSG3 RRC request (PDCCH+PUSCH) | PUSCH with 7 bytes payload, MCS 0, 2 PRBs, 3 DMRS symbols 11 symbols with PUSCH, With 7 re-transmissions (8 attempts), using different frequency for different attempts. No PDCCH errors. | -13.9, -12.4 |
| Uplink and Downlink Data Transmission |  |  |
| DL assignment or UL Grant (PDCCH) | PDCCH using aggregation level 16 and DCI format 0_0 or 1_0 with payload of 44bits+24bits CRC CORESET 273 PRBs, 2 symbols, non-interleaved mapping, precoder cycling | -11.3, -9.5 |
| DL data (PDSCH) | Link and rank adaption based on 20 slot wideband CSI feedback periodicity and HARQ with up to three retransmissions. 273 PRBs, 2 symbols with DMRS, PDSCH and DMRS mapped to 12 symbols (2 symbols reserved for PDCCH), overhead due to CSI-RS and TRS with 20ms period | depends on rate |
| ACK/NACK (PUCCH) | PUCCH Format 3 using 14 symbols, 1 PRB, 4 DMRS and frequency hopping 4 bits payload for ACK/NACKS (three bits for 3DL:1UL TDD asymmetry and another bit for scheduling request) Pr(DTX to ACK) <=1%, Pr(NACK to ACK) <=0.1%, Pr(ACK error) <=1% or 10% | -9.1, -5.4dB |
| CSI feedback PUCCH or PUSCH | 6 bits CSI part 1 (RI+CQI), 10 bits CSI part 2 (PMI1+PMI2) wideband reporting for type I feedback for an 8x2 port layout and up to rank four 1 PRB, no HARQ ACK/NACKs PUCCH format 3 with 4 DMRS, with frequency hopping PUSCH without multiplexing with data on PUSCH and no frequency hopping | PUCCH -6.6, -3.3 PUSCH -6.2, -1.5 |
| UL data (PUSCH) | Link and bandwidth adaption based on DMRS and HARQ with up to three retransmissions. Up to 273 PRBs, 2 symbols with DMRS, PDSCH and DMRS mapped to 14 symbols and no UCI overhead included | depends on rate |

# 3. System-level models and results

This chapter first presents system-level models and assumptions for low-band and mid-band, and then coverage results for different scenarios.

## 3.1	Models and assumptions

In Table 6 assumptions for desired and interfering signals for 700MHz are presented. In this case all channels are transmitted and received at the base station using regular sector beams. It is further assumed that SSB transmission is concurrent between cells, so that an SSB transmission is interfered with another SSB transmission, and that the interferer activity is 100%. For other downlink channels it is assumed that they are interfered by PDSCH with 50% activity. For the uplink it is assumed that PRACH and PUSCH are interfered by PUSCH with an activity of 50%, and that PUCCH is interfered by PUCCH. For the uplink channels power control with full pathloss compensation (alpha = 1) is used with SNR targets (P0 – noise per RB) according to Table 6. The noise figures are 5dB in uplink and 7dB downlink.

Table 6. Desired and interfering signal assumptions for 700MHz.

| Channel | Desired signal beam | Interfering signal | Interferer activity | Power control SNR target (P0) |
| --- | --- | --- | --- | --- |
| SSB | sector | SSB | 1.0 |  |
| Msg2 Pdcch | sector | PDSCH | 0.5 |  |
| Msg2 Pdsch | sector | PDSCH | 0.5 |  |
| PDCCH | Sector | PDSCH | 0.5 |  |
| PDSCH data | sector | PDSCH | 0.5 |  |
| Msg 1 PRACH | sector | PUSCH | 0.5 | 3dB (P0=-116) |
| PUCCH | sector | PUCCH | 0.5 | 3dB (P0=-113) |
| Msg3 PUSCH | sector | PUSCH | 0.5 | 10dB (P0=-106) |
| CSI PUSCH | sector | PUSCH | 0.5 | 10dB (P0=-106) |
| PUSCH Data | sector | PUSCH | 0.5 | 10dB (P0=-106) |

Table 7 presents the same information for 4GHz. In this case an AAS with 8x8 dual polarized element pairs portioned into an 8x2 array of 4x1 subarrays, denoted (8x8x2x2x8). The SSB is swept in four different horizontal directions, and the best direction (strongest received signal) is used for the coverage calculations. It is further assumed that the PRACH is received using the same beam, and also that the following PUCCH and PUSCH transmission for message 2 use this beam. Following transmissions are assumed to use based on Maximum Ratio Transmission (MRT) for downlink and Maximum Ratio Combination (MRC) for uplink. The interfering channels, interferer activity and power control targets are the same as for 700MHz.

Table 7. Desired and interfering signal assumptions for 4GHz.

| Channel | Desired signal beam | Interfering signal | Interferer activity | Power control SNR target |
| --- | --- | --- | --- | --- |
| SSB | GoB over four horiz directions | ssb | 1.0 |  |
| Msg2 Pdcch | GoB over four horiz directions | PDSCH | 0.5 |  |
| Msg2 Pdsch | GoB over four horiz directions | PDSCH | 0.5 |  |
| PDCCH | MRT | PDSCH | 0.5 |  |
| PDSCH data | MRT | PDSCH | 0.5 |  |
| Msg 1 PRACH | GoB over four horiz directions | PUSCH | 0.5 | 3dB |
| PUCCH | MRC | PUCCH | 0.5 | 3dB |
| Msg3 PUSCH | MRC | PUSCH | 0.5 | 10dB |
| CSI PUSCH | MRC | PUSCH | 0.5 | 10dB |
| PUSCH Data | MRC | PUSCH | 0.5 | 10dB |

Some additional parameters for 700MHz and 4GHz are presented in Table 8.

Table 8. Additional parameters for 700MHz and 4GHz.

| Parameter | 700MHz | 4GHz |
| --- | --- | --- |
| Duplex and bandwidth | FDD 2x10MHz | TDD 100MHz, 3:1 (DDSU) |
| BS antenna | Sector antenna (8x1x2), 17dBi gain, scenario specific tilt | AAS (8x8x2x2x8), 26dBi gain, scenario specific tilt |
| UE antenna | Isotropic, 0dBi | Isotropic, 0dBi |
| BS power | 43dBm | 53dBm |
| UE power | 23dBm | 23dBm |
| BS noise figure | 5dB | 5dB |
| UE noise figure | 7dB | 7dB |

## 3.2	Rural Macro, ISD 7km, 700MHz, 2x10MHz FDD

This modified version of the Rural Macro scenario, with the ISD adjusted to 7km, is selected a create a challenging scenario from a coverage perspective, and stress potential bottlenecks.

Figure 1 shows SINR distributions and coverage for different channels. It is seen that the SINR differs between channels because of the differences in transmit power, antenna gains, and interference. For each channel the SINR requirement (for two levels of quality requirements, 10% and 1% error) is marked with a star in the SINR distribution, and the relative coverage (or outage) can be read at the corresponding value on the vertical axis. Note that the legend only lists results for 10% BLER for control information and the lower data rate for PDSCH and PUSCH.  The SINR requirements and outage levels for the lower of the quality requirements are also included in the figure legend. Figure 2 shows the same information as a bar graph with the coverage per channel for both low and high quality requirements.

Looking at the coverage for the different channels it is seen that PUSCH data has the worst coverage. This is followed by the uplink control channels, which are relatively well-balanced, but where CSI on PUCCH is slightly worse than the others. Downlink control channels and data are more robust.

As discussed in [1], due to varying interference, there is no strict relationship between the control channel quality and isotropic loss or coupling loss. Neglecting the interference variation, it is however possible to associate each channel with the isotropic (or coupling) loss observed at the percentile corresponding to the relative coverage of the control channel. For example, a channel with 95% coverage is associated with the isotropic loss at the 95th percentile. This does not affect the relative order of the channels and which constitute bottlenecks, but allows a simple assessment of maximum isotropic and coupling loss.

Figure 3 shows the isotropic loss corresponding to the coverage for each of the channels. The order of the channels in terms of performance is the same as in Figure 2.  Figure 4 shows similar results in terms of coupling gain. Here an unloaded network is studied in order to match the setting for the 3GPP target in 38.913 [5] with zero interference margin. It is seen that the 3GPP target from 3GPP TR 38.913 of supporting 143dB coupling loss is met.

In summary, we make the following observations.

Observations:

- For low-band, uplink data with datarates exceeding 30kbps has the worst coverage.

- For low-band, uplink control channels are quite well balanced. There is no obvious bottleneck.

- For low-band, downlink control channels and data are quite well balanced and have better coverage than uplink channels.

- The 3GPP target from 38.913 on extreme coverage is met.

Figure 1. SINR distributions and coverage for different channels for Rural Macro with 7km ISD at 700MHz.

Figure 2. Coverage in terms of fraction of users with acceptable quality for Rural Macro with 7km ISD at 700MHz Red bars represent 1% error rates and blue bars 10% error rates for control, and higher or lower data rates for PDSCH and PUSCH.

Figure 3. Estimated maximum isotropic loss for different channels for Rural Macro with 7km ISD at 700MHz. Red bars represent 1% error rates and blue bars 10% error rates for control, and higher or lower data rates for PDSCH and PUSCH.

Figure 4. Estimated maximum coupling loss for different channels for Rural Macro with 7km ISD at 700MHz. Red bars represent 1% error rates and blue bars 10% error rates for control, and higher or lower data rates for PDSCH and PUSCH.

## 3.3	LMLC, ISD 6km, 700MHz, 2x10MHz FDD

Figure 5 shows SINR distributions and coverage for different channels for the LMLC scenario. It is seen that the SINR differs between channels because of the differences in transmit power, antenna gains, and interference. For each channel the SINR requirement is marked with a star in the SINR distribution, and the relative coverage (or outage) can be read at the corresponding value on the vertical axis. The SINR requirements and outage levels for the lower quality requirements are also included in the figure legend. The same information is also presented in Figure 6 as bars indicating the relative coverage per channel for the different quality requirements. Red bars in the figures represent 1% error rates and blue bars 10% error rates for control channels, and higher or lower data rates for PDSCH and PUSCH.

It is seen that very good coverage is achieved. The channel with the worst coverage is again PUSCH data. This is followed by uplink control.

Figure 7 shows the isotropic loss corresponding to the coverage for each of the channels.

Observation:

- Very good coverage is achieved in the LMLC scenario. Uplink data is limiting for data rates exceeding 30kbps. Other potential bottlenecks are not so evident in this scenario.

Figure 5. SINR distributions and coverage for different channels for LMLC at 700MHz.

Figure 6. Coverage in terms of fraction of users with acceptable quality for LMLC at 700MHz.

Figure 7. Estimated maximum isotropic loss for different channels for LMLC at 700MHz. Note that this is partly limited by the scenario studied, with quite good coverage.

## 3.4	Extreme Long Range, ISD 173km, 700MHz, 2x10MHz FDD

Figure 8 shows SINR distributions and coverage for the different channels in the extreme long range scenario. Relative coverage is also summarized in Figure 9. As in the figures above, red bars represent 1% error rates and blue bars 10% error rates for control channels, and higher or lower data rates for PDSCH and PUSCH.

It is seen that good coverage is achieved for the low quality requirements. For the high quality requirements worse coverage is observed for some of the channel. The reason for this is the poor cell-isolation, which in turn is a result of the propagation model used for this scenario (free-space plus a constant excess loss).

Observation:

- Good coverage can be achieved in the Extreme Long Range scenario.

Figure 8. SINR distributions and coverage for different channels for Extreme Long Range at 700MHz.

Figure 9. Coverage in terms of fraction of users with acceptable quality for Extreme Long Range at 700MHz.

Figure 10. Estimated maximum isotropic loss for different channels for Extreme Long Range at 700MHz.   Note that this is partly limited by the scenario studied, with quite good coverage.

## 3.5	Urban Macro, ISD 700m, 4GHz, 100MHz TDD

This modified version of the Urban Macro scenario, with the ISD adjusted to 700m, is selected a create a challenging scenario from a coverage perspective at 4GHz, and stress potential bottlenecks.

Figure 11 shows SINR distributions and coverage for different channels. The relative coverage per channel is also summarized in Figure 12, and an assessment of the maximum isotropic loss per channel is presented in Figure 13. As in the figures above, red bars represent 1% error rates and blue bars 10% error rates for control channels, and higher or lower data rates for PDSCH and PUSCH.

It is seen that 1Mbps PUSCH data has the worst coverage. This is followed by the uplink control channels, which are relatively well-balanced, but where CSI on PUCCH or PUSCH is slightly worse than the others. Downlink control channels and data are more robust. Message 2, for which limited CSI is assumed to be available for beamforming, is worse than the others, but better than uplink control.

Observations:

- For mid-band, uplink data (PUSCH 1 or 2Mbps) has the worst coverage.

- For mid-band, uplink control channels are quite well balanced. CSI on PUCCH or PUSCH are slightly worse than others.

- For mid-band, downlink control channels and data (PDSCH 10Mbps) are quite well balanced and have better coverage than uplink channels. Message 2, for which limited CSI is assumed to be available for beamforming, is worse than the others, but better than uplink control.

Figure 11. SINR distributions and coverage for different channels for Urban Macro with 700m ISD at 4GHz.

Figure 12. Coverage in terms of fraction of users with acceptable quality for Urban Macro with 700m ISD at 4GHz.

Figure 13. Estimated maximum isotropic loss for different channels for Urban Macro with 700m ISD at 4GHz.

## 3.6	Rural Macro, ISD 3km, 4GHz, 100MHz TDD

Like the Urban Macro scenario with 700m ISD, this modified version of the Rural Macro scenario, with the ISD adjusted to 3km, is selected a create a challenging scenario from a coverage perspective at 4GHz, and stress potential bottlenecks. One intention with studying both these scenarios is to see if the bottlenecks are the same across the scenarios.

Figure 14 shows SINR distributions and coverage for different channels. Figure 15 summarizes the relative coverage per channel and Figure 16 shows an assessment of isotropic loss. As in the figures above, red bars represent 1% error rates and blue bars 10% error rates for control channels, and higher or lower data rates for PDSCH and PUSCH.  It is seen that 1Mbps PUSCH data has the worst coverage. This is followed by uplink control channels. Downlink control and data are more robust.

Observation:

- The bottlenecks in the rural macro scenario are the same as in the urban macro scenario. This points to that the bottlenecks could be consistent across scenarios.

Figure 14. SINR distributions and coverage for different channels for Rural Macro with 3km ISD at 4GHz.

Figure 15. Coverage in terms of fraction of users with acceptable quality for Rural Macro with 3km ISD at 4GHz.

Figure 16. Estimated maximum isotropic loss for different channels for Rural Macro with 3km ISD at 4GHz.

## 3.7	Urban Macro, ISD 500m, 4GHz, 100MHz TDD

For completeness, also an Urban Macro scenario with the default ISD of 500m is studied. Figure 17 shows SINR distributions and coverage for different channels. Figure 18 summarized the relative coverage per channel and Figure 19 shows an assessment of isotropic loss.  As above, red bars represent 1% error rates and blue bars 10% error rates for control channels, and higher or lower data rates for PDSCH and PUSCH.

As expected, it is seen that coverage is better than in the case with 700m ISD.

It is seen that 1Mbps PUSCH data has the worst coverage. This is followed by uplink control channels. Downlink control and data are more robust.

Observation:

- For mid-band, the Urban Macro scenario with 500m ISD has quite good coverage. Uplink data is limiting followed by uplink control. Bottlenecks are not as evident as in more challenging scenarios.

Figure 17. SINR distributions and coverage for different channels for Urban Macro at4GHz.

Figure 18. Coverage in terms of fraction of users with acceptable quality for Urban Macro at 4GHz.

Figure 19. Estimated maximum isotropic loss for different channels for Urban Macro at 4GHz.

## 3.8	Rural Macro, ISD 1.7km, 4GHz, 100MHz TDD

For completeness, also a Rural Macro scenario with the default ISD of 1732m is studied.

Figure 20 shows SINR distributions and coverage for different channels. Figure 21 summarizes the relative coverage per channel and Figure 22 shows an assessment of isotropic loss. As in the figures above, red bars represent 1% error rates and blue bars 10% error rates for control channels, and higher or lower data rates for PDSCH and PUSCH.

As expected, it is seen that coverage is better than in the case with 3km ISD. It is further seen that 1Mbps PUSCH data has the worst coverage. This is followed by uplink control channels. Downlink control and data are more robust.

Figure 20. SINR distributions and coverage for different channels for Rural Macro at 4GHz.

Figure 21. Coverage in terms of fraction of users with acceptable quality for Rural Macro at 4GHz.

Figure 22. Estimated maximum isotropic loss for different channels for Rural Macro at 4GHz. Note that this is partly limited by the scenario studied, with quite good coverage.

## 3.9	Summary and discussion

Altogether, the above evaluations can be summarized as:

Observations:

- Across scenarios, uplink data is typically limiting, followed by uplink control. Downlink control and data are more robust. There is no obvious bottleneck among the control channels.

Based on this it is proposed to focus coverage enhancements efforts where most needed:

Proposals:

- Focus coverage enhancements efforts on uplink data and uplink control.

It should be noted that in this study the focus has been on coverage, and for most channels the configuration providing the best coverage has been selected. For example, aggregation level 16 is used for PDCCH. Some of these configurations are resource consuming and could form capacity bottlenecks. For capacity reasons it could be motivated to study efficiency enhancements of channels forming such bottlenecks.

Observation:

- Maintaining downlink coverage may require relatively high amounts of resource, and so techniques achieving downlink coverage that maintain spectral efficiency may be of interest.

# 4. Link Budget Tables based on Simulated Antenna Gains and Interference Margins

Link budgets are often presented in table format. While the simplicity of this format is attractive, the problem becomes how to set the values of components in a reasonable way. If we focus on identifying channels that form bottlenecks, isotropic loss (or coupling loss plus antenna gain) is a suitable metric, for which we need estimates of interference margins and antenna gains.   Isotropic gain can be further translated into cell range using suitable assumptions of fade margins, outdoor-to-indoor losses etc., and so is sufficient at least purely for the purpose of link budget calculation.

To estimate the interference margins and antenna gains from system level simulation, we identify a subset of users with SINRs corresponding to the requirement of the channel.   Here we define antenna gain as the sum of the transmit and receive antenna gains for a given UE drop.  Interference margin is defined as (I+N)/N, also called ‘rise over thermal’ or ‘IoT’, where I and N are the average interference and noise power for a given UE drop. For this subset we average the antenna gain and interference margins. See example for CSI on PUSCH in Figure 23 and message 2 in Figure 24.

As observed in [1], it is seen that for a given SINR, there is a variation in isotropic loss, antenna gain, and interference margin. The variation in isotropic loss is however smaller for the subset of users with SINR near the requirement. This indicates that isotropic loss can be a reasonable metric. The yellow and red markers indicate estimations of antenna gain and interference margins based on averages over the selected subset (+/-2 dB of the required SINR). The blue marker indicates the isotropic loss resulting from using these estimates to estimate the isotropic loss in link budget tables (like those presented in Table 9 and Table 10, further discussed below). We see that the estimation falls within the range of observed isotropic loss values. This is a good sign. To more accurately judge the ability to point out the right bottlenecks, in Figure 25 we plot the estimated isotropic loss as a function of the actual simulated outage. We see that for the most parts, a higher outage corresponds to a lower estimated supported isotropic loss. There is one exception, but the channels with the worst coverage are consistent in order. This indicates that the methodology can indeed identify the right bottlenecks (at least in the conditions simulated).

Using this methodology it is possible to create simple link budget tables like those in Table 9 and Table 10. Note that these also contain the maximum coupling loss, which is informative for channels with the same antenna gain.  Examining Table 9 for the 7km rural scenario at 700 MHz, we can see that the antenna gain is 12.9 dBi, which is roughly 4 dB less than the maximum antenna gain of 17 dBi.  The antenna gain does not vary between the physical channels, since adaptive beamforming is not used for these simulations.  This reduced antenna gain illustrates that effects such as element antenna patterns and tilt make it important to use a realistic value of antenna gain in link budgets.  Looking at the interference margin (‘IoT’) row, we see that the interference numbers vary significantly according to the physical channel: from 0.8 dB for PRACH to 3.2 dB for PUSCH.  This is despite the large cell spacing of 7 km, and illustrates that interference should not be neglected even in ‘coverage’ scenarios.

Examining results for the 700m urban scenario at 4 GHz in Table 10 where adaptive beamforming is used, the antenna gain has different behavior than for the rural scenario.  The antenna gain goes from a low of 12.9 dB for e.g. SSB to 19.0 dB for PDSCH, which is due to the different beamforming supported by the different channels.  Even though the full array is used for beamforming of PDSCH, its antenna gain is 7 dB less than the maximum of 26 dB.  This is again due to effects like tilt and antenna pattern.  The interference statistics vary less than in the 700 MHz case: from 0.2 to 1.6 dB, but are still significant, especially for UL channels.

Lastly, we should note that the values of tilt here are felt to be reasonable choices, but have not been optimized.  Further optimization could have different coverage and/or capacity.

Observations:

- Antenna gain observed at the limit of coverage may be substantially less than the maximum antenna gain.

- Antenna gains observed at the limit of coverage vary substantially between different channels.

- Interference margin varies significantly even in coverage limited scenarios.

- System level and link budget based methodologies tend to converge toward the same coverage bottlenecks if the methodologies incorporate suitable values of antenna gain and interference margin.

Proposal:

- Use antenna gain and interference margin values derived from system simulations in link budget analyses.

Figure 23. Estimated antenna gain and interference margin (IoT) for CSI on PUSCH for Urban Macro, 700m ISD, 4GHz.

Figure 24. Estimated antenna gain and interference margin (IoT) for message 2 for Urban Macro, 700m ISD, 4GHz.

Table 9. Link budget table for Rural Macro, 7km ISD, 700MHz.

|  | SSB | Msg2 PDCCH | Msg2 PDSCH | PDCCH | PDSCH Data | Msg1 PRACH | A/N PUCCH | CSI on PUCCH | Msg3 PUSCH | CSI on PUSCH | PUSCH Data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ptx [dBm] | 38.9 | 45.7 | 36.6 | 45.7 | 43.0 | 23.0 | 23.0 | 23.0 | 23.0 | 23.0 | 23.0 |
| Gant [dBi] | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 | 12.9 |
| Pnoise [dBm] | -101.4 | -94.6 | -103.6 | -94.6 | -97.3 | -108.6 | -116.4 | -116.4 | -113.4 | -116.4 | -113.4 |
| IoT[dB] | 2.2 | 1.1 | 1.1 | 1.1 | 1.1 | 0.8 | 1.8 | 1.8 | 3.2 | 3.2 | 3.2 |
| SNRreq[dB] | -9.8 | -9.2 | -9.0 | -9.2 | -8.9 | -13.9 | -8.1 | -5.0 | -10.0 | -6.0 | -8.7 |
| Max iso loss [dB] | 160.8 | 161.3 | 161.1 | 161.3 | 160.9 | 157.7 | 158.6 | 155.5 | 156.1 | 155.1 | 154.8 |
| Max coupling loss [dB] | 147.9 | 148.4 | 148.2 | 148.4 | 148.0 | 144.8 | 145.7 | 142.6 | 143.2 | 142.2 | 141.9 |

Table 10. Link budget table for Urban Macro, 700m ISD, 4GHz.

|  | SSB | Msg2 PDCCH | Msg2 PDSCH | PDCCH | PDSCH Data | Msg1 PRACH | A/N PUCCH | CSI on PUCCH | Msg3 PUSCH | CSI on PUSCH | PUSCH Data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ptx [dBm] | 41.7 | 48.5 | 39.4 | 48.5 | 53.0 | 23.0 | 23.0 | 23.0 | 23.0 | 23.0 | 23.0 |
| Gant [dBi] | 12.9 | 12.9 | 12.9 | 19.0 | 19.0 | 12.9 | 15.3 | 15.3 | 15.3 | 15.3 | 15.3 |
| Pnoise [dBm] | -98.4 | -91.6 | -100.6 | -91.6 | -87.1 | -105.6 | -113.4 | -113.4 | -110.4 | -113.4 | -103.4 |
| IoT[dB] | 0.2 | 0.1 | 0.4 | 0.1 | 0.1 | 0.5 | 0.7 | 0.7 | 1.6 | 1.6 | 1.6 |
| SNRreq[dB] | -12.9 | -11.3 | -9.2 | -11.3 | -9.6 | -20.0 | -9.1 | -6.6 | -13.9 | -6.2 | -0.5 |
| Max iso loss [dB] | 165.6 | 164.1 | 161.8 | 170.3 | 168.5 | 161.0 | 160.1 | 157.6 | 161.0 | 156.3 | 140.6 |
| Max coupling loss [dB] | 152.8 | 151.3 | 148.9 | 151.3 | 149.5 | 148.2 | 144.8 | 142.3 | 145.7 | 141.0 | 125.3 |

Figure 25. Estimated isotropic loss as a function of actual outage.

# Conclusion

In the previous sections we first made the following proposals related to link level evaluations:

Proposals:

- Consider two configurations as shown in Table 1,

- for low band a 2x10MHz FDD with 15kHz subcarrier spacing at 700 MHz with 2T/2R at base station and 1T/2R at terminal

- for mid band a 1x100MHz 3DL1UL TDD with 30kHz subcarrier spacing at 4GHz using up to 32T/32R at base station and 1T/4R at terminal

- For the baseline link level simulations, consider 2T for downlink 1T for uplink, whereas 2R and 4R are relevant for both uplink and downlink at 700MHz and 4GHz respectively, see also Table 2.

- For the link level simulations, initially consider TDL-A with medium spatial correlation, RMS delay spread 100ns. Later consider also 30ns delay spread and for the 700MHz case and 300ns for the 4GHz targeting rural macro and urban micro deployments (where a majority of users are indoors). Initially consider terminal speed of 3km/h and later consider also other higher speeds like 30km/h and 120km/h. This summarized in Table 3.

- For the coverage analysis for low band, consider, the channels and assumptions in Table 4.

- For the coverage analysis for mid band, consider, the channels and assumptions in Table 5.

Related to system-level evaluations and identification of bottlenecks, we then made the following Observations:

- For low-band, uplink data with datarates exceeding 30kbps has the worst coverage

- For low-band, uplink control channels are quite well balanced. There is no obvious bottleneck.

- For low-band, downlink control channels and data are quite well balanced and have better coverage than uplink channels.

- The 3GPP target from 38.913 on extreme coverage is met

- Very good coverage is achieved in the LMLC scenario. Uplink data is limiting for datarates exceeding 30kbps. Other potential bottlenecks are not so evident in this scenario.

- Good coverage can be achieved in the Extreme Long Range scenario.

- For mid-band, uplink data (PUSCH 1 or 2Mbps) has the worst coverage.

- For mid-band, uplink control channels are quite well balanced. CSI on PUCCH or PUSCH are slightly worse than others.

- For mid-band, downlink control channels and data (PDSCH 10Mbps) are quite well balanced and have better coverage than uplink channels. Message 2, for which limited CSI is assumed to be available for beamforming, is worse than the others, but better than uplink control.

- The bottlenecks in the rural macro scenario are the same as in the urban macro scenario. This points to that the bottlenecks could be consistent across scenarios.

- For mid-band, the Urban Macro scenario with 500m ISD has quite good coverage. Uplink data is limiting followed by uplink control. Bottlenecks are not as evident as in more challenging scenarios.

Regarding antenna gains and interference margins needed for link budget analyses and the relative behavior of system and link budget methodologies, we made the following observations and proposal.

Observations:

- Antenna gain observed at the limit of coverage may be substantially less than the maximum antenna gain.

- Antenna gains observed at the limit of coverage vary substantially between different channels.

- Interference margin varies significantly even in coverage limited scenarios.

- System level and link budget based methodologies tend to converge toward the same coverage bottlenecks if the methodologies incorporate suitable values of antenna gain and interference margin.

Proposal:

- Use antenna gain and interference margin values derived from system simulations in link budget analyses.

Altogether, we make the following observations and proposal:

Observation:

- Across scenarios, uplink data is typically limiting, followed by uplink control. Downlink control and data are more robust. There is no obvious bottleneck among the control channels.

- Maintaining downlink coverage may require relatively high amounts of resource, and so techniques achieving downlink coverage that maintain spectral efficiency may be of interest.

Proposal:

- Focus coverage enhancements efforts on uplink data and uplink control.

# References

R1-2004632, “Evaluation methodology for coverage enhancements”, Ericsson, RAN1 #101-e, May, 2020

3GPP TR 37.910, “Study on self evaluation towards IMT-2020 submission,” v16.1.0 (2019-09)

3GPP TS 36.101, v16.5.0 (2020-03)

3GPP TR 38.901, v16.1.0

3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies,” v15.0.0 (2018-06)