3GPP TSG RAN WG1 #101-e Meeting	R1-2005018

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.4.1

Source: 	Moderator (Samsung)

Title: 	Text Proposals from [101-e-NR-5G_V2X_NRSL-PHYstructure-01]

Document for:	Endorsement

Introduction

During WI and RAN1#101-e meeting, the following were agreed.

Agremeents: - For indication of PSFCH overhead in PSSCH TBS determination, one bit is used for N=2 or N=4 while no bit is used for N=0 or N=1. Agreements: - For PSSCH TBS determination, the following is used. - N_RE’ = N_sc^PRB (N_symb^sh - N_symb^PSFCH) - N_oh^PRB– N_RE’^DMRS - N_sc^PRB = 12 - N_symb^sh: the number of sidelink symbols within the slot, excluding one gap symbol and one first SL symbol (i.e, a total of two symbosls) in a slot - N_symb^PSFCH: 3 if indicated as “1” by SCI for PSFCH overhead, 0 if indicated as “0” by SCI for PSFCH overhead - For N=0, N_symb^PSFCH=0 - For N=1, N_symb^PSFCH=3 - N_oh^PRB as (pre-)configured by xOverhead - N_RE’^DMRS: the averaged DMRS overhead per PRB in PSSCH resource over a set of DMRS patterns as (pre-)configured for the resource pool by sl-PSSCH-DMRS-TimePattern - Discuss further interaction with PSCCH & PSFCH indication (if any) - N_RE = N_RE’ * n_PRB – N_RE^SCI1 – N_RE^SCI2 - n_PRB: total number of allocated PRBs for the PSSCH - N_RE^SCI1: total number of REs allocated for the corresponding PSCCH - N_RE^SCI2: total number of REs allocated for 2nd SCI Agreements: - The higher layer parameter sl-xOverhead for PSSCH TBS determination is (pre-)configured among {0, 3, 6, 9}. - How to handle the value of 0 as part of the signalling is up to RAN2 Agreements: - For average DMRS overhead, interaction of PSCCH and PSFCH indication - Alt 1. Consider PSFCH indication: averaged over only possible DMRS patterns as PSFCH indication. - Alt 2. Not consider PSFCH indication: averaged over all (pre-)configured patterns - Alt 2-1. Consider PSCCH resource - Alt 2-2. Not consider PSCCH resource Take Alt 2 + Alt 2-2 above. Agreements: - For the intermediate number of information bits (N_info) is obtained by N_info=N_RE*R*Qm*v, NR sidelink follows the conclusion of [101-e-NR-7.1CRs-11] in RAN1#101-e in calculation of N_info. Agreements: Use N_RE’^DMRS as below. sl-PSSCH-DMRS-TimePattern; N_RE’^DMRS. {2}; 12. {3}; 18. {4}; 24. {2,3}; 15. {2,4}; 18. {3,4}; 21. {2,3,4}; 18. Agreements: The 1st stage SCI indicates the PSFCH overhead for PSSCH TBS determination.

Summary of TPs

| TP1 | Target specification and version, Clauses affected | TS38.212 V16.1.0 Clause 8.3.1.1 |
| --- | --- | --- |
|  | Reason for change | It was agreed to indicate PSFCH overhead information in 1st SCI. |
|  | Summary of change | 1 or 0 bit of PSFCH overhead indication is added in SCI format 0-1. |
|  | Consequences if not approved | Incomplete specification |

| TP2 | Target specification and version, Clauses affected | TS38.214 V16.1.0 Clause 8.1.3.2 |
| --- | --- | --- |
|  | Reason for change | The current specification does not have TBS determination procedure for PSSCH, and the detailed procedure was agreed in RAN1#101-e. |
|  | Summary of change | TBS determination procedure for PSSCH is added. |
|  | Consequences if not approved | Incomplete specification |

Text Proposal 1 for TS38.212

----------------------------------Begin of text proposal for 38.212 Section 8.3.1.1---------------------------------

- 8.3.1.1	SCI format 0-1

SCI format 0-1 is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 0-1:

-	Priority – 3 bits as defined in clause x.x.x of [6, TS 38.214].

-	Frequency resource assignment – log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause x.x.x of [6, TS 38.214].

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause x.x.x of [6, TS 38.214].

-	Resource reservation period – log2(NreservPeriod) bits as defined in clause x.x.x of [6, TS 38.214], if higher layer parameter sl-MultiReserveResource is configured; 0 bit otherwise.

-	DMRS pattern – [x] bits as defined in clause 8.4.1.1.2 of [4, TS 38.211], if more than one DMRS patterns are configured by higher layer parameter sl-PSSCH-DMRS-TimePattern; 0 bit otherwise.

-	2nd-stage SCI format – [x] bits as defined in clause x.x.x of [6, TS 38.214].

-	Beta_offset indicator – [2] bits as provided by higher layer parameter sl-BetaOffsets2ndSCI.

-	Number of DMRS port – 1 bit as defined in Table 8.3.1.1-1.

-	Modulation and coding scheme – 5 bits as defined in clause 8.1.3 of [6, TS 38.214].

-    PSFCH overhead indication – 1 bit as defined clause 8.1.3.2 of [6, TS 38.214] if higher layer parameter sl-PSFCH-Period=2 or 4; 0 bit otherwise.

-	Reserved – [2 - 4] bits as determined by higher layer parameter sl-NumReservedBits, with value set to zero.

Table 8.3.1.1-1: Number of DMRS port

| Value of the Number of DMRS port field | Antenna ports |
| --- | --- |
| 0 | 1000 |
| 1 | 1000 and 1001 |

----------------------------------End of text proposal for 38.214 Section 8.3.1.1---------------------------------

Text Proposal 2 for TS38.214

----------------------------------Begin of text proposal for 38.214 Section 8.1.3.2---------------------------------

For the PSSCH assigned by SCI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, or a table other than Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤28, the UE shall first determine the TBS as specified below:

The UE shall first determine the number of REs (NRE) within the slot.

- A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NscRBNsymbsh-NsymbPSFCH-NohPRB-NREDMRS, where

  - NscRB=12 is the number of subcarriers in a physical resource block,

  - NsymbshNsymbslot = sl-lengthSLsymbols -2, where sl-lengthSLsymbols is the number of sidelink symbols within the slot provided by higher layers,

  - NsymbPSFCH = 3 if “PSFCH overhead indication” field of SCI format 1-A indicates “1”, and NsymbPSFCH = 0 otherwise, if higher layer parameter sl-PSFCH-Period is 2 or 4. If higher layer parameter sl-PSFCH-Period is 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3.

  - NohPRB is the overhead given by higher layer parameter sl-xOverhead,

  - NREDMRS is given by Table 8.1.3.2-xx according to higher layer parameter sl-PSSCH-DMRS-TimePattern.

Table 8.1.3.2-xx.  NREDMRS according to higher layer parameter sl-PSSCH-DMRS-TimePattern

| sl-PSSCH-DMRS-TimePattern | NREDMRS |
| --- | --- |
| {2} | 12 |
| {3} | 18 |
| {4} | 24 |
| {2,3} | 15 |
| {2,4} | 18 |
| {3,4} | 21 |
| {2,3,4} | 18 |

- A UE determines the total number of REs allocated for PSSCH ()NRE) by NRE=NRE'∙nPRB-NRESCI,1-NRESCI,2NRE= NRE'* nPRB, where

  - nPRB is the total number of allocated PRBs for the PSSCH,

  - NRESCI,1 is the total number of REs occupied by the PSCCH and PSCCH DMRS.

  - NRESCI,2 is the number of coded modulation symbols generated for 2nd-stage SCI transmission (prior to duplication for the 2nd layer, if present) according to Clause 8.4.4 of [TS38.212].

The UE determines TBS according to Steps 2), 3), and 4) in subclause 5.1.3.2

else if Table 5.1.3.1-2 is used and ,

-	the TBS is assumed to be as determined from the SCI for the same transport block using 0 ≤ IMCS ≤27..

else

-	the TBS is assumed to be as determined from the SCI for the same transport block using 0 ≤ IMCS ≤28.

----------------------------------- End of text proposal for 38.214 Section 8.1.3.2---------------------------------------