3GPP TSG RAN WG1 Meeting #101-e														R1-2005120

e-Meeting, May 25th – June 5th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0106 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of switched uplink operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| mmWork item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2020-06-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of switched uplink operation between two uplinks |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of a feature |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The switched uplink operation is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4, 6.1.6 (new), 6.2.1, 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | 38.101-1, 38.101-3, 38.133, 38.331, 38.213 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <omitted text>

## 5.4	UE CSI computation time

When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, the UE shall provide a valid CSI report for the n-th triggered report,

-	if the first uplink symbol to carry the corresponding CSI report(s) including the effect of the timing advance, starts no earlier than at symbol Zref, and

-	if the first uplink symbol to carry the n-th CSI report including the effect of the timing advance, starts no earlier than at symbol Z'ref(n),

where Zref is defined as the next uplink symbol with its CP starting  after the end of the last symbol of the PDCCH triggering the CSI report(s), and where Z'ref(n), is defined as the next uplink symbol with its CP starting after the end of the last symbol in time of the latest of: aperiodic CSI-RS resource for channel measurements, aperiodic CSI-IM used for interference measurements, and aperiodic NZP CSI-RS for interference measurement, when aperiodic CSI-RS is used for channel measurement for the n-th triggered CSI report, and where Tswitch is defined in subclause 6.4 and is applied only if Z1 of table 5.4-1 is applied.

### <omitted text>

### 6.1.6 	Uplink switching

The UE may omit uplink transmission during the uplink switching gap NTx1-Tx2 if the conditions defined in this sub-clause are met and the UE is configured with uplinkTxSwitchingPeriod-r16. The switching gap NTx1-Tx2 is indicated by UE capability UplinkTxSwitchingPeriod-r16:

-	If a UE indicated a capability for uplink switching with uplinkTxSwitchRequested-r16 for a band combination, and if it is for that band combination

- 	Configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), or

-	Configured with uplink carrier aggregation, or

-	Configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink.

the conditions under which the switching gap may be present and the location of the switchin gap are defined for each of the cases in sections 6.1.6.1, 6.1.6.2, and 6.1.6.3 respectively.

If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in subclause 5.3, subclause 5.4, subclause 6.2.1, subclause 6.4 and in subclause 9 of [6, TS 38.213].

The UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL,carrier1, µUL,carrier2), where the µUL,carrier1 corresponds to the subcarrier spacing of the uplink transmitted before the switching gap and the µUL,carrier2 corresponds to the subcarrier spacing of the uplink transmitted after the switching gap.

#### 6.1.6.1	Uplink switching for EN-DC

For a UE indicating a capability for uplink switching with uplinkTxSwitchRequested-r16 for a band combination, and if it is for that band combination configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC),  if the UE is configured with uplink switching with parameter uplinkTxSwitchingPeriod-r16,

- for the UE configured with option1 by the parameter uplinkTxSwitchingOption, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s):

- when the UE is to transmit an NR uplink that takes place after an E-UTRA uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

- when the UE is to transmit an E-UTRA uplink that takes place after an NR uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

- the UE is not expected to transmit simultaneously on the NR uplink and the E-UTRA uplink. If the UE is scheduled or configured to transmit any NR uplink transmission overlapping with an E-UTRA uplink transmission, the NR uplink transmission is dropped,

- for the UE configured with option2 by the parameter uplinkTxSwitchingOption, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s):

- when the UE is to transmit an NR two-port uplink that takes place after an E-UTRA uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.	.

- when the UE is to transmit an E-UTRA uplink that takes place after an NR two-port uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

- the UE is not expected to transmit simultaneously a two- port transmission on the NR uplink and the E-UTRA uplink.

-	in all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

-	when the UE is configured with tdm-PatternConfig-r15 or by tdm-PatternConfig-r16

-	for the E-UTRA subframes designated as uplink by the configuration, the UE assumes the operation state in which one-port E-UTRA uplink can be transmitted.

-	for the E-UTRA subframes other than the ones designated as uplink by the configuration, the UE assumes the operation state in which two-port NR uplink can be transmitted.

NTx1-Tx2

NTx1-Tx2

#### 6.1.6.2	Uplink switching for Carrier Aggregation

For a UE indicating a capability for uplink switching with uplinkTxSwitchRequested-r16 for a band combination, and if it is for that band combination configured with uplink carrier aggregation:

-	If the UE is configured with uplink switching with parameter uplinkTxSwitchingPeriod-r16, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s):

-	When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 1-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	When the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to  transmit for the duration of NTx1-Tx2 on any of the two carriers.

-    For the UE configured with option1 by the parameter uplinkTxSwitchingOption, when the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier, then the UE is not expected to  transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	For the UE configured with option2 by the parameter uplinkTxSwitchingOption, when the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on the same uplink carrier and the UE is under the operation state in which 2-port transmission cannot be supported in the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	For the UE configured with option2 by the parameter uplinkTxSwitchingOption, when the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier, and any transmission on another uplink carrier.

-	In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

#### 6.1.6.3	Uplink switching for Supplementary Uplink

For a UE indicating a capability for uplink switching with uplinkTxSwitchRequested-r16 for a band combination, and if it is for that band combination configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink:

-	If the UE is configured with uplink switching with parameter uplinkTxSwitchingPeriod-r16,

-	If the UE is to transmit any uplink channel or signal on a different uplink from the preceding transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the uplink channel or signal and Toffset is the preparation procedure time of the transmission occasion of the uplink channel or signal given in subclause 5.3, subclause 5.4, subclause 6.2.1, subclause 6.4 and in subclause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to transmit on any of the two uplinks.

NTx1-Tx2

NTx1-Tx2

-	In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

## <omitted text>

### 6.2.1	UE sounding procedure

## <omitted text>

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource is set to 'aperiodic':

-	the UE receives a configuration of SRS resource sets,

-	the UE receives a downlink DCI, a group common DCI, or an uplink DCI based command where a codepoint of the DCI may trigger one or more SRS resource set(s). For SRS in a resource set with usage set to 'codebook' or 'antennaSwitching', the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 + Tswitch Otherwise, the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 + Tswitch+14. The minimal time interval in units of OFDM symbols is counted based on the minimum subcarrier spacing between the PDCCH and the aperiodic SRS.

-	Tswitch is defined in subclause 6.4.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and except when SRS is configured with the higher layer parameter [SRS-for-positioning], the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot , if UE is configured with CA-slot-offset for at least one of the triggered and triggering cell,  Ks =, otherwise, and where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	Nslot, offset, PDCCHCAand μoffset,PDCCHare the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured CA-slot-offset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured CA-slot-offset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and when SRS is configured with the higher layer parameter [SRS-for-positioning], the UE transmits every aperiodic SRS resource in each of the triggered SRS resource set(s) in slot  where

-	k is configured via higher layer parameter slotOffset for each aperiodic SRS resource in each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	Nslot, offsetCA and the  for the {scheduling, scheduled} carrier pair is defined in [4, TS 38.211] clause 4.5.

-	if the UE is configured with the higher layer parameter spatialRelationInfo containing the ID of a reference 'ssb-Index', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo contains the ID of a reference 'csi-RS-Index', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, or of the latest reference aperiodic CSI-RS. If the higher layer parameter spatialRelationInfo contains the ID of a reference 'srs', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS or of the reference semi-persistent SRS or of the reference aperiodic SRS. When the SRS is configured by the higher layer parameter [SRS-for-positioning] and if the higher layer parameter spatialRelationInfo contains the ID of a reference ‘DL-PRS-ResourceId’, the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS.

-	when a UE receives an spatial relation update command, as described in clause 6.1.3.xx of [10, TS 38.321], for an SRS resource, and when the HARQ-ACK corresponding to the PDSCH carrying the update command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on updating spatial relation for the SRS resource shall be applied for SRS transmission starting from the first slot that is after slot n+3Nslotsubframe,µ. [The update command contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the updated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the update command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the update command if present, same serving cell and bandwidth part as the SRS resource set otherwise.] When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', the UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set.

## <omitted text>

## 6.4	UE PUSCH preparation procedure time

If the first uplink symbol in the PUSCH allocation for a transport block, including the DM-RS, as defined by the slot offset K2 and the start and length indicator SLIV of the scheduling DCI and including the effect of the timing advance, is no earlier than at symbol L2, where L2 is defined as the next uplink symbol with its CP starting after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, then the UE shall transmit the transport block.

-	N2 is based on µ of Table 6.4-1 and Table 6.4-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µDL, µUL) resulting with the largest Tproc,2, where the µDL corresponds to the subcarrier spacing of the downlink with which the PDCCH carrying the DCI scheduling the PUSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the PUSCH is to be transmitted, and κ is defined in clause 4.1 of [4, TS 38.211].

-	If the first symbol of the PUSCH allocation consists of DM-RS only, then d2,1 = 0, otherwise d2,1 = 1.

-	If the UE is configured with multiple active component carriers, the first uplink symbol in the PUSCH allocation further includes the effect of timing difference between component carriers as given in [11, TS 38.133].

-	If the scheduling DCI triggered a switch of BWP, d2,2 equals to the switching time as defined in [11, TS 38.133], otherwise d2,2=0.

-	For a UE that supports capability 2 on a given cell, the processing time according to UE processing capability 2 is applied if the high layer parameter processingType2Enabled in PUSCH-ServingCellConfig is configured for the cell and set to enable,

-	If the PUSCH indicated by the DCI is overlapping with one or more PUCCH channels, then the transport block is multiplexed following the procedure in clause 9.2.5 of [6, TS 38.213], otherwise the transport block is transmitted on the PUSCH indicated by the DCI.

-	If uplink switching gap is triggered as defined in subclause 6.1.6,  equals to the switching gap duration and for the UE configured with uplinkTxSwitchingOption2 µUL=min(µUL,carrier1, µUL,carrier2), otherwise .

Otherwise the UE may ignore the scheduling DCI.

The value of  is used both in the case of normal and extended cyclic prefix.

Table 6.4-1: PUSCH preparation time for PUSCH timing capability 1

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |

Table 6.4-2: PUSCH preparation time for PUSCH timing capability 2

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 5 |
| 1 | 5.5 |
| 2 | 11 for frequency range 1 |