3GPP TSG RAN WG1 Meeting #101-e	R1-2005179

e-Meeting, May 25th- June 05th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1332 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to LTE-based 5G terrestrial broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast-Core |  |  |  |  |  | Date: |  |  | 2020-06-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction to clarify TBS scaling and semistatic CFI in MIB MBMS for LTE-based 5G terrestrial broadcast. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - [100b-e-LTE-TerrBcast-02] TBS scaling Include the endorsed text proposal 3 of R1-2002855 clarifying TBS scaling in Section 11.1 - [101-e-LTE-TerrBcast-02] Issues related to control region and CFI Include the endorsed text proposal 2 of R1-2004879 with editorial correction to RRC parameter name and clarifying semistatic CFI is only applicable if the value in MIB is non-zero in Section 9.1.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of misinterpretation of TBS scaling functionality, and inconsitent specification of semistatic CFI in MIB MBMS for LTE-based 5G terrestrial broadcast. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3, 11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.211 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 9.1.3	Control Format Indicator (CFI) assignment procedure

For a serving cell, if a UE is configured with higher layer parameter semiStaticCFI-SlotSubslotNonMBSFN, the UE shall assume the CFI is equal to the value of the higher layer parameter semiStaticCFI-SlotSubslotNonMBSFN for non-MBSFN subframes for receiving physical downlink shared channel with slot/subslot duration.

<Unchanged parts are omitted>

For a MBMS-dedicated cell, if a UE is configured with higher layer parameter semiStaticCFI-MBMS included in MasterInformationBlock-MBMS, the UE shall assume the CFI is equal to the value of the higher layer parameter semiStaticCFI-MBMS for non-MBSFN subframes if a non-zero value is indicated by semiStaticCFI-MBMS.

<Unchanged parts are omitted>

# 11	Physical Multicast Channel (PMCH) related procedures

## 11.1	UE procedure for receiving the PMCH

A UE is not expected to receive PMCH with  (defined in [3]) other than  in an MBSFN subframe with non-zero-size non-MBSFN region.

A UE is not expected to receive PMCH with in an MBSFN subframe with zero-size non-MBSFN region.

The UE shall decode the PMCH when configured by higher layers. The UE may assume that an eNB transmission on the PMCH is performed according to Subclause 6.5 of [3].

The for the PMCH is configured by higher layers. To determine the modulation order () and TBS index () used in the PMCH the UE shall

-	if the UE is configured by higher layers to decode the PMCH based on QPSK, 16QAM, 64QAM, and 256QAM

-	if the PMCH is with and the UE is configured with higher layer parameter timeSeperation with value '4'

-	use and Table 11.1-2 for determining the modulation order () and TBS index ()

-	otherwise,

-	useand Table 7.1.7.1-1A for determining the modulation order () and TBS index ()

-	otherwise,

-	if the PMCH is with and the UE is configured with higher layer parameter timeSeperation with value '4'

-	use and Table 11.1-1 for determining the modulation order () and TBS index ()

-	otherwise,

-	use and Table 7.1.7.1-1 for determining the modulation order () and TBS index ().

The UE shall then follow the procedure in Subclause 7.1.7.2.1 to determine the transport block size, assuming is equal to. For PMCH with  the UE shall scale the derived transport block size by , then round to the closest valid transport block size in the union of Table 7.1.7.2.1-1 and Table 7.1.7.2.4-1 by including, in the rounding procedure, only the TBS entries in Table 7.1.7.2.1-1 with ITBS≤33B and the entries in 7.1.7.2.4-1 for which the TBS_L1 is present in Table 7.1.7.2.1-1 with ITBS≤33B if the UE is configured by higher layers to decode the PMCH based on QPSK, 16QAM, 64QAM, and 256QAM; and only the TBS entries in Table 7.1.7.2.1-1 with ITBS≤26A and the entries in 7.1.7.2.4-1 for which the TBS_L1 is present in Table 7.1.7.2.1-1 with ITBS≤26A otherwise. In case the scaled TBS has the same distance to two valid transport block sizes, it is rounded to the larger transport block size. The UE shall set the redundancy version to 0 for the PMCH.

A UE may optionally report parameters mbms-MaxBW (T), mbms-ScalingFactor1dot25 (A(1.25) , mbms-ScalingFactor7dot5 (A(7.5), mbms-ScalingFactor2dot5 (A(2.5), and mbms-ScalingFactor0dot37 (A(0.37) to indicate a limitation on baseband capability provided by the following inequality

T≥c=1C(RcBcIc(15kHz +A(7.5BcIc(7.5kHz +A(2.5 BcIc(2.5kHz +A(1.25 BcIc(1.25kHz +A(0.37 BcIc(0.37kHz )

where

-	T is the maximum bandwidth capability of the UE, indicated by mbms-MaxBW:

-	if mbms-MaxBW is set to implicitValue, then

T=roundMaximum number of DL-SCH transport block bits received within a TTIMaximum TBS for 2 layers for the maximum supported modulation scheme× 20MHz

where the numerator and the denominator correspond to the corresponding columns of the maximum indicated ue-CategoryDL in [12].

-	if mbms-MaxBW is set to explicitValue, then T=explicitValue× 40MHz

-	C is the number of serving cells the UE is configured with, or receiving PMCH from.

-	Rc is the number of spatial layers the UE can receive in the c-th serving cell according to MIMO-CapabilityDL.

-	Bc is the bandwidth (in MHz) of the c-th serving cell.

-	If the UE is receiving PMCH with 1.25kHz numerology in serving cell c, then

-	Ic(0.37kHz =0,Ic(1.25kHz =1, Ic(2.5kHz =0,Ic(7.5kHz =0,Ic(15kHz =0

-	else, if the UE is receiving PMCH with 7.5kHz numerology in serving cell c, then

-	Ic(0.37kHz =0,Ic(1.25kHz =0, Ic(2.5kHz =0,Ic(7.5kHz =1,Ic(15kHz =0

-	else, if the UE is receiving PMCH with 2.5kHz numerology in serving cell c, then

-	Ic(0.37kHz =0,Ic(1.25kHz =0, Ic(2.5kHz =1,Ic(7.5kHz =0,Ic(15kHz =0

-	else, if the UE is receiving PMCH with 0.37kHz numerology in serving cell c, then

-	Ic(0.37kHz =1,Ic(1.25kHz =0, Ic(2.5kHz =0,Ic(7.5kHz =0,Ic(15kHz =0

-	else,

-	Ic(0.37kHz =0,Ic(1.25kHz =0, Ic(2.5kHz =0,Ic(7.5kHz =0,Ic(15kHz =1

Table 11.1-1: Modulation and TBS index table for PMCH

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 4 | 10 |
| 12 | 4 | 11 |
| 13 | 4 | 12 |
| 14 | 4 | 13 |
| 15 | 4 | 14 |
| 16 | 4 | 15 |
| 17 | 4 | 16 |
| 18 | 4 | 17 |
| 19 | 4 | 18 |
| 20 | 4 | 19 |
| 21 | 6 | 19 |
| 22 | 6 | 20 |
| 23 | 6 | 21 |
| 24 | 6 | 22 |
| 25 | 6 | 23 |
| 26 | 6 | 24 |
| 27 | 6 | 25 |
| 28 | 6 | 26/26A |
| 29 | 2 | reserved |
| 30 | 4 |  |
| 31 | 6 |  |

Table 11.1-2: Modulation and TBS index table 2 for PMCH

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 2 |
| 2 | 2 | 4 |
| 3 | 2 | 6 |
| 4 | 2 | 8 |
| 5 | 2 | 10 |
| 6 | 4 | 11 |
| 7 | 4 | 12 |
| 8 | 4 | 13 |
| 9 | 4 | 14 |
| 10 | 4 | 15 |
| 11 | 4 | 16 |
| 12 | 4 | 17 |
| 13 | 4 | 18 |
| 14 | 4 | 19 |
| 15 | 6 | 20 |
| 16 | 6 | 21 |
| 17 | 6 | 22 |
| 18 | 6 | 23 |
| 19 | 6 | 24 |
| 20 | 6 | 25 |
| 21 | 8 | 27 |
| 22 | 8 | 28 |
| 23 | 8 | 29 |
| 24 | 8 | 30 |
| 25 | 8 | 31 |
| 26 | 8 | 32 |
| 27 | 8 | 33/33A/33B |
| 28 | 2 | reserved |
| 29 | 4 |  |
| 30 | 6 |  |
| 31 | 8 |  |